5	Self evaluation of eMBB technical performance

[Editor’s note: The grouping of the technical performance requirements for eMBB usage scenario are based on Report ITU-R M.2410 “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”. However, it does not imply that each of the following technical performance requirements is only related to eMBB. It is just the minimum requirement that shall be evaluated under eMBB. This should also be noted for Section 6 and 7.]

5.5	User experienced data rate

As defined in Report ITU-R M.2410, user experienced data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput. User throughput (during active time) is defined as the number of correctly received bits, i.e. the number of bits contained in the service data units (SDUs) delivered to Layer 3, over a certain period of time.

5.5.1	NR

User experienced data rate for NR are evaluated under Dense Urban – eMBB test environment.

5.5.1.1	Dense Urban – eMBB

For Dense Urban – eMBB test environment, single-band single-layer case using evaluation configuration A (carrier frequency = 4 GHz), and multi-band case using evaluation configuration C as defined in Report ITU-R M.2412 are considered in evaluation. Detailed evaluation assumptions and results can be found in RP-181817.

5.5.1.1.1	Evaluation configuration A (CF = 4 GHz)

For evaluation configuration A (single-band case), user experienced data rate for NR is evaluated based on 5th percentile user spectral efficiency, using the analytical way as provided in Report ITU-R M.2412.

The evaluation results of DL user experienced data rate for NR FDD and NR TDD for evaluation configuration A are provided in Table 5.5.1.1.1-1.

It is assumed that for FDD and TDD with 15 kHz SCS, a component carrier with 40 MHz is used; and for TDD with 30 kHz SCS, a component carrier with 100 MHz is used. Multiple component carriers are aggregated to achieve the DL target user experienced data rate. The assumed aggregated system bandwidth is given in Table 5.5.1.1.1-1.

It is observed that both NR FDD and TDD fulfill the DL user experienced data rate requirement in evaluation configuration A.

Table 5.5.1.1.1-1 DL user experienced data rate for NR in Dense Urban – eMBB 
(Evaluation configuration A, CF=4 GHz)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing | ITU Requirement [Mbps] | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Number of samples | Assumed system bandwidth [MHz] | User exp. data rate [Mbps] | Number of samples | Assumed system bandwidth [MHz] | User exp. data rate [Mbps] |
| 32x4 MU-MIMO Type II Codebook (128Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | 100 | 8 | 240 | 108.99 | 4 | 240 | 113.48 |
| 32x4 MU-MIMO Type I Codebook (128Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | 100 | 2 | 280 | 112.86 | / | / | / |
| 32x4 MU-MIMO Type II Codebook (256Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;1,16) | 15 | 100 | 1 | 160 | 104.66 | / | / | / |
| 32x4 MU-MIMO Type II Codebook (256Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (16,8,2,1,1;2,8) | 15 | 100 | 1 | 200 | 114.75 | 1 | 200 | 120.55 |
| 32x4 MU-MIMO Ideal CSI feedback (128Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | 100 | / | / | / | 1 | 160 | 106.21 |
| 4x4 MU-MIMO Type II Codebook (128Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,1) | 15 | 100 | 1 | 240 | 112.64 | / | / | / |
| 32x8 MU-MIMO Type II Codebook (256Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (16,8,2,1,1;2,8) | 15 | 100 | 1 | 280 | 110.35 | / | / | / |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing | Frame structure | ITU Requirement [Mbps] | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | Assumed system bandwidth [MHz] | User exp. data rate [Mbps] | Number of samples | Assumed system bandwidth [MHz] | User exp. data rate [Mbps] |
| 32x4 MU-MIMO, 4T SRS (128Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 30 | DDDSU, S=10DL:2GP:2UL | 100 | 1 | 300 | 110.78 | 2 | 300 | 129.73 |
| 32x4 MU-MIMO, 4T SRS (128Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 30 | DDDSU, S=10DL:2GP:2UL | 100 | 1 | 300 | 137.16 | / | / | / |
| 32x4 MU-MIMO, 4T SRS (128Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | DDDSU, S=10DL:2GP:2UL | 100 | 1 | 320 | 107.33 | 1 | 320 | 111.28 |
| 64x4 MU-MIMO, 4T SRS (192Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (12,8,2,1,1;4,8) | 30 | DDDSU, S=10DL:2GP:2UL | 100 | 1 | 300 | 144.34 | 2 | 200 | 102.80 |
| 64x4 MU-MIMO, 4T SRS (192Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (12,8,2,1,1;4,8) | 15 | DDDSU, S=10DL:2GP:2UL | 100 | 1 | 240 | 100.87 | 1 | 240 | 102.57 |
| 32x4 MU-MIMO, Type II Codebook (256Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;1,16) | 15 | DSUUD S=6DL:2GP:6UL | 100 | 1 | 400 | 105.37 | / | / | / |
| 32x4 MU-MIMO, Type II Codebook (256Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (16,8,2,1,1;2,8) | 15 | DSUUD S=11DL:1GP:2UL | 100 | 1 | 320 | 112.20 | 1 | 320 | 111.78 |
| 32x4 MU-MIMO, 4T SRS (128Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | DSUUD S=11DL:1GP:2UL | 100 | 2 | 440 | 101.81 | 1 | 400 | 106.75 |
| 32x4 MU-MIMO, Type II Codebook (256Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (16,8,2,1,1;2,8) | 30 | DSUUD S=11DL:1GP:2UL | 100 | 1 | 300 | 111.50 | 1 | 300 | 112.58 |
| 64x4 MU-MIMO, 4T SRS (192Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (12,8,2,1,1;4,8) | 15 | DSUUD S=11DL:1GP:2UL | 100 | 1 | 240 | 110.65 | 1 | 240 | 111.12 |
| 64x4 MU-MIMO, 4T SRS (256Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (16,8,2,1,1;4,8) | 15 | DSUUD S=11DL:1GP:2UL | 100 | 1 | 240 | 117.95 | 1 | 200 | 105.27 |
| 64x4 MU-MIMO, 4T SRS (192Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (12,8,2,1,1;4,8) | 30 | DSUUD S=11DL:1GP:2UL | 100 | 1 | 200 | 101.60 | 1 | 200 | 103.77 |
| 64x4 MU-MIMO, 4T SRS (192Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (12,8,2,1,1;4,8) | 30 | DSUUD S=6DL:2GP:6UL | 100 | 1 | 300 | 114.97 | 1 | 300 | 109.22 |
| 64x4 MU-MIMO, 4T SRS (256Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (16,8,2,1,1;4,8) | 30 | DSUUD S=11DL:1GP:2UL | 100 | 1 | 200 | 108.41 | 1 | 200 | 114.06 |
| 64x8 MU-MIMO, 4T SRS (256Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,32,2,1,1;1,32) | 15 | DDDDD | 100 | 1 | 320 | 114.11 | / | / | / |
| 64x4 MU-MIMO, 4T SRS (192Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (12,8,2,1,1;4,8) | 30 | DDDDD DDSUU S=6DL:4GP:4UL | 100 | 1 | 300 | 124.58 | / | / | / |
| 128x4, MU-MIMO, 4T SRS; (M,N,P, Mg,Ng,Mp,Np) = (8,16,2,1,1;4,16) | 30 | DDSU S=10DL:2GP:2UL | 100 | / | / | / | 1 | 360 | 111.91 |

The evaluation results of UL user experienced data rate for NR FDD and NR TDD for evaluation configuration A are provided in Table 5.5.1.1.1-2.

It is assumed that for FDD with 15 kHz SCS, a component carrier with 40 MHz is used; for TDD with 15 kHz SCS, a component carrier with 50 MHz is used; and for TDD with 30 kHz SCS, a component carrier with 100 MHz is used. Multiple component carriers are aggregated to achieve the UL target user experienced data rate. The assumed aggregated system bandwidth is given in Table 5.5.1.1.1-2.

It is observed that both NR FDD and TDD fulfill the UL user experienced data rate requirement in evaluation configuration A.

Table 5.5.1.1.1-2 UL user experienced data rate for NR in Dense Urban – eMBB 
(Evaluation configuration A, CF=4 GHz)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing | ITU Requirement [Mbps] | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Number of samples | Assumed system bandwidth [MHz] | User exp. data rate [Mbps] | Number of samples | Assumed system bandwidth [MHz] | User exp. data rate [Mbps] |
| 2x16 SU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;1,8) | 15 | 50 | 4 | 240 | 59.78 | 1 | 240 | 52.46 |
| 2x32 SU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | 50 | 2 | 160 | 58.49 | 2 | 160 | 50.68 |
| 4x32 SU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | 50 | 1 | 120 | 54.18 | 1 | 160 | 59.93 |
| 4x32 MU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;1,16) | 15 | 50 | 1 | 120 | 63.33 | / | / | / |
| 4x32 MU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (16,8,2,1,1;2,8) | 15 | 50 | 1 | 200 | 55.46 | / | / | / |
| 4x16 SU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;1,8) | 15 | 50 | 1 | 160 | 52.80 | 1 | 160 | 55.84 |
| 16x16 SU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;1,8) | 15 | 50 | / | / | / | 1 | 240 | 58.49 |
| 4x4 SU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,1) | 15 | 50 | 1 | 280 | 54.50 | / | / | / |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing | Frame structure | ITU Requirement [Mbps] | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | Assumed system bandwidth [MHz] | User exp. data rate [Mbps] | Number of samples | Assumed system bandwidth [MHz] | User exp. data rate [Mbps] |
| 2x32 SU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 30 | DDDSU S=10DL:2GP:2UL | 50 | 1 | 800 | 53.68 | 2 | 600 | 54.77 |
| 2x32 SU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | DDDSU S=10DL:2GP:2UL | 50 | 1 | 800 | 53.94 | / | / | / |
| 2x64 SU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (12,8,2,1,1;4,8) | 30 | DDDSU S=10DL:2GP:2UL | 50 | 1 | 600 | 51.80 | 1 | 700 | 55.27 |
| 2x64 SU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (12,8,2,1,1;4,8) | 15 | DDDSU S=10DL:2GP:2UL | 50 | 1 | 600 | 52.24 | 1 | 700 | 50.11 |
| 4x32 SU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 30 | DDDSU S=10DL:2GP:2UL | 50 | 1 | 700 | 56.21 | 1 | 700 | 52.61 |
| 4x32 SU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | DDDSU S=10DL:2GP:2UL | 50 | 1 | 700 | 54.96 | 1 | 800 | 55.98 |
| 4x32 MU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;1,16) | 15 | DSUUD S=6DL:2GP:6UL | 50 | 1 | 300 | 73.15 | / | / | / |
| 4x16 SU-MIMO, Non-codebook based,OFDMA; gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;1,8) | 15 | DSUUD S=11DL:1GP:2UL | 50 | 1 | 500 | 58.82 | 1 | 400 | 55.07 |
| 4x16 SU-MIMO, Non-codebook based,OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;1,8) | 30 | DSUUD S=11DL:1GP:2UL | 50 | 1 | 500 | 57.08 | 1 | 400 | 61.35 |
| 4x16 SU-MIMO, codebook based,OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;1,8) | 15 | DSUUD S=11DL:1GP:2UL | 50 | 1 | 500 | 54.90 | 1 | 500 | 60.35 |
| 4x16 SU-MIMO, codebook based,OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;1,8) | 30 | DSUUD S=11DL:1GP:2UL | 50 | 1 | 500 | 50.11 | 1 | 400 | 50.89 |
| 8x64 MU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (4,32,2,1,1;1,32) | 15 | UUUUU | 50 | 1 | 300 | 50.70 | / | / | / |
| 2x32 SU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (12,8,2,1,1;4,4) | 30 | DSUUD S=6DL:2GP:6UL | 50 | / | / | / | 1 | 200 | 55.44 |
| 2x64 SU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (12,8,2,1,1;4,8) | 30 | DSUUD S=6DL:2GP:6UL | 50 | 1 | 200 | 59.63 | 1 | 200 | 57.54 |
| 4x128, MU-MIMO, Codebook based, OFDMA; (M,N,P, Mg,Ng,Mp,Np) = (8,16,2,1,1;4,16) | 30 | DDSU S=10DL:2GP:2UL | 50 | / | / | / | 1 | 900 | 50.50 |

5.5.1.1.2	Evaluation configuration B (CF = 30 GHz)

TBD

5.5.1.1.3	Evaluation configuration C

For evaluation configuration C (multi-band), the UL user experienced data rate is evaluated using a TDD band (on 30 GHz) and a supplementary uplink (SUL) band (on 4 GHz). System level simulation is used. The “cell edge” users on TDD band are offloaded to SUL band. The evaluation results of NR TDD+SUL are provided in Table 5.5.1.1.3-1.

In the evaluation, it is assumed that for FDD with 15 kHz SCS, a component carrier with 20 MHz is used; for TDD with 60 kHz SCS, a component carrier with 200 MHz is used. Multiple component carriers on either TDD band or SUL band are aggregated to achieve the UL target user experienced data rate. The assumed aggregated system bandwidth is given in Table 5.5.1.1.3-1.

Table 5.5.1.1.3-1 UL user experienced data rate for NR in Dense Urban – eMBB 
(Evaluation configuration C)

(a) NR TDD+SUL, Macro layer only

| Scheme and antenna configuration | Sub-carrier spacing | Frame structure | ITU Requirement [Mbps] | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | Assumed system bandwidth [MHz] | User exp. data rate [Mbps] | Number of samples | Assumed system bandwidth [MHz] | User exp. data rate [Mbps] |
| 4 GHz (SUL band): 2x32 SU-MIMO, Codebook based OFDMA 30 GHz (TDD band): 8x32 SU-MIMO, Codebook based, OFDMA (2 panel@UE) 50% offload to SUL band | 4 GHz: 15 30 GHz: 60 | 4 GHz: full uplink; 30 GHz: DDDSU S=10DL:2GP:2UL | 50 | 1 | 4 GHz: 100 30 GHz: 1200 | 53.13 | 1 | 4 GHz: 100 30 GHz: 1200 | 51.39 |