3GPP TSG RAN meeting #81										RP-181845

Gold Coast, Australia, September 10-13, 2018

## Status Report to TSG

Agenda item:			10.2.17

| WI / SI Name | UE requirements for network-based CRS interference mitigation for LTE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| included in this status report | Core part: | Yes | Perf. part: | Yes |  | Testing part: | No |
| Acronym | LTE_NW_CRS_IM |  |  |  |  |  |  |
| Unique ID | 761001 |  |  |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-181315 |  |  |  |  |  |  |
| Target Completion Date | Core part: 09/2018 |  |  |  | Performance part: 12/2018 |  |  |
| Is the WI/SI complete? | Core part: Yes |  |  |  | Performance Part: No |  |  |

Source:

| Leading WG |  | TSG RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Iana SIOMINA |
|  | Company | Ericsson |
|  | Email | iana (dot) siomina (at) ericsson (dot) com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2 agreed on the CR to TS 36.331 introducing network-based CRS interference mitigation:

- R2-1813088, Implementing network-based CRS interference mitigation, Ericsson.

In the TS 36.331 ASN.1 review during RAN#103, it was agreed to merge the configuration and cell barring parameters for network-based CRS interference mitigation and CRS muting, see [11]. The changes were included in the CR above.

#### 2.2.2	Remaining Open issues

No open issues.

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4 agreed on a CR to TS 36.133, which is finalizing the open issues in the Core part of the WI:

- R4-1811735, Applicability requirement for network-based CRS-IM, Qualcomm Inc.

RAN4 agreed on a WF for the UE performance part:

- R4-1811864, WF for demodulation performance requirements for network-based CRS interference mitigation, Ericsson.

The WF contains the following agreements:

- Define performance requirements for the UE supporting nw-BasedCRS-InterferenceMitigation-r15

- At least define test case for TM3. FFS for TM9

- For the CRS on/off configuration of interference cell

  - Option 1: CRS is muted based on a pattern which may change with some probability

  - Option 2: Mutable CRS is always muted

  - Other options are not precluded

- Companies are encouraged to provide detail setup and simulation assumption in the next meeting, including the reference receiver assumptions

#### 2.4.2	Remaining Open issues

- Finalize UE performance requirements for UE capable of network-based CRS interference mitigation

- Specify if necessary RRM test cases

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

#### 2.5.4	Estimated Level of Completion

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT. Based on the joint RAN-SA#80 session the items relevant for this section are shown in http://www.3gpp.org/ftp/tsg_sa/TSG_SA/TSGS_80/Docs/SP-180561.zip and summarized below:

| Work Area | SA WIDs | Rapporteurs | RAN WIDs | Rapporteurs |
| --- | --- | --- | --- | --- |
| URLLC for 5G | FS_5G_URLLC FS_Vertical_LAN cyberCAV | SA2: Hui.ni@huawei.com SA2: Devaki.chandramouli@nokia.com SA1: joachim.walewski@siemens.com | FS_NR_L1enh_URLLC FS_NR_unlic FS_NR_IIOT | RAN1: chengyan.cheng@huawei.com RAN1: jingsun@qti.qualcomm.com RAN2: dawid.koziol@nokia.com |
| V2X for 5G | FS_eV2XARC | SA2: laeyoung.kim@lge.com SA1: sungduck.chun@lge.com | FS_NR_V2X | RAN1: Hanbyul.seo@lge.com Matthew.webb@huawei.com |
| Positioning | FS_eLCS 5G_HYPOS | SA2: aiming@catt.cn SA1: lionel.ries@esa.int | FS_NR_POS | RAN1:alexey.khoryaev@INTEL.COM asbjorn.grovlen@ERICSSON.COM |
| UE Capabilities | FS_RACS | SA2: harisz@qti.qualcomm.com | FS_RACS_RAN | RAN2: alex.hsu@mediatek.com |
| 5G Satellite Aspects | FS_5GSAT_ARCH 5GSAT | SA2: cyril.michel@thalesaleniaspace.com SA1: cyril.michel@thalesaleniaspace.com | FS_NR_NTN_solutions | RAN3: nicolas.chuberre@thalesaleniaspace.com |

RAN and SA rapporteurs are requested to fill in these clauses jointly.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

[1] R4-1810544, Discussion on the remaining open issues in the network-based CRS interference mitigation, Qualcomm Inc., Aug. 2018.

[2] R4-1809872, On WU and CD subframes for NW based CRS-IM, Intel Corporation, Aug. 2018.

[3] R4-1810639, Remaining on the warm-up and cool-down in network based CRS IM, Huawei, HiSilicon, Aug. 2018.

[4] R4-1811011, On remaining issues for RRM with network-based CRS interference mitigation, Ericsson, Aug. 2018.

[5] R4-1811241, Discussion on performance requirements for network-based CRS interference mitigation, Ericsson, Aug. 2018.

[6] R4-1811012, Updated RRM requirements for network-based CRS interference mitigation, Ericsson, Aug. 2018.

[7] R4-1810640, CR on the warm-up for network based CRS IM, Huawei, HiSilicon, Aug. 2018.

[8] R4-1810973, Applicability requirement for network-based CRS-IM, Qualcomm Inc., Aug. 2018.

[9] R4-1811735, Applicability requirement for network-based CRS-IM, Qualcomm Inc., Aug. 2018.

[10] R4-1811864, WF for demodulation performance requirements for network-based CRS interference mitigation,

Ericsson, Aug. 2018.

[11] R2-1813136, Offline #803: [Q235] Text proposals for ASN.1 issue related to CRS muting, Qualcomm, Aug. 2018.

[12] R2-1813088, Implementing network-based CRS interference mitigation, Ericsson.

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template