3GPP TSG RAN Meeting #81			RP-181863

Gold Coast, Australia, Sept 10 - 13, 2018

Agenda item:	9.6 NR UE capabilities/IoT bits/UE categories

Title:	Views on NR UE Capability List

Source:	MediaTek Inc.

Document for:	Discussion

# Introduction

In RAN1#94, an LS [1] is sent to RAN asking RAN to “decide whether the feature is mandatory/optional based on RAN WG recommendations.” In this contribution, we provide our view on the remaining feature groups that have not been decided to be mandatory or optional. The NR UE features list [2] compiled after RAN1#94 is given in the attachment with an additional column appended at the end expressing our recommendation. Some revisions and additions are also made to reflect the latest agreements in RAN1#94. A summary of our recommendation are given in the next section.

# Summary of MediaTek Recommendation

First of all, for the remaining features with RAN working group recommendation, we suggest that RAN accepts all recommendations. These include feature groups 1-13, 4-19 to 4-19c, 5-30 to 5-31, 6-16, 6-18, 6-23 and 1 to 7 and 9 to 14 of the URLLC features.

For those without WG recommendation, including those with square brackets, our recommendation is summarized in Table 1.

Table 1: Feature Group without WG Recommendation

| # | FG/component | WG Recommendation | MTK recommendation |
| --- | --- | --- | --- |
| 1-8 | RLM based on a mix of SS block and CSI-RS |  | Optional |
| 2-20 | Beam correspondence | [Mandatory at least for FR2] | Continue discussion in RAN1#94b and RAN4#88b until requirements are specified |
| 2-30 | UL beam management component 1 |  | Optional |
| 2-31 | Beam failure recovery |  | Mandatory for FR2, optional for FR1 |
| 2-34 | NZP-CSI-RS for IM | [Optional] | Optional |
| 2-38 | CSI report w/o PMI | FFS | Optional |
| 2-41 | Type II codebook |  | Optional |
| 2-43 | Type II codebook with port selection |  | Optional |
| 2-55 | SRS Tx switch |  | Optional |
| 2-56 | SRS carrier switch |  | Optional |
| 5-27a | Dynamic rate-matching control resource set for DL | [Mandatory with capability signaling] | Mandatory with capability signaling |
| 6-10a | Cross carrier scheduling for different numerologies |  | Not support in R-15 (and remove from feature list) as there are still multiple open issues. |
| 6-16 | Supplemental uplink |  | Optional |
| 6-18 | Supplemental uplink with dynamic switch |  | Optional |
| 8[5-30c] | Dynamic indication of MCS tables with new RNTI for PUSCH |  | Optional |

In addition, candidate values and the detailed descriptions of some features should be further discussed in RAN1#94b. Therese include, but are not limited to, CSI-RS resources related features (2-33 and 2-35) and BWP related features (6-1 to 6-4). Our recommended revisions can be found in the attached table and the motivations can be found in our previous RAN1 contribution [3].

# References

- R1-1810005, “LS on new UE features list for NR,” RAN WG1

- R1-1809998, “RAN1 NR UE feature list,” NTT DOCOMO

- R1-1809723, “Remaining issues in NR R-15 UE features,” MediaTek Inc.