3GPP TSG RAN meeting #81										RP-181871

Gold Coast, Australia, September 10-13, 2018

## Status Report to TSG

Agenda item:			10.2.2

| WI / SI Name | Even further enhanced MTC for LTE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| included in this status report | Core part: | Yes | Perf. part: | Yes |  | Testing part: | No |
| Acronym | LTE_eMTC4 |  |  |  |  |  |  |
| Unique ID | 750059 |  |  |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-172811 |  |  |  |  |  |  |
| Target Completion Date | Core part: 09/2018 |  |  |  | Performance part: 12/2018 |  |  |
| Is the WI/SI complete? | Core part: Yes |  |  |  | Performance Part: No |  |  |

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Johan BERGMAN |
|  | Company | Ericsson |
|  | Email | johan.bergman@ericsson.com |
| Rapporteur | Name | Alberto RICO |
|  | Company | Qualcomm |
|  | Email | albertor@qti.qualcomm.com |
| Rapporteur | Name | FANG Huiying |
|  | Company | ZTE |
|  | Email | fang.huiying@zte.com.cn |
| Rapporteur | Name | YE Qiaoyang |
|  | Company | Intel |
|  | Email | qiaoyang.ye@intel.com |
| Rapporteur | Name | SUN Feifei |
|  | Company | Samsung |
|  | Email | feifei.sun@samsung.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.1	RAN1

#### 2.1.1	Agreements

None (core part completed in RAN#80)

#### 2.1.2	Remaining Open issues

None

## 2.2	RAN2

#### 2.2.1	Agreements

Provision of 36.331 CRs to RAN#81 for approval

#### 2.2.2	Remaining Open issues

None

## 2.3	RAN3

#### 2.3.1	Agreements

None (core part completed in RAN#80)

#### 2.3.2	Remaining Open issues

None

## 2.4	RAN4

#### 2.4.1	Agreements

Core part:

RAN4#88 discussed BS RF requirements and agreed on the following CR:

- CR for REFSENS requirement for PUSCH sub-PRB allocation (R4-1811574 and R4-1810493)

RAN4#88 discussed UE RF requirements and agreed on the following CR:

- CR on the A-MPR for PUSCH sub-PRB allocation (R4-1811494)

- CR on the 14-dBm UE power class (R4-1811451)

RAN4#88 discussed RRM requirements for WUS and made the following agreements:

- Agreed way forward in R4-1811690

- Agreed to specify minimum WUS reception requirements for WUS transmissions based on 1 transmit antenna and 2 transmit antennas. The structure of the requirements was also agreed as well as simulation assumptions.

- No test is defined to verify the minimum WUS reception requirements.

- Agreed to complete the remaining work of WUS requirements for Rel-15 LTE-MTC under TEI15 agenda Q4-2019

RAN4#88 discussed high-velocity UE support and made the following agreements:

- Agreed 36.133 CR in R4-1810750

RAN4#88 discussed CRS muting support and made the following agreements:

- Agreed to use 8 warm-up subframes for legacy MTC UEs to read SIB1-BR

- Agreed 36.133 CR in R4-1811717

RAN4#88 discussed new gaps for dense PRS and made the following agreements:

- The following documents were agreed on measurement gaps for dense PRS:

  - R4-1811329, CR to 36.133, Clarification of the requirement for dense PRS gap in eFeMTC UE R15, Qualcomm Inc., Aug. 2018.

  - R4-1811331, CR to 36.133, Updated requirements for measurement gaps for dense PRS, Ericsson, Aug. 2018.

  - R4-1811737, LS on the applicability of dense PRS gap patterns, Huawei, HiSilicon, Aug. 2018.

  - R4-1811330, CR to 36.133, CR on RLM requirements for UE configured with dense PRS gaps, Huawei, HiSilicon, Aug. 2018.

- The above agreed documents contain the following agreements:

  - The impact of the new gaps on RLM

  - The impact of the new gaps on RRM requirements

  - Applicability conditions for the new patterns

RAN4 also discussed high-quality signal thresholds and sent the following reply to RAN2 in R4-1810761:

RAN WG4 would like to thank RAN WG2 for the LS on high quality signal threshold for both MTC and NB-IoT. RAN WG4 has discussed the high-quality signal thresholds used by MTC and NB-IoT UEs for performing PLMN selection and has reached the conclusion that there is no need to lower the current thresholds of -110 dBm.

Performance part:

RAN4 #88 discussed BS REFSENS test cases and made the following agreements:

- Agreed CR (R4-1811572, R4-1811573)

RAN4#88 discussed RRM test scenarios and test configurations and made the following agreements:

- Agreed test case list in R4-1811704

RAN4#88 discussed UE demodulation performance requirements and made the following agreements:

- Agreements on new UE demodulation requirements (from agreed WF i in R4-1811372):

  - DL 64QAM

    - CE Mode A with 3PRB allocation, TBS=968 and under EPA5 2x1 low

    - Transmission mode: TM2

  - High speed scenario:

    - TM2, QPSK 1/3, (TBS=504bits with 6PRB), EPA200, 2x1 Low, CE Mode A, without frequency hopping

    - No Repetition

  - CRS muting

    - Select test cases from

      - Cat-M1

        - 8.11.1.1.3.1 Tests 2 (TM2, 16QAM 1/2, 3PRB, CE Mode A, No repetitions)

      - Cat-M2

        - 8.11.1.1.3.2 Tests 2 (TM2, QPSK 1/10, 18PRB, CE Mode B, 32 repetitions)

- Agreements on new UE CQI reporting requirements (from agreed WF i in R4-1811372):

  - CQI definition test for CQI table 5 for BL/CE UEs

    - Companies are encouraged to provide the simulation results to set the SNR test points corresponding to 64QAM

  - CQI definition test for CQI table 6 for BL/CE UEs

    - Companies are encouraged to provide the simulation results to set the SNR test points corresponding to 16QAM

RAN4#88 discussed BS demodulation performance requirements and made the following agreements:

- Summary of PUSCH simulation results with high velocity scenario (R4-1810149)

- Agreements on new BS demodulation requirements (from agreed WF i in R4-1811373):

  - RAN4 will specify the demodulation requirements with 2 of 3 sub-carriers pi/2 BPSK

    - Simulation assumption for PUSCH with sub-PRB transmission

      - EPA5 1x2

      - Channel bandwidth: 3MHz, 5MHz, 10MHz, 15MHz, 20MHz

      - CE Mode A: 1RU without repetitions, TBS = [32] bits

      - CE Mode B: 2RU with 16 repetitions, TBS = [88] bits

#### 2.4.2	Remaining Open issues

Core part:

- None

Performance part:

- RAN4 demod:

  - Endorse draft CRs for test cases with final test requirements TBD

  - Produce ideal simulation results for alignment

  - Produce practical simulation results

  - Agree on final test requirements

  - Agree 36.101 CR including the final test requirements

  - Agree 36.104 CR including the final test requirements

  - Agree 36.141 CR including the final test requirements

  - Agree CR for test cases corresponding to final test requirements

- RAN4 RRM:

  - Develop the agreed test cases and agree on the CRs

## 2.5	RAN5

N/A

## 2.6	RAN6

N/A

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

N/A

## 4.	References

RAN4#88

72 contributions (for details see agenda item 6.5 in Tdoc list)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template