3GPP TSG-RAN Meeting #81	RP-181889

Gold Coast, Australia, Sept 10th – 13th, 2018

Source:	Ericsson, Sierra Wireless

Title:	IMT-2020 self evaluation: mMTC link budget

Agenda Item:	8.2

Document for:	Discussion and Decision

# Introduction

3GPP are as part of the New Study Item on Self Evaluation towards IMT-2020 Submission [1] preparing a link budget calculation for the use cases eMBB, mMTC and URLLC. In this contribution we present our input to the mMTC use case in TR 37.919 [4] based on link level simulations performed for NB-IoT and for LTE Bandwidth reduced Low complexity (BL) UEs operating in Coverage Enhanced (CE) modes A and B, hereafter referred to as LTE-M.

This is a revision of R1-1809781 [2] containing updated results.

# Link budget template

In this chapter we present our input to the IMT-2020 mMTC link budget template. Sections 2.1, 2.2, 2.3 and 2.4 presents a discussion around the SNR, Shadow fading margin, HARQ gain and penetration margin parameters, while section 2.5 presents the complete mMTC link budget.

## SNR and Data rate performance

The IMT-2020 link budget requires the proponents of a technology to present ‘transmission bit rates’ and ‘packet error ratios’ for the data and control channels of the presented technology. Here we present NB-IoT and LTE-M (N)/(M)PDCCH, (N)PDSCH, (N)PUSCH (F1/2) and PUCCH performance based on the assumptions summarized in Table 1. These assumptions are aligned with the IMT-2020 evaluation methodology [3].

Table 1: Urban Macro-mMTC test environment definition.

| Parameters | Config. A |
| --- | --- |
| Technologies | LTE-M FDD and NB-IoT FDD |
| Physical channels | NB-IoT: NPDCCH, NPDSCH, NPUSCH F1, NPUSCH F2 LTE-M: MPDCCH, PDSCH, PUSCH, PUCCH |
| Carrier frequency for evaluation | 700 MHz |
| Device mobility | 3 km/h |
| Channel models | TDL-iii (NLOS), TDL-v (LOS) |
| eNB RX/TX | 4/2 (2/2 in case of PUCCH) |
| Device RX/TX | 1/1 |
| Transmission mode | TM2 at eNB; TM1 at UE |

The simulator was configured to study the coverage achievable at 10% BLER and a physical layer data rate of at least 160 bps on the UL and DL data channels. The control channels were evaluated at a similar coverage as the data channels, but with a 1% BLER target. Table 3 and Table 2 presents the simulated results for NB-IoT and calculates the thereto associated coupling losses when assuming eNB power of 35 dBm/PRB and a device power class of 23 dBm.

Table 2: NB-IoT link level results for TDL-iii, 2 Hz.

| Physical channel | NPDSCH | NPDCCH | NPUSCH F1 | NPUSCH F2 |
| --- | --- | --- | --- | --- |
| Power | 35 | 35 | 23 | 23 |
| Bandwidth [Hz] | 180000 | 180000 | 15000 | 15000 |
| TBS [Bits] | 256 | 23 | 256 | 1 |
| #RU/#SF/Aggregation level | 10 | 2 | 10 | 1 |
| #Rep | 128 | 512 | 16 | 32 |
| #TX/#RX | 2TX/1RX | 2TX/1RX | 1TX/4RX | 1TX/4RX |
| BLER | 10% | 1% | 10% | 1% |
| Phy data rate [bps] | 180 | 44 | 180 | 15 |
| SNR [dB] | -17.7 | -16.7 | -16.6 | -16 |
| NF | 7 | 7 | 5 | 5 |
| Resulting coupling loss | 167.1 | 166.1 | 166.8 | 166.2 |

Table 3: NB-IoT link level results for TDL-v, 2 Hz.

| Physical channel | NPDSCH | NPDCCH | NPUSCH F1 | NPUSCH F2 |
| --- | --- | --- | --- | --- |
| Power | 35 | 35 | 23 | 23 |
| Bandwidth [Hz] | 180000 | 180000 | 15000 | 15000 |
| TBS [Bits] | 256 | 23 | 512 | 1 |
| #RU/#SF/Aggregation level | 10 | 2 | 10 | 1 |
| #Rep | 64 | 256 | 16 | 32 |
| #TX/#RX | 2TX/1RX | 2TX/1RX | 1TX/4RX | 1TX/4RX |
| BLER | 10% | 1% | 10% | 1% |
| Phy. data rate [bps] | 360 | 89 | 180 | 15 |
| SNR [dB] | -16.9 | -17.2 | -17.6 | -17.9 |
| NF | 7 | 7 | 5 | 5 |
| Resulting coupling loss | 166.3 | 166.6 | 167.8 | 168.1 |

Table 4 and Table 5 presents LTE-M results for TDL-iii and -v.

Note that the PUCCH for TDL-iii has so far been simulated using 2 eNB antennas. Simulations with 4 eNB antennas are ongoing, and the results will be provided in a revision.

The performance PUCCH for TDL-v is also under evaluation. Here we have taken the conservative approach to approximate the PUCCH performance for TDL-v using the performance recorded for TDL-iii.

Table 4: LTE-M link level results for TDL-iii, 2 Hz.

| Physical channel | PDSCH | MPDCCH | PUSCH | PUCCH |
| --- | --- | --- | --- | --- |
| Power | 36.8 | 36.8 | 23 | 23 |
| Bandwidth [Hz] | 1080000 | 1080000 | 180000 | 180000 |
| TBS [Bits] | 208 | 23 | 256 | 1 |
| #RU/#SF/Aggregation level | 1 | 24 | 1 | 1 |
| #Rep | 1024 | 256 | 768 | 128 |
| #TX/#RX | 2TX/1RX | 2TX/1RX | 1TX/4RX | 1TX/2RX |
| BLER | 10% | 1% | 4.0% | 1% |
| Phy data rate [bps] | 182 | 70 | 320 | 15 |
| SNR [dB] | -21.9 | -20.8 | -24.6 | -26.3 |
| NF | 7 | 7 | 5 | 5 |
| Resulting coupling loss | 165.3 | 164.2 | 164 | 165.7 |

Table 5: LTE-M link level results for TDL-v, 2 Hz.

| Physical channel | PDSCH | MPDCCH | PUSCH | PUCCHA) |
| --- | --- | --- | --- | --- |
| Power | 36.8 | 36.8 | 23 | TBD |
| Bandwidth [Hz] | 1080000 | 1080000 | 180000 | 23 |
| TBS [Bits] | 208 | 23 | 256 | 180000 |
| #RU/#SF/Aggregation level | 1 | 24 | 1 | 1 |
| #Rep | 1024 | 256 | 512 | 1 |
| #TX/#RX | 2TX/1RX | 2TX/1RX | 1TX/4RX | 128 |
| BLER | 10% | 1% | 8.7% | 1TX/2RX |
| Phy data rate [bps] | 182 | 70 | 457 | 1% |
| SNR [dB] | -24.1 | -21 | -24.6 | 15 |
| NF | 7 | 7 | 5 | -26.3 |
| Resulting coupling loss | 167.5 | 164.4 | 164 | 5 |

- Approximation based on the TDL-iii performance.

## Link budget template: Cell area reliability and Shadow fading margin

Since the Connection Density evaluations have been performed for a 99% Grade of service requirement it is in the link budget motivated to assume a ‘Cell area reliability’ of 99%. Based on this requirement a ‘Shadow fading margin’ can be calculated, based on the simplistic assumption of a lognormal distributed shadowing with standard deviation σ and a pie shaped cell with an area πR2/3.

Figure 1: Cell shape assumed in the derivation of shadow fading margin.

First assume that a maximum path loss PLMAX is supported by the studied technology. The outage likelihood due to shadow fading (SF) at a certain distance r from the base station can then be calculated based on the Normal distribution Cumulative distribution function:

ProbPLr+SF>PLMAX=12(1+erf⁡(PLr-PLMAX2σ))

Here erf corresponds to the Error function, and the path loss PL(r) is defined by the studied channel model.

The total outage can then be calculated by weighting ProbPLr+SF>PLMAX with the fraction of the cell area associated with a circular arc of length 2πr/3 and thickness dr, and integrating over the full cell radius rϵ[0,R]:

Outage= r=0r=R12(1+erfPLr-PLMAX2σ)∙ 2πrdrπR2

The cell radius R is given by the channel model and a nominal path loss that equals PLMAX minus the shadow fading margin. Figure 2 illustrates the relation between outage probability and the shadow fading margin for LTE-M and UMA A, LOS conditions.

Figure 2: Outage probability versus shadow fading margin.

The shadow fading margin that results in 1% outage is in Table 7 and Table 8 reported to provide 99% Cell area reliability.

## Link budget template: HARQ gain

The link budget template requires a ‘HARQ gain’ to be established. Figure 3 and Figure 4 therefore presents for TDL-iii and TDL-v:

- The BLER performance of the NPUSCH F1 reference case (1 sub-carrier, 10 RUs, 16 repetition) presented in Table 2 and Table 3.

- The initial and residual BLER performance of an 8-repetition NPUSCH F1 transmission using a single HARQ retransmission.

It can be observed that for TDL-iii there is a negligible 0.2 dB coverage gain by using 8 repetitions, and 1 HARQ retransmission compared to the 16 repetitions, no HARQ reference case. The gain can be attributed to the added time diversity due to the scheduling gap inserted between the first and second HARQ transmission.

For TDL-v there is no time diversity coverage gain. This is due to the limited diversity offered by this LOS type of channel.

Similar conclusions with regards to the HARQ cell edge coverage gain can be made for the NB-IoT DL (see link results presented in the Annex), as well as for LTE-M UL and DL. The HARQ gain for the data transmission is therefore reported as 0 dB in Table 7 and Table 8.

For both NB-IoT and LTE-M there is however an important data rate gain thanks to HARQ retransmissions that deserves to be highlighted. Table 6 presents the average (N)PUSCH PHY cell edge data rate with and without HARQ. These improvements in data rates are for simplicity not considered in Table 7 and Table 8.

Note that the simulations for LTE-M focused on achieving maximum data rate for the given coverage of 164 dB. For NB-IoT the focus was to maximize coverage given a data rate requirement of 160 bps. At the same coupling loss level, the performance is expected to be similar for the two technologies.

Table 6: Average (N)PUSCH cell edge PHY data rates for NB-IoT and LTE-M.

| Technology | Channel | Coupling loss | Ref: No HARQ | HARQ |
| --- | --- | --- | --- | --- |
| NB-IoT | TDL-iii | 166.8 dB | 180 bps | 274 bps |
| NB-IoT | TDL-v | 167.8 dB | 180 bps | 200 bps |
| LTE-M | TDL-iii | 164 dB | 320 bps | 700 bps |
| LTE-M | TDL-v | 164 dB | 457 bps | 700 bps |

Figure 3: Assessment of NB-IoT NPUSCH F1 cell edge HARQ performance for TDL-iii.

Figure 4: Assessment of NB-IoT NPUSCH F1 cell edge HARQ performance for TDL-v.

## Link budget template: Penetration margin

For the ‘Penetration margin’ parameter we are making a close to worst case assumption. This is motivated since the link budget is exploring the maximum range supported by mMTC.

For UMA A the penetration loss (PL) is calculated as [5]:

PLUMAA = PLtw + PLin + N(0, σP2)

With the PLtw equal to the external wall loss, PLin the indoor loss and σP being the standard deviation of the loss:

PLtw = 20 dB

PLin = 0.5d2D-in, with d2D-in ~ U(0, 25m)

σP = 4.4 dB

With d2D-in = 25 m and under the assumption that the N(0, σP2) delivers a value of 2 times σP, i.e. 8.8 dB (to cover ~95% of the N(0, σP2) distribution) we get a pessimistic assessment of the penetration loss:

PLUMAA = PLtw + PLin + N(0, σP2) = 20 + 12.5 + 8.8 = 41.3 dB

For UMA B the high loss model [5] gives a penetration loss (PL) calculated as:

PLUMAB = PLtw + PLin + N(0, σP2)

With:

PLtw =5-10log100.7⋅10-LIRRglass10+0.3⋅10-Lconcrete10, for LIRRglass = 23+0.3f, Lconcrete = 5+4f

PLin = 0.5d2D-in, with d2D-in = min(A,B), for A and B~ U(0, 25m)

σP = 6.5 dB

With a frequency f = 0.7 GHz,  d2D-in = 25 m and under the assumption that the N(0, σP2) delivers a value of 2 times σP, i.e. 13 dB we get a pessimistic assessment of the penetration loss that equals:

PLUMAB = PLtw + PLin + N(0, σP2) = 17.74 + 12.5 + 13 = 43.2 dB

These two values for UMA A and UMA B are used as ‘Penetration margin’ in the link budget table when taking indoor users into account.

## Link budget template: Proposal

Table 7 and Table 8 presents the link budget template for NB-IoT and LTE-M. Many of the parameter values are taken from the ITU-R report M.2411-0 [3]. The tables are also complemented with a set of notes explaining parts of the data. Worth to mention is that item “11a Receiver array gain” is not part of the IMT-2020 template but has been added here since its appears to be missing.

For LTE-M the PUCCH performance is in all cases currently based on simulated results for TDL-iii, 2 RX at the eNB.

Table 7: Urban Macro-mMTC link budget for NB-IoT.

| Channel model | UMA A/B | UMA A/B | UMA A | UMA A | UMA B | UMA B |
| --- | --- | --- | --- | --- | --- | --- |
| LOS/NLOS | LOS | LOS | NLOS | NLOS | NLOS | NLOS |
| Link direction | DL | UL | DL | UL | DL | UL |
| Carrier frequency (GHz) | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 |
| BS antenna heights (m) | 25 | 25 | 25 | 25 | 25 | 25 |
| UE antenna heights (m) | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| Cell area reliability(1) (%) (Please specify how it is calculated.) | 99% | 99% | 99% | 99% | 99% | 99% |
| Transmission bit rate for control channel (bit/s) | 89 A) | 15 A) | 44 A) | 15 A) | 44 A) | 15 A) |
| Transmission bit rate for data channel (bit/s) | 360 A) | 180 A) | 180 A) | 180 A) | 180 A) | 180 A) |
| Target packet error ratio for the required SNR in item (19a) for control channel | 1% | 1% | 1% | 1% | 1% | 1% |
| Target packet error ratio for the required SNR in item (19b) for data channel | 10% | 10% | 10% | 10% | 10% | 10% |
| Spectral efficiency(2) (bit/s/Hz) | 0.002 B) | 0.012 B) | 0.001 B) | 0.012 B) | 0.001 B) | 0.012 B) |
| Pathloss model(3) (select from LOS, NLOS or O-to-I) | UMA A/B. LOS | UMA A/B. LOS | UMA A. NLOS | UMA A. NLOS | UMA B. NLOS | UMA B. NLOS |
| UE speed (km/h) | 3 | 3 | 3 | 3 | 3 | 3 |
| Feeder loss (dB) | 3 | 3 | 3 | 3 | 3 | 3 |
| Transmitter |  |  |  |  |  |  |
| (1) Number of transmit antennas (The number shall be within the indicated range in § 8.4 of Report ITU-R M.2412-0) | 2 C) | 1 C) | 2 C) | 1 C) | 2 C) | 1 C) |
| (2) Maximal transmit power per antenna (dBm) | 32.0 | 23 | 32.0 | 23 | 32.0 | 23 |
| (3) Total transmit power = function of (1) and (2) (dBm) (The value shall not exceed the indicated value in § 8.4 of Report ITU-R M.2412-0) | 35.0 D) | 23.0 | 35.0 | 23.0 | 35.0 | 23.0 |
| (4) Transmitter antenna gain (dBi) | 8.0 | 0 | 8.0 | 0 | 8.0 | 0 |
| (5) Transmitter array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, CDD (Cyclic delay diversity), etc.) (dB) | 9.0 E) | 0 | 9.0 E) | 0 | 9.0 E) | 0 |
| (6) Control channel power boosting gain (dB) | 0 | 0 | 0 | 0 | 0 | 0 |
| (7) Data channel power loss due to pilot/control boosting (dB) | 0 | 0 | 0 | 0 | 0 | 0 |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (Feeder loss must be included for and only for downlink) | 3 | 0 | 3 | 0 | 3 | 0 |
| (9a) Control channel e.i.r.p. = (3) + (4) + (5) + (6) – (8) dBm | 50.0 | 23.0 | 50.0 | 23.0 | 50.0 | 23.0 |
| (9b) Data channel e.i.r.p. = (3) + (4) + (5) – (7) – (8) dBm | 50.0 | 23.0 | 50.0 | 23.0 | 50.0 | 23.0 |
| Receiver |  |  |  |  |  |  |
| (10) Number of receive antennas (The number shall be within the indicated range in § 8.4 of Report ITU-R M.2412-0) | 1 | 4 C) | 1 | 4 C) | 1 | 4 C) |
| (11) Receiver antenna gain (dBi) | 0 | 8 | 0 | 8 | 0 | 8 |
| (11a) Receiver array gain | 0 | 9.0 E) | 0 | 9.0 E) | 0 | 9.0 E) |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | 0 | 3 | 0 | 3 | 0 | 3 |
| (13) Receiver noise figure (dB) | 7 | 5 | 7 | 5 | 7 | 5 |
| (14) Thermal noise density (dBm/Hz) | -174 | -174 | -174 | -174 | -174 | -174 |
| (15) Receiver interference density (dBm/Hz) | -177 F) | -177 F) | -177 F) | -177 F) | -177 F) | -177 F) |
| (16) Total noise plus interference density = 10 log (10^(((13) + (14))/10) + 10^((15)/10)) dBm/Hz | -172.24 | -172.24 | -172.24 | -172.24 | -172.24 | -172.24 |
| (17) Occupied channel bandwidth (for meeting the requirements of the traffic type) (Hz) | 180000 G) | 15000 G) | 180000 G) | 15000 G) | 180000 G) | 15000 G) |
| (18) Effective noise power = (16) + 10 log((17)) dBm | -119.68 | -130.47 | -119.68 | -130.47 | -119.68 | -130.47 |
| (19a) Required SNR for the control channel (dB) | -17.2 H) | -17.9 H) | -16.7 H) | -16.0 H) | -16.7 H) | -16 H) |
| (19b) Required SNR for the data channel (dB) | -16.9 H) | -17.6 H) | -17.7 H) | -16.6 H) | -17.7 H) | -16.6 H) |
| (20) Receiver implementation margin (dB) | 2 | 2 | 2 | 2 | 2 | 2 |
| (21a) H-ARQ gain for control channel (dB) | 0 | 0 | 0 | 0 | 0 | 0 |
| (21b) H-ARQ gain for data channel (dB) | 0 | 0 | 0 | 0 | 0 | 0 |
| (22a) Receiver sensitivity for control channel = (18) + (19a) + (20) – (21a) dBm | -134.88 | -146.37 | -134.38 | -144.47 | -134.38 | -144.47 |
| (22b) Receiver sensitivity for data channel = (18) + (19b) + (20) – (21b) dBm | -134.58 | -146.07 | -135.38 | -145.07 | -135.38 | -145.07 |
| (23a) Hardware link budget for control channel = (9a) + (11) + (11a) -(22a) dB | 184.92 | 186.41 | 184.42 | 184.51 | 184.42 | 184.51 |
| (23b) Hardware link budget for data channel = (9b) + (11) + (11a)- (22b) dB | 184.62 | 186.11 | 185.42 | 185.11 | 185.42 | 185.11 |
| Calculation of available pathloss |  |  |  |  |  |  |
| (24) Lognormal shadow fading std deviation (dB) | 4 | 4 | 6 | 6 | 6 | 6 |
| (25) Shadow fading margin (function of the cell area reliability and (24)) (dB) | 6.3 I) | 6.3 I) | 10.3 I) | 10.3 I) | 10.3 I) | 10.3 I) |
| (26) BS selection/macro-diversity gain (dB) | 0 | 0 | 0 | 0 | 0 | 0 |
| (27a) Outdoor: Penetration margin (dB) | 0 | 0 | 0 | 0 | 0 | 0 |
| (27b) Indoor: Penetration margin (dB) | NA | NA | 41,3 | 41,3 | 43,2 | 43,2 |
| (28) Other gains (dB) (if any please specify) | 0 | 0 | 0 | 0 | 0 | 0 |
| (29aa) Outdoor: Available path loss for control channel = (23a) – (25) + (26) – (27a) + (28) – (12) dB | 179 | 177 | 174 | 171 | 174 | 171 |
| (29ab) Indoor: Available path loss for control channel = (23a) – (25) + (26) – (27b) + (28) – (12) dB | NA | NA | 133 | 130 | 131 | 128 |
| (29ba) Outdoor: Available path loss for data channel = (23b) – (25) + (26) – (27a) + (28) – (12) dB | 178 | 177 | 175 | 172 | 175 | 172 |
| (29bb) Indoor: Available path loss for data channel = (23b) – (25) + (26) – (27b) + (28) – (12) dB | NA | NA | 134 | 131 | 132 | 129 |
| Range/coverage efficiency calculation |  |  |  |  |  |  |
| (30aa) Outdoor: Maximum range for control channel (based on (29aa) and according to the system configuration section of the link budget) (m) | 58805 | 53883 | 19536 | 16450 | 19536 | 16450 |
| (30ab) Indoor: Maximum range for control channel (based on (29ab) (m) | NA | NA | 1715 | 1444 | 1533 | 1291 |
| (30ba) Outdoor: Maximum range for data channel (based on (29ba) and according to the system configuration section of the link budget) (m) | 57798 | 52960 | 20721 | 17042 | 20721 | 17042 |
| (30bb) Indoor: Maximum range for data channel (based on (29bb) (m) | NA | NA | 1819 | 1496 | 1626 | 1337 |
| (31aa) Outdoor: Coverage Area for control channel = (π (30a)2) (m2/site) | 10863601707 | 9121146823 | 1198961440 | 850099513 | 1198961440 | 850099513 |
| (31ab) Indoor: Coverage Area for control channel = (π (30a)2) (m2/site) | NA | NA | 9235433 | 6547697 | 7382718 | 5234066 |
| (31ba) Outdoor: Coverage Area for data channel = (π (30b)2) (m2/site) | 10494791919 | 8811491847 | 1348882060 | 912370099 | 1348882060 | 912370099 |
| (31bb) Indoor: Coverage Area for data channel = (π (30b)2) (m2/site) | NA | NA | 10390466 | 7027448 | 8306084 | 5617592 |

Table 8: Urban Macro-mMTC link budget for LTE-M.

| Channel model | UMA A/B | UMA A/B | UMA A | UMA A | UMA B | UMA B |
| --- | --- | --- | --- | --- | --- | --- |
| LOS/NLOS | LOS | LOS | NLOS | NLOS | NLOS | NLOS |
| Link direction | DL | UL | DL | UL | DL | UL |
| Carrier frequency (GHz) | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 |
| BS antenna heights (m) | 25 | 25 | 25 | 25 | 25 | 25 |
| UE antenna heights (m) | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| Cell area reliability(1) (%) (Please specify how it is calculated.) | 99% | 99% | 99% | 99% | 99% | 99% |
| Transmission bit rate for control channel (bit/s) | 70 A) | 8 A) | 70 A) | 8 A) | 70 A) | 8 A) |
| Transmission bit rate for data channel (bit/s) | 183 A) | 457 A) | 183 A) | 320 A) | 183 A) | 320 A) |
| Target packet error ratio for the required SNR in item (19a) for control channel | 1% | 1% | 1% | 1% | 1% | 1% |
| Target packet error ratio for the required SNR in item (19b) for data channel | 10% | 10% J) | 10% | 10% | 10% | 10% |
| Spectral efficiency(2) (bit/s/Hz) | 1.7·10-4 B) | 2.6·10-3 B) | 1.7·10-4 B) | 1.8·10-3 B) | 1.7·10-4 B) | 1.8·10-3 B) |
| Pathloss model(3) (select from LOS, NLOS or O-to-I) | UMA A/B. LOS | UMA A/B. LOS | UMA A. NLOS | UMA A. NLOS | UMA B. NLOS | UMA B. NLOS |
| UE speed (km/h) | 3 | 3 | 3 | 3 | 3 | 3 |
| Feeder loss (dB) | 2 | 2 | 2 | 2 | 2 | 2 |
| Transmitter |  |  |  |  |  |  |
| (1) Number of transmit antennas (The number shall be within the indicated range in § 8.4 of Report ITU-R M.2412-0) | 2 C) | 1 | 2 C) | 1 | 2 C) | 1 |
| (2) Maximal transmit power per antenna (dBm) | 32.0 | 23 | 33.8 | 23 | 33.8 | 23 |
| (3) Total transmit power = function of (1) and (2) (dBm) (The value shall not exceed the indicated value in § 8.4 of Report ITU-R M.2412-0) | 36.8 D) | 23.0 | 36.8 D) | 23.0 | 36.8 D) | 23.0 |
| (4) Transmitter antenna gain (dBi) | 8.0 | 0 | 8.0 | 0 | 8.0 | 0 |
| (5) Transmitter array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, CDD (Cyclic delay diversity), etc.) (dB) | 9.0 E) | 0 | 9.0 E) | 0 | 9.0 E) | 0 |
| (6) Control channel power boosting gain (dB) | 0 | 0 | 0 | 0 | 0 | 0 |
| (7) Data channel power loss due to pilot/control boosting (dB) | 0 | 0 | 0 | 0 | 0 | 0 |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (Feeder loss must be included for and only for downlink) | 3 | 0 | 3 | 0 | 3 | 0 |
| (9a) Control channel e.i.r.p. = (3) + (4) + (5) + (6) – (8) dBm | 51.8 | 23.0 | 51.8 | 23.0 | 51.8 | 23.0 |
| (9b) Data channel e.i.r.p. = (3) + (4) + (5) – (7) – (8) dBm | 51.8 | 23.0 | 51.8 | 23.0 | 51.8 | 23.0 |
| Receiver |  |  |  |  |  |  |
| (10) Number of receive antennas (The number shall be within the indicated range in § 8.4 of Report ITU-R M.2412-0) | 1 | 4 C) | 1 | 4 C) | 1 | 4 C) |
| (11) Receiver antenna gain (dBi) | 0 | 8 | 0 | 8 | 0 | 8 |
| (11a) Receiver array gain | 0 | 9.0 E) | 0 | 9.0 E) | 0 | 9.0 E) |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | 0 | 3 | 0 | 3 | 0 | 3 |
| (13) Receiver noise figure (dB) | 7 | 5 | 7 | 5 | 7 | 5 |
| (14) Thermal noise density (dBm/Hz) | -174 | -174 | -174 | -174 | -174 | -174 |
| (15) Receiver interference density (dBm/Hz) | -177 F) | -177 F) | -177 F) | -177 F) | -177 F) | -177 F) |
| (16) Total noise plus interference density = 10 log (10^(((13) + (14))/10) + 10^((15)/10)) dBm/Hz | -172.24 | -172.24 | -172.24 | -172.24 | -172.24 | -172.24 |
| (17) Occupied channel bandwidth (for meeting the requirements of the traffic type) (Hz) | 1080000 G) | 180000 G) | 1080000 G) | 180000 G) | 1080000 G) | 180000 G) |
| (18) Effective noise power = (16) + 10 log((17)) dBm | -111.90 | -119.68 | -111.90 | -119.68 | -111.90 | -119.68 |
| (19a) Required SNR for the control channel (dB) | -21 H) | -26.3 H) | -20.8 H) | -26.3 H) | -20.8 H) | -26.3 H) |
| (19b) Required SNR for the data channel (dB) | -24.1 H) | -24.6 H) | -21.9 H) | -24.6 H) | -21.9 H) | -24.6 H) |
| (20) Receiver implementation margin (dB) | 2 | 2 | 2 | 2 | 2 | 2 |
| (21a) H-ARQ gain for control channel (dB) | 0 | 0 | 0 | 0 | 0 | 0 |
| (21b) H-ARQ gain for data channel (dB) | 0 | 0 | 0 | 0 | 0 | 0 |
| (22a) Receiver sensitivity for control channel = (18) + (19a) + (20) – (21a) dBm | -130.9 | -144 | -130.7 | -144 | -130.7 | -144 |
| (22b) Receiver sensitivity for data channel = (18) + (19b) + (20) – (21b) dBm | -134.0 | -142.3 | -131.8 | -142.3 | -131.8 | -142.3 |
| (23a) Hardware link budget for control channel = (9a) + (11) + (11a) -(22a) dB | 182.7 | 184 | 182.5 | 184 | 182.5 | 184 |
| (23b) Hardware link budget for data channel = (9b) + (11) + (11a)- (22b) dB | 185.8 | 182.3 | 183.6 | 182.3 | 183.6 | 182.3 |
| Calculation of available pathloss |  |  |  |  |  |  |
| (24) Lognormal shadow fading std deviation (dB) | 4 | 4 | 6 | 6 | 6 | 6 |
| (25) Shadow fading margin (function of the cell area reliability and (24)) (dB) | 6.3 I) | 6.3 I) | 10.3 I) | 10.3 I) | 10.3 I) | 10.3 I) |
| (26) BS selection/macro-diversity gain (dB) | 0 | 0 | 0 | 0 | 0 | 0 |
| (27a) Outdoor: Penetration margin (dB) | 0 | 0 | 0 | 0 | 0 | 0 |
| (27b) Indoor: Penetration margin (dB) | NA | NA | 41,3 | 41,3 | 43,2 | 43,2 |
| (28) Other gains (dB) (if any please specify) | 0 | 0 | 0 | 0 | 0 | 0 |
| (29aa) Outdoor: Available path loss for control channel = (23a) – (25) + (26) – (27a) + (28) – (12) dB | 176 | 175 | 172 | 171 | 172 | 171 |
| (29ab) Indoor: Available path loss for control channel = (23a) – (25) + (26) – (27b) + (28) – (12) dB | NA | NA | 131 | 128 | 129 | 128 |
| (29ba) Outdoor: Available path loss for data channel = (23b) – (25) + (26) – (27a) + (28) – (12) dB | 180 | 173 | 173 | 169 | 173 | 169 |
| (29bb) Indoor: Available path loss for data channel = (23b) – (25) + (26) – (27b) + (28) – (12) dB | NA | NA | 132 | 128 | 130 | 126 |
| Range/coverage efficiency calculation |  |  |  |  |  |  |
| (30aa) Outdoor: Maximum range for control channel (based on (29aa) and according to the system configuration section of the link budget) (m) | 51841 | 46952 | 17478 | 15980 | 17467 | 15980 |
| (30ab) Indoor: Maximum range for control channel (based on (29ab) (m) | NA | NA | 1534 | 1402 | 1371 | 1402 |
| (30ba) Outdoor: Maximum range for data channel (based on (29ba) and according to the system configuration section of the link budget) (m) | 61968 | 42575 | 18648 | 14457 | 18636 | 14457 |
| (30bb) Indoor: Maximum range for data channel (based on (29bb) (m) | NA | NA | 1637 | 1269 | 1462 | 1134 |
| (31aa) Outdoor: Coverage Area for control channel = (π (30a)2) (m2/site) | 8442844929 | 6925622212 | 959647150 | 802240003 | 958483280 | 802240003 |
| (31ab) Indoor: Coverage Area for control channel = (π (30a)2) (m2/site) | NA | NA | 7391684 | 6178973 | 5901605 | 4939298 |
| (31ba) Outdoor: Coverage Area for data channel = (π (30b)2) (m2/site) | 12063930856 | 5694541652 | 1092439099 | 656624869 | 1091114179 | 656624869 |
| (31bb) Indoor: Coverage Area for data channel = (π (30b)2) (m2/site) | NA | NA | 8414753 | 5057111 | 6718483 | 4042449 |

Notes:

- Transmission bit rate: Simulated physical layer data rate based on selected TBS and BLER at the reported required SNR.

- Spectral efficiency: Transmission bit rate divided by the Occupied channel bandwidth.

- Number of transmit/receive antennas: Each reported antenna contains 8 vertically stacked antenna elements. The two antennas are transmitting in two polarizations.

- Maximal transmit power: The eNB power is based on an assumed 29 dBm/PRB with an additional 6 dB power boosting for NB-IoT.

- Transmitter/Receiver array gain: The eNB antenna gain is derived under the assumption of 8 antenna elements as 10log10(8). Note that the Receiver array gain is not part of the original template and was added to the table.

- Receiver interference density: This factor is dependent on cell deployment and traffic load. As a the coverage of the technology is explored here with a very large cell size, we make the simplistic assumption that the interference is 3 dB below the thermal noise floor.

- Occupied channel bandwidth: NPDCCH, NPDSCH uses 180 kHz. NPUSCH F1/F2 uses 15 kHz. MPDCCH, PDSCH uses 1.08 MHz. PUSCH and PUCCH uses 180 kHz.

- Required SNR: Based on link level simulations.

- Shadow fading margin: Calculated for a single cell, assuming a lognormal distributed shadowing and a cell radius r and area of πr2/3, targeting 99% Cell area reliability.

- Actual LTE-M PUSCH BLER 4.4%.

# Conclusions

In this contribution, we have presented initial input to the IMT-2020 mMTC link budget based on NB-IoT and LTE-M link level performance. It is proposed that this input is considered when updating TR 37.919 Study on Self Evaluation towards IMT-2020 Submission and preparing input to ITU-R on IMT-2020.

Proposal: The herein presented mMTC link budget analysis is considered when updating TR 37.919 Study on Self Evaluation towards IMT-2020 Submission [4] and preparing input on ITU-R on IMT-2020.

# References

RP-171451, “New Study Item on Self Evaluation towards IMT-2020 Submission”, RAN#75, Ericsson et al

R1-1809781, “IMT-2020 self-evaluation: mMTC link budget”, RAN1#94, Ericsson

Report ITU-R M.2411-0 “Requirements, evaluation criteria and submission templated for the development of IMT-2020”, ITU-R

RP-180523, TR 37.919 v0.1.0 Study on Self Evaluation towards IMT-2020 Submission (Rel-15), RAN#79

Report ITU-R M.2412-0 “Guidelines for evaluation of radio interface technologies for IMT-2020”, ITU-R

# Annex: NPDSCH HARQ performance

Figure 5: Assessment of NB-IoT NPDSCH cell edge HARQ performance for TDL-iii.

Figure 6: Assessment of NB-IoT NPDSCH cell edge HARQ performance for TDL-v.