3GPP TSG RAN meeting #82										RP-182421

Sorrento, Italy, December 10-13, 2018

## Status Report to TSG

Agenda item:			10.4.2

| WI / SI Name | Additional enhancements for NB-IoT |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NB_IOTenh3 |  |  |  |  |
| Unique ID | 800084 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-181674 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2019 | Performance part: 06/2020 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 25% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | COZZO, Carmela |
|  | Company | Huawei |
|  | Email | carmela.cozzo@huawei.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

Contributions [1] – [71] were submitted to RAN1#94bis meeting. A summary of the RAN1 agreements was submitted after RAN1#94bis meeting in R1-1811737.

Contributions [72] – [142] were submitted to RAN1#95 meeting. A summary of the RAN1 agreements was submitted after RAN1#94bis meeting in R1-1813796.

#### 2.1.1	Agreements

UE-group wake-up signal,

RAN1 discussed UE-group wake-up signal, with the following agreements

RAN1#94bis Agreements

UE-group NWUS is supported based on eNB’s and UE’s capability.

Whether the network supports UE-group NWUS is done by higher layer signaling.

FFS: The number of UE groups is configured by SIB.

Note that the UE-group NWUS is UE optional

From a RAN1, perspective, UE grouping is based on at least UE ID or some function of UE ID.

The legacy UE should not be prevented from using legacy WUS even in the case of Rel-16 group WUS is enabled

- Performance impact on legacy WUS should be carefully considered

Group WUS is based on at least legacy WUS and UE-group ID.

Configuration of group WUS is at least signaled in SI

A Rel-16 group WUS capable UE shall also be capable of Rel-15 legacy WUS

Rel-16 UE-groups WUS with same DRX/eDRX gap configuration are multiplexed with TDM and/or single sequence CDM

- FFS: Whether single sequence CDM can include legacy WUS

- FFS: Multiplexing between Rel-15 and Rel-16 UE groups

Note: Single sequence CDM is where different sequences share the same resource and only one sequence is transmitted at a given time

RAN1#95 Agreements

UE group ID is used as a parameter to generate WUS UE group sequence(s)

One group WUS is designed as a single sequence

Further study false detection (cross/auto correlation) performance properties for the following designs:

- legacy WUS + cover codes,

- legacy WUS + shifted scrambling codes,

- legacy WUS + phase shift + cover code + scrambling bits

  - Including combinations of phase shift, cover code, and/or scrambling bits

Other designs are not precluded.

The number of UE groups is configurable and broadcasted in SIB.

- FFS: Further details on the number of UE groups. For example, whether it is per PO or per gap configuration of a PO.

Multiplexing of Rel-16 UE groups is down-selected among

- Single sequence CDM is supported

- Single sequence CDM is supported and TDM can be additionally applied

  - FFS: Configurability of CDM and/or TDM

A common WUS is used to wake up all group WUS UEs monitoring the same WUS (time-frequency) resource if more than one UE group occupies the WUS resource.

- FFS: Whether the above is also applicable for Rel-15 WUS UEs

- FFS: Whether to support waking up a subset of all WUS UE groups

For multiplexing between Rel-16 UE-group WUS and Rel-15 legacy WUS within the same carrier, further evaluate and down select among the following options

- TDM

- single-seq. CDM

- single-seq. CDM+TDM

FFS whether legacy WUS is the common WUS for all new UEs or only a part of new UEs.

Rel-16 group WUS uses the same gap configurations as for Rel-15 legacy WUS except for differences from possible TDM.

- No new gap higher layer signaling will be introduced for TDM

Support for transmission in preconfigured UL resources

RAN1 discussed transmission in preconfigured UL resources, with the following agreements

RAN1#94bis Agreements

In idle mode, the UE will at least consider one or more of the following attributes when validating TA (combination of multiple attributes is allowed):

- Serving cell changes (serving cell refers the cell that the UE is camping on)

- Time Alignment Timer for idle mode

- Serving cell NRSRP changes (serving cell refers the cell that the UE is camping on)

- FFS Other attributes:

  - Neighbour cell NRSRP change

  - TDOA of >=2 eNBs

  - TA History

  - Subscription based UE differentiation

  - Others not precluded (for example, attributes that need to be considered for high mobility UEs)

Note that UE power consumption should be taken into account for the FFS attributes

For transmission in preconfigured UL resources, an RRC idle UE may use the latest TA that passed the validation criteria

Pre-configured UL resources for transmission of data are indicated by RRC signaling. At least UE-specific RRC signaling is supported.

The resource configuration includes at least the following

- Time domain resources including periodicity(s)

- Frequency domain resources

- TBS(s)/MCS(s)

For UL transmission in preconfigured resource, fallback mechanism to RACH/EDT procedures is supported.

Dedicated preconfigured UL resource is defined as an NPUSCH resource used by a single UE

- NPUSCH resource is time-frequency resource

- Dedicated PUR is contention-free

Contention-free shared preconfigured UL resource (CFS PUR) is defined as an NPUSCH resource simultaneously used by more than one UE

- NPUSCH resource is at least time-frequency resource

- CFS PUR is contention-free

Contention-based shared preconfigured UL resource (CBS PUR) is defined as an NPUSCH resource simultaneously used by more than one UE

- NPUSCH resource is at least time-frequency resource

- CBS PUR is contention-based (CBS PUR may require contention resolution)

In IDLE mode, HARQ is supported for transmission in dedicated PUR

- A single HARQ process is supported,

  - FFS whether two HARQ processes are supported

- FFS: The design of the corresponding NPDCCH search space

In idle mode, dedicated PUR is supported.

- Support for CFS PUR is FFS.

- Support for CBS PUR is FFS.

RAN1#95 Agreements

In idle mode, at least the following TA validation attributes are supported:

- Serving cell changes (serving cell refers the cell that the UE is camping on)

- Time Alignment Timer for idle mode

- Serving cell NRSRP changes (serving cell refers the cell that the UE is camping on)

  - Based on NRSRP measurement definition in existing Rel-15 TS36.214

Send LS to RAN2, RAN4 to consider in their work. All the agreements with ‘include in LS to RAN2, RAN4’ for NB-IoT and eMTC should be captured in this LS. The LS is endorsed in R1-1813778

The UE can be configured to use at least these TA validation attributes:

- Time Alignment Timer for idle mode

- Serving cell NRSRP changes

- Note: the configuration shall support disabling of the TA validation attributes

Include in LS to RAN2, RAN4.

For further study:

TA validation attributes:

- Subscription based UE differentiation (or Stationary indication in held in subscription)

- Cell specific indication where TA is valid within that cell

Include in LS to RAN2, RAN4:

RAN1 assumes that a UE transitioning from EDT/connected to idle mode can use the valid TA that was used while in EDT/connected mode.

For dedicated PUR in idle mode, the UE may skip UL transmissions.

- FFS: Resource release mechanism

- FFS: Whether or not to support mechanism to disable skipping by eNB

In idle mode, only one HARQ process is supported for dedicated PUR

For dedicated PUR in idle mode, UL grant for HARQ retransmission is transmitted in search space

FFS: Details on the search space (for example USS, CSS)

Scheduling of multiple DL/UL transport blocks

RAN1 discussed scheduling of multiple DL/UL transport blocks, with the following agreements

RAN1#94bis Agreements

The UE should only monitor one DCI size in the UE specific search space.

Individual feedback for each HARQ process is supported. FFS if HARQ bundling/multiplexing can be optionally supported.

Using one DCI to schedule multiple TBs for SC-MTCH is supported, and it is configured and enabled per SC-MTCH via SC-PTM configuration message in SC-MCCH.

Working Assumption

For UE supporting multiple TBs, the soft buffer size stays the same as that of the legacy UE

RAN1#95 Agreements

For multi-TBs scheduling

- UL: I_sc for each TB is same

Confirm the working assumption that for UE supporting multiple TBs, the soft buffer size stays the same as that of the legacy UE.

For UL/DL unicast, at least consecutive resource allocation in time is supported when multiple TBs are scheduled by one single DCI.

- ‘consecutive resource allocation in time’ means no new scheduling gap between the end of previous TB and the start of the next TB

FFS: Whether scheduling gaps is also supported

FFS: How to schedule repetitions within the consecutive resource allocation

For unicast, when multiple DL/UL transport blocks are assigned by a single DCI, the relationship(s) between HARQ process and TB is/are selected from the following two candidates(multiple choices are allowed)

- Relationship 1: 1 HARQ process corresponds to 1 TB

- Relationship 2: 1 HARQ process corresponds up to 2 TBs

Maximum UL HARQ process supported is 2.

Maximum DL HARQ process supported is 2.

The maximum number of TBs for multicast is one of [4, 8]. FFS: Whether the TBs are back to back without gap

Coexistence of NB-IoT with NR

RAN1 discussed coexistence of NB-IoT with NR, with the following agreements

RAN1#94bis Agreements

The following aspects are studied to improve the performance of coexistence of NB-IoT with NR

- Resource reservation in NB-IoT

- Overlap of NR SSB with NB-IoT

RAN1#95 Agreements

RAN1 continues to study the following techniques for performance improvements of NB-IoT resource allocation until the next meeting:

- Resource reservation at symbol level/slot level/subframe level/subcarrier level

- Whether the resource reservation for NB-IoT is dynamic or semi-static (if supported)

- Whether and how to support NB-IoT transmission in a portion of the subframe

- Impact of resource reservation to legacy UEs

- Whether NB-IoT transmission is postponed or dropped in reserved resources

- Whether resource reservation is used for anchor/non-anchors

Conclusion

Overlap of NR SSB with NB-IoT anchor carrier can be avoided.

Support of quality report in Msg3 for non-anchor access

RAN1 discussed the quality report in Msg3 for non-anchor access, with the following agreements

RAN1#94bis Agreements

RAN1 does not define search space for hypothetical NPDCCH for channel quality report in Msg3 on non-anchor access.

From RAN1 point of view, specification support for measurement period for non-anchor access in RAN1 specifications is not needed

RAN1 does not define measurement reference resource for non-anchor access.

For further study:

The following scenarios with regards to downlink channel quality reporting in msg3 for non-anchor carrier access.

- For EDT/non-EDT, msg3 associated with PDCCH order PRACH, IDLE

- PUR

RAN1#95 Agreements

In case 4 bits is used for a non-anchor carrier, all repetition i.e. 12 candidate values {1,2,4,8,16,32,64,128,256,512,1024,2048} can be reported in Msg3.

In case of 2 bits is used for a non-anchor carrier, 3 candidate values can be reported in Msg3. Select one of the following alternatives for determining the 3 values:

- Depending on Rmax, the maximum number of repetitions for NPDCCH Type 2 CSS.

- Depending on R, "DCI subframe repetition number" indicated in DCI format N1 for Msg2 scheduling.

- Depending on Rdecoded, based on the number of repetitions for NPDCCH scheduling Msg2 where UE decodes successfully.

Presence of NRS on a non-anchor carrier for paging

RAN1 discussed the presence of NRS on a non-anchor carrier for paging, with the following agreements

RAN1#94bis Agreements

The presence of NRS on subframes which will contain NRS even when no paging NPDCCH is transmitted is enabled by SIB.

Subframes which will contain NRS even when no paging NPDCCH is transmitted are associated to a PO.

- FFS: Whether PO is from UE perspective or NW perspective

A subset of the POs have associated subframes which will contain NRS even when no paging NPDCCH is transmitted.

FFS: Details of the subset, whether it depends on some existing parameters such as DRX cycle or new parameters by configuration

FFS: Whether the subset can be the whole set of POs by configuration.

RAN1#95 Agreements

RAN1 to discuss the issue of NRS presence for NPDCCH early termination when NWUS is enabled. Down-select among:

- Alt1: If NWUS is enabled (from network perspective), there is no NRS for NPDCCH early termination, but there may be NRS for NWUS early termination.

- Alt2: If NWUS is enabled (from UE perspective, i.e., enabled by network and supported by UE), there is no NRS for NPDCCH early termination, but there may be NRS for NWUS early termination.

- Alt3: Decouple configuration of NRS for NPDCCH early termination and NWUS early termination

  - e.g. eNB can enable NRS for NWUS and disable for NPDCCH, or vice versa.

- Alt4: NRS is always associated with NPDCCH for paging

In the POs that have associated subframes which will contain NRS even when no paging NPDCCH is transmitted for NPDCCH early termination:

- NRS is present in the first M subframes out of the 10 NB-IoT DL subframes before the PO, and the N first NB-IoT DL subframes of NPDCCH search space

  - FFS value(s) of N, M

The subset of POs that have associated subframes which will contain NRS even when no paging NPDCCH is transmitted can be the whole set of POs.

#### 2.1.2	Remaining Open issues

- Specify support for UE-group wake-up signal

- Specify support for transmission in preconfigured resources in idle and/or connected mode based on SC-FDMA waveform for UEs with a valid timing advance.

- Specify scheduling multiple DL/UL transport blocks with or without DCI for SC-PTM and unicast.

- Conclude on additional potential specification enhancement for improving the performance of coexistence of NB-IoT with NR.

- Specify support of Msg3 quality reporting for non-anchor access.

- Specify signalling to indicate on a non-anchor carrier for paging a set of subframes which will contain NRS even when no paging NPDCCH is transmitted.

## 2.2	RAN2

Contributions [143] – [187] were submitted to RAN2#103bis meeting. A summary of the RAN2 agreements was submitted after RAN2#103bis meeting in R2-1815744.

Contributions [188] – [243] were submitted to RAN2#104 meeting. A summary of the RAN2 agreements was submitted after RAN2#104 meeting in R2-1818633.

#### 2.2.1	Agreements

Organisational

RAN2#103bis agreements: To create document agreed after each meeting capturing agreements from all meetings. Copy/paste of chair notes, not intended to be a further discussion of agreements. One week email agreement after each meeting BlackBerry will be rapporteur of the document. RAN2#104 agreements: None

Mobile-terminated (MT) early data transmission (EDT)

RAN2 discussed Mobile-terminated (MT) early data transmission (EDT), with the following agreements:

RAN2#103bis agreements: RAN2 intends to support MT-initiated EDT for both CP and UP solutions. The intention to use MT-EDT is for user data, i.e. not for NAS signalling. RAN2#104 agreements: MT EDT are evaluated at least based on battery life, network resource efficiency, security, reliability and potential impact on core network. MT-EDT is intended for DL data which can be transmitted in one transport block. Use cases that require DL data transmission with or without UL data transmission as a response should be supported for MT-EDT.

RAN2 agreed to have the following email discussion until next meeting:

[104#49][eMTC & NB-IoT R16] MT EDT (Huawei): To evaluate the solution options based on the agreed criteria (Huawei)

UE-group wake-up signal (WUS)

RAN2 discussed UE-group wake-up signal (WUS), with the following agreements:

RAN2#103bis agreements: The aim of UE grouping for WUS is reducing the false alarm probability. At least UE_ID based grouping is supported for UE-Group based WUS. This doesn’t exclude other options. Further discuss whether the following are supported: - Service based grouping - DRX/eDRX based grouping - Gap based grouping. - Coverage based grouping RAN2#104 agreements: Further discuss the benefit and feasibility of using service based parameters for grouping in addition to UE-ID. Can discuss group distribution further, including Rel-15/16 mechanism interaction, once we know more about number of groups and more about the grouping solution (e.g. service based parameters). RAN2 will decide on the UE to WUS group mapping.

RAN2 agreed to have the following email discussion until next meeting:

[104#42][eMTC & NB-IoT R16] Analyse the benefit and feasibility of service based group WUS solutions (Nokia): Companies can provide solutions, with benefit analysis (e.g. based on reference traffic model) and feasibility.

Transmission in preconfigured resources

RAN2 discussed transmission in preconfigured resources, with the following agreements:

RAN2#103bis agreements: Transmission in dedicated preconfigured uplink resources in IDLE mode is supported for UEs with a valid timing advance. Initially we will focus on dedicated preconfigured uplink resources in idle mode - Shared resources can also be discussed RAN2#104 agreements: The eNB configures the dedicated preconfigured uplink resources via RRC dedicated signaling. Methods for eNB to obtain information used to help configuring the dedicated preconfigured uplink resource to the UE is FFS. Periodic D-PUR with duration is supported FFS if one shot D-PUR is supported. Release of the dedicated preconfigured resources are supported, details for NW triggered and UE triggered are FFS.

RAN2 agreed to have the following email discussion until next meeting:

[104#43][eMTC & NB-IoT R16] To progress on D-PUR (Sierra Wireless): Primary scope is to progress the FFS captured for D-PUR. Secondary scope, to progress further details on solutions addressing the agreements.

Scheduling multiple DL/UL transport blocks

RAN2 discussed scheduling multiple DL/UL transport blocks, with the following agreements:

RAN2#103bis agreements: Working assumption: For MTCH in SC-PTM, configuration for multiple scheduling is transmitted in MCCH. Backwards compatibility is FFS. SPS is not supported for NB-IoT in Release 16. Enhancements for “SR with SPS for BSR” can be considered. RAN2#104 agreements: RAN2 intends to support separate/shared SC-MTCH transmission. Multiple TB scheduling is supported for UEs in connected mode. It is FFS if it is supported for EDT. UEs in connected mode are configured with multiple TB scheduling via dedicated RRC signalling assuming that a new DCI format is introduced. This is pending RAN1 agreement. UEs report capability to indicate support of multiple TB scheduling in connected mode.

Network management tool enhancement

RAN2 discussed network management tool enhancement, with the following agreements:

RAN2#103bis agreements: ANR reporting for NB-IoT only uses idle-mode measurements (i.e. we won’t introduce connected mode measurements) Support RACH report for NB-IoT Will consider whether information in LTE RACH report is extended RAN2#104 agreements: SON-ANR: RAN2 understanding is that the purpose of SON/ANR reporting in NB-IoT is network optimisation rather than immediately updating neighbour relations like with LTE ANR, and is therefore not time critical. SON reporting does not trigger RRC connection establishment/resume - FFS whether this includes EDT. SON information can be reported along with EDT, FFS what and how. - RACH report: In addition to legacy parameters for RACH reporting, the first selected resource pool (E.g. CE level, EDT) is included in the RACH report. This agreement is also applicable for eMTC.

RAN2 agreed to have the following email discussions until next meeting:

[104#44][NB-IoT R16] SON/ANR report for NB-IoT (Huawei): How and when the measurements, configuration, reporting are done

[104#45][NB-IoT R16]  RACH Report (Qualcomm): How and what to report.

[[104#46][NB-IoT R16] RLF Report (Ericsson): Purpose and content of RLF report.

Improved multi-carrier operation

RAN2 discussed improved multi-carrier operation, with the following agreements:

RAN2#103bis agreements: None. RAN2#104 agreements: Re-use the code points defined in Rel-14. Study the impact of re-using the Rel-14 RRC reporting mechanism and consider whether a MAC mechanism should be used instead. RAN2 further study how to support the use case of enabling measurements in non-anchor carrier while reducing measurement on anchor carrier.

Inter-RAT cell selection

RAN2#103bis agreements: RAN2 understands that cell reselection between NB-IoT and any other RAT is not within the scope of this WI NB-IoT network may indicate eMTC/LTE/GERAN assistance information for inter-RAT cell selection eMTC/LTE network may indicate NB-IoT assistance information for inter-RAT cell selection Intention is not to provide cell reselection parameters It is up to UE implementation how and when to perform measurements for cell selection of the target RAT RAN2#104 agreements: NB-IoT network may indicate frequency identifiers of neighbouring eMTC/LTE/GERAN carriers to assist inter-RAT selection. eMTC/LTE network may indicate frequency identifiers of neighbouring NB-IoT carriers to assist inter-RAT selection. Indicate whether each LTE neighbour frequency supports eMTC/LTE/both. Indicate whether each GERAN neighbour frequency supports EC-GSM/PEO.

RAN2 agreed to have the following email discussion until next meeting:

[[104#47][NB-IoT R16]  Inter-RAT cell selection (Mediatek): Whether priority and/or suitability criteria can be broadcast. How the assistance information is signalled.

Coexistence with NR

RAN2 did not discuss coexistence with NR.

#### 2.2.2	Remaining Open issues

- Solution(s) for MT-EDT and details of the agreed MT-EDT solution(s).

- Solution for UE-grouping for WUS and RAN2 aspects of UE-Group WUS.

- Support for transmission in preconfigured dedicated resources:

  - Detailed solution for transmission in preconfigured dedicated resources in idle mode,

  - Solutions for transmission in preconfigured shared resources in idle mode if supported,

  - Solutions for transmission in preconfigured resources in connected mode if supported;

- Scheduling multiple DL/UL transport blocks with or without DCI for SC-PTM and unicast:

  - Signalling for scheduling multiple DL/UL transport blocks with or without DCI for SC-PTM and unicast,

  - MAC procedure for scheduling multiple DL/UL transport blocks in unicast.

- Network management tools:

  - SON/ANR: agree on the solution for ANR measurements in idle mode, details of the solution (configuration, measurements, reporting),

  - RACH report: Details of the RACH report and reporting procedure,

  - RLF report: Purpose of the RLF report, contents and reporting procedure.

- Improved multi-carrier operation

  - Support of Msg3 quality reporting for non-anchor access: reporting mechanism (whether to reuse rel-14 or introduce a new mechanism)

  - NRS presence on non-anchor even when no paging NPDCCH is transmitted:

    - Signalling to indicate on a non-anchor carrier the set of subframes which will contain NRS even when no paging NPDCCH is transmitted.

    - Whether to enable RRM measurements on non-anchor carrier.

- Inter-RAT cell selection: Details and signalling of the assistance information in the NB-IoT and LTE/eMTC RATs

- Signalling impact of coexistence of NB-IoT with NR, if any.

## 2.3	RAN3

Not started.

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

Not started.

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R1-1810081	Additional enhancements for NB-IoT workplan	Huawei, HiSilicon

- R1-1811664	Feature summary of UE-group wake-up signal for NB-IoT	Ericsson

- R1-1811679	Further topics for UE-group wake-up signal for NB-IoT	Ericsson

- R1-1810082	On support for UE-group wake-up signal	Huawei, HiSilicon

- R1-1810194	UE-group wake-up signal in NB-IoT	Ericsson

- R1-1810240	Discussion on UE-grouping wake up signal in NB-IoT	LG Electronics

- R1-1810359	Multiplexing methods for UE-group wake-up signal of NB-IoT	vivo

- R1-1810508	Discussion on Wake-up signal for NB-IoT	ZTE

- R1-1810606	UE-Group WUS in NB-IoT	MediaTek Inc.

- R1-1810741	UE-group wake-up signal for NB-IoT	Intel Corporation

- R1-1810827	UE-group wake-up signal for NB-IoT	Samsung

- R1-1810923	UE-group wake-up signal	Qualcomm Incorporated

- R1-1811073	UE group wake-up signal for NB-IoT	Nokia, Nokia Shanghai Bell

- R1-1811386	UE-group wake-up signal for Rel.16 IoT	NTT DOCOMO, INC.

- R1-1811576	Group Wake-Up Signal for NB-IoT	Fraunhofer IIS, Fraunhofer HHI

- R1-1811697	Feature lead summary of Support for transmission in preconfigured UL resources	Huawei, HiSilicon

- R1-1811678	Summary of offline discussions on PUR for eMTC and NB-IoT	Huawei, HiSilicon

- R1-1810083	On support for transmission in preconfigured UL resources	Huawei, HiSilicon

- R1-1810180	Considerations on transmission in preconfigured UL resources	Hisense

- R1-1810195	Support for transmission in preconfigured UL resources in NB-IoT	Ericsson

- R1-1810241	Discussion on preconfigured UL resources in NB-IoT	LG Electronics

- R1-1810490	NB-IOT Pre-configured UL Resources Design Considerations 	Sierra Wireless, S.A.

- R1-1810509	Support for transmission in preconfigured UL resources for NB-IoT	ZTE

- R1-1810607	Early Transmission on Preconfigured UL resources in NB-IoT	MediaTek Inc.

- R1-1810742	UL transmission in preconfigured resources for NB-IoT	Intel Corporation

- R1-1810828	Discussion on transmission in preconfigured UL reosurces for NB-IoT	Samsung

- R1-1810924	Support for transmission in preconfigured UL resources	Qualcomm Incorporated

- R1-1811074	Preconfigured Grant for Uplink transmission	Nokia, Nokia Shanghai Bell

- R1-1811388	UL transmission scheme in preconfigured resources	NTT DOCOMO, INC.

- R1-1811555	Discussion on transmission in preconfigured UL resources	III

- R1-1811660	Summary on Multiple TB scheduling enhancement for NB-IoT	ZTE

- R1-1810084	Scheduling multiple DL/UL transport blocks for SC-PTM and unicast	Huawei, HiSilicon

- R1-1810196	Scheduling of multiple DL/UL transport blocks in NB-IoT	Ericsson

- R1-1810242	Discussion on multiple transport blocks scheduling in NB-IoT	LG Electronics

- R1-1810488	NB-IOT Multiple Transport Block Grant Design	Sierra Wireless, S.A.

- R1-1810510	Consideration on scheduling enhancement for NB-IoT	ZTE

- R1-1810584	Design of scheduling of multiple DL/UL TB for Rel.16 NBIoT	Lenovo, Motorola Mobility

- R1-1810610	Scheduling of multiple DL/UL transport blocks	MediaTek Inc.

- R1-1810829	Scheduling of multiple transport blocks for NB-IoT	Samsung

- R1-1810925	Scheduling of multiple DL/UL transport blocks	Qualcomm Incorporated

- R1-1811075	Scheduling of multiple DL/UL transport blocks	Nokia, Nokia Shanghai Bell

- R1-1811698	Feature lead summary of Coexistence of NB-IoT with NR	Huawei, HiSilicon

- R1-1810085	Discussion on NB-IoT coexistence with NR	Huawei, HiSilicon

- R1-1810197	Coexistence of NB-IoT with NR	Ericsson

- R1-1810243	Discussion on coexistence of NB-IoT with NR	LG Electronics

- R1-1810472	On Coexistence of NB-IoT with NR	Dish Network

- R1-1810511	Performance improvement for coexistence of NB-IoT with NR	ZTE

- R1-1810605	Coexistence of NB-IoT with NR	MediaTek Inc.

- R1-1810926	Coexistence of NB-IoT with NR	Qualcomm Incorporated

- R1-1811079	Coexistence of NB-IoT with NR	Nokia, Nokia Shanghai Bell

- R1-1811699	Feature lead summary of support of quality report in Msg3 for non-anchor access	Huawei, HiSilicon

- R1-1810086	On support of Msg3 quality reporting for non-anchor access	Huawei, HiSilicon

- R1-1810198	Support of quality report in Msg3 for non-anchor access in NB-IoT	Ericsson

- R1-1810244	Downlink channel quality report during random access procedure on a non-anchor carrier  LG Electronics

- R1-1810512	Support of Quality report in Msg3 for non-anchor access	ZTE

- R1-1810608	Quality report in Msg3 on non-anchor carrier	MediaTek Inc.

- R1-1810743	DL quality reporting in Msg3 for non-anchor access in NB-IoT	Intel Corporation

- R1-1810830	Discussion on quality report in Msg3 for NB-IoT	Samsung

- R1-1810927	Support of quality report in msg3 for non-anchor	Qualcomm Incorporated

- R1-1811077	Support of Quality report in Msg3 on non-anchor carrier	Nokia, Nokia Shanghai Bell

- R1-1811326	Presence of NRS on a non-anchor for paging (feature lead summary)	Qualcomm Europe Inc. (Spain)

- R1-1810087	NRS presence on non-anchor carriers for paging	Huawei, HiSilicon

- R1-1810199	Presence of NRS on a non-anchor carrier for paging in NB-IoT	Ericsson

- R1-1810245	Discussion on NRS on an a non-anchor carrier	LG Electronics

- R1-1810513	Presense of NRS on non-anchor PRB	ZTE

- R1-1810609	NRS presence on non-anchor carrier	MediaTek Inc.

- R1-1810831	NRS presence on non-anchor carrier	Samsung

- R1-1810928	Presence of NRS on a non-anchor carrier for paging	Qualcomm Incorporated

- R1-1811078	Presence of NRS on a non-anchor carrier for paging	Nokia, Nokia Shanghai Bell

- R1-1811681	WF on Support of DL quality report in connected mode in NB-IoT	Ericsson, CMCC, Qualcomm, LG, ZTE, Lenovo, Nokia, NSB, SoftBank, MediaTek, NTT Docomo, AT&T, Sharp

- R1-1811590	Support of DL quality report in connected mode in NB-IoT	Ericsson

- R1-1813714	Summary of 6.2.2.1 Group WUS for NB-IoT	Ericsson

- R1-1813756	Further topics on 6.2.2.1 Group WUS for NB-IoT	Ericsson

- R1-1812127	UE-group wake-up signal in NB-IoT	Ericsson

- R1-1812133	UE-group wake-up signal	Huawei, HiSilicon

- R1-1812279	on UE-group wake-up signal of NB-IoT	vivo

- R1-1812429	Group Wake-Up Signal for NB-IoT	Fraunhofer IIS, Fraunhofer HHI

- R1-1812460	UE-group wake-up signal for NB-IoT	Intel Corporation

- R1-1812535	Discussion on UE-grouping wake up signal in NB-IoT	LG Electronics

- R1-1812712	UE-Group WUS in NB-IoT	MediaTek Inc.

- R1-1812773	Discussion on Wake-up signal for NB-IoT	ZTE

- R1-1812928	UE group wake-up signal for NB-IoT	Nokia, Nokia Shanghai Bell

- R1-1812946	UE-group wake-up signal for NB-IoT	Samsung

- R1-1813046	UE-group wake-up signal	Qualcomm Incorporated

- R1-1813195	Consideration on UE-group wake-up signal for Rel-16 NB-IoT	Sharp

- R1-1813294	Discussion on UE-group wake up signal in NB-IoT	NTT DOCOMO, INC.

- R1-1813635	UE-group wake-up signal for NB-IoT	Sony

- R1-1813717	Feature lead summary of Support for transmission in preconfigured UL resources	Huawei, HiSilicon

- R1-1813762	Further topics for transmission in preconfigured UL resources	Huawei, HiSilicon

- R1-1813754	LTE-M Preconfigured UL Resources – offline Tuesday	Sierra Wireless

- R1-1812128	Support for transmission in preconfigured UL resources in NB-IoT	Ericsson

- R1-1812134	Transmission in preconfigured UL resources	Huawei, HiSilicon

- R1-1812461	UL transmission in preconfigured resources for NB-IoT	Intel Corporation

- R1-1812536	Discussion on preconfigured UL resources in NB-IoT	LG Electronics

- R1-1812713	Early Transmission in Preconfigured UL resources in NB-IoT	MediaTek Inc.

- R1-1812726	NB-IOT Pre-configured UL Resources Design Considerations	Sierra Wireless, S.A.

- R1-1812774	Support for transmission in preconfigured UL resources for NB-IoT	ZTE

- R1-1812929	Preconfigured Grant for Uplink transmission	Nokia, Nokia Shanghai Bell

- R1-1812947	Discussion on transmission in preconfigured UL reosurces for NB-IoT	Samsung

- R1-1813047	Support for transmission in preconfigured UL resources	Qualcomm Incorporated

- R1-1813295	Discussion on preconfigured UL resources in NB-IoT	NTT DOCOMO, INC.

- R1-1813529	Discussion on transmission in preconfigured UL resources	III

- R1-1813740	Summary on Multiple TB scheduling enhancement for NB-IoT	ZTE

- R1-1813757	Summary on Multiple TB scheduling enhancement for NB-IoT	ZTE

- R1-1812129	Scheduling of multiple DL/UL transport blocks in NB-IoT	Ericsson

- R1-1812135	Scheduling multiple DL/UL transport blocks for SC-PTM and unicast	Huawei, HiSilicon

- R1-1812537	Discussion on multiple transport blocks scheduling in NB-IoT	LG Electronics

- R1-1812716	Scheduling of multiple DL/UL transport blocks in NB-IoT	MediaTek Inc.

- R1-1812723	NB-IOT Multiple Transport Block Grant Design 	Sierra Wireless, S.A.

- R1-1812775	Consideration on scheduling enhancement for NB-IoT	ZTE

- R1-1812790	Design of scheduling of multiple DL/UL transport blocks for Rel.16 NBIoT	Lenovo, Motorola Mobility

- R1-1812930	Scheduling of multiple DL/UL transport blocks	Nokia, Nokia Shanghai Bell

- R1-1812948	Scheduling of multiple transport blocks for NB-IoT	Samsung

- R1-1813048	Scheduling of multiple DL/UL transport blocks	Qualcomm Incorporated

- R1-1813718	Feature lead summary of Coexistence of NB-IoT with NR	Huawei, HiSilicon

- R1-1812130	Coexistence of NB-IoT with NR	Ericsson

- R1-1812136	Performance enhancements for NB-IoT coexistence with NR	Huawei, HiSilicon

- R1-1812538	Discussion on coexistence of NB-IoT with NR	LG Electronics

- R1-1812711	Coexistence of NB-IoT with NR	MediaTek Inc.

- R1-1812776	Coexistence of NB-IoT with NR	ZTE

- R1-1812932	Coexistence of NB-IoT with NR	Nokia, Nokia Shanghai Bell

- R1-1813049	Coexistence of NB-IoT with NR	Qualcomm Incorporated

- R1-1813719	Feature lead summary of support of quality report in Msg3 for non-anchor access	Huawei, HiSilicon

- R1-1812131	Support of quality report in Msg3 for non-anchor access in NB-IoT	Ericsson

- R1-1812137	Msg3 quality reporting for non-anchor access	Huawei, HiSilicon

- R1-1812462	DL quality reporting in Msg3 for non-anchor access in NB-IoT	Intel Corporation

- R1-1812539	Downlink channel quality report during random access procedure on a non-anchor carrier  LG Electronics

- R1-1812714	Quality report in Msg3 on non-anchor carrier in NB-IoT	MediaTek Inc.

- R1-1812777	Support of Quality report in Msg3 for non-anchor access	ZTE

- R1-1812934	Support of Quality report in Msg3 on non-anchor carrier	Nokia, Nokia Shanghai Bell

- R1-1812949	Discussion on quality report in Msg3 for NB-IoT	Samsung

- R1-1813050	Support of quality report in msg3 for non-anchor	Qualcomm Incorporated

- R1-1813716	Presence of NRS on a non-anchor for paging (feature lead summary)	Qualcomm

- R1-1812132	Presence of NRS on a non-anchor carrier for paging in NB-IoT	Ericsson

- R1-1812138	NRS presence on non-anchor carriers for paging	Huawei, HiSilicon

- R1-1812540	Discussion on NRS on a non-anchor carrier	LG Electronics

- R1-1812715	NRS presence on non-anchor carrier in NB-IoT	MediaTek Inc.

- R1-1812778	Presense of NRS on non-anchor PRB	ZTE

- R1-1812935	Presence of NRS on a non-anchor carrier for paging	Nokia, Nokia Shanghai Bell

- R1-1812950	NRS presence on non-anchor carrier	Samsung

- R1-1813051	Presence of NRS on a non-anchor carrier for paging	Qualcomm Incorporated

- R1-1813588	Support of DL quality report in connected mode in NB-IoT	Ericsson

- R2-1813903	Additional enhancements for NB-IoT workplan	Huawei, HiSilicon

- R2-1813915	UE-Group based Wake-up signal	Huawei, HiSilicon

- R2-1815148	Wake-up signal UE grouping	Ericsson

- R2-1813719	UE-group Wake-up Signal for NB-IoT	MediaTek Inc.

- R2-1813960	UE grouping on wake-up signal channel	Qualcomm Incorporated

- R2-1814382	Consideration on UE-group wake-up signal for NB-IoT and eMTC	ZTE Corporation

- R2-1814766	Consideration on wake up signal	Lenovo, Motorola Mobility

- R2-1814278	Supporting UL data transmission on pre-configured resources in IDLE	Qualcomm Incorporated

- R2-1814343	Use cases for transmission in preconfigured uplink resources	Ericsson	discussion

- R2-1814375	Analysis for UL transmission over preconfigured resources for NB-IoT and eMTC	ZTE Corporation

- R2-1814376	Support for UL transmission over preconfigured dedicated resource in idle mode for NB-IoT and eMTC	ZTE Corporation

- R2-1813916	Uplink transmission in preconfigured resource	Huawei, HiSilicon

- R2-1814342	Preconfigured uplink resources	Ericsson

- R2-1814413	Signaling Aspects for transmission in preconfigured resources	Nokia, Nokia Shanghai Bell

- R2-1815078	Pre-configured UL Resources Design Considerations	Sierra Wireless, S.A.

- R2-1815278	Data transmission using SPS in RRC_IDLE	LG Electronics UK

- R2-1815279	Support for common SPS resource allocation	LG Electronics UK

- R2-1813814	ANR Reporting Protocol for NB-IoT	Sequans Communications

- R2-1814107	RACH report enhancements for SON	Qualcomm Incorporated

- R2-1814379	Analysis on network management tool related measurement report in NB-IoT	ZTE Corporation

- R2-1813904	Discussion on RACH and RLF report	Huawei, HiSilicon

- R2-1813905	Discussion on ANR report	Huawei, HiSilicon

- R2-1814108	Cell Global Identity and strongest measured cell(s) (ANR)	Ericsson

- R2-1814412	SON measurements and reporting for NB-IoT	Nokia, Nokia Shanghai Bell

- R2-1814764	Consideration on SON in NB-IOT	Lenovo, Motorola Mobility

- R2-1814765	Consideration on SON in NB-IOT	Lenovo, Motorola Mobility

- R2-1814110	Supporting Presence of NRS on a non-anchor carrier for paging in NB-IoT	Ericsson

- R2-1814111	Support of quality report in Msg3 for non-anchor access in NB-IoT	Ericsson

- R2-1813906	Support of Quality report in Msg3 for non-anchor access	Huawei, HiSilicon

- R2-1813907	Presence of NRS on a non-anchor carrier for paging	Huawei, HiSilicon

- R2-1813961	Channel quality reporting for non-anchor carriers	Qualcomm Incorporated

- R2-1813962	Non-anchor carrier measurements for RRM	Qualcomm Incorporated

- R2-1814380	Support for quality report in Msg3 for non-anchor access in NB-IoT	ZTE Corporation

- R2-1813543	LS on RAN6 impact for Rel-16 NB-IoT enhancements related to Inter-RAT cell reselection

- R2-1813550	Reply LS on RAN6 impact for Rel-16 NB-IoT enhancements related to Inter-RAT cell reselection

- R2-1814414	Power Efficient mechanism for Inter RAT cell selection	Nokia, Nokia Shanghai Bell

- R2-1814208	Idle Cell Reselection from NB-IoT to LTE, eMTC	Qualcomm India Pvt Ltd

- R2-1814209	Idle Cell Reselection from LTE, eMTC to NB-IoT	Qualcomm India Pvt Ltd

- R2-1813908	Discussion on idle mode inter-RAT cell selection	Huawei, HiSilicon

- R2-1813816	Inter-RAT selection triggering for NB-IoT	Sequans Communications

- R2-1814109	IRAT Cell Selection for NB-IoT	Ericsson

- R2-1813720	Inter-RAT Cell Selection and Reselection for NB-IoT	MediaTek Inc.

- R2-1813721	NB-IoT Inter-RAT Cell Reselection upon State Transition	MediaTek Inc.

- R2-1814313	Consideration on inter-RAT cell selection/reselection in NB-IoT	ZTE Corporation

- R2-1814384	Consideration on issues related to coexistence of NB-IoT with NR	ZTE Corporation

- R2-1818210	Mobile-terminated Early Data Transmission	MediaTek Inc.

- R2-1817048	Analysis of Group WUS Options	Nokia, Nokia Shanghai Bell

- R2-1816546	UE grouping on wake-up signal channel	Qualcomm Incorporated

- R2-1817634	Further consideration on UE-group wake-up signal	ZTE Corporation

- R2-1818218	General aspects of group WUS	Ericsson

- R2-1816400	Group based wake-up signal	Huawei, HiSilicon

- R2-1816436	UE-Group Wake-Up Signal	Fraunhofer IIS, Fraunhofer HHI

- R2-1816956	UE-Group WUS in NB-IoT	MediaTek Inc.

- R2-1817047	Analysis of GWUS and common WUS	Nokia, Nokia Shanghai Bell

- R2-1817085	UE grouping for WUS	Sony

- R2-1817564	Consideration on wake up signal	Lenovo, Motorola Mobility

- R2-1817637	Sub-group ID calculation based on UE_ID for UE-group WUS	ZTE Corporation

- R2-1818238	UE_ID based WUS grouping	Ericsson

- R2-1816401	Uplink transmission in dedicated pre-configured resource	Huawei, HiSilicon

- R2-1816644	Transmission in preconfigured uplink resources	Ericsson

- R2-1816993	Main issues for transmission over preconfigured dedicated resource in IDLE	ZTE Corporation

- R2-1817053	Analysis for transmission over preconfigured shared resource in IDLE	ZTE

- R2-1816957	Early Transmission in Preconfigured UL Resources in NB-IoT	MediaTek Inc.

- R2-1817043	Signaling Aspects for transmission in preconfigured resources	Nokia, Nokia Shanghai Bell

- R2-1817054	Analysis for transmission over preconfigured shared resource in IDLE	ZTE

- R2-1817277	Transmission in preconfigured resources for MTC and NB-IoT	Fujitsu

- R2-1817911	Supporting UL data transmission on preconfigured resources in IDLE	Qualcomm Incorporated

- R2-1817940	Pre-configured UL Resources Design Considerations	Sierra Wireless, S.A.

- R2-1818052	Support for shared preconfigured resources in RRC_IDLE	LG Electronics UK

- R2-1818053	Discussion on transmission in preconfigured resources in RRC_IDLE	LG Electronics UK

- R2-1817558	Discussion on UE reporting the SON parameters	Ericsson

- R2-1816404	Discussion on ANR report	Huawei, HiSilicon

- R2-1817908	RACH report enhancements for SON	Qualcomm Incorporated

- R2-1816403	Discussion on RLF report	Huawei, HiSilicon

- R2-1816402	Discussion on RACH report	Huawei, HiSilicon

- R2-1816427	ANR considerations for NB-IoT	Gemalto N.V.	discussion

- R2-1817042	SON measurements and reporting for NB-IoT	Nokia, Nokia Shanghai Bell

- R2-1817501	ANR Reporting Protocol for NB-IoT	Sequans Communications

- R2-1817549	Reporting of RA and RLF in NB-IOT SON	Lenovo, Motorola Mobility

- R2-1817561	ANR procedure in NB-IOT	Lenovo, Motorola Mobility

- R2-1817784	Further consideration on RLF and ANR report	ZTE Corporation

- R2-1817807	Consideration on necessary information for RACH report	ZTE Corporation

- R2-1817808	Network management tool related measurement report procedure	ZTE Corporation

- R2-1817895	Support for quality report in Msg3 for non-anchor access in NB-IoT	ZTE Corporation

- R2-1817555	Support of quality report in Msg3 for non-anchor access in NB-IoT	Ericsson

- R2-1816547	Channel quality reporting for non-anchor carriers	Qualcomm Incorporated

- R2-1816405	Support of Quality report in Msg3 for non-anchor access	Huawei, HiSilicon

- R2-1816548	Non-anchor carrier measurements for RRM	Qualcomm Incorporated

- R2-1816406	Presence of NRS on a non-anchor carrier for paging	Huawei, HiSilicon

- R2-1817556	Supporting Presence of NRS on a non-anchor carrier for paging in NB-IoT	Ericsson

- R2-1816659	Reply LS on RAN6 impact for Rel-16 NB-IoT enhancements related to Inter-RAT cell selection

- R2-1816958	 Inter-RAT Cell Selection for NB-IoT	MediaTek Inc.

- R2-1817810	Network Assistance for UE Idle Mode Mobility between NB-IoT and LTE, eMTC, GERAN	Qualcomm India Pvt Ltd

- R2-1817553	IRAT Cell Selection for NB-IoT	Ericsson

- R2-1816407	Assistance information for idle mode inter-RAT cell selection	Huawei, HiSilicon

- R2-1816426	Considerations on NB-IoT cell selection	Gemalto N.V.

- R2-1817044	Power Efficient mechanism for Inter RAT cell selection	Nokia, Nokia Shanghai Bell

- R2-1817049	Common aspects of signaling of assistance information for Inter RAT cell selection across RATs	Nokia, Nokia Shanghai Bell

- R2-1817278	Improvements to Inter-RAT cell selection for IDLE mode	Fujitsu

- R2-1817502	Inter-RAT selection triggering for NB-IoT	Sequans Communications	discussion	R2-1813816

- R2-1817638	Further consideration on inter-RAT cell selection for NB-IoT	ZTE Corporation

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template