3GPP TSG-RAN Meeting #82	RP-182461

Sorrento, Italy, Dec 10 - 13, 2018

Agenda item:	9.6

Source: 		NTT DOCOMO, INC.

Title: 	Update of RAN4 Rel-15 NR UE features

Document for:	Discussion

1. Introduction

In this contribution, we update the RAN4 Rel-15 NR UE feature list [1] according to the RAN4 agreements. The updated feature list is attached in this contribution.

2. Discussion

PA calibration gap (UE feature 2-8)

At the last RAN4 meeting, RAN4 agreed not to specify the requirement for PA calibration gap in [2]:

It is proposed that power calibration gaps are not specified. Gaps for calibration can be scheduled by the UE itself autonomously.

Therefore, we suggest to remove this UE feature from the RAN4 UE feature list.

Proposal 1: Remove PA calibration gap (UE feature 2-8) from the RAN4 UE feature list since RAN4 decided that this feature is not specified.

SU-MIMO Interference Mitigation advanced receiver (UE feature 3-4)

At the RAN4 #89bis, RAN4 agreed to add a new row regarding intra-cell interference mitigation receiver [3]. We just capture this agreement into the list, and this feature should be optional without capability signalling as RAN4 agreed.

Agreement: Introduce R-ML SU-MIMO IM receiver as an additional RAN4 NR UE feature WI; #; Feature group; Components; Prerequisite feature groups; Need for gNB to know; Consequences if the feature is not supported by the UE; Type (See R4-17121 19); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; RAN5 implication; Remarks; Responsible WG; Recommendation for TSG-RAN. Baseband; 3-4; SU-MIMO Interference Mitigation advanced receiver; 1) R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2 with 2 RX antennas 2) R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2, 3, and 4 with 4 RX antennas; No; SU-MIMO Interference Mitigation advanced receiver capability is not possible; Type 4; No need; No need; UE supporting the feature is required to meet the Enhanced Receiver Type [TBD] requirements in TS 38.101-4; RAN4; Optional without capability signalling. NTT DOCOMO will capture it in the feature list for RAN4 for the next RAN plenary.

Proposal 2: SU-MIMO interference mitigation advanced receiver (UE feature 3-4) is optional without capability signalling as RAN4 agreed.

7.5kHz UL raster shift (UE feature 1-12)

RAN4 agreed that the feature is supported in the following additional bands in red [4]. Therefore, we just capture this agreement into the attachment.

For SUL bands and Bands n1, n2, n3, n5, n7, n8, n12, n20, n25, n28, n66, n70 and n71 defined in table 5.2-1, FREF_shift = FREF + Δshift, Δshift = 0 kHz or 7.5 kHz

Short measurement gap (UE feature 3-3)

At the RAN4 #89, RAN4 agreed the modification of UE feature 3-3 (short measurement gap) in [5]. We just capture this agreement into the list as the attachment.

3. Conclusion

In this contribution, we discuss the remaining issues on RAN4 related UE features.

Proposal 1: Remove PA calibration gap (UE feature 2-8) from the RAN4 UE feature list since RAN4 decided that this feature is not specified.

Proposal 2: SU-MIMO interference mitigation advanced receiver (UE feature 3-4) is optional without capability signalling as RAN4 agreed.

Also, the updated list of RAN4 UE features is attached in this contribution.

Proposal 3: Approve the updated list of RAN4 Rel-15 NR UE features in the attachment.

References

- RP-182171, Rel.15 NR UE feature list for RAN4, Sept. 2018

- R4-1814723, On the specification of PCG, Nov. 2018.

- R4-1814401, RAN4 #88bis report, Nov. 2018.

- R4-1816467, Support of 7.5 kHz carrier shift for additional operating bands, Nov. 2018.

- R4-1816137, On UE feature: short measurement gap, Nov. 2018.