3GPP TSG RAN Meeting #85																			RP-182486

Sorrento, Italy, Dec. 10 - 13, 2018

Source: 	Intel Corporation

Title:	Draft TP to TR 38.807 - Operational requirements: spectrum availability and regulatory requirements

Agenda item:	9.3.17

Document for:	Discussion

# Introduction

In this contribution, we provide a draft text proposal for capturing spectrum availability and regulatory requirements for frequency beyond 52.6 GHz based on submitted contributions [1] ~ [9] from RAN Plenary #82. The list of contributions that were used as reference for the text proposal is available in the reference section. We encourage companies to take a look at the text proposal and provide further feedback. The editor will use the draft text proposal as the basis for the TR38.807 update of the corresponding section in RAN#83 and RAN#84.

# Draft Text Proposal for Section 4 of TR38.807

The following is a draft text proposal for section 4 of TR38.807.

==================== Start of Text Proposal =====================

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TD 	RP-181435: "New SID: Study on NR beyond 52.6 GHz".

[3]	3GPP TR 38.805: "Study on New Radio access technology; 60 GHz unlicensed spectrum".

[4]	3GPP TR 38.913: "Study on New Radio access technology; Next Generation Access Technologies".

[5]		https://www.itu.int/pub/R-REG-RR (Edition of 2016)

[6]		https://www.itu.int/md/R15-CPM19.02-C-0001/en

[7]		APG19-3/INP-35, Australia, “PRELIMINARY VIEWS ON WRC-19 AGENDA ITEMS 1.13, 1.16, 9.1 (ISSUES 9.1.1, 9.1.5 AND 9.1.8)”, APG19-3, March, 2018, Australia

[8]		APG19-3/INP-87, China, “PRELIMINARY VIEWS ON WRC-19 AGENDA ITEMS 1.13, 1.16, 9.1 (ISSUES 9.1.1, 9.1.5, 9.1.8)”, APG19-3, March, 2018, Australia

[9]		Independent Communications Authority of South Africa. Standard procedures and guideline regarding the use of radio frequency spectrum in the E-band (71 – 76 GHz paired with 81 – 86 GHz). April 28, 2017.

[10]		Independent Communications Authority of South Africa. National Radio Frequency Plan 2018. May 25, 2018.

[11]		Independent Communications Authority of South Africa. Electronic Communications Act (36/2005): Notice regarding the Radio Frequency Spectrum Regulations 2015. March 30, 2015.

[12]		Independent Communications Authority of South Africa. Radio Frequency Spectrum Regulations (279/2015): Amendment to the Radio Frequency Spectrum Regulations, 2015. November 22, 2016.

[13]		Canada Spectrum Management and Telecommunications. Canadian Table of Frequency Allocations 2018 Edition. April 2018.

[14]		Canada Spectrum Management and Telecommunications. Technical Requirements for Fixed Lineof-Sight Radio Systems Operating in the Bands 71-76 GHz and 81-86 GHz. May 2017.

[15]		MIIT, “Notice on wireless technical applications with very low transmission power (short distance) in 60GHz. 2006-082”  (关于60GHz 频段微功率（短距离）无线电技术应用有关问题的通知 2006-082) http://www.miit.gov.cn/n1146295/n1146592/n1146754/n1235566/n1235603/n1235609/n1235611/c3176263/part/3176264.pdf .

[16]		MIIT, “Technical requiremeng of wireless devices with very low transmission power (short distance 2005-423).” (微功率（短距离）无线电设备的技术要求 2005-043) http://www.miit.gov.cn/n1146295/n1146592/n1146754/n1235566/n1235603/n1235609/n1235611/c3176241/part/3176242.pdf [DCM1]	ARIB STD-B43 v2.1, “PORTABLE MILLIMETER-WAVE DIGITAL TRANSMISSION SYSTEM FOR TELEVISION PROGRAM CONTRIBUTION,” Jan. 2018.

[17]	ARIB STD-B43 v2.1, “PORTABLE MILLIMETER-WAVE DIGITAL TRANSMISSION SYSTEM FOR TELEVISION PROGRAM CONTRIBUTION,” Jan. 2018.

[18]	ARIB STD-B69 v4.0, “LOW POWER DATA COMMUNICATION SYSTEM/MILLIMETER-WAVE VIDEO TRANSMISSION EQUIPMENT,” Sept. 2016.

[19]	ARIB STD-B117 v1.0, “LOW POWER DATA COMMUNICATION SYSTEM/60 GHz-BAND WIRELESS LAN FOR VERY HIGH THROUGHPUT DATA COMMUNICATIONS,” Sept. 2016.

[20]	ARIB STD-B48 v2.2, “MILLIMETER-WAVE RADAR EQUIPMENT FOR SPECIFIED LOW POWER RADIO STATION,” Dec. 2015.

[21]	ARIB STD-B111 v1.1, “79 GHz BAND HIGH-RESOLUTION RADAR,” Mar. 2017.

[22]		Republic of Korea Communications Agency. Republic of Korea Frequency Allocations Status. May 2018.

[23]		Republic of Korea National Radio Research Agency. National Radio Research Agency Publication 2014-12. July 02, 2014.

[24]		Republic of Korea Ministry of Science and ICT. Ministry of Science and ICT Publication 2018-3. July 02, 2014.

[25]		Republic of Korea Ministry of Science and ICT. Ministry of Science and ICT Publication 2018-4. July 02, 2014.

[26]	NFAP-18		“National Frequency Allocation Plan – 2018”, Government of India Ministry of Communications

[27]		IMDA (Infocomm Media Development Authority), IMDA TS SRD, Aprail 2018, https://www.imda.gov.sg/-/media/imda/files/regulation-licensing-and-consultations/ict-standards/telecommunication-standards/radio-comms/imdatssrd.pdf?la=en [SS2]		No. 2018-36 	Ministry of Science and ICT notice

[28]		ACMA (Australian Communications and Media Authority), “Radiocommunications (Low Interference Potential Devices) Class Licence 2015”, 2018. https://www.legislation.gov.au/Details/F2018C00500

[29]		No. 2018-71 	Ministry of Science and ICT notice

[30]		No. 2018-26 	National Radio Research Agency notice

[31]	No. 2018-36 	Ministry of Science and ICT notice

---------- unchanged text omitted -----------

# 4	Operational Requirements

## 4.1	Overview of Global Spectrum Availability

In 2015, the international telecommunication union (ITU) proposed 11 candidate millimeter-wave bands between 24 and 86 GHz for the deployment of future fifth mobile generation (5G/IMT-2020/NR) broadband systems. Table 4.1-1 provide the candidate band identified in WRC-19 between 24 and 86 GHz.

Table 4.1-1 IMT-2020 candidate bands in WRC-19 AI 1.13

| Candidate frequency bands | Candidate frequency bands of requirement additional conditions |
| --- | --- |
| 24.25-27.5 GHz | 31.8-33.4 GHz, |
| 37-40.5 GHz | 40.5-42.5 GHz |
| 42.5-43.5 GHz | 47-47.2 GHz |
| 45.5-47 GHz |  |
| 47.2-50.2 GHz |  |
| 50.4-52.6 GHz |  |
| 66-76 GHz |  |
| 81-86 GHz |  |

Table 4.1-2 provides information on the allocation within the frequency range 52.6 GHz to 116 GHz in ITU Radio Regulation [5]. Protection of some incumbent services may apply and may incur in-band and/or out-of-band limitations to IMT-2020 systems. Such incumbent services are documented in the comments column of Table 4.1-2, but there is no definition on the incurred limitations in Radio Regulation.

Table 4.1-2: Frequency bands in the range 52.6 to 116 GHz in radio regulation

| Frequency band (GHz) | Allocated to Mobile Service on a primary basis | Allocated to Fixed Service on a primary basis | Comments |
| --- | --- | --- | --- |
| 52.6-54.25 | No | No | EESS (passive) and SRS (passive), All emissions are prohibited in this band, footnote 5.340 |
| 54.25-55.78 | No | No | EESS (passive) and SRS (passive) |
| 55.78-59 | Yes | Yes | EESS (passive) and SRS (passive) This band available for high-density applications in the fixed service, footnote 5.547 |
| 59-59.3 | Yes | Yes | EESS (passive) and SRS (passive) Radiolocation |
| 59.3-64 | Yes | Yes | Radiolocation |
| 64-65 | Yes | Yes | This band available for high-density applications in the fixed service, footnote 5.547 |
| 65-66 | Yes | Yes | This band available for high-density applications in the fixed service, footnote 5.547 |
| 66-71 | Yes | No | WRC-19 AI 1.13 frequency band, sharing and compatibility studies and potential limitations information in section 2.2. |
| 71-76 | Yes | Yes | WRC-19 AI 1.13 frequency band, sharing and compatibility studies and potential limitations information in section 2.2. |
| 76-81 | No | No | Radiolocation |
| 81-86 | Yes | Yes | WRC-19 AI 1.13 frequency band, sharing and compatibility studies and potential limitations information in section 2.2. |
| 86-92 | No | No | EESS (passive) and SRS (passive), All emissions are prohibited in this band, footnote 5.340 |
| 92-94 | Yes | Yes | Radiolocation |
| 94-94.1 | No | No | Radiolocation |
| 94.1-95 | Yes | Yes | Radiolocation |
| 95-100 | Yes | Yes | Radiolocation |
| 100-102 | No | No | EESS (passive) and SRS (passive), All emissions are prohibited in this band, footnote 5.340 |
| 102-105 | Yes | Yes | N/A |
| 105-109.5 | Yes | Yes | SRS (passive) |
| 109.5-111.8 | No | No | EESS (passive) and SRS (passive), All emissions are prohibited in this band, footnote 5.340 |
| 111.8-114.25 | Yes | Yes | SRS (passive) |
| 114.25-116 | No | No | EESS (passive) and SRS (passive), All emissions are prohibited in this band, footnote 5.340 |

Within the range 52.6 to 116 GHz, the frequency bands 66-71, 71-76 and 81-86 GHz are being studied under WRC-19 Agenda Item 1.13 for potential IMT identification. Results of sharing and compatibility studies, potential technical and regulatory conditions are included in Draft CPM Report [6], and the final decisions are to be made in WRC-19 with respect to IMT identification or no IMT identification, along with the corresponding technical and regulatory conditions.

- For 66-71 GHz, Studies were carried out for the ISS and MSS (Earth-to-space), and sharing is feasible.

- For 71-76 GHz, Studies were carried out for the FS, RLS and FSS. Sharing with FS and FSS is feasible.

  - In order to protect the radiolocation service in the frequency band 76-81 GHz, the unwanted emission limits into the 76-81 GHz band from IMT BSs and UEs operating in the 71-76 GHz band shall comply with the limits given in Table below (note: the exact values are from individual compatibility studies):

| Limits of unwanted emissions into 76-81 GHz from IMT stations Station | 76-77 GHz dB(W/200 MHz) | 77-81 GHz dB(W/200 MHz) |
| --- | --- | --- |
| BS | [TBD/−29.6/−31.5/< −37] | [TBD/−33/< −37] |
| UE | [TBD/−20/< −37] | [TBD/−35/< −37] |

- For 81-86 GHz, Studies were carried out for the FS, FSS, RAS (in band and adjacent band), EESS (passive) and RLS. Sharing with FS, FSS and RAS (in band and adjacent band) is feasible.

  - In order to protect the radiolocation service in the frequency band 76-81 GHz, the unwanted emission limits into the 76-81 GHz band from IMT BSs and UEs operating in the 81-86 GHz band shall comply with the limits given in Table below (note: the exact values are from individual compatibility studies):

| Limits of unwanted emissions into 76-81 GHz from IMT stations Station | 76-77 GHz dB(W/200 MHz) | 77-81 GHz dB(W/200 MHz) |
| --- | --- | --- |
| BS | [TBD/−29.6/−31.5/< −37] | [TBD/−33/< −37] |
| UE | [TBD/−20/< −37] | [TBD/−35/< −37] |

  - In order to protect the EESS passive service in the frequency band 86-92 GHz, the unwanted emission limits into the 86-92 GHz band from IMT BSs and UEs operating in the 81-86 GHz bands shall comply with the limits given in Table below (note: the exact values are from individual compatibility studies):

| Limits of unwanted emissions into 86-92 GHz from IMT stations Station | dB(W/100 MHz) |
| --- | --- |
| BS | TBD/-43.6/-49.8/-31.3/-20 |
| UE | TBD/-43.5/-49.9/-31.2/-19.9 |

Moreover, in ITU region-3, the meeting of the APT Conference Preparatory Group for World Radiocommunication Conference 2019 (APG19-3) were held by Aisa-Pacfic Telecommunity (APT) to provide APT preliminary views on WRC-19 Agenda Items and issues associated with each WRC-19 Agenda Item. With regarding WRC-19 Agenda Item 1.13, both China and Australia expressed their interesting bands beyond 52.6GHz:

- China is considering to identify the frequency bands above 43.5GHz especially 66-71GHz, 71-76 GHz and 81-86 GHz under AI 1.13 for IMT towards WRC-19, if sharing with relevant radio services is feasible depending on needs [7].

- Australia is also interested in investigating the possibility of an IMT identification in some or all of the 37-43.5 GHz and 66-76 GHz bands [8].

It is notable that it is possible to be updated views by administration, depending on further meetings.

## 4.2	Country Specific Spectrum Availability and Spectrum Regulatory Requirements

Table 4.2-1 and 4.2-2 provide an overall summary of spectrum availability for data communication (excluding radar based telemetry) between frequencies 52.6GHz and 100GHz for various countries under ITU region 1, 2, and 3. Symbol ‘U’, ‘L’, and ‘G’ represent unlicensed spectrum, licensed spectrum (including light licensing and other licensing forms), government only spectrum, respectively.

Table 4.2-1: Spectrum availability for various countries between frequency 52.6GHz and 71GHz

| Region | Country | Frequency (GHz) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 52.6-54.25 | 54.25-55.78 | 55.78-56.9 | 56.9-57 | 57-58.2 | 58.2-59 | 59-59.3 | 59.3-64 | 64-65 | 65-66 | 66-71 |
| ITU Region 1 | Europe/CEPT |  |  |  |  | U |  |  |  |  |  |  |
|  | Israel |  |  |  |  |  |  |  |  |  |  |  |
|  | South Africa |  |  |  |  | U |  |  |  | U |  |  |
| ITU Region 2 | USA |  |  |  |  | U |  |  |  |  |  |  |
|  | Canada |  |  |  |  | U |  |  |  |  |  |  |
|  | Brazil |  |  |  |  | U |  |  |  |  |  |  |
|  | Mexico |  |  |  |  | U |  |  |  |  |  |  |
| ITU Region 3 | China |  |  |  |  |  |  | U |  |  |  |  |
|  | Japan |  |  |  |  | U |  |  |  |  |  |  |
|  | Korea |  |  |  |  | U |  |  |  |  |  |  |
|  | India |  |  |  |  |  |  |  |  |  |  |  |
|  | Taiwan |  |  |  |  |  |  |  |  |  |  |  |
|  | Singapore |  |  |  |  | U |  |  |  |  |  |  |
|  | Australia |  |  |  |  | U |  |  |  |  |  |  |

Table 4.2-2: Spectrum availability for various countries between frequency 71GHz and 100GHz

| Region | Country | Frequency (GHz) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 71-74 | 74-76 | 76-77 | 77-81 | 81-84 | 84-86 | 86-92 | 92-94 | 94-94.1 | 94.1-95 | 95-100 |
| ITU Region 1 | Europe/CEPT | L |  |  |  | L |  |  | L | L |  |  |
|  | Israel |  |  |  |  |  |  |  |  |  |  |  |
|  | South Africa | L |  |  |  | L |  |  |  |  |  |  |
| ITU Region 2 | USA | L |  |  |  | L |  |  | L | G | L |  |
|  | Canada | L |  |  |  | L |  |  |  |  |  |  |
|  | Brazil | L |  |  |  | L |  |  |  |  |  |  |
|  | Mexico | L |  |  |  | L |  |  |  |  |  |  |
| ITU Region 3 | China |  |  |  |  |  |  |  |  |  |  |  |
|  | Japan | L |  |  |  | L |  |  |  |  |  |  |
|  | Korea | L |  |  |  | L |  |  |  |  |  |  |
|  | India |  |  |  |  |  |  |  |  |  |  |  |
|  | Taiwan |  |  |  |  |  |  |  |  |  |  |  |
|  | Singapore | L |  |  |  | L |  |  |  |  |  |  |
|  | Australia |  |  |  |  |  |  |  |  |  |  |  |

### 4.2.1	ITU Region 1

#### 4.2.1.1	Europe and CEPT

General information for usages in CEPT can be found in THE EUROPEAN TABLE OF FREQUENCY ALLOCATIONS AND APPLICATIONS IN THE FREQUENCY RANGE 8.3 kHz to 3000 GHz (ECA TABLE). When it comes to Regulation about Short Range Devices additional information can be found in Recommendation ERC/REC 70-03 “Relating to the use of Short Range Devices (SRD)”.  Unwanted emission limits can be found in ECC Recommendation (02)/05 “Unwanted Emissions” and ERC/RECOMMENDATION 74-01 “UNWANTED EMISSIONS IN THE SPURIOUS DOMAIN”. It should however be pointed out that the Rec 74-01 is under review with estimated finalization date 2019-05-24.

The bands above 52.6 GHz that are discussed due to WRC-19 AI 1.13 (IMT), within CEPT are three, 66-71 GHz as IMT candidate band and the bands 71-76/81-86 GHz as no IMT bands. The band 71-76 GHz, paired with 81-86 GHz, is a fixed link band important for backhauling of 5G. Therefor fixed link usage is expected to increase in the future.

From a European perspective, the focus should be on bands that are proposed for IMT as well as on the unlicensed band 57-66 GHz. The current status, seen by CEPT, for IMT bands can be found in the draft ECP, European Common Proposal and Brief for WRC-19 Agenda Item 1.13. There are no limitations concerning output powers or channel bandwidth in the 66-71 GHz band, only that the band 66-71 GHz is planned for “Future Civil systems”.

Table 4.2.1.1-1 and Table 4.2.1.1-2 provide summary of frequency bands in the range of 52.6 and 100 GHz and regulatory requirements for spectrum between 52.6 GHz and 114.25 GHz, respectively.

Table 4.2.1.1-1: Frequency bands in the range 52.6 to 100 GHz

| Frequency range in GHz | Comments | Region |
| --- | --- | --- |
| 52.6 to 54.25 | EESS service, all emissions are prohibited, (ref ITU-R Radio Regulation footnote 5.340 | ITU/CEPT |
| 55.78 to 57 | Recommended to be used by fixed services (ref: ERC Recommendation 12-12 and ITU-R Radio Regulation footnote 5.547) | ITU/CEPT |
| 57 to 66 | Several usage of Short range devices (ref ERC/REC 70-03) and EC Decision about SRD (ref COMMISSION IMPLEMENTING DECISION (EU) 2017/1483 of 8 August 2017) 57-64 GHz recommended by CEPT for fixed service limited to point to point (ref: ECC recommendation (09)01) 64-66 GHz recommended by CEPT for fixed service limited to point to point (ref: ECC recommendation (05)02) 63-64 GHz ITS band in Europe (ref: ECC decision (09)01) | EC/CEPT |
| 66 to 71 | Planned for “Future civil systems” There is currently no limitations on channel bandwidths, output powers etc. Recommended by CEPT for IMT identification (ref: draft new resolution [A113-IMT 66/71 GHz, Output from ECC PT1 meeting in Sep 2018]). GSA considers this band as an important band for 5G | CEPT |
| 71 to 76 and 81 to 86 | Recommended by CEPT for fixed services (ref: ECC recommendation (05)07) | CEPT |
| 77 to 81 | Automotive RADAR in Europe (ref: ECC decision (04)03 and ref Commission Decision of 8 July 2004 on the harmonisation of radio spectrum in the 79 GHz range for the use of automotive short-range radar equipment in the Community | EC/CEPT |
| 86 to 92 | EESS service, all emissions are prohibited (ref ITU-R Radio Regulation footnote 5.340) | ITU/CEPT |
| 92 to 100 | Recommended by CEPT for fixed services (ref: ECC recommendation (18)02) | CEPT |

Table 4.2.1.1-2: Europe/CEPT – Regulatory Requirements for available and potential spectrum between 52.6 GHz and 114.25 GHz

| Frequency band (GHz) |  | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  | 57 – 66 | 40 dBm mean e.i.r.p.. This refers to the highest power level of the transmitter power control range during the transmission burst if transmitter power control is implemented | Adequate spectrum sharing mechanism (e.g. Listen-before-Talk, Detect-And-Avoid) shall be implemented by the equipment | Not specified |  | The maximum mean e.i.r.p. density is limited to 13 dBm/MHz. Point-to-point links of the Fixed Service are regulated by ECC/REC/ (05)02 and ECC/REC/ (09)01 |
|  | 66 – 71 | To be defined | To be defined | To be defined | To be defined |  |
|  | 71-76, 81-86 | Maximum power limit= 30dBm Maximum EIRP= 85dBm (55 dBW) Minimum antenna gain= Pout (dBm) + 15; or 38 (whichever is the greater) maximum antenna gain= 85-Pout(dBm) Provision for reduced EIRP, see ETSI EN 302 217 Minimum spectral efficiency, see ETSI Radio Interface capacity (RIC) Complete set of specification, ETSI EN 302 217 part 2-2 and part 3 | Access to the frequency bands by Authorization regimes or by Block assignment/auction regimes . | An aggregation of 250 MHz channels for wider channels as defined in ECC/REC/(05)07 (channel arrangements for channel sizes ranging from 250 to 4750 MHz) | Fixed Services To assist the planning of fixed links, self-coordination approach, similar to the “light licensing”, described in ECC Report 80 can be considered. |  |
|  | 92-94, 94.1-100, 102-109.5, 111.8-114.25 | For Fixed Service, defined in ECC Recommendation (18)02 | For Fixed Service, defined in ECC Recommendation (18)02 | Radio frequency channel/block arrangements for Fixed Service are defined in ECC Recommendation (18)02 | The choice of the appropriate assignment method and licensing regime for Fixed Service remains a decision for national administrations, based on several technical and not technical factors | To assist the planning of fixed links, self-coordination approach, similar to the “light licensing”, described in ECC Report 80 can be considered. |

#### 4.2.1.2	Israel

#### 4.2.1.3	South Africa

Table 4.2.1.3-1 shows summary of the spectrum allocations for South Africa for frequency between 52.6GHz and 100GHz, based on [9]. Primary services are listed in “capitals:”, e.g., MOBILE, and secondary services are listed in “normal characters;” e.g. Amateur.

Table 4.2.1.3-1: South Africa spectrum allocation between 52.6 GHz and 100 GHz

| Frequency [GHz] | Allocations | Notes and ITU Radio Regulations Footnotes |
| --- | --- | --- |
| 52.6-54.25 | EARTH EXPLORATION-SATELLITE (passive) SPACE RESEARCH (passive) | 5.340, 5.556; |
| 54.25-55.78 | EARTH EXPLORATION-SATELLITE (passive) INTER-SATELLITE SPACE RESEARCH (passive) | 5.556A; |
| 55.78-56.9 | EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE MOBILE SPACE RESEARCH (passive) | 5.557A, NF14, 5.556A, 5.558, 5.547; |
| 56.9-57 | EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE MOBILE SPACE RESEARCH (passive) | NF14, 5.558, 5.547; |
| 57-58.2 | EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE MOBILE SPACE RESEARCH (passive) | NF14, 5.556A, 5.558; |
| 58.2-59 | EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) | NF14, 5.547, 5.556 |
| 59-59.3 | EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE INTER-SATELLITE RADIOLOCATION SPACE RESEARCH (passive) | 5.556A, 5.558 |
| 59.3-64 | FIXED MOBILE INTER-SATELLITE RADIOLOCATION | 5.558, 5.559 The band 61-61.5 GHz is designated for ISM applications (5.138). The band 59 - 61 GHz reserved for government use. |
| 64-65 | FIXED INTER-SATELLITE MOBILE except aeronautical mobile | 5.547, 5.556 |
| 65-66 | EARTH EXPLORATION-SATELLITE FIXED INTER-SATELLITE MOBILE except aeronautical mobile SPACE RESEARCH | 5.547 |
| 66-71 | INTER-SATELLITE MOBILE MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION-SATELLITE | 5.553, 5.558, 5.554 |
| 71-74 | FIXED FIXED-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (space-to-Earth) | Paired with 81 – 86 GHz. |
| 74-76 | FIXED FIXED-SATELLITE (space-to-Earth) MOBILE BROADCASTING BROADCASTING-SATELLITE Space research (space-to-Earth) | 5.561 Paired with 81 – 86 GHz. |
| 76-77.5 | RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite Space research (space-to-Earth) | 5.149 Radio Frequency Spectrum Regulations as amended (Annex B) (GG. No. 38641, 30 March 2015) [10]. |
| 77.5-78 | AMATEUR AMATEUR-SATELLITE Radio astronomy Space research (space-to-Earth) | 5.149 |
| 78-79 | RADIOLOCATION Amateur Amateur-satellite Space research (space-to-Earth) | 5.149, 5.560 |
| 79-81 | RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite Space research (space-to-Earth) | 5.149 |
| 81-84 | FIXED FIXED-SATELLITE (Earth-to-space) MOBILE MOBILE-SATELLITE (Earth-to-space) RADIO ASTRONOMY Space research (space-to-Earth) | 5.338A, NF14, 5.149, 5.561A Paired with 71-76 GHz. |
| 84-86 | FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIO ASTRONOMY | 5.561B, 5.338A, NF14 |
| 86-92 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) | 5.340 |
| 92-94 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION | 5.149 |
| 94-94.1 | RADIOLOCATION Radio astronomy | 5.562, 5.562A |
| 94.1-95 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION | NF14, 5.149 |
| 95-100 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION RADIONAVIGATION RADIONAVIGATION-SATELLITE | 5.149, 5.554 |
| Note NF14: The frequency bands 71GHz – 76GHz paired with 81GHz – 86GHz are allocated to the fixed services and is earmarked for very high capacity Broadband Fixed Wireless Systems over very short hop lengths. Radio frequency channel arrangements for fixed service systems operating in the bands 71-76GHz and 81-86GHz are according to the Radio Frequency Spectrum Regulations (GG. No.38641, 30 March 2015) [10]. |  |  |

The use of E-band, 73.375 – 75.875GHz paired with 83.375 – 85.375GHz, is based on point-to-point light-licensing in units of 2 x 250 MHz channels. The band is segmented into two blocks, Block A and Block B. Block A is for self-co-ordinated spectrum. Block B is for ICASA-coordinated spectrum. The channel allocations for Block A and B are shown in Figure 4.2.1.3-1.

Figure 4.2.1.3-1: South Africa E-band channel arrangements (from [11])

Table 4.2.1.3-2 shows summary of regulatory requirements for frequency ranges that are designed for mobile and fixed link use between 52.6GHz and 100GHz in South Africa, which are based on [11] and [12].

Table 4.2.1.3-2: South Africa regulatory requirements for frequency between 52.6GHz and 100GHz

| Frequency band [GHz] |  | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  | 57 – 64 | Max EIRP 55dBm Max Tx power 10dBm. Min antenna gain 30dBi. | No requirements. |  | Point-to-point links | Unlicensed. Use EN 305 550 standard. |
|  | 57 – 66 | Max EIRP 40dBm | The use of these systems is as described in ITU-R Report ITU-R M.2227 and Recommendation ITU-R M.2003. |  | Multi-gigabit wireless systems. Fixed outdoor installers are not allowed. | Unlicensed. Use EN 302 567 standard. |
|  | 76 – 77 | Max peak power 55dBm | No duty cycle restriction. No channel spacing. |  | RTTT radar | Use EN 300 091, EN 301 489-1,3, and EN60950 standard |
|  | 71 – 76 | Max Tx power 35 dBm.. Max EIRP 85dBm (Gant > 55dBi), Max EIRP 85 – (55-Gant) dBm (55dBi ≥ Gant > 45 dBi), and Max EIRP 75 – 2*(45-Gant) (45dBi ≥ Gant > 38 dBi), where Gant: antenna gain. Max PSD 150mW/100MHz. Min antenna gain 38 dBi. | N/A |  | Fixed links | Light licensed |
|  | 81 – 86 | Max Tx power 35 dBm.. Max EIRP 85dBm (Gant > 55dBi), Max EIRP 85 – (55-Gant) dBm (55dBi ≥ Gant > 45 dBi), and Max EIRP 75 – 2*(45-Gant) (45dBi ≥ Gant > 38 dBi), where Gant: antenna gain. Max PSD 150mW/100MHz. Min antenna gain 38 dBi. | N/A |  | Fixed links | Light licensed |

### 4.2.2	ITU Region 2

#### 4.2.2.1	USA

Table 4.2.2.1-1 shows the spectrum allocations for USA for frequency between 52.6GHz and 100GHz. Primary services are listed in “capitals:”, e.g., MOBILE, and secondary services are listed in “normal characters;” e.g. Amateur. The FCC rules part(s) and additional requirements are rules and regulations that pertain to specified frequency ranges and detailed description of the rules and regulations are listed in FCC code of federal regulations Title 47 Part 2.

Table 4.2.2.1-1: USA spectrum allocation between 52.6 GHz and 100 GHz

| Frequency [GHz] | Allocations | Governing FCC Rule Part(s) |
| --- | --- | --- |
| 52.6-54.25 | EARTH EXPLORATION-SATELLITE (passive) SPACE RESEARCH (passive) | General Rules and Regulations (2.105 -US246) |
| 54.25-55.78 | EARTH EXPLORATION-SATELLITE (passive) INTER-SATELLITE SPACE RESEARCH (passive) | Satellite Communications (25) General Rules and Regulations (2.105 -5.556A) |
| 55.78-56.9 | EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE MOBILE SPACE RESEARCH (passive) | Satellite Communications (25) General Rules and Regulations (2.105 -US353, US532, US379, 5.556A, 5.558) |
| 56.9-57 | EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) | General Rules and Regulations (2.105 - US532, 5.558) |
| 57-58.2 | EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE MOBILE SPACE RESEARCH (passive) | RF Devices (15) Satellite Communications (25) General Rules and Regulations (2.105 -US532, 5.556A, 5.558) |
| 58.2-59 | EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) | RF Devices (15) General Rules and Regulations (2.105 -US353, US354) |
| 59-59.3 | EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE RADIOLOCATION SPACE RESEARCH (passive) | RF Devices (15) General Rules and Regulations (2.105 -US353, 5.558, 5.559) |
| 59.3-64 | FIXED MOBILE RADIOLOCATION | RF Devices (15) ISM Equipment (18) General Rules and Regulations (2.105 -5.138, US353, 5.559, 5.558) |
| 64-65 | FIXED MOBILE except aeronautical mobile | RF Devices (15) |
| 65-66 | EARTH EXPLORATION-SATELLITE FIXED INTER-SATELLITE MOBILE except aeronautical mobile SPACE RESEARCH | RF Devices (15) Satellite Communications (25) |
| 66-71 | INTER-SATELLITE MOBILE MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION-SATELLITE | RF Devices (15) Satellite Communications (25) General Rules and Regulations (2.105 -5.554, 5.553, 5.558) |
| 71-74 | FIXED FIXED-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (space-to-Earth) | Fixed Microwave (101) General Rules and Regulations (2.105 -US389) |
| 74-76 | FIXED FIXED-SATELLITE (space-to-Earth) MOBILE BROADCASTING BROADCASTING-SATELLITE Space research (space-to-Earth) | RF Devices (15) Fixed Microwave (101) General Rules and Regulations (2.105 -US389) |
| 76-77 | RADIO ASTRONOMY RADIOLOCATION Amateur Space research (space-to-Earth) | RF Devices (15) Personal Radio (95) Amateur Radio (97) General Rules and Regulations (2.105 -US342) |
| 77-81 | RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite Space research (space-to-Earth) | RF Devices (15) Personal Radio (95) Amateur Radio (97) General Rules and Regulations (2.105 -5.560, US342) |
| 81-84 | FIXED FIXED-SATELLITE (Earth-to-space) MOBILE MOBILE-SATELLITE (Earth-to-space) RADIO ASTRONOMY Space research (space-to-Earth) | RF Devices (15) Fixed Microwave (101) General Rules and Regulations (2.105 -US161, US342, US389, US297) |
| 84-86 | FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIO ASTRONOMY | RF Devices (15) Fixed Microwave (101) General Rules and Regulations (2.105 -US161, US342, US389) |
| 86-92 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) | RF Devices (15) Fixed Microwave (101) General Rules and Regulations (2.105 -US246, US74) |
| 92-94 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION | RF Devices (15) Fixed Microwave (101) General Rules and Regulations (2.105 -US161, US342) |
| 94-94.1 | RADIOLOCATION Radio astronomy | RF Devices (15) General Rules and Regulations (2.105 -5.562A) |
| 94.1-95 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION | RF Devices (15) Fixed Microwave (101) General Rules and Regulations (2.105 -US161, US342) |
| 95-100 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION RADIONAVIGATION RADIONAVIGATION-SATELLITE | General Rules and Regulations (2.105 -5.554, US342) |

Among the spectrum allocations for U.S.A., frequency ranges 57GHz to 71GHz are available for mobile use as part of unlicensed spectrum regulated by Title 47 Part 15 of the FCC regulations. Frequency ranges 92 to 95 GHz are available for fixed indoor wireless use as part of unlicensed spectrum regulated by Title 47 Part 15 of the FCC regulations. Frequency ranges 71GHz to 74GHz, 81GHz to 86GHz, and 92GHz to 95GHz are available for mobile use as part of licensed spectrum using universal licensing system regulated by Title 47 Part 101 of the FCC regulations.

The 70/80/90GHz bands in US are licensed on the basis of non-exclusive nationwide licenses. There is no limit to the number of non-exclusive nationwide licenses that may be granted for these bands, and these licenses will serve as a prerequisite for registering individual links. An entity may request any portion of the 71-76GHz and 81-86GHz bands, up to 5 gigahertz in each segment for a total of 10 gigahertz. Licensees are also permitted to register smaller segments. The 92-95GHz band is divided into three segments: 92.0-94.0GHz and 94.1-95.0 GHz for non-government and government users, and 94.0-94.1 GHz for Federal Government use. Pairing is allowed and segments may be aggregated without limit (possible 12.9 gigahertz maximum aggregation). Licensees may use the 70GHz, 80GHz and 90GHz bands for any point-to-point, non-broadcast service. The segments may be unpaired or paired, but paring will be permitted only in a standardized manner (e.g., 71-72.25GHz may be paired only with 81-82.25GHz, and so on).

Table 4.2.2.1-2 shows regulatory requirements for frequency ranges that are designed for mobile use between 52.6GHz and 100GHz in U.S. It should be noted that no channel occupancy time, occupied channel bandwidth, and listen-before-talk requirements exist for unlicensed operations in above 52.6GHz.

Table 4.2.2.1-2: USA regulatory requirements for frequency between 52.6GHz and 100GHz that are available for fixed and mobile use

| Frequency band [GHz] |  | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  | 57 – 71 | Max avg, EIRP 40dBm Max peak EIRP 43dBm If emission-BW is less than 100 MHz, max peak conducted output power is {500mW * emission-BW / 100MHz} [Note 1] Otherwise, max peak conducted output power is 500mW | No requirements. | No specified requirements. | Equipment other than fixed outdoor. [Note 4] | Unlicensed. Note 2 Note 3 Note 10 |
|  | 57 – 71 | Max avg. EIRP (82 – 2N) dBm Max peak EIRP (85 – 2N) dBm. N = max(0, 51 dBi – antenna-gain) If emission-BW is less than 100 MHz, max peak conducted output power is {500mW * emission-BW / 100MHz} [Note 1] Otherwise, max peak conducted output power is 500mW | No requirements. | No specified requirements. | Fixed outdoor equipment | Unlicensed. Note 10 |
|  | 71 – 76 | Max EIRP 55dBW, Max Tx Power 5dBW Max Tx PSD 150mW/100MHz [Note 8] [Note 9] | Sharing and coordination among non-government licensees and between non-government and government services. | Max of 5 GHz [Note 5] | Any point-to-point, non-broadcast service | Lightly licensed |
|  | 81 – 86 | Max EIRP 55dBW Max Tx power 5dBW Max Tx PSD 150mW/100MHz [Note 8] [Note 9] | Sharing and coordination among non-government licensees and between non-government and government services. | Max of 5 GHz [Note 5] | Any point-to-point, non-broadcast service | Lightly licensed |
|  | 92 – 94 | Max EIRP 55dBW [Note 8] [Note 9] | Sharing and coordination among non-government licensees and between non-government and government services. | Max of 2 GHz [Note 5] | Any point-to-point, non-broadcast service | Lightly licensed |
|  | 94 – 94.1 | Max EIRP 55dBW [Note 8] [Note 9] | N/A |  | Any point-to-point, non-broadcast service (Government use only) | Lightly Licensed |
|  | 94.1 – 95 | Max EIRP 55dBW [Note 8] [Note 9] | Sharing and coordination among non-government licensees and between non-government and government services. | Max of 0.9 GHz [Note 5] | Any point-to-point, non-broadcast service | Lightly Licensed |
|  | 92 – 95 | Max avg. PSD 9uW/sq.cm [Note 6] Max peak PSD 18uW/sq. cm [Note 6] Max Tx power 500mW | Sharing and coordination among non-government licensees and between non-government and government services. | No specified requirements. | Fixed Indoor Only | Unlicensed. Note 2 Note 7 Note 10 |
|  | 95 – 100, 102 – 109.5, 111.8 – 114.25 | Rules being defined by FCC as part of the Spectrum Horizons 95-275GHz proceeding | Rules being defined by FCC as part of the Spectrum Horizons 95-275GHz proceeding | Rules being defined by FCC as part of the Spectrum Horizons 95-275GHz proceeding | There are passive services like radio astronomy service (RAS), Earth exploration-satellite service (EESS) (passive), and space research service (SRS) (passive) in the gaps (100-102GHz and 109.5-111.8 GHz) between bands considered | Licensed Note 12 |
| Note 1: Emission bandwidth is defined as the instantaneous frequency range occupied by a steady state radiated signal with modulation, outside which the radiated power spectral density never exceeds 6 dB below the maximum radiated power spectral density in the band, as measured with a 100kHz resolution bandwidth spectrum analyzer. The center frequency must be stationary during the measurement interval, even if not stationary during normal operation (e.g., for frequency hopping devices). Peak transmitter conducted output power shall be measured with an RF detector that has a detection bandwidth that encompasses the 57-71GHz band and that has a video bandwidth of at least 10MHz. |  |  |  |  |  |  |
| Note 2: The power density of any emissions outside the 57-71GHz band shall consist solely of spurious emissions. Radiated emissions below 40GHz shall not exceed the general limits of 5 uV/m measured at a distance of 3 m. Between 40 GHz and 200 GHz, the level of these emissions shall not exceed 90 pW/cm2 at a distance of 3m. The levels of the spurious emissions shall not exceed the level of the fundamental emission. |  |  |  |  |  |  |
| Note 3: Equipment is presumed to operate over the temperature range −20 to + 50 degrees Celsius with an input voltage variation of 85% to 115% of rated input voltage, unless justification is presented to demonstrate otherwise. |  |  |  |  |  |  |
| Note 4: Operation on aircraft is permitted under the following conditions: (1) when the aircraft is on the ground, and (2) while airborne, only in closed exclusive on-board communication networks within the aircraft, with the following exceptions: (i) Equipment shall not be used in wireless avionics intra-communication (WAIC) applications where external structural sensors or external cameras are mounted on the outside of the aircraft structure. (ii) Equipment shall not be used on aircraft where there is little attenuation of RF signals by the body/fuselage of the aircraft. These aircraft include, but are not limited to, toy/model aircraft, unmanned aircraft, crop-spraying aircraft, aerostats, etc. |  |  |  |  |  |  |
| Note 5: Licensees are also permitted to register smaller bandwidth segments and segments may be aggregated without limit (up to 12.9 gigahertz maximum aggregation). |  |  |  |  |  |  |
| Note 6: measured at 3 meters from the radiating structure. |  |  |  |  |  |  |
| Note 7: The use of outdoor mounted antennas, e.g., antennas mounted on the outside of a building or on a telephone pole, or any other outdoors infrastructure is prohibited. The emissions from equipment operated under this section shall not be intentionally directed outside of the building in which the equipment is located, such as through a window or a doorway. Spurious emission Between 40 GHz and 200 GHz, the level of these emissions shall not exceed 90 pW/cm2 at a distance of 3 meters. |  |  |  |  |  |  |
| Note 8: The mean power of emissions must be attenuated below the mean output power of the transmitter in any 1 MHz band by A, where A = max{56, 11 + 0.4*(P-50) + 10*log10(B) } dB and A = Attenuation (in decibels) below the mean output power level, P = Percent removed from the center frequency of the transmitter bandwidth and between 50 and 250 percent, and B = Authorized bandwidth in MHz. Attenuation to an absolute power of less than -13dBm/1MHz is not required. |  |  |  |  |  |  |
| Note 9: Unless otherwise authorized upon specific request by the applicant, each station authorized must employ a directional antenna adjusted with the center of the major lobe of radiation in the horizontal plane directed toward the receiving station with which it communicates: provided, however, where a station communicates with more than one point, a multi- or omni-directional antenna may be authorized if necessary. Fixed stations must employ transmitting and receiving antennas (excluding second receiving antennas for operations such as space diversity) meeting the appropriate performance indicated in table below. Frequency (GHz); Max BW to 3 dB points (included angle in degrees); Min antenna gain (dBi); Min radiation suppression to angle in degrees from centerline of main beam in (dB). 5° to10°; 10° to 15°; 15° to 20°; 20° to 30°; 30° to 100°; 100° to 140°; 140° to 180°. 71 - 76 (co-polar); 1.2; 43; 35; 40; 45; 50; 50; 55; 55. 71 - 76 (cross-polar); 1.2; 43; 45; 50; 50; 55; 55; 55; 55. 81 – 86 (co-polar); 1.2; 43; 35; 40; 45; 50; 50; 55; 55. 81 – 86 (cross-polar); 1.2; 43; 45; 50; 50; 55; 55; 55; 55. 92 - 95; 0.6; 50.0; 36; 40; 45; 50; 55; 55; 55. Other than frequency 92 to 95 GHz, the antenna gain less than 50 dBi (but greater than or equal to 43 dBi) is permitted only with a proportional reduction in maximum authorized EIRP in a ratio of 2 dB of power per 1 dB of gain, so that the maximum allowable EIRP (in dBW) for antennas of less than 50 dBi gain becomes + 55−2(50-G), where G is the antenna gain in dBi. In addition, antennas in these bands must meet two additional standards for minimum radiation suppression: At angles between 1.2 and 5 degrees from the centerline of the main beam, co-polar discrimination must be G−28, where G is the antenna gain in dBi; and at angles of less than 5 degrees from the centerline of main beam, cross-polar discrimination must be at least 25 dB. |
| Note 10: Any transmitter that has received the necessary FCC equipment authorization may be mounted in a group installation for simultaneous operation with one or more other transmitter(s) that have received the necessary FCC equipment authorization, without any additional equipment authorization. However, no transmitter may be equipped with external phase-locking inputs that permit beam-forming arrays to be realized. |  |  |  |  |  |  |
| Note 11: First, a licensee applies for a non-exclusive nationwide license; second, the licensee registers individual point-to-point links with a database manager. Registration process verifies potential for harmful interference to or from all previously registered non-government links. |  |  |  |  |  |  |
| Note 12: On February 22nd, 2018, the FCC adopted their Spectrum Horizons Notice of Proposed Rulemaking (NPRM) that seeks comment on:bands above 95GHz; A mmW coalition [Nokia 6] has been active in this proceeding and has petitioned FCC to include these passive bands (100-102GHz and 109.5-111.8 GHz), and other passive bands above 114.25GHz for consideration for terrestrial fixed and mobile use. If aproved, then the whole 95-114.25GHz range could be available in USA. |  |  |  |  |  |  |

#### 4.2.2.2	Canada

Table 4.2.2.2-1 shows the spectrum allocations for Canada for frequency between 52.6GHz and 100GHz based on [13]. Primary services are listed in “capitals:”, e.g., MOBILE, and secondary services are listed in “normal characters;” e.g. Amateur.

Table 4.2.2.2-1: Canada spectrum allocation between 52.6GHz and 100GHz

| Frequency [GHz] | Allocations | Notes and ITU Radio Regulations Footnotes |
| --- | --- | --- |
| 52.6-54.25 | EARTH EXPLORATION-SATELLITE (passive) SPACE RESEARCH (passive) | 5.3.40, 5.556 |
| 54.25-55.78 | EARTH EXPLORATION-SATELLITE (passive) INTER-SATELLITE SPACE RESEARCH (passive) | 5.556A |
| 55.78-56.9 | EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE MOBILE SPACE RESEARCH (passive) | 5.557A, 5.556A, 5.5558, 5.547 |
| 56.9-57 | EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) | 5.558A, 5.558, 5.547 |
| 57-58.2 | EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE MOBILE SPACE RESEARCH (passive) | 5.556A, 5.558, 5.547 |
| 58.2-59 | EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) | 5.547, 5.556 |
| 59-59.3 | EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE INTER-SATELLITE RADIOLOCATION SPACE RESEARCH (passive) | 5.556A, 5.558, 5.559 |
| 59.3-64 | FIXED MOBILE INTER-SATELLITE RADIOLOCATION | 5.558, 5.559, 5.138 |
| 64-65 | FIXED INTER-SATELLITE MOBILE except aeronautical mobile | 5.547, 5.556 |
| 65-66 | EARTH EXPLORATION-SATELLITE FIXED INTER-SATELLITE MOBILE except aeronautical mobile SPACE RESEARCH | 5.547 |
| 66-71 | INTER-SATELLITE MOBILE MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION-SATELLITE | 5.553, 5.558, 5.554 |
| 71-74 | FIXED FIXED-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (space-to-Earth) |  |
| 74-76 | FIXED FIXED-SATELLITE (space-to-Earth) MOBILE BROADCASTING BROADCASTING-SATELLITE Space research (space-to-Earth) | 5.561 |
| 76-77.5 | RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite Space research (space-to-Earth) | 5.149 |
| 77.5-78 | AMATEUR AMEUR-SATELLITE RADIOLOCATION Radio astronomy Space research (space-to-Earth) | 5.149 |
| 78-79 | RADIOLOCATION Amateur Amateur-satellite Radio astronomy Space research (space-to-Earth) | 5.149, 5.560 |
| 79-81 | RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite Space research (space-to-Earth) | 5.149 |
| 81-84 | FIXED FIXED-SATELLITE (Earth-to-space) MOBILE MOBILE-SATELLITE (Earth-to-space) RADIO ASTRONOMY Space research (space-to-Earth) | 5.338A, 5.149, 5.561A |
| 84-86 | FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIO ASTRONOMY | 5.338A, 5.149 |
| 86-92 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) | 5.340 |
| 92-94 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION | 5.338A, 5.149 |
| 94-94.1 | EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) Radio astronomy | 5.562, 5.562A |
| 94.1-95 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION | 5.149 |
| 95-100 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION RADIONAVIGATION RADIONAVIGATION-SATELLITE | 5.149, 5.554 |

Canada has designed frequency bands 71-76GHz, 81-86GHz, and 92-95GHz for use by fixed service. Frequency bands 71-76GHz, 81-86GHz are subject to licensing and frequency band 92-95GHz will be subject to future revision of the Canada’s Standard Radio System Plan (SRSP). Figure 4.2.2.2-1 illustrates the band plan and associated usage for the bands 71-76 and 81-86GHz. The channel center frequencies are also described in Table 4.2.2.2-2 and 4.2.2.2-3. The frequency ranges 71.125 to 74.375 GHz and 81.125 to 84.375GHz provide for thirteen pairs of 250MHz channels (A1/A1’ to A13/A13’) for systems employing FDD. The frequency ranges 74.375GHz to 75.875GHz and 84.375GHz to 85.875GHz provide for 6 pairs of 250MHz channels (A14/A14’ to A19/A19’) for systems employing either FDD or TDD.

Figure 4.2.2.2-1: Canada’s 71-76GHz and 81-86GHz band plan and associated usage (from [11])

Table 4.2.2.2-2: 71-76GHz and 81-86GHz band plan and associated usage for systems using 250MHz wide channels for Canada

| Channel | Centre frequency (MHz) | Duplex |
| --- | --- | --- |
| A1/A1' | 71250/81250 | FDD |
| A2/A2' | 71500/81500 | FDD |
| A3/A3' | 71750/81750 | FDD |
| A4/A4' | 72000/82000 | FDD |
| A5/A5' | 72250/82250 | FDD |
| A6/A6' | 72500/82500 | FDD |
| A7/A7' | 72750/82750 | FDD |
| A8/A8' | 73000/83000 | FDD |
| A9/A9' | 73250/83250 | FDD |
| A10/A10' | 73500/83500 | FDD |
| A11/A11' | 73750/83750 | FDD |
| A12/A12' | 74000/84000 | FDD |
| A13/A13' | 74250/84250 | FDD |
| A14/A14' | 74500/84500 | FDD, TDD |
| A15/A15' | 74750/84750 | FDD, TDD |
| A16/A16' | 75000/85000 | FDD, TDD |
| A17/A17' | 75250/85250 | FDD, TDD |
| A18/A18' | 75500/85500 | FDD, TDD |
| A19/A19' | 75750/85750 | FDD, TDD |

Table 4.2.2.2-3: 71-76GHz and 81-86GHz band plan and associated usage for systems using aggregation of multiple 250MHz wide channels for Canada

| Channel | Bandwidth (MHz) | Centre frequency (MHz) | Duplex |
| --- | --- | --- | --- |
| B1/B1' | 500 | 71375 / 81375 | FDD |
| B2/B2' | 500 | 71875 / 81875 | FDD |
| B3/B3' | 500 | 72375 / 82375 | FDD |
| B4/B4' | 500 | 72875 / 82875 | FDD |
| B5/B5' | 500 | 73375 / 83375 | FDD |
| B6/B6' | 500 | 73875 / 83875 | FDD |
| B7/B7' | 500 | 74375 / 84375 | FDD |
| B8/B8' | 500 | 74875 / 84875 | FDD, TDD |
| B9/B9' | 500 | 75375 / 85375 | FDD, TDD |
| C1/C1' | 750 | 71500 / 81500 | FDD |
| C2/C2' | 750 | 72250 / 82250 | FDD |
| C3/C3' | 750 | 73000 / 83000 | FDD |
| C4/C4' | 750 | 73750 / 83750 | FDD |
| C5/C5' | 750 | 74500 / 84500 | FDD |
| C6/C6' | 750 | 75250 / 85250 | FDD, TDD |
| D1/D1' | 1000 | 71625 / 81625 | FDD |
| D2/D2' | 1000 | 72625 / 82625 | FDD |
| D3/D3' | 1000 | 73625 / 83625 | FDD |
| D4/D4' | 1000 | 74625 / 84625 | FDD |
| E1/E1' | 1250 | 71750 / 81750 | FDD |
| E2/E2' | 1250 | 73000 / 83000 | FDD |
| E3/E3' | 1250 | 74250 / 84250 | FDD |
| F1/F1' | 1500 | 71875 / 81875 | FDD |
| F2/F2' | 1500 | 73375 / 83375 | FDD |
| G1/G1' | 1750 | 72000 / 82000 | FDD |
| G2/G2' | 1750 | 73750 / 83750 | FDD |
| H1/H1' | 2000 | 72125 / 82125 | FDD |
| H2/H2' | 2000 | 74125 / 84125 | FDD |
| I1/I1' | 2250 | 72250 / 82250 | FDD |
| I2/I2' | 2250 | 74500 / 84500 | FDD |
| J1/J1' | 2500 | 72375 / 82375 | FDD |
| K1/K1' | 2750 | 72500 / 82500 | FDD |
| L1/L1' | 3000 | 72625 / 82625 | FDD |
| M1/M1' | 3250 | 72750 / 82750 | FDD |
| N1/N1' | 3500 | 72875 / 82875 | FDD |
| O1/O1' | 3750 | 73000 / 83000 | FDD |
| P1/P1' | 4000 | 73125 / 83125 | FDD |
| Q1/Q1' | 4250 | 73250 / 83250 | FDD |
| R1/R1' | 4500 | 73375 / 83375 | FDD |

Table 4.2.2.2-3 shows regulatory requirements for frequency ranges that are designed for mobile use between 52.6GHz and 100GHz in Canada.

Table 4.2.2.2-3: Canada regulatory requirements for frequency between 52.6GHz and 100GHz that are available for fixed and mobile use

| Frequency band [GHz] |  | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  | 57 – 64 | Max Peak conducted power: - If BW >100MHz, 500mW - If BW<100MHz; 500mW * (Emission BW/100MHz) In EIRP: Indoor: 40dBm avg/43dBm Peak Outdoor PtP: 82dBm when Gant>51dBi; 82-2*(51-Gant) when Gant≤51dBi | No requirements. | No specified requirements. | Indoor/outdoor | Unlicensed. |
|  | 64 – 71 | In June 2017, Canada proposed to extend the 60GHz band to this range. |  |  |  | Note 3 |
|  | 71 – 76 | Max power 5dBW. Max PSD -15dBW/MHz Minimum antenna gain 38 dBi Max EIRP 55dBm (Gant > 55dBi), Max EIRP 55 – (55-Gant) dBm (55dBi ≥ Gant > 45 dBi), and Max EIRP 45 – 2*(45-Gant) (45dBi ≥ Gant > 38 dBi), where Gant: antenna gain. [Note 1] [Note 2] | N/A |  | fixed point-to-point | Licensed |
|  | 81 – 86 | Max power 5dBW. Max PSD -15dBW/MHz Minimum antenna gain 38 dBi Max EIRP 55dBm (Gant > 55dBi), Max EIRP 55 – (55-Gant) dBm (55dBi ≥ Gant > 45 dBi), and Max EIRP 45 – 2*(45-Gant) (45dBi ≥ Gant > 38 dBi), where Gant: antenna gain. [Note 1] [Note 2] | N/A |  | fixed point-to-point | Licensed |
| Note 1: The following unwanted emission limits shall be applied. Maximum power spectral density (dBW/MHz); Frequency separation (% of channel bandwidth). −30; 50. −45; 57.5. −45; 70. −65; 125. −75; 250 or 150 + 500 MHz *. * The 250% value is applied for a channel bandwidth that is less than or equal to 500 MHz. For a bandwidth greater than 500 MHz, the 150%+500 MHz value is applied.. The power spectral density, delivered to the antenna input, outside the edges of the bands 71-76 GHz and 81-86 GHz shall be limited to a maximum of −55 dBW/MHz. Additionally, emissions into the band 86-92 GHz are furthermore limited. The power spectral density, delivered to the antenna input, shall be limited to a maximum of −41 dBW/100 MHz at 86 GHz and reducing proportionally to −55 dBW/100 MHz at and beyond 87 GHz |
| Note 2: Antenna pattern must satisfy Envelop A or B specified below. The co-polarized radiation pattern envelope in the horizontal plane of the antenna must remain within Envelope A for both vertical and horizontal polarizations. The deployment of a radio system using an RF channel bandwidth greater than 2000 MHz and an antenna radio pattern remaining within Envelope B is not permitted. The values for the radiation pattern envelopes are shown table below and specified as off-axis antenna gain in units of dBi, as opposed to dBs of attenuation from the main lobe gain. In order to convert the pattern values to attenuation from the main lobe, the antenna main beam gain in dBi can be subtracted from the off-axis dBi values of the applicable envelope. Azimuth (in degrees) from main lobe peak; Antenna directivity co-polarization (dBi). Envelope A; Envelope B. 5°; 16; 25. 10°; 9; 17.5. 15°; 5; 10. 20°; 1; 7. 40°; −0.33; 2. 50°; −1; 0.66. 70°; −4; −2. 88.75°; −16.18; −7. 90°; −17; −7. 100°; −17; −7. 100°; −17; −10. 180°; −17; −10. |
| Note: The June 2017 Consultation on Releasing Millimetre Wave Spectrum to Support 5G from Innovation, Science and Economic Development Canada (ISED) is accessible at http://www.ic.gc.ca/eic/site/smt-gst.nsf/eng/sf11298.html ISED proposal remains pending. |  |  |  |  |  |  |

#### 4.2.2.3	Brazil

Table 4.2.2.3-1 shows regulatory requirements for frequency ranges that are designed for mobile use between 52.6GHz and 100GHz in Brazil.

Table 4.2.2.3-1: Regulatory requirements for available spectrum between 52.6 GHz and 100 GHz in Brazil

| Frequency band (GHz) |  | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  | 52.6 – 57, 64 – 71, 86 – 100, | No regulation |  |  |  |  |
|  | 57 – 64 | Total Peak Power at transmitter output ≤ 27dBm (500mW) Max avg. Power Density: 9 microW/cm^2 @3m Max peak Power Density: 18 microW/cm^2 @3m | No channel occupancy time requirements No LBT requirements | No occupied channel bandwidth (OCB) requirements | Restricted Radiation Equipment (Low power devices) Not permitted for use in aircraft or satellites, and field disturbance sensors (including vehicular radar systems). | Anatel Resolution 680/2017 (act 14448) Spurious emissions: 40-200 GHz: ≤90 pW / cm^2 @3m Spurious emission levels shall not exceed the emission level at the fundamental radio frequency. |
|  | 71 – 76 | EIRP: a) below 85 dBm, for antennas with gain equal to or greater than 55 dBi; b) below 85 – (55 – G), for antennas with gain (G) equal to or greater than 45 dBi and less than 55 dBi; and, c) below 75 - 2 × (45 - G), for antennas with gain (G) of less than 45 dBi. | Must ensure that the power density in the “Itapetinga Radio Observatory”, located in Atibaia-SP, at 23º11'5.077'' South and 46º33'28.429'' West, do not exceed -228 dB (W/(m2Hz)). The main lobes of the antennas cannot be directed to the Radio station of Itapetinga when the distance between the antennas and the Radio station of Itapetinga is less than 60 km, and the compatibility at a lower distance must be proven by a study. | No occupied channel bandwidth (OCB) requirements | Point to Point Telecommunications Service | Anatel Resolution 642/2014 PSD for out of band emissions: PSD = -55 dBW / MHz, for frequencies below 71 GHz and between 76 GHz and 81 GHz; PSD = -41 dBW / 100 MHz at 86.05 GHz, decreasing linearly to -55 dBW / 100 MHz at 87 GHz, being flat from this frequency. |
|  | 76 – 77 | If the vehicle is not moving, the power density of any emission ≤ 200 nanoW / cm2 @ 3 m from the outer surface of the radiation structure. For field variation sensors installed in any part to be seen frontally in the vehicle, the power density of any emission when the vehicle is in motion ≤ 60 microW / cm2 @ 3 m from the outer surface of the radiation structure. For field variation sensors installed anywhere to be seen sideways or behind the vehicle, when the vehicle is in motion ≤ 30 microW / cm2 @ 3 m from the outer surface of the radiation structure. | No channel occupancy time requirements No LBT requirements | No occupied channel bandwidth (OCB) requirements | Restricted Radiation Equipment Variable electromagnetic field sensors installed in vehicle and used as vehicle radar systems Not permitted for use in aircraft or satellites. | Anatel Resolution 680/2017 (act 14448) Spurious emissions: ≤600 pW / cm^2 @3m, for field variation sensors installed anywhere to be viewed frontally on the vehicle ≤300 pW / cm^2 @3m, for field variation sensors installed anywhere to be viewed sideways or behind on the vehicle |
|  | 76 – 81 | The power at the transmitter output of a repeater station should be limited to 100 watts RMS. |  |  | Ham radio (Amateur radio) through specific authorization from Anatel arising from a substantiated request | Anatel Resolution 452/2006 |
|  | 81 – 86 | EIRP: a) below 85 dBm, for antennas with gain equal to or greater than 55 dBi; b) below 85 – (55 – G), for antennas with gain (G) equal to or greater than 45 dBi and less than 55 dBi; and, c) below 75 - 2 × (45 - G), for antennas with gain (G) of less than 45 dBi. | Must ensure that the power density in the “Itapetinga Radio Observatory”, located in Atibaia-SP, at 23º11'5.077'' South and 46º33'28.429'' West, do not exceed -228 dB (W/(m2Hz)). The main lobes of the antennas cannot be directed to the Radio station of Itapetinga when the distance between the antennas and the Radio station of Itapetinga is less than 60 km, and the compatibility at a lower distance must be proven by a study. | No occupied channel bandwidth (OCB) requirements | Point to Point Telecommunications Service | Anatel Resolution 642/2014 PSD for out of band emissions: PSD = -55 dBW / MHz, for frequencies below 71 GHz and between 76 GHz and 81 GHz; PSD = -41 dBW / 100 MHz at 86.05 GHz, decreasing linearly to -55 dBW / 100 MHz at 87 GHz, being flat from this frequency. |

#### 4.2.2.4	Mexico

Table 4.2.2.4-1 shows regulatory requirements for frequency ranges that are designed for mobile use between 52.6GHz and 100GHz in Mexico.

Table 4.2.2.4-1: Regulatory requirements for available spectrum between 52.6 GHz and 100 GHz in Mexico

| Frequency band (GHz) |  | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  | 57 – 64 | Min EIRP: 82 dBm Max EIRP: 85 dBm In cases where the antenna gain is less than 51 dBi, to the average EIRP and the maximum EIRP must be subtracted 2 dB for each dB that the gain is less than 51 dBi. |  |  | ITS systems that operate in the 63-64 GHz segment must operate with a maximum EIRP of 40 dBm and antenna gain ≤23 dBi. | For the 61-61.5 GHz segment, the fixed field disturbance sensors with bandwidth ≤500 MHz must operate with an average EIRP not exceeding 40 dBm and a maximum EIRP not exceeding 43 dBm For the 57-64 GHz band, except the 61-61.5 GHz segment, the fixed field disturbance sensors with bandwidth ≤500 MHz must operate with an average EIRP not exceeding 10 dBm and a maximum EIRP not exceeding 13 dBm. |
|  | 71 – 76, 81 – 86 | Max EIRP: 55 dBW with a max input power to the radiator of 3 W (5 dBW). Maximum limit of transmitted spectral density of 150 mW per 100 MHz. Directional antennas with a maximum of 1.2° opening angle of the main lobe of radiation at 3 dB, in the azimuthal and elevation planes. It is allowed the operation of antennas with gain less than 50 dBi and more than 38 dBi with a reduction of the maximum EIRP values at a ratio of 2 dB of power for every 1 dB of gain, so the max allowed EIRP will be calculated as: Maximum value of EIRP = 55 - 2 (50 - G) where, G is the gain of the antenna. |  |  |  | Out-of-band emissions: The attenuation of transmitters employing digital modulation techniques, in any MHz of the spectrum whose center frequency is separated from the carrier frequency allocated by more than 50 and up to 250% of the bandwidth used, shall be calculated according to the following equation and it cannot be less than 11 dB: A = 11 + 0.4 (P-50) + 10 Log10 B = 11 + 0.4 (P-50) + 27 Where: P.- Percentage of separation between the carrier frequency of the channel bandwidth used and the central frequency of the MHz under analysis. B = 500 MHz No attenuations greater than 56 dB or with absolute powers less than -13 dBm / MHz are necessary. |

### 4.2.3	ITU Region 3

#### 4.2.3.1	China

In China, MIIT (Ministry of Industry and Information Technology of the People's Republic China) publishes that short distant unlicensed applications can be applied between 59-64 GHz. While, 76-77GHz is considered for Vehicle Radar ranging application [15][16]. Details are summarized in Table 4.2.3.1-1.

Table 4.2.3.1-1: Regulatory requirements for available spectrum between 52.6 GHz and 100 GHz in China

| Frequency band (GHz) | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Notes |
| --- | --- | --- | --- | --- | --- |
| 59-64 | Output power of antenna port: ≤ 10 dBm Maximum EIRP：47dBm Mean EIRP: ≤ 44 dBm | No requirement | Not specified | very low transmission power and short distant unlicensed applications | The unlicensed devices operating in 59-64 GHz shall not interfere to the space research, earth resources exploration, radio astronomy and other legal wireless applications, and shall not demand radio interference protection. Carrier frequency tolerance: 500 x 10-6 Out of Band Emission (EIRP): -5dBm/MHz Spurious Emission (corresponding to frequency range outside 2.5*carrier bandwidth ): -20dBm/1MHz |
| 76-77 | Maximum EIRP：55dBm | No requirement | Not specified | Vehicle Radar ranging application | Spurious Emission (corresponding to frequency range outside 2.5*carrier bandwidth ): -20dBm/1MHz Spurious Emission in Idle/stand-by state (corresponding to frequency range outside 2.5*carrier bandwidth ): -47dBm/1MHz |

#### 4.2.3.2	Japan

Figure 4.2.3.2-1: Major spectrum usage in the frequency band above 52.6GHz in Japan

(source: http://www.tele.soumu.go.jp/e/adm/freq/search/myuse/0002/index.htm)

Figure 4.2.3.2-1 shows frequency usage for frequency above 52.6GHz in Japan. Table 1 below summarize the survey on Japanese regulatory requirements in the frequency band above 52.6GHz.

Table 2. Major spectrum usage and regulatory requirements between 52.6 GHz and 100 GHz in Japan*

| Frequency band (GHz) |  | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  | 52.6 – 54.25 |  |  |  | For Earth Exploration Satellite Service (passive), Space Research Service (passive) |  |
|  | 54.25 – 55.78 | Power below 1W |  | See Note 1 | For Broadcast Auxiliary Services (Mobile) | ARIB STD-B43 [17] See Note 2 for additional regulatory requirements. |
|  | 55.78 – 59 |  |  |  | For Commercial Telecommunications, Public and General Services (Fixed/Mobile) |  |
|  | 57 – 66 | Power below 250mW | See Note 3 | Occupied BW shall be below 9GHz | For Unlicensed low power data Communication systems | ARIB STD-T69 [18], ARIB STD-T117 [19] See Note 4 for additional regulatory requirements. |
|  | 60 – 61, 76 – 77 | Power below 10mW |  | See Note 5 | For Vehicle Radars (Unlicensed Low-Power) | ARIB STD-T48 [20] See Note 6 for additional regulatory requirements. |
|  | 77 – 81 | Power below 10mW or PSD below 5μW/MHz in case of occupied BW below 2GHz |  | Occupied BW shall be below 4GHz | For Vehicle Radars (Unlicensed Low-Power) | ARIB STD-T111 [21] See Note 7 for additional regulatory requirements. |
|  | 71 – 76, 81 – 86 | Peak power below 1W |  | Occupied BW shall be below 5GHz | For Commercial Telecommunications, Public and General Services (Fixed/Mobile) | See Note 8 for additional regulatory requirements. |
|  | 77.5 – 78 |  |  |  | For Amateur |  |
|  | 85.5 – 92 |  |  |  | For Radio Astronomy |  |
| Note 1: For system with 125MHz BW, PSK, QAM and OFDM are available. For system with 500MHz BW, PSK is available. For system with 1GHz BW, PSK and QAM are available. |  |  |  |  |  |  |
| Note 2: The unwanted emission shall satisfy below 50μW. |  |  |  |  |  |  |
| Note 3: The carrier sensing functionality is required for transmission power above 10dBm. |  |  |  |  |  |  |
| Note 4: The antenna absolute gain shall be less than 47dBi. In case of transmission power above 10mW, directivity with above 10dBi antenna absolute gain for main beam direction is necessary and EIRP shall be less than 40dBm. In addition, the unwanted emission shall satisfy below Table. Frequency; Unwanted emission requirement. Below 55.62GHz; Below -30dBm/MHz. Between 55.62GHz and 57GHz, Between 66GHz and 67.5GHz; Below -26dBm/MHz. Above 67.5GHz; Below -30dBm/MHz. |
| Note 5: The allowed value for occupied channel bandwidth (which is defined as bandwidth containing 99.5% of transmission power) is shown in below Table. Frequency; Occupied channel bandwidth. Between 60GHz and 61GHz; Below 500MHz. Between 76GHz and 77GHz; Below 1GHz. |
| Note 6: The antenna gain shall be less than 40dB. In addition, the unwanted emission shall satisfy below Table. Out-of-band domain emission limit; Spurious domain emission limit. Below 100μW; Below 50μW. |
| Note 7: The antenna absolute gain shall be less than 35dBi. In addition, the unwanted emission shall satisfy Table. Out-of-band domain emission limit; Spurious domain emission limit. Peak power below 100μW/MHz; Peak power below 50μW/MHz. |
| Note 8: The antenna absolute gain shall be less than 55dBi. In addition, the unwanted emission shall satisfy Table. Out-of-band domain emission limit; Spurious domain emission limit. Peak power below 100μW/MHz; Peak power below 50μW/MHz. |

Note *: Other radio stations may be used in the respective frequency bands.

#### 4.2.3.3	Korea

Table 5 shows summary of the spectrum allocations for South Korea for frequency between 52.6GHz and 100GHz, based on [22]. Primary services are listed in “capitals:”, e.g., MOBILE, and secondary services are listed in “normal characters;” e.g. Amateur.

Table 4.2.3.3-1: South Korea spectrum allocation between 52.6 GHz and 100 GHz

| Frequency [GHz] | Allocations | Notes |
| --- | --- | --- |
| 52.6-54.25 | EARTH EXPLORATION-SATELLITE (passive) SPACE RESEARCH (passive) |  |
| 54.25-55.78 | EARTH EXPLORATION-SATELLITE (passive) INTER-SATELLITE SPACE RESEARCH (passive) |  |
| 55.78-58.2 | EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE MOBILE SPACE RESEARCH (passive) |  |
| 58.2-59 | EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) |  |
| 59-59.3 | EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE INTER-SATELLITE RADIOLOCATION SPACE RESEARCH (passive) |  |
| 59.3-64 | FIXED MOBILE INTER-SATELLITE RADIOLOCATION |  |
| 64-65 | FIXED INTER-SATELLITE MOBILE except aeronautical mobile |  |
| 65-66 | EARTH EXPLORATION-SATELLITE FIXED INTER-SATELLITE MOBILE except aeronautical mobile SPACE RESEARCH |  |
| 66-71 | INTER-SATELLITE MOBILE MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION-SATELLITE |  |
| 71-74 | FIXED FIXED-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (space-to-Earth) |  |
| 74-76 | FIXED FIXED-SATELLITE (space-to-Earth) MOBILE BROADCASTING BROADCASTING-SATELLITE Space research (space-to-Earth) |  |
| 76-77.5 | RADIO ASTRONOMY RADIOLOCATION Space research (space-to-Earth) |  |
| 77.5-78 | AMATEUR AMATEUR-SATELLITE RADIOLOCATION Radio astronomy Space research (space-to-Earth) |  |
| 78-79 | RADIOLOCATION Amateur Amateur-satellite Radio astronomy Space research (space-to-Earth) |  |
| 79-81 | RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite Space research (space-to-Earth) |  |
| 81-84 | FIXED FIXED-SATELLITE (Earth-to-space) MOBILE MOBILE-SATELLITE (Earth-to-space) RADIO ASTRONOMY Space research (space-to-Earth) |  |
| 84-86 | FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIO ASTRONOMY |  |
| 86-92 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) |  |
| 92-94 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION |  |
| 94-94.1 | EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) Radio astronomy |  |
| 94.1-95 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION |  |
| 95-100 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION RADIONAVIGATION RADIONAVIGATION-SATELLITE |  |

Table 4.2.3.3-2 shows regulatory requirements for frequency ranges that are designed for mobile use between 52.6GHz and 100GHz in South Korea based on [23], [24], [25], [29], [30], and [31].

Table 4.2.3.3-2: South Korea regulatory requirements for frequency between 52.6 GHz and 100 GHz

| Frequency band [GHz] |  | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Additional Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  | 52.6 – 54.25 | All emission is prohibited | No requirement | No requirement | No requirement | All emission is prohibited |
|  | 57 – 66 | Max Tx power 500mW (beamformed antenna) Max Tx power 100mW (omni-direction antenna). Max PSD 13dBm/MHz Max EIRP 43dBm Max antenna gain 16dBi [Note 1] | No requirement. Note 3 | Occupied BW shall be within the range of 57-66 GHz | Other than fixed point-to-point | Unlicensed |
|  | 57 – 66 | Max Tx power 500mW (beamformed antenna) Max Tx power 100mW (omni-direction antenna). Max PSD 13dBm/MHz Max EIRP 57dBm [Note 1] | No requirement. Note 3 | Occupied BW shall be within the range of 57-66 GHz | Fixed point-to-point | Unlicensed |
|  | 71 – 76 | Max EIRP 85dBm Max Tx power 3W Max PSD 150mW/100MHz Min antenna gain 43dBi, antenna half beamwidth less than 1.2 degrees. [Note 2] | Permits use under condition that do not cause interference to Astronomy stations that are already in use. | Max bandwidth 5GHz. Occupied BW shall be within the range of 71-76 GHz | For licensed fixed point-to-point wireless communications For amateur radio station (see Note 4 for channelization) | Frequency error should be less than ±150×10-6. Note 4. |
|  | 76 – 77 | Max Tx power 10mW EIRP below 55dBm [Note 5] | Function for interference avoidance shall be required. (No Specific technical requirement is described) | Occupied BW shall be within the range of 76-77 GHz | Automotive crash prevention radar | Unlicensed |
|  | 76 – 81 | PSD below -3 dBm/MHz EIRP below 34dBm/50MHz Fixed antenna orientation with down tilt angle. The half power beamwidth of the antenna array must be smaller than 8 degrees Unwanted emission outside the 76-81GHz shall be at least attenuated 20dB more than the PSD | No requirement | Occupied BW shall be within the range of 76-81 GHz | For outdoor level detection radars. For amateur radio station (see Note 4 for channelization) |  |
|  | 81 – 86 | Max EIRP 55dBW Max Tx power 3W Max PSD 150mW/100MHz Min antenna gain 43dBi, antenna half beamwidth less than 1.2 degrees. [Note 2] | Permits use under condition that do not cause interference to Astronomy stations that are already in use. | Max bandwidth 5GHz. Occupied BW shall be within the range of 81-86 GHz | For licensed fixed point-to-point wireless communications | Frequency error should be less than ±150×10-6. |
|  | 86 – 92 | No requirement | No requirement | No requirement | No requirement | All emissions are prohibited |
| Note 1: Antenna absolute gain shall be less than 16dBi. If antenna gain greater than 16dBi may be employed than conducted output power shall be reduced as much as the increased antenna gain. A user identification code shall be used in each device to prevent malfunction of other devices and to prevent malfunction caused by signals from other devices. However, it does not apply to fixed point-to-point communication. Unwanted emission shall not exceed emission level given in followin Table. Spectrum Range; Unwanted Emission Limit; Measurement Bandwidth. Less than 1GHz; -36dBm; 100kHz. 1GHz to 40GHz; -30dBm; 1MHz. More than 40GHz; -10dBm; 1MHz. Additionally, Tx or Rx spurious emission for Standby state (not operating) shall not exceed emission level given in following Table. Spectrum Range; Unwanted Emission Limit; Measurement Bandwidth. Less than 1GHz; -54dBm; 100kHz. More than 1GHz; -47dBm; 1MHz. |
| Note 2: The mean power of emissions must be attenuated below the mean output power of the transmitter in any 1 MHz band by A, where A = max{56, 11 + 0.4*(S-50) + 10*log10(B) } dBc and A = attenuation (in decibels) below the mean output power level, S = percent removed from the center frequency of the transmitter bandwidth and between 50 and 250 percent, and B = occupied bandwidth in MHz. Attenuation to an absolute power of less than -13dBm/1MHz is not required. Spurious emission must be below the mean output of the transmission in any 1 MHz band by A = 43 + 10*log10(P), where A = attenuation (in decibels) below the mean output power level, and P = average transmit power. |  |  |  |  |  |  |
| Note 3: For equipment with more than 27dBm EIRP operating in 57-58GHz, the following instruction shall be indicated in user the instruction manual. “User who would like to use this equipment within 300 m from Astronomical antenna have to obtain consent from astronomical observatory” |  |  |  |  |  |  |
| Note 4: From 75.5 to 81GHz, 75.75, 77.75, 78.5, and 80 GHz are assigned for the carrier frequencies of amateur radio station. |  |  |  |  |  |  |
| Note 5: The unwanted emission outside the 76-77GHz shall not exceed emission level given in following table. Spectrum Range; Unwanted Emission Limit; Measurement Bandwidth. Less than 1GHz; -26dBm * or 0dBm **; 100kHz. More than 1GHz; -26dBm * or 0dBm **; 1MHz. * In case that Conducted power level shall be less than 10mW ** In case that EIRP shall be less than 55dBm. |

#### 4.2.3.4	India

The Government of India Ministry of Communications’ Telecommunications Department has recently released a national frequency allocation plan [26]. Table 4.2.3.4-1 is populated per this plan. For frequencies above 52.6 GHz, the power requirements, spectrum access, modulation and occupied bandwidth remain undefined. The NFAP is released typically every 3 to 4 years and the next one is not expected until after 2020.

Table 4.2.3.4-1: India – Regulatory Requirements for Available and Potential Spectrum between 52.6 GHz and 114.25 GHz

| Frequency band (GHz) |  | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  | 56.9 – 59.3 | Not defined | Not defined | Not defined | -ESS (Passive) -Space Research (Passive) - Fixed - Mobile |  |
|  | 59.3 – 71 | Not defined | Not defined | Not defined | -Fixed -Inter-satellite -Mobile -Radiolocation -Space Research |  |
|  | 71 – 78 | Not defined | Not defined | Not defined | -fixed satellite(space-to-earth) -Fixed -Broadcasting -Space Research -Radio astronomy |  |
|  | 78 – 86 | Not defined | Not defined | Not defined | -Radiolocation -Amateur Satellite -Radio astronomy -Space Research (Space-to-Earth) -Fixed -Mobile |  |
|  | 86-95 | Not defined | Not defined | Not defined | -Earth Exploration Satellite -Radio astronomy -Space Research |  |
|  | 95-111.8 | Not defined | Not defined | Not defined | -Fixed -Mobile -Radio Atronomy -Radiolocation -Radionavigation -Radionavigation-satellite -Space research |  |
|  | 111.8-114.25 | Not defined | Not defined | Not defined | -Fixed -Mobile -Radio astronomy -Space Research Inter satellite |  |

#### 4.2.3.5	Taiwan

#### 4.2.3.6	Singapore

In Singapore, IMDA (Info-communications Media Development Authority) shows that 57-66 GHz is considered for Wireless LAN and broadband access application [27]. On the other hand, short range radar systems, such as automatic cruise control and collision warning systems for vehicle, are applied from76 to 77GHz. Moreover, details of their regulatory requirements can be found in Annex. Furthermore, it is notable that both spectrum band, 71 – 76 GHz paired with 81 – 86 GH, will be available for short-term commercial use in fixed-wireless backhaul and inter-building fixed-wireless network.

Table 4.2.3.6-1: Regulatory requirements for available spectrum between 52.6 GHz and 100 GHz in Singapore

| Frequency band (GHz) | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Notes |
| --- | --- | --- | --- | --- | --- |
| 76 – 77 Note1 | Maximum EIRP： 23.5 dBm when stationary Maximum EIRP： 37 dBm when vehicle is in motion | No requirement | Not specified | Short range radar systems such as automatic cruise control and collision warning systems for vehicle | A short range radar system is a movement and position detection device which is used to give a warning of collision by identifying the delay between a transmitted pulse and a return pulse. Standards for Measurement Methods FCC Part 95 Subpart M; or EN 301 091-1 Transmitter Spurious Emissions limit: -30dBm for f≥1GHz, refer to ITU-R Rec. SM.329-12 §4.3, Table 3 Category B limits |
| 57 – 66 Note1 | Maximum EIRP： 10 W Indoor use is restricted to maximum mean EIRP density of 13 dBm/MHz Outdoor use is restricted to max e.i.r.p. of 25 dBm and max e.i.r.p. power spectral density of -2 dBm/MHz | No requirement | Not specified | Wireless LAN and broadband access | Standards for Measurement Methods EN 302 567 EN 305 550 Transmitter Spurious Emissions limit: -30dBm for f≥1GHz, refer to ITU-R Rec. SM.329-12 §4.3, Table 3 Category B limits |
| 71-76, 81-86 |  |  |  | fixed wireless system | See Note 2 for specific information |
| Note1: The development of spectrum allocations for short range devices is an ongoing policy review process. IMDA will revise and update its frequency allocations for short range devices as new technologies emerge. |  |  |  |  |  |
| Note2: IMDA will make available 71 – 76 GHz paired with 81 – 86 GHz of the E-band for commercial wireless applications such as fixed-wireless backhaul and inter-building fixed-wireless network. The spectrum will be available for short-term commercial use of up to 12 months until 31 December 2019. IMDA will monitor market and technology developments of the E-band, and take into consideration the World Radiocommunication Conference 2019 recommendations before allocating the spectrum in this band beyond 2020. |  |  |  |  |  |

#### 4.2.3.7	Australia

ACMA (Australian Communications and Media Authority) publishes sensors using radar which can be applied in some different bands from 52.6GHz to 85GHz in Australia [28]. Moreover, details of their regulatory requirements can be found in Table 4.2.3.7-1.

Table 4.2.3.7-1: Regulatory requirements for available spectrum between 52.6 GHz and 100 GHz in Australia

| Frequency band (GHz) | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Notes |
| --- | --- | --- | --- | --- | --- |
| 61-61.5 | Maximum EIRP：100 mW | No requirement | Not specified | Transmitters for non-specific applications |  |
| 59–63 | Maximum EIRP：150 W The maximum transmitter power must not exceed 20 mW. | No requirement | Not specified | WiFi and RLAN transmitters | The transmitter must not be operated on board an aircraft. The transmitter must only be used outdoors. The transmitter must not cause spurious emissions outside the band at or greater than –30 dBm/MHz. |
| 57–66 | Maximum EIRP：20 W | No requirement | Not specified | WiFi and RLAN transmitters | The transmitter must comply with FCC Rules Title 47 Part 15 Section 255. |
| 60-61 | Maximum EIRP：20 mW | No requirement | Not specified | Radio determination: sensors using radar for measurement |  |
| 75-85 | Maximum EIRP：75 nW | No requirement | Not specified | Same as 60-61GHz | The maximum EIRP applies outside the shielded room enclosure. The transmitter must meet the requirements of ETSI Standard EN 302 372. |
| 76-77 | Maximum EIRP：25 W | No requirement | Not specified | Same as 60-61GHz |  |
| 77-81 | See Note1 below | No requirement | Not specified | Same as 60-61GHz | The transmitter must meet the requirements of ETSI Standard EN 302 264. The transmitter must not be operated within a nominated distance of a specified Australian radio-astronomy site. |
| 57-64, 75-85 | See Note2 below | See Note3 below | Not specified | Same as 60-61GHz | The transmitter must be operated in a position such that emissions are directed towards: (i) the ground; or (ii) the floor or a wall of a building or similar structure. The transmitter must comply with ETSI Standard EN 302 729. The transmitter must not be operated within a nominated distance of a specified Australian radio-astronomy site. |
| Note1: Section 4.3.3.3 in ETSI EN 302 264 V2.1.1 The peak power for EUT with fixed beam or scanning antenna shall not be greater than 55 dBm. |  |  |  |  |  |
| Note2: Section 4.3.4.3 in ETSI EN 302 729 V2.1.1 57-64 GHz: Maximum peak power limit EIRP is 35dBm, measured in 50MHz within main beam. 75-85 GHz Maximum peak power limit EIRP is 34dBm, measured in 50MHz within main beam. |  |  |  |  |  |
| Note3: Section 4.7 in ETSI EN 302 729 V2.1.1 |  |  |  |  |  |

=================== End of Text Proposal ========================

# Reference

- RP-182317, “Survey on regulatory requirements beyond 52.6 GHz in Korea,” Samsung

- RP-182328, “NR beyond 52.6GHz - Spectrum Availability and Regulatory Requirements,” Nokia, Nokia Shanghai Bell

- RP-182402, “TP for TR 38.807 with regulatory survey for bands beyond 52.6GHz in Canada, Mexico and Brazil,” QUALCOMM Europe Inc. - Spain

- RP-182417, “Survey on Japanese spectrum usage and regulatory requirements in the frequency band above 52.6GHz,” NTT DOCOMO, INC.

- RP-182434, “Considerations for NR use cases and scenarios in frequencies above 52.6 GHz,” Huawei, HiSilicon

- RP-182435, “Summary of ITU radio regulations for the range 52.6-100 GHz,” Huawei, HiSilicon

- RP-182476, “Survey on spectrum availability and regulatory requirements in ITU Region 3,” ZTE Corporation,Sanechips

- RP-182485, “Spectrum availability and regulatory requirement survey for spectrum above 52.6GHz,” Intel Corporation (UK) Ltd

- RP-182629, “Survey on spectrum availability and regulatory requirements for Region CEPT/EC for frequencies above 52.6 GHz,” Ericsson España S.A.