3GPP TSG-RAN meeting #82	RP-182629

Sorrentino, Italy, December 10th – 13th, 2018

Agenda Item:	9.3.17

Source:	Ericsson

Title:	Survey on spectrum availability and regulatory requirements for Region
CEPT/EC for frequencies above 52.6 GHz

Document for:	Discussion, Information

# 1	Introduction

At RAN plenary meeting #80 the SI to study NR waveform beyond 52.6 GHz has been approved [1]. One of the objectives defined in [1] is to “Identify target spectrum ranges”. According to the work plan this objective should be discussed in RAN plenary meetings #82 to #84.

In this contribution we provide an overview over relevant documents and recommendations for region CEPT/EC (Europe). We plan to follow up this contribution with more technical details extracted from the identified documents and recommendations in the upcoming RAN plenary meetings.

# 2	Discussion

The study item “Study on NR beyond 52.6GHz” currently lists frequencies 52.6 GHz and 100 GHz as the frequency end points for this study. One of the objectives of the Study on NR beyond 52.6 GHz is to identify target spectrum ranges and perform a regulatory survey.

Table 1 identifies bands within the frequency range 52.6 to 100 GHz and provides short statements and references where to find detail information w.r.t. status of the band.

General information for usages in CEPT can be found in THE EUROPEAN TABLE OF FREQUENCY ALLOCATIONS AND APPLICATIONS IN THE FREQUENCY RANGE 8.3 kHz to 3000 GHz (ECA TABLE). When it comes to Regulation about Short Range Devices additional information can be found in Recommendation ERC/REC 70-03 “Relating to the use of Short Range Devices (SRD)”. 
Unwanted emission limits can be found in ECC Recommendation (02)/05 “Unwanted Emissions” and ERC/RECOMMENDATION 74-01 “UNWANTED EMISSIONS IN THE SPURIOUS DOMAIN”. It should however be pointed out that the Rec 74-01 is under review with estimated finalization date 2019-05-24.

The bands above 52.6 GHz that are discussed due to WRC-19 AI 1.13 (IMT), within CEPT are three, 66-71 GHz as IMT candidate band and the bands 71-76/81-86 GHz as no IMT bands. The band 71-76 GHz, paired with 81-86 GHz, is a Fixed link band important for backhauling of 5G. Therefor Fixed link usage is expected to increase in the future

From a European perspective, the focus should be on bands that are proposed for IMT as well as on the unlicensed band 57-66 GHz. The current status, seen by CEPT, for IMT bands can be found in the draft ECP, European Common Proposal and Brief for WRC-19 Agenda Item 1.13. There are no limitations concerning output powers or channel bandwidth in the 66-71 GHz band, only that the band 66-71 GHz is planned for “Future Civil systems”.

Table 1: Frequency bands in the range 52.6 to 100 GHz

| Frequency range in GHz | Comments | Region |  |
| --- | --- | --- | --- |
| 52.6 to 54.25 | EESS service, all emissions are prohibited, (ref ITU-R Radio Regulation footnote 5.340 | ITU/CEPT |  |
| 55.78 to 57 | Recommended to be used by fixed services (ref: ERC Recommendation 12-12 and ITU-R Radio Regulation footnote 5.547) | ITU/CEPT |  |
| 57 to 66 | Several usage of Short range devices (ref ERC/REC 70-03) and EC Decision about SRD (ref COMMISSION IMPLEMENTING DECISION (EU) 2017/1483 of 8 August 2017) 57-64 GHz recommended by CEPT for fixed service limited to point to point (ref: ECC recommendation (09)01) 64-66 GHz recommended by CEPT for fixed service limited to point to point (ref: ECC recommendation (05)02) 63-64 GHz ITS band in Europe (ref: ECC decision (09)01) | EC/CEPT |  |
| 66 to 71 | Planned for “Future civil systems” There is currently no limitations on channel bandwidths, output powers etc. Recommended by CEPT for IMT identification (ref: draft new resolution [A113-IMT 66/71 GHz, Output from ECC PT1 meeting in Sep 2018]). GSA considers this band as an important band for 5G | CEPT |  |
| 71 to 76 and 81 to 86 | Recommended by CEPT for fixed services (ref: ECC recommendation (05)07) | CEPT |  |
| 77 to 81 | Automotive RADAR in Europe (ref: ECC decision (04)03 and ref Commission Decision of 8 July 2004 on the harmonisation of radio spectrum in the 79 GHz range for the use of automotive short-range radar equipment in the Community | EC/CEPT |  |
| 86 to 92 | EESS service, all emissions are prohibited (ref ITU-R Radio Regulation footnote 5.340) | ITU/CEPT |  |
| 92 to 100 | Recommended by CEPT for fixed services (ref: ECC recommendation (18)02) | CEPT |  |

# Conclusion

In this contribution we provide links to relevant documents and recommendations for region EC/CEPT w.r.t. bands for NR beyond 52.6 GHz. Follow up contributions with more details extracted from the cited documents and recommendations are planned for the upcoming RAN plenary meetings.

# References

RP-181435, New SID: Study on NR beyond 52.6GHz, Intel, RAN Meeting #80, June 2018