3GPP TSG-RAN meeting #82	RP-182808

Sorrento, Italy, December 10th – 13th, 2018

Agenda Item:	9.3.6

Source:	FirstNet, AT&T, The UK Home Office, Telstra

Title:	Mission Critical Services and NR V2X

Document for:	Discussion, Decision

# 1	Introduction

The NR V2X Study item is scheduled to conclude at RAN#83 (March 2019) and be followed by a work item as part of NR Release 16. As a part of this work, an NR sidelink for V2X services was introduced. In this paper we ask for the support of ProSe public safety (PS) services (MCPTT, MCVideo, and MCData) as another use case for the NR sidelink V2X.

# 2	Discussion

ProSe D2D functionality was added to the 3GPP system in Release 12. The driving use case behind this was the need from public safety responder communities to replace traditional narrow-band mobile radio communication systems and add richer communication with video streaming, data services etc.

A set of summarized public safety priorities listed in Annex A that are derived from a contribution submitted to RAN Plenary via RP-131178 and further support requirements are retrieved from section 7A of TS 22.278.

To support these requirements a new form of communication link was introduced in Release 12, called the sidelink. In Releases 13 and 14, this service was extended to commercial use cases and V2X. V2X services include communication between vehicles and communication between vehicles and road side units which can provide an advanced set of services targeted to enhance safety and efficiency of road transportation.

In Release 16, the introduction of sidelink is being investigated in the NR V2X study item. The services studied so far in the V2X work include both periodic services (e.g. vehicular co-operative awareness messaging) and aperiodic services (e.g. sensor and data sharing). We note that there is no explicit mention of the ProSe public safety services in the V2X SI, so support for it cannot be expected in Rel-16 as per the current plan. However, we expect the amount of work required to support ProSe PS services in Rel-16 to be small compared to the work planned in NR V2X and also believe that some of the public safety functionalities can leverage advancements made in related areas during V2X SI study period thus far.

In addition, it is also to be noted that SA6 WG has initiated a study on the support of Mission Critical Services over 5GS (FS_MCOver5GS) which is expected to be completed in TSG#85 (09/2019).

# 4	Conclusion and proposal

Based on the above discussion we propose that the support of ProSe public safety services (MCPTT, MCVideo, and MCData) be included in the scope of the upcoming NR V2X work item.

Annex A:

The following set of summarized public safety priorities are derived from a contribution submitted to RAN Plenary via RP-131178 and further support requirements are retrieved from section 7A of TS 22.278.

- Direct 1:many communication out of coverage

- Direct 1:many Communication in coverage

- Direct 1:1 Communication out of coverage

- UE to NW Relay

- ProSe Communication Path via eNB

- ProSe Direct Discovery - Both in and out of coverage aspects need to be considered.

- Restricted ProSe Discovery - Without this feature component, only Open ProSe Discovery would be supported.

- Floor Control - functionality that moderates requests from users to transmit within a group communication. Includes also aspects of session control and group communication – such as notification when users leave an active group communication.

- Network usage and battery consumption should be minimized.