3GPP TSG RAN Meeting #82	RP-182854

Sorrento, Italy, Dec. 10-13, 2018

# Source:	ZTE Corporation, Sanechips

# Title:	New work item: Two step RACH for NR

# Document for:	Approval

Agenda Item:	9.1.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 	New work item: Two step RACH for NR

Acronym:

Unique identifier: 	{A number to be provided by MCC at the plenary}

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part |  |
| --- | --- |
| This WID includes a Performance part |  |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access | X |
|  | Core Network |  |
|  | Services |  |

1	Impacts 	{ For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study.}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  | X | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a … {Tick one box. "Feature / Building Block / Work Task" form a hierarchical structure. E.g. no Building Block can be proposed without a corresponding parent Feature. The full structure of all existing Work Items is shown in the 3GPP Work Plan in ftp://ftp.3gpp.org/Information/WORK_PLAN }

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, we define them as work tasks. If you are in doubt, please contact MCC.

2.2	Parent Work Item

{Not applicable for Feature nor for a Study Item}

{For a Building Block: list here the parent Feature }

{For a Work Task: list here the parent Building Block }

| Parent Work Items |  |
| --- | --- |
| Unique ID | Title |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one WID. Therefore the table above should just include the feature WI Unique ID and title.

2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as preceding SI or a preceding WI (e.g. if further enhancing a feature).}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  | New WID on NR-based Access to Unlicensed Spectrum | Work in this WID on 4 step RACH could be applied for 2-step RACH in this WI |

NOTE:	Also related or dependent WIs/SIs in other TSGs should be indicated.

3	Justification

MSG A comprising preamble and data was studied and evaluated for 2-step RACH. General higher layer aspects of 2-step RACH were also studied. In addition to the general enhancements for connection setup/resume procedure, it was also concluded that a 2-step RACH procedure potentially has benefits for channel access. RAN2 also agreed that triggers for 4-step RACH are also applicable to 2-step RACH and hence 2-step RACH could be applied for all RRC states.

Further, the way forward in RAN#81 [RP-182126] says “A common 2-step RACH design for various use cases is desirable” and “2-step RACH can be included in a later Rel-16 WI, per normal approval process”. This work item proposes a common design for 3 main scenarios (mMTC, URLLC and eMBB) and for both licensed and unlicensed spectrum. Additional enhancements specific for unlicensed spectrum are needed and these should hence be specified after completion of 4-step RACH in NR-U WI.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The work item aims to specify 2-step RACH as general MAC procedure covering both physical layer and high layer aspects. Only Contention based RACH procedures are specified for 2-step RACH. All the objects are applied for both licensed and unlicensed spectrum except for last bullet where additional work specific to NR-U will be done after completion of 4-step RACH in NR-U WI.

- Two-step RACH [RAN1, RAN2]

- 2 step RACH shall be able operate regardless of whether the UE has valid TA or not.

- 2-step RACH is applicable to any cell size supported in Rel-15 NR;

- 2 step RACH is applied for RRC_INACTIVE , RRC_CONNECTED and RRC_IDLE state

- Support contention-based RACH procedure (RAN2)

- Channel structure of msgA is Preamble and PUSCH carrying payload (RAN1)

  - Rel-15 NR PRACH Preambles design will be reused.

    - PUSCH carrying Payload without preamble is TBD.

    - Rel-15 NR reference signal as preamble other than Rel-15 NR preamble is TBD.

  - Reuse the Rel-15 NR PUSCH including Rel-15 DMRS for transmission of payload of MSG A)

    - No new CP length and no sub-PRB guard subcarrier(s)

Note 1: The above sub-bullet is there to ensure that signal structure optimizations for any specific cell size (e.g. cells with RTT larger than Rel-15 CP duration) are not pursued.

  - Specify the mapping between the PRACH preamble and the time-frequency resource of PUSCH in MSG A+ DMRS

    - PRACH Preamble and PUSCH in a MsgA is TDMed

  - Specify the supported MCS(s) and time-frequency resource size(s) of PUSCH in MsgA

    - Consider the MsgA payload contents determined by RAN2

  - Specify power control of PUSCH of Msg A

- Specify Msg A’s content: to include the equivalent contents of msg3 of 4-step RACH (RAN2/RAN1)

  - Inclusion of UCI in MsgA is not precluded

- Specify Msg B’s content: to include the equivalent contents of msg2 and msg4 of 4-step RACH (RAN1/RAN2)

- Contention resolution for 2-step RACH (RAN2)

- Design of RNTI for Msg B of 2-step RACH (RAN2)

- Specify the fall back procedure from 2-step RACH to 4-step RACH (RAN2/RAN1)

- After 4-step RACH design is completed for NR-U in the Rel-16 NR-U WI, identify and specify the necessary modification of 2-step RACH procedure for its application in NR-U(RAN1/RAN2)

- All triggers for Rel-15 4-step RACH are applied for 2-step RACH except for SI Request and BFR which are up to RAN2 discussion

Note 2: UP data transmission in RRC_IDLE and RRC_INACTIVE state is not in the scope. UP data transmission in RRC_CONNECTED mode as in Rel-15 is supported.

4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

- Specify BS demodulation requirements for the case of PUSCH resource assigned to single UE only

- Specify corresponding BS conformance tests

4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| {Possible values: "TS" or "Internal TR" or "External TR". See Note 1} | {E.g. "22.XXX" or actual number if known} | {Title of the specification (as per TR 21.801 §6.1.1), to be aligned as much as possible with the WI/SI title} | {E.g. "TSG#87"} | {E.g. "TSG#89"} | {e.g.: rapporteur: <FamilyName>, <GivenName>, <Company>, <email address>} |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.211 | Channel structure of MSG A | TSG#86 |  |
| 38.213 | RACH procedure | TSG#86 |  |
| 38.321 | RACH procedure | TSG#86 |  |
| 38.331 | Signaling to configure related parameters | TSG#86 |  |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

6	Work item Rapporteur(s)

ZTE Corporation

{The first listed Rapporteur is the work item primary Rapporteur. The role of a Rapporteur is further described in www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec. 
Secondary Rapporteur(s) are possible for specific secondary task(s).

7	Work item leadership

Leading working group: RAN1

Secondary responsible working group: RAN2, RAN4

8	Aspects that involve other WGs

{Specify all the other WG(s) to be involved and, if specific, their task. E.g.: "SA2, SA3, SA5. CT6 for storage, and potentially SA4". If not applicable, indicate "None" or "None identified yet".}

NOTE:	For RAN WIs: Section 8 applies only toWGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval.}

Supporting IM name

ZTE

Sanechipes

MediaTek

China Telecom

OPPO

Vivo

Xiaomi

CMCC

Intel

InterDigital

CAICT

SAICT

Fujitsu

Verizon

China Unicom

CATT

Google

Panasonic

Ericsson

Qualcomm

ITRI

Hughes

Apple

SONY