3GPP TSG RAN Meeting #82																	RP-182858

Sorrento, Italy, December 10-13, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0073 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Capability for aperiodic CSI-RS triggering with different numerology between PDCCH and CSI-RS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-12-13 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Aperiodic CSI-RS triggering with different numerologies between the PDCCH triggering the CSI-RS reporting and the actual CSI-RS to be measured is introduced in RAN1 specification. For this purpose, the slot offset of aperiodic CSI-RS relative to the triggering PDCCH is clarified for the case where the PDCCH and CSI-RS are transmitted with different numerologies. Capability signalling is required to determine whether a UE supports |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a capability signalling on band-combination level for the feature of “aperiodic CSI-RS triggereing with different numerologies (SCS)”. Imact analysis: Impacted Functionality: CSI-RS measurements and reporting If the NW is implemented according to the CR but the UE is not, there is no inter-operability issue since the NW discovers that the UE does not support the new functionality and the NW will hence refrain from configuring it. If the UE is implemented according to the CR but the NW is not, there is no inter-operability issue since the NW does not support CSI-RS triggering with different numerologies and will hence not configure that functionality anyway. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear if the UE supports aperiodic CSI-RS triggering with different numeroloiges. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 38.331 CR 0792 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 4.2.7.4	CA-ParametersNR

| Definitions for parameters | Per | M | FDDTDD DIFF | FR1 FR2 DIFF |
| --- | --- | --- | --- | --- |
| aperiodic-CSI-diffSCS Indicates whether the UE supports aperiodic CSI-RS triggering with different sub carrier spacing between PDCCH and CSI-RS. | BC | No | No | No |
| diffNumerologyAcrossPUCCH-Group Indicates whether different numerology across PUCCH groups in CA is supported by the UE. | BC | No | No | No |
| diffNumerologyWithinPUCCH-Group Indicates whether UE supports different numerology across carriers within a PUCCH group and a same numerology between DL and UL per carrier for data/control channel at a given time. | BC | No | No | No |
| multipleTimingAdvances Indicates whether multiple timing advances are supported by the UE. For NR CA band combination, if the band combination comprised of more than one band entry (i.e., inter-band or intra-band non-contiguous band combination), the field indicates that different timing advances on different band entries are supported. For EN-DC band combination, this field is not presented and it is mandatory for the UE supporting EN-DC band combination. In this release, up to two timing advances are supported for EN-DC band combination or NR CA band combination. Note: For NR CA, it is mandatory with IOT bit for inter-band NR CA, otherwise optional. For EN-DC, it is mandatory without IOT bit. | BC | Yes/No | No | No |
| parallelTxSRS-PUCCH-PUSCH Indicates whether the UE supports parallel transmission of SRS, PUCCH and PUSCH across CCs in an inter-band CA band combination. | BC | No | No | No |
| parallelTxPRACH-SRS-PUCCH-PUSCH Indicates whether the UE supports parallel transmission of PRACH, SRS, PUCCH and PUSCH across CCs in an inter-band CA band combination. | BC | No | No | No |
| simultaneousRxTxInterBandCA Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band NR CA. It is mandatory for certain TDD-FDD and TDD-TDD band combinations defined in TS 38.101-1 [2], 38.101-2 [3] and 38.101-3 [4]. | BC | Yes/No | No | No |
| simultaneousRxTxSUL Indicates whether the UE supports simultaneous reception and transmission for a NR band combination including SUL. Mandatory/Optional support depends on band combination and captured in TS 38.101-1 [2]. | BC | Yes/No | No | No |
| supportedNumberTAG Defines the number of timing advance groups are supported by the UE | BC | Tbd | No | No |