3GPP TSG RAN meeting #83										RP-190149

Shenzhen, China, March 18-21, 2019

## Status Report to TSG

Agenda item:			9.4.1

| WI / SI Name | Enhancements on MIMO for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_eMIMO |  |  |  |  |
| Unique ID | 800085 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-182863 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 12/2019 | Performance part: 09/2020 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 25% | Performance Part: 0% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Eko Onggosanusi |
|  | Company | Samsung |
|  | Email | eko.o@samsung.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

The following agreements and conclusions were made in RAN1 NR-AH 1901:

- CSI enhancement for MU-MIMO support

Agreement

On FD compression unit, agree on Alt1 (PMI subband size = CQI subband size) as the default, along with Alt2.2 (PMI subband size = CQI subband size / R) as secondary

- The value of R is fixed to 2

- FFS: Whether secondary implies a separate UE capability or restricted use cases

  - Include issues such as limitation on the number of FD compression units, CPU occupation, latency constraint and/or BW constraint

- FFS: Whether FD compression unit is higher-layer configured or reported by the UE

Agreement

On basis/coefficient subset selection for the first layer, the following is supported:

- Common selection for all beams with size-K0 subset of 2LM reported

  - The value of K0 is configured via higher-layer signaling

    - The number of reported non-zero coefficients can be smaller than or equal to K0

  - FFS: Whether the value of M is configurable

Working Assumption

On the choice of oversampling factor O3, O3 = 4 is supported

Agreement

On FD compression unit, the FD compression unit is higher-layer configured

Agreement

On basis/coefficient subset selection for the first layer, support the following:

- Size-K0K0 subset design: down select in RAN1#96 from the following alternatives

  - Alt1. Unrestricted subset (size=2LM)

  - Alt2. Polarization-common subset (size=LM)

  - Alt3. Restricted subset (for a given subset of beams and FD basis, size=2L+M)

- The value of K0:  K0=β×2LM where two values of β are supported

  - Down select in RAN1#96 from  β∈18,14,12,34

- The UCI consists of two parts:

  - Information pertaining to the number(s) of non-zero coefficients is reported in UCI part 1

    - Note: This does not imply whether this information consists of single or multiple values

  - The payload of UCI part 1 remains the same for different RI value(s)

- Bitmap is used to indicate non-zero coefficient indices

Agreement

On the values of L, support L={2,4}

- Decide whether to support L=3 and/or L=6 in the future meetings considering the performance-overhead trade-off for different RI values and/or different number of antenna ports

Agreement

Two values of M are supported. In RAN1#96, down select between the following alternatives (p∈14,12):

- Alt1. M=p×N3R

- Alt2. M=p×N3

- FFS: support for p=1/8 and/or p=3/4 in addition to 1/4 and 1/2

Agreement:

The value of M is higher-layer configured

- FFS: Whether UE reporting smaller value of M (in addition to the configured M) is supported

Agreement

Values of N3: For R∈1,2 and NSB is # CQI subbands, when NSB×R≤19,

Values of N3: For R∈1,2 and NSB is # CQI subbands, when NSB×R>19, downselect among the following alternatives in RAN1#96

- Alt1: N3N3 is smallest multiple of 2, 3, or 5 which is  ≥NSB×R

- Alt2: N3N3 is a multiple of 2, 3, or 5. Segment into 2 parts with overlapping between 2 parts. Note: no padding is needed to align the DFT size with the multiple of 2, 3, or 5

- Enhancements on multi-TRP/panel transmission

Agreement

For multi-DCI based multi-TRP/panel transmission, the total number of CWs in scheduled PDSCHs, each of which is scheduled by one PDCCH, is up to X and also the total number of MIMO layers of scheduled PDSCHs is up to reported UE MIMO capability, if resource allocation of PDSCHs are overlapped.

- X=2

- FFS: X=3

Agreement

For multiple-PDCCH based multi-TRP/panel transmission for eMBB, for the purposes of PDCCH detection, UE does not assume any dependency amongst the multiple PDCCHs

Agreement

For multiple-PDCCH based multi-TRP/panel downlink transmission for eMBB,

- Separate ACK/NACK payload/feedback for received PDSCHs is supported

  - FFS: Details on PUCCH carrying separate ACK/NACK payload/feedback

FFS: Whether to additionally support joint ACK/NACK payload/feedback for received PDSCHs

Agreement

For a UE supporting multiple-PDCCH based multi-TRP/panel transmission and each PDCCH schedules one PDSCH, at least for eMBB with non-ideal backhaul, down-select one alternative from following in RAN1 96

- Alt 1: the UE may be scheduled with full/partially/non-overlapped PDSCHs at time and frequency domain by multiple PDCCHs

- Alt 2:  the UE can be only scheduled with full/non-overlapped PDSCHs at time and frequency domain by multiple PDCCHs

- Alt 3: the UE may be scheduled with full/partially/non-overlapped PDSCHs at time and frequency domain by multiple PDCCHs with following restrictions:

  - Same DMRS configuration with respect to actual number of front loaded DMRS symbol(s), the actual number of additional DMRS, the DMRS symbol location and DMRS configuration type shall be assumed by the UE for full/partially overlapping PDSCHs.

  - The UE is not expected to have more than one TCI state with DMRS ports within the same CDM group for full/partially overlapping PDSCHs

  - Full scheduling information for receiving a PDSCH is indicated and carried only by the corresponding PDCCH.

Other restrictions are not excluded, for example BWP switching

Agreement

TCI indication framework shall be enhanced in Rel-16 at least for eMBB:

- Each TCI code point in a DCI can correspond to 1 or 2 TCI states

  - When 2 TCI states are activated within a TCI code point, each TCI state corresponds to one CDM group, at least for DMRS type 1

    - FFS design for DMRS type 2

  - FFS: TCI field in DCI, and associated MAC-CE signaling impact

Agreement

For multi-TRP specification support for URLLC, support at least one of following schemes for transmitting the same transport block from multiple TRPs. Study following schemes for further down-selection for one or more schemes in next meetings

- Scheme 1 (SDM):  n (n<=Ns) TCI states within the single slot, with overlapped time and frequency resource allocation

- Scheme 2 (FDM): n (n<=Nf) TCI states within the single slot, with non-overlapped frequency resource allocation

- Scheme 3 (TDM): n (n<=Nt1) TCI states within the single slot, with non-overlapped time resource allocation

- Scheme 4 (TDM): n (n<=Nt2) TCI states with K different slots.

- For further study:

  - Details on restriction related to MCS, modulation order for PDSCHs from different TRPs w.r.t. schemes 1 to 4.

  - Whether to support mini-slot PDSCH repetitions

  - Signalling mechanism

  - Companies to consider how the schemes apply for FR1 and FR2

  - Whether the number of repetitions can be larger than the number of TCI states (n)

- Further clarification for each scheme can be elaborated in RAN1 96

- Baseline scheme in addition to Rel-15 single-TRP scheme for evaluations

  - SFN transmission based on Rel-15 from multi-TRP with single TCI state

    - Companies to provide details on assumption on time/frequency synchronization and TRS transmission across TRPs

- Note that supporting multiple schemes in Rel-16 is not excluded.

- Note that control signalling mechanism for PDSCH reliability/robustness enhancement schemes can be discussed separately.

- Enhancements on multi-beam operation

Agreement

For UL beam management latency reduction in controlling PUCCH spatial relation, the maximum RRC configurable number of spatial relations for PUCCH (i.e., maxNrofSpatialRelationInfos) is increased to be 64 per BWP.

- FFS: RRC and/or MAC CE signaling overhead reduction related to this.

Agreement

For latency and overhead reduction for DL beam management,

- No new CSI-RS design and no new term such as ‘sub-time unit’ or ‘sub-symbol’ are introduced in Rel-16, i.e., no support of sub-time unit for beam management RS shorter than 1 OFDM symbol

Companies can provide further evaluation results and proposals for faster DL beam operation other than those requiring sub-time unit

Agreement

An identifier (ID), agreed in RAN1#95, that can be used at least for indicating panel-specific UL transmission is to be down-selected or merged from the following alternatives in next RAN1 meeting:

- Alt.1: an SRS resource set ID, where FFS on further association to other RS (if needed)

- Alt.2: an ID, which is directly associated to a reference RS resource and/or resource set

- Alt.3: an ID, which can be assigned for a target RS resource or resource set

- Alt.4: an ID which is additionally configured in spatial relation info

Agreement

For L1-SINR, interference can be measured based on dedicated resource(s) for interference measurement.

- FFS: UE assumes interference signal on the REs of the RS for signal part and REs for dedicated resource(s) for interference measurement similar to specified in 38.214

- FFS: whether resource(s) for interference measurement can be NZP based or ZP based or both

- FFS: whether/how to reuse NZP CSI-RS resource(s) configured for channel measurement as resource(s) for interference measurement

Agreement

Specification support will be provided for gNB to derive at least the failed CC index during SCell BFR procedure

- FFS: Whether the information is implicitly derived or explicitly conveyed by the UE

- FFS: Whether new beam information should be included

- FFS: Details on triggering for transmitting BFRQ

Agreement

- SCell BFD is based on periodic 1-port CSI-RS, which can be configured explicitly by RRC or implicitly by TCI state.

- Down-select one of the following alternatives in RAN1#96:

  - Alt 1: SCell BFD RS is in current CC

  - Alt 2: SCell BFD RS is in current CC for explicit configuration and can be in current CC or another CC for implicit configuration

  - Alt 3: SCell BFD RS can be in current CC or another CC for both explicit and implicit configuration

- SCell BFD is measured based on hypothetical BLER

Agreement

Down-select at least one of the following alternatives:

- Alt 1: For SCell BFR, BFRQ can be transmitted if UE declares beam failure and identifies a new candidate beam.

  - UE reports new beam information by or after BFRQ

- Alt 2: For SCell BFR, BFRQ can be transmitted if UE declares beam failure.

  - UE only indicates beam failure happens by BFRQ

  - Note: new beam identification can be done by using DL BM procedure

- Alt 3: For SCell BFR, BFRQ can be transmitted if UE declares beam failure

  - UE may report new beam information during BFR procedure

  - FFS: impact of new beam identification threshold

- Note: It is up to UE whether to do beam failure detection and new beam identification in parallel or not

- For Alt1 and Alt3, reference signals for new candidate DL beam(s) are configured, which are based on CSI-RS and/or SSB.

  - FFS: whether the CSI-RS and/or SSB can be in another CC

  - FFS: signaling details, e.g. RRC and/or MAC CE

Agreement

For SCell BFR

- Decide BFRQ solution for BFR on SCell with DL only first, PCell in FR1+FR2

- Above is to facilitate RAN1 discussion but not to prioritize certain scenarios

- Full TX power UL transmission

Agreement

Full TX power UL transmission with multiple power amplifier is supported at least for codebook based UL transmission for non-coherent and partial/non-coherent capable UEs. The support of this feature is indicated by the UE as part of UE capability signalling. For power class 3:

- UE capability 1: for the UE to support full Tx power in UL transmission, full rated PAs on each Tx chain is supported with a new UE capability

  - FFS: detailed power scaling description

  - Note: Full Tx power means UE delivers total power of 23dBm for PC3

- UE capability 2: for the UE to support full Tx power in UL transmission, no Tx chain is assumed to deliver full power with the new UE capability

  - FFS: detailed design

- UE capability 3: for the UE to support full Tx power in UL transmission, subset of Tx chains with full rated PAs is supported with a new UE capability

FFS: Whether all three capabilities will be specified or a subset will be specified

FFS: UE capability signalling/reporting details

Note: Two or more of the above capabilities could be merged depending on the further details

Send LS to RAN4 to provide their view on PC 2 applicability of the new UE capability (Rakesh, vivo).

- Low PAPR RS

Agreement

The RRC parameters DMRS-DownlinkConfig and DMRS-UplinkConfig are extended to configure the alternative use of Rel. 16 sequence selection and behavior for the 2nd and 3rd CDM group for PDSCH and PUSCH respectively and for CP-OFDM only. DCI is not used to select between Rel.15 and Rel.16 behaviour.

Agreement

For length 12,18, and 24 respectively, NR Rel-16 supports the binary CGS in the Table C1, C2, and C3 respectively followed by pi/2 BPSK modulation followed by DFT as DMRS sequence for π π/2 BPSK modulation for both PUSCH and PUCCH.

- The above is applicable to single-symbol DMRS configuration

- FFS: CGS for two-symbol DMRS configuration

Table C1, C2, and C3 can be found in R1-1901362

Agreement

- For pi/2 BPSK Rel.16 sequence design for PUSCH DMRS of length ≥30

  - sdf

  - Cell ID default scrambling parameter(s) unless configured otherwise

  - Use c_init formula from Rel.15 CP-OFDM DMRS and reuse Rel-15 Gold sequence generator

- Open issues for further study:

  - Whether new Rel.16 DMRS sequence is used for Msg3

  - For which DCI formats, search spaces and RNTIs the new Rel.16 DMRS sequence configuration is applicable

  - Configuration of Rel.16 sequence for pi/2 BPSK PUSCH DMRS

    - Alt.A One configuration of Rel.16 sequence applies to all lengths

    - Alt.B Independent configuration of Rel.16 sequence for sequence length ≥30  and <30

- Down-select between Alt.1 and Alt.2 in RAN1#96

  - Alt.1  Follow DFT-S-OFDM approach

    - nIDnSCID  is defined as nPUSCH-Identity

    - Only nSCID=0 is applicable

    - No change to DCI, only RRC is used to configure

  - Alt.2 Follow CP-OFDM approach

    - nIDnSCID   parameters are configured by RRC as for CP-OFDM DMRS

    - DCI is used to indicate nSCID as for CP-OFDM

      - 1 bit always present in DCI when Rel.16 DMRS is configured

    - FFS on how to interpret 1 bit in DCI for length<30

    - Possible benefit: gNB can dynamically select sequence to reduce probability of a bad sequence choice (e.g. nulls in PSD)

Agreement:

For length 6 CGS; 8-PSK is used

Decide the associated sequences in the next RAN1 meeting

Conclusion

Multiple companies in RAN1 have identified a power imbalance issue due to the some combinations of Rel.15 DMRS and some MIMO precoding matrices

- For example, it may occur for some cases of two symbol DMRS and two or four layer MIMO for a UE

- Some companies claim this can be avoided by implementation based solutions for PDSCH and PUSCH

Agreement

For one OFDM symbol DMRS and for PUSCH with pi/2 BPSK modulation, down select among the following alternatives

- Alt.0: Only a single DMRS port is supported (one comb is used)

- Alt.1: One DMRS port per comb is supported (in total 2 ports)

- Alt.2: Support two DMRS ports per comb (in total 4 ports)

- Study may take UL timing misalignment into account

The following agreements and conclusions were made in RAN1#96:

- CSI enhancement for MU-MIMO support

Agreement

Proposal 1 from R1-1902304 is agreed

Agreement

Extend the Type II DFT-based compression (designed for RI=1-2) to RI=3-4 with the following design principle:

- The resulting overhead for RI=3-4 is at least comparable to that for RI=2

Agreement

On subset selection for layer 0, agree on the following:

- Unrestricted (polarization-independent) subset selection which requires a size-2LM bitmap in UCI part 2

- β∈14,12,34

  - FFS: Further down selection of supported combinations of FD compression parameters

Agreement

On LCC quantization, agree on the following:

- The description of each of the five alternatives above is final (in R1-1902304, section 2.5)

- Agree on Alt2 (differential per polarization) per the description in R1-1902304

Agreement

On subset selection for RI=2, agree on the following

- SD basis selection (selection of L out of N1N2 SD DFT vectors) is layer-common

- Terms:

  - “FD basis subset selection” refers to the selection of M out of N3 FD DFT vectors

  - “Coefficient subset selection” refers to the selection of KNZ (# non-zero coefficients) out of 2LM where KNZ ≤ K0

- The size-K0 subset design for layer 0 is also applied to layer 1

- K0 is the maximum number of non-zero coefficients for each layer.

- Layer-independent FD basis subset selection

- Layer-independent coefficient subset selection

Agreement

On the values of N3, further discuss and clarify/refine both of the available alternatives with N3=NSB×RN3=NSB×R as the evaluation baseline

Agreement

For RI∈3,4, different layers are independently quantized just as RI=1 and 2

Agreement

On SD and FD basis selection for RI∈3,4

- The parameter R is layer-common and RI-common

- For the higher-layer setting of SD/FD basis parameters (L, p):

  - Down select among the following alternatives for the higher-layer setting of SD/FD basis parameters (L, p):

    - Alt1 RI-common for RI∈1,2,3,4, layer-common

    - Alt2 RI-common for RI∈1,2,3,4, layer-/layer-group-specific

    - Alt3 RI-common for RI∈3,4, layer-common

    - Alt4 RI-common for RI∈3,4, layer-/layer-group-specific

    - Alt5 RI-specific for RI∈3,4, layer-common

    - Alt6 RI-specific for RI∈3,4, layer-/layer-group-specific

  - Note: For RI=1 and 2, RI-common, layer-common setting has been agreed

  - Note: No other alternatives will be considered

Email discussion by 15th of March: Companies to provide more details on the alternatives listed above. Strive to converge on a single set of parameters for each alternative. (Eko, Samsung)

Agreement

On the max # NZ coefficients for RI∈3,4, down select from the following alternatives (no other alternatives will be considered)

- Alt0. For RI∈1,2,3,4, there is only one β value

- Alt1. Total max # NZ coefficients across all layers ≤ 2K0 (the K0 value set for RI∈1,2)

- Alt2. For RI∈3,4, there is only one value of max # NZ coefficients per layer <K0 where the K0 value is set for RI∈1,2

- Alt3. Total max # NZ coefficients across all layers ≤ 2αK0 (the K0 value set for RI∈1,2) where α is fixed and RI-specific

  - FFS: value of α per agreement that the overhead for RI=3 or 4 should at least be comparable to RI=2

- Alt4. For RI∈3,4, there is only one value of max # NZ coefficients per layer <αK0 where the K0 value is set for RI∈1,2 where α is fixed and RI-specific

  - FFS: value of α per agreement that the overhead for RI=3 or 4 should at least be comparable to RI=2

- Note: For RI∈1,2, there is only one K0 value (=max # NZ coefficients per layer)

Agreement

Decide in RAN1#96bis whether to support L=6 (in addition to L=2 and 4) taking into account RI∈3,4 and/or 32 ports

- Also consider possible restriction(s) on the use case for supporting the additional value(s) of L

- Enhancements on multi-TRP/panel transmission

Agreement

For multi-DCI based multi-TRP/panel transmission, the total number of CWs in scheduled PDSCHs, each of which is scheduled by one PDCCH, is up to 2.

Agreement

For a UE supporting multiple-PDCCH based multi-TRP/panel transmission and each PDCCH schedules one PDSCH, at least for eMBB with non-ideal backhaul, support following restrictions:

- The UE may be scheduled with fully/partially/non-overlapped PDSCHs at time and frequency domain by multiple PDCCHs with following restrictions:

  - The UE is not expected to assume different DMRS configuration with respect to actual number of front loaded DMRS symbol(s), the actual number of additional DMRS, the actual DMRS symbol location and DMRS configuration type if the UE may be scheduled with full/partially overlapping PDSCHs by multiple PDCCHs.

  - The UE is not expected to have more than one TCI index with DMRS ports within the same CDM group for fully/partially overlapped PDSCHs

  - Full scheduling information for receiving a PDSCH is indicated and carried only by the corresponding PDCCH.

  - The UE is expected to be scheduled with the same active BWP bandwidth and the same SCS if the UE is expected to receive multiple PDSCHs simultaneously at given symbols.

  - The number of active BWPs for a UE is 1 per CC

  - FFS: PDSCH mapping type from two co-scheduled PDSCHs

  - FFS: Alignment of PRG-level grid from multiple TRPs

  - FFS: How to ensure the same active BWP between multiple TRPs

  - Note that rate matching mechanisms (if need) to support multi-DCI based NCJT will be discussed separately.

Agreement

For multiple-PDCCH based multi-TRP/panel transmission, rate matching, puncturing, and pre-emption mechanisms shall be studied/enhanced if need, e.g. ratematchpattern, DMRS ports, ZP/NZP CSI-RS, SSB, configured CORESET, lte-CRS-ToMatchAround, pre-emption indications.

- to be discussed and down-selected in RAN1#96bis

Agreement

For TCI state configuration in order to enable one or two TCI states per a TCI code point,

- MAC-CE enhancement to map one or two TCI states for a TCI code point where further detailed design is determined in RAN2.

- FFS whether increasing the number of bits of TCI field in DCI

Include in LS to RAN2

R1-1903637	Draft LS on support of Enhancements on multi-TRP/panel transmission

The draft LS is endorsed in R1-1903697 with updates on new RAN1 agreements.

Agreement

To support multiple-PDCCH based multi-TRP/panel transmission with intra-cell (same cell ID) and inter-cell (different Cell IDs), following RRC configuration can be used to link multiple PDCCH/PDSCH pairs with multiple TRPs

- one CORESET in a “PDCCH-config” corresponds to one TRP

  - FFS whether to increase the number of CORESETs per “PDCCH-config” more than 3

FFS: UE monitoring/decoding behavior for multiple PDCCHs.

Include in LS to RAN2

Agreement

For separate ACK/NACK payload/feedback for received PDSCHs where multiple DCIs are used,

- PUCCH resources conveying ACK/NACK feedback can be TDM with separated HARQ-ACK codebook.

  - FFS TDM within a slot

  - FFS: the format of PUCCH from multiple TRP shall be same or different

For issues related to PUCCH resources, study including:

- FFS: if PUCCH resources conveying ACK/NACK feedback are overlapped at time, whether predefined dropping rule is needed to drop ACK/NACK feedback.

- FFS: how to handle ACK/NACK overlapping with CSI reporting for different TRPs

- FFS: how to handle PUCCH overlapping with PUSCH at the time domain for different TRPs

- FFS: whether the UE can assume simultaneous ACK/NACK transmission from multiple PUCCH resources, and associated details of configurations/indication/UE capability.

Include in LS to RAN2

- Enhancements on multi-beam operation

For purpose of further discussion on this topic for RAN1#96 and future meetings

Following multi-panel UE (MPUE) categories can be used for discussions on possible enhancements over Rel-15, if needed.

- MPUE-Assumption1: Multiple panels are implemented on a UE and only one panel can be activated at a time, with panel switching/activation delay of [X] ms

- MPUE-Assumption2: Multiple panels are implemented on a UE and multiple panels can be activated at a time and one or more panels can be used for transmission

- MPUE-Assumption3: Multiple panels are implemented on a UE and multiple panels can be activated at a time but only one panel can be used for transmission

Note: Above does not imply the support of either one or both of the categories but is only for efficient discussions at least for this meeting, which may also be updated further. Whether to support either one or both categories will depend on subsequent discussions

Note: There is no consensus among the companies in RAN1 whether MPUE-Assumption2 is in the work scope of Rel-16 WI

Agreement

If RAN1 cannot agree on the support of at least one of MPUE-Assumption1, MPUE-Assumption2, MPUE-Assumption3, enhancements on panel-specific beam selection for uplink will not be supported in Rel-16.

- Deadline for decision: RAN1#96bis

Agreement

For signaling overhead reduction on updating/configuring spatial relation for PUCCH, support simultaneous spatial relation update/configuration for multiple PUCCH resources

- FFS signaling details to be decided in next meeting, including down-selection/merging among the following options

  - Spatial relation update for all PUCCH resources in a CC by one MAC CE

  - Spatial relation update per Rel-15 PUCCH resource set

  - Spatial relation update per group of PUCCH (which might need to be introduced for Rel-16)

  - PUCCH spatial relation info configured in a BWP could be applied across different BWP or different cells

  - Other options are not precluded.

Agreement

In RAN1#96bis, determine whether to support the configuration of up to 64 candidate beams for BFR by RRC signaling.

- FFS signaling details including whether MAC-CE message can choose a subset of the candidate beams as active resources for new beam identification in Rel-16

Working Assumption

For UL beam management latency and overhead reduction, support MAC CE based spatial relation update for aperiodic SRS per resource level

- FFS: Whether this is a UE optional feature

- Note: Qualcomm prefers to have this as a UE optional feature

Agreement

- For SCell BFR, BFRQ shall be conveyed if UE declares beam failure

  - UE shall convey new beam information during BFR procedure if new candidate beam RS and corresponding threshold is configured and at least if channel quality of new beam is above or equal to threshold

    - FFS: whether no new beam identified could be included as a state of new beam information

    - FFS: details if no new beam is above or equal to threshold

Agreement

For interference measurement of L1-SINR, down select one of the following in RAN1#96bis

- Alt 1: dedicated ZP IMR

- Alt 2: dedicated NZP IMR

- Alt 3: dedicated ZP IMR and dedicated NZP IMR

Companies are encouraged to provide use cases and benefit, e.g. throughput and gNB/UE complexity benefit for different alternatives

- L1-RSRP/CSI based beam selection could be baseline

- Full TX power UL transmission

Agreement

Note: UE capability 1, 2, 3 agreed in RAN1#AH1901 mean the PA architectures.

At least for PC3, UE capability 1, 3 can support full power transmission.

Working assumption: For PC3, UE capability 2 can support full power transmission.

- Companies to check for any implementation issues and/or performance of Rel-16 full power transmission compared to Rel-15 non-coherent codebook subset uplink transmission)

Agreement

Down select among the following two alternatives by RAN1#96bis. As part of UE capabilities signalled the following is included:

Alt1: UE capability signaling of supported one or group of TPMI precoder(s)

Alt3: UE capability signaling of power scaling schemes for full uplink power transmission

- Note: This does not imply any restriction on UE antenna virtualization

- FFS: Whether full uplink power transmission needs to be supported for all precoders

- Low PAPR RS

Agreement

When Rel-16 sequence for pi/2 BPSK PUSCH DMRS is configured, the Rel-16 sequence is used for all sequence lengths.

Agreement

For the case of CGS sequence in case of two adjacent symbol DMRS, the same CGS sets are used for both single and two symbol DMRS for pi/2 BPSK modulation.

- FFS: Whether or not to support deterministic sequence hopping pattern

Agreement

For CP-OFDM, if Rel.16 DMRS is configured by RRC, it only applies when scheduled by DCI formats 0_1 and 1_1 or with a PUSCH transmission with configured grant.

- Note: It is up to the editor whether/how to capture this agreement

Agreement

If Rel-16 DMRS is configured by RRC for PUSCH with pi/2 BPSK modulation, the Rel-16 DMRS applies when scheduled by DCI formats 0_1 and 0_0 and using at least C-RNTI, CS-RNTI, SP-CSI-RNTI and MCS-C-RNTI, when applicable or PUSCH transmission with configured grant

- Search space for DCI format 0_0: Rel-16 DMRS for at least USS is supported

Agreement

The PUSCH multiplexing capacity when Rel-16 DMRS is configured for pi/2 BPSK PUSCH is up to two ports for one OFDM symbol, four ports (TD-OCC across OFDM symbols) for two OFDM symbols.

Note: Qualcomm showed strong concern on the benefits of pi/2 BPSK due to this agreement

Agreement

The PUCCH multiplexing capacity when Rel-16 DMRS is configured for pi/2 BPSK PUCCH is a single port

Note: Qualcomm showed strong concern on the benefits of pi/2 BPSK due to this agreement

Agreement

- For PUSCH DMRS with pi/2-BPSK modulation, two RRC parameters ScramblingID0 and ScramblingID1 can be configured by RRC as for CP-OFDM DMRS (16 bit per ID)

- Used in cinit for length ≥30 as n_ID^{n_SCID=0}, n_ID^{n_SCID=1}

- For length<30, these parameters are used as N_ID^PUSCH is configured as either ScramblingID0 or ScramblingID1 depending on DCI

- DCI indicates n_SCID={0,1} dynamically, without increasing DCI size to select the above parameter

  - Antenna port indication is re-used for n_SCID indication as follows:

    - DMRS port 1000,1002,1004 and 1006 are interpreted as port 1000,1002,1004 and 1006, respectively, with n_SCID=0

    - DMRS port 1001,1003,1005 and 1007 are interpreted as port 1000,1002,1004 and 1006, respectively, with n_SCID=1

Agreement

- Rel.16 DMRS is not supported for MSG3 transmissions, Rel.15 behavior is used for MSG3

#### 2.1.2	Remaining Open issues

- CSI enhancement for MU-MIMO support

  - Rank 3-4 extension of Type II DFT-based compression: detailed design for parameter setting and subset selection

  - UCI design: complete content of part 1 and 2

  - Other issues, e.g. CBSR, “Other schemes”, N3 for larger # subbands, additional optimization

- Enhancements on multi-TRP/panel transmission

  - Remaining issues on multi-PDCCH for supporting NCJT, e.g. details on partially-overlapping scheduling, PDSCH-related enhancements (PDSCH mapping, RM, puncturing, potential “pre-emption”), PUCCH/PUSCH-related enhancements (TRP differentiation mechanism of PUCCH/PUSCH for HARQ-ACK reporting)

  - Remaining issues on single-PDCCH for supporting NCJT, e.g. DMRS/PTRS port indication

  - Study of enhancements for URLLC use cases and, if specified, the pertinent specification enhancements

- Enhancements on multi-beam operation

  - Components of L1-SINR, e.g. SINR definition, interference measurement (resources and setting), reporting format

  - Exact SCell BFR scheme (e.g. BFRQ reporting)

  - Down selection from 3 UE assumptions and, if agreed, the corresponding UL “panel-specific” identification for UL beam indication

  - Further study of DL/UL beam indication with reduced overhead/latency and, if specified, the pertinent specification enhancements, e.g. whether to support the configuration of up to 64 candidate beams for BFR by RRC and if supported, the signaling details

- Full TX power UL transmission

  - Scheme selection for codebook-based UL transmission

  - Scheme down-selection for UE capability signaling/reporting for full power transmission

- Low PAPR RS

  - Length-6 DMRS sequences design for pi/2-BPSK

  - Other issues, e.g., whether to support deterministic sequence hopping, sequence reordering, and CSS for enhanced DMRS

- Evaluation Methodology

  - No open issue

## 2.2	RAN2

#### 2.2.1	Agreements

Not yet started

#### 2.2.2	Remaining Open issues

Not yet started

## 2.3	RAN3

n/a

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

Not yet started

#### 2.4.2	Remaining Open issues

Not yet started

## 2.5	RAN5

n/a

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

n/a

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

n/a

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

[1] 3GPP, RAN1, RAN1#94bis Chairman Notes

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template