3GPP TSG RAN meeting #83										RP-190181

Shenzhen, China, March 18-21, 2019

## Status Report to TSG

Agenda item:			9.4.8

| WI / SI Name | Integrated Access and Backhaul for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_IAB |  |  |  |  |
| Unique ID | 820070 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-182882 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 12/2019 | Performance part: 06/2020 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 10% | Performance Part: 0% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN-2 |
| --- | --- | --- |
| Rapporteur | Name | Georg Hampel |
|  | Company | Qualcomm Incorporated |
|  | Email | ghampel@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1 #AH1901

Agreements:

- For both SA and NSA, for initial cell selection, an IAB-node MT may assume that half frames with SS/PBCH blocks occur with a periodicity of 16 frames.

  - FFS cell re-selection

  - Study further the value of assumed SS/PBCH block periodicity for a backhaul link switching to a new parent node when the link to the current parent node fails.

Agreements:

- SSB transmission configuration information (i.e. the necessary information for the DU to determine when to transmit SSB) and SSB reception configuration information (i.e. the necessary information for the MT to determine when to search for SSB) for IAB inter-node measurements are determined in a centralized manner, i.e. not locally by the IAB node.

- An IAB node is provided with configuration of SSB transmission and reception information for IAB inter-node measurements.

  - FFS details

Agreement:

At least existing resource definitions (D/U/F) and semi-static and dynamic signaling methods defined in Rel-15 for access UEs are reused for configuration and indication of MT resources to be used by the backhaul link between the IAB node and its parent.

- FFS details

Agreement:

IAB-node/IAB-donor DU resources are provided by a semi-static configuration which is provided separately from the MT resource indication

- FFS: whether the configuration is per-link or per-DU

- FFS: details for the configuration

Agreement:

Support indication of the dynamic availability of soft resources at an IAB node from a parent

- FFS details, including explicit vs. implicit indication

Agreements:

An IAB node should set its DL TX timing ahead of its DL Rx timing by TA/2 + T_delta

- T_delta is signalled from the parent node, where the value is intended to account for factors such the offset between parent DL Tx and UL Rx, if any due to factors such as Tx to Rx switching time, HW impairments, etc.

- TA is the timing gap between UL Tx timing and DL Rx timing, which is derived based on existing Rel-15 mechanism

- FFS (not necessarily an exhaustive list):

  - value range and granularity of Tdelta

  - need for aperiodic/periodic updates of Tdelta

  - other timing impairment factors for adjusting IAB node timing to be included in Tdelta

  - timing alignment when the IAB node has multiple parents

  - Note: once the design of the above FFS points is in a good shape, an LS to RAN4 may be necessary to solicit their input

RAN1#96

Agreements

- Existing NR mechanisms are used by the network to signal to IAB MTs the SSB periodicity for cell re-selection.

- The IAB-node MT initial access assumption that half frames with SS/PBCH blocks occur with a periodicity of 16 frames does not have an impact on cell re-selection.

Agreements:

- The IAB-node MT initial access assumption that half frames with SS/PBCH blocks occur with a periodicity of 16 frames does not have an impact on backhaul link rerouting to a new parent node in response to a failure in the link to the current parent node.

- Send an LS to RAN2/3 capturing agreements from this meeting and RAN1 AdHoc1901 – R1-1903809

Agreements:

- The SSBs for IAB inter-node discovery and measurements are defined with a framework using the characteristics of the Rel 15 SMTC framework with the following enhancements:

  - The maximum number of SMTC windows that can be configured for an IAB node, referred as max( NRX ), is at least 3

    - FFS whether max( NRX ) needs to be greater than 3

    - FFS whether or not to extend existing SMTC window configuration

  - Each SMTC window is provided its own independent configuration (e.g. periodicity, offset, duration)

  - Introduction of SSB transmission configurations (STC) indicating SSB transmission(s)

    - FFS details

  - FFS how to resolve potential collision between SMTC and STC

Agreements:

- New RACH configurations specific to IAB nodes are derived with extension of existing Rel 15 RACH configurations obtained by:

  - scaling the parameter ‘x’ from the PRACH configuration table, and

  - by adding an offset y_offset to the parameter ‘y’ (frame-based offset) and/or adding an offset to the slot/subframe number (slot/subframe-based offset) from the PRACH configuration table.

- FFS values and ranges for scaling factor applicable to ‘x’ and for y_offset.

- FFS whether a simple extension rule can be generally applied to all existing configurations leaving it up to the network to not use resulting configurations that may not fit other system constraints.

Agreements:

- If a resource is configured as not available, the DU cannot assume it can use the resource

- In case of hard DU resources, the DU can assume it can use the resource regardless of the MT’s configuration

  - FFS: Exception cases for specific signals/channels to be transmitted or received by the MT in the same resource (e.g. SS/PBCH blocks, SI reception, RACH)

- In case of soft DU resources:

  - If the soft resource is indicated as available, the DU can assume it can use the resource

  - If the soft resource is not indicated as available, the DU cannot assume it can use the resource

  - The use of soft resources at least corresponds to transmission/reception of specific signals and channels (e.g. PDSCH/PUSCH) at the DU

    - FFS the use of soft resources in case of cell-specific (e.g. SS/PBCH blocks, SI reception, RACH) signals and channels to be potentially transmitted/received at the DU

  - Both implicit and explicit indication of the availability of soft resources at an IAB node is supported

    - In case of implicit indication of DU soft-resource availability, the IAB node knows that the DU resource can be used without impacting the MTs ability to transmit/receive according to its configuration and scheduling based on indirect means. Examples of such means may include:

      - the lack of uplink scheduling grant at the MT

      - no data available for MT transmission

      - the configured MT search space,

      - configured RS measurement occasions (e.g. SSB/CSI-RS)

      - FFS: consider whether the parent should be able to always be aware of/control the outcome of implicit indications at child nodes

    - Explicit indication that a resource is available is based on DCI indication. The following options can be considered:

      - SFI-like indication via DCI Format 2_0

      - Use of 2 SFI indications (e.g. based on multi-slot scheduling mechanism)

      - Define a new DCI format

      - Other options are not precluded

      - FFS: whether an explicit indication that a resource is available always has priority over any implicit determination of the availability of a resource

  - Further consider factors impacting the usage of soft resources at a child DU, including:

    - MT’s decoding delay

    - Information exchange delay between MT and DU

    - DU’s PDSCH preparation time

    - UE PUSCH preparation time

    - Accumulated delay across hops

Agreements:

- Inter-IAB node conflict resolution can be supported by the following options (to be down-selected)

  - Alt1: the parent node is aware of all of the DU resource configurations (D/U/F/H/S/NA) of its child IAB node DUs,

  - Alt2: the parent node may be made aware of a subset of the DU resource configurations (D/U/F/H/S/NA) of its child IAB node DUs

  - FFS: whether the indication of the child DU resources at the parent is via explicit (e.g. F1-AP signaling) or implicit (e.g. based on child MT configuration) means

Agreements:

- Both slot and symbol-level multiplexing of access and backhaul links are supported.

Agreements: (R1-1903583)

- T_delta is indicated by a parent to the child node independently from the existing Rel.15 TA indication from the parent node used to set the UL Tx timing of the child IAB node’s MT

  - T_delta is updated on an aperiodic basis determined by the parent node

  - The child IAB node should trigger its DL TX timing adjustment by TA/2 + T_delta after it receives the timing offset T_delta indication from its parent node, if it is using OTA Timing Case 1 to obtain its DL timing.

    - FFS: behavior if TA/2 + T_delta results in an effective negative timing offset

    - FFS: delay between receiving T_delta and application of T_delta at the child node

  - Separate value ranges/granularities may be considered for T_delta in FR1 and T_delta in FR2

- Send LS to RAN4 asking them to determine the exact values and granularity of T_delta and provide confirmation on RAN1’s assumption on the DL timing accuracy requirements for IAB nodes in case of OTA Case 1 timing is applied across multiple hops – R1-1903810

#### 2.1.2	Remaining Open issues

- Physical layer specification [RAN1-led, RAN2, RAN3, RAN4]:

  - Specification of extensions to Rel. 15 to support the use of SSBs orthogonal to SSBs used for UEs (via TDM and/or FDM), for inter-IAB-node discovery and measurements, including additional SMTC periodicities and time-domain mapping of SSB locations (e.g. enable muting patterns to deal with half-duplex constraint).

  - Specification of extension of RACH occasions and periodicities for backhaul RACH resources. w.r.t. access RACH resources, and associated network coordination mechanisms for selection of such parameters (in order to orthogonalize access and BH due to the half-duplex constraints)

  - Specification of mechanisms for resource multiplexing among backhaul and access links. This includes:

    - Specification of semi-static configuration for IAB-node/IAB-donor DU resources in case of TDM operation subject to half-duplex constraint. This shall be forward compatible to allow the support of half-duplex scenarios with FDM and SDM resource sharing among backhaul and access links.

    - Specification of time resource types for the DU’s child links: DL hard, DL soft, UL hard, UL soft, Flexible hard, Flexible soft, Not Available

    - Specification of dynamic indication (L1 signalling) of the availability of soft resources for a child IAB-node DU

    - Specification of required transmission/reception rules for IAB-nodes and associated behaviours regarding time resource utilization as discussed in TR 38.874 clause 7.3.3.

  - Specification of mechanism to support the “case-1” OTA timing alignment.

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2 #105

Agreements: Adaptation layer

- RAN2 confirms that routing and bearer mapping (e.g. mapping of BH RLC channels) are adaptation layer functions

- RAN2 assumes that the TX part of the adaptation layer performs routing and “bearer mapping”, and the RX part of the adaptation layer performs “bearer de-mapping”.

- RAN2 assumes that SDUs are forwarded from the RX part of the adaptation layer to the TX part of the adaptation layer (for the next hop) for packets that are relayed by the IAB node.

- It is FFS how to model adaptation layer protocol entities, e.g. whether separate for DU and MT or not, and how these are configured, i.e. via F1-AP or RRC.

Agreements: L2 configuration

- RAN2 assumes that IAB-donor CU is controlling the setup and modification of all backhaul channels in the IAB network below the IAB-donor.

- RAN2 assumes that a separate BH RLC channel should be setup for each UE DRB with one-to-one bearer mapping.

- RAN2 assumes that for a UE DRB with many-to-one bearer mapping, a BH RLC channel associated with IAB node existing BH RLC channel might be reused as BH RLC channel to forward traffic of this UE DRB (e.g. if the BH RLC channel supports the required UE DRB QoS).

- RAN2 assumes that IAB-donor CU configures the adaptation layer.

- RAN2 assumes that routing is a function of the adaptation layer.

- The details of the routing functionality, e.g. what is configured vs. what is decided locally, is FFS.

Agreements: RLF handling

- RAN2 assumes that there is a RLF-notification at BH RLF, at least to downstream node(s).

- Alternate routes and/or Dual Connectivity could be utilised at recovery at a failure of a BH link.

- Current UE RLF detection and recovery is reused as baseline

- It is FFS, whether other indications are needed, e.g. when link has recovered, or when recovery is in progress.

#### 2.2.2	Remaining Open issues

- Specification of an IAB-node following architecture 1a including [RAN2-led, RAN3]:

  - Routing function on IAB-node to support forwarding across the multi-hop topology based on routing identifier.

  - Hop-by-hop propagation of signalling to support low latency scheduling (e.g. TR 38.874 clause 8.6), BH RLF handling (e.g. TR 38.874 clause 9.7.14-15) and resource coordination across the multi-hop topology (e.g. TR 38.874 clause 7.3.3).

  - UE-bearer to BH RLC-channel mapping and mapping between ingress and egress BH RLC channels functions for support of one-to-one and many-to-one bearer mapping.

- Enhancements to gNB functionality to serve as an IAB-donor following architecture 1a [RAN3, RAN2]

  - Functions on gNB CU-CP for topology, route and resource management [RAN3-led].

  - Support for IP routability to IAB-node (e.g. from CU, OAM) [RAN3-led].

  - Bearer mapping function on gNB DU to map downlink traffic of one or many UE-bearers to a BH RLC-channel [RAN2-led].

- Specification of possible enhancements to E1, F1 and X2/Xn interfaces [RAN3-led, RAN2]:

  - On F1:

    - security protection over the wireless backhaul links.

    - setting up and reconfiguring IAB-nodes and IAB-donor DUs

  - On X2 and Xn, necessary functions to enable DC operation with IAB.

  - On E1, configuration of necessary IAB-specific transport and/or security protection of F1-U.

- Specification of procedures for IAB-node integration and topology adaptation, including [RAN3-led, RAN2]:

  - Procedures for IAB-node integration for SA and NSA modes, including enhancements needed to E-UTRAN for NSA mode.

  - Specification of IAB-node migration underneath the same IAB-donor (with or without a change of IAB-donor DU), and between different IAB-donors. Migration of IAB-node could be network-controlled or could be due to BH RLF.

Support for route redundancy and route selection based on multi-connectivity (e.g. TR 38.874 clause 9.7), leveraging existing NR solutions as well as NR-NR DC, without additional RAN1 work. (see NOTE1).

- Specification of enhancements to L2 wireless transport [RAN2-led, RAN3]:

  - Specification of an adaptation layer above RLC layer. The adaptation layer supports routing across the wireless backhaul and IP as next protocol layer.

  - Extension of LCID space and potentially LCG space to support one-to-one mapping of UE bearers to BH RLC channels. The extension of LCID space and LCG space is applicable only to IAB-nodes.

  - Specification of a flow control mechanism (for DL and, if necessary, for UL) to handle congestion.

  - Specification of mechanisms to enable lossless delivery in hop-by-hop ARQ.

- Specification of signalling for L2 transport and resource management [RAN2-led, RAN3, RAN1]:

  - Specification of RRC and F1-AP procedures and messages for: the setup and release of IAB-nodes; configuration of adaptation layer at the IAB-nodes and IAB-donor DU; configuration of BH RLC channels, QoS information, routing tables, bearer-mappings; configuration of means for network synchronization; and configuration for sharing of time-domain resources among backhaul and access links (see physical layer specification).

  - Specification of enhancements to bearer context setup/release procedures to support flow QoS across multiple hops.

  - Specification of signalling to enable aspects of radio-aware scheduling on IAB-nodes and IAB-donor DUs (e.g. as discussed in TR 38.874 clauses 8.2.4.2-3).

  - Specification of enhancement for uplink resource request procedure and related signalling to enable low latency uplink data scheduling.

  - Specification of BH RLF handling (e.g. downstream BH RLF notification).

## 2.3	RAN3

#### 2.3.1	Agreements

RAN3 #103

Endorsement of CR#0033, Baseline to TS 38.401 v15.4.0, Rel-16, Cat. B (R3-191143)

Agreements: IAB-node setup (R3-191006)

- The donor needs to know that the IAB-node MT is not a normal UE

- SA for IAB node:

AMF includes “IAB Authorized” IE in the INITIAL CONTEXT SETUP REQUEST/CONTEXT MODIFICATION REQUEST messages

- NSA for IAB node:

MME includes “IAB Authorized” IE in the INITIAL CONTEXT SETUP REQUEST/CONTEXT MODIFICATION REQUEST messages

- The eNB should include “IAB Authorized” IE in SgNB ADDITION REQUEST/MODIFICATION REQUEST messages

Agreement: Forwarding/routing

- Routing/forwarding for F1-C and for F1-U should be the same

Endorsement of CR#0063, TS 38.413 v15.2.0, Rel-16, Cat. B (R3-191018)

Endorsement of CR#1303, TS 36.423 v15.4.0, Rel-16, Cat. B (R3-191144)

Endorsement of CR#1661, TS 36.413 v15.4.0, Rel-16, Cat. B (R3-191078)

#### 2.3.2	Remaining Open issues

- Specification of an IAB-node following architecture 1a including [RAN2-led, RAN3]:

  - Routing function on IAB-node to support forwarding across the multi-hop topology based on routing identifier.

  - UE-bearer to BH RLC-channel mapping and mapping between ingress and egress BH RLC channels functions for support of one-to-one and many-to-one bearer mapping.

- Enhancements to gNB functionality to serve as an IAB-donor following architecture 1a [RAN3, RAN2]

  - Functions on gNB CU-CP for topology, route and resource management [RAN3-led].

  - Support for IP routability to IAB-node (e.g. from CU, OAM) [RAN3-led].

  - Bearer mapping function on gNB DU to map downlink traffic of one or many UE-bearers to a BH RLC-channel [RAN2-led].

- Specification of possible enhancements to E1, F1 and X2/Xn interfaces [RAN3-led, RAN2]:

  - On F1:

    - security protection over the wireless backhaul links.

    - setting up and reconfiguring IAB-nodes and IAB-donor DUs

  - On X2 and Xn, necessary functions to enable DC operation with IAB.

  - On E1, configuration of necessary IAB-specific transport and/or security protection of F1-U.

- Specification of procedures for IAB-node integration and topology adaptation, including [RAN3-led, RAN2]:

  - Procedures for IAB-node integration for SA and NSA modes, including enhancements needed to E-UTRAN for NSA mode.

  - Specification of IAB-node migration underneath the same IAB-donor (with or without a change of IAB-donor DU), and between different IAB-donors. Migration of IAB-node could be network-controlled or could be due to BH RLF.

Support for route redundancy and route selection based on multi-connectivity (e.g. TR 38.874 clause 9.7), leveraging existing NR solutions as well as NR-NR DC, without additional RAN1 work.

- Specification of enhancements to L2 wireless transport [RAN2-led, RAN3]:

  - Specification of an adaptation layer above RLC layer. The adaptation layer supports routing across the wireless backhaul and IP as next protocol layer.

  - Extension of LCID space and potentially LCG space to support one-to-one mapping of UE bearers to BH RLC channels. The extension of LCID space and LCG space is applicable only to IAB-nodes.

  - Specification of a flow control mechanism (for DL and, if necessary, for UL) to handle congestion.

  - Specification of mechanisms to enable lossless delivery in hop-by-hop ARQ.

- Specification of signalling for L2 transport and resource management [RAN2-led, RAN3, RAN1]:

  - Specification of RRC and F1-AP procedures and messages for: the setup and release of IAB-nodes; configuration of adaptation layer at the IAB-nodes and IAB-donor DU; configuration of BH RLC channels, QoS information, routing tables, bearer-mappings; configuration of means for network synchronization; and configuration for sharing of time-domain resources among backhaul and access links (see physical layer specification).

  - Specification of an IP address allocation mechanism for the IAB-nodes [RAN3].

  - Specification of enhancements to bearer context setup/release procedures to support flow QoS across multiple hops.

  - Specification of signalling to enable aspects of radio-aware scheduling on IAB-nodes and IAB-donor DUs (e.g. as discussed in TR 38.874 clauses 8.2.4.2-3).

  - Specification of enhancement for uplink resource request procedure and related signalling to enable low latency uplink data scheduling.

  - Specification of BH RLF handling (e.g. downstream BH RLF notification).

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

- Core Part: Specification of RF and RRM requirements [RAN4-led]:

  - Define RF requirements for both backhaul and access links of an IAB-node including requirements for co-existence (e.g. ACLR, ACS). This may include defining a new power class for MT.

  - Define RRM core requirements for both backhaul and access links of IAB node.

  - Define latency requirements for switching between communication over parent backhaul link (i.e. MT) and child backhaul/access links (i.e. DU).

  - Define timing requirements for IAB specific network synchronization. This may include (a) requirement for “case 1” timing (e.g. accuracy of DL transmission timing alignment between an IAB-node and its parent node), and (b) cell phase synchronization accuracy for multi-hop IAB network.

- Performance Part: Define RRM performance requirements.

- Performance Part: Define demodulation performance requirements for both backhaul and access links of an IAB-node. [RAN4]

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1 #AH1901

| R1-1900031 | Initial access procedure for IAB | Huawei, HiSilicon |
| --- | --- | --- |
| R1-1900032 | SSB-based discovery and measurement for IAB | Huawei, HiSilicon |
| R1-1900033 | RACH design for IAB | Huawei, HiSilicon |
| R1-1900034 | Resource multiplexing between backhaul and access in IAB | Huawei, HiSilicon |
| R1-1900035 | DL transmission timing alignment for IAB | Huawei, HiSilicon |
| R1-1900115 | Discussion on extension of SSBs for inter-IAB-node discovery and measurements | vivo |
| R1-1900116 | Discussion on resource multiplexing among backhaul and access links | vivo |
| R1-1900307 | SSB/RMSI periodicity for NR initial access | OPPO |
| R1-1900393 | Discussion on discovery and measurement for IAB network | Lenovo, Motorola Mobility |
| R1-1900394 | Discussion on resource partitioning for IAB network | Lenovo, Motorola Mobility |
| R1-1900403 | Discussion on extensions of SSBs for inter-IAB-node discovery and measurements | CMCC |
| R1-1900404 | Discussions on PRACH enhancements to support NR Backhaul links | CMCC |
| R1-1900427 | Enhancements to support IAB node initial access | AT&T |
| R1-1900428 | Enhancements to support inter-IAB node discovery and measurements | AT&T |
| R1-1900429 | Enhancements to support IAB node RACH | AT&T |
| R1-1900430 | Mechanisms for supporting access and backhaul link multiplexing | AT&T |
| R1-1900431 | Mechanisms for supporting IAB node timing alignment | AT&T |
| R1-1900432 | Mechanisms for resource coordination and radio-aware scheduling for IAB | AT&T |
| R1-1900475 | SSB/RMSI periodicity for NR IAB initial access | Intel Corporation |
| R1-1900476 | SSBs for inter-IAB node discovery and measurement | Intel Corporation |
| R1-1900477 | RACH enhancement for NR IAB | Intel Corporation |
| R1-1900478 | Resource allocation for NR IAB | Intel Corporation |
| R1-1900479 | Mechanisms to support case-1 OTA timing alignment | Intel Corporation |
| R1-1900533 | SSB/RMSI Periodicity for Initial Access in IAB | Samsung |
| R1-1900534 | Support of SSBs for IAB Node Discovery and Measurement | Samsung |
| R1-1900535 | Support of RACH Occasions and Periodicity for IAB RACH | Samsung |
| R1-1900536 | Resource Multiplexing between Backhaul and Access Links | Samsung |
| R1-1900537 | Support of Case 1 OTA Timing Alignment | Samsung |
| R1-1900611 | Discussion on SSB/RMSI periodicity for NR initial access | LG Electronics |
| R1-1900612 | Discussion on SSBs for inter-IAB-node discovery and measurements | LG Electronics |
| R1-1900613 | Discussion on RACH occasions and periodicities for backhaul RACH resources | LG Electronics |
| R1-1900614 | Discussions on resource multiplexing among backhaul and access links | LG Electronics |
| R1-1900615 | Discussions on timing alignment for IAB nodes | LG Electronics |
| R1-1900616 | Discussion on scenario with same cell ID among IAB nodes | LG Electronics |
| R1-1900640 | Discussion on IAB-assumed SSB periodicity in stage-1 initial access | ZTE, Sanechips |
| R1-1900641 | Discussion of SSB for inter-IAB-node discovery and measurements | ZTE, Sanechips |
| R1-1900642 | Discussion on backhaul RACH resources configuration | ZTE, Sanechips |
| R1-1900655 | SSB/RMSI periodicity for NR initial access assumed by IAB-node | Nokia, Nokia Shanghai Bell |
| R1-1900656 | Extension of SSBs for IAB discovery and measurements | Nokia, Nokia Shanghai Bell |
| R1-1900657 | Enhancements on RACH for IAB | Nokia, Nokia Shanghai Bell |
| R1-1900658 | Mechanisms for resource multiplexing among backhaul and access links | Nokia, Nokia Shanghai Bell |
| R1-1900659 | IAB Case #1 timing | Nokia, Nokia Shanghai Bell |
| R1-1900662 | Discussion on IAB | NEC |
| R1-1900725 | SSB periodicity for NR initial access | Ericsson |
| R1-1900726 | SSB-based IAB-node discovery and measurements | Ericsson |
| R1-1900727 | IAB RACH resources | Ericsson |
| R1-1900732 | IAB semi-static configuration for DU resources | Ericsson |
| R1-1900733 | IAB OTA timing alignment | Ericsson |
| R1-1900765 | Resource multiplexing among backhaul and access links | ZTE, Sanechips |
| R1-1900773 | Discussion on OTA timing alignment mechanism | ZTE, Sanechips |
| R1-1900790 | On SSB and RMSI for IAB Initial Access in NR | InterDigital, Inc. |
| R1-1900791 | On RACH Resources for IAB Random Access in NR | InterDigital, Inc. |
| R1-1900792 | On Integrated Access and Backhaul Link Enhancement in NR | InterDigital, Inc. |
| R1-1900830 | L1 signaling supports resource management for IAB! | Sharp |
| R1-1900878 | Proposal for SSB/RMSI periodicity | Qualcomm Incorporated |
| R1-1900879 | Proposal for SSB extensions for IAB | Qualcomm Incorporated |
| R1-1900880 | Proposal for RACH extension for IAB | Qualcomm Incorporated |
| R1-1900881 | IAB resource management framework | Qualcomm Incorporated |
| R1-1900882 | IAB node synchronization framework | Qualcomm Incorporated |
| R1-1900883 | On the IAB scope | Qualcomm Incorporated |
| R1-1900957 | SSB/RMSI periodicity for IAB initial access | NTT DOCOMO, INC. |
| R1-1900958 | Extensions of SSBs for inter-IAB-node discovery and measurements | NTT DOCOMO, INC. |
| R1-1900959 | Extension of RACH occasions and periodicities for backhaul RACH resources | NTT DOCOMO, INC. |
| R1-1900960 | Mechanisms for resource multiplexing among backhaul and access links | NTT DOCOMO, INC. |
| R1-1900961 | Mechanism to support the “case-1” OTA timing alignment | NTT DOCOMO, INC. |
| R1-1901042 | SSB/RMSI Periodicity for Initial Access in IAB | Samsung |
| R1-1901043 | Support of SSBs for IAB Node Discovery and Measurement | Samsung |
| R1-1901044 | Support of RACH Occasions and Periodicity for IAB RACH | Samsung |
| R1-1901045 | Resource Multiplexing between Backhaul and Access Links | Samsung |
| R1-1901046 | Support of Case 1 OTA Timing Alignment | Samsung |
| R1-1901125 | Discussion on extensions of SSBs for inter-IAB-node discovery and measurements | CAICT |
| R1-1901126 | Discussion on RACH occassions and periodicities for backhaul RACH resources | CAICT |
| R1-1901264 | On backhaul link radio link failure handling for IAB | Huawei, HiSilicon |
| R1-1901367 | Summary of section 7.2.3.1 – SSB periodicity for IAB nodes | Qualcomm |
| R1-1901369 | Summary of section 7.2.3.2 – SSB extensions for IAB | Qualcomm |
| R1-1901370 | Summary of section 7.2.3.3 – RACH extensions for IAB | Qualcomm |
| R1-1901390 | FL Summary of 7.2.3.4 - IAB Resource Allocation | AT&T |
| R1-1901391 | FL Summary of 7.2.3.5 - IAB Timing Alignment | AT&T |
| R1-1901400 | Summary of section 7.2.3.1 – SSB periodicity for IAB nodes | Qualcomm |
| R1-1901417 | FL Summary of 7.2.3.5 - IAB Timing Alignment | AT&T |
| R1-1901455 | FL Summary of 7.2.3.4 - IAB Resource Allocation | AT&T |

RAN1 #96

| R1-1901531 | SSB-based discovery and measurement for IAB | Huawei, HiSilicon |
| --- | --- | --- |
| R1-1901532 | RACH design for IAB | Huawei, HiSilicon |
| R1-1901533 | Resource multiplexing between backhaul and access in IAB | Huawei, HiSilicon |
| R1-1901534 | DL transmission timing alignment for IAB | Huawei, HiSilicon |
| R1-1901535 | On backhaul link radio link failure handling for IAB | Huawei, HiSilicon |
| R1-1901680 | Discussion on extension of SSBs for inter-IAB-node discovery and measurements | vivo |
| R1-1901681 | Discussion on resource multiplexing among backhaul and access links | vivo |
| R1-1901889 | Enhancements to support IAB node initial access | AT&T |
| R1-1901890 | Enhancements to support inter-IAB node discovery and measurements | AT&T |
| R1-1901891 | Enhancements to support IAB node RACH | AT&T |
| R1-1901892 | Mechanisms for supporting access and backhaul link multiplexing | AT&T |
| R1-1901893 | Mechanisms for supporting IAB node timing alignment | AT&T |
| R1-1901894 | Mechanisms for resource coordination and radio-aware scheduling for IAB | AT&T |
| R1-1902077 | Discussion on SSB/RMSI periodicity for NR initial access | LG Electronics |
| R1-1902078 | Discussion on SSBs for inter-IAB-node discovery and measurements | LG Electronics |
| R1-1902079 | Discussion on RACH occasions and periodicities for backhaul RACH resources | LG Electronics |
| R1-1902080 | Discussions on resource multiplexing among backhaul and access links | LG Electronics |
| R1-1902081 | Discussions on timing alignment for IAB nodes | LG Electronics |
| R1-1902082 | Discussion on scenario with same cell ID among IAB nodes | LG Electronics |
| R1-1902141 | Discussion on IAB-assumed SSB periodicity in stage-1 initial access | ZTE, Sanechips |
| R1-1902142 | Discussion on backhaul RACH resources configuration | ZTE, Sanechips |
| R1-1902143 | Resource multiplexing among backhaul and access links | ZTE, Sanechips |
| R1-1902144 | Discussion on OTA timing alignment mechanism | ZTE, Sanechips |
| R1-1902154 | Discussion on discovery and measurement for IAB network | Lenovo, Motorola Mobility |
| R1-1902155 | Discussion on resource partitioning for IAB network | Lenovo, Motorola Mobility |
| R1-1902197 | Discussion of SSB for inter-IAB-node discovery and measurements | ZTE, Sanechips |
| R1-1902268 | SSB/RMSI Periodicity for Initial Access in IAB | Samsung |
| R1-1902269 | Support of SSBs for IAB Node Discovery and Measurement | Samsung |
| R1-1902270 | Support of RACH Occasions and Periodicity for IAB RACH | Samsung |
| R1-1902271 | Resource Multiplexing between Backhaul and Access Links | Samsung |
| R1-1902272 | Support of Case 1 OTA Timing Alignment | Samsung |
| R1-1902328 | Discussion on extensions of SSBs for inter-IAB-node discovery and measurements | CMCC |
| R1-1902329 | Discussions on PRACH enhancements to support NR Backhaul links | CMCC |
| R1-1902409 | SSB periodicity at initial access 7.2.3.1 | Ericsson |
| R1-1902410 | SSB-based IAB node discovery and measurement | Ericsson |
| R1-1902411 | IAB Enhancements for RACH resources | Ericsson |
| R1-1902412 | IAB resource configuration and assignment | Ericsson |
| R1-1902413 | IAB OTA Timing Alignment | Ericsson |
| R1-1902415 | Resource for IAB up-stream (MT) transmission | Ericsson |
| R1-1902430 | SSB periodicity for IAB initial access | Nokia, Nokia Shanghai Bell |
| R1-1902431 | SSB TX/RX for IAB stage 2 discovery | Nokia, Nokia Shanghai Bell |
| R1-1902432 | Enhancements on RACH for IAB | Nokia, Nokia Shanghai Bell |
| R1-1902433 | Mechanisms for resource multiplexing among backhaul and access links | Nokia, Nokia Shanghai Bell |
| R1-1902434 | IAB Case #1 timing | Nokia, Nokia Shanghai Bell |
| R1-1902476 | SSB/RMSI periodicity for NR initial access | Intel Corporation |
| R1-1902477 | SSBs for inter-IAB-node discovery and measurements | Intel Corporation |
| R1-1902478 | RACH enhancements for NR IAB | Intel Corporation |
| R1-1902479 | Mechanisms for resource multiplexing among backhaul and access links | Intel Corporation |
| R1-1902480 | Mechanism to support the "case-1" OTA timing alignment | Intel Corporation |
| R1-1902685 | Discussion on IAB | NEC |
| R1-1902743 | SSB/RMSI periodicity for NR initial access | OPPO |
| R1-1902744 | Extension of RACH configuration for backhaul link | OPPO |
| R1-1902793 | SSB/RMSI periodicity for IAB initial access | NTT DOCOMO, INC. |
| R1-1902794 | Extensions of SSBs for inter-IAB-node discovery and measurements | NTT DOCOMO, INC. |
| R1-1902795 | Extension of RACH occasions and periodicities for backhaul RACH resources | NTT DOCOMO, INC. |
| R1-1902796 | Mechanisms for resource multiplexing among backhaul and access links | NTT DOCOMO, INC. |
| R1-1902797 | Mechanism to support the “case-1” OTA timing alignment | NTT DOCOMO, INC. |
| R1-1902921 | Discussion on extensions of SSBs for inter-IAB-node discovery and measurements | CAICT |
| R1-1902922 | Discussion on RACH occassions and periodicities for backhaul RACH resources | CAICT |
| R1-1902990 | Proposal for SSB extensions for IAB | Qualcomm Incorporated |
| R1-1902991 | Proposal for RACH extension for IAB | Qualcomm Incorporated |
| R1-1902992 | IAB resource management framework | Qualcomm Incorporated |
| R1-1902993 | IAB node synchronization framework | Qualcomm Incorporated |
| R1-1903065 | Remaining issues on SSB periodicity for IAB | Huawei, HiSilicon |
| R1-1903226 | IAB resource configuration and assignment | Ericsson |
| R1-1903573 | Summary for 7.2.3.1 SSB periodicity for IAB NR initial acces | Qualcomm |
| R1-1903574 | Summary for 7.2.3.2 Extensions of SSBs for IAB | Qualcomm |
| R1-1903575 | Summary for 7.2.3.3 Extensions of RACH for IAB | Qualcomm |
| R1-1903582 | Summary of 7.2.3.4 Mechanisms for resource multiplexing among backhaul and access links | AT&T |
| R1-1903583 | Summary of 7.2.3.5 Mechanism to support the “case-1” OTA timing alignment | AT&T |
| R1-1903692 | [Draft] LS on initial access SSB periodicity assumption for IAB MT | Qualcomm |
| R1-1903693 | [Draft] LS on OTA timing alignment for IAB | Huawei |
| R1-1903809 | LS on initial access SSB periodicity assumption for IAB MT | Qualcomm |
| R1-1903810 | LS on OTA timing alignment for IAB | RAN1, Huawei |

RAN2 #105

| R2-1900809 | IAB WI workplan | Qualcomm Inc |
| --- | --- | --- |
| R2-1900810 | IAB baseline for 38300 | Qualcomm Inc |
| R2-1901192 | On the impact of IAB work on Core Network procedures and signalling | Samsung R&D Institute UK |
| R2-1901193 | Draft LS on IAB impact to CN | Samsung R&D Institute UK |
| R2-1901187 | Some considerations on LCID space extension | Samsung R&D Institute UK |
| R2-1901198 | On pre-emptive SR and BSR in IAB networks | Samsung R&D Institute UK |
| R2-1901191 | Further details on the LCID space extension | Samsung R&D Institute UK |
| R2-1901196 | Solutions for reliable IAB end-to-end transmission for the hop-by-hop RLC ARQ case | Samsung R&D Institute UK |
| R2-1901159 | Fundamental issues for adaptation layer design | Samsung R&D Institute UK |
| R2-1901258 | Discussion on MAC subheader structure for IAB | Nokia, Nokia Shanghai Bell |
| R2-1901065 | Overview of flow control solutions | Samsung |
| R2-1900902 | QoS framework in IAB network | Samsung |
| R2-1900918 | Consideration of multi-hop RLC ARQ | Kyocera |
| R2-1901030 | Discussion on SR/BSR triggering in multi-hop IAB network | ASUSTeK |
| R2-1900811 | Functional view of IAB adaptation layer | Qualcomm Inc |
| R2-1900925 | Consideration of low latency scheduling for multi-hop backhauling | Kyocera |
| R2-1900977 | Discussion on IAB bearer-to-RLC-channel mapping | ITRI |
| R2-1900978 | Function of Adaptation Layer to cope with multi-hop RLC ARQ | ITRI |
| R2-1900979 | RLC Enhancement in IAB | ITRI |
| R2-1900954 | IAB scheduling | Potevio |
| R2-1900943 | Radio-aware scheduling in IAB | AT&T |
| R2-1900944 | Route selection in case of route redundancy | AT&T |
| R2-1900946 | A better name for the IAB Adaptation layer | AT&T |
| R2-1900221 | Consideration of flow control | CATT |
| R2-1900310 | Triggering condition of Pre-BSR | vivo |
| R2-1900243 | Common design and QoS Handling for IAB | CATT |
| R2-1900626 | Adaptation layer contents and configuration | Nokia, Nokia Shanghai Bell |
| R2-1900565 | Discussion on BH RLC channel configuration in IAB network | ShenZhen Zhongxing Shitong |
| R2-1900566 | Discussion on flow control in IAB | ShenZhen Zhongxing Shitong |
| R2-1900567 | Consideration on UL lossless delivery in IAB | ShenZhen Zhongxing Shitong |
| R2-1900568 | Discussion on low latency scheduling in IAB | ShenZhen Zhongxing Shitong |
| R2-1900561 | Discussion on IAB UP protocol stack and adaptation layer | ShenZhen Zhongxing Shitong |
| R2-1900562 | Consideration on user plane one-to-one bearer mapping | ShenZhen Zhongxing Shitong |
| R2-1900563 | Consideration on user plane many-to-one bearer mapping | ShenZhen Zhongxing Shitong |
| R2-1901480 | IAB protocol stack options | Nokia, Nokia Shanghai Bell |
| R2-1901387 | Flow Control in IAB networks | Ericsson |
| R2-1901389 | Adaptation layer for IAB | Intel Corporation |
| R2-1901391 | Uplink latency reduction | Intel Corporation |
| R2-1901393 | Flow control mechanism for DL | Intel Corporation |
| R2-1901394 | Flow control mechanism for UL | Intel Corporation |
| R2-1901395 | Need for Loss less delivery with hop-by-hop ARQ | Intel Corporation |
| R2-1901396 | Handling of fairness in IAB | Intel Corporation |
| R2-1901405 | Issues to be decided in IAB Stage2 design | KDDI Corporation |
| R2-1901406 | Consideration on routing management procedure | KDDI Corporation |
| R2-1901418 | Security aspects and considerations for IAB | Nokia, Nokia Shanghai Bell |
| R2-1901419 | IAB user plane aspects | Sony |
| R2-1901328 | Uplink scheduling in IAB networks: BSR, SR, and LCP | Ericsson |
| R2-1901329 | LCID and LCG extension for IAB nodes | Ericsson |
| R2-1901330 | Time stamping for User Plane Latency Control | Ericsson |
| R2-1901646 | [Draft] LS on destination address for IAB routing | Futurewei Technologies |
| R2-1901639 | Routing function and configuration | Futurewei Technologies |
| R2-1901801 | Security protection of F1 over wireless backhaul | Futurewei Technologies |
| R2-1901824 | General RLC impacts | Huawei, HiSilicon, CMCC |
| R2-1901825 | Lossless data delivery | Huawei, HiSilicon |
| R2-1901826 | Bearer mapping in IAB network | Huawei, HiSilicon |
| R2-1901827 | Remaining issues of the UP protocol architecture | Huawei, HiSilicon |
| R2-1901828 | Support of traffic terminated at MT of IAB node | Huawei, HiSilicon |
| R2-1901829 | Adaptation layer modelling | Huawei, HiSilicon |
| R2-1901830 | Adaptation layer header format design | Huawei, HiSilicon |
| R2-1901831 | Header compression in adaptation layer | Huawei, HiSilicon |
| R2-1901832 | LCID extension for IAB backhaul link | Huawei, HiSilicon |
| R2-1901833 | LCG space extension for IAB backhaul link | Huawei, HiSilicon |
| R2-1901834 | Uplink scheduling enhancement | Huawei, HiSilicon |
| R2-1901835 | Flow control for IAB networks | Huawei, HiSilicon |
| R2-1901836 | Multiple connectivity in IAB networks | Huawei, HiSilicon |
| R2-1901549 | Queue Management vs Flow Control for Congestion Handling | Sequans Communications |
| R2-1902021 | Consideration on LCID and LCG space extension | LG Electronics Inc. |
| R2-1902015 | QoS based many-to-one bearer mapping | LG Electronics Inc. |
| R2-1902016 | Downlink flow control mechanism in IAB | LG Electronics Inc. |
| R2-1902017 | Consideration on the adaptation layer | LG Electronics Inc. |
| R2-1902018 | BH RLF reporting to IAB donor node | LG Electronics Inc. |
| R2-1902019 | Consideration on RLC channel type for MT’s own traffic | LG Electronics Inc. |
| R2-1902023 | Consideration on uplink data congestion handling | LG Electronics Inc. |
| R2-1902022 | SN length for multi-hop RLC ARQ | LG Electronics Inc. |
| R2-1902164 | LCG based UL grant | LG Electronics Inc. |
| R2-1902165 | E2E reliability in hop-by-hop RLC ARQ | LG Electronics |
| R2-1902167 | Enhancement for low latency uplink scheduling | LG Electronics |
| R2-1902130 | IAB bearer mapping decision and configuration | Futurewei Technologies |
| R2-1902128 | Discussion on flow control | ITL |
| R2-1902307 | Radio-aware scheduling in IAB | AT&T |
| R2-1902184 | Overview on control signaling transmission in IAB | Samsung |
| R2-1902170 | Discussion on route selection in IAB | LG Electronics |
| R2-1902188 | Backhaul Link Failure Recovery | LG Electronics France |
| R2-1902189 | Detection of BH Failure | LG Electronics France |
| R2-1902192 | Tansport of BH RLF notification | LG Electronics France |
| R2-1902154 | Access Control for IAB node | LG Electronics Inc |
| R2-1901725 | Topology in IAB system | Lenovo, Motorola Mobility |
| R2-1902020 | LCG based UL grant | LG Electronics Inc. |
| R2-1902063 | RLF handling in intermediate IAB nodes | Samsung R&D Institute UK |
| R2-1902065 | RRC signaling structure for IAB | Futurewei Technologies |
| R2-1901996 | Down selection on CP protocol options | Futurewei Technologies |
| R2-1902024 | QoS and Fairness enforcement in IAB networks | Futurewei Technologies |
| R2-1901909 | Bearer mapping for control plane signalling | Futurewei Technologies |
| R2-1901636 | Supporting of using SA IAB in NSA network | Futurewei Technologies |
| R2-1901717 | Downstream notification of BH RLF in architecture 1a | SHARP Corporation |
| R2-1901718 | Access restrictions (barring) in IAB | SHARP Corporation |
| R2-1901719 | Cell Selection and Cell Reselection procedures of IAB | Futurewei Technologies |
| R2-1901647 | IAB RRC Connection Establishment | Futurewei Technologies |
| R2-1901638 | Discussion on IAB NSA procedure and SRB transmission | Futurewei Technologies |
| R2-1901728 | IP address management for IAB nodes | Futurewei Technologies |
| R2-1901771 | Congestion reporting and handling for IAB networks | Futurewei Technologies |
| R2-1901634 | Backhaul RLF Recovery | Futurewei Technologies |
| R2-1901323 | QoS Mapping to Backhaul Bearers in IAB Network | Ericsson |
| R2-1901324 | Backhaul Channel Setup and Modification Procedure for IAB Networks | Ericsson |
| R2-1901325 | Adaptation Layer Header Content | Ericsson |
| R2-1901326 | Packets Forwarding in IAB Networks | Ericsson |
| R2-1901327 | IAB Node Integration Procedure | Ericsson |
| R2-1901420 | IAB control Plane aspects | Sony |
| R2-1901421 | IAB System information handling | Sony |
| R2-1901392 | Backhaul RLF handling | Intel Corporation |
| R2-1901390 | Configuration of Bearer Mapping for IAB | Intel Corporation |
| R2-1901382 | Support for LTE deployment at IAB node sites | Ericsson |
| R2-1901383 | IP address assignment for IAB nodes | Ericsson |
| R2-1901384 | Minimizing CN functionalities for IAB networks | Ericsson |
| R2-1901385 | IAB node authentication and authorization | Ericsson |
| R2-1901386 | Handling of link failures in IAB networks | Ericsson |
| R2-1901497 | IAB node in EN-DC | Nokia, Nokia Shanghai Bell |
| R2-1900569 | Discussion on IAB CP protocol stack | ShenZhen Zhongxing Shitong |
| R2-1900570 | Discussion on control plane bearer mapping | ShenZhen Zhongxing Shitong |
| R2-1900571 | Discussion on IAB BH RLF handling | ShenZhen Zhongxing Shitong |
| R2-1900572 | Consideration on routing in IAB | ShenZhen Zhongxing Shitong |
| R2-1900627 | Backhaul link RLF handling | Nokia, Nokia Shanghai Bell |
| R2-1900628 | Route redundancy support with multi-connectivity | Nokia, Nokia Shanghai Bell |
| R2-1900242 | Route Adaptation upon Backhaul Failure | CATT |
| R2-1900945 | Control plane signaling to support IAB routing | AT&T |
| R2-1900812 | Backhaul RLF notification for IAB | Qualcomm Inc |
| R2-1901031 | On topology adaptation upon backhaul-link-failure recovery | ASUSTeK |
| R2-1900919 | Consideration of topology adaptation upon BH RLF | Kyocera |
| R2-1900903 | Overview on IAB node setup (SA) | Samsung |
| R2-1900904 | Overview on IAB node setup (NSA) | Samsung |
| R2-1901059 | IAB RRC state machine | Samsung |
| R2-1901023 | Consideration on RLF handling in IAB | Hisense |
| R2-1901068 | RLF in backhaul link | Lenovo, Motorola Mobility |
| R2-1901067 | Security framework for the NR integrated access backhaul | Samsung |
| R2-1901582 | Overview of RAN1 impacts | Futurewei Technologies |

RAN3 #103

| R3-190187 | IAB WI workplan | Qualcomm Incorporated (Rapporteur) |
| --- | --- | --- |
| R3-190188 | IAB baseline for 38401 | Qualcomm Incorporated |
| R3-190189 | IAB initial parent-node selection | Qualcomm Incorporated |
| R3-190190 | IAB DL flow control using NUPP | Qualcomm Incorporated |
| R3-190243 | Discussion on Backhaul RLF recovery | CATT |
| R3-190246 | Flow and congestion control for IAB | AT&T |
| R3-190247 | Route redundancy for IAB Nodes in NSA mode | AT&T |
| R3-190248 | Procedures for NSA mode node integration | AT&T |
| R3-190350 | IAB Aspects in TS 38.401 | Ericsson |
| R3-190351 | CR TS 38.401 IAB Aspects | Ericsson |
| R3-190352 | IP Address Assignment for IAB Nodes | Ericsson |
| R3-190353 | QoS Mapping to Backhaul Bearers in IAB Networks | Ericsson |
| R3-190354 | IAB node Integration Procedure | Ericsson |
| R3-190355 | CR TS 38.401 IAB node Integration Procedure | Ericsson |
| R3-190356 | Backhaul Channel Setup and Modification Procedure for IAB Networks | Ericsson |
| R3-190357 | CR TS 38.401 Backhaul Channel Setup and Modification Procedure for IAB Networks | Ericsson |
| R3-190358 | Flow Control in IAB Networks | Ericsson |
| R3-190359 | IAB Node Authentication and Authorization | Ericsson |
| R3-190360 | Support for LTE Deployment at IAB Node Sites | Ericsson |
| R3-190361 | Minimizing CN Functionalities for IAB Networks | Ericsson |
| R3-190362 | Packet Forwarding in IAB Networks | Ericsson |
| R3-190363 | Recovery From Link Failure in IAB | Ericsson |
| R3-190364 | CR TS 38.401 Recovery From Link Failure in IAB | Ericsson |
| R3-190407 | IAB Node Integration Procedure | Huawei |
| R3-190410 | Adaptation Layer Configuration | Huawei |
| R3-190414 | IP Address Management of IAB Node | Huawei |
| R3-190415 | End-to-end flow control for congestion handling | LG Electronics Inc. |
| R3-190417 | QoS Management and Configuration of IAB Node | Huawei |
| R3-190418 | IAB-node migration between different IAB-donors | LG Electronics Inc. |
| R3-190420 | Support of Flow Control for IAB Network | Huawei |
| R3-190421 | Remaining issues of the UP protocol architecture | Huawei |
| R3-190434 | Overview on IAB node setup (SA) | Samsung |
| R3-190435 | Overview of flow control solutions | Samsung |
| R3-190436 | Overview on IAB node setup (NSA) | Samsung |
| R3-190437 | QoS framework in IAB network | Samsung |
| R3-190438 | Overview on IP routing in IAB network | Samsung |
| R3-190439 | Overview on IAB node reestablishment | Samsung |
| R3-190440 | Overview on IAB node mitigation | Samsung |
| R3-190458 | Security protection of F1 over wireless backhaul | Huawei |
| R3-190459 | Support of multiple connectivity in IAB network | Huawei |
| R3-190488 | Issues to be decided in IAB Stage2 design | KDDI Corporation |
| R3-190490 | Consideration on routing management procedure | KDDI Corporation |
| R3-190492 | RAN3 Impact Analysis of IAB Usage in NSA network | Huawei |
| R3-190493 | Routing Address Design and Configuration | Huawei |
| R3-190494 | Routing Table Design and Configuration | Huawei |
| R3-190495 | Intra-Donor CU Topology Adaptation | Huawei |
| R3-190496 | Inter-Donor CU Topology Adaptation | Huawei |
| R3-190497 | Backhaul RLF Recovery | Huawei |
| R3-190498 | Analysis on IP address management | Nokia, Nokia Shanghai Bell |
| R3-190499 | IAB Integration | Nokia, Nokia Shanghai Bell |
| R3-190500 | Bearer Mapping in Donor-DU and IAB node | Huawei |
| R3-190501 | Congestion reporting and handling for IAB networks | Huawei |
| R3-190502 | Resource coordination between multi-hop BH links | Huawei |
| R3-190503 | Discussion on flow control in IAB | ZTE Corporation |
| R3-190541 | Discussion on IAB BH RLF handling | ZTE Corporation |
| R3-190542 | Discussion on network-controlled IAB migration handling | ZTE Corporation |
| R3-190543 | Control plane signaling delivery in NSA deployment | ZTE Corporation |
| R3-190544 | Consideration on IP Address Allocation in IAB | ZTE Corporation |
| R3-190545 | Consideration on routing in IAB | ZTE Corporation |
| R3-190546 | Discussion on BH RLC channel configuration in IAB network | ZTE Corporation |
| R3-190547 | Integration of IAB-node | Huawei |
| R3-190551 | Introduction of overall architecture for IAB | Huawei |
| R3-190562 | Intra Donor-DU topology adaptation procedure | Huawei |
| R3-190579 | IAB protocol stack options | Nokia, Nokia Shanghai Bell |
| R3-190580 | Security aspects and considerations for IAB | Nokia, Nokia Shanghai Bell |
| R3-190850 | Security protection for Wireless backhaul in IAB | TCL Communication Ltd. |

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template