3GPP TSG-RAN Meeting #83	RP-190187

Shenzhen, China, 18th - 21st Mar 2018

Agenda Item:	9.3.12

Source:	Charter Communications Inc.

Title:	Use-cases for FS_NR_beyond_52GHz

Document for: Discussion and Agreement

# Introduction

This contribution aims to identify use-cases which could be employed when using frequencies >52.6GHz.

# Discussion

The following objectives together with work plan identified in [1] were approved at RAN#82:

This study item includes the following objectives for frequency range between 52.6 GHz and 114.25 GHz:

- Survey on global spectrum availability and regulatory requirements (including channelization and licensing regimes)

  - For 60GHz bands, TR 38.805 can be a reference.

- Identify potential use cases and deployment scenarios

- Identify NR design requirements and considerations on top of regulatory requirements

Note: Work plan

- RAN#81: TR skeleton

- RAN#82-85: Objective 1

- RAN#83-85: Objective 2 and 3

- RAN#86: Conclude the SI and approve the TR

In this contribution, the authors suggest two potential use-cases for when frequencies > 52.6GHz are employed.

## Wide Area Network Applications

Under this set of use-cases, the following is envisioned:

- Outdoor fixed broadband network

Today, wireless ethernet networks (ala 802.11 based) offer broadband connectivity to confined outdoor venues such as university campuses, stadiums, malls etc. When operating in ƒ>52.6GHz NR/NR-U should offer feature parity (if not exceed) to such wireless ethernet based networks to serve either as replacement or complimentary network. Here, the expectation is for a single node (eg gNB) to offer broadband service (in order of ~>= 20Gbps) to a few fixed (non-mobile) devices (e.g. UE) over a distance of ~300-500m. The resulting network could either be public or non-public. Both point to point, and point-to-multipoint transmissions should be supported.

Figure 1 PtP and PtMP outdoor fixed broadband transmissions

- Outdoor Backhaul/Relay network

Where a wired connection is not a feasible option, various wireless technologies e.g. PtP microwave links are used to offer backhaul / relay service. 3GPP Rel-16 offers backhaul / relay service under the IAB umbrella. However, this service is presently limited to ƒ<=52.6GHz. NR/NR-U operating in ƒ>52.6GHz NR/NR-U should extend such capabilities to this higher frequency range. Here, the expectation is for a single node (eg gNB) to offer broadband services to a few devices (e.g. UE) or to offer relay services (up to certain hops) to a few devices (e.g. other nodes) or a combination thereof. The expected coverage range is ~300-500m.

## Industrial IoT (IIoT) Applications

This set of use-cases is expected to target indoor industrial automation applications over a distance of ~50-100m, with speed <= 5km/h, and other characteristics such as latency, packet error rate etc in accordance with performance requirements mentioned in Annex E of [2].

Figure 2 IIoT factory robots

# Conclusions

It is proposed to adopt Section 2.1 and 2.2 as TPs to TR 38.807 [3].

# References

- RP-182861, “Revised SID on Study on NR beyond 52.6GHz”

- TS 22.261, “Service requirements for next generation new services and markets”

- TR 38.807, “Study on NR beyond 52.6 GHz”