3GPP TSG RAN Meeting #83																			RP-190197

Shenzhen, China, March 18-21, 2019

Source: 	Intel Corporation

Title:	Discussion on NR MIMO maintenance work

Agenda item:	9.4.1

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss motivations for follow up RAN1 maintenance work in Rel-16 to address unresolved issues from Rel-15 due to late stage of the specification and potential backward compatibility issues.

# Discussion

During Rel-15 NR maintenance work in RAN1 [1] several “essential” issues were not resolved due to late stage of the Rel-15 specification and potentially non-backwards compatible changes. Although some of the corresponding modifications may not be critical for operation of the first wave of Rel-15 NR networks relying on the basic functionality, the existing ambiguities in the specification and unresolved issues may prevent from implementation and usage of the more advanced capabilities of UE and gNB in the practical networks. Moreover, considering future evolution of the NR systems, keeping some functionality issues unresolved in NR may not be appropriate due to potential impact on the new NR features to be introduced in the future releases.

In particular the description of the following Rel-15 aspects was not completely addressed in the current RAN1 specifications according to our understanding:

Default QCL assumption for aperiodic CSI-RS triggered with scheduling offset below threshold

NR supports beam management procedure based on aperiodic CSI-RS (A-CSI-RS). In FR2 A-CSI-RS can be scheduled by gNB with time offset which can be below or above beam switching threshold indicated by the UE capability using FG 2-28.

| 2-28 | A-CSI-RS beam switching timing | 1. Minimum time between the DCI triggering of AP-CSI-RS and aperiodic CSI-RS transmission shall be at least KBi symbols. (Symbols measured from last symbol containing the indication to first symbol of CSI-RS), where i is the index of SCS, l=1,2 corresponding to 60,120 kHz SCS. | Optional with capability signaling Only applicable to FR2 Candidate values: {14, 28, 48, 224, 336} |
| --- | --- | --- | --- |

According to current specification, if the A-CSI-RS scheduling offset is below threshold, UE assumption on the default Rx beam (default QCL assumption) is unspecified in Rel-15, introducing ambiguity between gNB and UE and, therefore, preventing from such scheduling decision in the practical NR systems. On the other hand, A-CSI-RS triggering with scheduling offset below threshold is expected to be useful, comparing to A-CSI-RS triggering with offsets above thresholds that can be relatively large (e.g. 336 OFDM symbols) according to FG 2-28 description above.

Simultaneous transmission and reception in FR2

The issue of simultaneous transmission and reception was extensively discussed in RAN1 and RAN4 during Rel-15 timeframe. The issue occurs in FR2, due to a single analog Rx beamforming capability at the UE allowing reception using one Rx beam at time. With such constraint simultaneous processing of the different physical channels or reference signals scheduled with different QCL assumptions (targeting different and conflicting Rx beamforming at the UE) on the same or different CCs is not feasible. The issue of simultaneous processing of reference signal used for RRM and BM measurements with physical channels or reference signal configured for other purpose was clearly addressed in RAN4 through scheduling restrictions and prioritization of the measurements.

Simultaneous processing for some combinations was also addressed in RAN1 (see table below)

| Group-1 | Group-2 | UE behavior |
| --- | --- | --- |
| PDSCH scheduled before timeDurationForQCL | PDCCH (CORESET) | 38.214: Prioritize reception of PDCCH (CORESET) if QCL Type D is different but PDSCH behavior not specified. Also applies to intra-band CA |
| CSI-RS with repetition=ON | PDCCH (CORESET) | 38.214: Scheduling not allowed. Also applies to intra-band CA |
| All other CSI-RS without repetition=ON (includes TRS, CSI and repetition=OFF) | PDCCH (CORESET) | 38.214: UE may assume same QCL-Type-D. Also applies to intra-band CA [NW ensures same QCL] |
| CSI-RS for L1-RSRP | SSB | 38.214: UE may assume same QCL-Type-D |
| A-CSI-RS for CSI (without trs-Info and without repetition) scheduled before beamSwitchTiming | PDSCH scheduled after timeDurationForQCL, A-CSI scheduled after beamSwitchTiming | 38.214: UE applies QCL-Type-D of Group-2 to receive |
| PDCCH (CORESET) | PDCCH (CORESET) | 38.213: Select monitoring search spaces in the cell with lowest index, then select CSS over USS, then lowest search space id. Only search spaces with same QCL-Type-D required to be monitored |
| PDSCH (DM-RS) | SSB | 38.214: UE may assume same QCL-Type-D for DM-RS and SSB |

However, several remaining combinations identified by RAN1 for specification [2] were not finalized due to lack of time and late stage of Rel-15 work

- PSDSCH+PDSCH

- PDSCH scheduled after timeDurationForQCL + PDCCH

- A-CSI-RS scheduled after beamSwitchTiming or periodic, semi-persistent CSI-RS + PDSCH scheduled before Threshold-Sched-Offset

- SRS+SRS (including SRS of different types)

Although in the first wave of Rel-15 systems, simultaneous transmission and reception of the physical channels and reference signals may not occur due to use of a single transmit beamforming at gNB, the issue is likely to become a real life in the future NR deployments supporting multiple Tx beamforming capability at time at the gNB. Having UE behavior undefined in RAN1 specification for the remaining cases could create inefficiencies due to different assumptions at gNB and UE regarding processing priorities in the above combinations.

QCL assumption of PDCCH/PDSCH/PUSCH after beam failure recovery (BFR)

According to BFR procedure in case of beam failure after transmitting PRACH, UE starts to monitor CORESET-BFR based on new Tx beam identified from BFR. However according to specification, UE is also required to monitor previous CORESET based on previous Tx beam (TCI state) as well, where previous Tx beam (TCI state) for PDSCH is still valid according to the current specification irrespective of BFR. However such requirement clearly exceeds UE capability, e.g., when UE is supporting tracking of a single Tx beam (single active TCI state FG 2-4).

Somewhat similar issue occurs in the uplink, where UE is required to reset only the PUCCH beam (spatial relation info) after BFR based on new Tx beam identified from BFR. This means that for PUSCH scheduled by DCI format 0_1, UE is still required to maintain previous Tx beam. If UE is capable of supporting single Tx beam (single active spatial relation info FG 2-60), current specification exceeds signaled UE capability.

Spatial relation info assumption for configured grant based PUSCH

For single-slot PDSCH, Tx beam (QCL assumption) is selected from the candidate Tx beams (TCI states) in PDSCH slot. However, for multi-slot PDSCH, the timing (actual slot) for a Tx beam (TCI state) selection is not specified in Rel-15 therefore creating ambiguity between UE and gNB regarding the assumed Tx beamforming at gNB (QCL assumption).

Summarizing discussion above the following observation can be made.

Observation:

- Some of the practical NR system capabilities are not functional in Rel-15 and the issues cannot be resolved in Rel-15 timeframe due late stage and potential backwards compatibility issues

Based on the discussion above it seems essential to consider limited follow up work in RAN1 to correct essential issues taking into account backwards compatibility. In our view, Rel-16 TEI work or Rel-16 eMIMO WI in RAN1 may be considered with the scopes decided in RAN.

Proposal:

- Consider follow up maintenance work for NR in RAN1, e.g. as part of Rel-16 TEI work or Rel-16 eMIMO WI, to resolve these issues in backwards compatible manner if applicable

  - The actual scope can be decided in RAN with the following open issues used as a starting point

  - Issues related to beam management

      - QCL assumption of aperiodic CSI-RS for BM

      - QCL assumption of PDCCH/PDSCH/PUSCH after beam failure recovery

      - Spatial relation info assumption for configured grant based PUSCH

  - Issues related to simultaneous transmission and receptions for two groups of RS / physical channels

      - UE assumption for processing of the following combinations PSDSCH+PDSCH, PDSCH+PDCCH, A-CSI-RS+PDSCH, SRS+SRS

# Conclusions

In this contribution, we discuss motivations for follow up maintenance work in Rel-16 to address unresolved issues from Rel-15 due to late stage of the specification and potential backward compatibility issues. It was observed that some of the practical NR capabilities are not functional in Rel-15 and the issue cannot be resolved in Rel-15 timeframe, due late stage and potential backwards compatibility issues. Based on the observation the following proposal was made

- Consider follow up maintenance work for NR in RAN1, e.g. as part of Rel-16 TEI work or Rel-16 eMIMO WI, to resolve these issues in backwards compatible manner if applicable

  - The actual scope can be decided in RAN with the following open issues used as a starting point

  - Issues related to beam management

      - QCL assumption of aperiodic CSI-RS for BM

      - QCL assumption of PDCCH/PDSCH/PUSCH after beam failure recovery

      - Spatial relation info assumption for configured grant based PUSCH

  - Issues related to simultaneous transmission and receptions for two groups of RS / physical channels

      - UE assumption for processing of the following combinations PSDSCH+PDSCH, PDSCH+PDCCH, A-CSI-RS+PDSCH, SRS+SRS

# References

[1]		RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #96, Athens, Greece, February 25th – March 1st, 2019.

[2] 	RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #95, Spokane, USA, November 12th – 16th, 2018.