3GPP TSG RAN Meeting #83								RP-190235

Shenzhen, China, March 18-21, 2019

Agenda Item:	9.3.5

Source:	Apple

Title:	Views on NR UE Power Saving WI

Document for:	Discussion/Decision

# Introduction

The UE power saving SI was concluded in RAN1#96[2] and outcomes were captured in [1]. Various power saving schemes in time / frequency / antenna domains for UE to better adapt to dynamic traffic arrival and DRX operation were studied. A lot of efforts were made to reduce unnecessary processing/buffering of signals and transitions among different states. One of the focus points was to help UE reducing unnecessary PDCCH monitoring when there is no grant . It has been also identified that UE assistance information could improve UE power saving by helping network to make better decisions on parameter configuration or network operation.

Although the study item in RAN1 has finished, there are still many topics under consideration. Considering the limited time for (potential) WI, reducing the scope of WI is inevitable. In this contribution, we provide our views on WI scope with focus on UE assistance information and dynamic PDCCH monitoring skipping.

# UE Assistance Information

There are certain types of information, e.g. from application layer, that are available to the UE, but not directly accessible by the network. In some cases, that information, if provided to the network, could potentially help the network to enhance scheduling decision and UE configurations, which further improves UE power consumption. In this section, we provide a couple simple examples, including assistant information available at UE from application layer, to help network configuration on UE specific DRX and SCell activation/deactivation.

## UE-assisted DRX Configuration

CDRX is an important feature, directly impacting UE power consumption. This mechanism itself is quite flexible in the sense that it could be configured with wide range of parameters to support diverse applications with different traffic patterns (delay and burstiness). For example, drx-LongCycle value have a range from 10ms to 12400ms. drx-onDurationTimer has a range from 1/32ms to 1600ms. drx-InactivityTimer could be configured in the range from 0ms to 2560ms.

In real deployment, however, as we also observed from field logs, only very limited set of DRX configurations are used and mostly very static. For example, one configuration for VoIP and the other configuration for regular data transfer, which is fixed in most cases. The exact configured parameters may vary from network to network, but in most cases DRX configuration is very static compared to dynamically changing user traffic. For a normal UE usage, the user can quickly initiate / terminate / switch across different applications, which generate different types of traffic. However, in most cases, we do not observe network adaptation of its configuration to those changes accordingly.

One of the potential limitations preventing network to adapting the configurations to traffic pattern is the lack of full knowledge of the traffic generated from upper layer, which is beyond RAN and CN. Although network can use QoS parameter to capture different traffic characteristics in configuring DRX parameters, existing 5QI (5G QoS Indicator) values in QoS framework cannot fully capture different details of traffic patterns. For example, 5QI defines packet delay budget (PDB) which could be used to determine drx-LongCycle, e.g., real delay requirement of application layer traffic, periodicity of the traffic, and burstiness of traffic, total traffic size are not captured in QoS framework although they could be used to further optimize configuration.

That information, on the other hand, are typically available from UE side. When users initiate a certain application, e.g., video call (e.g., FaceTime video), voice call (FaceTime audio), downloading of Apps, uploading of video clips to cloud server, watching buffered video such as YouTube, or start stock/weather applications, UE may deduct certain statistics (with higher layer information), e.g., how much traffic is to be generated and what the traffic pattern will look like. We believe it would be very helpful to provide a mechanism that UE can provide such information to the network to assist network making better decisions in configuration.

Observation 1

- UE can provide application layer information to the network and potentially assist the network in selecting CDRX configuration parameters.

## UE-assisted SCell activation/deactivation

It has been observed that LTE CA consumes significant amount of power. But, the real problem is not that it consumes high power but that a lot portion of power for secondary carriers is being wasted instead of being used for data transmission. In typical CA scenarios, the following problems have been observed.

- Deactivation delay : Network does not quickly deactivate after data transfer finishes although there is no active data transfer happening. Even though there is MACCE-based dynamic activation/deactivation mechanism, it is hardly used in the field and deactivation mostly rely on SCell deactivation timer which is typically configured in the order of hundreds of ms (typically on the next on-duration interspersed by a long CDRX cycle)

- SCells causing low overall achievable throughput: Sometimes it is observed that SCells with low achievable throughput do not always contribute much to the overall achievable throughput. In such situation, the advantages of additional carriers for higher achievable throughput is limited with power consumption to support additional carriers being large.

In such situations, a UE’s request to the network for SCell activation / deactivation could be helpful in terms of UE power saving. In case 1) network can quickly deactivate unused SCells for power saving after file transfer finishes. Note that in many cases UE knows actual file size (with the help of upper layers) to be transmitted / received. In case 2), network could deactivate a set of SCells which drags down overall UE energy efficiency based on UE’s indication to network.

Observation 2

- UE can significantly reduce SCell activation/deactivation delay and reduce UE power consumption using UE-assisted SCell activation/deactivation mechanism based on UE’s knowledge from UE’s application layer (e.g., file size to download/upload). Since such information is available only in UE side , UE-assisted approach could be better than network-initiated approach in terms of UE power saving.

Proposal 1

- NR UE PS WI scope includes specifying mechanisms to support UE-assisted information to the network.

# PDCCH Monitoring Skipping (a.k.a Go-To-Sleep)

It has been observed from typical “day of use” device logs, that a dominant contributor to UE power consumption is the unnecessary PDCCH monitoring (i.e. without valid grant or scheduling) in connected mode. When a UE in RRC CONNECTED mode is scheduled by gNB to receive DL/UL data, the grant transmission time to the UE of interest could be random. This is because that traffic arrival for the UE from application server is random and the gNB need to serve multiple UEs in the cell. Figure 1 shows an exemplary CDF of grant inter arrival time of video and web-browsing in LTE. It is observed the mean grant-inter arrival time is 5ms for video traffic and 6.2ms for web-browsing. Note that the tail of distribution reaches to ~ 20ms for video and ~40ms for web-browsing. This indicates that there could be time gap on the order of ms to tens of ms during which UE can enter sleep state for power saving.

Figure 1 CDF of grant inter-arrival time for video (left) and web-browsing (right)

To take advantage of such time gap between grants, it is desirable to support explicit sleep indication based UE power saving mechanism as shown in Figure 2. Due to the dynamic change of traffic arrivals, configuring static PDCCH monitoring period or CDRX parameters may not be enough to effectively handle dynamic data transmission / reception in power efficient manner. With explicit signaling approach, network can dynamically handle such situation while maximizing UE power saving gain.

Figure 2 Expected UE power consumption with explicit signal based sleep indication

Proposal 2

- NR UE PS WI scope includes specifying dynamic PDCCH monitoring skipping (go-to-sleep) for UE power saving.

# Conclusion

In this contribution, we discussed UE assistance information and PDCCH monitoring skipping. We think these two mechanisms provide substantial benefits to UE power saving and recommend to be included in the corresponding R16 work item.

Note that UE assistance information could be used to help network configuration/operation. In case it is used for configuration, it may not necessarily require new enhancements / features in terms of UE behavior. For example, UE assisted DRX configuration could be used on top of R15 DRX framework without changing UE behavior on DRX operation. Likewise, SCell activation/deactivation does not require any change in terms of UE behavior in SCell operation. The only enhancement is UE’s feedback to network. Thus, the UE assistance information approach has minimal complexity impact in terms of specification work yet still provide potential benefits in UE power saving by helping network to make efficient decisions.

Unnecessary PDCCH monitoring without grant is one of the biggest contributors to UE power consumption. In that sense, we believe that dynamic indication making UE skipping PDCCH decoding would be a good trade-off between network scheduling flexibility and UE power saving. It has been shown through extensive study in the SI phase that this approach, by allowing UE to opportunistically enter deeper sleep mode, has the highest potential among potential techniques.

Observation 1

- UE can provide application layer information to the network and potentially assist the network in selecting CDRX configuration parameters.

Observation 2

- UE can significantly reduce SCell activation/deactivation delay and reduce UE power consumption using UE-assisted SCell activation/deactivation mechanism based on UE’s knowledge from UE’s application layer (e.g., file size to download/upload). Since such information is available only in UE side , UE-assisted approach could be better than network-initiated approach in terms of UE power saving.

Proposal 1

- NR UE PS WI scope includes specifying mechanisms to support UE-assisted information to the network.

Proposal 2

- NR UE PS WI scope includes specifying dynamic PDCCH monitoring skipping (go-to-sleep) for UE power saving.

# References

- 38.840 Study on UE Power Saving, v 0.1.1

- Chairman Notes, 3GPP RAN1#96 Meeting, Athens, Greece, 25th February – 1st March, 2019