3GPP TSG RAN meeting #83										RP-190246

Shenzhen, China, March 18-21, 2019

Status Report to TSG

Agenda item:			10.3.6

| WI / SI Name | Further performance enhancement for LTE in high speed scenario |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | LTE_high_speed_enh2 |  |  |  |  |
| Unique ID | 800079 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-181482 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: September/2019 | Performance part: December/2019 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 10% | Performance Part: 15% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Takuma TAKADA |
|  | Company | NTT DOCOMO, INC. |
|  | Email | takuma.takada.nv@nttdocomo.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

- Specify corresponding signaling support, if needed.

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

    - Agreements

General

- Work plan for LTE high speed in Rel.16 was agreed in [1].

For UE RRM

- Way forward on HST enhancement for RRM was agreed in [2]:

  - Enhanced intra-frequency cell identification and measurement period are defined for SCC.

    - Using the existing highSpeedEnhancedMeasFlag or introducing new IE is FFS

    - How to define the UE capability for enhanced intra-frequency cell identification and measurement period for SCC is FFS.

    - Note: it is expected to send LS to RAN2 in next meeting (RAN4#90bis)

  - RAN4 RRM shall focus on the following requirement impacts for HST feature:

    - Cell detection (agreed in RAN4#90 chairman’s note)

    - Cell reselection requirements in idle mode

    - Measurement requirements in connected mode (single carrier)

      - DRX

      - Non-DRX

    - Measurement requirements of SCC with both active SCells and deactivated SCells

    - RLM

    - Measurement accuracy

    - FFS: UE Tx timing, i.e., Tq

  - High speed CA scenarios

    - CA scenario 1 is a typical deployment in high speed scenario

    - Whether scenario 4/5 are implemented needs operator input

    - Note: CA scenarios are listed in [3]

    - Preclude scenario #2 and #3 from above scenarios (agreed in RAN4#90 chairman’s note)

  - For high speed scenarios, ISD requires operator input for typical setting/range

For UE demodulation performance

- Way forward on UE demodulation enhancement for HST was agreed in [4]:

  - Companies’ view on CA (1)

    - Define HST-SFN CA requirements based on Rel.14 requirements

      - Ericsson, Qualcomm, NTT DOCOMO, CMCC

    - Not Define HST-SFN CA requirements

      - Huawei, Intel, MTK

  - Way forward on uni-directional and bi-directional HST-SFN

    - Demand for “bi-directional + 500km/h” scenario by CMCC and NTT DOCOMO has been observed.

    - Demand for “uni-directional + 500km/h” scenario, need more operator’s inputs.

  - Which HST-SFN scenario is specified:

    - Option 1: Adopt HST-SFN Bi-directional scenario

      - Huawei, CMCC, Intel, NTT DOCOMO, Qualcomm, MTK

      - Maximum Doppler shift must be capped

      - Details are FFS

    - Option 2: Adopt HST-SFN uni-directional scenario

      - Ericsson, Qualcomm

        - Details are FFS

    - Option 3: Adopt HST-SFN Bi-directional scenario

      - NTT DOCOMO

        - Details are FFS

  - Whether original HST and multipath scenario are considered or not:

    - Option 1: Specify HST, multipath and HST-SFN scenario

      - NTT DOCOMO

    - Option 2: Specify only HST-SFN scenario

      - Huawei, Qualcomm, Intel, Ericsson, MTK

  - DMRS-based transmission modes are considered or not in this WI

    - Option 1: Consider

      - Huawei

    - Option 2: Not consider

      - Qualcomm, Intel, MTK

  - 4Tx is not considered in this WI.

  - CSI is not considered in this WI.

For BS demodulation performance

- Way Forward on BS demodulation scenarios and requirements for HST was agreed in [5]:

  - Following deployment scenarios should be considered.

    - UE speed:

      - 500km/h;

    - Reference frequency will be agreed based on different companies’ input. Considerations would focus as much as possible on reusing the existing designs in previous 3GPP standards.

      - Other options are not precluded.

    - Maximum Doppler shift (fd) for HST will be agreed based on different companies’ input;

      - note: according to [6], new or modified physical layer reference signals shall not be considered

    - BS-Railway track distance (Dmin) and Initial distance of the train from BS (Ds/2):

      - Option 1: Same as Scenario 1 (Open space) and Scenario 3 (Tunnel) specified in Rel.14 LTE;

      - Other options are not precluded.

    - If needed operators can provide a detailed description of the HST deployment scenarios.

  - Define enhanced PUSCH requirements for following scenarios:

    - High speed train (HST) scenario;

    - FFS Multipath fading propagation scenario (i.e., define ETU with Doppler shift more than 600Hz);

    - FFS moving propagation scenario;

    - Other scenarios are not precluded.

  - Define PRACH requirements HST scenarios is FFS.

  - Define PUCCH requirements for multipath fading propagation scenario is FFS.

2.4.2	Remaining Open issues

- For RRM perspective

  - Specify intra-frequency cell identification and measurement period for SCC

    - Using the existing highSpeedEnhancedMeasFlag or introducing new IE

    - Decide how to define the UE capability for enhanced intrafrequency cell identification and measurement period for SCC.

  - Specify RRM requirements in high speed CA scenario

    - Decide which requirements are enhanced.

    - Decide which are the typical CA deployments in high speed scenario.

  - Specify RRM requirements in 500km/h velocity

    - Decide which requirements are enhanced.

  - Specify the corresponding RRM test cases.

- For UE demodulation perspective

  - Decide whether HST-SFN CA requirements based on Rel.14 requirements are specified or not.

  - Specify UE demodulation performance requirements for 500km/h velocity.

    - Decide how to handle the optional HST-SFN scenarios.

    - Specify maximum Doppler shit for 500km/h velocity.

    - Decide whether original HST and multipath scenario are considered or not:

      - Option 1: Specify HST, multipath and HST-SFN scenario

      - Option 2: Specify only HST-SFN scenario

  - Decide whether DMRS-based transmission modes are considered or not.

- For BS demodulation perspective

  - Specify the BS demodulation performance requirements.

    - Decide BS-Railway track distance (Dmin) and Initial distance of the train from BS (Ds/2):

      - Option 1: Same as Scenario 1 (Open space) and Scenario 3 (Tunnel) specified in Rel.14 LTE;

        - Other options are not precluded.

    - Specify maximum Doppler shit for 500km/h velocity.

    - Decide whether multipath fading propagation and moving propagation scenario.

    - Decide whether PRACH requirements for HST scenario are specified or not.

    - Decide whether PUCCH requirements for multipath fading propagation scenario are specified or not.

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

[1] R4-1900804, “Work plan for LTE high speed in Rel.16”, NTT DOCOMO, INC., Huawei, HiSilicon

[2] R4-1902520, “Way forward on HST enhancement for RRM”, Huawei

[3] R4-1901531, “On SFN deployment with carrier aggregation for high speed train scenarios”, Nokia, Nokia Shanghai Bell

[4] R4-1902521, “Way forward on UE demodulation enhancement for HST”, NTT DOCOMO, INC.,

[5] R4-1902522, “Way forward on BS enhancement for HST”, Ericsson, NTT DOCOMO, INC.,

[6] RP-181482, “New Work Item on Further performance enhancement for LTE in high speed scenario”

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion level swith colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template