3GPP TSG RAN meeting #83										RP-190336

Shenzhen, China, March 18-21, 2019

## Status Report to TSG

Agenda item:			10.3.2

| WI / SI Name | Additional enhancements for NB-IoT |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NB_IOTenh3 |  |  |  |  |
| Unique ID | 800084 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-182902 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 03/2020 (changed) | Performance part: 06/2020 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 35% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | COZZO, Carmela |
|  | Company | Huawei |
|  | Email | carmela.cozzo@huawei.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

Since Rel-16 functional freeze was changed to March 2020, the core part completion date is updated to March 2020 and TUs are added for RAN2/3/4 in the first quarter of 2020.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

Contributions [1] – [72] were submitted to RAN1#96 meeting. A WI work plan was provided in R1-1901501. A summary of the RAN1 agreements was submitted after RAN1#96 meeting in R1-1903665.

#### 2.1.1	Agreements

UE-group wake-up signal,

RAN1 discussed UE-group wake-up signal, with the following agreements:

RAN1#96 Agreements For further evaluation and study (especially from UE implementation perspective): Benefits and costs with more grouping levels, i.e., subgroups of subgroups. A UE is required to monitor WUS(s) in only one WUS (T/F) resource location Up to 2 time-multiplexed WUS resources may be configured. FFS whether a group WUS resource may be shared with legacy WUS or not. For further study until next meeting Companies are encouraged to provide results for the different sequence candidates, taking into account, e.g., - Detection window lengths - Frequency errors - Channel model where a starting point for evaluations is the Rel-15 WUS assumptions in R1-1714993 and Rel-15 WUS functionality (w.r.t. frequency error and detection window lengths).

Support for transmission in preconfigured UL resources

RAN1 discussed transmission in preconfigured UL resources, with the following agreements:

RAN1#96 Agreements When the UE is configured to use several TA validation criteria, the TA is valid only when all the configured TA validation criteria are satisfied. For dedicated PUR in idle mode, the PUR search space configuration shall be included in the PUR configuration. - PUR search space is the search space where UE monitors for NPDCCH - FFS: Whether PUR search space is common or UE specific In idle mode, the TA validation configuration can include “PUR Time Alignment Timer” - Where the UE considers the TA as invalid if the (current time – time at last TA update) > the PUR Time Alignment Timer - Details on how to specify the “PUR Time Alignment Timer” is up to RAN2 In idle mode, when the UE validates TA, the UE considers the TA for the previous serving cell as invalid if the serving cell changes - Above applies for the case where the UE is configured to use the serving cell change attribute For dedicated PUR in idle mode, the dedicated PUR ACK is at least sent on NPDCCH - FFS: Whether to introduce new field in DCI or reuse existing field - RAN2 can decide if a higher layer PUR ACK is also supported When the TA is validated and found to be invalid and the UE has data to send, the UE can obtain a valid TA and may send data via legacy RACH or EDT procedures - FFS whether only TA is acquired and then data sent on PUR is supported - FFS other approaches to obtain a valid TA In idle mode, at least the following PUR configurations and PUR parameters may be updated after a PUR transmission: - Timing advance adjustment - UE TX power adjustment - FFS: Repetition adjustment for NPUSCH FFS: Whether the above update is done in L1 and/or higher layer In idle mode, the PUR search space configuration includes at least the following: - NPDCCH repetitions and aggregation levels - NPDCCH starting subframe periodicity (variable G) - Starting subframe position (alpha_offset) For dedicated PUR, in idle mode, the PUR resource configuration includes at least the following - Time domain resources including periodicity(s) - Note: also includes number of repetitions, number of RUs, starting position - Frequency domain resources - TBS(s)/MCS(s) - Power control parameters - Legacy DMRS pattern

Scheduling of multiple DL/UL transport blocks

RAN1 discussed scheduling of multiple DL/UL transport blocks, with the following agreements:

RAN1#96 Agreements One DCI can be used to schedule both initial and retransmission of different HARQ processes. For unicast, when all the TBs are scheduled by one DCI - MCS, repetition number, resource allocation, are common across all UL transport blocks - There is a single field for each of the following as in Rel-15: Scheduling delay, DCI subframe repetition number, Flag for differentiation - MCS, repetition number, resource assignment are common across all DL transport blocks - There is a single field for each of the following as in Rel-15: Scheduling delay, DCI subframe repetition number, NPDCCH order indicator, Flag for differentiation - FFS: HARQ-ACK resource For unicast, relationship 1 is supported: 1 HARQ process corresponds to 1 TB - FFS: Whether to support relationship 2 (1 HARQ process corresponds up to 2 TBs) in addition to relationship 1 - RAN1 will make decision on the support for the FFS part in RAN1#96bis For unicast, scheduling gaps between TBs scheduled by one single DCI are not supported for relationship 1 For TBs scheduled by one DCI that are contiguous, the ACK/NACK resources are back-to-back. FFS details. For SC-MTCH, the maximum number of TBs scheduled is 8. For SC-MTCH, all the TBs scheduled by one DCI use the same resource assignment, MCS and repetition number. For SC-MTCH multiple TBs scheduling, down-select from the following options: - Modify existing DCI to indicate the number of scheduled TBs (e.g. by adding new field) - Reuse Rel-15 DCI and use SC-MCCH to indicate TB numbers. - Support both a) and b)

Coexistence of NB-IoT with NR

RAN1 discussed coexistence of NB-IoT with NR, with the following agreements:

RAN1#96 Agreements For further study in future meeting (including): - NB-IoT resource reservation on non-anchor carrier to improve the performance of coexistence of FDD/TDD NB-IoT with NR - Subframe-level resource reservation in NB-IoT - Symbol-level resource reservation in NB-IoT - Slot-level resource reservation in NB-IoT For further study in future meeting (including): NB-IoT transmission is dropped/postponed in reserved resources. Conclusion From RAN1 perspective, no issues were identified that would prevent the coexistence of NR and FDD/TDD NB-IoT.

Support of quality report in Msg3 for non-anchor access

RAN1 discussed the quality report in Msg3 for non-anchor access, with the following agreements:

RAN1#96 Agreements For further study in future meetings: For channel quality report in Msg3 on non-anchor access, RAN1 identified the following scenarios in which UE may monitor a carrier that is different from the carrier in which the UE receives Msg2 (i.e. RAR): - Idle mode - Non-EDT random access procedure - EDT procedure - Connected mode - NPDCCH ordered NPRACH procedure - Random access procedure to apply for UL-SCH resource For the measurement on carrier(s) other than the one UE is receiving RAR for non-anchor access, if supported, RAN1 to select one or more among the following candidates: - Paging carrier - Anchor carrier - Carrier(s) configured by CarrierConfigDedicated-NB in connected mode - Other carriers configured by network with implicit or explicit signaling The following issues are identified for RAN1 further study - Whether amount of time or gap is needed for the measurement - Which carrier(s) UE reported if more than one carrier is measured - What kind of NRS UE can use for the measurement and whether needs indication In case of 2 bits are used for a non-anchor carrier, one of 3 candidate values can be reported in Msg3 - Which depends on Rmax, the maximum number of repetitions for NPDCCH Type 2 CSS. For further study in future meetings: If supported, RAN1 further discuss how to derive the candidate values reported in Msg3 if no NPDCCH Type2 CSS on the carrier.

Presence of NRS on a non-anchor carrier for paging

RAN1 discussed the presence of NRS on a non-anchor carrier for paging, with the following agreements:

RAN1#96 Agreements For the issue of NRS presence for NPDCCH early termination when NWUS is enabled, down-select among: - Alt3: Decouple configuration of NRS for NPDCCH early termination and NWUS early termination - e.g. eNB can enable NRS for NWUS and disable for NPDCCH, or vice versa. - Alt4: NRS is always associated with NPDCCH for paging For further study in future meetings: RAN1 to consider enabling presence of CRS in non-anchor carriers in subframes not containing NRS. The decimation pattern (i.e., the pattern that determines which POs have subframes with NRS even when no NPDCCH is transmitted) is designed at least with the following principles: - P1: The decimation pattern shall be fair across UEs, i.e., all UEs see the same/similar percentage of POs with NRS. - FFS: the case for eDRX - P2: A UE belonging to a given UE group (being a UE group the group of UEs that monitor paging in the same PO) can use NRS belonging to a PO of a different group in addition to the NRS of its own UE group. - The maximum gap between the PO with NRS and the PO the UE monitors is not larger than X (to ensure that the UE can reliably estimate the SNR for NPDCCH early termination) - P3: The POs with NRS are quasi-uniformly/uniformly distributed from UE perspective. - P4: The POs with NRS are quasi-uniformly/uniformly distributed from network perspective. NOTE: It was already agreed in previous meetings that a subset of the POs have associated subframes which will contain NRS even when no paging NPDCCH is transmitted. Further study details of decimation pattern until RAN1#96bis.

#### 2.1.2	Remaining Open issues

- Specify further details to support UE-group wake-up signal.

- Specify further details to support transmission in preconfigured resources in idle and/or connected mode based on SC-FDMA waveform for UEs with a valid timing advance.

- Specify further details for scheduling multiple DL/UL transport blocks with or without DCI for SC-PTM and unicast.

- Conclude on potential specification enhancement(s) for improving the performance of coexistence of NB-IoT with NR, from the list agreed in RAN1.

- Specify further details to support Msg3 quality reporting for non-anchor access.

- Specify further details of the signalling to indicate on a non-anchor carrier for paging a set of subframes which will contain NRS even when no paging NPDCCH is transmitted.

## 2.2	RAN2

Contributions [73] – [139] were submitted to RAN2#105 meeting. A WI work plan was provided in R2-1901124. A summary of the RAN2 agreements was submitted after RAN2#105 meeting in R2-1902405.

#### 2.2.1	Agreements

Organisational

RAN2#105 agreements: None (except for the summary document in R2-1902405).

Mobile-terminated (MT) early data transmission (EDT)

RAN2 discussed Mobile-terminated (MT) early data transmission (EDT), with the following agreements:

RAN2#105 agreements: DL data in paging message is excluded (Opt A). RNTI in paging message to schedule the DL data is excluded (Opt B). Working assumption: DL data scheduled, i.e. DL grant, in paging message is excluded (Opt C). Working assumption: DL data scheduled in paging occasion is excluded (Opt D).

UE-group wake-up signal (WUS)

RAN2 discussed UE-group wake-up signal (WUS), with the following agreements:

RAN2#105 agreements: Feasibility of the solution based on the following attributes for deriving the service-type for GWUS can be studied further: - Paging Probability - Mobility.

Transmission in preconfigured resources

RAN2 discussed transmission in preconfigured resources, with the following agreements:

RAN2#105 agreements: Multi-shot D-PUR is supported with the possibility to configure as a single shot. UE may perform a D-PUR request/information, if D-PUR is indicated as enabled in the cell. Network makes the decision on the D-PUR configuration. Request/information can include: - Requested TBS - Requested periodicity - Other information FFS. The eNB can (re)configure and release D-PUR by dedicated RRC signalling. D-PUR configuration is released when the eNB doesn’t detect “m” consecutive UE transmissions. The UE must release the D-PUR when it does a RA procedure on a new cell. D-PUR configuration can be set up without a pre-defined end (infinite).

Scheduling multiple DL/UL transport blocks

No discussion at RAN2#105

Network management tool enhancement

RAN2 discussed network management tool enhancement, with the following agreements:

RAN2#105 agreements: - RLF report: The UE informs network about having an RLF report available via rlf-InfoAvailable indication during the initial RRC connection re-establishment process after RLF. The rlf-InfoAvailable parameter is included in Msg5 of the RRC connection re-establishment process. It is up to the network to decide when to request for the RLF report, using UEInformationRequest/UEInformationResponse procedure. In case of no/failed RRC connection re-establishment, the UE may store the RLF report for indicating rlf-InfoAvailable in Msg5 of later connection establishment. The following legacy parameters are supported for RLF reporting in NB-IoT: - Failed cell ECGI - Last Serving Cell RSRP/RSRQ (Whether OK for CP-solution is FFS) - Absolute Time Stamp (optional, if available) - Location Info (optional, if available) - FFS: further information. RLF reporting does not trigger RRC connection establishment/resume, including EDT. Neither rlf-InfoAvailable indication nor RLF report is included in the Msg3 of the EDT procedure to indicate to the network about an available RLF report. - RACH report: For NB-IoT RACH report, range of numberOfPreamblesSent is INTEGER (1..64). contentionDetected is Boolean. Initial CEL is included in NB-IoT RACH report. Working assumption: A Boolean flag indicating EDT fallback (i.e., the UE started with EDT NPRACH resources and went through a fallback to non-EDT NPRACH resources) is included in NB-IoT RACH report. Take the ASN.1 structure provided in the report (proposal 5) as baseline for NB-IoT running CR. Initial CEL is included in eMTC RACH report. Working assumption: A Boolean flag indicating EDT fallback (i.e., the UE started with EDT PRACH resources and went through a fallback to non-EDT PRACH resources) is included in eMTC RACH report. Take the ASN.1 structure provided in the report (proposal 8) as baseline for eMTC running CR. RAN2 aims on standardizing a single method of sending NB-IoT RACH report, i.e., avoid multiple solutions. Proposed solutions should be discussed considering signalling overhead, UE power consumption, security aspects and other reports being considered in other email discussions. NB-IoT RACH report includes the information from the latest successful RACH procedure. NB-IoT RACH report information is discarded when the UE goes to IDLE. FFS how to report. SON-ANR: Solution direction based on option a: - Single set of measurements only. - No new measurement requirements. - ANR measurement reporting using the UE Information Request / Response framework is supported. Other methods FFS. - ANR reporting for the CP solution is not supported in Rel-16.

Improved multi-carrier operation

No discussion at RAN2#105.

Inter-RAT cell selection

RAN2#105 agreements: Priorities of eMTC/LTE/GERAN frequencies are not provided by NB-IoT network as assistance information for inter-RAT cell selection. Priorities of NB-IoT frequencies are not provided by eMTC/LTE network as assistance information for inter-RAT cell selection. Suitability criteria of eMTC/LTE/GERAN frequencies are not provided by NB-IoT network as assistance information for inter-RAT cell selection. Suitability criteria of NB-IoT frequencies are not provided by eMTC/LTE network as assistance information for inter-RAT cell selection.

Coexistence with NR

No discussion at RAN2#105.

Connection to 5GC

RAN2 discussed the LS from SA2 on Completion of Study on Cellular IoT support and evolution for the 5G System and sent a reply in R2-1902436.

RAN2 discussed the LS from SA2 on eDRX cycles for CM-CONNECTED with RRC inactive and sent a reply in R2-1902431.

#### 2.2.2	Remaining Open issues

- Agree solution(s) for MT-EDT and details of the agreed MT-EDT solution(s).

- Solution for UE-grouping for WUS and RAN2 aspects of UE-Group WUS.

- Support for transmission in preconfigured dedicated resources:

  - Detailed solution for transmission in preconfigured dedicated resources in idle mode,

  - Solutions for transmission in preconfigured shared resources in idle mode if supported,

  - Solutions for transmission in preconfigured resources in connected mode if supported;

- Scheduling multiple DL/UL transport blocks with or without DCI for SC-PTM and unicast:

  - Signalling for scheduling multiple DL/UL transport blocks with or without DCI for SC-PTM and unicast,

  - MAC procedure for scheduling multiple DL/UL transport blocks in unicast.

- Network management tools:

  - SON/ANR:

    - Whether there is a time limit for the reporting and whether to have additional method for reporting

    - Configuration (procedure and parameters)

    - Contents of the report

  - RACH report: Procedure for the reporting and stage 3 aspects

  - RLF report: FFS additional info in the report and stage 3 aspects.

- Improved multi-carrier operation

  - Support of Msg3 quality reporting for non-anchor access: reporting mechanism (whether to reuse rel-14 or introduce a new mechanism)

  - NRS presence on non-anchor even when no paging NPDCCH is transmitted:

    - Signalling to indicate on a non-anchor carrier the set of subframes which will contain NRS even when no paging NPDCCH is transmitted.

    - Whether to enable RRM measurements on non-anchor carrier.

- Inter-RAT cell selection: Stage 3 aspects of the signalling of the assistance information in the NB-IoT and LTE/eMTC RATs

- Signalling impact of coexistence of NB-IoT with NR, if any.

- Connection to 5GC.

- Solution(s) for small data transfer for the user plane solution, pending on SA2

- All other aspects

## 2.3	RAN3

Contributions [140] – [155] were submitted to RAN3#103 meeting. A WI work plan was provided in R3-190642.

#### 2.3.1	Agreements

RAN3 discussed support of mobile-terminated (MT) early data transmission (EDT) jointly with eMTC, with the following observations:

- Same as eMTC: - Wait for RAN2 LS reply to SA2 for further progress (before RAN3 #103bis) - At least the following potential functions to support 5GC connection may be discussed: - Support of Extended DRX in CM-idle - Support of Restriction of Enhanced Coverage - Delivery of Expected UE Behaviour information to NG-RAN - Support of QoS - Relocation / mobility - UP solution - Dedicated observations were identified for NB-IoT: - Support for Inter-UE QoS for Data over NAS - RAT-Type information delivery to AMF

The Intra eNB CP Relocation for NB-IoT was also discussed without conclusion.

RAN3 had initial discussions on connection to 5GC, jointly with eMTC, with no agreements.

#### 2.3.2	Remaining Open issues

- Specify support for mobile-terminated (MT) early data transmission (EDT)

- Specify support for connection to 5GC

## 2.4	RAN4

Contributions [156] – [167] were submitted to RAN4#90 meeting. A WI work plan was provided in R4-1901115.

#### 2.4.1	Agreements

RAN4 discussed coexistence with NR jointly with eMTC, with the following agreement:

- Agreed WF on study scope definition in R4-1902681

RAN4 discussed UE-group wake-up signal, with the following agreement (from agreed WF in R4-1902371):

- RAN4 shall discuss whether the release 15 WUS requirements can be reused for release 16 legacy WUS and whether new requirements are needed for release 16 UE-group WUS based on the RAN1/RAN2 agreements.

RAN4 discussed pre-configured uplink resources, with the following agreement (from agreed WF in R4-1902371):

- RAN4 is to discuss whether to specify requirements for following methods to validate TA - Serving cell change - Time Alignment Timer (TAT) - Serving cell RSRP change - FFS if RAN4 should define criteria for TA validity, e.g. threshold for the RSRP change, threshold for the TA change, threshold for the time period during which TA is considered as valid.

RAN4 discussed multi-carrier operation, with the following agreement (from agreed WF in R4-1902371):

- RAN4 is to specify requirements for MSG3 quality reporting on non-anchor carriers based on RAN1/RAN2 outcome. - RAN4 is to identify the scenarios under which RRM measurements based on NRS on non-anchor carriers are feasible.

#### 2.4.2	Remaining Open issues

- Specify support for UE-group wake-up signal

- Specify support for transmission in preconfigured resources in idle and/or connected mode based on SC-FDMA waveform for UEs with a valid timing advance.

- Conclude on potential specification enhancement(s) for improving the performance of coexistence of NB-IoT with NR.

- Specify support for Msg3 quality reporting for non-anchor access.

- Identify scenarios under which RRM measurements based on NRS on non-anchor carriers are feasible.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R1-1901501	Revision of additional enhancements for NB-IoT workplan	Huawei, HiSilicon

- R1-1903247	Feature lead summary for Group wake-up signal for NB-IoT	Ericsson

- R1-1903508	Further topics for 6.2.2.1 Group WUS for NB-IoT	Ericsson

- R1-1901502	UE-group wake-up signal	Huawei, HiSilicon

- R1-1901648	UE-group wake-up signal for NB-IoT	vivo

- R1-1901732	UE-Group WUS in NB-IoT	MediaTek Inc.

- R1-1901746	UE-group wake-up signal in NB-IoT	Ericsson

- R1-1901865	Discussion on Wake-up signal for NB-IoT	ZTE

- R1-1901963	UE group wake-up signal for NB-IoT	Nokia, Nokia Shanghai Bell

- R1-1902063	Discussion on UE-grouping wake up signal in NB-IoT	LG Electronics

- R1-1902148	UE-group wake-up signal for Rel-16 NB-IoT	Lenovo, Motorola Mobility

- R1-1902196	UE group wake up signal for NB-IoT	Sony

- R1-1902216	UE-group wake-up signal for NB-IoT	Samsung

- R1-1902374	UE-group wake-up signal	Qualcomm Incorporated

- R1-1902457	UE-group wake-up signal for NB-IoT	Intel Corporation

- R1-1902642	Consideration on UE-group wake-up signal for Rel-16 NB-IoT	Sharp

- R1-1902778	Discussion on UE-group wake up signal in NB-IoT	NTT DOCOMO, INC.

- R1-1903254	Feature lead summary of Support for transmission in preconfigured UL resources	Huawei, HiSilicon

- R1-1903513	Updated feature lead summary of Support for transmission in preconfigured UL resources	Huawei, HiSilicon

- R1-1901503	Transmission in preconfigured UL resources	Huawei, HiSilicon

- R1-1901629	NB-IOT Pre-configured UL Resources Design Considerations 	Sierra Wireless, S.A.

- R1-1901733	Early Transmission in Preconfigured UL resources in NB-IoT	MediaTek Inc.

- R1-1901747	Support for transmission in preconfigured UL resources in NB-IoT	Ericsson

- R1-1901866	Support for transmission in preconfigured UL resources for NB-IoT	ZTE

- R1-1901964	Preconfigured Grant for Uplink transmission	Nokia, Nokia Shanghai Bell

- R1-1902064	Discussion on preconfigured UL resources in NB-IoT	LG Electronics

- R1-1902149	UL transmission in preconfigured resources for NBIoT	Lenovo, Motorola Mobility

- R1-1902217	Discussion on transmission in preconfigured UL reosurces for NB-IoT	Samsung

- R1-1902375	Support for transmission in preconfigured UL resources	Qualcomm Incorporated

- R1-1902458	UL transmission in preconfigured resources for NB-IoT	Intel Corporation

- R1-1902553	Consideration for preconfigured uplink resources (PUR)	Sequans Communications

- R1-1902779	Discussion on preconfigured UL resources in NB-IoT	NTT DOCOMO, INC.

- R1-1902933	Discussion on transmission in preconfigured UL resources	III

- R1-1903257	Summary on Multiple TB scheduling enhancement for NB-IoT	ZTE

- R1-1901504	Scheduling multiple DL/UL transport blocks for SC-PTM and unicast	Huawei, HiSilicon

- R1-1901631	NB-IOT Multiple Transport Block Grant Design Considerations	Sierra Wireless, S.A.

- R1-1901736	Scheduling of multiple DL/UL transport blocks in NB-IoT	MediaTek Inc.

- R1-1901748	Scheduling of multiple DL/UL transport blocks in NB-IoT	Ericsson

- R1-1901867	Consideration on scheduling enhancement for NB-IoT	ZTE

- R1-1901965	Scheduling of multiple DL/UL transport blocks	Nokia, Nokia Shanghai Bell

- R1-1902065	Discussion on multiple transport blocks scheduling in NB-IoT	LG Electronics

- R1-1902150	Design of scheduling of multiple DL/UL transport blocks for NBIoT	Lenovo, Motorola Mobility

- R1-1902218	Scheduling of multiple transport blocks for NB-IoT	Samsung

- R1-1902376	Scheduling of multiple DL/UL transport blocks	Qualcomm Incorporated

- R1-1902459	Scheduling of multiple DL/UL TBs for NB-IoT	Intel Corporation

- R1-1903255	Feature lead summary of Coexistence of NB-IoT with NR	Huawei, HiSilicon

- R1-1901505	Performance enhancements for NB-IoT coexistence with NR	Huawei, HiSilicon

- R1-1901731	Coexistence of NB-IoT with NR	MediaTek Inc.

- R1-1901749	Coexistence of NB-IoT with NR	Ericsson

- R1-1901868	Coexistence of NB-IoT with NR	ZTE

- R1-1901966	Coexistence of NB-IoT with NR	Nokia, Nokia Shanghai Bell

- R1-1902377	Coexistence of NB-IoT with NR	Qualcomm Incorporated

- R1-1903256	Feature lead summary of Support of Quality report in Msg3 for non-anchor access	Huawei, HiSilicon

- R1-1901506	Msg3 quality reporting for non-anchor access	Huawei, HiSilicon

- R1-1901734	Quality report in Msg3 on non-anchor carrier in NB-IoT	MediaTek Inc.

- R1-1901750	Support of quality report in Msg3 for non-anchor access in NB-IoT	Ericsson

- R1-1901869	Support of Quality report in Msg3 for non-anchor access	ZTE

- R1-1901967	Support of Quality report in Msg3 on non-anchor carrier	Nokia, Nokia Shanghai Bell

- R1-1902066	Downlink channel quality report during random access procedure on a non-anchor carrier	LG Electronics

- R1-1902219	Discussion on Msg3 report for non-anchor access	Samsung

- R1-1902378	Support of quality report in msg3 for non-anchor	Qualcomm Incorporated

- R1-1902460	Quality report in Msg3 for NB-IoT non-anchor access	Intel Corporation

- R1-1903251	Presence of NRS on a non-anchor for paging (feature lead summary)	Qualcomm Incorporated

- R1-1903515	Presence of NRS on a non-anchor for paging (feature lead summary)	Qualcomm Incorporated

- R1-1901507	NRS presence on non-anchor carriers for paging	Huawei, HiSilicon

- R1-1901735	NRS presence on  non-anchor carrier in NB-IoT	MediaTek Inc.

- R1-1901751	Presence of NRS on a non-anchor carrier for paging in NB-IoT	Ericsson

- R1-1901870	Presense of NRS on non-anchor PRB	ZTE

- R1-1901968	Presence of NRS on a non-anchor carrier for paging	Nokia, Nokia Shanghai Bell

- R1-1902067	Discussion on NRS on a non-anchor carrier	LG Electronics

- R1-1902220	NRS presence on non-anchor carrier	Samsung

- R1-1902379	Presence of NRS on a non-anchor carrier for paging	Qualcomm Incorporated

- R2-1900105	RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC	Document Rapporteur (BlackBerry)

- R2-1901124	Revision of additional enhancements for NB-IoT workplan	Huawei, HiSilicon

- R2-1900317	Report on email discussion [104#42][NB-IoT R16]  Service based Group Wake Up signal analysis	Nokia, Nokia Shanghai Bell

- R2-1900318	Analysis of Service based GWUS	Nokia, Nokia Shanghai Bell

- R2-1901066	Consideration on wake up signal	Lenovo, Motorola Mobility

- R2-1901148	General aspects on group based WUS	Huawei, HiSilicon

- R2-1901149	Performance analysis on UE_ID based WUS grouping	Huawei, HiSilicon

- R2-1901194	Paging multiplexing and UE mobility of group WUS	Ericsson

- R2-1901195	UE_ID based WUS grouping	Ericsson

- R2-1901488	Further consideration on UE grouping for WUS	ZTE Corporation, Sanechips

- R2-1901491	Sub-group ID calculation based on UE_ID for UE-group WUS	ZTE Corporation, Sanechips

- R2-1900120	Email discussion [104#43][eMTC & NB-IoT R16] D-PUR report	Sierra Wireless, S.A.

- R2-1900321	Signaling Aspects for D-PUR	Nokia, Nokia Shanghai Bell

- R2-1900322	Further Analysis on shared PUR	Nokia, Nokia Shanghai Bell

- R2-1900736	PUR Configuration	Ericsson

- R2-1900737	PUR TA validation & release	Ericsson

- R2-1900738	PUR UE Multiplexing	Ericsson

- R2-1900880	HARQ process for PUR transmission in idle mode	Intel Corporation

- R2-1901042	Discussion on D-PUR configuration in respect to radio condition	ASUSTeK

- R2-1901043	Discussion on TA timer for D-PUR	ASUSTeK

- R2-1901145	General aspects on PUR with dedicated resource	Huawei, HiSilicon

- R2-1901146	DRAFT LS on D-PUR	Huawei

- R2-1901147	Analysis on resource types for PUR	Huawei, HiSilicon

- R2-1901472	DRX considerations for preconfigured resources	Sequans Communications

- R2-1901477	Further consideration on UL aspects of D-PUR in IDLE	ZTE Corporation, Sanechips

- R2-1901479	Further consideration on DL aspects of D-PUR in IDLE	ZTE Corporation, Sanechips

- R2-1901731	DL Data after PUR transmission	Sierra Wireless, S.A.

- R2-1901881	Discussion on time alignment timer for PUR	LG Electronics UK

- R2-1901883	Support for S-PUR	LG Electronics UK

- R2-1901884	Further discussion on D-PUR	LG Electronics UK

- R2-1902027	Remaining aspects not covered by [104#43] on data transmission on PUR	Qualcomm Incorporated

- R2-1901150	Applicability of multiple TB scheduling to EDT	Huawei, HiSilicon

- R2-1900323	Idle mode ANR Measurements for NB-IoT	Nokia, Nokia Shanghai Bell

- R2-1901055	Procedure of RACH and RLF reporting in NB-IOT	Lenovo, Motorola Mobility

- R2-1901123	Report of Email discussion [104bis#44][NB-IoT R16] SON/ANR report	Huawei

- R2-1901185	Email discussion report on RLF for SON	Ericsson

- R2-1901447	Remaining issues for ANR report in NB-IoT	ZTE Corporation, Sanechips

- R2-1901463	Remaining issues for RLF report in NB-IoT	ZTE Corporation, Sanechips

- R2-1901466	Remaining issues for RACH report in NB-IoT	ZTE Corporation, Sanechips

- R2-1901640	Email discussion [104#45]: RACH Report – What and how to report	Qualcomm Incorporated

- R2-1901643	A framework for SON Configuration/Reporting	Sequans Communications

- R2-1901644	NB-IoT ANR Reporting	Sequans Communications

- R2-1900422	Channel quality reporting for non-anchor carriers	Qualcomm Incorporated

- R2-1900423	Non-anchor carrier measurements for RRM	Qualcomm Incorporated

- R2-1901120	DL channel quality reporting for non-anchor access	Huawei, HiSilicon

- R2-1901121	RRM measurements on non-anchor carrier	Huawei, HiSilicon

- R2-1901122	DRAFT LS on enabling RRM measurements on the non-anchor paging carrier in Idle mode	Huawei

- R2-1901184	Msg3 Quality Report Format for NB-IoT	Ericsson

- R2-1901186	Supporting Presence of NRS on a non-anchor carrier for paging in NB-IoT	Ericsson

- R2-1901498	Further consideration on quality report in Msg3 for non-anchor access	ZTE Corporation, Sanechips

- R2-1901511	Further consideration on non-anchor carrier measurement	ZTE Corporation, Sanechips

- R2-1900163	Report of email discussion 104#47NB-IoT R16 Inter-RAT cell selection	MediaTek Inc.

- R2-1900324	Analysis of IRAT cell selection assistance information signaling	Nokia, Nokia Shanghai Bell

- R2-1901541	On Inter-RAT assistance information for NB-IoT and LTE(-eMTC)	Sequans Communications

- R2-1901645	Inter-RAT selection triggering for NB-IoT	Sequans Communications

- R2-1901892	Improvements to Inter-RAT cell selection for IDLE mode	Fujitsu

- R2-1901505	Consideration on coexistence of NB-IoT with NR	ZTE Corporation, Sanechips

- R2-1900745	NB-IoT connectivity to 5GC	Ericsson

- R2-1900786	Access stratum changes to support idle mode eDRX for NB-IoT connected 5GC Qualcomm Incorporated

- R2-1900942	Access stratum changes to support connected mode eDRX for NB-IoT connected 5GC Qualcomm Incorporated

- R2-1901125	Discussion on Connection to 5G CN for NB-IoT	Huawei, HiSilicon

- R2-1901134	Discussion on support of UP optimisation and RRC_INACTIVE for NB-IoT and eMTC connected to 5GC	Huawei, HiSilicon

- R2-1901135	Discussion on eDRX for RRC Inactive	Huawei, HiSilicon

- R2-1901469	Consideration on connection to 5GC for NB-IoT	ZTE Corporation, Sanechips

- R2-1901564	RAN2 impacts on introduction of NB-IoT and MTC connected to 5GC	Nokia, Nokia Shanghai Bell

- R2-1901565	UP solution support for NB-IoT and MTC connected to 5GC	Nokia, Nokia Shanghai Bell

- R2-1901568	Reply LS on Completion of Study on Cellular IoT support and evolution for the 5G System	Nokia, Nokia Shanghai Bell

- R3-190910	Additional Enhancements for NB-IoT WI session report	Vice-chairman

- R3-190345	Scope of RAN3 work for NBIOT Enhancements	Nokia, Nokia Shanghai Bell

- R3-190642	Revision of additional enhancements for NB-IoT work plan	Huawei

- R3-190561	Consideration on support for MT EDT	LG Electronics

- R3-190801	MT early data transmission options for NB-IoT	Ericsson

- R3-190802	MT early data transmission indication in Msg4 for NB-IoT	Ericsson

- R3-190803	UP solution for NB-IoT MT EDT	Ericsson

- R3-190804	CP solution for NB-IoT MT EDT	Ericsson

- R3-190805	Data Notification for CioT UEs (NB-IoT)	Ericsson

- R3-190806	Data Notification	Ericsson

- R3-190271	Consideration on NB-IoT UE connected to 5G CN	ZTE

- R3-190346	RAN3 functions for NBIOT connected to 5GC	Nokia, Nokia Shanghai Bell

- R3-190347	Support of extended DRX for NBIOT	Nokia, Nokia Shanghai Bell

- R3-190348	Reply LS on Completion of Study on Cellular IoT support and evolution for the 5G System	Nokia, Nokia Shanghai Bell

- R3-191012	Summary of Discussion on NB-IoT	Huawei

- R3-190807	Enhancement on Intra eNB CP Relocation for NB-IoT	Ericsson

- R4-1900507	Discussion on NR and NB-IoT coexistence	ZTE Corporation

- R4-1901400	NB-IoT NR coexistence - Channel raster	Ericsson

- R4-1901401	NB-IoT and NR coexistence scope	Ericsson

- R4-1901487	Recommendations on NB-IoT coexistence with NR	Nokia, Nokia Shanghai Bell

- R4-1900327	Discussion R16 Further Enhancements to NB-IoT	Qualcomm Incorporated

- R4-1901115	Discussion on the RRM work plan for NB R16	Huawei, HiSilicon

- R4-1901116	Initial discussions on the UE group WUS RRM requirements	Huawei, HiSilicon

- R4-1901117	Initial discussions on the RRM impact from PUR	Huawei, HiSilicon

- R4-1901118	Discussion on the presence of NRS on a non-anchor carrier for paging	Huawei, HiSilicon

- R4-1901119	WF on RRM impact of the R16 enhancement for multi carrier NB	Huawei, HiSilicon

- R4-1901587	Discussions on RRM requirements for transmissions using PUR for NB-IoT	Ericsson

- R4-1902371	WF on RRM impact of the R16 enhancement for multi carrier NB	Huawei, HiSilicon, Ericsson

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion level swith colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template