3GPP TSG RAN meeting #83                                                            RP-190359

Shenzhen, China, March 18-21, 2019

Source:	NTT DOCOMO, INC., KDDI, Softbank

Title:	Survey on Japanese spectrum usage and regulatory requirements in the frequency band above 52.6GHz

Agenda Item:	9.3.12

Document for: 	Discussion and Decision

- Introduction

At the RAN#82 meeting, the updated SID on study on NR beyond 52.6GHz was agreed to change the upper bound frequency from 100GHz to 114.25GHz.

In this contribution, we provide the corresponding update on Japanese spectrum usage and regulatory requirements in the frequency band above 52.6GHz, based on [1].

- Survey on Japanese spectrum availability and regulatory requirements for above 52.6GHz

Figure 1: Major spectrum usage in the frequency band above 52.6GHz in Japan

(source: http://www.tele.soumu.go.jp/e/adm/freq/search/myuse/0002/index.htm)

Figure 1 shows frequency usage for frequency above 52.6GHz in Japan. Table 1 below summarize the survey on Japanese regulatory requirements in the frequency band above 52.6GHz.

Table 1. Major spectrum usage and regulatory requirements between 52.6 GHz and 100 GHz in Japan*

| Frequency band (GHz) |  | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  | 52.6 – 54.25 |  |  |  | For Earth Exploration Satellite Service (passive), Space Research Service (passive) |  |
|  | 54.25 – 55.78 | Max Tx power 1W |  | See Note 1 | For Broadcast Auxiliary Services (Mobile) | ARIB STD-B43 [2] See Note 2 for additional regulatory requirements. |
|  | 55.78 – 59 |  |  |  | For Commercial Telecommunications, Public and General Services (Fixed/Mobile) |  |
|  | 57 – 66 | Max Tx power 250mW | See Note 3 | Max occupied BW 9GHz | For Unlicensed low power data Communication systems | ARIB STD-T69 [3], ARIB STD-T117 [4] See Note 4 for additional regulatory requirements. |
|  | 60 – 61, 76 – 77 | Max Tx power 10mW |  | See Note 5 | For Vehicle Radars (Unlicensed Low-Power) | ARIB STD-T48 [5] See Note 6 for additional regulatory requirements. |
|  | 77 – 81 | Max Tx power 10mW or Max Tx PSD 5μW/MHz in case of occupied BW equal to or less than 2GHz |  | Max occupied BW 4GHz | For Vehicle Radars (Unlicensed Low-Power) | ARIB STD-T111 [6] See Note 7 for additional regulatory requirements. |
|  | 71 – 76, 81 – 86 | Max peak Tx power 1W |  | Max occupied BW 5GHz | For Commercial Telecommunications, Public and General Services (Fixed/Mobile) | See Note 8 for additional regulatory requirements. |
|  | 77.5 – 78 |  |  |  | For Amateur |  |
|  | 85.5 – 92, 100 – 102, 105 – 116 |  |  |  | For Radio Astronomy |  |
| Note 1: For system with 125MHz BW, PSK, QAM and OFDM are available. For system with 500MHz BW, PSK is available. For system with 1GHz BW, PSK and QAM are available. |  |  |  |  |  |  |
| Note 2: The unwanted emission shall be equal to or less than 50μW. |  |  |  |  |  |  |
| Note 3: The carrier sensing functionality is required for transmission power above 10dBm. |  |  |  |  |  |  |
| Note 4: The antenna absolute gain shall be equal to or less than 47dBi. In case of transmission power above 10mW, directivity with above 10dBi antenna absolute gain for main beam direction is necessary and EIRP shall be equal to or less than 40dBm. In addition, the unwanted emission shall satisfy below Table. Frequency; Unwanted emission requirement. Below 55.62GHz; Equal to or less than -30dBm/MHz. Between 55.62GHz and 57GHz, Between 66GHz and 67.5GHz; Equal to or less than -26dBm/MHz. Above 67.5GHz; Equal to or less than -30dBm/MHz. |
| Note 5: The allowed value for occupied channel bandwidth (which is defined as bandwidth containing 99.5% of transmission power) is shown in below Table. Frequency; Occupied channel bandwidth. Between 60GHz and 61GHz; Equal to or less than 500MHz. Between 76GHz and 77GHz; Equal to or less than 1GHz. |
| Note 6: The antenna gain shall be less than 40dB. In addition, the unwanted emission shall satisfy below Table. Out-of-band domain emission limit; Spurious domain emission limit. Equal to or less than 100μW; Equal to or less than 50μW. |
| Note 7: The antenna absolute gain shall be less than 35dBi. In addition, the unwanted emission shall satisfy Table. Out-of-band domain emission limit; Spurious domain emission limit. Peak power shall be equal to or less than 100μW/MHz; Peak power shall be equal to or less than 50μW/MHz. |
| Note 8: The antenna absolute gain shall be less than 55dBi. In addition, the unwanted emission shall satisfy Table. Out-of-band domain emission limit; Spurious domain emission limit. Peak power shall be equal to or less than 100μW/MHz; Peak power shall be equal to or less than 50μW/MHz. |

Note *: Other radio stations may be used in the respective frequency bands.

- Conclusion

In this contribution, we provided an updated survey on Japanese spectrum usage and regulatory requirements in the frequency band above 52.6GHz. We propose to capture the survey to TR38.807.

References

- 3GPP, RP-182417, NTT DOCOMO, INC., “Survey on Japanese spectrum usage and regulatory requirements in the frequency band above 52.6GHz,” Dec. 2018.

- ARIB STD-B43 v2.1, “PORTABLE MILLIMETER-WAVE DIGITAL TRANSMISSION SYSTEM FOR TELEVISION PROGRAM CONTRIBUTION,” Jan. 2018.

- ARIB STD-B69 v4.0, “LOW POWER DATA COMMUNICATION SYSTEM/MILLIMETER-WAVE VIDEO TRANSMISSION EQUIPMENT,” Sept. 2016.

- ARIB STD-B117 v1.0, “LOW POWER DATA COMMUNICATION SYSTEM/60 GHz-BAND WIRELESS LAN FOR VERY HIGH THROUGHPUT DATA COMMUNICATIONS,” Sept. 2016.

- ARIB STD-B48 v2.2, “MILLIMETER-WAVE RADAR EQUIPMENT FOR SPECIFIED LOW POWER RADIO STATION,” Dec. 2015.

- ARIB STD-B111 v1.1, “79 GHz BAND HIGH-RESOLUTION RADAR,” Mar. 2017.