3GPP TSG-RAN WG4 Meeting #90	R4-1902135

# Athens, GR, 25 Feb - 1 Mar, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.101 | CR | 1113 | rev | 1 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NR protection in TS25.101 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-1-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of requirements for the NR FR1 new band of n77, n78 and n79 |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following requirements are updated. -6.6.3.1 Minimum requirement -6.6.3.1A Additional requirement for DC-HSUPA -6.6.3.1E Additional requirement for DB-DC-HSUPA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements of UTRA FDD UE protection to NR new band n77, n78 and n79 are not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS34.121-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is revision of R4-1900298 to resolve the coverpage issue. |  |  |  |  |  |  |  |  |

<Start of changes>

### 6.6.3	Spurious emissions

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emission, intermodulation products and frequency conversion products, but exclude out of band emissions.

The frequency boundary and the detailed transitions of the limits between the requirement for out band emissions and spectrum emissions are based on ITU-R Recommendations SM.329 [2].

#### 6.6.3.1	Minimum requirement

These requirements are only applicable for frequencies, which are greater than 12.5 MHz away from the UE centre carrier frequency.

Table 6.12: General spurious emissions requirements

| Frequency Bandwidth | Measurement Bandwidth | Minimum requirement | Note |
| --- | --- | --- | --- |
| 9 kHz f < 150 kHz | 1 kHz | -36 dBm |  |
| 150 kHz f < 30 MHz | 10 kHz | -36 dBm |  |
| 30 MHz f < 1000 MHz | 100 kHz | -36 dBm |  |
| 1 GHz f < 12.75 GHz | 1 MHz | -30 dBm |  |
| 12.75 GHz f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | 1 MHz | -30 dBm | Note 1 |
| NOTE 1: Applies only for Band XXII. |  |  |  |

Table 6.13: Additional spurious emissions requirements

| Operating Band | Frequency Bandwidth | Measurement Bandwidth | Minimum requirement |
| --- | --- | --- | --- |
| I | 460 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 703 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm * |
|  | 925 MHz f 935 MHz | 100 kHz 3.84MHz | -67 dBm * -60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz 3.84MHz | -79 dBm * -60 dBm |
|  | 1427 MHz  f  1518 MHz | 1 MHz | -50 dBm |
|  | 1452 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1805 MHz  f  1880 MHz | 100 kHz 3.84MHz | -71 dBm * -60 dBm |
|  | 1839.9 MHz  f  1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz <f< 1915.7 MHz | 300 kHz | -41 dBm |
|  | 2010 MHz <f< 2025 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz  f  2200 MHz | 1 MHz | -50 dBm |
|  | 2300 MHz f 2400 MHz | 3.84 MHz | -60 dBm |
|  | 2496 MHz  f  2570 MHz | 1 MHz | -50 dBm |
|  | 2570 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3300 MHz  f  3800 MHz | 1 MHz | -50 dBm |
|  | 3300 MHz  f 4200 MHz | 1 MHz | -50 dBm** |
|  | 4400 MHz f 5000 MHz | 1 MHz | -50 dBm |
| II | 617 MHz f 652 MHz | 1 MHz | -50 dBm |
|  | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 728 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 758 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 768 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1427 MHz  f  1518 MHz | 1 MHz | -50 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 1995 MHz f 2020 MHz | 1 MHz | -50 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz  f  2690 MHz | 1 MHz | -50 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm** |
| III | 460 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 703 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz  f  894 MHz | 3.84 MHz | -60 dBm ***** |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm * |
|  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -67 dBm * - 60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz 3.84 MHz | -79 dBm * - 60 dBm |
|  | 1427 MHz  f  1518 MHz | 1 MHz | -50 dBm |
|  | 1452 MHz  f  1496 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm ***** |
|  | 1805 MHz f 1880 MHz | 3.84 MHz | -60 dBm |
|  | 1880 MHz f 1920 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm ***** |
|  | 2010 MHz <f< 2025 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz  f  2200 MHz | 1 MHz | -50 dBm |
|  | 2300 MHz f 2400 MHz | 3.84 MHz | -60 dBm |
|  | 2496 MHz  f  2570 MHz | 1 MHz | -50 dBm |
|  | 2570 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm ** |
|  | 3300 MHz  f  4200 MHz | 1 MHz | -50 dBm ** |
|  | 4400 MHz  f 5000 MHz | 1 MHz | -50 dBm |
| IV | 617 MHz f 652 MHz | 1 MHz | -50 dBm |
|  | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 728 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 768 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1427 MHz  f  1518 MHz | 1 MHz | -50 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 1995 MHz f 2020 MHz | 1 MHz | -50 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz  f  2690 MHz | 1 MHz | -50 dBm |
|  | 2620 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm** |
| V | 460 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 617 MHz f 652 MHz | 1 MHz | -50 dBm |
|  | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 703 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 728 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 859 MHz f 869 MHz | 1 MHz | -27 dBm |
|  | 869 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1427 MHz  f  1518 MHz | 1 MHz | -50 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1805 MHz f 1880 MHz | 3.84 MHz | -60 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 1995 MHz f 2020 MHz | 1 MHz | -50 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2300 MHz f 2400 MHz | 3.84 MHz | -60 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz  f  2690 MHz | 1 MHz | -50 dBm** |
|  | 2570 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3300 MHz  f  3800 MHz | 1 MHz | -50 dBm |
|  | 3300 MHz  f  4200 MHz | 1 MHz | -50 dBm** |
|  | 4400 MHz  f 5000 MHz | 1 MHz | -50 dBm** |
| VI | 758 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 860 MHz  f < 875 MHz | 1 MHz | -37 dBm |
|  | 875 MHz  f  890 MHz | 3.84 MHz | -60 dBm |
|  | 945 MHz  f  960 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1839.9 MHz  f  1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm |
|  | 2010 MHz <f< 2025 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz  f  2200 MHz | 1 MHz | -50 dBm |
|  | 2545 MHz  f  2575 MHz | 1 MHz | -50 dBm |
|  | 2595 MHz  f  2645 MHz | 1 MHz | -50 dBm |
|  | 3400 MHz  f  3600 MHz | 1 MHz | -50 dBm |
| VII | 461 MHz f 466 MHz | 1 MHz | -50 dBm |
|  | 462.5 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 728 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 738 MHz f 758 MHz | 1 MHz | -50 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 768 MHz f 791 MHz | 1 MHz | -50 dBm |
|  | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm * |
|  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -67 dBm * -60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz 3.84 MHz | -79 dBm * -60 dBm |
|  | 1427 MHz < f  1518 MHz | 1 MHz | -50 dBm |
|  | 1452 MHz < f  1496 MHz | 3.84 MHz | -60 dBm |
|  | 1805 MHz  f  1880 MHz | 100 kHz 3.84 MHz | -71 dBm * -60 dBm |
|  | 1900 MHz f 1920 MHz | 3.84 MHz | -60 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 2010 MHz <f< 2025 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz  f  2200 MHz | 1 MHz | -50 dBm |
|  | 2300 MHz <f< 2400 MHz | 3.84 MHz | -60 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2620 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 2595 MHz  f  2620 MHz | 1 MHz | -40 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3300 MHz  f  4200 MHz | 1 MHz | -50 dBm |
| VIII | 460 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 703 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 860 MHz f 890 MHz | 1 MHz | -37 dBm **** |
|  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -67 dBm * -60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz 3.84 MHz | -79 dBm * -60 dBm |
|  | 1427 MHz  f  1518 MHz | 1 MHz | -50 dBm |
|  | 1452 MHz  f  1496 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm **** |
|  | 1805 MHz < f  1830 MHz | 100 kHz 3.84 MHz | -71 dBm ** & * -60 dBm ** |
|  | 1830 MHz < f  1880 MHz | 100 kHz 3.84 MHz | -71 dBm * -60 dBm |
|  | 1880MHz  f  1920 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f 1915.7 MHz | 300 kHz | -41 dBm **** |
|  | 2010 MHz  f  2025 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz  f  2200 MHz | 1 MHz | -50 dBm |
|  | 2300 MHz <f< 2400 MHz | 3.84 MHz | -60 dBm |
|  | 2496 MHz  f  2570 MHz | 1 MHz | -50 dBm |
|  | 2570 MHz  f  2640 MHz | 3.84 MHz | -60 dBm |
|  | 2640 MHz < f  2690 MHz | 3.84 MHz | -60 dBm ** |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm ** |
|  | 3300 MHz  f  4200 MHz | 1 MHz | -50 dBm ** |
|  | 4400 MHz f 5000 MHz | 1 MHz | -50 dBm ** |
| IX | 758 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 859 MHz  f  894 MHz | 3.84 MHz | -60 dBm |
|  | 945 MHz  f  960 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1839.9 MHz  f  1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm |
|  | 2010 MHz  f  2025 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz  f  2200 MHz | 1 MHz | -50 dBm |
|  | 2545 MHz  f  2575 MHz | 1 MHz | -50 dBm |
|  | 2595 MHz  f  2645 MHz | 1 MHz | -50 dBm |
|  | 3400 MHz  f  3600 MHz | 1 MHz | -50 dBm ** |
| X | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 728 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 768 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 1995 MHz f 2020 MHz | 1 MHz | -50 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz  f  2570 MHz | 1 MHz | -50 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm ** |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm ** |
| XI | 758 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 860 MHz  f  890 MHz | 3.84 MHz | -60 dBm |
|  | 945 MHz  f  960 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1839.9 MHz  f  1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm |
|  | 2010 MHz  f  2025 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz  f  2200 MHz | 1 MHz | -50 dBm |
|  | 2545 MHz  f  2575 MHz | 1 MHz | -50 dBm |
|  | 2595 MHz  f  2645 MHz | 1 MHz | -50 dBm |
|  | 3300 MHz  f  4200 MHz | 1 MHz | -50 dBm |
|  | 4400 MHz  f 5000 MHz | 1 MHz | -50 dBm |
| XII | 617 MHz f 652 MHz | 1 MHz | -50 dBm |
|  | 728 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1427 MHz  f  1518 MHz | 1 MHz | -50 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 1995 MHz f 2020 MHz | 1 MHz | -50 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz f 2690 MHz | 1 MHz | -50 dBm |
| XIII | 617 MHz f 652 MHz | 1 MHz | -50 dBm |
|  | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 728 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 763 MHz f 775 MHz | 6.25 kHz | -35 dBm*** |
|  | 793 MHz f 805 MHz | 6.25 kHz | -35 dBm*** |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1427 MHz  f  1518 MHz | 1 MHz | -50 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm** |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 1995 MHz f 2020 MHz | 1 MHz | -50 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm** |
|  | 2496 MHz f 2690 MHz | 1 MHz | -50 dBm |
| XIV | 617 MHz f 652 MHz | 1 MHz | -50 dBm |
|  | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 728 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 769 MHz f 775 MHz | 6.25 kHz | -35 dBm *** |
|  | 799 MHz f 805 MHz | 6.25 kHz | -35 dBm *** |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 1995 MHz f 2020 MHz | 1 MHz | -50 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz f 2690 MHz | 1 MHz | -50 dBm |
| XIX | 758 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 860 MHz  f < 875 MHz | 1 MHz | -37 dBm |
|  | 875 MHz  f  890 MHz | 3.84 MHz | -60 dBm |
|  | 945 MHz  f  960 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1839.9 MHz  f  1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm |
|  | 2010 MHz  f  2025 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2545 MHz  f  2575 MHz | 1 MHz | -50 dBm |
|  | 2595 MHz  f  2645 MHz | 1 MHz | -50 dBm |
|  | 3400 MHz  f  3600 MHz | 1 MHz | -50 dBm |
|  | 3300 MHz  f  4200 MHz | 1 MHz | -50 dBm** |
|  | 4400 MHz  f 5000 MHz | 1 MHz | -50 dBm |
| XX | 461 MHz f 466 MHz | 1 MHz | -50 dBm |
|  | 738 MHz f 758 MHz | 1 MHz | -50 dBm |
|  | 758 MHz f 788 MHz | 1 MHz | -50 dBm |
|  | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm * |
|  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -67 dBm * -60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz 3.84 MHz | -79 dBm * -60 dBm |
|  | 1427 MHz < f  1518 MHz | 1 MHz | -50 dBm |
|  | 1452 MHz < f  1496 MHz | 3.84 MHz | -60 dBm |
|  | 1805 MHz  f  1880 MHz | 100 kHz 3.84 MHz | -71 dBm * -60 dBm |
|  | 2010 MHz  f  2025 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz  f  2200 MHz | 1 MHz | -50 dBm |
|  | 2300 MHz  f  2400 MHz | 3.84 MHz | -60 dBm |
|  | 2620 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 2570 MHz  f  2620 MHz | 3.84 MHz | -60 dBm** |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3300 MHz  f  4200 MHz | 1 MHz | -50 dBm ** |
| XXI | 758 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 860 MHz  f  890 MHz | 3.84 MHz | -60 dBm |
|  | 945 MHz  f  960 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 1 MHz | -35 dBm |
|  | 1839.9 MHz  f  1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm |
|  | 2010 MHz  f  2025 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz  f  2200 MHz | 1 MHz | -50 dBm |
|  | 2545 MHz  f  2575 MHz | 1 MHz | -50 dBm |
|  | 2595 MHz  f  2645 MHz | 1 MHz | -50 dBm |
|  | 3300 MHz  f  4200 MHz | 1 MHz | -50 dBm |
|  | 4400 MHz  f 5000 MHz | 1 MHz | -50 dBm |
| XXII | 461 MHz f 466 MHz | 1 MHz | -50 dBm |
|  | 738 MHz f 758 MHz | 1 MHz | -50 dBm |
|  | 758 MHz f 791 MHz | 1 MHz | -50 dBm |
|  | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm * |
|  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -67 dBm * -60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz 3.84 MHz | -79 dBm * -60 dBm |
|  | 1452 MHz < f  1496 MHz | 3.84 MHz | -60 dBm |
|  | 1805 MHz  f  1880 MHz | 100 kHz 3.84 MHz | -71 dBm * -60 dBm |
|  | 1880 MHz  f  1920 MHz | 3.84 MHz | -60 dBm |
|  | 2010 MHz  f  2025 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz  f  2200 MHz | 1 MHz | -50 dBm |
|  | 2300 MHz  f  2400 MHz | 3.84 MHz | -60 dBm |
|  | 2620 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 2570 MHz  f  2620 MHz | 3.84 MHz | -60 dBm |
|  | 3510 MHz  f  3525 MHz | 1 MHz | -40 dBm |
|  | 3525 MHz  f  3590 MHz | 1 MHz | -50 dBm |
|  | 3600 MHz  f  3800 MHz | 3.84 MHz | -50 dBm |
| XXV | 617 MHz f 652 MHz | 1 MHz | -50 dBm |
|  | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 728 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 768 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 3.84 MHz | -60 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 1995 MHz f 2020 MHz | 1 MHz | -50 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz f 2690 MHz | 1 MHz | -50 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3400 MHz f 3800 MHz | 1 MHz | -50 dBm ** |
| XXVI | 460 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 617 MHz f 652 MHz | 1 MHz | -50 dBm |
|  | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 728 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 768 MHz f 799 MHz | 1 MHz | -50 dBm |
|  | 799 MHz f 803 MHz | 1 MHz | -40 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 945 MHz  f  960 MHz | 3.84 MHz | -60 dBm |
|  | 1427 MHz  f  1518 MHz | 1 MHz | -50 dBm |
|  | 1475.9 MHz f 1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1805 MHz  f  1880 MHz | 3.84 MHz | -60 dBm |
|  | 1839.9 MHz f 1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 1995 MHz f 2020 MHz | 1 MHz | -50 dBm |
|  | 2010 MHz f 2025 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2300 MHz f 2400 MHz | 3.84 MHz | -60 dBm |
|  | 2496 MHz f 2690 MHz | 1 MHz | -50 dBm ** |
|  | 3400 MHz f 3800 MHz | 1 MHz | -50 dBm |
|  | 3300 MHz  f 4200 MHz | 1 MHz | -50 dBm** |
| Note * The transmitter additional spurious emission measurements are made on frequencies which are integer multiples of 200 kHz. As exceptions, up to five measurements with a level up to the applicable requirements defined in Table 6.12 are permitted for each UARFCN used in the measurement. Note ** The transmitter additional spurious emission measurements are made on frequencies which are integer multiples of 200 kHz. As exceptions, measurements with a level up to the applicable requirements defined in Table 6.12 are permitted for each UARFCN used in the measurement due to 2nd, 3rd and 4th harmonic spurious emissions. Note *** This requirement is applicable also for frequencies, which are between 2.5 MHz and 12.5 MHz away from the UE transmit centre carrier frequency. Note **** This requirement is applicable only when UE transmission is allocated within 900MHz to 915MHz. Note ***** This requirement is applicable only when UE transmission is allocated within 1744.9 MHz to 1784.9 MHz. |  |  |  |

<Unchanged subclause skipped>

#### 6.6.3.1A	Additional requirement for DC-HSUPA

The requirements in Table 6.12A are only applicable for frequencies, which are greater than 20 MHz away from the centre of the assigned carrier frequencies when dual adjacent carriers are assigned on the uplink.

Table 6.12A: General spurious emissions requirements for DC-HSUPA

| Frequency Bandwidth | Measurement Bandwidth | Minimum requirement | Note |
| --- | --- | --- | --- |
| 9 kHz f < 150 kHz | 1 kHz | -36 dBm |  |
| 150 kHz f < 30 MHz | 10 kHz | -36 dBm |  |
| 30 MHz f < 1000 MHz | 100 kHz | -36 dBm |  |
| 1 GHz f < 12.75 GHz | 1 MHz | -30 dBm |  |
| 12.75 GHz f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | 1 MHz | -30 dBm | Note 1 |
| NOTE 1: Applies only for Band XXII. |  |  |  |

The requirements in Table 6.13A are only applicable for frequencies, which are greater than 25 MHz away from the centre of the assigned frequencies when dual adjacent carriers are assigned on the uplink.

Table 6.13A: Additional spurious emissions requirements for DC-HSUPA

| Operating Band | Frequency Bandwidth | Measurement Bandwidth | Minimum requirement |
| --- | --- | --- | --- |
| I | 460 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 703 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm * |
|  | 925 MHz f 935 MHz | 100 kHz 3.84MHz | -67 dBm * -60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz 3.84MHz | -79 dBm * -60 dBm |
|  | 1427 MHz  f  1518 MHz | 1 MHz | -50 dBm |
|  | 1452 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1805 MHz  f  1880 MHz | 100 kHz 3.84MHz | -71 dBm * -60 dBm |
|  | 1844.9 MHz  f  1879.9 MHz | 3.84 MHz | -55 dBm |
|  | 1884.5 MHz <f< 1915.7 MHz | 300 kHz | -41 dBm |
|  | 2010 MHz <f< 2025 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz  f  2200 MHz | 1 MHz | -50 dBm |
|  | 2300 MHz <f< 2400 MHz | 3.84 MHz | -60 dBm |
|  | 2496 MHz  f  2570 MHz | 1 MHz | -50 dBm |
|  | 2570 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3300 MHz  f  3800 MHz | 1 MHz | -50 dBm |
|  | 3300 MHz  f 4200 MHz | 1 MHz | -50 dBm** |
|  | 4400 MHz f 5000 MHz | 1 MHz | -50 dBm |
| II | 617 MHz f 652 MHz | 1 MHz | -50 dBm |
|  | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 728 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 758 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 768 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1427 MHz  f  1518 MHz | 1 MHz | -50 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 1995 MHz f 2020 MHz | 1 MHz | -50 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz  f  2620 MHz | 1 MHz | -50 dBm |
| III | 460 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 703 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 852 MHz f 869 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm * |
|  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -67 dBm * - 60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz 3.84 MHz | -79 dBm - 60 dBm * |
|  | 1427 MHz  f  1518 MHz | 1 MHz | -50 dBm |
|  | 1452 MHz < f  1496 MHz | 3.84 MHz | -60 dBm |
|  | 1805 MHz f 1880 MHz | 3.84 MHz | -60 dBm |
|  | 1880 MHz f 1920 MHz | 3.84 MHz | -60 dBm |
|  | 2010 MHz f 2025 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz  f  2200 MHz | 1 MHz | -50 dBm |
|  | 2300 MHz f 2400 MHz | 3.84 MHz | -60 dBm |
|  | 2496 MHz  f  2570 MHz | 1 MHz | -50 dBm |
|  | 2570 MHz  f  2620 MHz | 3.84 MHz | -60 dBm |
|  | 2620 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm ** |
|  | 3300 MHz  f  4200 MHz | 1 MHz | -50 dBm ** |
|  | 4400 MHz  f 5000 MHz | 1 MHz | -50 dBm |
| IV | 617 MHz f 652 MHz | 1 MHz | -50 dBm |
|  | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 728 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 768 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1427 MHz  f  1518 MHz | 1 MHz | -50 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 1995 MHz f 2020 MHz | 1 MHz | -50 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz  f  2690 MHz | 1 MHz | -50 dBm |
|  | 2620 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
| V | 460 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 617 MHz f 652 MHz | 1 MHz | -50 dBm |
|  | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 703 MHz f 803 MHz | 3.84 MHz | -50 dBm |
|  | 728 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 859 MHz f 869 MHz | 1 MHz | -27 dBm |
|  | 869 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1427 MHz  f  1518 MHz | 1 MHz | -50 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1805 MHz f 1880 MHz | 3.84 MHz | -60 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 1995 MHz f 2020 MHz | 1 MHz | -50 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz  f  2200 MHz | 1 MHz | -50 dBm |
|  | 2170 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2300 MHz f 2400 MHz | 3.84 MHz | -60 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz  f  2620 MHz | 1 MHz | -50 dBm ** |
|  | 2570 MHz  f  2690 MHz | 1 MHz | -50 dBm ** |
|  | 3300 MHz  f  3400 MHz | 1 MHz | -50 dBm |
|  | 3300 MHz  f  4200 MHz | 1 MHz | -50 dBm** |
|  | 4400 MHz  f 5000 MHz | 1 MHz | -50 dBm** |
| VI | 860 MHz  f < 875 MHz | 1 MHz | -37 dBm |
|  | 875 MHz  f  890 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1844.9 MHz  f  1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm |
|  | 2010 MHz <f< 2025 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz  f  2200 MHz | 1 MHz | -50 dBm |
|  | 2545 MHz  f  2575 MHz | 1 MHz | -50 dBm |
|  | 2595 MHz  f  2645 MHz | 1 MHz | -50 dBm |
|  | 3400 MHz  f  3600 MHz | 1 MHz | -50 dBm |
| VII | 462.5 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 728 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 738 MHz f 758 MHz | 1 MHz | -50 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 791 MHz | 1 MHz | -50 dBm |
|  | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 852 MHz f 869 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm * |
|  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -67 dBm * -60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz | -79 dBm * |
|  | 1427 MHz < f  1518 MHz | 1 MHz | -50 dBm |
|  | 1452 MHz < f  1496 MHz | 3.84 MHz | -60 dBm |
|  | 1805 MHz  f  1880 MHz | 100 kHz | -71 dBm * |
|  | 1900 MHz f 1920 MHz | 3.84 MHz | -60 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 1995 MHz f 2020 MHz | 1 MHz | -50 dBm |
|  | 2010 MHz <f< 2025 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz  f  2200 MHz | 1 MHz | -50 dBm |
|  | 2300 MHz <f< 2400 MHz | 3.84 MHz | -60 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2620 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 2595 MHz  f  2620 MHz | 1 MHz | -37 dBm |
|  | 3300 MHz  f  4200 MHz | 1 MHz | -50 dBm |
| VIII | 460 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 703 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -57 dBm *, *** -50 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz 3.84 MHz | -79 dBm * -60 dBm |
|  | 1427 MHz  f  1518 MHz | 1 MHz | -50 dBm |
|  | 1452 MHz < f  1496 MHz | 3.84 MHz | -60 dBm |
|  | 1805 MHz < f  1830 MHz | 100 kHz 3.84 MHz | -71 dBm ** & * -60 dBm ** |
|  | 1830 MHz < f  1880 MHz | 100 kHz 3.84 MHz | -71 dBm * -60 dBm |
|  | 1880MHz  f  1920 MHz | 3.84 MHz | -60 dBm |
|  | 2010MHz  f  2025 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz  f  2200 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz  f  2570 MHz | 1 MHz | -50 dBm |
|  | 2300MHz  f  2400 MHz | 3.84 MHz | -60 dBm |
|  | 2570 MHz f 2620 MHz | 3.84 MHz | -60 dBm |
|  | 2620 MHz  f  2640 MHz | 3.84 MHz | -60 dBm |
|  | 2640 MHz < f  2690 MHz | 3.84 MHz | -60 dBm ** |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm ** |
|  | 3300 MHz  f  4200 MHz | 1 MHz | -50 dBm ** |
|  | 4400 MHz f 5000 MHz | 1 MHz | -50 dBm ** |
| IX | 758 MHz  f  803 MHz | 1 MHz | -50 dBm |
|  | 859 MHz  f  894 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1844.9 MHz  f  1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm |
|  | 2010 MHz  f  2025 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz  f  2200 MHz | 1 MHz | -50 dBm |
|  | 2545 MHz  f  2575 MHz | 1 MHz | -50 dBm |
|  | 2595 MHz  f  2645 MHz | 1 MHz | -50 dBm |
|  | 3400 MHz  f  3600 MHz | 1 MHz | -50 dBm ** |
| X | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 728 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 768 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 1995 MHz f 2020 MHz | 1 MHz | -50 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz  f  2570 MHz | 1 MHz | -50 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm ** |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm ** |
| XI | 758 MHz  f  803 MHz | 1 MHz | -50 dBm |
|  | 860 MHz  f  890 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1844.9 MHz  f  1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm |
|  | 2010 MHz  f  2025 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz  f  2200 MHz | 1 MHz | -50 dBm |
|  | 2545 MHz  f  2575 MHz | 1 MHz | -50 dBm |
|  | 2595 MHz  f  2645 MHz | 1 MHz | -50 dBm |
|  | 3300 MHz  f  4200 MHz | 1 MHz | -50 dBm |
|  | 4400 MHz f 5000 MHz | 1 MHz | -50 dBm |
| XIX | 758 MHz  f  803 MHz | 1 MHz | -50 dBm |
|  | 860 MHz  f < 875 MHz | 1 MHz | -30 dBm |
|  | 875 MHz  f  890 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1844.9 MHz  f  1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm |
|  | 2010 MHz  f  2025 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz  f  2200 MHz | 1 MHz | -50 dBm |
|  | 2545 MHz  f  2575 MHz | 1 MHz | -50 dBm |
|  | 2595 MHz  f  2645 MHz | 1 MHz | -50 dBm |
|  | 3400 MHz  f  3600 MHz | 1 MHz | -50 dBm |
|  | 3300 MHz  f  4200 MHz | 1 MHz | -50 dBm** |
|  | 4400 MHz  f 5000 MHz | 1 MHz | -50 dBm |
| XX | 738 MHz f 758 MHz | 1 MHz | -50 dBm |
|  | 758 MHz  f  788 MHz | 1 MHz | -50 dBm |
|  | 811 MHz f 821 MHz | 3.84 MHz | -50 dBm *** |
|  | 791 MHz f 811 MHz | 3.84 MHz | -60 dBm |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm * |
|  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -67 dBm * -60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz 3.84 MHz | -79 dBm * -60 dBm |
|  | 1427 MHz < f  1518 MHz | 1 MHz | -50 dBm |
|  | 1452 MHz < f  1496 MHz | 3.84 MHz | -60 dBm |
|  | 1805 MHz  f  1880 MHz | 100 kHz 3.84 MHz | -71 dBm * -60 dBm |
|  | 2010 MHz  f  2025 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz  f  2200 MHz | 1 MHz | -50 dBm |
|  | 2300 MHz  f  2400 MHz | 3.84 MHz | -60 dBm |
|  | 2620 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 2570 MHz  f  2620 MHz | 3.84 MHz | -60 dBm ** |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3300 MHz  f  4200 MHz | 1 MHz | -50 dBm ** |
| XXII | 738 MHz f 758 MHz | 1 MHz | -50 dBm |
|  | 758 MHz  f  791 MHz | 1 MHz | -50 dBm |
|  | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 852 MHz f 869 MHz | 1 MHz | -50 dBm |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm * |
|  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -67 dBm * -60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz 3.84 MHz | -79 dBm * -60 dBm |
|  | 1452 MHz < f  1496 MHz | 3.84 MHz | -60 dBm |
|  | 1805 MHz  f  1880 MHz | 100 kHz 3.84 MHz | -71 dBm * -60 dBm |
|  | 1880 MHz  f  1920 MHz | 3.84 MHz | -60 dBm |
|  | 2010 MHz  f  2025 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz  f  2200 MHz | 1 MHz | -50 dBm |
|  | 2300 MHz  f  2400 MHz | 3.84 MHz | -60 dBm |
|  | 2620 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 2570 MHz  f  2620 MHz | 3.84 MHz | -60 dBm |
|  | 3510 MHz  f  3525 MHz | 1 MHz | -40 dBm |
|  | 3525 MHz  f  3590 MHz | 1 MHz | -50 dBm |
|  | 3600 MHz f  3800 MHz | 3.84 MHz | -50 dBm |
| XXV | 617 MHz f 652 MHz | 1 MHz | -50 dBm |
|  | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 728 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 768 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 3.84 MHz | -60 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 1995 MHz f 2020 MHz | 1 MHz | -50 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz f 2690 MHz | 1 MHz | -50 dBm |
|  | 3400 MHz f 3800 MHz | 1 MHz | -50 dBm |
| XXVI | 460 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 617 MHz f 652 MHz | 1 MHz | -50 dBm |
|  | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 728 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 768 MHz f 799 MHz | 1 MHz | -50 dBm |
|  | 799 MHz f 803 MHz | 1 MHz | -40 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1427 MHz  f  1518 MHz | 1 MHz | -50 dBm |
|  | 1475.9 MHz f 1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1805 MHz  f  1880 MHz | 3.84 MHz | -60 dBm |
|  | 1844.9 MHz f 1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f 1915.7 MHz | 300 kHz | -41 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 1995 MHz f 2020 MHz | 1 MHz | -50 dBm |
|  | 2010 MHz f 2025 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2170 MHz  f  2200 MHz | 1 MHz | -50 dBm |
|  | 2170 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2300 MHz f 2400 MHz | 3.84 MHz | -60 dBm |
|  | 2496 MHz f 2690 MHz | 1 MHz | -50 dBm ** |
|  | 3400 MHz f 3800 MHz | 1 MHz | -50 dBm |
|  | 3300 MHz  f 4200 MHz | 1 MHz | -50 dBm** |
| Note * The measurements are made on frequencies which are integer multiples of 200 kHz. As exceptions, up to five measurements with a level up to the applicable requirements defined in Table 6.12 are permitted for each UARFCN used in the measurement Note ** The measurements are made on frequencies which are integer multiples of 200 kHz. As exceptions, measurements with a level up to the applicable requirements defined in Table 6.12 are permitted for each UARFCN used in the measurement due to 2nd, 3rd and 4th harmonic spurious emissions Note *** This requirement is applicable also for frequencies, which are between 5 MHz and 25 MHz away from the UE centre carrier frequency. |  |  |  |

<Unchanged subclause skipped>

#### 6.6.3.1E	Additional requirement for DB-DC-HSUPA

For UE supporting DB-DC-HSUPA operation, the spurious emission requirements in Table 6.13C and Table 6.13D apply for frequencies, which are greater than 12.5MHz away from the UE centre carrier frequency for each carrier. If for some frequency a spurious emission requirement of individual carrier overlaps with the spectrum emission mask or channel bandwidth of another carrier then it does not apply.

Table 6.13C: General spurious emissions requirements for DB-DC-HSUPA

| Frequency Bandwidth | Measurement Bandwidth | Minimum requirement | Note |
| --- | --- | --- | --- |
| 9 kHz f < 150 kHz | 1 kHz | -36 dBm |  |
| 150 kHz f < 30 MHz | 10 kHz | -36 dBm |  |
| 30 MHz f < 1000 MHz | 100 kHz | -36 dBm |  |
| 1 GHz f < 12.75 GHz | 1 MHz | -30 dBm |  |

Table 6.13D: Additional spurious emissions requirements for DB-DC-HSUPA

| Operating Band combination | Frequency Bandwidth | Measurement Bandwidth | Minimum requirement |
| --- | --- | --- | --- |
| I-1-V-1 I-1-V-2 I-2-V-1 I-2-V-2 | 460 MHz  f  467.5 MHz | 1 MHz | -50 dBm |
|  | 703 MHz  f  803 MHz | 1 MHz | -50 dBm |
|  | 859 MHz  f  869 MHz | 1 MHz | -27 dBm |
|  | 869 MHz  f  894 MHz | 3.84 MHz | -60 dBm |
|  | 1427 MHz  f  1518 MHz | 1 MHz | -50 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2496 MHz  f  2570 MHz | 1 MHz | -50 dBm** |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3300 MHz  f  3800 MHz | 1 MHz | -50 dBm |
|  | 3300 MHz  f 4200 MHz | 1 MHz | -50 dBm** |
|  | 4400 MHz f 5000 MHz | 1 MHz | -50 dBm** |
| I-1-VIII-1 I-2-VIII-1 I-2-VIII-2 I-1-VIII-2 I-1-VIII-3 | 460 MHz  f  467.5 MHz | 1 MHz | -50 dBm |
|  | 703 MHz  f  803 MHz | 1 MHz | -50 dBm |
|  | 791 MHz  f  821 MHz | 3.84 MHz | -60 dBm |
|  | 859 MHz  f  894 MHz | 3.84 MHz | -60 dBm |
|  | 860 MHz  f  890 MHz | 1 MHz | -37 dBm *** |
|  | 925 MHz f 935 MHz | 100 kHz | -67 dBm * |
|  |  | 3.84MHz | -60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz | -79 dBm * |
|  |  | 3.84 MHz | -60 dBm |
|  | 1427MHz  f  1518 MHz | 1 MHz | -50 dBm |
|  | 1452MHz  f  1496 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm *** |
|  | 1805 MHz < f  1830 MHz | 100 kHz | -71 dBm ** & * |
|  |  | 3.84 MHz | -60 dBm ** |
|  | 1830 MHz < f  1880 MHz | 100 kHz | -71 dBm * |
|  |  | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f 1915.7 MHz | 300 kHz | -41 dBm *** |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2496 MHz  f  2570 MHz | 1 MHz | -50 dBm |
|  | 2570 MHz  f  2640 MHz | 3.84 MHz | -60 dBm |
|  | 2640 MHz < f  2690 MHz | 3.84 MHz | -60 dBm ** |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm ** |
|  | 3300 MHz  f  4200 MHz | 1 MHz | -50 dBm ** |
|  | 4400 MHz  f  5000 MHz | 1 MHz | -50 dBm ** |
| II-1-V-1 II-1-V-2 | 617 MHz f 652 MHz | 1 MHz | -50 dBm |
|  | 717 MHz  f  728 MHz | 1 MHz | -50 dBm |
|  | 728 MHz  f  746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz  f  756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz  f  768 MHz | 3.84 MHz | -60 dBm |
|  | 859 MHz  f  869 MHz | 1 MHz | -27 dBm |
|  | 869 MHz  f  894 MHz | 3.84 MHz | -60 dBm |
|  | 1427 MHz  f 1518 MHz | 1 MHz | -50 dBm |
|  | 1525 MHz  f  1559 MHz | 1 MHz | -50 dBm |
|  | 1930 MHz  f  1995 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2180 MHz  f  2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz  f  2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz  f  2690 MHz | 1 MHz | -50 dBm** |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm |
| Note *: The measurements are made on frequencies which are integer multiples of 200 kHz. As exceptions, up to five measurements with a level up to the applicable requirements defined in Table 6.13C are permitted for each UARFCN used in the measurement Note **: The measurements are made on frequencies which are integer multiples of 200 kHz. As exceptions, measurements with a level up to the applicable requirements defined in Table 6.13C are permitted for each UARFCN used in the measurement due to 2nd, 3rd and 4th harmonic spurious emissions Note ***: This requirement is applicable only when transmission is made between 900MHz to 915MHz. |  |  |  |

<End of changes>