3GPP TSG-RAN4 Meeting #90	R4-1902523

# Athens, Greece, 25th February – 1st March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6287 | rev | 1 | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Applicability rule for 8Rx RLM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_8Rx_AP_DL-Perf |  |  |  |  |  | Date: |  |  | 2019-02-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is agreed in [RP-182551] - Applicability rule for RLM - For an 8Rx-capable UE with 2Rx supported band, RLM requirement is tested on 2Rx-supported band. - For an 8Rx-capable UE with 4Rx supported band but without any 2Rx supported band, RLM requirement is tested on 4Rx supported band. - Existing applicability rules for 4Rx UE apply. - For an 8Rx-capable UE without any 2Rx or 4Rx supported bands, RLM requirement is tested on 8Rx-supported band - Existing applicability rules for 4Rx UE apply. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The applicability rule for 8rx RLM is specified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The gap switching time are imcompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.3.8	Antenna Configuration

Unless otherwise specified, E-UTRA FDD or E-UTRA TDD cells in all RRM Test cases in AWGN propagation condition are configured with Antenna Configuration 1x2.

A.3.8.1	Antenna connection for 4 Rx capable UEs

A.3.8.1.1 Introduction

All tests in sections A.4 to A.9 are specified for UEs supporting either category 0 (1RX) or 2RX. In this section, the antenna connection method for applying 2RX tests to UEs supporting 4RX antenna ports is specified. No tests are currently specified in section A.4-A.9 which are applicable only to 4RX antenna ports, so 4RX capable UEs are always tested by reusing tests which were originally specified for 2RX UEs.

A.3.8.1.2 Principle of testing

A.3.8.1.2.1 Single carrier tests

For 4RX capable UEs supporting at least one 2RX band, all single carrier tests specified in section A.4 to A.8 shall be tested on any band where 2RX is supported with the antenna connection specified in A.8.3.1.2.3. For single carrier tests specified in section A.9, all tests shall be tested with the antenna connection specified in A.3.8.1.2.3 for bands where 2RX is supported, and the antenna connection specified in A.3.8.1.2.4 for bands where 4RX is supported.

For 4RX capable UEs which do not support any 2RX band, all tests specified in sections A.4 to A.9 shall be tested using the antenna connection specified in section A.3.8.1.2.4. For radio link monitoring tests, the SNR levels are modified according to table A.3.8.1.2.1-1 and table A.3.8.1.2.1-2.

Table A.3.8.1.2.1-1 Modified parameters for RLM out of sync testing with 4 RX antenna connection

| Test case | SNR during T3 (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 3 | Test 4 |
| A.7.3.1 | -17 | -17 | -15 | -15.7 |
| A.7.3.3 | -16.6 | -16.7 | -14.8 | -15.4 |
| A.7.3.5 | -15.7 | -17 | N/A | N/A |
| A.7.3.7 | -15.4 | -16.6 | N/A | N/A |
| A.7.3.9 (cell 1) | Note 1 | N/A | N/A | N/A |
| A.7.3.10 (cell 1) | Note 1 | N/A | N/A | N/A |
| A.7.3.13 (cell 1) | Note 1 | N/A | N/A | N/A |
| A.7.3.14 (cell 1) | Note 1 | N/A | N/A | N/A |
| A.7.3.17 (cell 1) | Note 1 | N/A | N/A | N/A |
| A.7.3.18 (cell 1) | Note 1 | N/A | N/A | N/A |
| A.7.3.23 | -15.7 | N/A | N/A | N/A |
| Note 1: For 4Rx capable UEs without any 2Rx supported RF bands,this test can be skipped. |  |  |  |  |

Table A.3.8.1.2.1-2 Modified parameters for RLM in sync single carrier testing with 4 RX antenna connection

| Test case | SNR during T3 (dB) |  | SNR during T4 (dB) |  |
| --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 1 | Test 2 |
| A.7.3.2 | -14 | -15.7 | -9.9 | -10.8 |
| A.7.3.4 | -14.8 | -15.4 | -9.9 | -10.8 |
| A.7.3.6 | -17 | N/A | -12.2 | N/A |
| A.7.3.8 | -16.6 | N/A | -12.6 | N/A |
| A.7.3.11 (cell 1) | Note 1 | N/A | Note 1 | N/A |
| A.7.3.12 (cell 1) | Note 1 | N/A | Note 1 | N/A |
| A.7.3.15 (cell 1) | Note 1 | N/A | Note 1 | N/A |
| A.7.3.16 (cell 1) | Note 1 | N/A | Note 1 | N/A |
| A.7.3.19 (cell 1) | Note 1 | N/A | Note 1 | N/A |
| A.7.3.20 (cell 1) | Note 1 | N/A | Note 1 | N/A |
| A.7.3.21(cell 1) | Note 1 | N/A | Note 1 | N/A |
| A.7.3.22 (cell 1) | Note 1 | N/A | Note 1 | N/A |
| A.7.3.24 | -15.7 | N/A | -10.8 | N/A |
| A.7.3.25 | -15.7 | N/A | -10.8 | N/A |
| Note 1: For 4Rx capable UEs without any 2Rx supported RF bands,this test can be skipped. |  |  |  |  |

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests

All carrier aggregation and dual connectivity tests are performed using the antenna connection in section A.3.8.1.2.3 for the PCell antenna connection if the PCell is on a band where 2RX is supported or the antenna connection in A.3.8.1.2.4 for the PCell antenna connection if the PCell is on a band where 4RX is supported.

All carrier aggregation and dual connectivity tests are performed using the antenna connection in section A.3.8.1.2.3 for the SCell or PSCell antenna connection if an SCell or PSCell is on band where 2RX is supported or the testing procedure in A.3.8.1.2.4 for the SCell or PSCell antenna connection if an SCell or PSCell is on a band where 4RX is supported.

For dual connectivity radio link monitoring tests with the PSCell on a band where 4RX is supported, the PSCell SNR levels are modified according to table A.3.8.1.2.2 -1 and table A.3.8.1.2.2 -2.

Tabke A.3.8.1.2.1-1 Modified parameters for dual connectivity RLM out of sync testing with 4 RX antenna connection

| Test case | SNR during T3 (dB) |
| --- | --- |
| A.7.3.38 (cell 2) | -15.7 (5MHz) -15.7 (10MHz) -16.3 (20 MHz) |
| A.7.3.39 (cell 2) | -15.7 (5MHz) -15.7 (10MHz) -16.3 (20 MHz) |
| A.7.3.40 (cell 2) | -15.4 (5MHz) -15.4 (10MHz) -16.1 (20MHz) |

Table A.3.8.1.2.1-1 Modified parameters for RLM out of sync testing with 4 RX antenna connection

| Test case | SNR during T3 (dB) | SNR during T4 (dB) |
| --- | --- | --- |
| A.7.3.41 (cell 2) | -15.7 (5 Mhz) -17.0 (10 Mhz) -17.0 (20 Mhz) | -10.8 (5MHz) -12.2 (10MHz) -12.2 (20 MHz) |
| A.7.3.42 (cell 2) | -15.7 (5 Mhz) -17.0 (10 Mhz) -17.0 (20 Mhz) | -10.8 (5MHz) -12.2 (10MHz) -12.2 (20 MHz) |
| A.7.3.43 (cell 2) | -16.6 (5MHz) -16.6 (10 MHz) -16.6 (20 MHz) | -12.6 (5MHz) -12.6 (10 MHz) -12.6 (20 MHz) |

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported

For bands where 2RX is supported, it is left to the UE declaration and AP configuration to decide which 2 of the 4 Rx ports are connected with data source from system simulator. The remaning 2 Rx ports shall be connected with zero input. No test parameters or requirements are modified.

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported

For bands where 4RX is supporetd, all 4 Rx are connected with data source from system simulator. The system simulator shall provide independent noise and fading (low correlation) for each antenna port. Except for the modifications to radio link monitoring theresholds described in sections A.3.8.1.2.1 and A.3.8.1.2.2, no test parameters or requirements are modified.

A.3.8.2	Antenna connection for 8 Rx capable UEs

A.3.8.2.1 Introduction

In this section, the antenna connection method for applying 2RX tests or 4RX tests to UEs supporting 8RX antenna ports is specified. No tests are currently specified in section A.4-A.9 which are applicable only to 8RX antenna ports, so 8RX capable UEs are always tested by reusing tests which were originally specified for 2RX UEs or 4Rx UEs.

A.3.8.2.2 Principle of testing

A.3.8.2.2.1 Single carrier tests

For 8RX capable UEs supporting at least one 2RX band, all single carrier tests specified in section A.4 to A.8 shall be tested on any band where 2RX is supported with the antenna connection specified in A.8.3.2.2.3. For single carrier tests specified in section A.9, all tests shall be tested with the antenna connection specified in A.3.8.2.2.3 for bands where 2RX is supported, and the antenna connection specified in A.3.8.2.2.5 for bands where 8RX is supported.

For 8RX capable UEs supporting at least one 4RX band but without supporting any 2RX band, all single carrier tests specified in section A.4 to A.8 shall be tested on any band where 4RX is supported with the antenna connection specified in A.3.8.2.2.4. For single carrier tests specified in section A.9, all tests shall be tested with the antenna connection specified in A.3.8.2.2.4 for bands where 4RX is supported, and the antenna connection specified in A.3.8.2.2.8 for bands where 8RX is supported. For radio link monitoring tests, the SNR levels are modified according to table A.3.8.1.2.1-1 and table A.3.8.1.2.1-2.

For 8RX capable UEs which do not support any 2RX or 4RX band, all tests specified in sections A.4 to A.9 shall be tested using the antenna connection specified in section A.3.8.1.2.5. For radio link monitoring tests, the SNR levels are modified according to table A.3.8.1.2.1-1 and table A.3.8.1.2.1-2.

A.3.8.2.2.2	Carrier aggregation and Dual connectivity tests

All carrier aggregation and dual connectivity tests are performed using the antenna connection in section A.3.8.2.2.3 for the PCell antenna connection if the PCell is on a band where 2RX is supported or the antenna connection in A.3.8.2.2.4 for the PCell antenna connection if the PCell is on a band where 4RX is supported but without supporting any 2RX band or the antenna connection in A.3.8.2.2.5 for the PCell antenna connection if the PCell is on a band where 8RX is supported.

All carrier aggregation and dual connectivity tests are performed using the antenna connection in section A.3.8.2.2.3 for the SCell or PSCell antenna connection if an SCell or PSCell is on band where 2RX is supported or the testing procedure in A.3.8.2.2.4 for the SCell or PSCell antenna connection if an SCell or PSCell is on a band where 4RX is supported but without supporting any 2RX band or the testing procedure in A.3.8.2.2.5 for the SCell or PSCell antenna connection if an SCell or PSCell is on a band where 8RX is supported.

For dual connectivity radio link monitoring tests with the PSCell on a band where 8RX is supported, the PSCell SNR levels are modified according to table A.3.8.1.2.2 -1 and table A.3.8.1.2.2 -2.

A.3.8.2.2.3	Antenna connection for bands where 2RX is supported

For bands where 2RX is supported, it is left to the UE declaration and AP configuration to decide which 2 of the 8 Rx ports are connected with data source from system simulator. The remaning 6 Rx ports shall be connected with zero input. No test parameters or requirements are modified.

A.3.8.2.2.4	Antenna connection for bands where 4RX is supported

For bands where 4RX is supporetd, it is left to the UE declaration and AP configuration to decide which 4 of the 8 Rx ports are connected with data source from system simulator. The remaning 4 Rx ports shall be connected with zero input. Except for the modifications to radio link monitoring theresholds described in sections A.3.8.1.2.1 and A.3.8.1.2.2, no test parameters or requirements are modified.

A.3.8.2.2.5	Antenna connection for bands where 8RX is supported

For bands where 8RX is supporetd, all 8 Rx are connected with data source from system simulator. The system simulator shall provide independent noise and fading (low correlation) for each antenna port. Except for the modifications to radio link monitoring theresholds described in sections A.3.8.1.2.1 and A.3.8.1.2.2, no test parameters or requirements are modified.

<End of Change 1>