3GPP TSG-RAN4 WG4 Meeting #90	R4-1902055

# Athens, Greece,  Feb.25 - Mar.01, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5369 | rev | 1 | Current version: | 13.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR: Correction to 4Rx TC 9.9.1.4.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_4Rx_AP_DL-Perf |  |  |  |  |  | Date: |  |  | 2018-12-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For 4 antenna ports, when the CodeBookSubsetRestriction bitmap is set to “0x0000 0020 0000 0000”, only one PMI value is available, there is no second PMI value. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the first and second PMI distinguish in the test parameters in Table 9.9.1.4.2-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistence between CodeBookSubsetRestriction and following PMI reporting test parameters |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.1.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

##### 9.9.1.4.2	TDD

The following requirements apply to UE Category ≥5. For the parameters specified in table 9.9.1.4.2-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported offset level of the wideband spatial differential CQI for codeword #1 (Table 7.2-2 in TS 36.213 [6]) shall be used to determine the wideband CQI index for codeword #1 as

wideband CQI1 = wideband CQI0 – Codeword 1 offset level

The wideband CQI1 shall be within the set {median CQI1 -1, median CQI1, median CQI1 +1} for more than 90% of the time, where the resulting wideband values CQI1 shall be used to determine the median CQI values for codeword #1. For both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 – 1 and median CQI1 – 1 shall be less than or equal to 0.1. Furthermore, for both codewords #0 and #1, the PDSCH BLER using the transport format indicated by the respective median CQI0 + 1 and median CQI1 + 1 shall be greater than or equal to 0.1.

Table 9.9.1.4.2-1: PUCCH 1-1 static test (TDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 9 |  |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |  |
| Special subframe configuration |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | -3 |  |  |  |
|  |  | dB | -3 |  |  |  |
| CRS reference signals |  |  | Antenna ports 0, 1 |  |  |  |
| CSI reference signals |  |  | Antenna ports 15,…,18 |  |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 3 |  |  |  |
| CSI reference signal configuration |  |  | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (4 x 4) |  |  |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0020 0000 0000 |  |  |  |
| SNR (Note 2) |  | dB | 5 | 6 | 11 | 12 |
|  |  | dB[mW/15kHz] | -93 | -92 | -87 | -86 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note 3) |  |  |  |
| PUCCH Report Type for CQI/ PMI |  |  | 2 |  |  |  |
| Physical channel for RI reporting |  |  | PUSCH |  |  |  |
| PUCCH Report Type for RI |  |  | 3 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| CQI delay |  | ms | 10 or 11 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 3 |  |  |  |
| ri-ConfigIndex |  |  | 805 (Note 4) |  |  |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |  |  |
| Note 1: Reference measurement channel RC.22 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink SF#7 and #2. Note 4: RI reporting interval is set to the maximum allowable length of 160ms to minimise collisions between RI, CQI/PMI and HARQ-ACK reports. In the case when all three reports collide, it is expected that CQI/PMI reports will be dropped, while RI and HARQ-ACK will be multiplexed. At eNB, CQI report collection shall be skipped every 160ms during performance verification. |  |  |  |  |  |  |