3GPP TSG-TSG RAN WG1 Meeting # 96	R1-1903313

# Athens, Greece,  25th Feb - 1st March, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1235 | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Power control for RACH-less HO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMob-Core |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Power control for RACH-less handover is not defined in RAN1 specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that the PUSCH (re)transmissions corresponding to RACH-less HO follow the same power control procedures as those scheduled by the random access response grant. - Clarify that for the computaiton of f(0) and g(0), the UE should assume ΔPrampuprequested,c=δmsg2,c=0 – since these parameters do not exist for RACH-less HO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1, 5.1.1.1, 5.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 5.1.1	Physical uplink shared channel

If the UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG

-	When the procedures are applied for MCG, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the MCG respectively.

-	When the procedures are applied for SCG, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including PSCell), serving cell, serving cells belonging to the SCG respectively. The term 'primary cell' in this clause refers to the PSCell of the SCG.

If the UE is configured with a PUCCH-SCell, the UE shall apply the procedures described in this clause for both primary PUCCH group and secondary PUCCH group

-	When the procedures are applied for primary PUCCH group, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the primary PUCCH group respectively.

-	When the procedures are applied for secondary PUCCH group, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the secondary PUCCH group respectively.

For PUSCH (re)transmissions corresponding to ul-ConfigInfo-r14, the UE shall apply the procedures corresponding to PUSCH (re)transmission corresponding to the random access response grant.

#### 5.1.1.1	UE behaviour

<Unchanged parts are omitted>

-	If the UE receives the random access response message for a serving cell

-	, where

-	 is the TPC command indicated in the random access response corresponding to the random access preamble transmitted in the serving cell , see Subclause 6.2, and

and  is provided by higher layers and corresponds to the total power ramp-up requested by higher layers from the first to the last preamble in the serving cell , is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks valid for the subframe of first PUSCH transmission in the serving cell , and TF,c(0) is the power adjustment of first PUSCH transmission in the serving cell c. For a PUSCH transmission corresponding to ul-ConfigInfo-r14, the UE shall assume ΔPrampuprequested,c=δmsg2,c=0

<Unchanged parts are omitted>

#### 5.1.2.1	UE behaviour

<Unchanged parts are omitted>

-	Else

-	, where

-	 is the TPC command indicated in the random access response corresponding to the random access preamble transmitted in the primary cell, see Subclause 6.2 and

-	if UE is transmitting PUCCH in subframe i,

.

Otherwise,

and  is provided by higher layers and corresponds to the total power ramp-up requested by higher layers from the first to the last preamble in the primary cell. A UE configured with ul-ConfigInfo-r14, the UE shall assume ΔPrampuprequested=δmsg2=0

<Unchanged parts are omitted>