3GPP TSG-RAN WG1 Meeting #96	R1-1903598

# Athens, Greece,25th February – 1st March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0014 | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to dynamic HARQ codebook in NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Fujitsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-2-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In subclause 9.1.3.1 there is pseudo-code for determining the dynamic HARQ-ACK codebook. This contains two nested loops, which should consider every serving cell, c, for each of the M monitoring occasions. Unfortunately c is only intialized to zero outside the outer loop, so the full set of cells will only be considered for the first monitoring occasion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Initialization of c is moved inside the first loop as follows: Set to the number of PDCCH monitoring occasion(s) while Set – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell while Also editorial corrections are made (adjusting indentation) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The dynamic HARQ-ACK codebook will be determined incorrectly for cases with more than one PDCCH monitoring occasion |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Impact analysis This CR has isolated impact on the determination of the dynamic HARQ-ACK codebook when there is more than one PDCCH monitoring occasion If either the UE or the network, or both, are not implemented according to the CR, HARQ will not operate correctly if the use of the dynamic HARQ codebook is configured with more than one PDCCH monitoring occasion |  |  |  |  |  |  |  |  |

#### 9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

A UE determines monitoring occasions for PDCCH with DCI format 1_0 or DCI format 1_1 for scheduling PDSCH receptions or SPS PDSCH release on an active DL BWP of a serving cell , as described in Subclause 10.1, and for which the UE transmits HARQ-ACK information in a same PUCCH in slot  based on

-	PDSCH-to-HARQ_feedback timing values for PUCCH transmission with HARQ-ACK information in slot  in response to PDSCH receptions or SPS PDSCH release

-	slot offsets  [6, TS 38.214] provided by time domain resource assignment field in DCI format 1_0 or DCI format 1_1 for scheduling PDSCH receptions or SPS PDSCH release and by pdsch-AggregationFactor, when provided.

The set of PDCCH monitoring occasions for DCI format 1_0 or DCI format 1_1 for scheduling PDSCH receptions or SPS PDSCH release is defined as the union of PDCCH monitoring occasions across active DL BWPs of configured serving cells, ordered in ascending order of start time of the search space set associated with a PDCCH monitoring occasion. The cardinality of the set of PDCCH monitoring occasions defines a total number  of PDCCH monitoring occasions.

A value of the counter downlink assignment indicator (DAI) field in DCI format 1_0 or DCI format 1_1 denotes the accumulative number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s) or SPS PDSCH release associated with DCI format 1_0 or DCI format 1_1 is present, up to the current serving cell and current PDCCH monitoring occasion, first in ascending order of serving cell index and then in ascending order of PDCCH monitoring occasion index , where .

The value of the total DAI, when present [5, TS 38.212], in DCI format 1_1 denotes the total number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s) or SPS PDSCH release associated with DCI format 1_0 or DCI format 1_1 is present, up to the current PDCCH monitoring occasion  and is updated from PDCCH monitoring occasion to PDCCH monitoring occasion.

Denote by  the value of the counter DAI in DCI format 1_0 or DCI format 1_1 for scheduling on serving cell  in PDCCH monitoring occasion  according to Table 9.1.3-1. Denote by  the value of the total DAI in DCI format 1_1 in PDCCH monitoring occasion  according to Table 9.1.3-1. The UE assumes a same value of total DAI in all DCI formats 1_1 in PDCCH monitoring occasion .

If the UE transmits HARQ-ACK information in a PUCCH in slot  and for any PUCCH format, the UE determines the , for a total number of  HARQ-ACK information bits, according to the following pseudo-code:

Set  – PDCCH with DCI format 1_0 or DCI format 1_1 monitoring occasion index: lower index corresponds to earlier PDCCH with DCI format 1_0 or DCI format 1_1 monitoring occasion

Set

Set

Set

Set

Set  to the number of serving cells configured by higher layers for the UE

Set  to the number of PDCCH monitoring occasion(s)

while

Set  – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell

while

if PDCCH monitoring occasion  is before an active DL BWP change on serving cell  or an active UL BWP change on the PCell and an active DL BWP change is not triggered by a DCI format 1_1 in PDCCH monitoring occasion

;

else

if there is a PDSCH on serving cell  associated with PDCCH in PDCCH monitoring occasion , or there is a PDCCH indicating SPS PDSCH release on serving cell

if

end if

if

else

end if

if harq-ACK-SpatialBundlingPUCCH is not provided and  is a monitoring occasion for PDCCH with DCI format 1_0 or DCI format 1_1 and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of at least one serving cell,

= HARQ-ACK information bit corresponding to the first transport block of this cell

= HARQ-ACK information bit corresponding to the second transport block of this cell

elseif harq-ACK-SpatialBundlingPUCCH is provided to the UE and  is a monitoring occasion for PDCCH with DCI format 1_1 and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks in at least one configured DL BWP of a serving cell,

= binary AND operation of the HARQ-ACK information bits corresponding to the first and second transport blocks of this cell

else

= HARQ-ACK information bit of this cell

end if

end if

end if

end while

end while

if

end if

if harq-ACK-SpatialBundlingPUCCH is not provided to the UE and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of a serving cell,

else

end if

for any

Set

while

if SPS PDSCH reception is activated for a UE and the UE is configured to receive SPS PDSCH in a slot  for serving cell , where  is the PDSCH-to-HARQ-feedback timing value for SPS PDSCH on serving cell

= HARQ-ACK information bit associated with the SPS PDSCH reception

end if

;

end while

For a PDCCH monitoring occasion with DCI format 1_0 or DCI format 1_1 in the active DL BWP of a serving cell, when a UE receives a PDSCH with one transport block and the value of maxNrofCodeWordsScheduledByDCI is 2, the HARQ-ACK information is associated with the first transport block and the UE generates a NACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is not provided and generates HARQ-ACK information with value of ACK for the second transport block if harq-ACK-SpatialBundlingPUCCH is provided.

-----------------------------------------------------end of change------------------------------------------------------------------------