3GPP TSG RAN WG1 Meeting #96 R1-1903726 Athens, Greece, February 25th – March 1st, 2019 CR-Form-v11.2. CHANGE REQUEST. 38.213; CR; 0022; rev; Current version:; 15.4.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; (Late Drop) CR on PRACH Power Ramping Counter Suspension. Source to WG:; Samsung. Source to TSG:; R1. Work item code:; NR_newRAT-Core; Date:; 2019-02-25. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; In EN-DC, if due to power allocation the UE does transmit a PRACH, Layer 1 notifies higher layers to suspend the corresponding power ramping counter – Layer 1 may also do so if the UE transmits PRACH with reduced power. The same situation can occur in NE-DC but the agreement for EN-DC was not updated after the NE-DC operation was specified.. Summary of change:; Add NE-DC as an operating scenario where the power ramping counter can be suspended when the UE needs to transmit PRACH with reduced power.. Consequences if not approved:; Incomplete specifications.. Clauses affected:; 7.4. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:; Isolated impact analysis: The UE operation is same as for EN-DC. There is no impact on the gNB operation. CR is for ‘late drop’ Rel-15.

## 7.4	Physical random access channel

*** Unchanged text is omitted ***

If due to power allocation to PUSCH/PUCCH/PRACH/SRS transmissions as described in Subclause 7.5, or due to power allocation in EN-DC or NE-DC operation as described in Subclause 7.6.1 or Subclause 7.6.1A, respectively, the UE does not transmit a PRACH in a transmission occasion, Layer 1 notifies higher layers to suspend the corresponding power ramping counter. If due to power allocation to PUSCH/PUCCH/PRACH/SRS transmissions as described in Subclause 7.5, or due to power allocation in EN-DC or NE-DC operation as described in Subclause 7.6.1 or Subclause 7.6.1A, respectively, the UE transmits a PRACH with reduced power in a transmission occasion, Layer 1 may notify higher layers to suspend the corresponding power ramping counter.