3GPP TSG-RAN WG1 Meeting #96	  R1-1903748

# Athens, Greece, February 25th - March 1st, 2019

| CR-Form-v11.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0025 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifying DL reception and UL transmision related restrictions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 28.02.2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-14 (Release 15) Rel-14 (Release 16) |  |
| Reason for change: |  | Simultaneous DL reception and UL transmission related restrictions are unneccsary for paired spectrum single cell operation, on following parts: - PDCCH, PDSCH and CSI-RS reception is cancelled due to scheduled UL transmission, - UL transmission cancellation due to scheduled DL reception and - SS/PBCH block location related UL transmission restrictions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce wording to determine that the simultaneous DL reception and UL transmission restrictions and procedures apply only with single carrier in unpaired spectrum. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unneccesary restriction for RACH, PUSCH and PUCCH transmission and PDCCH, PDSCH and CSI-RS reception in case of paired spectrum. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Isolated Impact Analysis: On paired spectrum, if the gNB is implemented according to this CR and the UE is not: - A UE assuming that the PDCCH, PDSCH and CSI-RS reception related restrictions apply for also for paired spectrum, will receive gNB scheduled data or measure CSI-RS, resulting a retransmission request or lost/degraded report quality. - A UE assuming that the PUSCH and PUCCH transmission restrictions apply for also for paired spectrum, will not follow the gNB scheduling (PUSCH), or transmit feedback (PUCCH) as expected by gNB, resulting a retransmission request or lost report. On paired spectrum, if the UE is implemented according to this CR and the gNB is not: - gNB would not transmit PDCCH or PDSCH or CSI-RS for the UE corresponding to UL transmission - gNB would avoid scheduling PUCCH/PUSCH overlapping with SSB in time, losing uplink capacity. No impact on compatibility. |  |  |  |  |  |  |  |  |

# 11	UE-group common signalling

If the UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG

-	When the procedures are applied for MCG, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the MCG respectively.

-	When the procedures are applied for SCG, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including PSCell), serving cell, serving cells belonging to the SCG respectively. The term 'primary cell' in this clause refers to the PSCell of the SCG.

## 11.1	Slot configuration

<omitted text>

For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to receive a PDCCH, or a PDSCH, or a CSI-RS in a set of symbols of a slot, the UE receives the PDCCH, the PDSCH, or the CSI-RS if the UE does not detect a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, or DCI format 2_3 that indicates to the UE to transmit a PUSCH, a PUCCH, a PRACH, or a SRS in at least one symbol of the set of symbols of the slot; otherwise, the UE does not receive the PDCCH, or the PDSCH, or the CSI-RS in the set of symbols of the slot.

For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format 1_0, DCI format 1_1, or DCI format 0_1 indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then

-	the UE does not expect to cancel the transmission in symbols from the subset of symbols that occur, relative to a last symbol of a CORESET where the UE detects the DCI format 1_0 or the DCI format 1_1 or the DCI format 0_1, after a number of symbols that is smaller than the PUSCH preparation time  for the corresponding UE processing capability [6, TS 38.214] assuming

-	the UE cancels the PUCCH, or PUSCH, or PRACH transmission in remaining symbols from the set of symbols and cancels the SRS transmission in remaining symbols from the subset of symbols

<omitted text>

For a set of symbols of a slot that are indicated to a UE as flexible by TDD-UL-DL-ConfigurationCommon, or TDD-UL-DL-ConfigDedicated, the UE does not expect to receive both dedicated configuring transmission from the UE in the set of symbols of the slot and dedicated configuring reception by the UE in the set of symbols of the slot.

For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot that are indicated to a UE by ssb-PositionsInBurst in SystemInformationBlockType1 or ssb-PositionsInBurst in ServingCellConfigCommon, for reception of SS/PBCH blocks, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by TDD-UL-DL-ConfigurationCommon, or TDD-UL-DL-ConfigDedicated, when provided to the UE.