3GPP TSG RAN Meetings #83	RP-190655

Shenzhen, China, 18th  – 21st   March 2019

# WI template dec2016 (RAN #75 adaptation3)

# Source:	CATT

# Title:	New WID: UE Power Saving in NR

# Document for:	Approval

Agenda Item:

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

Title:	UE Power Saving in NR

Acronym:

Unique identifier: 	{A numberto be provided by MCC at the plenary}

NOTE:	For new WIs/Sis leave the Unique identifier empty but you may make a proposal for an Acronym.

If this is a RAN WID including Core and Perf. Part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | x |
| --- | --- |
| This WID includes a Performance part | x |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

- Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | x | x |  |  |
| No | X |  |  | X | X |
| Don’t know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a work item.

|  | Feature |
| --- | --- |
|  | Building Block |
| X | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, we define them as work tasks.If you are in doubt, please contact MCC.

2.2	Parent and child Work Items

| Parent and child Work Items |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. Part WI or Testing part WI in one WID. Therefore the table above should just include the feature WI Unique ID and title and Nature of relationship is “parent WID”.

2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 700058 | Study on Scenarios and Requirements for Next Generation Access Technologies |  |
| 710062 | Study on New Radio (NR) Access Technology |  |
| 700022 | Study on Stage 1 for New Services and Markets Technology Enablers |  |

NOTE:	Classical examples: List a preceding SI or a preceding WI (e.g. if you further enhance a topic). Also related or dependent Wis in other TSGs should be indicated.

3	Justification

UE battery life is an important aspect of the user’s experience, which will influence the adoption of 5G handsets and/or services. The UE power saving study to target that UE power efficiency for 5G NR UEs is better than that of LTE.    The study of the Rel-16 UE power saving had shown substantial power saving gain up to 98% comparing to Rel-15 Rel-15 NR feature with power saving feature, such as DRX.

The UE power saving study completes the UE power consumption model for evaluation based on the inputs from many sources.  Evaluation results have been shown based on the UE power consumption model through calibration and subsequent evaluations.   The UE power model could be used as the reference model in evaluating the UE power consumption for any other features if applicable.

RAN1’s study shows the following power saving gains over the agreed baseline in UE power saving schemes with UE adaptation in frequency domain, time domain, antenna domain, DRX operations, and reducing PDCCH monitoring with different traffic types, such as FTP, IM, web browsing, video streaming, gaming and VoIP,  and network configurations,

- The power saving schemes with UE adaptation to BWP switching show the power saving gains ranged from 16% - 45% over the agreed baseline configuration.  The power saving schemes with UE adaptation to SCell operation show12% - 57.5% power saving gain with average latency increase 0.1% - 2.6%.

- The power saving schemes with cross-slot scheduling shows up to 2%-28% power saving gains with UPT degradation 0.3%-25%.  The power saving gain decreases and the UPT degradation increases as the K0 increase.   The power saving gain 15%-17% and 0% -93% overhead increases is observed for same-slot scheduling only with small packet.  The power saving gain less than 2% is observed for multi-slot scheduling.

- The power saving schemes with UE adaptation to the number of MIMO layers or number of Tx/Rx antenna (panels) provides 3%-30% power saving gain and 4% latency increase for dynamic antenna adaptation.   The power saving gain 6%-30% is observed for semi-static antenna adaptation with expected latency and UPT degradation.  Additional network resource is used for compensation of the loss of multi-antenna processing gain.

- The power saving schemes with UE adaptation to the DRX operation shows 8%~50% power saving gains with latency increase in the range of 2%~13%.  The power saving gain for dynamic DRX configuration/adaptation is 8%-70% with latency increase 2% - 323%. Rel-15 enabled DRX operation shows negative 37% to 47 % power saving gain over the agreed baseline.

- The power saving schemes for dynamic adaption of UE PDCCH monitoring shows 5% - 85% power saving gains with the latency increase/UPT degradation in the range of (0% - 115%)/( 5%-  43%).

It is noted that the combined power saving techniques (as listed above) may not necessarily provide linear addition of individual power saving gains. It is noted that the power saving gains shown in the specific context as cited above are relative numbers, while it is understood that the absolute percentage of power consumption for a specific context may vary depending on the actual usage.

The UE assistance information provides the gNB additional information in facilitating UE adaptation to the traffic and reducing the power consumption of some respective power saving schemes shown aforementioned.  UE preferred configurations are fed back to the gNB to help the network optimizing the resource utilization and assisting UE in achieving power consumption reduction.

The power saving schemes with the power saving signal/channel triggering UE adaptation shows the power saving with adaptation in different domains, such as DRX operation, BWP switching, and reducing PDCCH monitoring aforementioned.

For adapting/relaxing RRM measurement in time domain

- 11.1% - 26.6% and 7.4% - 17.8% power saving gains are shown for increasing measurement period 4 times and 2 times respectively for RRC CONNECTED state, 17.9%-19.7% and 0.89%-5.36% power saving gains are shown RRC IDLE/INACTIVE state by increasing measurement period for 1 SSB burst set for measurement and periodic activities and 2 or 3 SSB burst set for measurement and periodic activities respectively.

- For stationary or low mobility (e.g., 3km/h) case, increasing measurement period has less impact (e.g., handover failure rate changes from 0% to 0.26% for 3km/h by extending 4 times measurement period) to the mobility performance compared to high mobility cases (e.g., handover failure rate changes from 0%-1% for 60km/h by extending 4 times measurement period).

For adapting/relaxing RRM measurement for intra-frequency measurement by reducing the number of measured cells,

- By assuming number of neighbouring cells to be measured is reduced, it is shown that about 4.7% - 7.1% power saving gain can be observed if reducing the number of measured cells for IDLE state. 1 source shows that about 1.8% - 21.3% power saving gain can be observed if reducing the number of measured cells for CONNECTED state.

- In additional to that, by also assuming that UE can limit the processing for measurement within a constrained time period and/or with reduced complexity, 26.43% - 37.5% power saving gain is shown.

- The corresponding performance impact was not reported and summarized

For additional resource for RRM measurement

- For RRC IDLE/INACTIVE, by using additional resources for RRM measurement, the UE power saving gain is 19%~38%. Analysis on network architecture impact, higher-layer/PHY-layer signaling overhead and network energy consumption is not provided. Two sources report 1.6% of SSB overhead for 160ms additional reference signals periodicity assuming that SSB burst set periodicity is 20ms. If static UE specific signals (e.g., CSI-RS) for connected mode UEs can be reused as additional resource for RRM, reference signal overhead may not increase.

- For RRC CONNECTED, one source showed that, by having CSI-RS repetition for CSI-RS based RRM, the UE power saving gain is 11.8%~20.7% by assuming that CSI-RS overhead of baseline is identical to CSI-RS overhead with enhancement. The analysis on handover failure rate is not provided under the assumed overhead.

For reducing the number of measured inter-frequency layers

- 2 sources show that reducing the number of measured inter-frequency layers can provide 21%~38% power saving gain for RRC CONNECTED states, and 1 source shows 14%~35% power saving gain for RRC IDLE states by assuming reducing inter-frequency layers from 6 to 3. The mobility performance impact is not provided.

The UE power saving study has demonstrated power saving from the one power saving scheme with UE adaptation to the traffic and power consumption, e.g., adaptation to the DRX operation, or combination of power saving schemes, e.g., adaptation to the DRX operation with reduced PDCCH monitoring.   Therefore, the proposed UE power saving work item is to select the appropriate power saving schemes with substantial power saving gain and tolerable system performance impact.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective is to specify the UE power saving techniques with UE adaption in achieving UE power saving.  The power saving technique should address latency and performance in NR as well as network impact.  The objective of the UE power saving includes the following,

- Specify power saving techniques with UE adaptation with focus in RRC_CONNECTED mode [RAN1, RAN4]

  - Specify the power saving techniques with power saving signal/channel

    - Specify the power saving signal/channel as the wakeup signal/channel in RRC_CONNECTED

    - Note: no duplication of DRX operation

    - Note: Impact to DRX is studied at RAN2

  - Specify DCI-based mechanism in skipping PDCCH monitoring or switching PDCCH monitoring periodicity

    - Note: no duplication of DRX operation

  - Specify the procedure of cross-slot scheduling power saving techniques

  - Specify the power saving techniques of UE adaptation to the BWP/SCell operation in RRC_CONNECTED

    - Note: no duplication of the NR CA/DC enhancement

  - Specify the power saving techniques of UE dynamic adaptation to the maximum number of MIMO layers in RRC_CONNECTED

  - Specify the UE assistance information for the associated power saving techniques.

- Specify the power saving techniques in RRM measurements in synchronous and asynchronous network deployment

  - Specify  the relaxation and adaptation of RRM measurement period [RAN4, RAN1]

  - Specify additional resources for RRM in RRC_IDLE/RRC_INACTIVE [RAN1, RAN4]

Note:  These objectives are RAN1/RAN4 focus.   RAN2 objectives will be incorporated with possible revision of existing objectives after RAN2 complete the study.

4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

Specify necessary performance requirements related to the above-mentioned core requirements [RAN4]

- UE performance requirements

- UE RRM performance requirements

4.3	RAN time budget request (not applicable to RAN5 Wis/Sis)

NOTE:	For all RAN related Wis/Sis which are not led by RAN WG5 the WI/SI apporteurs has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed leave the field empty otherwise enter a number in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID’s zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each of them in the attached Excel table.

Additional comments to the time budget request in the attached Excel table:

5	Expected Output and Time scale

| New specifications{One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Proposed Spec no. or series | Type (see note 1) | Title | For info at TSG# | For approval at TSG# | Remarks |

Note 1:	Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
“Internal TR” is intended for 3GPP internal use only whereas “External TR” may be transposed by Ops.

NOTE:	If this is a RAN WID including Core and Perf. Part, then all new Core part specs have to be listed first and then all new Perf. Part specs. Indicate “Core part” or “Perf. Part” under Remarks for each spec.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.133 | Necessary RAN4 requirements | 86 | Core parts |
| 38.101-1 | Necessary RAN4 requirements | 86 | Core parts |
| 38.101-2 | Necessary RAN4 requirements | 86 | Core parts |
| 38.101-3 | Necessary RAN4 requirements | 86 | Core parts |
| 38.300 | Possible changes to NR Stage-3 | 86 | Core parts |
| 38.213 | Possible changes to physical layer procedures | 86 | Core parts |
| 38.101-4 | Necessary RAN4 requirements | 88 | Performance parts |
| 38.133 | Necessary RAN4 requirements | 88 | Performance parts |

By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

NOTE:	If this is a RAN WID including Core and Perf. Part, then all new Core part specs have to be listed first and then all new Perf. Part specs. Indicate “Core part” or “Perf. Part” under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. Part), then it has to be listed twice with appropriate approval dates.

6	Work item Rapporteur(s)

Fang-Chen Cheng

Company: CATT

Email:	fcc@catt.cn

7	Work item leadership

RAN WG1

Secondary RAN WAG4

8	Aspects that involve other WGs

NOTE:	For RAN WIDs: Section 8 applies only toWGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

9	Supporting Individual Members

Supporting IM name

Apple

AT&T

CATT

CAICT

CMCC

China Unicom

China Telecom

CHTTL

Blackberry

Ericsson

Fujitsu

Huawei

Huawei Device

HiSilicon

III

Intel

InterDigital

ITRI

KT Corp

LG

Lenovo

MediaTek

Motorola Mobility

Nokia

Nokia Shanghai Bell

NTT DoCoMo

OPPO

Panasonic

Qualcomm

Samsung

Sharp

Sierra Wireless

SK Telecom

Softbank

Sony

Telecom Italia

Verizon

vivo

Xiaomi