3GPP TSG RAN Meeting #84		 RP-190991

Newport Beach, CA, U.S.A., June 3rd – 6th, 2019

Agenda item:	9.2.1

Source: 	Qualcomm Incorporated

Title: 	Single Tx UL LTE-NR DC

Document for:	Discussion/Decision

# Discussion

In the current Rel-15 RAN1 feature list [1], there are two features targeting SUO Case 1 operation; FG 6-13 and 8-1a, both listed below.

| 6-13 | Case 1 Single Tx UL LTE-NR DC | 1) Case 1: DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell 2) HARQ subframe offset |  | Yes |  | Type 2 | Yes | Yes |  | This is a UE feature for LTE for a LTE/NR dual connectivity UE |  |  | Mandatory with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8-1a | Operation A with single UL Tx case 1 |  | EN-DC | Yes |  | Type 2 | Yes | Yes |  | RP-172833 |  | Mandatory with capability signaling conditioned that UE does not support dynamic power sharing, optional for UEs supporting dynamic power sharing | Mandatory with capability signaling conditioned that UE does not support dynamic power sharing, i.e., UE indicate “0” as non-support for 8-1, optional for UEs supporting dynamic power sharing |

These two features were introduced for two different use cases:

- FG 6-13: Introduced for single Tx capability, such as band combinations with difficult IMD, or certain intra-band contiguous EN-DC band combinations, e.g. b71-n71

  - Based on the agreed Way Forward in [2]

- FG 8-1a: Introduced for power sharing for Type 2 (not dynamic power sharing capable) UEs

  - Based on the agreed Way Forward in [3]

Mandatory / optional status for FG 6-13 has not been discussed in RAN1. The current mandatory designation was made at the Plenary, but not based on prior agreements.

We observe that the underlying functionality for FG 6-13 and FG 8-1a is identical. However, there is a conflict between the FG 6-13 and FG 8-1a mandatory / optional designation. As it was agreed [3], for Type 1 UEs, SUO Case 1 operation is optional, as captured in FG 8-1a. But for the same UEs, SUO Case 1 operation would be mandatory based on FG 6-13. We propose to resolve the conflict in definitions.

Therefore, we make the following proposal.

Proposal:  For FG 6-13, change the mandatory designation to “mandatory with capability signaling for UEs not supporting simultaneous dual-Tx operation in the band combination”.

Isolated impact analysis:

Note that agreeing to the proposal above would not introduce backward compatibility issues and there is no need to introduce any new signaling.

# Conclusions

In this contribution, the following proposal has been made:

Proposal:  For FG 6-13, change the mandatory designation to “mandatory with capability signaling for UEs not supporting simultaneous dual-Tx operation in the band combination”.

# References

- R1-1907862, “RAN1 NR UE features”, NTT DOCOMO

- RP-172064, “Single Tx switched UL”, Qualcomm, Intel

- RP-172833, “Way Forward on NR EN-DC power sharing capability”, Qualcomm

- RP-172100, “LS on single Tx switched UL”, RAN, Qualcomm, Intel