3GPP TSG-RAN WG1 Meeting #97	R1-1907587

# Reno, USA, May 13th  - 17th , 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.212 | CR | 0319 | rev |  | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the bitwidth of report type 1a for FD-MIMO Class A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_EBF_FDMIMO-Core |  |  |  |  |  | Date: |  |  | 2019-05-15 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | PUCCH reporting mode 2-1 is supported for FD-MIMO Class A. However, the table in 36.212 that specifies the bitwidth of the subband CQI and subband PMI does not cover FD-MIMO Class A |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the bitwidth table of the report type 1a for FD-MIMO Class A, i.e. updating Table 5.2.3.3.2-2B for FD-MIMO Class A with CodebookConfig=2/3/4 and defining a new table for CodebookConfig=1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support of PUCCH mode 2-1 for FD-MIMO Class A |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.2.3.3.2	Channel quality information formats for UE-selected sub-band reports

Table 5.2.3.3.2-1 shows the fields and the corresponding bit widths for the sub-band channel quality information feedback for UE-selected sub-band reports for PDSCH transmissions associated with transmission mode 1, transmission mode 2, transmission mode 3, transmission mode 7, transmission mode 8 configured without PMI/RI reporting, transmission mode 9 configured without PMI/RI reporting or configured with 1 antenna port, transmission mode 10 configured without PMI/RI reporting or configured with 1 antenna port, and transmission mode 9/10 configured with higher layer parameter eMIMO-Type and eMIMO-Type2, and eMIMO-Type2 is set to 'CLASS B' configured without PMI/RI reporting or configured with 1 antenna port.

Table 5.2.3.3.2-1A shows the fields and the corresponding bit widths for the sub-band channel quality information feedback for UE-selected sub-band reports for PDSCH transmissions associated with transmission mode 9/10 configured without PMI reporting and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS B' with 2/4/8 antenna ports, and transmission mode 9/10 configured with higher layer parameter eMIMO-Type and eMIMO-Type2, and eMIMO-Type2 is set to 'CLASS B' configured without PMI reporting with 2/4/8 antenna ports.

Table 5.2.3.3.2-1: UCI fields for channel quality information feedback for UE-selected sub-band CQI reports (transmission mode 1, transmission mode 2, transmission mode 3, transmission mode 7, transmission mode 8 configured without PMI/RI reporting, transmission mode 9 configured without PMI/RI reporting or configured with 1 antenna port, transmission mode 10 configured without PMI/RI reporting or configured with 1 antenna port, transmission mode 9/10 configured without PMI/RI with higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS B', and transmission mode 9/10 configured with higher layer parameter eMIMO-Type and eMIMO-Type2, and eMIMO-Type2 is set to 'CLASS B' configured without PMI/RI reporting or configured with 1 antenna port)

| Field | Bit width |
| --- | --- |
| Sub-band CQI | 4 |
| Sub-band label | 1 or 2 |

Table 5.2.3.3.2-1A: UCI fields for channel quality information feedback for UE-selected subband CQI reports (transmission mode 9/10 configured without PMI reporting and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS B' with 2/4/8 antenna ports, and transmission mode 9/10 configured with higher layer parameter eMIMO-Type and eMIMO-Type2, and eMIMO-Type2 is set to 'CLASS B' configured without PMI reporting with 2/4/8 antenna ports)

| Field | Bit width |  |
| --- | --- | --- |
|  | Rank = 1 | Rank > 1 |
| Wide-band CQI | 4 | 4 |
| Spatial differential CQI | 0 | 3 |
| Sub-band label | 1 or 2 | 1 or 2 |

Table 5.2.3.3.2-2 shows the fields and the corresponding bit widths for the sub-band channel quality information feedback for UE-selected sub-band reports for PDSCH transmissions associated with transmission mode 4, transmission mode 5, transmission mode 6, and transmission mode 8 configured with PMI/RI reporting except with alternativeCodeBookEnabledFor4TX-r12=TRUE.

Table 5.2.3.3.2-2: UCI fields for channel quality information feedback for UE-selected sub-band reports (transmission mode 4, transmission mode 5, transmission mode 6 and transmission mode 8 configured with PMI/RI reporting except with alternativeCodeBookEnabledFor4TX-r12=TRUE)

| Field | Bit width |  |  |  |
| --- | --- | --- | --- | --- |
|  | 2 antenna ports |  | 4 antenna ports |  |
|  | Rank = 1 | Rank = 2 | Rank = 1 | Rank > 1 |
| Sub-band CQI | 4 | 4 | 4 | 4 |
| Spatial differential CQI | 0 | 3 | 0 | 3 |
| Sub-band label | 1 or 2 | 1 or 2 | 1 or 2 | 1 or 2 |

Table 5.2.3.3.2-2A and Table 5.2.3.3.2-2B show the fields and the corresponding bit widths for the sub-band channel quality information feedback for UE-selected sub-band reports for PDSCH transmissions associated with transmission mode 9 configured with PMI/RI reporting with 2/4/8 antenna ports except with alternativeCodeBookEnabledFor4TX-r12=TRUE, and transmission mode 10 configured with PMI/RI reporting with 2/4/8 antenna ports except with alternativeCodeBookEnabledFor4TX-r12=TRUE, and transmission mode 9/10 configured with PMI/RI reporting and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS A' with CodebookConfig=2/3/4, transmission mode 9/10 configured with PMI/RI reporting with 2/4/8 antenna ports and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS B' with K=1 and except with alternativeCodebookEnabledCLASSB_K1=TRUE, or K>1, except with alternativeCodeBookEnabledFor4TX-r12=TRUE, transmission mode 9/10 configured with higher layer parameter eMIMO-Type and eMIMO-Type2, and eMIMO-Type2 configured with PMI/RI reporting with 2/4/8 antenna ports except with alternativeCodebookEnabledCLASSB_K1=TRUE or alternativeCodeBookEnabledFor4TX-r12=TRUE. The number of configured CSI-RS resources in a CSI process K is defined in [3] and alternativeCodebookEnabledCLASSB_K1 is configured by higher layers [6].

Table 5.2.3.3.2-2A-1 shows UCI fields for channel quality information feedback for UE-selected sub-band reports (transmission mode 9/10 configured with 2/4/8 antenna ports and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS B' with K=1 and alternativeCodebookEnabledCLASSB_K1=TRUE, transmission mode 9/10 configured without PMI reporting and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS B' with 2/4/8 antenna ports, transmission mode 9/10 configured with higher layer parameter eMIMO-Type and eMIMO-Type2, and eMIMO-Type2 is set to 'CLASS B' configured without PMI reporting with 2/4/8 antenna ports.

Table 5.2.3.3.2-2B-1 shows the fields and the corresponding bit widths for the sub-band channel quality information feedback for UE-selected sub-band reports for PDSCH transmissions associated with transmission mode 9/10 configured with PMI/RI reporting and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS A' with CodebookConfig=1.

Table 5.2.3.3.2-2C shows the fields and the corresponding bit widths for the sub-band channel quality information feedback for UE-selected sub-band reports for PDSCH transmissions associated with transmission modes 8, 9 and 10 configured with PMI/RI reporting and alternativeCodeBookEnabledFor4TX-r12 =TRUE, transmission mode 9/10 configured with PMI/RI reporting with 4 antenna ports and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS B' with K=1 and except with alternativeCodebookEnabledCLASSB_K1=TRUE, and with alternativeCodeBookEnabledFor4TX-r12=TRUE, and transmission mode 9/10 configured with higher layer parameter eMIMO-Type and eMIMO-Type2, and eMIMO-Type2 configured with PMI/RI reporting with 4 antenna ports except with alternativeCodebookEnabledCLASSB_K1=TRUE , and with alternativeCodeBookEnabledFor4TX-r12=TRUE.

Table 5.2.3.3.2-2A: UCI fields for channel quality information feedback for UE-selected sub-band reports (transmission mode 9 configured with PMI/RI reporting with 2/4 antenna ports except with alternativeCodeBookEnabledFor4TX-r12=TRUE, transmission mode 10 configured with PMI/RI reporting with 2/4 antenna ports except with alternativeCodeBookEnabledFor4TX-r12=TRUE, and transmission mode 9/10 configured with 2/4 antenna ports and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS B' with K=1 and except with alternativeCodebookEnabledCLASSB_K1=TRUE, K>1 except with alternativeCodeBookEnabledFor4TX-r12=TRUE, and transmission mode 9/10 configured with higher layer parameter eMIMO-Type and eMIMO-Type2, and eMIMO-Type2 configured with PMI/RI reporting with 2/4 antenna ports except with alternativeCodebookEnabledCLASSB_K1=TRUE or alternativeCodeBookEnabledFor4TX-r12=TRUE)

| Field | Bit width |  |  |  |
| --- | --- | --- | --- | --- |
|  | 2 antenna ports |  | 4 antenna ports |  |
|  | Rank = 1 | Rank = 2 | Rank = 1 | Rank > 1 |
| Wide-band CQI | 0 | 0 | 0 | 0 |
| Sub-band CQI | 4 | 4 | 4 | 4 |
| Spatial differential CQI | 0 | 3 | 0 | 3 |
| Wide-band i2 | 0 | 0 | 0 | 0 |
| Sub-band i2 | 0 | 0 | 0 | 0 |
| Sub-band label | 1 or 2 | 1 or 2 | 1 or 2 | 1 or 2 |

Table 5.2.3.3.2-2A-1: UCI fields for channel quality information feedback for UE-selected sub-band reports (transmission mode 9/10 configured with 2/4/8 antenna ports and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS B' with K=1 and alternativeCodebookEnabledCLASSB_K1=TRUE, transmission mode 9/10 configured without PMI reporting and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS B' with 2/4/8 antenna ports, and transmission mode 9/10 configured with higher layer parameter eMIMO-Type and eMIMO-Type2, and eMIMO-Type2 is set to 'CLASS B' configured without PMI reporting with 2/4/8 antenna ports)

| Field | Bit width |  |
| --- | --- | --- |
|  | 2/4/8 antenna ports |  |
|  | Rank = 1 | Rank > 1 |
| Wide-band CQI | 0 | 0 |
| Sub-band CQI | 4 | 4 |
| Spatial differential CQI | 0 | 3 |
| Wide-band i2 | 0 | 0 |
| Sub-band i2 | 0 | 0 |
| Sub-band label | 1 or 2 | 1 or 2 |

Table 5.2.3.3.2-2B: UCI fields for channel quality feedback for UE-selected sub-band reports (transmission mode 9 configured with PMI/RI reporting with 8 antenna ports, transmission mode 10 configured with PMI/RI reporting with 8 antenna ports, transmission mode 9/10 configured with PMI/RI reporting and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS A' with CodebookConfig=2/3/4, and transmission mode 9/10 configured with 8 antenna ports and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS B' with K=1 except with alternativeCodebookEnabledCLASSB_K1=TRUE, and K>1, and transmission mode 9/10 configured with higher layer parameter eMIMO-Type and eMIMO-Type2, and eMIMO-Type2 configured with PMI/RI reporting with 8 antenna ports except with alternativeCodebookEnabledCLASSB_K1=TRUE)

| Field | Bit width |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/12/16/20/24/28/32 antenna ports |  |  |  |  |  |  |  |  |  |
|  | Rank = 1 |  | Rank = 2, 3 |  | Rank = 4 |  | Rank = 5, 6, 7 |  | Rank = 8 |  |
|  | PTI=0 | PTI=1 | PTI=0 | PTI=1 | PTI=0 | PTI=1 | PTI=0 | PTI=1 | PTI=0 | PTI=1 |
| Wide-band CQI | 4 | 0 | 4 | 0 | 4 | 0 | 4 | 0 | 4 | 0 |
| Sub-band CQI | 0 | 4 | 0 | 4 | 0 | 4 | 0 | 4 | 0 | 4 |
| Spatial differential CQI | 0 | 0 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Wide-band i2 | 4 | 0 | 4 | 0 | 3 | 0 | 0 | 0 | 0 | 0 |
| Sub-band i2 | 0 | 4 | 0 | 2 | 0 | 2 | 0 | 0 | 0 | 0 |
| Sub-band label | 0 | 1 or 2 | 0 | 1 or 2 | 0 | 1 or 2 | 0 | 1 or 2 | 0 | 1 or 2 |

Table 5.2.3.3.2-2B-1: UCI fields for channel quality feedback for UE-selected sub-band reports (transmission mode 9/10 configured with PMI/RI reporting and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS A' with CodebookConfig=1)

| Field | Bit width |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8/12/16/20/24/28/32 antenna ports |  |  |  |  |  |  |  |  |  |
|  | Rank = 1 |  | Rank = 2 |  | Rank = 3, 4 |  | Rank = 5, 6, 7 |  | Rank = 8 |  |
|  | PTI=0 | PTI=1 | PTI=0 | PTI=1 | PTI=0 | PTI=1 | PTI=0 | PTI=1 | PTI=0 | PTI=1 |
| Wide-band CQI | 4 | 0 | 4 | 0 | 4 | 0 | 4 | 0 | 4 | 0 |
| Sub-band CQI | 0 | 4 | 0 | 4 | 0 | 4 | 0 | 4 | 0 | 4 |
| Spatial differential CQI | 0 | 0 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Wide-band i2 | 2 | 0 | 2 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| Sub-band i2 | 0 | 2 | 0 | 2 | 0 | 1 | 0 | 0 | 0 | 0 |
| Sub-band label | 0 | 1 or 2 | 0 | 1 or 2 | 0 | 1 or 2 | 0 | 1 or 2 | 0 | 1 or 2 |

Table 5.2.3.3.2-2C: UCI fields for channel quality feedback for UE-selected sub-band reports with 4 antenna ports (transmission modes 8, 9 and 10 configured with PMI/RI reporting, 4 antenna ports and alternativeCodeBookEnabledFor4TX-r12=TRUE, and transmission mode 9/10 configured with 4 antenna ports and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS B' with K=1 and except with alternativeCodebookEnabledCLASSB_K1=TRUE, and K>1, with alternativeCodeBookEnabledFor4TX-r12=TRUE, and transmission mode 9/10 configured with higher layer parameter eMIMO-Type and eMIMO-Type2, and eMIMO-Type2 configured with PMI/RI reporting with 4 antenna ports except with alternativeCodebookEnabledCLASSB_K1=TRUE, and with alternativeCodeBookEnabledFor4TX-r12=TRUE)

| Field | Bit width |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 4 antenna ports |  |  |  |  |  |
|  | Rank = 1 |  | Rank = 2 |  | Rank = 3 | Rank=4 |
|  | PTI=0 | PTI=1 | PTI=0 | PTI=1 | PTI=1 | PTI=1 |
| Wide-band CQI | 4 | 0 | 4 | 0 | 0 | 0 |
| Sub-band CQI | 0 | 4 | 0 | 4 | 4 | 4 |
| Spatial differential CQI | 0 | 0 | 3 | 3 | 3 | 3 |
| Wide-band i2 | 4 | 0 | 4 | 0 | 0 | 0 |
| Sub-band i2 | 0 | 4 | 0 | 2 | 2 | 2 |
| Sub-band label | 0 | 1 or 2 | 0 | 1 or 2 | 1 or 2 | 1 or 2 |

<Unchanged parts are omitted>