3GPP TSG RAN meeting #84							 	RP-191483

Newport Beach, USA

June 3-6, 2019

Agenda Item:	9.4.3

Source: 	Qualcomm

Title:	Guidance on essential functionality for NR-U

Document for:	Discussion

# 1	Background and Agenda

RP-191361 presented NR-U rapporteur’s views on prioritization of NR-U remaining work by identify essential/non-essential items on each WGs for ontime completion of WI.

The following proposals were given in RP-191361:

Proposal 1: RAN to instruct WGs to focus on completion of essential items leaving optimization items as best-effort from WG time-management perspective.

Proposal 2: 10MHz is supported in NR-U via NR-U/NR-U CA or NR/NR-U CA without air-interface optimizations specific to 10MHz. Note: 10MHz Pcell or SpCell is not supported in NR-U.

- It is only for 5GHz band (main target is 5GHz band in a certain country) and thus there will be clear definition by means of bands spelled out like LTE LAA.

- the absence of Wi-Fi channels should be guaranteed

Suggested agenda:

- Agree on Proposal 2?

- Review RAN4 prioritization topics

- Review RAN2 prioritization topics

- Review RAN1 prioritization topics

- Agree on list of essential items vs. high-level Proposal 1.

Note that this may not be an exhaustive list of essential/optimization items. If something is not mentioned as essential, no guidance from RAN is provided.

Note that the completion of essential items in Proposal 1 means completion of the discussion of the item, and not necessarily mean the item will be supported in NR-U.

Note that further discussion on completing the design to support features already agreed should be considered as high priority as well.

# 3	Discussion

## 3.1	RAN1 topics

7.2.2.1.1 Initial access signals and channels

Essential

- Wideband PRACH design (long sequence vs repetition)

- Whether to introduce LBT gap between ROs

- Supported PRACH formats (legacy PRACH and new PRACH)

- RMSI PDSCH to SSB rate matching (impacted by RAN4 sync raster decision, if the decision is to put the SSB in the middle of the initial DL BWP)

  - Also impact default PDSCH SLIV table configuration

Optimizations

- RMSI PDSCH to SSB rate matching (impacted by RAN4 sync raster decision)

  - Also impact default PDSCH SLIV table configuration

- CSI-RS FDM with SSB (impacted by RAN4 sync raster decision)

- RMSI (PLMN) transmission in Scell

- Additional PRACH numerology

- Multiplexing of PRACH and other channels

7.2.2.1.2 DL signals and channels

Essential

- Channel occupancy time and frequency domain structure indication for LBE

- UE COT detectionfor FBE

- gNB control for the Cat 4 UL transmission (including CG) switching to cat 2 when fall in gNB COT

- Behavior when P/SP-CSI-RS fail to transmit due to LBT

Optimizations

- CSI-RS enhancement

  - Behavior when P/SP-CSI-RS fail to transmit due to LBT

  - More opportunities for CSI-RS to compensate the possible LBT failure

  - A-TRS directly QCL with SSB

- Power saving with DL burst detection: Any more discussion on spec non-transparent way?

- DMRS sequence generation: Remove (some) time dependency?

7.2.2.1.3 UL signals and channels

Essential

- PUCCH EPF0/1/2/3 design

- Interlaced PUSCH resource allocation design in DCI

Optimizations

- Multiple starting position within a PUSCH

- 60KHz PUSCH interlaced waveform

- SRS waveform enhancement (interlace or stagger)

- P/SP-SRS multiple opportunities

- A-SRS triggering mechanism enhancement

7.2.2.2.1 Channel access

Essential

- 16us cat 2 LBT detailed design

- Signaling support for LBT type/priority indication

- CWS adjustment, including reference slot definition, processing timeline, what if no feedback expected or available, CWS adjustment for wideband operation and CG

- Channel access mechanism details for FBE

- UE to gNB COT sharing

Optimizations

- Max TX duration follow cat 1 LBT

- Max number of Cat 2 LBT attempts in a 5ms DRS transmission window

- Wideband LBT with common measurement

- Directional LBT

- LBT to facilitate spatial reuse

- Receiver assisted LBT

7.2.2.2.2 Initial access procedures

Essential

- Timing recovery from SSB

- Cross SSB QCL relationship by Q and A: Which A, how to indicate or determine Q, range of Q, how to determine Q for RRM

- Msg1 enhancements with more opportunities in freq domain (over multiple LBT subbands) and time domain

- RLM enhancement: IS and OOS

Optimizations

- Msg1 enhancements with more opportunities in freq domain (over multiple LBT subbands) and time domain

- Msg 3 enhancement with multiple Msg3

- RRM enhancement including subband level RSSI and channel occupancy measurement

- Actually transmitted SSB indication (any need to change from NR?)

7.2.2.2.3 HARQ enhancements

Essential

- Details for group based HARQ ACK retransmission for dynamic codebook

- Details for DCI support for multi-TTI grant

- One-shot HARQ ACK retransmission

- Details to handle non-numerical value for K1

Optimizations

- Group based HARQ ACK retransmission for semi-static codebook

- Larger K1 values

- Multiple freq domain opportunities for HARQ ACK in UL CA case and/or wideband CC case

- Larger K2 values

- 2-stage UL grant

7.2.2.2.4 Configured grant enhancements

Essential

- DFI design, including content and CG PUSCH to DFI  timing

- UCI design, including content and multiplexing

Optimizations

- Ending symbol flexibility

- CBG based transmission with CG resource

7.2.2.2.5 Wideband operations

Essential

- Coreset for wideband (multiple coresets one per LBT subband or multi-cluster coreset with one cluster per subband)

Optimizations

- PDCCH restrictions for multi-cluster coreset

- If PUSCH Alt 2 is also supported

## 3.2	RAN2

11.2.1.1 User plane - RACH

Essential

- MSG3

  - Multiple opportunities (RAN2 sent an LS to RAN1 for feasibility) can impact MAC.

- RAR

  - Timer window value and modification for RA-RNTI (or another method) to handle extended window.

- MSG1

  - Existing agreements are essential. Further enhancements are considered optimization.

Optimizations

- MSG3

  - Multiple opportunities (RAN2 sent an LS to RAN1 for feasibility) can impact MAC.

11.2.1.2 User plane - MAC

Essential

- UL LBT Failures

  - RAN2 agreed to have a mechanism for handling consistent LBT failures. In addition, since RACH counters are only updated upon successful transmission, this mechanism is needed for RACH failures.

- CG finalization

  - Configuration and retransmission are the main remaining issues. Many agreements have been made.

Optimizations

- DRX

  - Better management of active time is good for power optimization and latency but can be second priority, considering that there are different opinions among companies including some companies who do not want any optimizations.

11.2.2.1 Control plane – Inactive and idle mode

Optimization

- System Information optimizations for NR-U

  - No enhancements were agreed for single SI message case and it could be sufficient for Rel-16. Whether SIB9 for IIoT needs a second SI message should be discussed.

11.2.2.2 Control plane – Connected mode and RRC

Essential

- RLM RLF

  - The main remaining issue is how the missing RS samples are handled at PHY layer which should be decided by RAN1.

- RRM

  - Current progress in RAN2 should be sufficient. Measurement details for RSSI and CO should be decided by RAN1/RAN4.

11.2.3 Others

Essential

- The CAPC determination for CG has an FFS for multiplexing with high priority data and the Table for 5QI to CAPC mapping should be finalized.

## 3.3	RAN4

Frequency band definition: proceed according to WID

Essential

- Specify new unlicensed band for the 5 GHz frequency range

  - RAN4 agreed to refarm band46 (i.e. band n46) but RAN4 is still discussing channel raster, channel bandwidths, sub-bands, etc.

- Specify new unlicensed band for the 6 GHz frequency range

  - Band plan definition work is prioritized

System parameters

Essential

- Wideband operation (UE and BS)

  - Prioritize Alt 1 (all-or-nothing) for UL operation

  - RAN4 is currently discussing UL and DL wideband operation and related emission requirements for in-channel leakage/selectivity.

- Spectrum utilization

  - Agreement on number of RBs per 20MHz and 60kHz SCS is conditioned to agreement on emission requirements (ACLR, SEM, …)

- Sync raster

  - RAN4 is discussing synchronization raster for NR-U operation in standalone mode.

Other

Essential

- UE RF requirements

  - Limited discussions in RAN4 on this item. No agreements made so far.

- BS RF Requirements

  - Limited discussions in RAN4 on this item. Some agreements were made so far.

- RRM Requirements

  - Limited discussions in RAN4 on this item. Some agreements were made so far.

# 4	Conclusion