3GPP TSG RAN Meeting #84	RP- 191497

Newport Beach, USA, June 3-6, 2019

Agenda Item:		7.2

Source:		AT&T, Telecom Italia

Title:		3GPP ITU-R Contribution - Final IMT-2020 Submission Overview

Document for:		Approval

----------------- [remove 3GPP review part before submission to ITU] -----------------

| Radiocommunication Study Groups |  |
| --- | --- |
|  | Document 5D/TBD-E |
|  | TECHNOLOGY ASPECTS |

ATIS

3GPP 5G CANDIDATE FOR INCLUSION IN IMT-2020:

SUBMISSION 1 (SRIT)

In response to the ITU-R Circular Letter 5/LCCE/59 which invites proposals for candidate radio interface technologies for the terrestrial component of IMT-2020, the 3GPP PROPONENT joins with the Third Generation Partnership Project (3GPP) in providing a technology submission under Step 3 of the IMT-2020 process.

In this final view, intended to assist WP 5D and the evaluation groups in its preparation work on IMT-2020, the Proponent of the 3GPP submissions (hereafter known as the 3GPP Proponent) is providing at this time all the required components of the complete submissions, including the Description Template, Compliance Templates, and Self Evaluation Report, of its 5G solution:

3GPP 5G CANDIDATE SUBMISSION 1 consists of the following:

    - Submission 1: SRIT

      - Component RIT: NR

      - Component RIT: E-UTRA/LTE

Each submission is self-contained and complete unto itself, consisting of Releases 15 & 16

STATEMENT OF COMPLIANCE WITH THE PROVISIONS IN DOC. IMT-2020/2 REV 1, STEP 2

    - ITU-R Criteria:  An SRIT consists of a number of component RITs complementing each other, with each component RIT fulfilling the minimum requirements of at least two test environments and together as an SRIT fulfilling the minimum requirements of at least four test environments comprising the three usage scenarios.

- 3GPP Proponent Statement: The individual RIT Components as well as the Complete SRIT fulfills this ITU-R Criteria as indicated below

[3GPP Editor’s Note:  Confirm the entries in each table]

For Component RIT: NR  (ITU-R criteria: fulfils at least 2 test environments)

TABLE 4

(source ITU-R Report M.2412)

Mapping of test environments and usage scenarios

| Usage Scenarios | eMBB |  |  | mMTC | URLLC |
| --- | --- | --- | --- | --- | --- |
| Test Environments | Indoor Hotspot – eMBB | Dense Urban – eMBB | Rural – eMBB | Urban Macro – mMTC | Urban Macro – URLLC |
| COMPONENT RIT: NR | fufills | fufills | fufills | fufills | fufills |

For Component RIT: E-UTRA/LTE (ITU-R Criteria: fulfils at least 2 test environments)

TABLE 4

(source ITU-R Report M.2412)

Mapping of test environments and usage scenarios

| Usage Scenarios | eMBB |  |  | mMTC | URLLC |
| --- | --- | --- | --- | --- | --- |
| Test Environments | Indoor Hotspot – eMBB | Dense Urban – eMBB | Rural – eMBB | Urban Macro – mMTC | Urban Macro – URLLC |
| COMPONENT RIT: E-UTRA/LTE | fufills | fufills | fufills | fufills | User plane latency : fulfils Reliability : not assessed |

For Complete SRIT (ITU-R Criteria: fulfils at least four test environments comprising the three usage scenarios)

TABLE 4

(source ITU-R Report M.2412)

Mapping of test environments and usage scenarios

| Usage Scenarios | eMBB |  |  | mMTC | URLLC |
| --- | --- | --- | --- | --- | --- |
| Test Environments | Indoor Hotspot – eMBB | Dense Urban – eMBB | Rural – eMBB | Urban Macro – mMTC | Urban Macro – URLLC |
| COMPLETE SRIT | fufills | fufills | fufills | fufills | fufills |

This 3GPP Proponent submission for IMT-2020 is based upon the following currently in force ITU-R Reports, and the minimum technical requirements, evaluation criteria, and other information contained there-in:

| ITU-R Report Number & Date | Title | Status |
| --- | --- | --- |
| M.2410-0 (11/2017) | Minimum requirements related to technical performance for IMT-2020 radio interface(s) | In force (Main) |
| M.2411-0 (11/2017) | Requirements, evaluation criteria and submission templates for the development of IMT-2020 | In force (Main) |
| M.2412-0 (11/2017) | Guidelines for evaluation of radio interface technologies for IMT-2020 | In force (Main) |

The 3GPP Submission 1 meets or exceeds the specific requirements, evaluation criteria, and/or other information specified in the ITU-R Reports.

Attachments:

  - RP-191xy1: Characteristics Template – SRIT

  - RP-191xy2: Compliance Template - SRIT

  - RP-191xy3: Link Budget Template - SRIT

  - RP-191zzz: TR 37.910 Study on self evaluation towards IMT-2020 submission V.xyz

This contribution was developed in 3GPP TSG RAN, approved by the Organizational Partners of the 3GPP PCG, and it is being submitted ATIS on behalf of the 3GPP Proponent. Source: PCG43_xy [RP-191494]).

Developed by 3GPP as 5G, Release 15 and beyond.

The 3GPP Proponent of the 3GPP submission is collectively the 3GPP Organizational Partners (OPs). The Organizational Partners of 3GPP are ARIB, ATIS, CCSA, ETSI, TSDSI, TTA and TTC (http://www.3gpp.org/partners).

Source: RP-191497 for the 3GPP Submission 1 SRIT to ITU-R.