3GPP TSG RAN meeting #85										RP-191756

Newport Beach, USA, September 16-20, 2019

## Status Report to TSG

Agenda item:			9.4.5

| WI / SI Name | Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_L1enh_URLLC |  |  |  |  |
| Unique ID | 830074 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-191584 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 03/2020 | Performance part: 09/2020 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 60% | Performance Part: 10% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Yan Cheng |
|  | Company | Huawei |
|  | Email | chengyan.cheng@huawei.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1#98

PDCCH enhancements

Agreements:

- Introduce one new DCI format for DL scheduling and one new DCI format for UL scheduling with configurable sizes for some fields in Rel-16.

Agreements:

Support separate configurable number of bits (2 or 3 or 4 bits) for “HARQ process number” for new DCI formats for scheduling DL and UL

- FFS 0 or 1 bits

Agreements:

Support (2, 2) (4, 3) (7, 3) defined in UE feature 3-5b as the combination (X, Y) for Rel-16 PDCCH monitoring capability on the per-CC limit on the maximum number of non-overlapping CCEs   for URLLC.

- Combination (2, 1) (4, 1) (4, 2) (7, 1) (7, 2) are not additionally introduced

- FFS (3, 3) or (3,2)

- UE reports the supported combinations per SCS

- (2, 2)(4, 3)(7, 3) applicable for 15 kHz and 30 kHz

  - FFS for 60 kHz and 120 kHz

Agreements:

For a Rel-16 UE supporting enhanced PDCCH monitoring capability, down-select between option 1 and option 2:

- Option 1: PDCCH monitoring based on Rel-15 capability for eMBB and PDCCH monitoring based on Rel-16 capability for URLLC can be configured to a UE on the same carrier

  - UE monitors PDCCH for eMBB following reported Rel-15 capability, and monitors PDCCH for URLLC following reported Rel-16 capability

  - For Rel-16 PDCCH monitoring capability, the limit C on the maximum number of non-overlapping CCEs for channel estimation per PDCCH monitoring span is the same across different spans within a slot. Each span for Rel-16 PDCCH only cover USS for URLLC (FFS for CSS)

- Option 2: PDCCH monitoring for both eMBB and URLLC can be configured based on either Rel-15 capability or Rel-16 capability

  - gNB configures which capability is used

  - For Rel-16 PDCCH monitoring capability,

    - The limit C on the maximum number of non-overlapping CCEs for channel estimation per PDCCH monitoring span is the same across different spans within a slot, each span can cover CSS and/or USS

- Note: the value C is to be separately discussed

Agreements:

If UE reports the support of more than one combination of C(X, Y) for a given SCS, and if multiple combinations of C(X, Y) are valid for the span pattern, the maximum value of C of the valid combinations is applied.

- A combination C(X, Y) is valid if the span pattern satisfies X and Y of the given combination in every slot, including cross slot boundary

- FFS the impact from empty span(s) on the span pattern

The compromised proposals in Section 3.2 for email discussion/approval till 9/6 – Chengyan (HW)

Note: The above email discussion is still not concluded yet till 9/9

UCI enhancements

Agreements:

At least one sub-slot configuration for PUCCH can be UE-specifically configured to a UE.

- At least support following two sub-slot configurations for PUCCH: “2-symbol*7” and “7-symbol*2”.

  - FFS other configurable sub-slot configurations, e.g. 4, 14 sub-slots in a slot.

  - For the above two sub-slot configurations (“2-symbol*7” and “7-symbol*2”), support a single configuration for PUCCH resource following R15 applicable for all the sub-slots in a slot.

    - FFS whether or not to additionally support that PUCCH resource configuration can be different for different sub-slots

    - FFS for other sub-slot configurations, if any.

- FFS: If a PUCCH resource across sub-slot boundary is supported.

Agreements:

When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE, following can be separately configured for different HARQ-ACK codebooks:

- PUCCH-SpatialRelationInfo

- Sub-slot configuration (only applied for the sub-slot-based HARQ-ACK codebook)

  - FFS whether or not to support the case when there are at least two HARQ-ACK codebooks configured with sub-slots, with the same or different sub-slot configurations

Agreements:

When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE, the PHY identification of HARQ-ACK codebook is also used to determine the priority of the HARQ-ACK codebook for collision handling.

Agreements:

When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE,

- In case of SPS PDSCH, the following options for identifying a HARQ-ACK codebook (to down-select, combinations are not precluded)

  - Opt.1: By SPS PDSCH configurations

  - Opt.2: By the DCI activating the SPS PDSCH

  - Opt.3: By the CORESET where the activating DCI is received

Agreements:

Reuse the R15 mechanism for the following scenarios:

- A URLLC SR collides with a URLLC HARQ-ACK (no other UL signals/channels), except for (to conclude the FFSs by RAN1#98b)

  - FFS if the case in which SR with PF0 vs HARQ-ACK with PF1 needs to be considered.

  - FFS SR with HARQ-ACK in PF 2, 3, 4

- URLLC HARQ-ACK collides with URLLC PUSCH (no other UL signals/channels) when the corresponding timelines are met

  - To conclude by RAN1#98b for the error cases per R15 (especially for the cases when the timeline is not met)

Agreements:

In case URLLC (i.e., high priority) HARQ-ACK collides with eMBB (i.e., low priority) SR, down-select from options below (to conclude RAN1#98b):

- Option 1: Drop eMBB SR

- Option 2: Multiplex URLLC HARQ-ACK and eMBB SR if the multiplexing rule is met. Otherwise, drop eMBB SR.

  - FFS the details of the rule, e.g.

    - Timeline

    - Latency

    - Reliability

    - PUCCH formats

In case eMBB HARQ-ACK (i.e., low priority) collides with URLLC (i.e., high priority) SR, down-select from options below.

- Option 1: Drop eMBB HARQ-ACK

- Option 2: Multiplex eMBB HARQ-ACK and URLLC SR if the multiplexing rule is met. Otherwise, drop eMBB HARQ-ACK

  - FFS the details of the rule, e.g.

    - Timeline

    - Latency

    - Reliability

    - PUCCH formats, e.g. SR on PF0 collides with HARQ-ACK on PF1/3/4

- FFS: Resending HARQ-ACK or not after dropping.

In case eMBB HARQ-ACK (i.e., low priority) collides with URLLC (i.e., high priority) HARQ-ACK, down-select from options below.

- Option 1: Drop eMBB HARQ-ACK.

- Option 2: Multiplex eMBB HARQ-ACK and URLLC HARQ-ACK if the multiplexing rule is met. Otherwise, drop eMBB HARQ-ACK

  - FFS the details of the rule, e.g.

    - Timeline

    - Latency

    - Reliability

    - Pre-defined rules or configurable rules or dynamically-indicated multiplexing

- FFS: Resending HARQ-ACK or not after dropping.

FFS details in case of a channel/signal being dropped in handling of collision of UL channels/signals

High priority vs. low priority HARQ-ACK is made known at the PHY layer (note: for SR, it’s agreed earlier)

Email discussion on how to determine the priority of SR, A/N, and PUSCH in PHY till next meeting – Jia (OPPO)

PUSCH enhancements

Agreements:

In terms of how to interpret L and K for all PUSCH transmissions, down-select between the following two:

- Alt 1: The time window within which valid symbols are used for transmission is L*K.

  - FFS the definition of “valid symbols”

- Alt 2: The time window within which valid symbols are used for transmission can be longer than L*K symbols, and it is extended at least in case of semi-static DL symbols.

  - FFS extension of the time window in case of dynamic DL symbols and/or semi-static flexible symbols and/or reserved symbols (if defined) and/or SSB symbols and/or type-0 CSS in CORESET#0 (as indicated by MIB)

  - FFS the definition of “valid symbols”

  - FFS whether to define a maximum time window size and if so, details

Conclusion:

In terms of how to handle the interaction of enhanced PUSCH with DL/UL directions, consider the following options:

- For DG PUSCH

  - If dynamic SFI is not configured,

    - Semi-static flexible symbols are used for PUSCH. Segmentation occurs only around semi-static DL symbols.

  - If dynamic SFI is configured

    - Option 1: behavior not dependent on dynamic SFI

      - Option 1-1: Semi-static flexible symbols are used for PUSCH. Segmentation occurs only around semi-static DL symbols.

        - FFS whether the conflict between dynamic SFI and symbols used for PUSCH transmission is considered as an error case, e.g.

          - Option 1-1a: The UE does not expect any semi-static flexible symbol to be indicated as DL within the PUSCH transmission time window.

          - Option 1-1b: No error case is defined and in general all semi-static flexible symbols are used for PUSCH within the PUSCH transmission time window.

      - Option 1-2: Semi-static DL/flexible symbols are not used for PUSCH. Segmentation occurs around semi-static DL/flexible symbols.

      - Option 1-3: Dynamic indication in UL grant on which set of semi-static flexible symbols are used for PUSCH. Segmentation occurs around semi-static DL and the dynamically indicated invalid symbols.

      - Option 1-4: Pre-defined rules to determine which set of semi-static flexible symbols are used for PUSCH. Segmentation occurs around semi-static DL and the invalid symbols as defined in the rules.

    - Option 2: the UE uses SFI to determine the symbols to transmit

      - In case SFI is configured and received

        - Option 2-1: Segmentation occurs around semi-static DL symbols and dynamic DL/flexible symbols

        - Option 2-2: Dynamic flexible symbols are used for PUSCH. Segmentation occurs around semi-static DL symbols and dynamic DL symbols

        - Option 2-3: Dynamic flexible symbols are used for PUSCH. A repetition is not transmitted if it conflicts with a dynamic DL symbol.

        - Option 2-4: A repetition is not transmitted if it conflicts with a dynamic DL/flexible symbol

      - In case SFI is configured and not received

        - A repetition is not transmitted if it conflicts with a semi-static flexible symbol.

- For CG PUSCH other than the first Type 2 CG PUSCH (including all the repetitions) activated by an UL grant

  - If dynamic SFI is not configured,

    - Semi-static flexible symbols are used for PUSCH. Segmentation occurs only around semi-static DL symbols.

  - If dynamic SFI is configured

    - Option 1: behavior not dependent on dynamic SFI

      - Option 1-1: Semi-static flexible symbols are used for PUSCH. Segmentation occurs only around semi-static DL symbols.

        - This does not seem to make much sense for CG. If semi-static flexible symbols are always used for CG PUSCH, the gNB can essentially configure these symbols as UL in semi-static configuration. – no need for this option?

      - Option 1-2: Semi-static DL/flexible symbols are not used for PUSCH. Segmentation occurs around semi-static DL/flexible symbols.

      - Option 1-3 from DG is not applicable for CG.

      - Option 1-4: Pre-defined rules to determine which set of semi-static flexible symbols are used for PUSCH. Segmentation occurs around semi-static DL and the invalid symbols as defined in the rules.

    - Option 2: the UE uses SFI to determine the symbols to transmit

      - In case SFI is configured and received

        - Option 2-1: Segmentation occurs around semi-static DL symbols and dynamic DL/flexible symbols

        - Option 2-2 does not make sense for CG. (Dynamic flexible symbols are used for PUSCH. Segmentation occurs around semi-static DL symbols and dynamic DL symbols)

        - Option 2-3 does not make sense for CG. (Dynamic flexible symbols are used for PUSCH. A repetition is not transmitted if it conflicts with a dynamic DL symbol.)

        - Option 2-4: a repetition is not transmitted if it conflicts with a semi-static DL symbol and a dynamic DL/flexible symbol

      - In case SFI is configured and not received

        - A repetition is not transmitted if it conflicts with a semi-static flexible symbol.

- For the first Type 2 CG PUSCH (including all the repetitions) activated by an UL grant,

  - Alt 1: same behavior as DG PUSCH

  - Alt 2: same behavior as CG PUSCH without an associated UL grant

  - …

- FFS: in case of a repetition not being transmitted (as in the above bullets), whether a repetition is a nominal repetition or a repetition after segmentation due to semi-static DL symbol(s)/slot boundary

- FFS: whether to postpone or not, and if yes, under what condition(s)

- FFS: whether/how guard period is handled

- Note that segmentation at slot boundary is always performed, even though it is not explicitly mentioned in the bullets above.

- FFS: the handling of conflict with SSB/PRACH symbols, the handling of conflict with semi-statically configured DL reception, etc.

Other options are not precluded

Enhancements to scheduling/HARQ

Proposals:

For Rel. 16 NR, the following cases are supported:

- Case 1: different minimum processing timeline capabilities can be configured for PDSCHs on the same carrier. The minimum processing timeline capability for each PDSCH is indicated at the PHY layer.

- Case 2: additional DMRS and PDSCH processing time capability #2 can be configured simultaneously on the same carrier.

Email discussion till 9/30 for the above proposals & other related DL OoO issues – Kianoush (QC)

Enhanced inter-UE Tx prioritization/multiplexing

Agreements:

- Reuse the existing methods for search space configuration to support UL CI monitoring

  - FFS possible restrictions

  - Note: this means both symbol level and slot level monitoring periodicities are possible from specification perspective

Agreements:

- The UE DCI size budget is not increased by UL CI monitoring

- Further discuss methods to reduce the UE monitoring for UL CI, e.g.

  - The number of aggregation levels and/or candidates for the UL CI monitoring should be limited

  - Conditions for eMBB UE UL CI monitoring:

    - For UL transmission with associated PDCCH,

      - Option 1: UE starts UL CI monitoring after the PDCCH is decoded

      - Option 2: UE monitors UL CI at least at the latest monitoring occasion ending no later than X symbols before the start of the UL transmission, and X is related to UL CI processing time.

    - For UL transmission without associated PDCCH, UE monitors UL CI at least at the latest monitoring occasion that ends no later than X symbols before the start of the UL transmission, and X is related to UL CI processing time.

    - Other conditions?

  - Others?

- FFS the enhancement of UE capability (number of non-overlapping CCE and/or blind decodes) for UL CI monitoring

Agreements:

- Upon detecting an UL cancelation indication, for the transmission of UL signal/channels, “stop with resuming” is not supported

  - Except:

    - SRS can still be transmitted on the non-cancelled symbols (conditioned on if SRS can be pre-empted)

    - FFS for the PUSCH repetition (Rel-15 & Rel-16) case

    - FFS for the PUCCH repetition case (conditioned on if PUCCH can be pre-empted)

  - FFS whether another PUSCH can be scheduled in non-pre-empted resource

  - FFS impact (e.g. phase continuity issue) to a different carrier due to UL cancelation

Agreements:

- The following UL channel/signals can be cancelled by UL cancelation indication

  - PUSCH (including DG-, CG- and SP-)

  - FFS for SRS

  - FFS for PUCCH

    - Option 1: PUCCH (all types) can be cancelled

    - Option 2: Some PUCCH can be cancelled, e.g. PUCCH carrying CSI

    - Option 3: PUCCH cannot be cancelled

  - FFS for PRACH (preamble and/or MSG 3 PUSCH)

Agreements:

- The UE processing time requirement for UL cancelation indication based on N2 defined in Rel-15 UE cap#2 is supported

  - FFS whether the processing time requirement for UL cancelation indication larger than N2 as defined in Rel-15 UE cap#2 can also be supported as an UE capability

  - FFS whether the processing time requirement for UL cancelation indication shorter than N2 as defined in Rel-15 UE cap#2 as can also be supported an UE capability

Agreements:

- For a DG-PUSCH, an open-loop parameter set indicated to the UE by scheduling DCI using a separate field than SRI is supported.

  - FFS number of bits for the indication

Enhanced UL configured grant transmission

The agreements below are relevant for RAN2 work on RAN1-led eURLLC WI (NR_L1enh_URLLC):

Agreements:

- M<=4 bits indication in the Release DCI is used for indicating which CG configuration(s) is/are released, where the association between each state indicated by the indication and the CG configuration(s) is

  - Up to 2^M states are higher layer configurable, where each of the state can be mapped to a single or multiple CG configurations to be released

  - In case of no higher layer configured state(s), separate release is used where the release corresponds to the CG configuration index indicated by the indication

Conclusion:

- No support of joint activation in a DCI for two or more configured grant Type 2 configurations in Rel-16

Working assumption:

- For activation and release of UL CG, same field(s) is/are used for a DCI format

The LS reply below is relevant for SA2 and is related to SA2 LS in S2-1813386:

R1-1909805	Draft response to LS on Combination 2 of Uu QoS characteristics values for V2X services Huawei [RAN WG1]

Updated to R1-1909890, which is approved with final LS in R1-1909898

#### 2.1.2	Remaining Open issues

- PDCCH enhancements

  - Remaining details of DCI format(s)

  - Remaining details of increased PDCCH monitoring capability

- UCI enhancements

  - Remaining details of more than one PUCCH for HARQ-ACK transmission within a slot

  - Remaining details of at least two HARQ-ACK codebooks simultaneously constructed

- Remaining details of option 4 for PUSCH enhancements

- Enhancements to scheduling/HARQ

- Enhanced inter-UE Tx prioritization/multiplexing

  - Remaining details of UL cancelation scheme

  - Remaining details of enhanced UL power control scheme

- Remaining details of enhanced UL configured grant transmission

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2#107

The agreements below are relevant for both RAN1 (NR_L1enh_URLLC) and RAN2 (NR_IIOT):

UL intra-UE prioritization and enhanced UL CG transmission should be discussed and addressed under RAN2 IIOT WI while the other objectives should be discussed under RAN2 eURLLC WI.

For cases that overlap with IIOT, R2 don’t expect separate CRs for eURLLC. CR coversheet for joint IIOT eURLLC CRs should indicate WI applicability on some level.

There may be MAC impact due to PUSCH repetitions. R2 wait for R1 LS.

Noted, wait for R1 progress/conclusions on OOO HARQ to determine whether there is any impact to MAC

#### 2.2.2	Remaining Open issues

- Remaining details of enhanced UL configured grant transmission

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RRM core requirements

R4-1910025	Work plan for eURLLC RRM

Decision:		Approved

Agreement: There is no need for the new BLER pair and PDCCH transmission parameter for RLM and BFD for URLLC.

Agreement: The current RRM requirements do not need to be updated for low latency for URLLC.

Agreement: There is no RRM impact for Rel-16 eURLLC.

Demodulation performance requirements

R4-1910043	Ad hoc minutes for URLLC demodulation requirements

Decision:		Approved

R4-1910044	Way forward on URLLC demodulation performance requirements

Decision:		Approved

R4-1910127	Work Plan for URLLC demodulation requirements

Decision:		Approved

#### 2.4.2	Remaining Open issues

- RRM core requirements

  - Decide whether mechanism to prevent RLF for URLLC service is needed

  - If needed, discuss the detailed solution of the mechanism (if it is agreed to be introduced)

- Demodulation performance requirements

  - Study the remaining details of test methodology for both BS and UE

  - Specify the following performance requirements based on Rel-15 URLLC functionalities

    - Investigate and specify the RLM test cases

    - Study and specify the US/BS demodulation performance and UE CQI reporting requirements for high reliability

    - Study and specify the UE/BS demodulation performance and UE CQI reporting requirements for low latency

  - Specify the performance requirements based on Rel-16 URLLC functionalities

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#98

- “Draft Minutes report_RAN1#98_v010”, 3GPP TSG RAN WG1 Meeting 98, August 2019.

- R1-1904831	Work Plan for NR Industrial IoT WI	Nokia

- R1-1908051	PDCCH enhancements for URLLC	Huawei, HiSilicon

- R1-1908408	PDCCH enhancements for eURLLC	MediaTek Inc.

- R1-1908121	PDCCH Enhancements for NR URLLC	Ericsson

- R1-1908158	PDCCH enhancements for URLLC	vivo

- R1-1908226	PDCCH enhancement for URLLC	China Unicom

- R1-1908235	On PDCCH enhancements for NR URLLC	ZTE

- R1-1908436	On PDCCH enhancements for NR URLLC	Nokia, Nokia Shanghai Bell

- R1-1908490	DL Control for URLLC	Samsung

- R1-1908541	PDCCH enhancements for NR URLLC	LG Electronics

- R1-1908594	PDCCH enhancements for URLLC	CATT

- R1-1908645	Downlink control enhancements for eURLLC	Intel Corporation

- R1-1908667	PDCCH enhancements for URLLC	OPPO

- R1-1908777	DCI Format for URLLC	Sony

- R1-1908798	PDCCH enhancements for NR URLLC	Panasonic Corporation

- R1-1908954	Discussion on PDCCH enhancements for URLLC	Spreadtrum Communications

- R1-1909099	Discussion on PDCCH enhancements	Sharp

- R1-1909148	PDCCH enhancements for URLLC	Motorola Mobility, Lenovo

- R1-1909193	PDCCH enhancements for URLLC	NTT DOCOMO, INC.

- R1-1909264	PDCCH Enhancements for eURLLC	Qualcomm Incorporated

- R1-1909366	On DCI enhancement for NR URLLC	WILUS Inc.

- R1-1909413	PDCCH enhancements for URLLC	Sequans Communications

- R1-1909451	PDCCH Enhancements for URLLC	InterDigital, Inc.

- R1-1909491	Summary on UCI enhancements for URLLC	OPPO

- R1-1909055	On UCI enhancements for NR URLLC	Apple Inc.

- R1-1909575 	UCI Enhancements for eURLLC	Qualcomm Incorporated

- R1-1908052	UCI enhancements for URLLC	Huawei, HiSilicon

- R1-1908122	UCI Enhancements for NR URLLC	Ericsson

- R1-1908159	UCI enhancements for URLLC	vivo

- R1-1908236	UL control enhancements for NR URLLC	ZTE

- R1-1908280	UCI enhancements for NR URLLC	NEC

- R1-1908319	UCI enhancements for URLLC	Fujitsu

- R1-1908409	Multiple HARQ procedures and intra-UE UCI prioritization	MediaTek Inc.

- R1-1908437	On UCI Enhancements for NR URLLC	Nokia, Nokia Shanghai Bell

- R1-1908491	UL Control for URLLC	Samsung

- R1-1908542	UCI enhancements for NR URLLC	LG Electronics

- R1-1908595	UCI enhancements for URLLC	CATT

- R1-1908646	UCI enhancements for eURLLC	Intel Corporation

- R1-1908668	UCI enhancements for URLLC	OPPO

- R1-1908778	UCI enhancements for URLLC	Sony

- R1-1908799	Discussion on UCI enhancement for URLLC	Panasonic Corporation

- R1-1908813	UCI enhancements	ETRI

- R1-1908867	Discussion on UCI enhancements for URLLC	CMCC

- R1-1908883	UCI enhancements for URLLC	China Telecommunications

- R1-1908931	UCI enhancements for NR URLLC	Asia Pacific Telecom co. Ltd

- R1-1908955	Discussion on UCI enhancements for URLLC	Spreadtrum Communications

- R1-1909065	UCI Enhancements for eURLLC	InterDigital, Inc.

- R1-1909100	UCI enhancements for URLLC	Sharp

- R1-1909153	UCI enhancement and intra-UE prioritization for URLLC	Motorola Mobility, Lenovo

- R1-1909163	Discussion on UCI enhancements for URLLC	ITRI

- R1-1909194	UCI enhancements for URLLC	NTT DOCOMO, INC.

- R1-1909351	UCI enhancements for URLLC	CAICT

- R1-1909367	On UCI enhancement and intra-UE prioritization/multiplexing for NR URLLC	WILUS Inc.

- R1-1908939	On PUSCH enhancements for NR URLLC	Panasonic

- R1-1908053	PUSCH enhancements for URLLC	Huawei, HiSilicon

- R1-1908123	PUSCH Enhancements for NR URLLC	Ericsson

- R1-1908160	PUSCH enhancements for URLLC	vivo

- R1-1908237	PUSCH enhancements for NR URLLC	ZTE

- R1-1908320	PUSCH enhancements for URLLC	Fujitsu

- R1-1908333	PUSCH enhancement for PUSCH	China Unicom

- R1-1908410	On repetition schemes for NR PUSCH	MediaTek Inc.

- R1-1908438	On PUSCH enhancements for NR URLLC	Nokia, Nokia Shanghai Bell

- R1-1908492	PUSCH enhancement for eURLLC	Samsung

- R1-1908543	PUSCH enhancements for NR URLLC	LG Electronics

- R1-1908596	PUSCH enhancements for URLLC	CATT

- R1-1908647	Enhanced PUSCH repetitions	Intel Corporation

- R1-1908669	PUSCH enhancements for URLLC	OPPO

- R1-1908779	PUSCH Enhancements for URLLC	Sony

- R1-1908946	Discussion on PUSCH enhancements for URLLC	Spreadtrum Communications

- R1-1909101	PUSCH enhancements for NR URLLC	Sharp

- R1-1909154	PUSCH enhancement for URLLC	Motorola Mobility, Lenovo

- R1-1909195	PUSCH enhancements for URLLC	NTT DOCOMO, INC.

- R1-1909266	PUSCH enhancements for eURLLC	Qualcomm Incorporated

- R1-1909352	PUSCH enhancements for URLLC	CAICT

- R1-1909368	On PUSCH enhancement for NR URLLC	WILUS Inc.

- R1-1909412	Remaining issues for PUSCH enhancements	Sequans Communications

- R1-1909452	PUSCH Enhancements for eURLLC	InterDigital, Inc.

- R1-1908124	Scheduling/HARQ Enhancements for NR URLLC	Ericsson

- R1-1908544	Enhancements to scheduling/HARQ for NR URLLC	LG Electronics

- R1-1908054	Enhancements to scheduling/HARQ	Huawei, HiSilicon

- R1-1908161	Enhancement for Scheduling/HARQ	vivo

- R1-1908238	On scheduling/HARQ enhancements for NR URLLC	ZTE

- R1-1908290	Non-Periodic Scheduling Request Transmitter for NR Release 16	Idaho National Laboratory

- R1-1908411	Enhancements to eURLLC scheduling/HARQ	MediaTek Inc.

- R1-1908493	Scheduling/HARQ enhancement for eURLLC	Samsung

- R1-1908597	Discussion on out-of-order scheduling/HARQ	CATT

- R1-1908648	Enhancements to scheduling/HARQ for eURLLC	Intel Corporation

- R1-1908670	Ehancements to scheduling and HARQ	OPPO

- R1-1908755	Discussion on enhancements to scheduling/HARQ for NR URLLC	Nokia, Nokia Shanghai Bell

- R1-1908780	Scheduling enhancements for URLLC	Sony

- R1-1908800	Discussion on scheduling/HARQ enhancement for URLLC	Panasonic Corporation

- R1-1909066	Enhancements to Scheduling/HARQ for eURLLC	InterDigital, Inc.

- R1-1909102	Discussion on enhancements to scheduling/HARQ	Sharp

- R1-1909106	On scheduling/HARQ enhancements for eURLLC	III

- R1-1909155	Discussion on out-of-order scheduling and HARQ	Motorola Mobility, Lenovo

- R1-1909196	Enhancements to scheduling/HARQ for URLLC	NTT DOCOMO, INC.

- R1-1909267	Enhancements to Scheduling and HARQ for eURLLC	Qualcomm Incorporated

- R1-1908162	UL inter UE Tx prioritization for URLLC	vivo

- R1-1908055	UL inter-UE transmission prioritization and multiplexing	Huawei, HiSilicon

- R1-1908125	Inter-UE Prioritization and Multiplexing of  UL Transmissions	Ericsson

- R1-1908239	UL inter-UE multiplexing between eMBB and URLLC	ZTE

- R1-1908281	Enhanced inter-UE Tx prioritisation and multiplexing	NEC

- R1-1908321	Discussion on UL cancellation indication	Fujitsu

- R1-1908412	On uplink inter-UE transmission prioritization and multiplexing	MediaTek Inc.

- R1-1908494	UL Inter-UE Multiplexing/Prioritization	Samsung

- R1-1908545	Discussion on UL inter UE Tx prioritization	LG Electronics

- R1-1908598	Discussion on inter-UE UL multiplexing	CATT

- R1-1908649	Enhanced inter-UE Tx prioritization/multiplexing	Intel Corporation

- R1-1908671	Inter UE Tx prioritization and multiplexing	OPPO

- R1-1908781	UL Inter UE transmission prioritisation & multiplexing	Sony

- R1-1908814	Enhanced inter UE Tx prioritization/multiplexing	ETRI

- R1-1908868	Discussion on UL inter UE Tx prioritization/multiplexing	CMCC

- R1-1908884	UL inter-UE multiplexing between eMBB and URLLC	China Telecommunications

- R1-1908969	On inter UE Tx prioritization/multiplexing enhancements for NR URLLC	Panasonic

- R1-1908970	UL inter-UE eMBB and URLLC multiplexing enhancements	Nokia, Nokia Shanghai Bell

- R1-1909056	Considerations on URLLC UL Inter UE Tx prioritization/multiplexing	Apple Inc.

- R1-1909103	UL cancelation indication design for enhanced inter UE Tx prioritization/multiplexing	Sharp

- R1-1909149	Enhanced inter UE Tx prioritization/multiplexing for URLLC	Motorola Mobility, Lenovo

- R1-1909197	UL inter-UE transmission prioritization/multiplexing	NTT DOCOMO, INC.

- R1-1909268	Uplink Inter-UE Tx Multiplexing and Prioritization	Qualcomm Incorporated

- R1-1909353	Enhanced inter UE Tx prioritization/multiplexing for URLLC	CAICT

- R1-1909369	On UL cancellation scheme for NR URLLC	WILUS Inc.

- R1-1909411	Considerations on UL inter-UE multiplexing for URLLC	Sequans Communications

- R1-1909453	Enhanced Inter UE Transmit prioritization/multiplexing for eURLLC	InterDigital, Inc.

- R1-1908971	On Enhanced UL Configured Grant Transmission for NR URLLC and activation/release of multiple SPS configurations	Nokia, Nokia Shanghai Bell

- R1-1908056	Enhanced UL configured grant transmission	Huawei, HiSilicon

- R1-1908126	Enhancements to UL Configured Grant Transmission for NR URLLC	Ericsson

- R1-1908163	Enhanced UL configured grant transmissions  for URLLC	vivo

- R1-1908240	Enhancements for UL configured grant transmission	ZTE

- R1-1908322	UL configured grant transmission enhancements for URLLC	Fujitsu

- R1-1908413	Enhancements for NR configured-grant	MediaTek Inc.

- R1-1908495	UL configured grants for eURLLC	Samsung

- R1-1908546	Enhanced UL configured grant transmission for NR URLLC	LG Electronics

- R1-1908599	Discussion on enhanced UL configured grant transmission	CATT

- R1-1908650	Enhanced CG PUSCH transmission	Intel Corporation

- R1-1908672	Configured grant enhancements for URLLC	OPPO

- R1-1908782	Enhanced UL configured grant transmission for URLLC	Sony

- R1-1908801	Discussion on URLLC enhancements for grant-free transmission	Panasonic Corporation

- R1-1908944	Discussion on UL grant-free transmission enhancements for URLLC	Spreadtrum Communications

- R1-1909104	Enhanced UL configured grant transmission	Sharp

- R1-1909198	Enhanced UL transmission with configured grant for URLLC	NTT DOCOMO, INC.

- R1-1909269	Enhanced Grant-Free Transmissions for eURLLC	Qualcomm Incorporated

- R1-1909354	Consideration on enhanced UL configured grant transmissions for URLLC	CAICT

- R1-1909370	Discussion on UL configured grant enhancements for NR URLLC	WILUS Inc.

RAN2#107

- “Draft_RAN2_107_Report_v1”, 3GPP TSG RAN WG2 Meeting 107, August 2019.

- R2-1911257	RAN2 work plan for eURLLC	Huawei, HiSilicon

- R2-1911258	Discussion on RAN2 impact of PUSCH enhancements	Huawei, HiSilicon

- R2-1911259	Considerations on out-of-order HARQ issues	Huawei, HiSilicon

- R2-1911260	Draft LS on overlapping SPS PDSCH and dynamic PDSCH	Huawei, HiSilicon

RAN4#92

- “RAN4 #92 Meeting report”, 3GPP TSG RAN WG4 Meeting 92, August 2019.

- R4-1910025	Work plan for eURLLC RRM

- R4-1908187	Discussion on the need for URLLC-specific RLM requirement   MediaTek

- R4-1908193	Discussion on RLM requirements for URLLC  Intel

- R4-1908593	RLM design for NR URLLC  NTT DOCOMO

- R4-1909685	General discussion on RRM impacts of URLLC 	Huawei, HiSilicon

- R4-1909687	[draft] LS on RLM requirements for URLLC  Huawei, HiSilicon

- R4-1909688	WF on RRM requriements for URLLC  Huawei, HiSilicon

- R4-1910044	Way forward on URLLC demodulation performance requirements  Huawei, HiSilicon

- R4-1909872	Work Plan for URLLC demodulation requirements  Huawei, HiSilicon

- R4-1910127	Work Plan for URLLC demodulation requirements  Huawei, HiSilicon

- R4-1908198	Discussion on test feasability of URLLC requirements  Intel

- R4-1908055	NR demodulation test methodology for the test metric of 99.999% reliability  Ericsson

- R4-1908136	On the test methodology for BS for test metric of 99.999% reliability  Nokia, Nokia Shanghai Bell

- R4-1908145	Views on Testability for URLLC  Qualcomm Incorporated

- R4-1909873	Discussion on URLLC test methodology 	Huawei, HiSilicon

- R4-1909894	Views on testability for URLLC performance requirements  NTT DOCOMO

- R4-1909926	On the test methodology for low latency requirements  Nokia Corporation

- R4-1909170	UE/BS demodulation requirements for low latency  Ericsson

- R4-1909249	General overview of URLLC feature  Ericsson

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template