3GPP TSG-RAN Meeting #85	RP-192133

Newport Beach, California, US, 16th – 20th Sep 2019

Agenda item:	14 - Maintenance for closed WIs/SIs for REL-15 and earlier

Source: 	Nokia, Nokia Shanghai Bell

Title: 	On SFN alignment in Rel-15 synchronous NR-NR DC

Document for:	Discussion/Decision

# Introduction & Discussion

RAN#84 discussed the question of SFN alignment in Rel-15 NR Dual Connectivity based on [1, 2, 3] and noted the following:

after offline discussion: no way forward

conclusion: discussion can be taken up in RAN1 again but not much online time will be used so try to solve this before the RAN1 meeting; if no progress we can take it up at RAN #85 again

RAN1#98 took the discussion up based on [4, 5, 6], but was unable to conclude and reported the status in an LS to RAN [7]:

There is no consensus in RAN1 on how to clarify what synchronous in synchronous NR-DC means in Rel-15, although it is commonly understood in RAN1 that synchronous means at least slot-level synchronization

A possibility of having two UE capabilities was brought up in RAN#84, one requiring SFN-alignment, another not requiring one. This would not help the system deployment that needs to be able to support the least common denominator of the UE capabilities, and thus there should be no such division of UE types.

Further, requiring the network to SFN-align cells across frequency ranges is setting a new requirement to the network deployments and operations, while the benefit has remained unclear.

# Conclusion

Proposal: RAN should not introduce a new requirement for the deployment and operation and mandate SFN-alignment across cells for NR-DC

Proposal: RAN should refrain from introducing new NR-DC UE capabilities for UE type needing SFN alignment

# References

- RP-190992	Synchronous NR-NR DC, Qualcomm Inc.

- RP-191543	List of issues of NR-NR DC synchronous mode, Qualcomm Inc.

- RP-191583	Way forward on SFN-sync, Samsung

- R1-1908245 Clarification of sync NR-DC and remaining issues on cross-carrier scheduling, ZTE

- R1-1908517 Remaining Topics on CA and DC, Samsung

- R1-1908877 Discussion on necessity of frame boundary alignment of NR CA, CMCC

- RP-191655 LS on SFN sync, RAN WG1