3GPP TSG RAN Meeting #85	RP-192176

Newport Beach, CA, USA, Sep 16th – 20th, 2019

Agenda Item:	9.4.6

Source:	Apple Inc.

Title:	R16 UE power saving: PDCCH monitoring reduction

Document for:	Discussion, Decision

# Introduction

After the discussion for R16 UE PS WI scope in RP#84, WID was approved [1]. However, there was no conclusion made regarding PDCCH skipping and it was decided to revisit the issue in RP#85. Followings was captured in meeting note [2].

conclusion: existing agreements of the WGs shall be honored and in case of conflicts WGs should solve this otherwise RAN will address this in Sep. in RAN #85, DCI skipping aspect could be rediscussed further in Sep.

We see there are clear reasons to support this feature in R16 for UE power saving. First, it is noted that 5G handsets would potentially suffer from higher power consumption compared to LTE only handset due to increased complexity, higher data rate, and larger operating bandwidth. Increased power consumption might eventually lead to larger battery size leading to larger form factor and potentially device overheating.

Given that large portion of power consumption is contributed by empty PDCCH monitoring, we see that PDCCH skipping can provide significant power saving effect, which could potentially solve above problems.

In this contribution, we provide evaluation results of PDCCH skipping on top CDRX (short + long), R16 PS techniques such as wake up signal, and cross slot scheduling technique.

# PDCCH Skipping

## PDCCH Skipping with Short CDRX

We first show that PDCCH skipping could provide additional gain on top of CDRX. NR CDRX is very flexible mechanism. The choice of good parameter set for DRX could provide fairly good power saving gain. However, even with a lot of potential, it has not been fully utilized so far in live networks since choosing right parameter sets for current application is not an easy task. The only exception is VoIP application, in which case traffic pattern is fairly predictable. This allows network to use fairly optimized CDRX parameters e.g., 40ms of CDRX cycle with 4ms for ON duration, etc. In other cases, always fixed parameter set is used (e.g., LDC=160ms, ITA=100ms, ODT=10ms). To optimize power consumption, these parameters need to be adjusted according to application and even short CDRX could be considered.

Although the use of short CDRX is not that popular in live networks, it has been shown that there is a potential power saving gain of using short CDRX. Therefore, we have compared following three cases to see the additional gain of PDCCH skipping on top of CDRX mechanism.

- Long CDRX only

- Long CDRX + short CDRX

- Long CDRX + short CDRX + PDCCH skipping

| Parameter | Value |
| --- | --- |
| Frequency | FR1 |
| Long CDRX only | LDC=160ms, IAT=100ms, ODT=8ms |
| Long CDRX & Short CDRX | set D1) LDC=160, IAT=40, ODT=8, SDC=20ms, SDCT=3 set D2) LDC=160, IAT=5, ODT=8, SDC=20ms, SDCT=5 |
| PDCCH skipping | duration = 10ms PDCCH skipping is sent only with scheduling DCI when DL buffer gets empty after current transmission. |
| Traffic | FTP3 with 2.5MB/sec set T1) packet size = 0.5MB, mean inter arrival time = 0.2sec set T2) packet size = 0.1MB, mean inter arrival time = 0.04sec |
| Parameters not shown here follows baseline configuration given in 38.840. |  |

CDRX Parameter Set D1) LDC=160, IAT=40, ODT=8, SDC=20ms, SDCT=3

The evaluation results are shown in Figure 2 for two different traffic set T1 and set T2. The DRX parameter set D1. For traffic set T1 (left), the additional power saving gain of PDCCH skipping is 16.9% compared to Long & short CDRX. For traffic set T2(right) which is smaller packet size and mean smaller inter-arrival time, the power saving gain is 34.5% on top of short CDRX.

Figure 2 Power saving gain of PDCCH skipping compared to CDRX (Long + Short) with DRX parameter set D1 (LDC=160ms, IAT=40ms, ODT=8ms, SDC=20ms, SDCT=3). Note that the power consumption reduction in (%) is computed w.r.t Long + Short DRX case.

CDRX Parameter set D2) LDC=160, IAT=5, ODT=8, SDC=20ms, SDCT=5

In D2, for further optimization, we reduce inactivity timer (ITA) to 5ms and increase the short DRX cycle timer to 5. This CDRX configuration has very short ITA and longer short CDRX cycle timer, so it could be understood as a version of CDRX for replacing continuous monitoring period (from long inactivity timer) with short periodic monitoring (by short cycle).

This configuration, as shown below, could further reduce power consumption. However, considering that inactivity timer (ITA) is determined considering scheduling flexibility of network and the burstness of traffic, the choice of too short ITA value could have negative impact on network scheduler giving restriction on scheduling and potential decrease of system throughput. Figure 3 shows the results with DRX parameter set D2. The power consumption of short CDRX is reduced, but still PDCCH skipping could provide good power saving gain for traffic set T1 and T2. The gain for traffic T2 is large since smaller packet size with more frequent arrival makes UE stay longer in DRX ON state, where PDCCH skipping has more chance to make the UE skip PDCCH monitoring.

Figure 3 Power saving gain of PDCCH skipping compared to CDRX (Long + Short) with DRX parameter set D2 (LDC=160ms, IAT=5ms, ODT=8ms, SDC=20ms, SDCT=6). Note that the power consumption reduction in (%) is computed w.r.t Long + Short DRX case

Observation 1

- PDCCH skipping can provide additional power saving gain of 5.2%~34.5% on top of Long+Short CDRX.

## PDCCH Skipping with R16 Features (Wake Up Signal, Cross Slot Scheduling)

R16 has two power saving features already in its WI scope : wake up signal(WUS) and cross slot scheduling (CS). Wake up signal is useful when traffic arrival is sporadic and UE spends large amount of time in alternating CDRX ON/OFF state. In such case UE could effectively skip ON durations without grant arrival. Cross slot scheduling technique is useful when traffic arrival rate is low resulting in PDCCH only monitoring in many slots. It has been shown throughout the SI that there are fair amount of power saving gain from these two techniques. Question is how much additional power saving gain PDCCH skipping can bring on top of these two. In this evaluation, we show the PDCCH skipping can provide still good power saving gain on top of R16 features and both long and short CDRX.

Long DRX only

Figure 4 shows the power consumption and power saving gain with long DRX only case. The additional PS gain of PDCCH skipping ranges from 5.8% to 25.6% depending on the skip duration and traffic pattern (mostly packet inter arrival time). It is observed that gain increases in the traffic pattern with higher arrival rate (0.1MB/0.04sec).

Figure 4 Power saving gain of PDCCH skipping compared to R16 features (wake up signal(WUS), cross slot scheduling(CS)) with Long DRX only  (LDC=160ms, IAT=100ms, ODT=8ms). Note that the power consumption reduction in (%) is computed w.r.t WUS+CS case.

Long DRX + Short DRX

Figure 5 shows the power consumption and power saving gain with long+short DRX case. The additional PS gain of PDCCH skipping ranges from 7.8% to 28.2% depending on the skip duration and traffic pattern (mostly packet inter arrival time). Similarly, it is observed that gain increases in the traffic pattern with higher arrival rate(0.1M/0.04sec).

Figure 5 Power saving gain of PDCCH skipping compared to R16 features (wake up signal(WUS), cross slot scheduling(CS)) with Long DRX + short DRX (LDC=160ms, IAT=40ms, ODT=8ms, SDC=20ms, SDCT=3ms). Note that the power consumption reduction in (%) is computed w.r.t WUS+CS case.

Observation 2

- PDCCH skipping can provide additional power saving gain of 5.8%~28.2% on top of short CDRX and R16 features (wake up signal and cross slot scheduling).

Proposal 1

- NR supports PDCCH skipping indication in R16 UE PS WI.

# Conclusions

In this contribution, we have shown that PDCCH skipping could provide significant power saving gain even with DRX (Long+Short) mechanism and R16 features such as cross slot scheduling and wake up signal. Based on the results, we made following proposal.

Observation 1

- PDCCH skipping can provide additional power saving gain of 5.2%~34.5% on top of Long+Short CDRX.

Observation 2

- PDCCH skipping can provide additional power saving gain of 5.8%~28.2% on top of short CDRX and R16 features (wake up signal and cross slot scheduling).

Proposal 1

- NR supports PDCCH skipping indication in R16 UE PS WI.

# References

- RP-191607 Revised WID: UE Power Saving in NR, CATT, Jun 2019

- draft_MeetingReport_RAN_84_Newport_Beach_190606_eom.rtf