3GPP TSG RAN Meeting #85	  	RP-192239

Newport Beach, CA, USA, 16th -20th   September, 2019   		was RP-192068

Agenda Item: 10.2.2

Source: MediaTek Inc., China Telecom, Huawei, HiSilicon, Telstra

Title: Rel-16 16QAM support for NB-IoT

Document for: Discussion and Decision

# Introduction

A new RAN1-led Working Item on Rel-16 enhancements for NB-IoT was approved at RAN Plenary  #80 [1]. In RAN#82, the scope of the WID was extended to include Connection to 5GC in alignment with the corresponding work in SA2 [2]. In RAN#83, a further WID extension was approved to include  support for quality reporting in connected mode for anchor and non-anchor carriers [3].

This contribution aims to discuss the benefits of  higher order modulation 16QAM for IoT application typcially requiring higher data rates and propose a further WID extension to support include support of 16QAM in NB-IoT.

# Discussion

NB-IoT is in many markets replacing GSM/EDGE for IoT usage. Requirements are diverse and while NB-IoT meet some part of requirements well (e.g. for stationary meters), others need enhancement, in particular:

- Tracker application reporting at high rate, e.g. once/s, once/2s.

- File download

- Media applications, e.g. kids toy with audio, smart home VAD (Voice Assistant Device)

In [4], the theoretical peak data rate for Rel-14 2 HARQ with TBS 2536 bits is provided. For non-anchor carrier in standalone mode, the DL and UL peak data rate is summarized in Table 1.

| R14 stand alone, non-anchor carrier | DL | UL |
| --- | --- | --- |
| Peak data rate | 126.8 kbps | 158.5 kbps |

- Table 1: R14 peak data rate for non-anchor carrier in standalone mode

Examples of IoT application that demands higher data rates than currently possible with NB-IoT are given in Table 2. NB-IoT often provides worse real time characteristics than GSM/EDGE. For interactive embedded UEs this is problematic, for example, sales or payment terminal (a high volume application).

Observation 1: The data rates achievable with NB-IoT are too restricted for some applications that would otherwise benefit from NB-IoT.

Observation 2: Higher data rates in NB-IoT can make NB-IoT viable for a greater number of applications.

| Applications | Description | Data Rate Requirement |
| --- | --- | --- |
| FOTA | Software upgrade | 250 kbps~300 kbps |
| Health/fitness | Heartbeat monitoring | 192 kbps~256 kbps |
| Smart home VAD (Voice Assistant Device) | Voice command for remote control | 200 kbps |

Table 2: IoT application that demand higher NB-IoT data rate

Examples of IoT application that demand lower NB-IoT latency are given in Table 3. Compared to GSM/EDGE, NB-IoT provides worse spectrum efficiency and worse latency performance. If traffic load on a carrier is high, there may not be sufficient availability of resources to transmit packets for cell edge UEs with required reliability. Increasing the data rates of UEs allows these UEs to receive data more quickly. This will result in better latency overall.

Observation 3: The latency achievable with NB-IoT is too large for some applications that would otherwise benefit from NB-IoT.

Observation 4: Higher data rates in NB-IoT can enable lowering the latency hence make NB-IoT viable for a greater number of applications.

| Applications | Latency Requirement |
| --- | --- |
| POS (point of sale) Machine High volume application | 500 ms ~ few sec |
| Smart Grid Demand Response | 500 ms |
| Smart Door (lock/unlock) | 600 ms |

Table 3: IoT application that demand lower NB-IoT latency

The typical amount of data in a transmission varies from application to application depending on the use cases for low-end embedded UEs (IoT UEs). The spectral efficiency should be at least as good as with EDGE in normal coverage. Figure 1 shows the CDF DL geometry for mMTC in IMT2020 evaluation, where the assumed bandwidth is 180 kHz [5]. Based on typical NB-IoT DL demodulator performance, it is expected that 16QAM can be used at SINR in the region of 7 dB or higher. It is expected that up to 30%-40% UEs could be scheduled with 16QAM with ISD=500 m. Similar observations can be made for UL. Increasing the data rates of UEs in normal coverage allows to use more resources for UEs nearer to the cell edge. This allows a higher spectrum efficiency.

Observation 5: Higher data rates in NB-IoT through higher order modulation 16QAM allows a higher spectrum efficiency hence better radio resource usage.

Figure 1: Coupling gain and DL geometry of Urban Macro - mMTC test environment

Support of 16QAM in NB-IoT will require the introduction of a new UE category Cat NB3. The carrier bandwidth is not proposed to be changed i.e. it shall remain 200 kHz. The expected impact to introduce support of 16QAM for NB-IoT on the specifications is expected to be minor i.e. extension of TBS tables for NPDSCH and NPUSCH in the specifications will be needed [6, 7].

Observation 6: The expected impact to introduce support of 16QAM for NB-IoT on the specifications is expected to be minor i.e. extension of TBS tables for NPDSCH, NPUSCH and introduction of a new UE Category Cat NB3.

NB-IoT can be provided as a standalone solution or as part of multi-mode implementation. For the latter, hardware sharing allows to expand the capabilities of NB-IoT at no added complexity cost. For the former, the additional hardware complexity to support 16QAM is deemed to have a minor effect on the overall cost of a NB-IoT UE.

Observation 7: The additional hardware complexity to support 16QAM is expected to have only a minor effect on the overall cost of a NB-IoT UE.

Making NB-IoT viable for a greater number of applications, especially very common IoT applications using GSM/EDGE today as shown above, will result in a greater market and unprecedented economies of scale for this technology. This will not only allow keeping the costs very low as originally intended, it will also allow operators to truly migrate their solution towards a single core network i.e. EPC (and, eventually 5GC).

Observation 8: As for earlier releases, economies of scale will be a primary contributor to keeping the overall cost very low with Cat NB3.

Observation 9: 16QAM is a true low-hanging fruit that can transform NB-IoT and its addressable market whilst retaining its key low-cost attribute.

Since the impact on the specifications is expected to be small, it is preferred to introduce 16QAM in NB-IoT in Release 16 to meet market demand.

Proposal 1: Revise the Release 16 NB-IoT WID to specify support for higher order modulation 16QAM for PDSCH and PUSCH.

# Conclusion

Observation 1: The data rates achievable with NB-IoT are too restricted for some applications that would otherwise benefit from NB-IoT.

Observation 2: Higher data rates in NB-IoT can make NB-IoT viable for a greater number of applications.

Observation 3: The latency achievable with NB-IoT is too large for some applications that would otherwise benefit from NB-IoT.

Observation 4: Higher data rates in NB-IoT can enable lowering the latency hence make NB-IoT viable for a greater number of applications.

Observation 5: Higher data rates in NB-IoT through higher order modulation 16QAM allows a higher spectrum efficiency hence better radio resource usage.

Observation 6: The expected impact to introduce support of 16QAM for NB-IoT on the specifications is expected to be minor i.e. extension of TBS tables for NPDSCH, NPUSCH and introduction of a new UE Category Cat NB3.

Observation 7: The additional hardware complexity to support 16QAM is expected to have only a minor effect on the overall cost of a NB-IoT UE.

Observation 8: As for earlier releases, economies of scale will be a primary contributor to keeping the overall cost very low with Cat NB3.

Observation 9: 16QAM is a true low-hanging fruit that can transform NB-IoT and its addressable market whilst retaining its key low-cost attribute.

Proposal 1: Revise the Release 16 NB-IoT WID to specify support for higher order modulation 16QAM for PDSCH and PUSCH.

# References

- RP-181451, New WID on Rel-16 enhancements for NB-IoT, Ericsson, TSG RAN#80, June 2018

- RP-182292, WID revision: Additional enhancements for NB-IoT, Huawei, TSG RAN#82, December 2018

- RP-190337, WID revision: Additional enhancements for NB-IoT, Huawei, TSG RAN#83, March 2019

- R1-1702749, Max TBS and soft channel bits support for Rel-14 NB-IoT 2 HARQ processes, MediaTek Inc.

- TR 37.910, Study on Self Evaluation towards IMT-2020 Submission, rel-16, v1.0.0, 2018-12

- R1-1906654, Support of 16QAM in NB-IoT, MediaTek Inc, RAN1#97, Reno, May 2019

- R1-160445, NB-PDSCH design for NB-IoT, Nokia, RAN1#84, Malta, February 2016