3GPP TSG RAN meeting #86										  RP-192414

Sitges, Spain, December 9-12, 2019

## Status Report to TSG

Agenda item:			9.4.4

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: Yes/No |
| Acronym | NR_2step_RACH |  |  |  |  |
| Unique ID | 820068 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-192330 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 03/2020 | Performance part: 06/2020 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 95% | Performance Part: 0% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Li Tian |
|  | Company | ZTE Corporation |
|  | Email | tian.li150@zte.com.cn |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

The following agreements were made in RAN1#96:

Agreements:

- PUSCH occasion for 2-step RACH is defined as

  - the time-frequency resource for payload transmission

- Consider the following methods for PUSCH occasion of msgA transmission:

  - Opt 1: PUSCH occasions are separately configured from PRACH occasions

    - For one PUSCH occasion, it is derived based on:

      - Alt 1: reuse the resource allocation for NR configured grant in principle

      - Alt 2: other potential configurations (e.g., reuse semi-static SFI + BWP,  reuse PRACH RO, etc.)

    - FFS detailed association rule between the PRACH and PUSCH for msgA transmission

  - Opt 2: Specify/configure the relative location (in time and/or frequency) of the PUSCH occasion with respect to the associated PRACH occasion

    - Alt 1: Time/frequency relation between PRACH preambles in PRACH occasion(s) and PUSCH occasions are single specification fixed value.

    - Alt 2: Time/frequency relation between each PRACH preamble in PRACH occasion(s) to the PUSCH occasion is single specification fixed value. Different preambles in different PRACH occasions can have different values.

    - Alt 3: Time/frequency relation between PRACH preambles in PRACH occasion(s) and PUSCH occasions are single semi-statically configured value.

    - Alt 4: Time/frequency relation between each PRACH preamble in PRACH occasion(s) to the PUSCH occasion is semi-statically configured value. Different preambles in different PRACH occasions can have different values.

    - Note: The time and frequency relation is not required to be the same alternative.

  - FFS detailed mapping between preamble and PUSCH resource + DMRS

Agreements:

- Both DFT-s-OFDM and CP-OFDM are supported for the payload transmission in msgA

  - FFS how to indicate/configure the waveform

- Consider the following numerology for msgA PUSCH (for possible down-selection)

  - Alt 1: ​follow the numerology configured for the UL BWP

    - FFS initial vs. active UL BWP

  - Alt 2:  same as msgA preamble numerology at least for some cases

    - E.g., when short preamble is used (L=139)

Agreements:

- For the relation of PRACH resources between 2-step and 4-step RACH, further study the following options (for possible down-selection or combination(s) of the options)

  - Option 1: Separate ROs are configured for 2-step and 4-step RACH

  - Option 2: Shared RO but separate preambles for 2-step and 4-step RACH

  - Option 3: Shared RO and shared preambles for 2-step and 4-step RACH

Agreements:

- The beam association rule between SSB and RACH occasion of 4-step RACH is to be used for 2-step RACH

  - FFS beam association for PUSCH

Agreements:

- At least open loop power control for PUSCH transmission in MsgA should be supported

  - FFS PC for preamble vs. PC for PUSCH

Agreements [Post RAN1#96 Email discussion]

- Adopt the link-level simulation assumptions in the following table for the initial evaluation of feasible payload size, and for the potential down-selection of schemes, e.g.

  - whether to have shared ROs and/or preambles between 2-step RACH and 4-step RACH, and if yes the percentage for partitioning

  - whether to have the guard time between PRACH and PUSCH and/or guard period within PUSCH, and if yes the length of GT/GP

  - the mapping scheme between preamble and PUSCH+DMRS, .e.g 1-to-1, multiple-to-1, or 1-to-multiple

  - appropriate power offset(s) between preamble and PUSCH, and whether to support repetition of MsgA PUSCH

  - whether to have UCI in msgA PUSCH, if a PUCCH transmission overlaps the PUSCH part of MsgA

  - whether to dynamically adapt the payload size and indicate by UCI in msgA PUSCH, and if yes the content and structure of UCI

  - FFS other schemes, e.g. whether guard band is included

- Additional system-level simulations or analytical evaluations can be considered for the following analysis:

  - Latency

  - Signalling overhead

  - Resource reservation overhead

  - PUSCH collision, with definition FFS, e.g. overlapped PUSCH occasion, with shared or separate DMRS port, and with same or different scrambling ID

  - Rx detection complexity

- Note 1: the supported/recommended payload size from RAN1 perspective may also need to take other factors into account, e.g. use cases, resource utilization.

- Note 2: the WID scope should be strictly followed when using the evaluation results for the comparison of schemes.

Table 2: Link-level evaluation assumptions

| Parameters | Values |
| --- | --- |
| Scenario | - For evaluation of schemes: 200m, UMi, 4 GHz. FFS: 500m, UMa, 4 GHz. - For evaluation of payload size: 200m, UMi, 4 GHz; or 500m, UMa, 4 GHz; or 1732m, RMa, 700 MHz; or 25km, RMa, 700 MHz. Other values can be reported if applicable. Note: this does not restrict preamble format selection. |
| Preamble format | Company report |
| Waveform (data part) | CP-OFDM, or DFT-s-OFDM |
| Subcarrier spacing for PUSCH | 15kHz at 700MHz, 30/60kHz at 4 GHz, 120kHz at 30GHz |
| TBS | - 56 bits as starting point for minimum payload size, other values are not precluded - Company report for the evaluation of payload size |
| MCS and Resource size | Company report the MCS, time/frequency resource size, and DMRS overhead. Strive to agree to some common values in RAN1#96bis. |
| Number of UEs | 1 as a starting point; FFS: 2 or more for evaluation of shared PUSCH occasion or interference from the adjacent PUSCH resource, including how to model relative SINR, timing, etc. |
| Traffic model | Full buffer |
| UE antenna configuration | 1Tx. FFS: 2 Tx |
| gNB antenna configuration | 2Rx or 4Rx, 8Rx as optional |
| Propagation channel & UE velocity | TDL/CDL-A 30ns, or TDL/CDL-C 300ns, 3km/h or 30km/h |
| Timing offset | Uniform [0, RTT]. |
| Frequency offset | 0.05ppm (fixed) at TRP, and 0.1 ppm (fixed) at UE |
| Max number of HARQ transmission | 1 as starting point, other values are not precluded and company should report the details of HARQ |
| Receiver | MMSE-IRC as baseline |
| Channel estimation | Realistic for both channel estimation and TO/FO estimation. Ideal can be considered for calibration, if needed. |
| Target BLER | [10%, 1%] for 1st transmission of msgA as starting points. |
| Performance metrics | 1) Missed detection probability vs. SNR for a given false alarm rate, e.g. 0.1%; 2) BLER vs. SNR; MCL can be reported using link budget calculations. 3) Optional: False alarm probability vs. SNR |

The following agreements were made in RAN1#96bis:

Agreements:

- One or more PUSCH occasion(s) within an msgA PUSCH configuration period are configured.

  - FFS msgA PUSCH configuration period, e.g.

    - For opt. 1 with separate PUSCH configuration, msgA PUSCH configuration period may or may not be the same as PRACH configuration period

    - For opt. 2 PUSCH configuration with relative location, msgA PUSCH configuration period is the PRACH configuration period

Agreements:

- PUSCH resource unit for 2-step RACH is defined as

  - The PUSCH occasion and DMRS port / DMRS sequence used for an msgA payload transmission.

    - FFS support only one or both of DMRS port / DMRS sequence

    - The DMRS sequence generation mechanism should follow Rel.15.

Working assumption:

- At least support one-to-one and multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit.

  - Configurable number of preambles (including one or multiple) mapped to one PUSCH resource unit

  - FFS one-to-multiple mapping

- Companies are strongly encouraged to perform additional evaluations/analysis

Agreements:

- Support the PRACH and PUSCH for msgA transmission in different slots. In this case, the numerology for msgA PUSCH follow the numerology configured for the UL BWP for msgA transmission.

  - FFS whether to support PRACH and PUSCH in the same slot for msgA transmission. If supported, down-select from the following option

    - Opt 1: the numerology for msgA PUSCH follows that of msgA preamble

    - Opt 2: gNB configure whether the numerology for msgA PUSCH follows that of msgA preamble or UL BWP

    - Opt 3: a UE is not expected to be configured with different numerology among PRACH preamble, msgA PUSCH and UL BWP for msgA transmission

    - Note: in Rel.15 the PRACH and PUSCH transmitted in the same slot for a UE are not supported

Agreements:

- Adopt the following table for the link-level evaluation:

| Parameters | Values |
| --- | --- |
| The number of PUSCH symbols & PUSCH mapping type | 14, Type A; [6], Type B as optional |
| 1) Total Number of PRBs for msgA PUSCH Or 2) number of PRBs per PUSCH occasion Note: either of them should be aligned for scheme comparison | [6, 12] Or [1,2,3] |
| PUSCH DMRS overhead | [2 or 3] DMRS symbols |
| Frequency hopping for msgA PUSCH | Company report, enabled/disabled |
| Preamble format | Format 0/[A1]; [32, 64] preambles in each RO. Other preamble formats or number of preambles are not precluded Note: company report number of SSBs per RO |
| Number of UEs | 1 as a starting point; FFS: 2 or more for evaluation of shared PUSCH occasion or can be reported Power modelling for FFS (Note: before the FFS is resolved, companies to report the detailed modelling) FFS: interference from the adjacent PUSCH resource occasion, including how to model relative SINR, timing, etc. |
| TBS | - 56 72 bits as starting point for minimum payload size, other values are not precluded - Company report for the evaluation of payload size |

Agreements:

- For the relation of PRACH resources between 2-step and 4-step RACH, the network has the flexibility to configure the following options:

  - Option 1: Separate ROs are configured for 2-step and 4-step RACH

  - Option 2: Shared RO but separate preambles for 2-step and 4-step RACH

Agreements:

- Further study the granularity of the time advance command, if supported in MsgB:

  - E.g., Based on the subcarrier spacing of MsgA PUSCH using a 12-bit TA command, where the granularity of the TA command is determined according to the following table.

| Subcarrier Spacing (kHz) of the msgA PUSCH data part | Unit |
| --- | --- |
| 15 | 16*64 Tc |
| 30 | 8*64 Tc |
| 60 | 4*64 Tc |
| 120 | 2*64 Tc |

  - Other options/variations are not precluded

Agreements:

For 2-step RACH preamble power control parameter configuration, further study and down select from the following options:

- Option 1: Power control parameters can be separately configured for 2-step and 4-step RACH.

  - If a power control parameter is not configured for 2-step RACH, the corresponding power control parameter of 4-step RACH is used instead for 2-step.

- Option 2: The corresponding power control parameter of 2-step RACH preamble follows that of 4-step RACH preamble.

Agreements:

For the determination of the PUSCH Tx power, further study at least the following components including possible down selection:

- An offset relative to the preamble received target power

  - Option 1.1: Offset configured for 2-step RACH

  - Option 1.2: Offset is the release 15 delta_preamble_msg3

  - Option 1.3: Offset is the release 15 delta_preamble_msg3 + configurable delta

- An offset relative to the MsgA PRACH Tx power for the MsgA PUSCH Tx power configured for 2-step RACH.

- Transmission bandwidth of MsgA PUSCH

- MsgA PUSCH Transport format (ΔTF). Further study the following options for further down selection

  - Option 2.1: deltaMCS configured for 2-step separate from 4-step

  - Option 2.2: reuse deltaMCS of 4-step RACH

- Preamble received target power.

- Pathloss. Further study the following options for further down selection

  - Option 4.1: Full pathloss compensation (α = 1)

  - Option 4.2: Partial pathloss compensation alpha configured for 2-step separate from that of 4-step RACH.

  - Option 4.3: Partial pathloss compensation using msg3-alpha.

- RS resource index for pathloss estimation.

- Total power ramp-up requested by higher layers for MsgA PUSCH Tx:

  - Option 6.1: from the first to the current MsgA PUSCH transmission (∆Prampuprequested).

  - Option 6.2: from the first to the latest random access MsgA preamble transmission (∆Prampuprequested).

  - Note: Latest means most recent transmitted.

- Power reduction priority rule in CA/DC

Agreements:

- For MsgA Tx beam selection further study at least the following options:

  - Option 1: The MsgA PRACH and MsgA PUSCH use the same Tx spatial filter (beam).

  - Option 2: The MsgA PRACH and MsgA PUSCH use same or different Tx spatial filter (beam) up to UE implementation.

    - No spec impact expected.

    - Note: in 4-step RACH it is up to UE implementation to decide the beams for Msg1 and Msg3.

  - Option 3: The MsgA PRACH and MsgA PUSCH use same or different Tx spatial filter (beam) under network control/assistance.

- MsgA retransmission, if supported, is defined as a retransmission of MsgA PRACH (with a re-selection of preamble) and MsgA PUSCH. Further study at the following options:

  - Option 1: Using the same payload for MsgA PUSCH.

  - Option 2: MsgA PUSCH payload can be different.

  - FFS: Conditions for MsgA retransmission and relation to fall back.

- FFS: retransmission of PUSCH only.

- FFS: retransmission of PRACH only.

The following agreements were made in RAN1#97:

Agreements:

- The following parameters are defined per msgA PUSCH configuration:

  - Common parameters for both option 1 (separate configuration) and option 2 (relative location), at least include:

    - MCS and/or TBS (to be further decided)

    - Number of FDMed POs

      - POs (including guard band or guard period, if exist) under the same msgA PUSCH configurations are consecutive in frequency domain

    - Number of PRBs per PO

    - Number of DMRS symbols/ports/sequences (if support) per PO

    - FFS whether or not support repetitions for msgA PUSCH

    - FFS bandwidth of PRB-level guard band or duration of guard time

    - FFS PUSCH mapping type

  - Parameters specific to option 1, at least include:

    - Periodicity (msgA PUSCH configuration period)

      - FFS value range

    - Offset(s) (e.g., symbol, slot, subframe, etc.)

    - Time domain resource allocation, details FFS, e.g., in a slot for msgA PUSCH: starting symbol, number of symbols per PO, number of time-domain POs, etc.

    - Frequency starting point

  - Parameters specific to option 2, at least include:

    - Single time offset (combination of slot-level and symbol-level indication) with respect to a reference point

      - FFS, e.g., each PRACH slot (e.g., start or end of the PRACH slot), etc.

    - Number of symbols per PO

      - FFS explicit or implicit indication

    - Single frequency offset with respect to FFS (the start of the first RO in frequency or the end of the last RO in frequency)

  - FFS: Number of TDMed POs

- Support multiple msgA PUSCH configurations for a UE

  - FFS the maximum number of configurations

  - FFS which parameters, if any, are common for all configurations

  - FFS indication of different msgA PUSCH configurations, e.g. by different ROs, by different preamble groups, or by UCI

  - FFS whether or not resources for different msgA PUSCHs can be overlapped in time-frequency, and if so, any spec impact

- FFS whether the frequency resource of msgA PUSCH should be limited to the bandwidth of PRACH

- FFS validation rule of msgA PUSCH

Agreements:

- The c_init for msgA PUSCH scrambling is at least derived based on a RNTI, preamble index, and/or n_ID (which can be  cell ID or configurable, to be FFS).

  - FFS details of the RNTI

  - FFS the inclusion of DMRS index.

Agreements:

- MsgA shall support all the preamble formats specified for NR release 15.

Agreements:

- From RAN1 perspective, when re-transmitting MsgA, and if the MsgA PRACH is on a different spatial filter (beam) than the latest MsgA PRACH transmission, layer 1 notifies higher layer to suspend the power ramping counter of MsgA PRACH,

  - FFS: How to determine the retransmitted MsgA PUSCH Tx power.

Conclusion:

In the reply LS to the RAN2 on power ramping,

- Include the agreements right above, and

- Mention that RAN1 discussed the suspension of power ramping counter when retransmitting MsgA, and if MsgA preamble is associated with a different SSB than the latest MsgA preamble transmission. The suspension of the power ramping counter for this scenario in case of 4-step RACH is described in the RAN2 specifications. It is up to RAN2 to agree on a similar behavior for 2-step RACH.

Agreements:

- The proposals in 5.2.6 of R1-1907900 is agreed

[Copied as below, also captured in the approved LS to RAN2 R1-1907948]

During MsgA PUSCH retransmissions, the MsgA PUSCH Tx power in transmission instance i is PPUSCHi, where

PPUSCHi=maxPCMAX,[MsgA]preambleReceivedTargetPower+∆MsgA_PUSCH+10log102μMRBPUSCHi+αPL(i)+∆TFi+∆rampupi

- ∆MsgA_PUSCH is an offset relative to the preamble received target power that could be configured for 2-step RACH. If the offset parameter is absent, the parameter delta_preamble_msg3 of 4-step RACH is used.

- [Working Assumption] The power component from the transport format ∆TFi is determined based on the same mechanism and the same parameter deltaMCS of Rel-15 Msg3 for the current transmission instance.

- The power component from pathloss compensation, αPLi, is determined by an alpha parameter, which is UE specific that is configured for 2-step separate from that of 4-step RACH. If the 2-step RACH alpha parameter is absent, the parameter msg3-alpha of 4-step RACH is used.

  - FFS: cell-specific MsgA PUSCH alpha.

- For the downlink pathloss estimate for MsgA PUSCH power control, the UE uses the same RS resource index as that used for the corresponding MsgA PRACH

- The power ramping component is given by;

∆rampup(i)=minmax0,PCMAX-10log102μMRBPUSCHi+[MsgA]preambleReceivedTargetPower+∆MsgA_PUSCH+αPLi+∆TFi,∆rampuprequested

  - Where, ∆rampuprequested is the requested ramp up from higher layers

  - Further study and down select from the following alternatives:

    - Alt1: Same ramp up for MsgA PUSCH and MsgA PRACH

∆rampuprequested=PREAMPLE_POWER_RAMPING_COUNTER-1×[MsgA]powerRampingStep

      - FFS: same power ramping counters for 2-step RACH MsgA PRACH and 4-step RACH Msg1.

    - Alt 2: Separate ramp up for MsgA PUSCH and MsgA PRACH, with different counters

∆rampuprequested=MSGAPUSCH_POWER_RAMPING_COUNTER-1×PUSCHpowerRampingStep

    - Alt3: Separate ramp up for MsgA PUSCH and MsgA PRACH, with the same counter

∆rampuprequested=PREAMBLE_POWER_RAMPING_COUNTER-1×PUSCHpowerRampingStep

Agreements:

- RACH preamble power control parameters include: powerRampingStep and preambleReceivedTargetPower.

The following agreements were made in RAN1#98:

Agreements:

- The following parameters are further defined per msgA PUSCH configuration

  - Common parameters for both option 1 (separate configuration) and option 2 (relative location)

    - Number of slots (in active UL BWP numerology) containing one or multiple POs, each slot has the same time domain resource allocation

    - Number of time domain POs in each slot

      - POs including guard period are contiguous in time domain within a slot

    - SLIV-based, indicating the start symbol of the first PO in each slot, and the number of occupied symbols of each PO in time domain

      - the number of occupied symbols excludes the guard period

    - PUSCH mapping type A or B

    - Configurable guard period, value range in the unit of symbols FFS

    - Frequency start point with respect to the first PRB of the active UL BWP

      - FFS: configurable PRB-level guard band, up to 1 PRB

Agreements:

  - At least support same configuration periodicity for msgA PRACH and PUSCH

    - Single time offset with respect to the start of each PRACH slot, counted as the number of slots (based on the numerology of active UL BWP)

      - Note: The symbol level offset is implied in SLIV-based indication

    - FFS how to handle the overlapping between POs

    - FFS whether and how to support different configuration periodicities

Agreements:

- For the definition of PRU, support both DMRS ports and DMRS sequences at least for CP-OFDM

  - More than 1 DMRS sequence can be configured, FFS the value

  - FFS whether/how to support multiple sequences for DFT-s-OFDM

  - The conditions under which only DM-RS ports are to be specified. FFS details

- Confirm the working assumption that both one-to-one and multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit (PRU) are supported

  - Configurable number of preambles (including one or multiple) mapped to one PRU, explicitly or implicitly

  - FFS 1-to-multiple mapping

Agreements:

- The offline agreement 5.2.1 in R1-1909726 is agreed

[copied below]

Correct typo in formula from RAN1#97

PPUSCHi=maxminPCMAX,[MsgA]preambleReceivedTargetPower+∆MsgA_PUSCH+10log102μMRBPUSCHi+αPL(i)+∆TFi+∆rampupi

Agreements:

- Any performance difference of 2-step and 4-step preambles (e.g. probability of missed-detection) is influenced by parameters some of which are under the control of the network (which the gNB has the flexibility to make the same or different) such as the preamble format (if supported and allowed to be configured differently), number of configured preambles (pool size), number of users attempting random access (traffic loads) and when applicable, power control parameters (such as preambleReceivedTargetPower and powerRampingStep).

  - This applies to shared ROs and separately configured ROs.

- Switching to 4-step RACH doesn’t just depend on MsgA PRACH performance, but on the impact of MsgA PUSCH on performance as well.

- Based on the above points, the preamble performance of 2-step RACH and 4-step can be different.

- RAN1 views that it can be beneficial to allow UE to switch to 4-step RACH.

Agreements:

- If a single RACH type is to be selected and when a UE is configured with 4-step RACH and 2-step RACH procedures, during random access procedure initialization:

  - One criterion for determination of random access procedure type can be based on an SSB-based RSRP threshold.

    - An SSB-based RSRP threshold can be optionally configured.

    - If the threshold is configured, if and how the UE can decide on which RACH type to use when above the threshold.

    - FFS: Which SSB-based RSRP is used.

  - This does not preclude any further criteria being defined by RAN1 and RAN2, including leaving the RACH type selection to UE implementation.

- It is up to RAN2 to decide whether a single RACH type is selected or both RACH types can be selected.

Agreements:

For shared ROs with 4-step RACH and 2-step RACH configured with separate preambles:

- 2-step RACH preambles are allocated from the non-CBRA preambles associated with each SSB.

Agreements:

For shared ROs with 4-step RACH and 2-step RACH configured with separate preambles:

- All 4-step RACH ROs can be shared with 2-step RACH.

- FFS: Whether only a subset of 4-step RACH ROs can be shared with 2-step RACH

  - FFS: How to indicate the shared ROs.

Agreements:

- 2-step RACH at least reuses the 4-step RACH configuration tables (Table 6.3.3.2-2/3/4 of TS 38.211).

  - FFS: Whether in case of 4-step RACH and 2-step RACH with separately configured ROs, additional PRACH configurations for 2-step RACH are needed.

- In case of 4-step RACH and 2-step RACH with separately configured ROs, the network can configure a separate prach-ConfigurationIndex for 2-step RACH

  - If the prach-ConfigurationIndex for 2-step RACH is not configured, 2-step RACH reuses the corresponding 4-step RACH parameter.

  - FFS: Whether the preamble formats of 2-step RACH and 4-step RACH are the same or different.

Agreements:

In case of 4-step RACH and 2-step RACH with separately configured ROs, for the frequency domain location of the PRACH occasions of 2-step RACH,

- Network can configure separate msg1-FDM and msg1-FrequencyStart for the 2-step RACH ROs

  - If any of these parameters is not configured for 2-step RACH, 2-step RACH reuses the corresponding 4-step RACH parameter.

Agreements:

- The rules for a UE for invalidating 2-step RACH ROs follow the same rules that are used for the invalidation of 4-step RACH ROs as described in section 8.1 of TS 38.213.

- FFS: For separately configured 2-step RACH and 4-step RACH ROs, if 2-step RACH ROs overlap with 4-step RACH ROs in time and frequency,

  - Option 1: the 2-step RACH ROs become invalid.

  - Option 2: This is not expected by UE.

  - Other options are not precluded

The following agreements were made by email approval after RAN1#98

- For RRC_INACTIVE/IDLE state, at least support up to two msgA PUSCH configurations for Rel.16

  - Using different preamble groups for the indications of different configurations in case of two configurations

  - Support of more than two configurations is not precluded, and if supported FFS the following mechanisms for the indications of different configurations

    - Alt.1: Using different preamble groups

    - Alt 2: Using different preamble groups and/or RO partitioning

    - Alt.3: Using UCI based indication

    - Alt. 4: Using different DMRS ports/sequences

- At least up to two msgA PUSCH configurations are supported for RRC_CONNECTED state for Rel.16

  - FFS details

- FFS whether the MsgA PUSCH configurations are the same among different RRC states (IDLE, INACTIVE, CONNCETED)

- FFS the rule or BS signaling the criterion for the UE’s selection of msgA PUSCH configuration

The following agreements were made in RAN1#99:

Agreements:

- For RRC_CONNECTED UEs in non-initial UL BWP, PUSCH mapping type and SLIV of the first PUSCH occasion can be explicitly configured.

  - Value ranges for startSymbolAndLengthMsgAPO is INTEGER (0,127);

  - If startSymbolAndLengthMsgAPO is not configured, and if the TDRA list PUSCH-TimeDomainResourceAllocationList is provided in PUSCH-ConfigCommon, use msgA-timeDomainAllocation (INTEGER (1..16 maxNrofUL-Allocations)) to indicate which values are used in the list

  - otherwise, use the default table 6.1.2.1.1-2 in 38.214, msgA-timeDomainAllocation (INTEGER (1,16)) indicates which values are used in the default table

    - the parameter K2 in the default table is not used for msgA PUSCH

- For RRC_IDLE/INACTIVE state and RRC_CONNECTED UEs in initial UL BWP,

  - If the TDRA list PUSCH-TimeDomainResourceAllocationList is provided in PUSCH-ConfigCommon, use msgA-timeDomainAllocation (INTEGER (1.. 16 maxNrofUL-Allocations)) to indicate which values are used in the list

  - otherwise, use the default table 6.1.2.1.1-2 in 38.214, msgA-timeDomainAllocation indicates which values are used in the default table.

    - the parameter K2 in the default table is not used for msgA PUSCH

- Slot offset between MsgA PRACH and PUSCH is always indicated by msgAPUSCH-timeDomainOffset.

  - Value range for msgAPUSCH-timeDomainOffset to be downselected from INTEGER ([1.. 32]) or ([0.. 31])

Agreements:

- For RRC_CONNECTED state

  - Confirm the working assumption that the preamble group based method as defined for RRC_IDLE/INACTIVE state is reused for the indication of multiple configurations.

  - The number of msgA PUSCH configuration(s) can be different from that in RRC RRC_IDLE/INACTIVE state.

  - PRACH configuration(s) and msgA PUSCH configuration(s) are both cell specific and configured per BWP

  - The number of preamble groups for 2-step RACH should be aligned with the number of msgA PUSCH configurations in a BWP

- If the active UL BWP and the initial UL BWP have same SCS and same CP length and the active UL BWP includes all RBs of the initial UL BWP, or the active UL BWP is the initial UL BWP,

  - The preamble grouping and msgA PUSCH configurations are left to gNB implementation

    - Note: Preamble grouping is configured per UL BWP

    - Note: for this overlapped UL BWP it could be up to 4 PUSCH configurations from gNB perspective

    - Note: for this overlapped UL BWP it could be up to 2 PUSCH configurations from UE perspective

Agreements:

- For shared ROs, the starting position of 2-step CBRA preambles for a SSB is determined from the end of the 4-step CBRA preambles for that SSB.

Agreements:

- The value range of MCS index is

  - 0~15

  - The modulation order for MCS entries 0~1 is QPSK

  - No restriction on the modulation order for other MCS entries, i.e., the modulation order is derived from the MCS table

Agreements:

- The value range of nrofPRBsperMsgAPO is [1-32]

- The MCS table for 2-step is given by ‘qam64’ (Table 6.1.4.1-1 for DFT-s-OFDM and Table 5.1.3.1-1 for CP-OFDM in 38.214) for both RRC_IDLE/INACTIVE state and RRC_CONNECTED state

Agreements:

- Support a configurable guard period between the hops, if intra-slot frequency hopping per PO for msgA is configured.

  - Reuse the value of guard period between POs if configured; otherwise, no guard period

Agreements:

- For separately configured 2-step RACH and 4-step RACH ROs, if 2-step RACH ROs overlap with 4-step RACH ROs in time and frequency,

  - No standardized solution, which implies that in the case of overlapping, gNB may have to prepare response assuming both 2-step and 4-step

Agreements:

- Preamble to PRU mapping ratio is:

  - A single value per configuration, which is implicitly derived by the total numbers of valid preambles and valid PRUs in the SSB-to-RO association pattern period

    - It is up to gNB implementation to make sure the value does not vary across different periods.

  - M preambles are mapped to one PRU, M=ceiling(N_pre/N_pru)

    - where N_pre and N_pru are respectively the total numbers of valid preambles and valid PRUs in the SSB-to-RO association pattern period

Agreements:

Update the previous agreement as the following:

- Each M (M>=1) consecutive PRACH preambles are mapped to valid PUSCH resource units (PRUs) within an msgA association period is updated in the following order:

  - First, in increasing order of frequency resource indexes for frequency multiplexed PUSCH occasions

  - Second, in increasing order of DMRS indexes within a single PUSCH occasion

    - FFS DMRS indexes for DMRS ports and/or sequences

    - DMRS indexes within a single PUSCH occasion are in increasing order of DMRS port first and DMRS sequences second.

  - Third, in increasing order of time resource indexes for time multiplexed PUSCH occasions within a PUSCH slot

  - Fourth, in increasing order of indexes for PUSCH slots

Agreements:

- The preambles in a preamble group in each PRACH slot are mapped to the PRUs that are determined by the signaled single offset related to the PRACH slot per MsgA PUSCH configuration

Agreements:

- The PRUs without associated preambles are not used for msgA transmission for 2-step RACH.

- (Working Assumption) The preambles without associated PRUs can be used for msgA transmission (preamble only) for 2-step RACH

Agreements:

- It is up to gNB implementation to guarantee the PUSCH occasion including guard period does not span across the slot boundaries

Agreements:

- The timing for MsgA PRACH and PUSCH transmission should be both assumed as N_TA =0.

Agreements:

- For the msgA-ssb-sharedROmaskindex, reuse all the mask index values defined in Table 7.4-1, 38.321.

Agreements:

- msgA-DMRS-Configuration is a list of parameters including at least:

  - FFS msgA-dmrs-Type, ENUMERATED{type2}, indicates the DMRS type for msgA PUSCH. If the field is absent, the UE uses DMRS type1

  - msgA-dmrs-AdditionalPosition, ENUMERATED{pos0, pos1, pos3}, indicates the position for additional DM-RS. If the filed is absent, the UE applies value ‘pos2’.

  - msgA-maxLength, ENUMERATED{len2}, indicates single-symbol or double-symbol DMRS. If the filed is absent, the UE applies value ‘len1’.

  - FFS msgA-nrOfantennaPorts, ENUMERATED{[1,2,3,4,6,8]}, number of DMRS ports configured for the PRUs in an msgA PUSCH occasion. If the field is absent, all the antenna ports can be used.

Agreements

The following RRC parameters are defined if interlaced PUSCH is configured

- interlace index of the first PO in frequency domain, interlaceIndexFirstPOMsgAPUSCH

- number of consecutive interlaces per PO, nrofInterlacesPerMsgAPO

- FFS: interlace level guard band

Agreements:

- For CP-OFDM,

  - Reuse the same formula in Rel.15 for the DMRS sequence generation,

  - value range of the number of DMRS sequences is

    - [1,2]

      - nSCIDnSCID is {0,1}, and

      - NID0,NID1∈0,1,…,65535NID0, NID1 {0, 1, …, 65535} are given by the higher layer configurations if configured

        - Else N_ID^{n_SCID}=N_ID^{cell}

Agreements:

For DFT-s-OFDM

  - Not support multiple DMRS sequences

Agreements:

- The following parameter(s) are included in msgA-DMRS-Configuration

  - 1-bit indication of index(-es) of CDM group(s): msgAPUSCHDMRSCDMgroup  INTEGER (0,1); if not configured then both CDM groups are used; and

  - 1-bit indication of port number msgAPUSCHNrOfPort  INTEGER (0,1); 0 indicates 1 port per CDM group, 1 indicates 2 ports per CDM group, if not configured then 4 ports per CDM group are used;

  - Regarding the parameter msgA-maxLength

    - Keep the agreement for msgA-maxLength, if UE does not support ‘len2’, and if msgA-maxLength is configured as ‘len2’, the UE cannot use 2-step RACH resources.

      - Note: it is UE capability issue, no spec impact

  - The ordering the DMRS ports follow that in Rel.15

  - DMRS scrambling ID(s)

- DMRS Type for msgA PUSCH is type 1.

Agreements:

- In case of single PUSCH configuration or if the DMRS symbols of multiple configurations are not overlapped, if the DMRS resources configured in one PUSCH occasion is no larger than 8 (for len2) or 4 (for len1), then only DMRS port is configured.

  - Note: this is based on the previous common understanding when we agreed to support multiple sequences

- In case of multiple PUSCH configurations with overlapped DMRS symbols, network can configure different CDM groups for each msgA PUSCH configure.

  - Note: this is based on the previous agreement “Each msgA PUSCH configurations can identify sub-sets of DMRS port/sequence combination”

Agreements:

RRC parameters for msgA PRACH:

| Parameter Name (suggested) | Values |
| --- | --- |
| msgA-prach-ConfigurationIndex | INTEGER (0..[255]) |
| msgA-RO-FDM | ENUMERATED {one, two, four, eight} |
| msgApreamble-powerRampingStep | ENUMERATED {dB0, dB2, dB4, dB6} |
| msgA-CB-PreamblesPerSSB | Same value range as CB-PreamblesPerSSB |
| msgA-ssb-sharedROmaskindex | INTEGER (0…15) |

Agreements:

RRC parameters for msgA PUSCH:

| Parameter Name (suggested) | Values |
| --- | --- |
| guardPeriodMsgAPUSCH | INTEGER {0,1,2,3}, 2bits In the unit of symbols |
| nrofMsgAPOperSlot | INTEGER {1,2,3,6}, 2bits |
| nrofSlotsMsgAPUSCH | INTEGER {1,2,3,4}, 2bits |
| nrMsgAPO-FDM | INTEGER {1,2,4,8}, 2bits |
| msgA-Alpha | Same range as msg3 ENUMERATED {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08, alpha09, alpha1}, 3bits |
| msgAPUSCH-timeDomainOffset | INTEGER (1.. 32), 5bits |
| nrofInterlacesPerMsgAPO | INTEGER {1...10} for 15kHz, INTEGER {1...5} for 30kHz, 4bits |
| interlaceIndexFirstPOMsgAPUSCH | INTEGER {1...10} for 15kHz, INTEGER {1...5} for 30kHz, 4bits |

Agreements

msgA-prach-ConfigurationIndexNew: INTEGER (256.. 262).

Agreements:

- The minimum transmission gap between the end of msgA PRACH and the beginning of msgA PUSCH (guard time excluded) is no less than Ngap symbols, as specified in TS 38.213, i.e., 2 or 4 symbols depending on the SCS

  - This is not applied for NR-U

  - Note: This is aligned with Rel-15

Working assumption:

- The granularity of the TA command in msgB is determined by the same table for the granularity of the TA command in 4step RACH RAR with the subcarrier spacing (kHz) value is based on the UL BWP SCS at least for the case when MsgA PRACH and MsgA PUSCH are in different slots.

Agreements:

- For CA/DC,

  - Note: MsgA is not supported in SCell.

  - The power reduction priority of MsgA PUSCH is the same as PUSCH transmission in Rel-15

Agreements:

- The power ramping counter is the same for MsgA PRACH and MsgA PUSCH.

  - Layer 1 will notify the higher layer to suspend the power ramping counter if msgA PRACH is not transmitted (if applicable)

  - Layer 1 may notify the higher layer to suspend the power ramping counter if msgA PRACH tx power is reduced (if applicable)

Agreements:

- RV=0 for msgA PUSCH initial transmission and msgA reattempt

- RV=0 for the msg3 transmission in case of fallback

Agreements:

For the PUCCH Resource index used for the HARQ-ACK feedback of a user that finds its contention resolution ID in the successRAR with a PDSCH scheduled by a PDCCH that has a CRC scrambled with the MsgB-RNTI:

- PUCCH Resource Index is only signalled explicitly in the successRAR

  - Number of bits used to indicate PUCCH resource index is 4 bits.

Agreements:

- PDSCH-to-HARQ_feedback timing indicator is only signalled in the successRAR

  - Number of bits used to indicate the PDSCH-to-HARQ_feedback timing indicator is 3 bits.

  - FFS the timing for UE to transmit HARQ-ACK information should be in the slot of  n+k+delta, where delta is defined by Table 6.1.2.1.1-5 in 38.214, where n is the slot index of receiving MsgB, and k is given by PDSCH-to-HARQ-timing-indicator with value range of {1, 2, 3, 4, 5, 6, 7, 8}.

Agreements:

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by msgB-RNTI (parameters in squared brackets depend on the other proposals):

-	Frequency domain resource assignment – bits

-	Time domain resource assignment – 4 bits

-	VRB-to-PRB mapping – 1 bit

-	Modulation and coding scheme – 5 bits

-	TB scaling – 2 bits

-	[LSBs of SFN – 2 bits]

-	Reserved bits – [14] bits

Note: The interpretation of the “LSBs of SFN” may be different for the hand over case

Agreements

- Msg1 pPower ramping counter should inherit continue increasing (if not suspended) msgA preamble power ramping counter after 2-step RACH switches to 4-step RACH.

- The power ramping step size follows the 4-step RACH configuration after 2-step RACH switches to 4-step RACH

- After 2-step RACH switches to 4-step RACH, the target power level follow the 4-step RACH formula plus an offset, where the offset is

  - OFFSET = (N -1) × (PREAMBLE_POWER_RAMPING_STEP_2_step  -  PREAMBLE_POWER_RAMPING_STEP_4_step)

    - N is the power ramping counter when switching back to 4-step

Agreements:

- The UE transmits the PUCCH responding to msgB successRAR using the same spatial domain transmission filter in the same BWP as the last msgA PUSCH transmission/retransmission.

Agreements:

- The minimum gap between the last symbol of msgB PDSCH and the first symbol of PUCCH transmission carrying HARQ-ACK to msgB PDSCH for a UE is assumed to be no less than the UE PDSCH processing procedure time given by NT,1+0.5+t ms, where NT,1 NT,1is the PDSCH processing time for UE processing capability 1 in TS 38.214-5.3.

  - The indicated timing for UE to transmit HARQ-ACK information should be in the slot determined by ceiling(n+k+delta+t/slot duration), where n is the slot index of receiving MsgB PDSCH, and k is given by PDSCH-to-HARQ-timing-indicator with value range of {1, 2, 3, 4, 5, 6, 7, 8}.

    - [working assumption] where delta is defined by Table 6.1.2.1.1-5 in 38.214

  - t >=0, to address in the CR stage the value of t

    - If t =0, the consequence needs to be discussed

#### 2.1.2	Remaining Open issues

None

## 2.2	RAN2

#### 2.2.1	Agreements

The following agreements were made in RAN2#105bis:

Agreements:

- Criteria on whether the UE uses 2-step RACH or 4-step RACH shall be clearly specified

- The start of the msgB reception window is after the PUSCH transmission opportunity of msgA.  Details are FFS for 2-step RACH and fallback.

- If CCCH SDU was included in MsgA, then the contention resolution will be based on the contention resolution ID included in MsgB.  FFS for other conditions.

Approved LS to RAN1

1. RAN2 has discussed the contents and content size of MsgA data part and concluded that the following could be included into MsgA depending on the Random Access trigger:

- RRC message for requesting RRC connection setup/re-establishment/resume for IDLE/INACTIVE/CONNECTED UEs;

- RRC message for requesting on-demand system information for IDLE/INACTIVE UEs;

- C-RNTI MAC CE + possibly other MAC CE(s) like BSR + UL data (CP or UP) for CONNECTED mode UEs.

Based on the above, it was concluded that minimum payload sizes the MsgA shall be able to support are 56 and 72 bits depending on whether short or long I-RNTI is requested by the NW from RRC INACTIVE UEs inside the RRCResumeRequest message, respectively.

On the other hand, no upper bound for the content size is defined by RAN2 but any limit based on the RAN1 design would be good to be known to be able to design the signaling options properly.

2. In addition, RAN2 would appreciate any early information for the power ramping for MsgA transmission.

ACTION: 	RAN2 respectfully asks RAN1 to take the minimum MsgA content sizes into account for their design as well as provide any information about the upper bound of the MsgA content size if/when available. Furthermore, RAN2 would respect any early information for the power ramping design for MsgA transmission.

The following agreements were made in RAN2#106:

Agreements:

- From RAN2 perspective, 2-step RACH selections can be based on indicating to all UEs via SIB, or dedicated configuration in RRC_CONNECTED/INACTIVE/IDLE states.  FFS if radio quality is used for 2-step RACH selection.

- From RAN2 perspective, for msgA retransmission (i.e. preamble and PUSCH) we assume that the UE retries on 2-step RACH

- FFS whether the UE can fallback to 4-step RACH after certain time.  Ask RAN1 whether the preamble transmission performance for 2-step RACH and 4-step RACH is the same.

- For MsgA with C-RNTI, the UE shall monitor the PDCCH addressed to C-RNTI for success response and msgB-RNTI (e.g. RA-RNTI or new RNTI)

- Contention resolution:

  - If the PDU PDCCH addressed to the C-RNTI (i.e. C-RNTI included in MsgA) containing the 12 bit TA command is received, the UE should consider the contention resolution to be successful and stop the reception of MsgB or with UL grant if the UE is synchronized already.

  - If the corresponding fallback RAR is detected, the UE should stop the monitoring of PDCCH addressed to the corresponding C-RNTI for success response and process the fallback operation accordingly.

  - If neither corresponding fallback RAR nor PDCCH addressed C-RNTI is detected within the response window, the UE should consider the msgA attempt failed and do back off operation based on the backoff indicator if received in MsgB.

  - FFS if a new MAC CE with 12bits Timing Advanced Command shall be introduced

- For CCCH, MsgB can include the SRB RRC message.  The format should be designed for both with and without RRC message.

- For CCCH, for success or fallback RAR MsgB can multiplex messages for multiple UEs.  FFS if we can multiplex SRB RRC messages of multiple UEs.

- Network response to msgA (i.e. msgB/msg2) can include the following:

  - SuccessRAR

  - FallbackRAR

  - Backoff Indication

FFS: format of successRAR and whether successRAR is split into more than one message and format of fallbackRAR and whether legacy msg2 can be reused for fallbackRAR

- Proposal 10: The following fields can be included in the successRAR when CCCH message is included in msgA.

  - Contention resolution ID

  - C-RNTI

  - TA command

- Upon receiving the fallbackRAR, the UE shall proceed to msg3 step of 4-step RACH procedure

- FallbackRAR should contain the following fields

  - RAPID

  - UL grant (to retransmit the msgA payload).  FFS on restrictions on the grant and UE behavior if different grant and rebuilding

  - TC-RNTI

  - TA command

- From RAN2 perspective, no further offset is needed for the start of msgB monitoring window (i.e. no offset is needed to cover the RRC processing delay and/or F1 delay).

- The UE will monitor for response message using the single msgB agreed window

- MsgB containing the succcessRAR shall not be multiplexed with the legacy 4-step RACH RAR in the same MAC PDU

The following agreements were made in RAN2#107:

- Working assumption: SRB RRC messages of multiple UEs cannot be multiplexed in same msg B (i.e. same MAC PDU).

- SuccessRAR cannot be split into more than one message (i.e.Contention resolution ID will also be included in successRAR).

- SuccessRAR and fallbackRAR can be multiplexed

- HARQ feedback for msgB would be needed from RAN2 point of view

- The fallback RAR shall be included in the general MsgB format, i.e., be able to be multiplexed with the successRAR for 2-step RACH.

- TB size offered in UL grant in the fallback RAR shall be the same as the TB size offered for payload transmission in MsgA; otherwise, the UE behavior is not defined (i.e. it is up to UE implementation).

- RA type selection is performed before beam selection

- No need to reexecute RA selection criteria upon fallback failure (i.e if reception of msg3 fails).  The UE re-transmits using msgA

- If the random access procedure is not successfully completed even after transmitting the msgA 'N' times, UE fallbacks to 4 step RACH procedure i.e. UE only transmits the PRACH preamble.

- Network can configure the number of times 'N', a UE can attempt to re-transmit msgA during the random access procedure.

- For MsgA with C-RNTI or CCCH SDU, upon receiving fallbackRAR corresponding to random access preamble transmitted by UE, UE may stop monitoring PDCCH addressed to msgB-RNTI.

- For MsgA with CCCH SDU, if neither fallbackRAR or successRAR is received within the response window, the UE should consider the msgA attempt failed and do back off operation based on the backoff indicator if received in MsgB

- RA type selection is NOT left up to UE implementation.

- If the UE is configured with 2-step RA, the RSRP is above a configurable threshold then the UE shall use the 2-step RA procedure.

- 2-step RA type selection is done after carrier type selection (UL/SUL).  FFS if we have separate threshold for different carriers (UL/SUL)

- we will not work on this (overload control for 2-step RACH) for now

- From RAN2 point of view, there is benefits to support 2-step CFRA for HO (dedicated preamble and dedicated PUSCH).

- 2-step CFRA (dedicated preamble and dedicated PUSCH) can be an alternative RACH-less HO.  It is up to the plenary to decide how to handle this and whether we chose to do anything at all.

Approved LS to RAN1

RAN2 has discussed the overall 2-step RACH procedure for the success case and fallback case. The following agreements have been made by RAN2.

Agreements:

- Working assumption: SRB RRC messages of multiple UEs cannot be multiplexed in same msg B (i.e. same MAC PDU).

- successRAR cannot be split into more than one message (i.e.Contention resolution ID will also be included in successRAR).

- SuccessRAR and fallbackRAR can be multiplexed

and

=>	HARQ feedback for msgB would be needed from RAN2 point of view

and

2.   TB size offered in UL grant in the fallback RAR shall be the same as the TB size offered for payload transmission in MsgA; otherwise, the UE behavior is not defined (i.e. it is up to UE implementation).

Also, according to the previous agreements in RAN2, the current successRAR has no UL grant in the message. RAN2 design allows multiplexing successRARs of multiple UEs in msgB. HARQ feedback for the msgB is needed for this multiplexed case to confirm contention resolution for the success UEs.

RAN2 kindly requests RAN1 to take the above agreements into account and provide feedback on any RAN1 mechanisms for sending HARQ feedback in UL for msgB in case of multiple UEs are multiplexed.

ACTIONS: RAN2 kindly requests RAN1 to take the above agreements into account and provide feedback on any RAN1 mechanisms for sending HARQ feedback in UL for msgB in case successRARs of multiple UEs are multiplexed in the msgB.

The following agreements were made in RAN2#107bis:

- The RSRP threshold for 2-step vs 4step CBRA can be configured separately for NUL and SUL.

- Use MSGA buffer to store the MSGA payload in case of 2-step RACH, unless the implementation in the MAC spec it becomes too complex.

- UE monitors the MSGB window regardless of the occurrence of the measurement gap

- The name of the new timing advance command MAC CE is: Absolute Timing Advance Command MAC CE

- Introduce preambles group A and B for 2-step RACH.

- Apply the same selection formulas to select between 2-step preambles group A and B as specified for 4-step in Rel-15. For the purpose of data threshold, ra-MsgASizeGroupA parameter can be introduced.

- Support configuration where fallback from 2-step RA to 4-step RA is not allowed

- TB size offered in UL grant in the Msg2 RAR in 4-step RACH shall be the same as the TB size offered for payload transmission in MsgA in 2-step RACH; otherwise, the UE behavior is not defined (i.e. it is up to UE implementation).  Rebuilding is not supported in the specification (i.e. it is up to UE implementation).

- If switching to 4-step RACH is expected to be supported, then support network configuration where the same TB sizes offered for 2-step RACH preamble groups are the same with those of 4-step RACH preamble groups.

- No UE specific RNTI will be designed for 2-step RACH in case CCCH SDU was included in MsgA.

- Confirm the Working Assumption: SRB RRC messages of multiple UEs cannot be multiplexed in same MsgB (i.e. same MAC PDU).

- RAN2 will work on specifying a new RA-RNTI design for msgB

- For 2 step CBRA, RA prioritisation is supported at least for handover and beam failure recovery.

- RA prioritisation for 2 step CBRA is controlled by networkRA prioritisation parameters are separately configured for 2 step CBRA and 4 step CBRA. -ra-Prioritization2Step is optionally added to BeamFailureRecoveryConfig IE and RACH-ConfigDedicated IE.

- 2-step RACH resources can only be configured on SpCell

- The 2-step RACH resources can be configured on a BWP where 4-step CBRA resources are not configured.  In that case we will not have 4-step switch.

- The PDCCH triggered 2-step CFRA RACH will not be supported in Rel-16

- The 2-step CBRA for SpCell BFR is supported in Rel-16.

- Legacy UEs are not required to decode msgB

- The 12-bit TA command, 16-bit C-RNTI and 48-bit UE Contention Resolution Identity are included in successRAR MAC subPDU.

- The UE Contention Resolution Identity field should be placed in the payload of the successRAR MAC subPDU. As a baseline, UE Contention Resolution Identity field is in front of the payload of MAC subPDU.

- We will discuss whether we include SFN after NR-U design is completed.

- As a baseline, the indication for presence of RRC message following the successRAR MAC subPDU is needed. The detailed indication method is FFS.

- For the case of msgB with RRC message, the SRB RRC message is included in a separate MAC subPDU, not inside the successRAR subPDU.

- More than one RRC message for a given UE can be included in msgB (i.e. for re-establishment case).  FFS whether we need to indicate number of RRC messages

- The fallbackRAR MAC subPDU is composed of 12-bit TA command, 16-bit TC-RNTI, 27-bit UL grant, and 6-bit RAPID. The RAPID is in the MAC subheader of fallbackRAR subPDU.

- For NR 2-step RACH, the payload of fallbackRAR MAC subPDU should reuse msg2 RAR format.

- The sub-header should differentiate the different types of MAC subPDU (e.g. fallback, success, backoff).  FFS how to indicate.

- The MAC subheader for SRB MAC subPDU should use a full LCID field (6-bit)

- The MAC subheader for SRB MAC subPDU should reuse the Rel-15 MAC subheader which supports both 8-bit and 16-bit Length field with additional F indicator field

- As a baseline, the RAPID field is not included in the MAC subheader for successRAR MAC subPDU.

The following agreements were made in RAN2#108:

- Include a 1 bit field in MAC subheader of SuccessRAR MAC subPDU to indicate presence/absence of SRB MAC subPDU(s) following successRAR MAC subPDU.

- If the MAC subheader of SuccessRAR MAC subPDU indicates presence of SRB MAC subPDU(s):

- SRB MAC subPDU(s) are present immediately after the SuccessRAR MAC subPDU;

- the last SRB MAC subPDU is followed by padding MAC subPDU, if padding is present;

- padding MAC subPDU includes R/R/LCID padding subheader (as used to indicate padding in DL SCH MAC PDU).

- 1 bit E field is included in MAC subheader of SuccessRAR MAC subPDU, Backoff MAC subPDU and FallbackRAR MAC subPDU.

- The E field is set to "1" to indicate at least another MAC subPDU (other than SRB MAC subPDU) follows. E field is set to "0" to indicate that the MAC subPDU including this MAC subheader is the last MAC subPDU (other than SRB MAC subPDU) in the MAC PDU.

- 1 bit T1 and 1 bit T2 field in MAC subheader are set as follows:

- MAC subheader in Backoff MAC subPDU : T1 = 0, T2 = 0

- MAC subheader in SuccessRAR MAC subPDU : T1 = 0, T2 = 1

- MAC subheader in FallbackRAR MAC subPDU: T1 = 1, T2 is not included.

- Format in figure 10 of R2-1914430 as a baseline. The format can be further updated based on final outcome of discussions.

- The RA response window is extended up to 40ms for 2-step RACH.

- From RAN2 perspective, 2bit LSB of the SFN is included in the DCI scheduling msgB

- A “fixed” offset is added to the RA-RNTI formula to extend the RA-RNTI space.  The Rapporteur of MAC will included it in the MAC CR.

- When 2-step RACH was triggered by UL data arrival while UL is non-sync grant is not included in random access message (i.e. no change to current MAC running CR).

- When 2-step RACH was triggered beam failure recovery or PDCCH order when uplink is in-sync, only C-RNTI addressed PDCCH on SPCell is needed for contention resolution

- If 4-step CFRA resource is configured, the UE should select 4-step RACH in “5.1.1 Random Access procedure initialization”.

- If no qualified beam with 4-step CFRA resource can be found, the UE should fallback to 4-step CBRA.  Assumption: 2-step CFRA and 4-step CFRA are not configured at the same time

- For random access initiated by PDCCH order, if PDCCH order includes non-zero RA preamble index, UE selects 4 step RA i.e. UE will perform legacy CFRA

- RA prioritization for MCS and MPS should be applicable for both 2-step and 4-step RACH procedures.

- RA prioritization parameters for MCS and MPS for both 2-step and 4-step RACH procedures are obtained from the SIB1 ra-PrioritizationForAccessIdentity field as defined in TS 38.321 Subclause 5.1.1, as agreed in main session for 4-step RA.

- Same mechanism as 4-step RA will be applied for 2-step RA

- Allow configurations where switching to 4-step RA is not supported even when 2-step RA and 4-step RA are configured in the BWP.

- HARQ process ID 0 is used for MsgA PUSCH transmission

- 2-step CFRA and 4-step CFRA cannot be configured simultaneously for a BWP

- For HO 2-step CFRA, the UE will monitor the PDCCH of the target cell for the response from the gNB identified by the C-RNTI while the msgB-ResponseWindow is running. The RA procedure is considered successful upon reception of a transmission addressed to the C-RNTI containing at least the 12 bit TAC MAC CE.

- Rebuilding is NOT supported: This means the CFRA payload size matches one of the payload sizes for CBRA and UE includes C-RNTI in MSGA for CFRA

- In case of 2-step CFRA, once MSGA is transmitted the UE monitors MSGB-RNTI (in addition to C-RNTI – i.e. same as CBRA)

- The initial RA type is always determined to be 2-step RA if 2-step CFRA is configured in HO

- Similar to 4-step RA, the UE then searches for a suitable CFRA beam with configured 2-step CFRA resources

- RAN2 assumes that SSB and CSI-RS based 2-step CFRA can be supported.  We assume that if there are RAN1 impact then CSI-RS configuration will not be supported.

- The PUSCH resource for 2-step CFRA associated with the dedicated preamble will be configured to the UE via dedicated signalling (i.e. will not be included in SIB1).  FFS how and when the PUSCH resources is releases

- 2-step CFRA is configured only on BWP where 2-step CBRA is configured

In addition to the above, the following agreements were also made as part of the discussion under NR-U work item at R2#108:

- From MAC perspective, if LBT fails for the preamble, the UE also cancel PUSCH transmission

- The PREAMBLE_TRANSMISSION_COUNTER is not increased if the preamble is not transmitted due to failure of the LBT for the preamble

- If preamble is transmitted but LBT for msgA PUSCH fails, the UE monitors downlink PDCCH for fallback RAR. FFS how and whether to deal with the C-RNTI case for connected mode

- The 2 LSBs for the SFN corresponding to msgA transmission time is included in msgB DCI, as for licenced case (pending RAN1)

#### 2.2.2	Remaining Open issues

Finalization of the running CRs for the following RAN2 specs:

- 38.300 (stage-2) – Nokia

- 38.321 (MAC) – ZTE

- 38.331 (RRC) – Ericsson

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- 3GPP, RAN1, RAN1#96 Chairman Notes

- R1-1901625 	Work plan for 2-step RACH 	ZTE

- R1-1902027 	Discussion on Channel Structure for 2-Step RACH	CATT

- R1-1902821 	Use Cases and Scenarios for 2-Step RACH, Ericsson

- R1-1903147 	Considerations on two-step RACH for NTN	THALES

- R1-1901626	Considerations on the channel structure of msgA	ZTE

- R1-1902466	Channel structure for two-step RACH	Intel Corporation

- R1-1902977	Channel Structure for Two-Step RACH	Qualcomm Incorporated

- R1-1901669	Discussion on channel structure for 2-step RACH	vivo

- R1-1901793	On channel structure for two-step RACH	MediaTek Inc.

- R1-1902133	Channel Structure for Two-Step RACH Considerations 	Sierra Wireless, S.A.

- R1-1902135	On 2-step RACH Channel Structure	Nokia, Nokia Shanghai Bell

- R1-1902165	On Channel Structure for Two-Step RACH	Sony

- R1-1902241	Channel Structure for Two-Step RACH	Samsung

- R1-1902326	Discussion on channel structure for two-step RACH	CMCC

- R1-1902393	Discussion on channel structure for 2-step RACH	Panasonic Corporation

- R1-1902533	Discussion on 2step RACH Structure	LG Electronics

- R1-1902582	On Signal Structure for Two-Step RACH 	InterDigital, Inc.

- R1-1902747	On Channel Structure for 2-step RACH	OPPO

- R1-1902784	Discussion on Channel Structure for Two-Step RACH	NTT DOCOMO, INC.

- R1-1902822	Channel Structure for Two-step RACH	Ericsson

- R1-1902917	Considerations on Resource Pool Design for PUSCH in MsgA of 2-step RACH	CAICT

- R1-1902961	Discussion on channel structure for two-step RACH	KDDI Corporation

- R1-1903056	Channel structure for 2-step RACH	Huawei, HiSilicon

- R1-1902136	On 2-step RACH Procedure	Nokia, Nokia Shanghai Bell

- R1-1902785	Discussion on Procedure for Two-step RACH	NTT DOCOMO, INC.

- R1-1903057	Discussion on 2-step RACH procedure	Huawei, HiSilicon

- R1-1901627	Considerations on 2-step RACH procedures	ZTE

- R1-1901670	Discussion on 2-step RACH procedure	vivo

- R1-1901794	On procedure for two-step RACH	MediaTek Inc.

- R1-1902134	Procedure for Two-Step RACH Considerations 	Sierra Wireless, S.A.

- R1-1902166	Discussion on Procedure for 2-step RACH	Sony

- R1-1902242	Procedure for Two-step RACH	Samsung

- R1-1902327	Discussion on procedure for two-step RACH	CMCC

- R1-1902394	Discussion on 2-step RACH procedure	Panasonic Corporation

- R1-1902467	Procedure  for two-step RACH	Intel Corporation

- R1-1902534	Discussion on 2step RACH Procedure	LG Electronics

- R1-1902583	Perspectives on operation of Two-Step RACH 	InterDigital, Inc.

- R1-1902748	On Procedure for 2-step RACH	OPPO

- R1-1902823	Procedure for Two-step RACH	Ericsson

- R1-1902843	2-step RACH procedure	Motorola Mobility, Lenovo

- R1-1902918	A scheme of fallback from 2-step RACH to 4-step RACH	CAICT

- R1-1902964	Discussion on procedure for two-step RACH	KDDI Corporation

- R1-1903321	Procedures for Two-step RACH	Qualcomm Incorporated

- R1-1903186	Discussion on 2-step RACH for NR	Fraunhofer HHI, Fraunhofer IIS

- R1-1903878	On the channel structure of msgA	ZTE, Sanechips

- R1-1903923	Further discussion on channel structure for 2-step RACH	Huawei, HiSilicon

- R1-1904392	Channel Structure for Two-Step RACH	Samsung

- R1-1904059	Discussion on channel structure for 2-step RACH	vivo

- R1-1904196	Channel Structure for 2-Step RACH	LG Electronics

- R1-1904246	Considerations on Channel Structure for Two-Step RACH	Sony

- R1-1904280	Channel structure for 2-step RACH	Intel Corporation

- R1-1904357	Discussion on channel structure for 2-step RACH	Panasonic Corporation

- R1-1904542	Discussion on channel structure for 2-step RACH	CATT

- R1-1904584	Discussion on channel structure for Two-Step RACH	Fujitsu

- R1-1904715	On 2-step RACH Channel Structure	Nokia, Nokia Shanghai Bell

- R1-1904724	Discussion on channel structure for two-step RACH	CMCC

- R1-1904778	Considerations on the channel structure on 2-step RACH	Spreadtrum Communications

- R1-1904819	Channel Structure for Two-Step RACH Considerations	Sierra Wireless, S.A.

- R1-1904842	Channel Structure for Two-step RACH	Ericsson

- R1-1904848	On PUSCH Transmission in msgA 	InterDigital, Inc.

- R1-1904944	Discussion on Channel Structure for Two-Step RACH	NTT DOCOMO, INC.

- R1-1904992	Channel Structure for Two-Step RACH	Qualcomm Incorporated

- R1-1905043	Discussion on channel structure for 2-Step RACH	ITRI, NTPU

- R1-1905051	On Channel Structure for 2-step RACH	OPPO

- R1-1905125	Considerations on Channel Structure for Two-step RACH	CAICT

- R1-1905205	Discussion on channel structure for two-step RACH	KDDI Corporation

- R1-1904716	On 2-step RACH Procedure	Nokia, Nokia Shanghai Bell

- R1-1904843	Procedure for Two-step RACH	Ericsson

- R1-1904993	Procedures for Two-step RACH	Qualcomm Incorporated

- R1-1903879	On 2-step RACH procedures	ZTE, Sanechips

- R1-1903924	Further discussion on 2-step RACH procedure	Huawei, HiSilicon

- R1-1904060	Discussion on 2-step RACH procedure	vivo

- R1-1904197	Discussion on 2-Step RACH Procedure	LG Electronics

- R1-1904247	Discussion on Procedure for 2-step RACH	Sony

- R1-1904281	Procedure for 2-step RACH	Intel Corporation

- R1-1904358	Discussion on 2-step RACH procedure	Panasonic Corporation

- R1-1904393	Procedure for Two-step RACH	Samsung

- R1-1904725	Discussion on procedure for two-step RACH	CMCC

- R1-1904779	Considerations on the random access procedure on 2-step RACH	Spreadtrum Communications

- R1-1904820	Procedure for Two-step RACH Considerations	Sierra Wireless, S.A.

- R1-1904850	Discussion on Procedures for Two-Step RACH 	InterDigital, Inc.

- R1-1904928	2-step RACH procedure	Motorola Mobility, Lenovo

- R1-1904945	Discussion on Procedure for Two-step RACH	NTT DOCOMO, INC.

- R1-1905044	Discussion on procedure for 2-Step RACH	ITRI, NTPU

- R1-1905052	On Procedure for 2-step RACH	OPPO

- R1-1905126	Considerations on Procedure for Two-step RACH	CAICT

- R1-1905207	Discussion on procedure for two-step RACH	KDDI Corporation

- R1-1903880	Analysis on the collision probability of msgA	ZTE, Sanechips

- R1-1903881	Preliminary evaluation results for 2-step RACH	ZTE, Sanechips

- R1-1904061	Evaluation for 2-step RACH	vivo

- R1-1904198	Discussion on preamble selection for 2-Step RACH	LG Electronics

- R1-1904199	Discussion on RAR detection in 2step RACH procedure	LG Electronics

- R1-1904684	Preliminary results for 2-step RACH evaluations	Huawei, HiSilicon

- R1-1904685	Other issues for 2-step RACH	Huawei, HiSilicon

- R1-1904717	2-step RACH latency and resource reservation	Nokia, Nokia Shanghai Bell

- R1-1904844	Evaluation methodology and metrics for Two-Step RACH	Ericsson

- R1-1905515	Initial results on coverage and msgA configurations	Ericsson

- R1-1904994	Other Considerations for Two-Step RACH	Qualcomm Incorporated

- R1-1907179	Use Cases and Scenarios for 2-Step RACH	Ericsson

- R1-1905989	Further discussions on the channel structure of msgA	ZTE

- R1-1907752 	Channel Structure for Two-step RACH	Ericsson

- R1-1907691	Channel Structure for Two-Step RACH	Qualcomm Incorporated

- R1-1907673	Feature lead summary for 2-step RACH channel structure	ZTE

- R1-1906050	Discussion on channel structure of 2-step RACH	Huawei, HiSilicon

- R1-1906124	Discussion on channel structure for 2-step RACH	vivo

- R1-1906192	Discussion on Channel Structure for Two-Step RACH	NTT DOCOMO, INC.

- R1-1906366	Considerations on the channel structure on 2-step RACH	Spreadtrum Communications

- R1-1906429	Discussion on channel structure for Two-Step RACH	Fujitsu

- R1-1906511	Discussion on Channel Structure for Two-step RACH	CMCC

- R1-1906580	On Channel Structure for 2-step RACH	OPPO

- R1-1906717	Discussion on Channel Structure for 2-step RACH	LG Electronics

- R1-1906746	On 2-step RACH Channel Structure	Nokia, Nokia Shanghai Bell

- R1-1906779	Discussion on channel structure for 2-step RACH	Intel Corporation

- R1-1906848	Considerations on Channel Structure for Two-Step RACH	Sony

- R1-1906858	On preamble and PUSCH mapping	InterDigital, Inc.

- R1-1906863	Discussion on channel structure for 2-step RACH	Panasonic Corporation

- R1-1906905	Channel Structure for Two-Step RACH	Samsung

- R1-1907127	Channel Structure for Two-Step RACH Considerations	Sierra Wireless, S.A.

- R1-1907196	Considerations on Channel Structure for Two-step RACH	CAICT

- R1-1907446	Discussion on channel structure for two-step RACH	KDDI Corporation

- R1-1905990	Further discussions on 2-step RACH procedures	ZTE

- R1-1906125	Discussion on 2-step RACH procedure	vivo

- R1-1906367	Considerations on 2-step RACH procedure	Spreadtrum Communications

- R1-1906581	On Procedure for 2-step RACH	OPPO

- R1-1906582	On recognition of msgB or msg2	Fujitsu

- R1-1906718	Discussion on Procedure for 2-step RACH	LG Electronics

- R1-1906780	Discussion on procedure for 2-step RACH	Intel Corporation

- R1-1906831	Discussion on Procedure for 2-step RACH	Sony

- R1-1906859	On 2-step RACH procedure	InterDigital, Inc.

- R1-1906864	Discussion on 2-step RACH procedure	Panasonic Corporation

- R1-1906906	Procedure for Two-step RACH	Samsung

- R1-1907123	Views on 2-step RA Procedures	Charter Communications

- R1-1907128	Procedure for Two-step RACH Considerations	Sierra Wireless, S.A.

- R1-1907181	Procedure for Two-step RACH	Ericsson

- R1-1907197	Considerations on Procedure for Two-step RACH	CAICT

- R1-1907238	2-step RACH procedure	Motorola Mobility, Lenovo

- R1-1907256	Procedures for Two-step RACH	Qualcomm Incorporated

- R1-1907447	Discussion on procedure for two-step RACH	KDDI Corporation

- R1-1905991	Simulation results for 2-step RACH	ZTE

- R1-1906126	Evaluation for 2-step RACH	vivo

- R1-1906604	Further results for 2-step RACH payload size evaluations	Huawei, HiSilicon

- R1-1906605	Evaluation results for other issues for 2-step RACH	Huawei, HiSilicon

- R1-1906748	2-step RACH latency and resource reservation	Nokia, Nokia Shanghai Bell

- R1-1906749	2-step RACH evaluation results	Nokia, Nokia Shanghai Bell

- R1-1906781	Initial evaluation results for 2-step RACH	Intel Corporation

- R1-1907182	Evaluation methodology and metrics for Two-Step RACH	Ericsson

- R1-1907183	Power control of msgA PUSCH transmissions	Ericsson

- R1-1907187	Modeling UE power differences in link simulations	Ericsson

- R1-1907677	Initial results on DMRS antenna ports and scrambling	Ericsson

- R1-1907257	Other Considerations for Two-Step RACH	Qualcomm Incorporated

- R1-1907428	Discussion on RAR and  msgB PDCCH monitoring for 2-step RACH	LG Electronics

- R1-1907495	Latency analysis on 2-step and 4-step RACH	Huawei, HiSilicon

- R1-1907540	Considerations for simultaneous RACH operation	Huawei, HiSilicon

- R1-1907541	Other issues for 2-step RACH	Huawei, HiSilicon

- R1-1908181	On the reamaining isuses of msgA channel structure	ZTE, Sanechips

- R1-1909122	Channel Structure for Two-step RACH	Ericsson

- R1-1909239	Channel Structure for Two-Step RACH	Qualcomm Incorporated

- Continue discussion offline – Li (ZTE)

- R1-1909478	FL Summary of Channel Structure for 2-step RACH	ZTE

- R1-1908033	Discussion on channel structure of 2-step RACH	Huawei, HiSilicon

- R1-1908134	Discussion on channel structure for 2-step RACH	vivo

- R1-1908211	Discussion on channel structure for Two-Step RACH	Fujitsu

- R1-1908230	Discussion on Channel Structure for 2-step RACH	InterDigital, Inc.

- R1-1908341	Considerations on 2-step RACH channel structure	Nokia, Nokia Shanghai Bell

- R1-1908459	Channel Structure for Two-Step RACH	Samsung

- R1-1908618	Discussion on channel structure for 2-step RACH	Intel Corporation

- R1-1908691	Discussion on Channel Structure for 2-step RACH	LG Electronics

- R1-1908753	Channel Structure for Two-Step RACH	Sierra Wireless, S.A.

- R1-1908762	Channel Structure for Two-Step RACH	Sony

- R1-1909503	Discussion on channel structure for 2-step RACH	Panasonic Corporation

- R1-1908860	Discussion on Channel Structure for 2-step RACH	CMCC

- R1-1909040	On Channel Structure for 2 Step RACH	Apple Inc.

- R1-1909172	Discussion on Channel Structure for Two-Step RACH	NTT DOCOMO, INC.

- R1-1909222	On Channel Structure for 2-step RACH	OPPO

- R1-1909344	Considerations on Channel Structure for Two-step RACH	CAICT

- R1-1909410	Discussion on channel structure for two-step RACH	KDDI Corporation

- R1-1908034	Discussion on 2-step RACH procedure	Huawei, HiSilicon

- R1-1908342	On 2-step RACH Procedure	Nokia, Nokia Shanghai Bell

- R1-1909173	Discussion on Procedure for Two-step RACH	NTT DOCOMO, INC.

- R1-1909499	Feature lead summary#1 on 2 step RACH procedures	Nokia

- R1-1908135	Discussion on 2-step RACH procedure	vivo

- R1-1908182	On the remaining issues of 2-step RACH procedures	ZTE, Sanechips

- R1-1908212	On recognition of msgB or msg2	Fujitsu

- R1-1908231	Views on 2-step RACH Procedures	InterDigital, Inc.

- R1-1908339	Views on 2-step RA Procedures	Charter Communications

- R1-1908460	Procedure for Two-step RACH	Samsung

- R1-1908619	Discussion on procedure for 2-step RACH	Intel Corporation

- R1-1908692	Discussion on 2-Step RACH Procedure	LG Electronics

- R1-1908754	Procedure for Two-step RACH	Sierra Wireless, S.A.

- R1-1908763	Discussion on procedures for 2-Step RACH	Sony

- R1-1908793	Discussion on 2-step RACH procedure	Panasonic Corporation

- R1-1908861	Discussion on Procedure for 2-step RACH	CMCC

- R1-1908967	Discussion on 2-step RACH procedure	Spreadtrum Communications

- R1-1909041	On Procedure for 2 Step RACH	Apple Inc.

- R1-1909123	Procedure for Two-step RACH	Ericsson

- R1-1909152	2-step RACH procedure	Motorola Mobility, Lenovo

- R1-1909223	On Procedure for 2-step RACH	OPPO

- R1-1909240	Procedures for Two-step RACH	Qualcomm Incorporated

- R1-1909331	On procedure for Two-step RACH	Xiaomi Technology

- R1-1909345	Considerations on Procedure for Two-step RACH	CAICT

- R1-1909403	Discussion on two-step RACH procedures	Google Inc.

- R1-1908136	Evaluation for 2-step RACH	vivo

- R1-1908183	Evaluation results for 2-step RACH	ZTE, Sanechips

- R1-1908343	2-step RACH evaluation results	Nokia, Nokia Shanghai Bell

- R1-1908461	Discussion on PUSCH resource unit	Samsung

- R1-1908620	Evaluation results for 2-step RACH	Intel Corporation

- R1-1909467	Performance and design details of 2-step RACH	Ericsson

- R1-1909241	Other Considerations for Two-Step RACH	Qualcomm Incorporated

- R1-1909313	Further discussion and evaluations for 2-step RACH	Huawei, HiSilicon

- R1-1913173	Introduction of two-step RACH	Qualcomm

- R1-1913174	Introduction of two-step RACH	Ericsson

- R1-1913175	Introduction of two-step RACH	Samsung

- R1-1913176	Introduction of two-step RACH	Nokia

- R1-1911827	Reamaining isuses of msgA channel structure	ZTE, Sanechips

- R1-1911871	Discussion on channel structure of 2-step RACH	Huawei, HiSilicon

- R1-1912006	Discussion on channel structure for 2-step RACH	vivo

- R1-1912067	Discussion on 2-step RACH Channel Structure	InterDigital Communications

- R1-1912069	Discussion on channel structure for 2-step RACH	Fujitsu

- R1-1912093	Discussion on the gap between preamble and data for 2-step RACH	MediaTek Inc.

- R1-1912191	On remaining details of channel structure for 2-step RACH	Intel Corporation

- R1-1912255	Discussion on Channel Structure for 2-step RACH	Asia Pacific Telecom co. Ltd

- R1-1912262	Discussion on Channel Structure for 2-step RACH	LG Electronics

- R1-1912337	Remaining issues on 2-step RACH Channel Structure	Sony

- R1-1912443	Channel Structure for Two-Step RACH	Samsung

- R1-1912531	Discussion on Channel Structure of 2-step RACH	CMCC

- R1-1912557	Further considerations on 2-step RACH channel structure	Nokia, Nokia Shanghai Bell

- R1-1912644	Channel structure for 2-step RACH	Sharp, NICT

- R1-1912650	On Channel Structure for 2-step RACH	OPPO

- R1-1912671	Channel Structure for Two-step RACH	Ericsson

- R1-1912745	Discussion on channel structure for 2-step RACH	Panasonic Corporation

- R1-1912803	Discussion on channel structure for 2-step RACH	Apple Inc.

- R1-1912869	Discussion on Channel Structure for Two-Step RACH	NTT DOCOMO, INC.

- R1-1913278	Channel Structure for Two-Step RACH	Qualcomm Incorporated

- R1-1913025	Considerations on Channel Structure for Two-step RACH	CAICT

- R1-1911828	Remaining issues of 2-step RACH procedures	ZTE, Sanechips

- R1-1911872	Discussion on 2-step RACH procedure	Huawei, HiSilicon

- R1-1912007	Discussion on 2-step RACH procedure	vivo

- R1-1912068	Discussion on 2-step RACH Procedures	InterDigital Communications

- R1-1912070	Discussion on procedure for 2-step RACH	Fujitsu

- R1-1912192	On remaining details of procedure for 2-step RACH	Intel Corporation

- R1-1912263	Discussion on Procedure for 2-step RACH	LG Electronics

- R1-1912338	Discussion on Procedure for 2-step RACH	Sony

- R1-1912444	Procedure for Two-step RACH	Samsung

- R1-1912532	Discussion on Procdure of 2-step RACH	CMCC

- R1-1912558	Remaining details of 2-step RACH Procedure	Nokia, Nokia Shanghai Bell

- R1-1912570	Discussion on 2-step RACH procedure	Spreadtrum Communications

- R1-1912651	On Procedure for 2-step RACH	OPPO

- R1-1912672	Procedure for Two-step RACH	Ericsson

- R1-1912746	Discussion on 2-step RACH procedure	Panasonic Corporation

- R1-1912804	Discussion on 2-step RACH procedure	Apple Inc.

- R1-1912849	Remaining issues of 2-step RACH procedure	Motorola Mobility, Lenovo

- R1-1912870	Discussion on Procedure for Two-step RACH	NTT DOCOMO, INC.

- R1-1912933	Procedures for Two-step RACH	Qualcomm Incorporated

- R1-1913026	Considerations on Procedure for Two-step RACH	CAICT

- R1-1911829	Evaluation results for 2-step RACH	ZTE, Sanechips

- R1-1912008	Other aspects for 2-step RACH and UE features	vivo

- R1-1912193	Evaluation results for 2-step RACH	Intel Corporation

- R1-1912264	Discussion on designing 2-step RACH for NR-U	LG Electronics

- R1-1912445	Discussion on PUSCH validation and overlaping handling	Samsung

- R1-1912559	2-step RACH evaluation results	Nokia, Nokia Shanghai Bell

- R1-1912673	Performance and Design Details of 2-Step RACH	Ericsson

- R1-1912912	Further discussion and evaluation results for 2-step RACH	Huawei, HiSilicon

- R1-1912934	Other Considerations for Two-Step RACH	Qualcomm Incorporated

- R2-1903534	Workplan for 2-step RACH	ZTE Corporation, Sanechips

- R2-1904033	Draft LS on MsgA content size	Nokia, Nokia Shanghai Bell

- R2-1904377	LS on message sizes for 2-step RACH	Ericsson

- R2-1903536	LS on msgA payload size for 2-step RACH	ZTE Corporation, Sanechips

- R2-1903428	MsgA and msgB size for 2-step RACH	Ericsson

- R2-1903535	MsgA payload size for 2-step RACH	ZTE Corporation, Sanechips

- R2-1904109	Discussion on MsgA payload content	Huawei, HiSilicon

- R2-1904979	Discussion on payload size of msgA for 2-step RACH	LG Electronics

- R2-1904439	Procedure for 2-Step RACH	Intel Corporation

- R2-1903112	2 Step RA_MsgA Aspects	Samsung Electronics Co., Ltd

- R2-1903077	Considerations on the 2-step RACH Procedure	vivo

- R2-1903113	2 Step RA_MsgB Aspects	Samsung Electronics Co., Ltd

- R2-1903152	Consideration on 2-step RACH	CATT

- R2-1903246	Procedures for 2-step RACH	ETRI

- R2-1903426	2-step RACH General Aspects	Ericsson

- R2-1903794	Discussion on 2-step RACH procedure	Spreadtrum Communications

- R2-1903914	Discussion on 2-step RACH procedure	Panasonic Corporation

- R2-1904110	Discussion on the 2-step RACH procedure	Huawei, HiSilicon

- R2-1904426	RNTI and HARQ aspects of 2-step RACH	MediaTek Inc.

- R2-1904438	Consideration of MsgA contents and size	Intel Corporation

- R2-1903073	Discussion on the Trigger Events of 2-step RACH	vivo

- R2-1904108	Discussion on supported triggers and scenarios for 2-step RACH	Huawei, HiSilicon

- R2-1904031	Applicable triggers for 2-step RACH	Nokia, Nokia Shanghai Bell

- R2-1904429	SI Request and BFR triggers for 2-step RACH	MediaTek Inc.

- R2-1904428	On the contents of msgA and msgB in 2-step RACH	MediaTek Inc.

- R2-1903292	Contents for msgA and msgB	OPPO

- R2-1904032	MsgA for 2-step RACH	Nokia, Nokia Shanghai Bell

- R2-1903548	General consideration on the content of MsgB	ZTE Corporation, Sanechips

- R2-1903608	Msg-A Content of 2-step RACH	Apple

- R2-1903716	msgB design	Nokia, Nokia Shanghai Bell

- R2-1903731	Contents in MAC layer of msgA and msgB	LG Electronics Inc.

- R2-1904969	two-step RACH msgA contents and UE ID for two-step RACH	Qualcomm

- R2-1904970	two-step RACH msgB contents and RNTI for msgB	Qualcomm

- R2-1905118	Initial consideration on 2 step RACH	NTT DOCOMO INC.

- R2-1903115	2 Step RA_MAC PDU Format for MsgB	Samsung Electronics Co., Ltd

- R2-1904113	Discussion on MsgB for 2-step RACH	Huawei, HiSilicon

- R2-1904325	Discussion of MsgA and MsgB identification	CMCC

- R2-1904971	two-step RACH fall back to four-step RACH	Qualcomm Incorporated

- R2-1904210	Consideration on fall back procedure from 2-step RACH to 4-step RACH	Sony

- R2-1903114	Fallback from 2 step to 4 step RACH	Samsung Electronics Co., Ltd

- R2-1903294	Fall back mechanisms for 2-steps RACH	OPPO

- R2-1903427	Fallback for 2-step RACH	Ericsson

- R2-1903715	Fall back from 2-step RACH to 4-step RACH	Nokia, Nokia Shanghai Bell

- R2-1904071	Fall back to 4 step RACH procedure	NEC

- R2-1904324	Discussion on RACH resources and fallback from 2-step to 4-step random access	CMCC

- R2-1903075	Differentiation between 2-step and 4-step RACH	vivo

- R2-1903782	On procedure selection among 2-step and 4-step RACH	Fujitsu

- R2-1903783	On recognition of msgB or msg2	Fujitsu

- R2-1904111	Discussion on the MsgA resource configuration	Huawei, HiSilicon

- R2-1904112	Discussion on RACH type switching between 2-step, 4-step RACH and CFRA	Huawei, HiSilicon	R2-1904211	PUSCH Configurations for MsgA of 2-step RACH	Sony

- R2-1904947	Random Access Resource selection procedure for 2-step CBRA	LG Electronics

- R2-1903293	Contention resolution for 2-step RACH	OPPO

- R2-1903429	RNTIs and Contention Resolution for 2-step RACH	Ericsson

- R2-1903729	Contention-based 2-Step RACH procedure	LG Electronics Inc.

- R2-1903074	Clarification on the Cntention-based 2-step RACH	vivo

- R2-1903291	Clarification on contention based 2-step RACH	OPPO

- R2-1903717	Network control of 2-step RACH	Nokia, Nokia Shanghai Bell

- R2-1903732	RNTI for msgB in 2-step RACH	LG Electronics Inc.

- R2-1904034	RNTI design for MsgB reception	Nokia, Nokia Shanghai Bell

- R2-1903781	Commonality of MsgB for RAR	Fujitsu

- R2-1903076	Prioritized RA Parameters for 2-step RACH	vivo

- R2-1906308	email discussion report: Procedures and mgsB content [105bis#30][NR/2-step RACH]	ZTE Corporation (email disc Rapporteur)

- R2-1905596	Procedure of contention resolution for 2-step RACH	OPPO	discussion

- R2-1905597	Considering msgB reception window in NR and NR-U	OPPO	discussion

- R2-1905598	Draft LS on MsgB reception window	OPPO

- R2-1905650	The Starting Point of the MsgB Reception Window	vivo

- R2-1905686	Discussion on 2-step RACH procedure	Spreadtrum Communications

- R2-1905720	2 Step RA_MsgB Aspects	Samsung Electronics Co., Ltd

- R2-1905753	2-step RACH procedure	CATT

- R2-1906303	Consideration on the contention resolution in 2-step RACH	ZTE Corporation, Sanechips

- R2-1906304	Ack in UL for MsgB reception	ZTE Corporation, Sanechips

- R2-1906307	Transmission of MAC SDU for SRB/DRB in 2-step RACH	ZTE Corporation, Sanechips

- R2-1906555	General 2-Step RACH procedure	LG Electronics Inc.

- R2-1906572	Completion of 2-Step RACH procedure	LG Electronics Inc.

- R2-1906661	Need for different window between fallback and contention resolution	LG Electronics Inc.

- R2-1906746	Contention resolution and SRB/DRB delivery for 2-step RACH	ETRI	discussion

- R2-1906953	RNTIs and Contention Resolution for 2-step RA	Ericsson

- R2-1906955	Back-off for 2-step RA	Ericsson

- R2-1906956	RAR and msgB window in 2-step RA	Ericsson

- R2-1907070	Contention resolution for CONNECTED mode UE	Nokia, Nokia Shanghai Bell

- R2-1907071	RNTI design for MsgB reception	Nokia, Nokia Shanghai Bell

- R2-1907073	RNTI and HARQ aspects of 2-step RACH	MediaTek Inc.

- R2-1907179	Contention Resolution in 2-step RACH	Apple

- R2-1907639	TA handling in 2-step RACH	Samsung

- R2-1907727	Discussion on MsgB reception	Huawei, HiSilicon

- R2-1907728	Discussion on the contention resolution for 2-step RACH	Huawei, HiSilicon

- R2-1907732	Discussion on the MsgB window for 2-step RACH	Huawei, HiSilicon

- R2-1907899	Contention resolution for 2-step RACH	Qualcomm Incorporated

- R2-1907909	RAR window for msgB reception	Qualcomm Incorporated

- R2-1905599	Fall back mechanisms for 2-steps RACH	OPPO

- R2-1905651	Fallback Procedure from 2-step to 4-step RACH	vivo

- R2-1905721	Fallback from 2 step to 4 step RACH	Samsung Electronics Co., Ltd

- R2-1905923	Fallback from 2-step to 4-step RA	CMCC

- R2-1906279	Fallback procedure for 2 -step RACH	Intel Corporation

- R2-1906406	2-Step RA procedure and fallback to 4-step RA	InterDigital

- R2-1906570	Fall back procedure to 4-step RACH	LG Electronics Inc.

- R2-1906755	Details on fallback mechanisms	NEC

- R2-1906759	2-step RACH overload control	Nokia, Nokia Shanghai Bell

- R2-1906954	Fall-back for 2-step RA	Ericsson

- R2-1907046	Consideration on fall back procedure from 2-step RACH to 4-step RACH	Sony

- R2-1907733	Discussion on RACH type switching between 2-step, 4-step RACH and CFRA	Huawei, HiSilicon

- R2-1907910	2-step RACH fall back to 4-step RACH	Qualcomm Incorporated

- R2-1908021	Open aspects of the fallback operation	ZTE Corporation

- R2-1905600	msgB contents and formats in 2-step RACH	OPPO	discussion

- R2-1905652	A Single MsgB Format for both RRC CONNECTED and IDLE&INACTIVE UE	vivo

- R2-1905722	2 Step RA_MAC PDU Format for MsgB	Samsung Electronics Co., Ltd

- R2-1905754	Reception of Msg2 and msgB	CATT

- R2-1906280	Consideration of MsgB contents and design	Intel Corporation

- R2-1906306	MsgB content and format	ZTE Corporation, Sanechips

- R2-1906420	RA response for 2-step RACH	ETRI	discussion

- R2-1906577	Considerations on msgB design	LG Electronics Inc.

- R2-1906579	RNTI design for msgB	LG Electronics Inc.

- R2-1906952	Multiplexing and msgB in 2-step RA	Ericsson

- R2-1907731	Discussion on MsgB content for 2-step RACH	Huawei, HiSilicon

- R2-1907911	msgB content in 2-step RACH	Qualcomm Incorporated

- R2-1905601	RACH initialization and resource selection for 2-step RACH	OPPO

- R2-1905644	Criteria on RACH type selection	OPPO

- R2-1905653	Selection Between 2-step and 4-step RACH	vivo

- R2-1905654	Differentiation Between 2-step and 4-step RACH	vivo

- R2-1905655	Discussion on the MsgA Resource Selection	vivo

- R2-1905719	2 Step RA_MsgA Aspects	Samsung Electronics Co., Ltd

- R2-1905922	System information related aspects for 2-step RACH resources configuration	CMCC

- R2-1906256	On procedure selection among 2-step and 4-step RACH	Fujitsu

- R2-1906257	On recognition of msgB or msg2	Fujitsu

- R2-1906305	Consideration on the load balance between 2-step RACH and 4-step RACH	ZTE Corporation, Sanechips, CATT

- R2-1906580	Criteria for 2-step RACH selection	LG Electronics Inc.

- R2-1906957	Selection of 2-step or 4-step RA	Ericsson

- R2-1907047	PUSCH Configurations for MsgA of 2-step RACH	Sony

- R2-1907725	Discussion on the MsgA resource selection	Huawei, HiSilicon

- R2-1907730	Discussion on the MSGA resource configuration	Huawei, HiSilicon

- R2-1907913	Criteria for selecting 2-step RACH and 4-step RACH	Qualcomm Incorporated

- R2-1907914	2-step RACH for CFRA BFR	Qualcomm Incorporated

- R2-1907925	Random Access Resource selection procedure for 2-step RACH	LG Electronics

- R2-1905595	Clarification on contention free 2-step RACH	OPPO	discussion

- R2-1905656	Considerations on the MsgA Without PRACH Transmission	vivo	discussion

- R2-1905657	Prioritized RA Parameters for 2-step RACH	vivo	discussion

- R2-1905920	Discussion of MsgA content for different cases	CMCC

- R2-1905921	PUSCH segmentation and repetition	CMCC

- R2-1906574	Need for RAPID MAC CE in payload of msgA	LG Electronics Inc.

- R2-1907726	Discussion on MsgA transmisison	Huawei, HiSilicon

- R2-1907729	Discussion on power control for MsgA	Huawei, HiSilicon

- R2-1907734	Backoff for two-step random access	Huawei, HiSilicon

- R2-1907735	RACH differentiation for two-step random access	Huawei, HiSilicon

- R2-1907736	Discussion on supported triggers and scenarios for 2-step RACH	Huawei, HiSilicon

- R2-1907919	Discussion on scope of 2-step RACH WI	LG Electronics

- R2-1908621	LS Reply on 2-step RACH MsgA content and power control (R1-1907948; contact: Nokia)	RAN1

- R2-1908698	Remaining Issues on the Content and Usage of SuccessRAR	vivo

- R2-1908699	Remaining Issues on Fallback Procedure from 2-step to 4-step RACH	vivo

- R2-1908700	Selection between 2-step and 4-step RACH	vivo

- R2-1908701	Discussion on the Resource Selection for MsgA	vivo

- R2-1908702	Considerations on MAC PDU Format for 2-step RACH	vivo

- R2-1908703	Considerations on the MsgA without PRACH Transmission	vivo

- R2-1908704	Differentiation between 2-step and 4-step RACH	vivo

- R2-1908705	Prioritized Random Access Parameters for 2-step RACH	vivo

- R2-1908769	Contention resolution for C-RNTI MAC CE case	OPPO

- R2-1908770	Contention resolution for CCCH SDU case	OPPO

- R2-1908771	Fall back mechanisms for 2-steps RACH	OPPO

- R2-1908772	RACH initialization and resource selection for 2-step RACH	OPPO

- R2-1908773	msgB contents and formats in 2-step RACH	OPPO

- R2-1908774	Clarification on contention free 2-step RACH	OPPO

- R2-1908804	2 Step RA_12bit Timing Advanced Information	Samsung Electronics Co., Ltd

- R2-1908805	2 Step RA_HARQ Feedback for MsgB	Samsung Electronics Co., Ltd

- R2-1908806	2 Step RA_Msg2 or MsgB for Fallback and Backoff	Samsung Electronics Co., Ltd

- R2-1908807	2 Step RA_Network Response Timing and Contention Resolution Aspects	Samsung Electronics Co., Ltd

- R2-1908808	2 Step RA_MsgA Details	Samsung Electronics Co., Ltd

- R2-1908809	2 Step RA_One or Multiple RRC messages for a UE in Network Response	Samsung Electronics Co., Ltd

- R2-1908810	2 Step RA_RACH prioritisation	Samsung Electronics Co., Ltd

- R2-1908811	2 step RA_Remaining Issues for Fallback from 2 step to 4 step RACH	Samsung Electronics Co., Ltd

- R2-1908812	2 Step RA_Selection between 2 step RACH and 4 step RACH	Samsung Electronics Co., Ltd

- R2-1908813	2 Step RACH_RNTI for Receiving Network Response	Samsung Electronics Co., Ltd

- R2-1908814	2 Step RA_MAC PDU Format for MsgB_Approach 1	Samsung Electronics Co., Ltd

- R2-1908815	2 Step RA_MAC PDU Format for MsgB_Approach 2	Samsung Electronics Co., Ltd

- R2-1908816	2 Step RA_Preamble & RO Configuration	Samsung Electronics Co., Ltd

- R2-1908837	Remaining issues of successRAR in 2-step RACH	CATT

- R2-1908838	Remaining Issues of FallbackRAR in 2-step RACH	CATT

- R2-1908930	MsgB Handling for 2-step RACH	ETRI

- R2-1909097	Discussion on procedure for successful case of 2-step RACH	Spreadtrum Communications

- R2-1909127	TA handling in 2-step RACH	Samsung

- R2-1909147	Details on fallback mechanisms	NEC

- R2-1909153	Consideration on the MsgB content and format	ZTE Corporation, Sanechips

- R2-1909154	Remaining issues on the usage of 2-step RACH	ZTE Corporation, Sanechips

- R2-1909155	Stage 3 issues for 2-step RACH	ZTE Corporation, Sanechips

- R2-1909156	Resource configuration and selection for 2-step RACH	ZTE Corporation, Sanechips

- R2-1909157	Running MAC CR for 2-step RACH	ZTE Corporation, Sanechips

- R2-1909158	Stage2 CR for 2-step RACH	ZTE Corporation, Sanechips, China Southern Power Grid Co., Ltd, Nokia (Stage2 spec rapporteur)

- R2-1909159	MsgB Open Issues	ZTE Corporation, Sanechips, Nokia, Nokia Shanghai Bell

- R2-1909160	Open aspects of fallback operation	ZTE Corporation, Sanechips, Nokia, Nokia Shanghai Bell

- R2-1909238	Discussion on the RACH Type Selection	PANASONIC R&D Center Germany

- R2-1909452	Fallback from 2-step to 4-step RA	CMCC

- R2-1909453	Discussion of the PUSCH transmission of MsgA	CMCC

- R2-1909454	PUSCH segmentation and repetition	CMCC

- R2-1909455	System information related aspects for 2 step RA	CMCC

- R2-1909566	Further criteria on whether the UE uses 2-step RACH or 4-step RACH	Intel Corporation

- R2-1909567	Further consideration of the fallback to 4-step RACH and BI	Intel Corporation

- R2-1909568	Further consideration of MsgB design and reception when C-RNTI is included in MsgA	Intel Corporation

- R2-1909607	Fallback procedure to 4-step RA	InterDigital

- R2-1909656	Discussion on RA resource selection	NTT DOCOMO, INC.

- R2-1909756	MsgB PDU Format for Idle/Inactive State	Potevio

- R2-1909770	2-step RACH power control	CMCC

- R2-1909819	consideration of UL ACK for MSGB reception	Beijing Xiaomi Mobile Software

- R2-1909820	consideration of UL ACK for MSGB reception	Beijing Xiaomi Mobile Software

- R2-1909821	Contention resolution of 2-step RACH when CCCH message is included in msgA	LG Electronics Inc.

- R2-1909822	Issues on SRB inclusion in successRAR	LG Electronics Inc.

- R2-1909823	Contents of successRAR	LG Electronics Inc.

- R2-1909824	Issue on whether legacy msg2 can be reused for fallbackRAR	LG Electronics Inc.

- R2-1909825	Need for RAPID MAC CE in payload of msgA	LG Electronics Inc.

- R2-1909826	RNTI design for msgB	LG Electronics Inc.

- R2-1909827	Criteria for 2-step RACH selection	LG Electronics Inc.

- R2-1909828	MAC PDU formats for msgB	LG Electronics Inc.

- R2-1909834	consideration of UL ACK for MSGB reception	Beijing Xiaomi Mobile Software

- R2-1909870	Consideration on  MsgB Content	Apple

- R2-1909871	Consideration on 2-step RACH Fallback Case	Apple

- R2-1909872	RRC Message Transmisson in 2-step RACH	Apple

- R2-1909905	Fall back procedure from 2-step RACH to 4-step RACH msg1	Sony

- R2-1909906	PUSCH Configurations for MsgA of 2-step RACH	Sony

- R2-1909930	HARQ use in 2-step RA	Ericsson

- R2-1909931	2-step RA for BFR	Ericsson

- R2-1909932	New RAR formats for 2-step RA	Ericsson

- R2-1909933	RNTIs for 2-step RA	Ericsson

- R2-1909934	Fallback for 2-step RA	Ericsson

- R2-1909935	Back-off for 2-step RA	Ericsson

- R2-1909936	Configuration of 2-step RA	Ericsson

- R2-1909937	Selection of 2 or 4-step RA	Ericsson

- R2-1909938	PUSCH selection and Support of larger MsgA payload in 2-step RA	Ericsson

- R2-1909997	Aspects on msgB in 2-step RACH	Fujitsu

- R2-1909998	Discussion on backoff indication in 2-step RACH	Fujitsu

- R2-1910081	MsgB format for 2-step RACH	MediaTek Inc.

- R2-1910082	On contention resolution in 2-step RACH	MediaTek Inc.

- R2-1910095	2 step RACH overload control	Nokia, Nokia Shanghai Bell

- R2-1910106	Criteria on RACH type selection	OPPO

- R2-1910377	Discussion on fallbackRAR in msg2	Google Inc.

- R2-1910378	Further consideration of MsgB contents and procedure when CCCH SDU is included in MsgA	Intel Corporation

- R2-1910379	MsgB format design when CCCH message is included in MsgA	Intel Corporation

- R2-1910619	Contention resolution for CONNECTED mode UE	Nokia, Nokia Shanghai Bell

- R2-1910620	Design of MsgB MAC PDU format	Nokia, Nokia Shanghai Bell

- R2-1910621	RNTI design for MsgB reception	Nokia, Nokia Shanghai Bell

- R2-1910668	Discussion on the MsgA resource selection	Huawei, HiSilicon

- R2-1910669	Discussion on fallbackRAR	Huawei, HiSilicon

- R2-1910670	Discussion on the MSGA resource configuration	Huawei, HiSilicon

- R2-1910671	Discussion on content for successRAR	Huawei, HiSilicon

- R2-1910672	Discussion MAC PDU format for msgB	Huawei, HiSilicon

- R2-1910675	Discussion on HARQ for MsgA	Huawei, HiSilicon

- R2-1910676	Discussion on power control for MsgA	Huawei, HiSilicon

- R2-1910677	Backoff for two-step random access	Huawei, HiSilicon

- R2-1910678	Discussion on the timers for 2-step RACH	Huawei, HiSilicon

- R2-1910679	Differentiation for two-step random access	Huawei, HiSilicon

- R2-1910680	Discussion on HARQ for msgB	Huawei, HiSilicon

- R2-1910681	Measurement gap and 2-step RACH	Huawei, HiSilicon

- R2-1910682	2-step RACH and preamble transmission counters	Huawei, HiSilicon

- R2-1910683	Discussion on supported scenarios for fallbacks in two-step RACH	Huawei, HiSilicon

- R2-1910984	Discussion on MsgA payload rebuilding issue	LG Electronics Inc.

- R2-1911347	2-step RACH fall back to 4-step RACH	Qualcomm Incorporated

- R2-1911349	msgB content in 2-step RACH	Qualcomm Incorporated

- R2-1911351	Discussion on TA command for RRC connected UE	Qualcomm Incorporated

- R2-1911365	RAR window for msgB reception and RNTI design for msgB	Qualcomm Incorporated

- R2-1911367	Criteria for selecting 2-step RACH and 4-step RACH	Qualcomm Incorporated

- R2-1911368	Discussion on using dedicated preamble for 2-step RACH	Qualcomm Incorporated

- R2-1911370	msgB packet format	Qualcomm Incorporated

- R2-1911371	Random Access Resource selection procedure for 2-step RACH	LG Electronics

- R2-1911402	Dedicated preamble support in 2-step RACH	Fujitsu, Qualcomm Incorporated, OPPO

- R2-1911501	Consideration on the load balance between 2-step RACH and 4-step RACH	ZTE Corporation, Sanechips, CATT

- R2-1911862	LS to RAN1 on RAN2 agreements related to 2-step RACH	CATT

- R2-1911774	Summary of offline discussion on overload control	CATT

- R2-1911775	Summary of offline discussion on CFRA RAN2 impacts/recommendations and other failure case handling	ZTE

- R2-1912009	LS Reply on overall procedure for 2-step RACH (R1-1909554; contact: ZTE)	RAN1

- R2-1912084	Remaining issues on supporting only 2-step RACH configuration on a UL BWP	OPPO

- R2-1912085	msgB MAC PDU format in 2-step RACH	OPPO

- R2-1912086	Remaining stage-2 open issues	OPPO

- R2-1912087	Stage3 issues on contention resolution for 2-step RACH	OPPO

- R2-1912088	Contention free 2-step RACH	OPPO

- R2-1912186	Differentiation Between 2-step and 4-step RACH	vivo

- R2-1912187	Remaining Issue on Contention Resolution in 2-step RACH	vivo

- R2-1912188	Remaining Issue on Fallback from 2-step to 4-step RACH	vivo

- R2-1912189	Resource Selection for MsgA	vivo

- R2-1912190	Prioritized RA Parameters for 2-step RACH	vivo

- R2-1912191	Discussion on the 2-step CFRA	vivo

- R2-1912223	On the remaining open issues of 2-step RACH	CATT

- R2-1912224	Load balancing between RACH Types	CATT

- R2-1912225	Remaining Issue of RA Type Selection	CATT

- R2-1912226	Open issues of 2-step CFRA	CATT

- R2-1912251	Discussion on the SuccessRAR for SI request	Spreadtrum Communications

- R2-1912430	2 Step RA_12bit Timing Advanced Information	Samsung Electronics Co., Ltd

- R2-1912431	2 Step RA_RACH prioritisation	Samsung Electronics Co., Ltd

- R2-1912432	2 Step RACH_RNTI for Receiving Network Response	Samsung Electronics Co., Ltd

- R2-1912433	2 Step RA_RACH Configuration	Samsung Electronics Co., Ltd

- R2-1912468	MsgB Retransmission Scheme	Apple

- R2-1912469	2-step CFRA	Apple

- R2-1912478	TA handling in 2-step RACH	Samsung

- R2-1912678	Open issues for msgB formats for 2-step RA	Ericsson

- R2-1912679	RNTI design and HARQ aspects for 2-step RA	Ericsson

- R2-1912680	Configuration of 2-step RA	Ericsson

- R2-1912681	2-step CFRA	Ericsson

- R2-1912682	PUSCH selection and MsgA payloads in 2-step RA	Ericsson

- R2-1912683	2-step RA 38.331 Draft CR	Ericsson

- R2-1912692	Loading Control in the RACH Type Selection	PANASONIC R&D Center Germany

- R2-1912794	Further consideration on the SucessRAR design and Fallback RAR	Intel Corporation

- R2-1912795	Preamble grouping selection or PUSCH TBS size selection for 2-step RACH	Intel Corporation

- R2-1912796	Support of CFRA in 2-step RACH	Intel Corporation

- R2-1912947	HARQ feedback for MsgB	CMCC

- R2-1912948	PUSCH segmentation and repetition	CMCC

- R2-1912949	Discussion of the PUSCH transmission of MsgA	CMCC

- R2-1912954	Further discussion on RA type selection	CMCC

- R2-1912955	System information for 2-step RACH resources configuration	CMCC

- R2-1913006	PDU format for msgB	Huawei, HiSilicon

- R2-1913007	msgA resource configuration and selection	Huawei, HiSilicon

- R2-1913008	Discussion on the MsgA transmission	Huawei, HiSilicon

- R2-1913009	Discussion on MsgB reception	Huawei, HiSilicon

- R2-1913010	fallback procedure for 2-step RACH	Huawei, HiSilicon

- R2-1913011	Differentiation for two-step random access	Huawei, HiSilicon

- R2-1913144	Discussion on HARQ and RNTI design for msgB	Google

- R2-1913168	Considerations on MsgB reception	Fujitsu

- R2-1913169	On open questions to 2-step CF-RACH	Fujitsu

- R2-1913217	RNTI design for MsgB reception	Nokia, Nokia Shanghai Bell

- R2-1913218	Draft LS on MsgB RNTI design	Nokia, Nokia Shanghai Bell

- R2-1913219	Further design aspects of MsgB MAC subheader formats	Nokia, Nokia Shanghai Bell

- R2-1913220	New TAC MAC CE for 2-step RACH	Nokia, Nokia Shanghai Bell

- R2-1913221	Preamble groups for 2-step RACH	Nokia, Nokia Shanghai Bell

- R2-1913222	CFRA for 2-step RACH	Nokia, Nokia Shanghai Bell

- R2-1913264	MsgB format proposal for 2-step RACH	MediaTek Inc.

- R2-1913291	Stage-2 running CR for 2-step RACH	Nokia (rapporteur), Nokia Shanghai Bell

- R2-1913356	Fallback to 4-step RACH with BI	Sony

- R2-1913357	HARQ support for MsgB of 2-Step RACH	Sony

- R2-1913366	Differentiating between MSG2 and MSGB	ZTE Corporation, Sanechips

- R2-1913367	Remaining Stage 2 issues for 2-step RACH	ZTE Corporation, Sanechips

- R2-1913368	Support of CFRA with 2-step RACH	ZTE Corporation, Sanechips, Qualcomm, Fujitsu

- R2-1913369	Configuration aspects of 2-step RACH	ZTE Corporation, Sanechips

- R2-1913370	Running MAC CR for 2-step RACH	ZTE Corporation (email rapporteur)

- R2-1913401	Discussion on HARQ feedback for msgB and RNTI design for msgB	Qualcomm Incorporated

- R2-1913402	Prioritized 2-step RACH	Qualcomm Incorporated

- R2-1913403	Report of email discussion [107#68] [NR/2-step RACH]: MAC PDU format for msgB	Qualcomm Incorporated

- R2-1913404	msgB MAC PDU format	Qualcomm Incorporated

- R2-1913405	Remaining issue on contention resolution and fall back procedure	Qualcomm Incorporated

- R2-1913448	Discussion on some open issues in running MAC CR	Email discussion rapporteur (ZTE Corporation)

- R2-1913449	Summary of running MAC CR review issue list	ZTE Corporation (email disc Rapporteur)

- R2-1913756	MAC PDU format of Success RAR	ETRI

- R2-1913870	RNTI design for msgB	LG Electronics Inc.

- R2-1913872	Remaining issues on 2-step random access resource	LG Electronics

- R2-1913874	MAC PDU formats for msgB	LG Electronics Polska

- R2-1913875	Need for RAPID MAC CE in payload of msgA	LG Electronics Inc.

- R2-1913877	Logical channel based RA type selection	LG Electronics Inc.

- R2-1913879	Consideration on 2-step CFRA	LG Electronics

- R2-1913914	Clarification on criteria selection for 2-step RACH and 4-step RACH	Qualcomm Incorporated

- R2-1914051	[DRAFT] LS to RAN1 on 2-step RACH	ZTE

- R2-1914052	Summary of discussion on rebudiling and fallback	Nokia

- R2-1914067	[DRAFT] LS on differentiating between MSG2 and MSGB	ZTE

- R2-1914191	LS on differentiating between MSG2 and MSGB	RAN2

- R2-1914311	Reply to LS on differentiating between MSG2 and MSGB (R1-1911656; contact: Nokia)	RAN1

- R2-1914319	Reply to LS on RAN2 agreements related to 2-step RACH (R1-1911739; contact: Nokia)	RAN1

- R2-1914373	Differentiation Between Msg2 and MsgB by MsgB-RNTI	vivo

- R2-1914374	Discussion on the 2-step CFRA	vivo

- R2-1914375	Contention Resolution of 2-step RACH Triggered by Uplink Data Arrival Without Valid TA	vivo

- R2-1914376	Remaining Issues on MsgA Transmission and Fallback	vivo

- R2-1914377	Resource Selection for 2-step RACH	vivo

- R2-1914378	Discussion on the BFR-initiated 2-step RACH	vivo

- R2-1914388	Remaining open issues for 2-step RACH	OPPO

- R2-1914389	Draft TP for msgB MAC PDU format	OPPO

- R2-1914390	Remaining issues on 2-step RACH configuration without 4-step resources	OPPO

- R2-1914391	Contention resolution for 2-step RACH in C-RNTI MAC CE case	OPPO

- R2-1914392	Contention free 2-step RACH	OPPO

- R2-1914396	[Draft] LS on preamble group for 2-step RACH	OPPO

- R2-1914421	On the remaining open issues of 2-step RACH	CATT

- R2-1914422	Load Balancing between RACH Types	CATT, OPPO, vivo, Spreadtrum, Panasonic, Xiaomi

- R2-1914423	Consideration on msgB-RNTI	CATT

- R2-1914424	RA Type Selection between Legacy CFRA and 2-step CBRA	CATT

- R2-1914430	Report of Email discussion 107bis#76- MSG B Format Design	Samsung Electronics Co., Ltd

- R2-1914431	MAC TP_MSGB Format Design	Samsung Electronics Co., Ltd

- R2-1914432	Legacy CFRA and Selection between 2 step and 4 step RA	Samsung Electronics Co., Ltd

- R2-1914433	2 Step RACH_RNTI for Receiving Network Response	Samsung Electronics Co., Ltd

- R2-1914434	2 Step RA_RACH Configuration	Samsung Electronics Co., Ltd

- R2-1914435	Open issues for supporting 2 step CFRA	Samsung Electronics Co., Ltd

- R2-1914617	Discussion on 2-step RACH for BFR	Asia Pacific Telecom co. Ltd

- R2-1914796	Remaining open issues for 2-step CBRA	ZTE Corporation, Sanechips

- R2-1914797	Consideration on the MsgB RA-RNTI calculation	ZTE Corporation, Sanechips

- R2-1914798	Running MAC CR for 2-step RACH	ZTE Corporation (email rapporteur)

- R2-1914799	Summary of running MAC CR review issue list	ZTE Corporation (email rapporteur)

- R2-1914800	Support of CFRA with 2-step RACH	ZTE Corporation, Sanechips, Vivo, Oppo, Fujitsu, Qualcomm, InterDigital, Intel

- R2-1914842	Other User Plane Stage-3 issues on 2-Step RACH	Intel Corporation

- R2-1914843	Remaining issues on PUSCH resource unit selection for 2-step RACH	Intel Corporation

- R2-1914862	UE capability for 2-step RACH	Intel Corporation

- R2-1915001	Considerations on MsgB reception	Fujitsu

- R2-1915002	On open questions to 2-step CF-RACH	Fujitsu

- R2-1915005	RA Prioritization for MPS for 2-step RACH	Perspecta Labs, ECD, AT&T, T-Mobile, Sprint, Ericsson, Qualcomm

- R2-1915069	RA variables for 2-step RACH fallback procedure	ITRI

- R2-1915215	Discussion on the new RA-RNTI for MsgB	CMCC

- R2-1915216	Discussion on 2-step CFRA	CMCC

- R2-1915225	TA handling in 2-step RACH	Samsung

- R2-1915231	Fallback to 4-step RACH with BI	Sony

- R2-1915239	Applying un-used RNTIs for MsgB of 2-Step RACH	Sony

- R2-1915240	Differentiating MsgB with and without RRC messages	Sony

- R2-1915255	Consideration on the variables for 2-step RACH	ZTE Corporation, Sanechips

- R2-1915268	Draft CR for 2-step CFRA	ZTE Corporation, Sanechips

- R2-1915326	RNTI design for MsgB reception	Nokia, Nokia Shanghai Bell

- R2-1915327	2-step RACH only BWP	Nokia, Nokia Shanghai Bell

- R2-1915328	HARQ ACK resources for successRAR	Nokia, Nokia Shanghai Bell

- R2-1915329	Draft LS on HARQ ACK resources for successRAR	Nokia, Nokia Shanghai Bell

- R2-1915330	CFRA for 2-step RACH	Nokia, Nokia Shanghai Bell

- R2-1915450	Loading Control in the RACH Type Selection	PANASONIC R&D Center Germany

- R2-1915602	RA type switch and fallback in 2-step RA	Ericsson

- R2-1915603	New msgB format for 2-step RA	Ericsson

- R2-1915604	Preamble group handling for 2-step RA	Ericsson

- R2-1915605	CFRA in 2-step RA	Ericsson

- R2-1915606	Beam-specific 2-step RA support	Ericsson

- R2-1915607	msgB RNTI design for 2-step RA	Ericsson

- R2-1915786	2-step RA 38.331 Running CR	Ericsson (Email disc rapporteur)

- R2-1915787	Report on email [107bis#77][2-step RACH] RRC details and running RRC CR (Ericsson)	Ericsson (Email disc rapporteur)

- R2-1915889	Stage-2 running CR for 2-step RACH	Nokia (rapporteur), Nokia Shanghai Bell

- R2-1915928	RAN1 Impact on MsgB Design	Apple

- R2-1915929	2-Step CFRA	Apple

- R2-1916006	Remaining issues on user plane aspects	LG Electronics

- R2-1916013	Remaining issues on PDU format for MsgB	Huawei, HiSilicon

- R2-1916014	MsgA resource configuration and selection	Huawei, HiSilicon

- R2-1916015	Remaining issues on MsgB reception	Huawei, HiSilicon

- R2-1916016	Discussion on MsgB-RNTI design	Huawei, HiSilicon

- R2-1916017	Remaining issues on the msgA transmission	Huawei, HiSilicon

- R2-1916018	Further Discussions on Fallback Procedure for 2-step RACH	Huawei, HiSilicon

- R2-1916024	RNTI for msgB	LG Electronics

- R2-1916027	Draft LS to RAN1 on RNTI for msgB	LG Electronics

- R2-1916028	Consideration on 2-step CFRA	LG Electronics

- R2-1916029	Logical channel based RA type selection	LG Electronics Inc.

- R2-1916060	RNTI design for msgB	Qualcomm Incorporated

- R2-1916062	Contention resolution for msgA with C-RNTI	Qualcomm Incorporated

- R2-1916064	msgB MAC PDU format	Qualcomm Incorporated

- R2-1916065	Remaining issue on 2-step RACH fall back procedure	Qualcomm Incorporated

- R2-1916083	Discussion on MSGB-RNTI design	Google

- R2-1916084	Discussion on msgB HARQ process	Google

- R2-1916163	Discussion on indication for presence of RRC messages	LG Electronics Polska

- R2-1916269	RA Prioritization for MPS for 2-step RACH	Perspecta Labs, CISA ECD, AT&T, T-Mobile, Sprint, Verizon, Ericsson, Qualcomm

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template