3GPP TSG RAN meeting #86									               RP-192723

Sitges, Spain, December 9-12, 2019

Status Report to TSG

Agenda item:			9.4.16

| WI / SI Name | Private Network Support for NG-RAN |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes |  | Performance part: No |  |  | Testing part: No |
| Acronym | NG_RAN_PRN |  |  |  |  |  |  |
| Unique ID | 830081 |  |  |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-191563 |  |  |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: |  | Core part: 03/2020 |  | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: |  | Core part: 66% |  | Performance Part: | Testing part: |  |

Source:

| Leading WG |  | RAN3 |
| --- | --- | --- |
| Rapporteur | Name | Jiang Zheng |
|  | Company | China Telecom |
|  | Email | jiangzheng@chinatelecom.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.2.1	Agreements

2.3.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

2.2.1.1 RAN2#107 Agreements

Agreements on SNPN were as follows:

Agreements

1	The SNPNs (identified by PLMN ID + NID) are broadcasted in SIB1,

FFS whether this is achieved by extending the legacy network list or by introducing a new SNPN specific network list or both.

2	The size and format of the NID will not be discussed in RAN2 (we will be informed by other groups)

3	Up to 12 different SNPNs can be broadcasted in a cell.

4	If “mixed” network sharing is allowed (i.e. a cell can contain both PLMNs and NPNs), the total number of networks indicated in SIB1 (i.e. #PLMN + #SNPN + #PNI-NPN) shall not exceed 12.

5	If HRNN are broadcast then the HRNN should a be broadcasted in a separate SIB (i.e. different from SIB1).

6	SNPN selection functions similar to normal PLMN selection: AS reports the found SNPNs (identified by PLMN ID + NID) to NAS which selects the network. In case of manual selection, the human readable network name (if broadcasted) may also be provided from AS to NAS.

7	Once the UE has selected an SNPN, cell selection/re-selection is only performed within the SNPN, i.e. a cell is only considered suitable if the broadcasted SNPN identifier matches the selected SNPN.

Agreements on PNI-NPN were as follows:

Agreements

1	The PNI-NPNs (identified by PLMN ID + CAG ID) are broadcasted in SIB1

FFS whether this is achieved by extending the legacy network list or by introducing a new PNI-NPN specific network list or both

2	The size and format of the CAG ID will not be discussed in RAN2 (we will be informed by other groups)

3	Up to 12 different PNI-NPNs can be broadcasted in a cell.

4	If HRNN are broadcast then the HRNN should a be broadcasted in a separate SIB (i.e. different from SIB1).

5	Network selection is triggered by NAS whereby AS reports the available PNI-NPNs (identified by PLMN ID + CAG ID) to NAS which selects the network to use. In case of manual network selection, the human readable network name (if broadcasted) may also be provided from AS to NAS.

6	The Allowed CAG list and “CAG only” indication received from upper layers are taken into account in the cell suitability check during cell selection/re-reselection.

Agreements on the scenario support for RAN sharing and Emergency services with NPN were as follows:

Agreements

1	There is no issue identified to support E1 for Rel-16 UEs.

2	(Regarding question E2) Rel-16 UEs not supporting the CAG feature can camp on a CAG cell as an acceptable cell to obtain limited service

3	There is no issue identified to support RS1 for Rel-16 UEs

4	RS2 and RS3 can be supported from RAN2 point of view

There was a Reply LS [57] approved to inform SA2 the RAN2 decision on the support for RAN sharing and Emergency services with Non-Public Networks.

2.2.1.2 RAN2#107b Agreements

Agreements on CAG Aspects were as follows:

Agreements:

- no new mechanism is introduced to handle the priority of a frequency layer of a CAG cell on which the UE is camped (beyond what cellReselectionPriority provides in SIB4 and in RRCRelease).

- the UE can optionally implement an autonomous search function of CAG cells. FFS on the relationship with dedicated priorities.

- reserving a PCI range for CAG cells is purely a deployment issue (does not need to be reflected in the spec)

- the PCI list of CAG cells can optionally be signalled to UEs. FFS on details of the list

- FFS whether proximity indication in CONNECTED mode is needed

- no preliminary access check for CAG cells in CONNECTED mode. The Allowed CAG list is provided to the gNB by the AMF.

- no new mechanism is introduced to handle the priority of a frequency layer of an SNPN cell on which the UE is camped (beyond what cellReselectionPriority provides in SIB4 and in RRCRelease).

- There is no autonomous search function of SNPN cells.

- reserving a PCI range for SNPN cells is purely a deployment issue (does not need to be reflected in the spec)

- FFS whether PCI range of SNPN cells can optionally be signalled to UEs.

- No proximity indication in CONNECTED mode is needed for SNPN.

- no preliminary access check for SNPN cells in CONNECTED mode.

Agreements on SIB1 design were as follows:

Agreements:

- SIB1 of NPN-only cell prevents access attempts by Rel-15 UEs for normal services.

- SIB1/MIB supports prevention of access attempts by Rel-15 UEs on a SNPN-only cell for emergency services.

- SIB1/MIB supports prevention of access attempts by Rel-15 UEs on a CAG-only cell for emergency services (this does not mean that access attempts by Rel-15 UEs for emergency services on CAG-only cell are always not allowed. This is still FFS.The feasibility of allowing emergency services on CAG-only for Rel-15 UEs will be discussed in the email discussion on RRC aspects/SIB1 design)

- Access attempts by Rel-15 UEs for emergency services on SNPN-only cell are not allowed.

- In a NPN-only cell, access attempts for normal services by Rel-16 UEs without support for NPN is not allowed.

- In a SNPN-only cell, access attempts for emergency services by Rel-16 UEs without support for SNPNs is not allowed.

- For a PLMN+NPN cell, Rel-15 UEs should be able to access PLMNs associated with the cell for normal and/or limited service.

- A new Rel-16 IE is needed with a role similar to role of cellReservedForOtherUse for Rel-15 UEs (FFS whether this will be PLMN specific)

- SIB1 allows indication of TAC, RANAC, cellIdentity per SNPN (per PLMN ID + NID). FFS on other IEs. FFS whether Rel-15 IEs or Rel-16 IEs are used for the indication.

- SIB1 allows indication of TAC, RANAC, cellIdentity for each CAG. FFS on other IEs. The fields are indicated per PLMN-ID. FFS whether Rel-15 IEs or Rel-16 IEs are used for the indication.

2.2.1.3 RAN2#108 Agreements

Agreements on SIB1 design were as follows:

Agreements:

- Access attempts by Rel-15 UEs for emergency services on CAG cell could be allowed based on operator's preference

- cellReservedForOtherUse is used to prevent Rel-15 UEs to access the cell.

- NPN information is outside PLMN-IdentityInfoList as a new Rel-16 IE for NPN-only cell and PLMN+NPN cell (the total number of network IDs is still 12)

Working assumption:

- The new Rel-16 IE with a role similar to role of cellReservedForOtherUse for Rel-15 UEs is cell specific.

Agreements on suitable and acceptable NPN cells as follows:

Agreements:

1.	At least one of the following conditions must be satisfied for a cell to be considered as suitable by a Rel-16 UE not in SNPN AM:

a.	Cell is part of either the selected PLMN or the registered PLMN or PLMN of the Equivalent PLMN list of the UE for which the PLMN-ID is broadcast by the cell with no associated CAG-IDs and for which CAG-only indication is absent or false;

b.	Cell is part of either the selected PLMN or the registered PLMN or PLMN of the Equivalent PLMN list of the UE for which Allowed CAG list includes a CAG-ID broadcast by the cell.

2.	Each SNPN-only cell is treated by Rel-16 UEs not in SNPN AM as if cell status is barred.

3.	A CAG cell which is not considered as suitable can be an acceptable cell for a Rel-16 UE not in SNPN AM.

4.	A PLMN-only cell or an SNPN+PLMN cell be an acceptable cell for a Rel-16 UE not in SNPN AM for which CAG-only indication is true for any PLMN-ID broadcast by the cell.

5.	The following are necessary conditions for an SNPN cell to be considered as a suitable cell by a Rel-16 UE in SNPN AM:

a.	the cell is part of either the selected SNPN or the registered SNPN of the UE;

b.	the cell is part of at least one TA that is not part of the list of "Forbidden Tracking Areas" which belongs to either the selected SNPN or the registered SNPN of the UE

c.	the cell is not barred,

d.	the cell selection criteria in clause 5.2.3.2 are fulfilled.

Agreements on manual network selection and cell barring as follows:

Agreements:

- Add the following note in TS 38.304 :

NOTE:	UE is not required to support manual search and selection of PLMN or CAG or SNPN while in RRC CONNECTED state. The UE may use local release of RRC connection to perform manual search if it is not possible to perform the search while RRC connected.

2.	In the UE on request of NAS, the AS shall scan all RF channels in the NR bands according to its capabilities to find available CAGs. On each carrier, the UE shall at least search for the strongest cell, read its system information and report available CAG ID(s) together with their HRNN (if broadcast) and PLMN(s) to the NAS. The search for available CAGs may be stopped on request of the NAS.

If NAS has selected a CAG and provided this selection to AS, the UE shall search for an acceptable or suitable cell belonging to the selected CAG to camp on.

3.	In the UE on request of NAS, the AS shall scan all RF channels in the NR bands according to its capabilities to find available SNPNs. On each carrier, the UE shall at least search for the strongest cell, read its system information and report available SNPN identifiers together with their HRNN (if broadcast) to the NAS. The search for available SNPNs may be stopped on request of the NAS.”

- All the R16 UEs will treat the cell as barred when the legacy IE cellReservedForOtherUse is set to “True” and this cell does not broadcast any CAG-IDs or NIDs.

Agreements on autonomous search function as follows:

Agreements:

- Allow autonomous cell search even in situations when frequency priorities are broadcast in system information.

- UE follows dedicated frequency priorities as in legacy behaviour. If UE run autonomous cell search and at the same time have dedicated frequency priorities, the result from autonomous cell search should not go against that indicated by dedicated frequency priorities (when they are valid).

Agreements on connected mode as follows:

Agreements:

- From RAN2 point of view there is no requirement for CAG ID to be included in RRC signalling at RRC connection establishment.

- For SNPN, include the SNPN ID in the RRCSetupComplete message. Stage 3 detalls are FFS

- For SNPN, there is no need to include SNPN ID in the RRCResumeComplete message since the UE context is known to the network.

- Send a LS to SA3 with Agreement#1 with SA2 and RAN3 in To.

2.2.2	Remaining Open issues

Conclude Connected Mode

Resolve remaining issues for NR

2.3	RAN3

2.3.1	Agreements

2.3.1.1. RAN3#105 Agreements

Agreements on NPN were as follows:

SNPN

1. At Exchange over NG setup, signal via configuration update the list of supported (PLMN ID, NID)s between NG-RAN node and AMF. Can be taken into account by NG-RAN node during NNSF.

2. Add NID into the Initial UE Message

3. Indicate serving (PLMN ID, NID) in the mobility restriction list

4. No equivalent (PLMN ID, NID) in the mobility restriction list

5. At mobility, we assume that source NG-RAN node knows the (PLMN ID, NID)s supported by the candidate target cells

6. At mobility, target RAN node needs to be informed of (serving PLMN, NID)

7. At mobility, target RAN node shall fail the handover if the serving (PLMN, NID) does not match any of the target cell supported list of (PLMN ID, NID)s

8. Need to exchange the list of supported (PLMN ID, NID)s per cell over Xn setup, Xn configuration update, in the two directions.

9. Over F1, we exchange list of supported (PLMN ID, NID)s between DU and CU: DU-configured [PLMN ID, NID] to CU.

10. Over E1, send the list of (PLMN ID, NID)s supported by CU-UP to CU-CP.

FFS how to signal multiple PLMN IDs+NIDs

PNI-NPN

1. Indicate in the mobility restriction list the UE supported list of CAG IDs per plmn

2. Indicate in the mobility restriction list the CAG-only indication per plmn (i.e. allowance of CAG UE to access non-CAG cells)

3. At mobility, we assume that source NG-RAN node knows the list of CAG IDs supported by the candidate target cells

4. At mobility, target RAN shall fail the handover if UE allowed CAG list does not match any of target cell supported list of CAG IDs (assuming target cell is a CAG cell)

5. Need to exchange list of cell supported CAG IDs at Xn setup, and configuration update.

6. all CAG information configured in DU

7. Over F1, need to signal cell supported list of CAG IDs from DU to CU

RAN sharing and emergency services

The emergency scenarios:

UE’s camping behaviour is in RAN2’s scope. RAN3 does not foresee any impacts from scenarios E1 and E2 in e.g. mobility or network access control.

The RAN sharing scenarios:

RAN3 has started discussions on RAN sharing and expects that it should be possible to support all cases, but details need further work.

There was a Reply LS [128] approved to inform SA2 the RAN3 decision on the support for RAN sharing and Emergency services with Non-Public Networks.

There was a LS on clarification for private networks [129] sent to SA2.

2.3.1.2. RAN3#105b Agreements

Agreements on NPN were as follows:

Agreed to have common CRs for PNI NPN and S-NPN

Stage2 BL CRs maintainers: TS 38.300 Nokia; TS 38.401 CT

Stage3 BL CRs maintainers: TS 38.413 QC; TS 38.423 Ericsson; TS 38.473 HW; TS 38.463 ZTE

2.3.1.3. RAN3#106 Agreements

Agreements on NPN were as follows:

NGAP shall support signalling multiple SNPN IDs from AMF to RAN; “Only signalling a single SNPN ID from AMF to RAN is supported in this release”

Both options for NID shall be supported

Xn signalling shall support the scenario where a single logical cell supports more than one SNPN ID

WA: introduce a CHOICE structure for CAG and SNPN (where applicable); further details to be refined

WA: include cell-supported CAG ID list in the Initial UE Message over NG

There was a reply LS on Sending CAG ID in NAS layer [413] (R3-197591) sent to SA3, SA2, RAN2.

2.3.2	Remaining Open issues

- The open issues are as follows:

  - Check where there is any need to further send selected cag ID in initial context setup. UE allowed cag list controls only the access.

  - Check cag involved in UAC?

  - Can AMF be shared between regular PLMN and SNPN?

  - Remove cause value for now. To be continued.

  - Whether concept of “serving” cag id exists? If yes, and whether it would need to be transferred during mobility?

  - Mobility in inactive state to be looked at?

  - The NPN support functions for F1 and E1;

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

RAN and SA rapporteurs are requested to fill in these clauses jointly.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE: This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

Latest WID:

- RP-191563, “Revised WID: Core part: Private Network Support for NG-RAN”, China Telecom, Nokia, Vodafone, WID approved at RAN#84.

RAN2#107:

- R2-1908651, “LS on RAN sharing and Emergency services with Non-Public Networks”, SA2

- R2-1909112, “NPN Work Plan”, Nokia, China Telecom (Rapporteurs), discussion

- R2-1909113, “NPN Stage 2 Draft CR”, Nokia, China Telecom (Rapporteurs), draftCR

- R2-1908977, “Overview of SNPN”, Ericsson, discussion

- R2-1908978, “Overview of PNI-NPN”, Ericsson, discussion

- R2-1909305, “Overview of Non-Public Networks”,	Nokia, Nokia Shanghai Bell, discussion

- R2-1909469, “Consideration on the scope and ways of RPN”, CMCC, discussion

- R2-1909470, “Consideration on standalone NPN”, CMCC, discussion

- R2-1909471, “Consideration on public network integrated NPN”,	CMCC, discussion

- R2-1911165, “Discussion on the deployment for CAG”, China Telecom, Huawei, HiSilicon, discussion

- R2-1911482, “Review of NPN requirements and potential work areas”, LG Electronics France, discussion

- R2-1910766, “Discussion of SA2 LS on RAN sharing and Emergency services with Non-Public Networks”,	Qualcomm Incorporated, discussion

- R2-1908979, “Discussion on SA2 LS on RAN sharing and Emergency services with Non-Public Networks”, Ericsson, discussion

- R2-1908980, “Draft Reply LS on RAN sharing and Emergency services with Non-Public Networks”, Ericsson, LS out

- R2-1909311, “Draft Reply LS on RAN Sharing and Emergency services with Non-Public Networks”, Nokia, Nokia Shanghai Bell, LS out

- R2-1909400, “Emergency support for SNPN and CAG”, Intel Corporation, discussion

- R2-1909401, “RAN sharing for SNPN and CAG”,	Intel Corporation, discussion

- R2-1909402, “[DRAFT] Reply LS on RAN sharing and Emergency services with Non-Public Networks”, Intel Corporation, LS out

- R2-1910552, “Discussion on RAN sharing and emergency services with NPN”, Huawei, HiSilicon, discussion

- R2-1910551, “Reply LS on RAN sharing and emergency services with Non-Public Networks”, Huawei, HiSilicon, LS out

- R2-1910767, “Supporting SNPNs, CAGs and RAN sharing”, Qualcomm Incorporated, discussion

- R2-1909309, “Adding Human-Readable network names for NPNs to SIB”, Nokia, Nokia Shanghai Bell, discussion

- R2-1908981, “Discussion on the need for Human Readable Network Name (HRNN) in broadcast”, Ericsson, discussion

- R2-1908982, “Draft LS on the need for Human Readable Network Name (HRNN) in broadcast”, Ericsson, LS out

- R2-1909267, “Consideration on the System Information of the Private Network”, ZTE Corporation, Sanechips, discussion

- R2-1909306, “SNPN selection”, Nokia, Nokia Shanghai Bell, discussion

- R2-1909308, “Adding NIDs and CAG Identifiers to SIB”, Nokia, Nokia Shanghai Bell, discussion

- R2-1910011, “Discussion on the SNPN and CAG related parameters broadcast by RAN”, vivo, discussion

- R2-1910553, “Consideration on impact on system information”, Huawei, HiSilicon, China Telecom, discussion

- R2-1911157, “Discussion on human-readable name of NPN”, China Telecom, Huawei, HiSilicon, discussion

- R2-1909310, “Preventing non-NPN UEs from selecting NPN cells”, Nokia, Nokia Shanghai Bell, discussion

- R2-1909269, “Consideration on the cell selection of the Private Network”, ZTE Corporation, Sanechips, discussion

- R2-1909307, “CAG impacts on cell selection and reselection”, Nokia, Nokia Shanghai Bell, discussion

- R2-1909912, “Cell selection/reselection with NPN cells”, Sony, discussion

- R2-1910012, “Discussion on CAG and SNPN cell (re)selection”, vivo, discussion

- R2-1910555, “LS to CTI on PLMN and cell selection for NPN”, Huawei, HiSilicon，LS out

- R2-1911054, “PLMN and cell selection for NPN”, Huawei, HiSilicon, discussion

- R2-1910013, “Discussion on CAG and SNPN cell access control”, vivo, discussion

- R2-1910556, “Access control for SNPN and CAG”,	Huawei, HiSilicon, discussion

- R2-1911311, “Discussion on signalling to enable NPN/CAG support in NR”, Samsung, discussion

- R2-1909268, “Consideration on the Access control for the Private Network”, ZTE Corporation, Sanechips, discussion

- R2-1909270, “Consideration on the PCI split for the Private Network”, ZTE Corporation, Sanechips,	discussion

- R2-1909271, Consideration on the autonomous search function of the private network, ZTE, Sanechips, discussion

- R2-1909403, Access Control for SNPN and CAG, Intel Corporation, discussion

- R2-1909674, Discussion on stand-alone non-public networks, OPPO, discussion

- R2-1909675, Discussion on closed access groups, OPPO, discussion

- R2-1910014, Consideration on mobility control of connected UE for CAG, vivo, discussion

- R2-1910554, Support for SNPN and CAG cell reselection,	Huawei, HiSilicon, discussion

- R2-1910557, Support inactive for SNPN and CAG, Huawei, HiSilicon, China Telecom, discussion

- R2-1910558, Discussion on RNA configuration for SNPN and CAG, Huawei, HiSilicon, discussion

- R2-1910559, Discussion on Proximity Indication of NPN, Huawei, HiSilicon, discussion

- R2-1911055, Discussion on handover cases, Huawei, HiSilicon, discussion

- R2-1911056, Enhancement for ANR, Huawei, HiSilicon, discussion

- R2-1911057, Discussion on RRC configuration for PLMN and NPN with RAN sharing and Dual radio, Huawei, HiSilicon, discussion

- R2-1911312, Discussion on Private Network Support for NG-RAN, Samsung R&D Institute India, discussion

- R2-1911827, Reply LS on RAN sharing and Emergency services with NPN, CMCC, LS out, Approved

RAN3#105:

- R3-193832, On Private Network Support for NG-RAN, China Telecommunication, discussion

- R3-193833, Work Plan for Private Network Support for NR-RAN WI, China Telecommunication, discussion

- R3-193960, Draft 38.300 BL CR for Information, Nokia, Nokia Shanghai Bell, China Telecom, discussion

- R3-194165, Explaining NPN, Ericsson, discussion

- R3-194208, Discussion of NPN access control, Qualcomm Incorporated, discussion

- R3-193624, Consideration on NPN solution, ZTE Corporation, discussion

- R3-193626, Discussion on CAG support for NR, ZTE Corporation, discussion

- R3-193327, LS on RAN sharing and Emergency services with Non-Public Networks, 3GPP SA2, Intel, Mediatek, LS in

- R3-194136, Network Sharing for NPN, Ericsson, discussion

- R3-194137, [Draft] Reply LS on RAN sharing with Non-Public Networks, Ericsson, LS out

- R3-194411, RAN sharing and emergency services for NPN, Huawei, China Telecom, discussion

- R3-193834, Reply LS on RAN sharing and Emergency services with Non-Public Networks, China Telecommunication, discussion

- R3-193726, [DRAFT] Reply LS RAN sharing and Emergency services with Non-Public Networks, Intel Corporation, LS out

- R3-193574, Discussion on the received LS on NPN scenarios for emergency calls and RAN sharing, Qualcomm Incorporated, discussion

- R3-193575, [Draft] Reply LS on RAN sharing and Emergency services with Non-Public Networks, Qualcomm Incorporated, LS out

- R3-194382, Support of NPN on NG and Xn interface, Ericsson, discussion

- R3-193667, On common solution for CAG and SNPN, NEC, discussion

- R3-193999, Private Network Support Access Control, Samsung, discussion

- R3-194000, CR for Private Network Impact in NG, Samsung, CR

- R3-194001, CR for Private Network  Impact in Xn, Samsung, CR

- R3-194210, Discussion on identities and common procedures for NPN, Qualcomm Incorporated, discussion

- R3-193721, NPN support in NG-RAN, Intel Corporation, discussion

- R3-193723, Introduction of NPN support in NG, Intel Corporation, CR

- R3-193724, Introduction of NPN support in Xn, Intel Corporation, CR

- R3-193577, Introduction of Non-Public Networks, Qualcomm Incorporated, CR

- R3-194408, Public network integrated NPNs, CMCC, discussion

- R3-194414, Discussion on Paging for NPN, Huawei, discussion

- R3-194415, CR to TS 38.423 on support of NPN, Huawei, CR

- R3-194416, CR to TS 38.413 on support of NPN, Huawei, CR

- R3-194417, CR to TS 38.300 on support of NPN, Huawei, draftCR

- R3-194418, CR to TS 36.300 on support of NPN, Huawei, draftCR

- R3-193828, Introduction of support Public network integrated NPN on NG interface, China Telecommunication, discussion

- R3-193829, CR to 38.413 for introducing PNI-NPN in Rel-16, China Telecommunication, CR

- R3-194413, General support of public network integrated NPN, Huawei, LGU+, discussion

- R3-193961, Overview of RAN3 impacts of PNI NPN, Nokia, Nokia Shanghai Bell, discussion

- R3-193962, Introduction of PNI NPN, Nokia, Nokia Shanghai Bell, CR

- R3-193830, Introduction of support Standalone NPN on NG interface, China Telecommunication, discussion

- R3-193831, CR to 38.413 for introducing SNPN in Rel-16, China Telecommunication, CR

- R3-194412, General support of standalone NPN, Huawei, discussion

- R3-193963, Overview of RAN3 impacts of Stand-alone NPN, Nokia, Nokia Shanghai Bell, discussion

- R3-193964, Introduction of Stand-alone NPN , Nokia, Nokia Shanghai Bell, CR

- R3-193965, LS on mobility in Stand-alone NPN, Nokia, Nokia Shanghai Bell , LS out

- R3-193576, Discussion of NPN support for connected mode mobility, Qualcomm Incorporated, discussion

- R3-193722, Mobility aspects of NPN, Intel Corporation, discussion

- R3-193727, [DRAFT] LS on NPN clarifications, Intel Corporation, LS out

- R3-194419, Handover for public network integrated NPN, Huawei, discussion

- R3-194420, Handover within SNPN and Interworking between SNPN and PLMN, Huawei, discussion

- R3-194421, Connection management between PLMN and SNPN, Huawei, discussion

- R3-194422, LS on connection management between PLMN and SNPN, Huawei, LS out

- R3-193803, Architecture for SNPN with a CU DU Split, AT&T, discussion

- R3-194166, Support for CU-DU Split – F1 interface impact, Ericsson, discussion

- R3-194207, Discussion for the NPN impact in F1 interface, Samsung Electronics Co., Ltd, discussion

- R3-194002, Private Network Support in F1 interface, Samsung, CR

- R3-193578, Discussion of NPN impacts on the F1 interface, Qualcomm Incorporated, discussion

- R3-193725, Introduction of NPN support in F1, Intel Corporation, CR

- R3-194238, Introduction of Public Network Integrated NPN, Nokia, Nokia Shanghai Bell, CR

- R3-194239, Introduction of Stand-alone NPN, Nokia, Nokia Shanghai Bell, CR

- R3-193835, On introducing Non Public Network in NG-RAN architecture, China Telecommunication, discussion

- R3-193836, CR for introducing Non Public Netwrok in NG-RAN architecture, China Telecommunication, CR

- R3-194485, General support of NPN over F1, Huawei, China Telecom, discussion

- R3-194487, CR to 38.470 on support of  NPN over F1, Huawei, China Telecom, CR

- R3-194486, CR to 38.473 on support of  NPN over F1, Huawei, China Telecom, CR

- R3-194491, Discussion on the human-readable name of NPN for disaggregated gNB, Huawei, discussion

- R3-193804, Architecture for SNPN with a CP UP Split, AT&T, discussion

- R3-194167, Support for CP-UP Split – Impact on E1 interface, Ericsson, discussion

- R3-194240, Introduction of Stand-alone NPN, Nokia, Nokia Shanghai Bell, CR

- R3-194488, General support of NPN over E1, Huawei, China Telecom, discussion

- R3-194489, CR to 38.463 on support of NPN over E1, Huawei, China Telecom, CR

- R3-194490, CR to 38.460 on on support of NPN over E1, Huawei, China Telecom, CR

- R3-194686, Way Forward with private networks - Summary of offline discussion,	Nokia, Nokia Shanghai Bell, discussion

- R3-194785, [Draft] Reply LS on RAN sharing and Emergency services with Non-Public Networks, Qualcomm Incorporated, LS out, Approved.

- R3-194786, LS on Clarification for private networks, Nokia, LS out, Approved.

RAN2#107b:

- R2-1912003, LS on sending CAG ID during resume procedure (C1-195157; contact: Samsung), CT1, LS in

- R2-1912035, Reply LS on RAN sharing and Emergency services with Non-Public Networks (R3-194785; contact: Qualcomm), RAN3, LS in

- R2-1912036, LS on clarifications on Private Networks (R3-194786; contact: Nokia), RAN3, LS in

- R2-1912059, LS on Sending CAG ID in NAS layer (S3-193142; contact: Qualcomm), SA3, LS in

- R2-1912153, Cell Selection and Reselection of NPN cell, CATT, discussion

- R2-1912154, Connection and Mobility Issue for UEs in Idle and Inactive Mode, CATT, discussion

- R2-1912309, Consideration on idle and inactive mode procedures, Huawei, HiSilicon, China Telecom, discussion

- R2-1912310, Consideration on connected mode procedures, Huawei, HiSilicon, discussion

- R2-1912360, Further aspects of SNPN, Ericsson, discussion

- R2-1912361, Further aspects of PNI-NPN, Ericsson, discussion

- R2-1912437, Discussion on (preventing) access to NPN only cell, Samsung Electronics Co., Ltd, discussion

- R2-1912507, Clarification on HRNN reporting, OPPO, discussion

- R2-1912508, Discussion on SIB1 design for NPN, OPPO, discussion

- R2-1912656, Network and cell (re)selection in SNPN access mode, Nokia, Nokia Shanghai Bell, discussion

- R2-1912657, RRC considerations to support SNPNs, Nokia, Nokia Shanghai Bell, discussion

- R2-1912675, Further consideration on the System Information of the Private Network, ZTE Corporation, Sanechips, discussion

- R2-1912676, Considerations on the remaining Idle/Inactive state issues of the Private Network, ZTE Corporation, Sanechips, discussion

- R2-1912713, Access Control for SNPN and CAG, Intel Corporation, discussion

- R2-1912714, Suitability criteria for NPN, Intel Corporation, discussion

- R2-1912841, NPN Work Plan, Nokia, China Telecom (Rapporteurs), discussion

- R2-1912842, NPN Stage 2 Draft CR, Nokia, China Telecom (Rapporteurs), draftCR

- R2-1912843, Report of Email Discussion [107#41] on CSG Aspects, Nokia (Rapporteur), report

- R2-1912924, Left issues in idle and inactive mode for NPN, CMCC, discussion

- R2-1912925, Left issues in active mode for NPN, CMCC, discussion

- R2-1912952, [DRAFT] LS to SA3 on privacy concern of transmission of CAG ID, CMCC, LS out

- R2-1913005, Discussion on reselection between NPNs and public networks, KDDI Corporation, discussion

- R2-1913052, Discussion on human-readable network name, China Telecom, Huawei, HiSilicon, discussion

- R2-1913058, Discussion on human-readable network name, China Telecom, Huawei, HiSilicon, discussion

- R2-1913060, Discussion on human-readable network name, China Telecom, Huawei, HiSilicon, discussion

- R2-1913069, Discussion on the deployment for CAG, China Telecommunications, discussion

- R2-1913070, Discussion on the deployment for CAG, China Telecom, Huawei, HiSilicon, discussion

- R2-1913363, Cell selection/reselection with NPN cells, Sony, discussion

- R2-1913633, Report of email discussion [107#40][NR/NPN] SIB1 design (Qualcomm), Qualcomm Incorporated, report

- R2-1913634, Suitable and acceptable NPN cells, Qualcomm Incorporated, discussion

- R2-1913635, Support for NPN HRNN broadcast, Qualcomm Incorporated, discussion

- R2-1913721, Discussion on providing CAG ID to AS during resume procedure, Huawei, HiSilicon, discussion

- R2-1913722, draft Reply LS to CTI on providing CAG ID to AS during resume procedure, Huawei, HiSilicon, LS out

- R2-1913804, Resue of CSG features for NPN cells, LG Electronics France, discussion

- R2-1913906, DISC_Resume_procedure_CAG_ID, Samsung Electronics Polska, discussion

- R2-1913946, Draft Reply LS on Sending CAG ID in NAS layer, Qualcomm Incorporated, LS out

- R2-1914178, LS on NID structure and length (C4-194332; contact: Ericsson), CT4, LS in

RAN3#105b:

- R3-194926	LS on Sending CAG ID in NAS layer	3GPP SA3	LS in	noted

- R3-195015	Access control for SNPN and CAG	Samsung	discussion	noted

- R3-195016	LS on NID and CAG ID reporting for mobility access control	Samsung	LS out	available

- R3-195037	Introduction of NPN on F1	Nokia, Nokia Shanghai Bell	CR	available

- R3-195039	Introduction of NPN on E1	Nokia, Nokia Shanghai Bell	CR	available

- R3-195174	Information on RAN2 BL CR	Nokia, Nokia Shanghai Bell, China Telecom	discussion	noted

- R3-195175	Introduction of Non Public Networks	Nokia, Nokia Shanghai Bell, China Telecom	draftCR	revised

- R3-195176	Organization of stage 3 work for NPN	Nokia, Nokia Shanghai Bell	discussion	noted

- R3-195177	Introduction of Non Public Networks	Nokia, Nokia Shanghai Bell	CR	noted

- R3-195178	Introduction of Non Public Networks	Nokia, Nokia Shanghai Bell	CR	available

- R3-195179	(TP for RAN3 BL CR 38.300) NPN Configuration aspects	Nokia, Nokia Shanghai Bell	other	revised

- R3-195180	(TP for NPN BL CR for 38.300) NPN Mobility aspects	Nokia, Nokia Shanghai Bell	other	available

- R3-195181	(TP for NPN BL CR for 38.300) NPN Paging aspects	Nokia, Nokia Shanghai Bell	other	available

- R3-195279	Discussion on open issue for configuration in NPN	CATT	discussion	noted

- R3-195396	CR to 38.413 for introducing PNI-NPN in Rel-16	China Telecommunication	CR	available

- R3-195397	CR to 38.413 for introducing SNPN in Rel-16	China Telecommunication	CR	available

- R3-195398	Discussion on support of Public network integrated NPN over NG interface	China Telecommunication	discussion	available

- R3-195399	Discussion on support of Standalone NPN over NG interface	China Telecommunication	discussion	available

- R3-195403	BL CR for introducing Non Public Netwrok in NG-RAN architecture	China Telecommunication, Nokia, Nokia Shanghai Bell, Huawei	CR	revised

- R3-195404	On introducing NPN in NG-RAN CU-DU architecture	China Telecommunication	discussion	noted

- R3-195606	Consideration on NPN paging	ZTE Corporation	discussion	available

- R3-195607	CR for TS38.413 on NPN paging	ZTE Corporation	CR	available

- R3-195608	CR for TS38.423 on NPN paging	ZTE Corporation	CR	available

- R3-195609	CR for TS38.473 on NPN paging	ZTE Corporation	CR	available

- R3-195611	pCR for TS38.300 on NPN paging	ZTE Corporation	other	available

- R3-195613	Impact of NPN on F1	ZTE Corporation	discussion	available

- R3-195614	CR for TS38.473 on NPN	ZTE Corporation	CR	available

- R3-195615	Impact of NPN on E1	ZTE Corporation	discussion	available

- R3-195616	CR for TS38.463 on NPN	ZTE Corporation	CR	available

- R3-195617	Consideration on Serving NPN expired in Connected mode	ZTE Corporation	discussion	available

- R3-195618	Draft LS to SA2 for NPN expired handling	ZTE Corporation	LS out	available

- R3-195698	CR for NGAP Handover Preparation to support PNI-NPN	ETRI	CR	available

- R3-195701	Discussion on NGAP HANDOVER REQUIRED message to support PNI-NPN	ETRI	discussion	available

- R3-195754	Introduction of NPN capturing agreements	Huawei	discussion	available

- R3-195755	CR to TS 38.413 on support of NPN	Huawei	CR	noted

- R3-195756	CR to TS 38.423 on support of NPN	Huawei	CR	available

- R3-195757	CR to TS 38.473 on support of NPN	Huawei, China Telecom	CR	available

- R3-195758	CR to TS 38.470 on support of  NPN	Huawei, China Telecom	CR	available

- R3-195759	CR to TS 38.463 on support of NPN	Huawei, China Telecom	CR	available

- R3-195760	CR to TS 38.460 on support of NPN	Huawei, China Telecom	CR	available

- R3-195761	Configuration aspects over NG	Huawei, LGU+	discussion	noted

- R3-195762	Mobility restriction list for NPN	Huawei	discussion	available

- R3-195763	Initial UE message for NPN	Huawei	discussion	available

- R3-195764	Initial UE Message for NPN	Huawei	CR	available

- R3-195765	[Draft] Reply LS on sending CAG ID in NAS layer	Huawei	LS out	available

- R3-195766	Connected mobility for NPN	Huawei	discussion	available

- R3-195767	Discussion on paging for NPN	Huawei	other	available

- R3-195768	Discussion on paging for NPN	Huawei	CR	available

- R3-195769	Discussion on paging for NPN	Huawei	CR	available

- R3-195770	Self-Configuration aspect for NPN	Huawei	discussion	available

- R3-195771	Remaining issues on support of NPN over F1	Huawei, China Telecom	discussion	available

- R3-195772	Remaining issues on support of NPN over F1	Huawei, China Telecom	CR	available

- R3-195773	Remaining issues on support of NPN over E1	Huawei, China Telecom	discussion	available

- R3-195774	Remaining issues on support of NPN over E1	Huawei, China Telecom	CR	available

- R3-195775	Support of MR-DC for NPN	Huawei	other	available

- R3-195776	Support of MR-DC for NPN	Huawei	CR	available

- R3-195777	Support of MR-DC for NPN	Huawei	draftCR	available

- R3-195778	Support of RAN sharing with NPN	Huawei	discussion	available

- R3-195779	Support of RAN sharing with NPN	Huawei	draftCR	available

- R3-195780	Support of RAN sharing with NPN	Huawei	draftCR	available

- R3-195781	Connection management between PLMN and SNPN	Huawei	discussion	available

- R3-195782	[Draft] LS on connection management between PLMN and SNPN	Huawei	LS out	available

- R3-195815	Discussion on configuration exchange over NG	Qualcomm Incorporated	discussion	noted

- R3-195816	Discussion on access control	Qualcomm Incorporated	discussion	available

- R3-195817	Discussion on open issues in mobility	Qualcomm Incorporated	discussion	available

- R3-195818	NPN Paging Aspects	Qualcomm Incorporated	discussion	available

- R3-195819	NPN Self-Configuration	Qualcomm Incorporated	discussion	available

- R3-195841	Mobility aspects for PNI NPN	CMCC	discussion	available

- R3-195842	Discussion of configuration aspects over NG	CMCC	discussion	noted

- R3-195875	NPN – Configuration Aspects over NG	Ericsson	discussion	noted

- R3-195876	Introducing NPN	Ericsson	other	available

- R3-195877	Introducing NPN	Ericsson	other	available

- R3-195878	Introducing NPN	Ericsson	other	available

- R3-195879	TPs for TSs 38.460 and 38.470 for support of NPN	Ericsson	other	available

- R3-195880	On Initial UE Message related aspects for NPN	Ericsson	discussion	available

- R3-195881	[Draft] Reply LS on Sending CAG ID in NAS layer	Ericsson	LS out	available

- R3-195882	NPN aspects for the Mobility Restriction List	Ericsson	discussion	available

- R3-195883	NPN related Mobility topics	Ericsson	discussion	available

- R3-195884	NPN topics related to Paging	Ericsson	discussion	available

- R3-195885	NPN topics related to Self Configuration	Ericsson	discussion	available

- R3-195886	NPN - F1 aspects	Ericsson	discussion	available

- R3-195887	Introducing NPN	Ericsson	other	available

- R3-195888	NPN - E1 aspects	Ericsson	discussion	available

- R3-195889	Introducing NPN	Ericsson	other	available

- R3-195890	NPN - Dual Connectivity Aspects	Ericsson	discussion	available

- R3-196089	Introduction of NPN in F1	LG Electronics	CR	available

- R3-196090	Introduction of NPN in E1	LG Electronics	CR	available

- R3-196151	Introduction of Non Public Networks	Nokia, Nokia Shanghai Bell, China Telecom	draftCR	endorsed (BL)

- R3-196152	BL CR for introducing Non Public Netwrok in NG-RAN architecture	China Telecommunication, Nokia, Nokia Shanghai Bell, Huawei	CR	endorsed (BL)

- R3-196153	Summary of offline on NPN NG Configuration Exchange	Qualcomm	discussion	noted

- R3-196173	(TP for RAN3 BL CR 38.300) NPN Configuration aspects	Nokia, Nokia Shanghai Bell	other	revised

- R3-196265	(TP for RAN3 BL CR 38.300) NPN Configuration aspects	Nokia, Nokia Shanghai Bell	other	agreed

RAN2#108:

- R2-1914326, LS on NID structure and length (C4-194332; contact: Ericsson), CT4, LS in

- R2-1914351, Reply LS on NID structure and length (S2-1910784; contact: Ericsson), SA2, LS in

- R2-1914355, Reply LS on clarifications on Private Networks (S2-1910803; contact: Ericsson), SA2, LS in

- R2-1914439, Open Issues for Cell Selection and Reselection of NPN cell, CATT, discussion

- R2-1914440, Discussion on Connected mode aspects for NPN, CATT, discussion

- R2-1914492, Discussion on reselection between NPNs and public networks, KDDI Corporation, discussion

- R2-1914493, Open Issues on NPN for UE in Inactive, CATT, discussion

- R2-1914598, NPN Work Plan, Nokia, China Telecom (Rapporteurs), discussion

- R2-1914599, NPN Stage 2 Draft CR, Nokia (Rapporteur), draftCR

- R2-1914600, LS on Emergency Services in SNPN, Nokia (Rapporteur), LS out

- R2-1914626, Cell Selection and Reselection aspects of NPN, Ericsson, discussion

- R2-1914627, Connected mode aspects of NPN, Ericsson, discussion

- R2-1914628, SIB1 design for NPN, Ericsson, discussion

- R2-1914732, Access Control for SNPN and CAG, Intel Corporation, discussion

- R2-1915089, Consideration on the PCI Range, ZTE Corporation, Sanechips, discussion

- R2-1915090, Overview of the Connected State Issues, ZTE Corporation, Sanechips, discussion

- R2-1915091, Consideration on the HRNN and Access control, ZTE Corporation, Sanechips, discussion

- R2-1915156, Clarification on HRNN reporting, OPPO, discussion

- R2-1915236, Cell selection/reselection with NPN cells, Sony, discussion

- R2-1915384, Network and cell (re)selection in SNPN access mode, Nokia, Nokia Shanghai Bell, discussion

- R2-1915385, RRC considerations to support SNPNs, Nokia, Nokia Shanghai Bell, discussion

- R2-1915386, Discussion on PCI list, Nokia, Nokia Shanghai Bell, discussion

- R2-1915387, Report from Email Discussion [107bis#63] on SIB1 design, Nokia (Rapporteur), discussion

- R2-1915388, Draft RRC CR for NPN, Nokia (Rapporteur), draftCR

- R2-1915739, Further considerations on mobility for CAG, Intel Corporation, discussion

- R2-1915743, RRC Connection establishment/resumption in SNPN and CAG, Intel Corporation, discussion

- R2-1915788, Report for email discussion [107bis#64][PRN] Suitable and acceptable NPN cells (Qualcomm), Qualcomm Incorporated, report

- R2-1915800, Running 38.304 CR for PRN, Qualcomm Incorporated, draftCR

- R2-1915804, Draft Reply LS on Sending CAG ID in NAS layer, Qualcomm Incorporated, LS out

- R2-1915805, Support for NPN HRNN broadcast, Qualcomm Incorporated, discussion

- R2-1915807, Views on cell reselection for CAGs, Qualcomm Incorporated, discussion

- R2-1915957, General considerations on idle and inactive mode for NPN, Huawei, HiSilicon, discussion

- R2-1915958, General considerations on connected mode for NPN, Huawei, HiSilicon, discussion

- R2-1915959, Considerations on SI Validity Checking, Huawei, HiSilicon, discussion

- R2-1915989, Cell selection and reselection for NPN, CMCC, discussion

- R2-1915990, Access and mobility control for NPN, CMCC, discussion

- R2-1915991, CAG ID transmission during both initial access and resume, CMCC, discussion

- R2-1915992, Draft Reply LS on CAG ID Transmission, CMCC, LS out

- R2-1916003, Discussion on human-readable network name, China Telecommunications, discussion

- R2-1916048, Discussion on human-readable network name, China Telecom, Huawei, HiSilicon, discussion

- R2-1916063, Discussion on the deployment for CAG, China Telecom, Huawei, HiSilicon, discussion

- R2-1916097, Discussion on Network identifier for SNPN, vivo, discussion

- R2-1916098, Discussion on the proximity indication in CONNECTED mode, vivo, discussion

- R2-1916170, Access control for PRN, LG Electronics France, discussion

- R2-1916225, Cell Reselection remaining issues, Samsung R&D Institute India, discussion

- R2-1916336, [DRAFT] Reply LS on NID structure and length, Ericsson, LS out

- R2-1916337, NPN Stage 2 Draft CR, draftCR

- R2-1916341, [Draft] CMAS/ETWS and emergency services for SNPNs, LS out

- R2-1916342, Report of Offline Discussion 211 (CATT), discussion

- R2-1916344, Reply LS on NID structure and length, RAN2, LS out

- R2-1916345, CMAS/ETWS and emergency services for SNPNs, RAN2, LS out

- R2-1916346, Draft Reply LS on Sending CAG ID in NAS layer, LS out

- R2-1916349, Reply LS on Sending CAG ID in NAS layer, RAN2, LS out

- R2-1916520, Reply LS on Sending CAG ID in NAS layer (R3-197591; contact: Ericsson), RAN3, LS in

RAN3#106:

- R3-196427	Reply LS on clarifications on Private Networks	3GPP SA2, Ericsson	LS in	noted

- R3-196436	LS on NID structure and length	3GPP CT4, Ericsson	LS in	noted

- R3-196514	Introduction of Non Public Networks	Nokia, Nokia Shanghai Bell, China Telecom	draftCR	endorsed (BL)

- R3-196547	Information on RAN2 BL CR	Nokia, Nokia Shanghai Bell	discussion	noted

- R3-196548	(TP for NPN BL CR 38.423) Configuration Aspects of NPN	Nokia, Nokia Shanghai Bell	other	noted

- R3-196549	(TP for NPN BL CR 38.300) Access Control Aspects on NPN	Nokia, Nokia Shanghai Bell	other	revised

- R3-196550	(TP for NPN BL CR for 38.413) Access control aspects of NPN	Nokia, Nokia Shanghai Bell	other	revised

- R3-196551	(TP for NPN BL CR for 38.423) Mobility Restriciton List for NPN	Nokia, Nokia Shanghai Bell	other	revised

- R3-196552	(TP for NPN BL CR for 38.300) NPN Mobility aspects	Nokia, Nokia Shanghai Bell	other	revised

- R3-196553	(TP for NPN BL CR for 38.413) NPN Mobility aspects	Nokia, Nokia Shanghai Bell	other	revised

- R3-196554	(TP for NPN BL CR for 38.300) NPN Paging aspects	Nokia, Nokia Shanghai Bell	other	revised

- R3-196555	(TP for NPN BL CR for 38.413) NPN Paging aspects	Nokia, Nokia Shanghai Bell	other	merged

- R3-196556	(TP for NPN BL CR for 38.423) NPN Paging aspects	Nokia, Nokia Shanghai Bell	other	available

- R3-196557	(TP for NPN BL CR for 38.473) NPN Paging aspects	Nokia, Nokia Shanghai Bell	other	noted

- R3-196558	(TP for NPN BL CR for 38.300) Dual Connectivity aspects of NPN	Nokia, Nokia Shanghai Bell	other	available

- R3-196621	Discussion on NPN for F1	Nokia, Nokia Shanghai Bell	discussion	available

- R3-196622	Introduction of NPN on F1	Nokia, Nokia Shanghai Bell	CR	available

- R3-196623	Discussion on NPN for E1	Nokia, Nokia Shanghai Bell	discussion	available

- R3-196625	Introduction of NPN on E1	Nokia, Nokia Shanghai Bell	CR	available

- R3-196701	(TP for NPN BL CR for 38.413) RAN sharing aspects of NPN	Nokia, Nokia Shanghai Bell	other	available

- R3-196803	Introduction of Non-Public Networks	Qualcomm Incorporated	CR	revised

- R3-196804	Passing RAN’s CAG configuration to the AMF	Qualcomm Incorporated	discussion	available

- R3-196805	Discussion on access control	Qualcomm Incorporated	discussion	available

- R3-196806	Discussion on open issues in mobility	Qualcomm Incorporated	discussion	available

- R3-196807	NPN Paging Aspects	Qualcomm Incorporated	discussion	available

- R3-196808	NPN Self-Configuration	Qualcomm Incorporated	discussion	available

- R3-196824	Discussion on initial access procedure in NPN	LG Electronics	discussion	available

- R3-196858	BL CR NPN for 38.401	China Telecommunication	CR	endorsed (BL)

- R3-196859	TP for NPN BL CR for 38.401	China Telecommunication, Nokia, Nokia Shanghai Bell, Huawei	other	noted

- R3-196860	TP for Configuration Aspects over NG interface	China Telecommunication	other	available

- R3-196861	TP for initial UE message for NPN over NG interface	China Telecommunication	other	available

- R3-196862	Discussion on mobility of NPN	China Telecommunication	discussion	available

- R3-196863	TP for NPN paging	China Telecommunication	other	available

- R3-196946	UE Allowed CSG List at the NG-RAN node	NEC	discussion	noted

- R3-197000	Consideration on NPN subscription expiration handling in Connected mode	ZTE Corporation	discussion	available

- R3-197010	Draft LS to SA2 for NPN subscription expiration handling	ZTE Corporation	LS out	reserved

- R3-197011	Draft LS to SA2 for NPN subscription expiration handling	ZTE Corporation	LS out	available

- R3-197012	Consideration on NPN paging	ZTE Corporation	discussion	reserved

- R3-197013	Consideration on NPN paging	ZTE Corporation	discussion	reserved

- R3-197014	Consideration on NPN paging	ZTE Corporation	discussion	available

- R3-197040	CR for TS38.413 on NPN paging	ZTE Corporation	CR	merged

- R3-197041	CR for TS38.423 on NPN paging	ZTE Corporation	CR	available

- R3-197042	CR for TS38.473 on NPN paging	ZTE Corporation	CR	noted

- R3-197043	Impact of NPN on F1	ZTE Corporation	discussion	available

- R3-197044	Introduction of NPN (to be BL CR)	Huawei, China Telecom	CR	revised

- R3-197048	(TP for NPN BL CR for TS 38.300): Configuration aspects over NG	Huawei	other	noted

- R3-197049	(TP for NPN BL CR for TS 38.413): Initial UE message for NPN	Huawei	other	available

- R3-197050	[Draft] Reply LS on sending CAG ID in NAS layer	Huawei	LS out	available

- R3-197051	(TP for NPN BL CR for TS 38.413): Mobility restriction list	Huawei	other	revised

- R3-197052	(TP for NPN BL CR for TS 38.300): Connected mobility for NPN	Huawei	other	noted

- R3-197053	(TP for NPN BL CR for TS 38.413): Paging for NPN	Huawei, CMCC, Samsung	other	merged

- R3-197054	(TP for NPN BL CR for TS 38.423): Paging for NPN	Huawei, CMCC, Samsung	other	available

- R3-197055	(TP for NPN BL CR for TS 38.300): Paging for NPN	Huawei, CMCC, Samsung	other	merged

- R3-197056	(TP for NPN BL CR for TS 38.423): Self-configuration aspects for NPN	Huawei	other	available

- R3-197057	(TP for NPN BL CR for TS 38.473): Further support of NPN over F1	Huawei, China Telecom	other	available

- R3-197058	CR to TS 38.470 on support of  NPN	Huawei, China Telecom	CR	available

- R3-197059	(TP for NPN BL CR for TS 38.463): Further support of NPN over E1	Huawei, China Telecom	other	available

- R3-197060	CR to TS 38.460 on support of NPN	Huawei, China Telecom	CR	available

- R3-197061	(TP for NPN BL CR for TS 38.423): Support of MR-DC for NPN	Huawei	other	available

- R3-197062	Support of MR-DC for NPN	Huawei	draftCR	available

- R3-197063	(TP for NPN BL CR for TS 38.300): Support of RAN sharing with NPN	Huawei	other	available

- R3-197064	Support of RAN sharing with NPN	Huawei	draftCR	available

- R3-197065	Connection management between PLMN and SNPN	Huawei	discussion	available

- R3-197066	CR for TS38.473 on NPN	ZTE Corporation	CR	available

- R3-197067	Impact of NPN on E1	ZTE Corporation	discussion	available

- R3-197068	BLCR for TS38.463 on NPN	ZTE Corporation	CR	revised

- R3-197070	CR for TS38.463 on NPN	ZTE Corporation	CR	available

- R3-197080	(TP for NPN BL CR for TS 38.413) NPN NG Handover aspects	ETRI	other	merged

- R3-197112	Updated work plan for PRN WI	China Telecommunications	Work Plan	noted

- R3-197153	Discussion on open issue for access control in NG-RAN	CATT	discussion	revised

- R3-197154	(TP for NPN Stage2 BL CR) Support for initiall access control in Non-Public Networks	CATT	other	noted

- R3-197155	(TP for NPN NGAP BL CR)support of Iniitial access control in Non-Public Networks	CATT	other	available

- R3-197255	LS on RAN sharing model for Stand-alone NPN	Nokia, Nokia Shanghai Bell	LS out	available

- R3-197270	Mobility aspects for PNI NPN	CMCC	discussion	available

- R3-197271	Left issues on configuration aspects for PNI NPN	CMCC	discussion	available

- R3-197353	NPN – Configuration Aspects over NG	Ericsson	discussion	noted

- R3-197354	Introducing NPN	Ericsson	CR	available

- R3-197355	On Initial UE Message related aspects for NPN	Ericsson	discussion	available

- R3-197356	[Draft] Reply LS on Sending CAG ID in NAS layer	Ericsson	LS out	revised

- R3-197357	NPN aspects for the Mobility Restriction List	Ericsson	discussion	available

- R3-197358	NPN related Mobility topics	Ericsson	discussion	available

- R3-197359	NPN topics related to Paging	Ericsson	discussion	merged

- R3-197360	NPN topics related to Self Configuration	Ericsson	discussion	available

- R3-197361	NPN – F1 Aspects	Ericsson	discussion	available

- R3-197362	NPN – E1 Aspects	Ericsson	discussion	available

- R3-197363	NPN – Dual Connectivity Aspects	Ericsson	discussion	available

- R3-197364	NPN-RAN Sharing Aspects	Ericsson	discussion	available

- R3-197461	Reply LS on NID structure and length	3GPP SA2, Ericsson	LS in	noted

- R3-197517	Discussion on open issue for access control in NG-RAN	CATT	discussion	available

- R3-197584	Introduction of Non-Public Networks	Qualcomm Incorporated	CR	endorsed (BL)

- R3-197585	Introduction of NPN (to be BL CR)	Huawei, China Telecom	CR	endorsed (BL)

- R3-197586	BLCR for TS38.463 on NPN	ZTE Corporation	CR	endorsed (BL)

- R3-197587	TP on configuration exchange for baseline CR on NPN/NGAP (38.413 CR#0290)	Qualcomm	other	agreed

- R3-197588	[TP for XnAP BL CR] TP for Xn Setup/Configuration Update	Ericsson	other	agreed

- R3-197589	(TP for NPN BL CR 38.300) Access Control Aspects on NPN	Nokia, Nokia Shanghai Bell	other	agreed

- R3-197590	(TP for NPN BL CR for 38.413) Access control aspects of NPN	Nokia, Nokia Shanghai Bell	other	revised

- R3-197591	Reply LS on Sending CAG ID in NAS layer	Ericsson	LS out	approved

- R3-197592	(TP for NPN BL CR for TS 38.413): Mobility restriction list	Huawei	other	agreed

- R3-197593	(TP for NPN BL CR for 38.423) Mobility Restriciton List for NPN	Nokia, Nokia Shanghai Bell	other	agreed

- R3-197594	(TP for NPN BL CR for 38.300) NPN Mobility aspects	Nokia, Nokia Shanghai Bell	other	revised

- R3-197595	(TP for NPN BL CR for 38.413) NPN Mobility aspects	Nokia, Nokia Shanghai Bell	other	agreed

- R3-197596	TP to NGAP BL CR on paging	Qualcomm	other	agreed

- R3-197597	TP to XNAP BL CR on paging	Ericsson	other	agreed

- R3-197598	(TP for NPN BL CR for 38.300) NPN Paging aspects	Nokia, Nokia Shanghai Bell	other	agreed

- R3-197667	Summary of offlines on NPN	Nokia, Nokia Shanghai Bell	discussion	noted

- R3-197776	(TP for NPN BL CR for 38.413) Access control aspects of NPN	Nokia, Nokia Shanghai Bell	other	agreed

- R3-197777	(TP for NPN BL CR for 38.300) NPN Mobility aspects	Nokia, Nokia Shanghai Bell	other	agreed

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template