3GPP TSG-RAN WG1 Meeting #100-e	R1-2001224

# e-Meeting, February 24th- March 6th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1300 | rev |  | Current version: | 14.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on priority value in LTE V2X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sharp |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_V2X-Core |  |  |  |  |  | Date: |  |  | 2020-03-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - For LTE V2X mode 4 sensing, PSSCH-RSRP measurement threshold is the i-th field in the threshold list, the parameter i is larger than 64 and out of indexed if the priority to be transmitted indicated by higher layers is 8 based on current calculation of i in Section 14.1.1.6. - It is not clear how the UE sets the Priority field according to the priority(s) indicated by higher layers. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change i=a×8+b+1 as i=(a-1)×8+b. - Clarify that “Priority field ‘000’ in SCI format 1 corresponds to priority ‘1’, priority field ‘001’ in SCI format 1 corresponds to priority ‘2’, and so on.”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE is not able to determine the PSSCH RSRP threshold in mode 4 sensing when the priority to be transmitted indicated by the higher layers is 8. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.1.1.6, 14.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 14.1.1.6	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink transmission mode 4

When requested by higher layers in subframe n, the UE shall determine the set of resources to be reported to higher layers for PSSCH transmission according to the following steps. Parameters  the number of sub-channels to be used for the PSSCH transmission in a subframe,  the resource reservation interval, and  the priority to be transmitted in the associated SCI format 1 by the UE are all provided by higher layers.  is determined according to Subclause 14.1.1.4B.

If partial sensing is not configured by higher layers then the following steps are used:

1)	A candidate single-subframe resource for PSSCH transmission  is defined as a set of  contiguous sub-channels with sub-channel x+j in subframe  where . The UE shall assume that any set of  contiguous sub-channels included in the corresponding PSSCH resource pool (described in 14.1.5) within the time interval  corresponds to one candidate single-subframe resource, where selections of  and  are up to UE implementations under  and . UE selection of  shall fulfil the latency requirement. The total number of the candidate single-subframe resources is denoted by.

2)	The UE shall monitor subframes ,, …,  except for those in which its transmissions occur, where  if subframe n belongs to the set , otherwise subframe is the first subframe after subframe n belonging to the set . The UE shall perform the behaviour in the following steps based on PSCCH decoded and S-RSSI measured in these subframes.

3)	The parameter  is set to the value indicated by the i-th SL-ThresPSSCH-RSRP field in SL-ThresPSSCH-RSRP-List where i=a(a-1)*8+b+1.

4)	The set  is initialized to the union of all the candidate single-subframe resources. The set  is initialized to an empty set.

5)	The UE shall exclude any candidate single-subframe resource  from the set  if it meets all the following conditions:

-	the UE has not monitored subframe  in Step 2.

-	there is an integer j which meets  where j=0, 1, …, , , k is any value allowed by the higher layer parameter restrictResourceReservationPeriod and q=1,2,…,Q. Here,  if  and , where  if subframe n belongs to the set , otherwise subframe  is the first subframe belonging to the set  after subframe n; and  otherwise.

6)	The UE shall exclude any candidate single-subframe resource  from the set  if it meets all the following conditions:

-	the UE receives an SCI format 1 in subframe , and "Resource reservation" field and "Priority" field in the received SCI format 1 indicate the values  and , respectively according to Subclause 14.2.1.

-	PSSCH-RSRP measurement according to the received SCI format 1 is higher than .

-	the SCI format received in subframe or the same SCI format 1 which is assumed to be received in subframe(s)  determines according to 14.1.1.4C the set of resource blocks and subframes which overlaps with  for q=1, 2, …, Q and j=0, 1, …, . Here, if  and , where  if subframe n belongs to the set , otherwise subframe is the first subframe after subframe n belonging to the set ; otherwise .

7)	If the number of candidate single-subframe resources remaining in the set  is smaller than , then Step 4 is repeated with  increased by 3 dB.

8)	For a candidate single-subframe resource  remaining in the set, the metric  is defined as the linear average of S-RSSI measured in sub-channels x+k for  in the monitored subframes in Step 2 that can be expressed by  for a non-negative integer j if , and  for a non-negative integer j otherwise.

9)	The UE moves the candidate single-subframe resource  with the smallest metric  from the set  to . This step is repeated until the number of candidate single-subframe resources in the set  becomes greater than or equal to ,

The UE shall report set  to higher layers.

If partial sensing is configured by higher layers then the following steps are used:

1)	A candidate single-subframe resource for PSSCH transmission  is defined as a set of  contiguous sub-channels with sub-channel x+j in subframe  where . The UE shall determine by its implementation a set of subframes which consists of at least  subframes within the time interval  where selections of  and  are up to UE implementations under  and . UE selection of  shall fulfil the latency requirement and  shall be greater than or equal to the high layer parameter minNumCandidateSF. The UE shall assume that any set of  contiguous sub-channels included in the corresponding PSSCH resource pool (described in 14.1.5) within the determined set of subframes correspond to one candidate single-subframe resource. The total number of the candidate single-subframe resources is denoted by.

2)	If a subframe  is included in the set of subframes in Step 1, the UE shall monitor any subframe  if k-th bit of the high layer parameter gapCandidateSensing is set to 1. The UE shall perform the behaviour in the following steps based on PSCCH decoded and S-RSSI measured in these subframes.

3)	The parameter  is set to the value indicated by the i-th SL-ThresPSSCH-RSRP field in SL-ThresPSSCH-RSRP-List where i=a(a-1)*8+b+1.

4)	The set  is initialized to the union of all the candidate single-subframe resources. The set  is initialized to an empty set.

5)	The UE shall exclude any candidate single-subframe resource  from the set  if it meets all the following conditions:

-	the UE receives an SCI format 1 in subframe , and "Resource reservation" field and "Priority" field in the received SCI format 1 indicate the values  and , respectively according to Subclause 14.2.1.

-	PSSCH-RSRP measurement according to the received SCI format 1 is higher than .

-	the SCI format received in subframe or the same SCI format 1 which is assumed to be received in subframe(s)  determines according to 14.1.1.4C the set of resource blocks and subframes which overlaps with  for q=1, 2, …, Q and j=0, 1, …, . Here, if  and , where  is the last subframe of the  subframes , and  otherwise.

6)	If the number of candidate single-subframe resources remaining in the set  is smaller than , then Step 4 is repeated with  increased by 3 dB.

7)	For a candidate single-subframe resource  remaining in the set, the metric  is defined as the linear average of S-RSSI measured in sub-channels x+k for  in the monitored subframes in Step 2 that can be expressed by  for a non-negative integer j.

8)	The UE moves the candidate single-subframe resource  with the smallest metric  from the set  to . This step is repeated until the number of candidate single-subframe resources in the set  becomes greater than or equal to .

The UE shall report set  to higher layers.

#### 14.2.1	UE procedure for transmitting the PSCCH

For sidelink transmission mode 1and PSCCH period i,

-	the UE shall determine the subframes and resource blocks for transmitting SCI format 0 as follows.

-	SCI format 0 is transmitted in two subframes in the subframe pool and one physical resource block per slot in each of the two subframes, wherein the physical resource blocks belong to the resource block pool, where the subframe pool and the resource block pool are indicated by the PSCCH resource configuration (as defined in Subclause 14.2.3)

-	the two subframes and the resource blocks are determined using "Resource for PSCCH" field () in the configured sidelink grant (described in [8]) as described in Subclause 14.2.1.1.

-	the UE shall set the contents of the SCI format 0 as follows:

-	the UE shall set the Modulation and coding scheme field according to the Modulation and coding scheme indicated by the higher layer parameter mcs-r12 if the parameter is configured by higher layers.

-	the UE shall set the Frequency hopping flag according to the "Frequency hopping flag" field in the configured sidelink grant.

-	the UE shall set the Resource block assignment and hopping resource allocation according to the "Resource block assignment and hopping resource allocation" field in the configured sidelink grant.

-	the UE shall set the Time resource pattern according to the "Time resource pattern" field in the configured sidelink grant .

-	the UE shall set the eleven-bit Timing advance indication to  to indicate sidelink reception timing adjustment value using the NTA (defined in [3]) value for the UE in the subframe that is no earlier than subframe  (described in Subclause 14.2.1.1).

For sidelink transmission mode 2,

-	SCI format 0 is transmitted in two subframes in the subframe pool and one physical resource block per slot in each of the two subframes, wherein the physical resource blocks belongs to the resource block pool, where the subframe pool and the resource block pool are indicated by the PSCCH resource configuration (as defined in Subclause 14.2.3)

-	the two subframes and the resource blocks are determined using the procedure described in Subclause 14.2.1.2

-	the UE shall set the eleven-bit Timing advance indication  in the SCI format 0 to zero.

For sidelink transmission mode 3,

-	The UE shall determine the subframes and resource blocks for transmitting SCI format 1 as follows:

-	SCI format 1 is transmitted in two physical resource blocks per slot in each subframe where the corresponding PSSCH is transmitted.

-	If the UE receives in subframe n DCI format 5A with the CRC scrambled by the SL-V-RNTI, one transmission of PSCCH is in the PSCCH resource  (described in Subclause 14.2.4) in the first subframe tqSL that is included in  and that starts not earlier than TDL-NTA2×TS+4+m×10-3.  is the value indicated by "Lowest index of the sub-channel allocation to the initial transmission" associated with the configured sidelink grant (described in [8]) if the field “Lowest index of the sub-channel allocation to the initial transmission” in the corresponding DCI format 5A is present and LInit=0 otherwise,  is determined by Subclause 14.1.5, the value m is indicated by 'SL index' field in the corresponding DCI format 5A according to Table 14.2.1-1 if this field is present and m=0 otherwise, TDL is the start of the downlink subframe carrying the DCI, and NTA and TS are described in [3].

-	If "Time gap between initial transmission and retransmission" in the configured sidelink grant (described in [8]) is not equal to zero, another transmission of PSCCH is in the PSCCH resource  in subframe , where  is the value indicated by "Time gap between initial transmission and retransmission" field in the configured sidelink grant.  corresponds to the value  determined by the procedure in Subclause 14.1.1.4C with the RIV set to the value indicated by "Frequency resource location of the initial transmission and retransmission" field in the configured sidelink grant.

-	If the UE receives in subframe n DCI format 5A with the CRC scrambled by the SL-SPS-V-RNTI , the UE shall consider the received DCI information as a valid sidelink semi-persistent activation or release only for the SPS configuration indicated by the SL SPS configuration index field. If the received DCI activates an SL SPS configuration, one transmission of PSCCH is in the PSCCH resource  (described in Subclause 14.2.4) in the first subframe tqSL  that is included in  and that starts not earlier than TDL-NTA2×TS+4+m×10-3.  is the value indicated by "Lowest index of the sub-channel allocation to the initial transmission" associated with the configured sidelink grant (described in [8]) if the field “Lowest index of the sub-channel allocation to the initial transmission” in the corresponding DCI format 5A is present and LInit=0 otherwise,  is determined by Subclause 14.1.5, the value m is indicated by 'SL index' field in the corresponding DCI format 5A according to Table 14.2.1-1 if this field is present and m=0 otherwise, TDL is the start of the downlink subframe carrying the DCI, and NTA and TS are described in [3]..

-	If "Time gap between initial transmission and retransmission" in the configured sidelink grant (described in [8]) is not equal to zero, another transmission of PSCCH is in the PSCCH resource  in subframe , where  is the value indicated by "Time gap between initial transmission and retransmission" field in the configured sidelink grant.  corresponds to the value  determined by the procedure in Subclause 14.1.1.4C with the RIV set to the value indicated by "Frequency resource location of the initial transmission and retransmission" field in the configured sidelink grant.

-	The UE shall set the contents of the SCI format 1 as follows:

-	the UE shall set the Modulation and coding scheme as indicated by higher layers.

-	the UE shall set the "Priority" field according to the highest priority among those priority(s) indicated by higher layers corresponding to the transport block. Priority field ‘000’ corresponds to priority ‘1’, priority field ‘001’ corresponds to priority ‘2’, and so on.

-	the UE shall set the Time gap between initial transmission and retransmission field, the Frequency resource location of the initial transmission and retransmission field, and the Retransmission index field such that the set of time and frequency resources determined for PSSCH according to Subclause 14.1.1.4C is in accordance with the PSSCH resource allocation indicated by the configured sidelink grant.

-	the UE shall set the Resource reservation to zero.

-	Each transmission of SCI format 1 is transmitted in one subframe and two physical resource blocks per slot of the subframe.

-	The UE shall randomly select the cyclic shift  among {0, 3, 6, 9} in each PSCCH transmission.

For sidelink transmission mode 4,

-	The UE shall determine the subframes and resource blocks for transmitting SCI format 1 as follows:

-	SCI format 1 is transmitted in two physical resource blocks per slot in each subframe where the corresponding PSSCH is transmitted.

-	If the configured sidelink grant from higher layer indicates the PSCCH resource in subframe , one transmission of PSCCH is in the indicated PSCCH resource m (described in Subclause 14.2.4) in subframe .

-	If "Time gap between initial transmission and retransmission" in the configured sidelink grant (described in [8]) is not equal to zero, another transmission of PSCCH is in the PSCCH resource  in subframe  where  is the value indicated by "Time gap between initial transmission and retransmission" field in the configured sidelink grant,  corresponds to the value  determined by the procedure in Subclause 14.1.1.4C with the RIV set to the value indicated by "Frequency resource location of the initial transmission and retransmission" field in the configured sidelink grant.

-	the UE shall set the contents of the SCI format 1 as follows:

-	the UE shall set the Modulation and coding scheme as indicated by higher layers.

-	the UE shall set the "Priority" field according to the highest priority among those priority(s) indicated by higher layers corresponding to the transport block. Priority field ‘000’ corresponds to priority ‘1’, priority field ‘001’ corresponds to priority ‘2’, and so on.

-	the UE shall set the Time gap between initial transmission and retransmission field, the Frequency resource location of the initial transmission and retransmission field, and the Retransmission index field such that the set of time and frequency resources determined for PSSCH according to Subclause 14.1.1.4C is in accordance with the PSSCH resource allocation indicated by the configured sidelink grant.

-	the UE shall set the Resource reservation field according to table 14.2.1-2 based on indicated value X, where X is equal to the Resource reservation interval provided by higher layers divided by 100.

-	Each transmission of SCI format 1 is transmitted in one subframe and two physical resource blocks per slot of the subframe.

-	The UE shall randomly select the cyclic shift  among {0, 3, 6, 9} in each PSCCH transmission.

Table 14.2.1-1: Mapping of DCI format 5A offset field to indicated value m

| SL index field in DCI format 5A | Indicated value m |
| --- | --- |
| '00' | 0 |
| '01' | 1 |
| '10' | 2 |
| '11' | 3 |

Table 14.2.1-2: Determination of the Resource reservation field in SCI format 1

| Resource reservation field in SCI format 1 | Indicated value X | Condition |
| --- | --- | --- |
| '0001', '0010', …, '1010' | Decimal equivalent of the field | The higher layer decides to keep the resource for the transmission of the next transport block and the value X meets . |
| '1011' | 0.5 | The higher layer decides to keep the resource for the transmission of the next transport block and the value X is 0.5. |
| '1100' | 0.2 | The higher layer decides to keep the resource for the transmission of the next transport block and the value X is 0.2. |
| '0000' | 0 | The higher layer decides not to keep the resource for the transmission of the next transport block. |
| '1101', '1110', '1111' | Reserved |  |

#### 14.2.1.1	UE procedure for determining subframes and resource blocks for transmitting PSCCH for sidelink transmission mode 1

### < Unchanged parts are omitted >