3GPP TSG RAN meeting #87e									RP-200237

e-Meeting,   16th – 19th  March, 2020

Status Report to TSG

Agenda item:		9.3.6

| WI / SI Name | UE Power Saving in NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_UE_pow_sav_CORE |  |  |  |  |
| Unique ID | 830175 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-191607 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 06/2020 | Performance part: 12/2020 | Testing part: NA |  |
| Overall Completion level | Study Item: | Core part: 85% | Performance Part: 0% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Fang-Chen Cheng |
|  | Company | CATT |
|  | Email | fcc@catt.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

    - Agreements

RAN1 completed

    - Remaining Open issues

none

2.2	RAN2

2.2.1	Agreements

Agreements made in RAN2#109e meeting (24th  February– 6th  March 2020):

PDCCH-based power saving signals/channel

Agreements - RAN2 to confirm the MAC-PHY modelling in the MAC running CR for DCP Send LS to RAN1 to inform the RAN2 understanding - RAN2 does not expect to discuss partial overlapping for DCP monitoring and will following RAN1 CR on this issue - No special handling for DCP monitoring in case DCP is overlapped with HARQ-RTT-timer, i.e. DCP needs to be monitored - ps-TransmitPeriodicCSI-r16 and ps-TransmitPeriodicL1-RSRP-r16 can be configured independently - FFS ps-TransmitPeriodicCSI-r16 covers periodic CSI reporting apart from L1-RSRP related CSI (i.e. cri-RSRP and ssb-Index-RSRP) - ask RAN1 what the intention in the LS R2-2002201

Agreements - The issue of capturing CSI reporting when the drx-onDurationTimer is not started due to DCP indication, but the MAC entity is in Active Time during on-duration due to other reasons will be addressed in MAC specification. - The TP in R2-2000254 is used to capture the solution to agreement 1 in MAC. - No ambiguity period is needed when considering DCP for on-duration determination. - The below TP is used to capture agreement 3 in MAC. 1> if DCP is configured for the active DL BWP: 2> in current symbol n, if the symbol occurs within drx-onDurationTimer duration and drx-onDurationTimer would not be running considering DCP occurrence(s) associated with the current DRX cycle until [x] ms prior to symbol n as specified in this clause: 3> not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7]; 3> not report semi-persistent CSI; 3> if ps-Periodic_CSI_Transmit is not configured with value true: ## 4> not report periodic CSI on PUCCH.. - Since DCP is only monitored outside Active Time, there is a 4-ms ambiguity period associated with the DCP monitoring, to be captured in MAC specification. Further agreements from RAN1 will be taken into account in the next meeting. - The TP in R2-2001037 is used as a baseline to capture Proposal 12 in MAC. - If DCP only applies when UE is in Long DRX, no change is needed to the current 38.321 CR to capture this behaviour. FFS whether DCP applies to short DRX - The UE behavior when it receives DCP regarding the monitoring of following DCP occasions is not addressed in RAN2. No change is needed to the current 38.321 CR for this issue, if any. - No change to the current specification is required to support notification of SI/PWS change when DCP is configured. - No mechanism for periodical wake up and/or always wake up in poor radio condition is specified to address DCP miss-detection - Given the split views in this session, the discussion on ASN.1 options for capturing the search space for the DCP is moved to the RRC/ASN.1 review. - No change to the specifications is required to address any potential DCP miss during handover. - The issue of network not being able to perform beam management actions when WUS has not indicated UE to wake-up but UE has transmitted CSI/SRS requires no change to current specifications. - The issue of the coexistence of DRX groups and Power Saving features will be addressed together with the main discussion on DRX groups support, following RAN1 reply LS. - Configuring the UE to report CSI/SRS in sparse mode, i.e. report once per N DRX cycles is not supported in the specifications. - FFS what the UE actually monitors if it misses DCP when configured with SCell dormancy - FFS UE behavior when a DCP occasion occurs during RAR window will be decided at the next meeting

UE assistance

Agreements 1. The UE assistance IE to transition out of connected mode is named ‘releasePreference’ 2. The UE assistance configuration for power savings is released during the RRC resume procedure. 3. A UE can report a preference of 0MHz aggregated bandwidth for power savings. 4. The reported long DRX-cycle preference is a multiple of the reported short DRX-cycle preference 5. The search space for DCP can be configured independent of search spaces for Rel-15 DCIs 6. The search space for DCP can be configured such that it is also used to monitor other Rel-15 DCIs. FFS the details of ASN.1 7. The prohibit timer for UE assistance on DRX, aggregated bandwidth, number of cell, number of MIMO layers, releasePreference and minimum scheduling offset for power savings can be configured up to 30s. 8. All fields in the DRX-Preference IE in the UE assistance information message are optional fields. FFS what it means when the UE omits the value and what it means 9. All fields in the MinSchedulingOffsetPreference IE in the UE assistance information message are optional fields. FFS what it means when the UE omits the values and what it means 10. If a UE wants to cancel an earlier indicated preference to leave connected mode, the UE can transmit a release preference IE with a connected mode state preference, when not prevented by a prohibit timer 11. A UE can report a preference of one layer as the minimum number of preferred DL MIMO layers for the downlink. 12. The reported values of UE assistance on reduced bandwidth, cells and MIMO layers for power savings can range up to at least the corresponding value in the current active configuration. FFS if it can be up to UE capability 13. A UE can report a preferred aggregated bandwidth for a frequency range on the configured serving cell. FFS if it is allowed even if it is not configured with serving cells on that frequency range 14.

Agreements

- In MR-DC with NR SN, support SCG specific UAI for power saving, which includes drx-Preference, maxBW-Preference, maxCC-Preference, maxMIMO-LayerPreference, and minSchedulingOffsetPreference.

- UE transmits SCG specific UAI for power saving in a transparent container to the MN and the MN then forwards the received container to the NR SN.   FFS if UAI can also be reported for power saving directly via SRB3 if configured.  FFS on the signalling details.

- UE implicitly can indicate a preference for NR SCG release by indicating zero number of carriers or zero aggregated maximum bandwidth in both FR1 and FR2.

- UE assistance for NR SCG setup is not supported in Rel-16.

- Indication of “Connected” for cancelling a previous release preference is subject to prohibit timer.

- Preferred carrier grouping for SCell dormancy is not supported in Rel-16

RRM measurement relaxation

Agreements - The network broadcasts corresponding parameters of relaxation triggering criteria to enable RRM measurement relaxation feature. - When network configures the parameters of both low mobility and not-at-cell-edge criteria. UE can perform measurement relaxation according one of the following options, which is indicated by the network: - Option a: UE uses both low mobility criterion and not-at-cell-edge criteria, i.e. UE can perform relaxation only when both criteria are fulfilled. And detailed relaxation behaviour is up to RAN4 discussion and decision; - Option b: UE uses either low mobility criterion or not-at cell-edge criterion, i.e. UE can perform relaxation when either low mobility or not-at-cell-edge criterion is fulfilled. And detailed relaxation behaviours are same as case that network only configures low mobility or not-at-cell-edge criterion; - The values of parameter SSearchDeltaP are 3, 6, 9, 12, 15 dB. - The infinity value for parameter SSearchDeltaP is not needed. - When network configures both RSRP and RSRQ thresholds for not-at-cell-edge criterion, UE can perform relaxation only when both RSRP and RSRQ based criteria are met. - Srxlev/Squal (Cell selection RX level value (dB)) is used to compare with threshold SsearchThresholdP/ SsearchThresholdQ. The corresponding text has been updated in running 38.304 CR. - The threshold SsearchThresholdP/ SsearchThresholdQ reuse the value range of ReselectionThreshold/ReselectionThresholdQ or RSRP-Range/RSRQ-Range - An LS is sent to RAN4 on RAN2 conclusions for the RRM measurement relaxation and ask RAN4 to discuss the measurement relaxation approach(es). - The parameter SrxlevRef is set according to the LTE mechanism as captured in current running 38.304 CR for power saving - FFS on RAN4 - if and what parameters we need (e.g. time interval for measurement relaxation since last measurement for cell reselection and the value range for the time interval)

RRC rapporteur should be able to use this as a baseline and companies can provide further views over email

Proposal 1: The terminology of relaxed measurement (i.e. option 2) is used for RRM measurement relaxation in NR.

Proposal 4: The IE highPriorityMeasRelax is defined as an optional fieldIE with the value of “ENUMERATED {true}” to configure higher priority frequencies can be relaxed.

Proposal 17: The parameter SSearchDeltaP is optional and default value can be 6dB.

Proposal 7: If the indication highPriorityMeasRelax is optional and when it is not present, the legacy behaviour should be followed, i.e. no measurement relaxation is performed on the high priority frequency.

Proposal 9: If proposal 8 is agreeable, this “and/or” indication is mandatory if network configures the parameters of both low mobility and not-at-cell-edge.

Proposal 12: The parameter TSearchDeltaP is optional, and the default value can be 1 minute or 60s.

Proposal 20: Whether the parameter SsearchThresholdP/SsearchThresholdQ is optional or mandatory if RRM measurement relaxation is supported can be discussed during the meeting after the decision on Proposal 2.

Proposal 11: RAN2 have a short discussion on the detailed values and the granularity for parameter TSearchDeltaP:

-	Option 2: Values in number of seconds, e.g. 5s, 10s, 20s, 30s, 60s, 120s, 180s, 240s, and 300s.

Agreements: - Relaxed RRM measurement is applied in the same way irrespective of whether the priorities are provided by dedicated signalling or broadcast signalling. - Ask RAN4 (In the same LS to RAN4 listing the RAN2 agreements) about the behaviour of relaxation of higher priority carriers: - For the case where Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, does RAN4 envision to relax higher priority carriers measurements further than Thigher_priority_search if RAN2-defined relaxation criterion(s) is/are met? - For the case where Srxlev < SnonIntraSearchP or Squal < SnonIntraSearchQ, does it make sense / is there a performance benefit to only relax equal/lower priority carriers but not higher priority carriers measurements if RAN2-defined relaxation criterion(s) is/are met? - The UE shall perform intra-frequency and inter-frequency neighbour cell measurement during TsearchDeltaP after cell selection/re-selection. - No indication to the network that UE has performed measurement relaxation is introduced. - From RAN2 perspective, there is no consensus on a method for reducing the carriers to measure in Rel-16. We can come back to this if RAN4 agrees otherwise. - A method for reducing the cells to measure on a carrier is not supported in Rel-16 - FFS on the UE behaviour if T330 is running

2.2.2	Remaining Open issues

- DCI with CRC scrambled by PS-RNTI (DCP)

| Open issues | Related specs |
| --- | --- |
| FFS whether DCP applies to short DRX. | 38.321 |
| FFS what the UE actually monitors if it misses DCP when configured with SCell dormancy | 38.321 |
| FFS UE behavior when a DCP occasion occurs during RAR window will be decided at the next meeting. | 38.321 |
| DCP-related UE feature list and capabilities | 38.306 |

- UE assistance

| Open issues | Related specs |
| --- | --- |
| The reported values of UE assistance on reduced bandwidth, cells and MIMO layers for power savings can range up to at least the corresponding value in the current active configuration. FFS if it can be up to UE capability. | 38.300 38.331 |
| A UE can report a preferred aggregated bandwidth for a frequency range on the configured serving cell. FFS if it is allowed even if it is not configured with serving cells on that frequency range. | 38.300 38.331 |
| UE transmits SCG specific UAI for power saving in a transparent container to the MN and the MN then forwards the received container to the NR SN. FFS if UAI can also be reported for power saving directly via SRB3 if configured. FFS on the signaling details (MR-DC signaling of UAI needs to be captured). | 38.331 36.331 |
| DCP-related UE feature list and capabilities | 38.306 |

- RRM measurement relaxation

| Open issues | Related specs |
| --- | --- |
| RRM measurement relaxation related UE feature list and capabilities | 38.306 |

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

Agreements made in RAN4#94 e-meeting (24th February – 6th March 2020):

Agreements in switching and interruption time for MIMO layer adaption (R4-2002799)

- No further evaluation on power saving gain for combined features in Rel-16 UE power saving WI.

- MIMO layer adaption for Case 2 follows general BWP switch delay requirement as for Case 1, e.g. reuse Rel-15 BWP switching delay requirements

- The interruption time for MIMO layer adaption based on BWP switching is defined consistently with BWP switching interruption defined in TS 38.133.

Agreements in RRM measurement relaxation for Power Saving (R4-2002338)

RRM measurement relaxation

- Applicability of RRM relaxation methods for scenario#1 and scenario#2:

  - Scenario #1:

    - Agreement - RRM measurement relaxation with longer intervals

  - Scenario #2:

    - Agreement - RRM measurement relaxation with longer intervals

- FFS on the scaling factor of measurement interval

  - Option 1: Fixed value

    - FFS on one fixed value for both scenario#1 and scenario #2

    - FFS on separate values for scenario#1 and scenario #2

  - Option 2: Network configurable value

- FFS on the transition period between scenario#1 or scenario#2 and scenario#3

  - Option1:

    - When switching from scenario#2 to scenario#3, UE shall fulfill the requirements corresponding to scenario#2 for N DRX cycles and thereafter switch to requirements corresponding to scenario#3

    - When switching from scenario#3 to scenario#2, UE shall fulfill the requirements corresponding to scenario #2 immediately.

  - Other option is not precluded.

- RRM measurement relaxation for inter-frequency layer with higher priority

  - Option 1:

    - When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ,  no relaxation of the current measurement delay requirement is expected for inter-frequency measurement with higher priority.

    - When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, the relaxed requirement for the frequency layer of higher priority shall use the same  relaxed measurement requirement as those for the frequency layer of equal/lower priority.

  - Option 2:

    - Measurements of higher priority carriers shall not be relaxed in high mobility scenarios (scenario #2)

  - Option 3:

    - Measurements of higher priority carriers shall not be relaxed

- RRM measurement relaxation by reducing the number of frequency layer to be measured

  - No consensus to introduce the RRM measurement relaxation by reducing the number of frequency layer in RAN4 94e.

  - Interested companies are encouraged to provide analysis on achievable power saving from reducing number of frequency layers for intra/inter-frequency measurements in RAN4 94bis

- EMR impact in power saving mode

  - Option 1: Up to UE’s implementation

  - Option 2: EMR frequency layer shall not be relaxed if T331 is running

  - Option 3: EMR frequency layer shall be relaxed

- RRM measurement relaxation threshold for inter-frequency measurement

  - Option 1: Introduce carrier specific search thresholds for measurement relaxation

  - Option 2: Up to RAN2’s decision

- RRM impact due to cross-slot scheduling power saving technique

  - Option 1: have RRM impact on DCI based delay requirement

  - Option 2: no RRM impact

  - Option 3: wait for RAN1 conclusion

4Rx demod requirement impact due to MIMO layer adaption feature

- Postpone discussion on Demod requirement impacts due to MIMO layer adaption to the WI performance part

2.4.2	Remaining Open issues

Open issues

FFS on the scaling factor of measurement interval

FFS on the transition period between scenarios

FFS on RRM measurement relaxation for inter-frequency layer with higher priority

FFS on EMR impact in power saving mode

FFS on RRM measurement relaxation threshold for inter-frequency measurement

FFS on RRM impact due to cross-slot scheduling power saving technique

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTx

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN2#109e

- R2-2001913	Summary of open issues for PDCCH 	CATT

- R2-2000253	Contributions summary on further impacts of DCP	CATT

- R2-2000254	New issue on CSI reporting with DCP	CATT

- R2-2000349	Open issues DCP	Ericsson

- R2-2000367	PDCCH-WUS not applicable for short DRX cycle	vivo

- R2-2000368	WUS impact on CSI reporting	vivo

- R2-2000412	Remaining issues on DCP	OPPO

- R2-2000413	Impacts of power saivng signalling on CSI reporting	OPPO

- R2-2000450	Open issues of DCP feature	Intel Corporation

- R2-2000584	PDCCH-WUS Mechanism	Apple

- R2-2000599	PDCCH-WUS and Short DRX Cycle	Apple

- R2-2000665	Discussion on introduction of search space for the DCP	ZTE Corporation, Sanechips

- R2-2000666	Introduction of search space for the DCP in TS38.331	ZTE Corporation, Sanechips

- R2-2000811	Discussion on PDCCH-WUS missing problems during handover	Xiaomi Communications

- R2-2001037	On DRX ambiguous period	Nokia, Nokia Shanghai Bell

- R2-2001038	On DCP monitoring and CSI/SRS transmission	Nokia, Nokia Shanghai Bell

- R2-2001040	On short DRX cycle applicability for DCP	Nokia, Nokia Shanghai Bell

- R2-2001300	Consideration on Short DRX cycle on DCP	LG Electronics Inc.

- R2-2001463	Remaining issues on WUS signal for Power Saving	ZTE Corporation, Sanechips

- R2-2001482	Wakeup signaling with DRX groups	Qualcomm Inc, Samsung

- R2-2001914	Summary of open issues for UE assistance 	Qualcomm

- R2-2000255	Reporting UE Assistance Info to NR SN	CATT

- R2-2000350	Open issues for UE assistance	Ericsson

- R2-2000351	Open issues for MR-DC scenarios	Ericsson

- R2-2000369	UE assistance information for power saving	vivo

- R2-2000451	Open issues of new UE assistance information for PWS	Intel Corporation

- R2-2000585	UE Assistance Information for MR-DC	Apple	discussion

- R2-2000596	UE Assistance Information for Scell	Apple	discussion

- R2-2000649	Remaining open issues on UE assistance information	OPPO

- R2-2000826	Power Saving UE assistance information	Sony

- R2-2000869	Power Saving UE assistance information	Sony Europe B.V.

- R2-2001301	Remaining issue on UE assistance	LG Electronics Inc.

- R2-2001330	Remaining issues on UE assistance information	Huawei, HiSilicon

- R2-2001483	Remaining issues on UE Assistancec Information	Qualcomm Inc

- R2-2002025	Summary of open issues on UE assistance	Qualcomm

- R2-2001915	Summary of RRM measurement relaxation open issues 	Huawei

- R2-2002199	Summary of RRM measurement relaxation open issues 	Huawei

- R2-2000256	Way forward on measurement relaxation with high priority frequencies	CATT

- R2-2000312	Configurations for RRM Measurement Relaxation in NR	MediaTek Inc.

- R2-2000352	Open issues RRM measurement relaxation	Ericsson

- R2-2000370	UE Power Consumption Reduction in RRM Measurement	vivo

- R2-2000595	Open Issues of RRM Measurement Relaxation	Apple

- R2-2000827	UE power saving for inter frequency measurements	Sony

- R2-2000913	Discussion on power saving for inter-frequency measurements	CMCC

- R2-2001039	On RRM measurement relaxation	Nokia, Nokia Shanghai Bell

- R2-2001063	On SrxlevRef adaptation in relaxed monitoring	Huawei, HiSilicon

- R2-2001064	Reducing the number of neighbour cells/carriers to measure	Huawei, HiSilicon

- R2-2001401	Coexistence issues of measurement relaxation and early measurements	LG Electronics, Ericsson, MediaTek

- R2-2001402	Per-frequency measurement relaxation based on neighbour cell quality	LG Electronics

- R2-2001577	RRM measurement relaxation	Samsung

- R2-2001643	On the frequency selection for RRM relaxation 	Samsung R&D Institute UK

RAN4#94e

- R4-2002688	Email discussion summary for RAN4#94e_#15_NR_UE_pow_sav_RF	CATT

- R4-2002884	Email discussion summary for RAN4#94e_#15_NR_UE_pow_sav_RF	CATT

- R4-2002799	 WF on NR UE power saving	CATT

- R4-2000601	Further discussion on switching and interruption for maximum MIMO layer adaptation	CATT

- R4-2000767	Switching time for MIMO only change	Qualcomm, Inc.

- R4-2000969	Discussion on switching time for MIMO layer adaptation	 ZTE Corporation

- R4-2001758	On switching time for MIMO layer/antenna number adaption	 Huawei, HiSilicon

- R4-2001793	Discussion on UE dynamic adaptation to the maximum number of MIMO layers	MediaTek inc.

- R4-2000153	Regarding switching and interruption timing requirement for MIMO layer adaption	vivo

- R4-2000602	CR on RRM requirement for maximum MIMO layer adaptation	vivo, CATT

- R4-2000787	On baseband requirement with MIMO layer adaptation	Apple

- R4-2000990	On requirements for MIMO layer adaptation for power saving	OPPO

- R4-2001655	Discussion on RRM requirement for MIMO layer adaption case2	Huawei, HiSilicon

- R4-2002136	Interruption time during MIMO layer adaptation	Qualcomm

- R4-2002178	Email discussion summary for RAN4#94e_#55_NR_UE_pow_sav_RRM	CATT

- R4-2002198	Email discussion summary for RAN4#94e_#55_NR_UE_pow_sav_RRM 	CATT

- R4-2002309	Email discussion summary for RAN4#94e_#55_NR_UE_pow_sav_RRM	CATT

- R4-2002239	WF on RRM measurement relaxation for power saving	CATT

- R4-2002338	WF on RRM measurement relaxation for power saving	CATT, vivo

- R4-2000152	Remaining issues on NR UE power saving	vivo

- R4-2000157	Draft LS on introducing thresholds for inter-frequency RRM relaxation for UE Power Saving	 vivo

- R4-2000158	Evaluation of RRM relaxation for power saving	 vivo

- R4-2000575	Further discussion on RRM measurement relaxation for NR power saving	 CATT

- R4-2000576	CR on measurement relaxation in IDLE mode for UE power saving	CATT

- R4-2002326	CR on measurement relaxation in IDLE mode for UE power saving	CATT

- R4-2000577	CR for DCI based TCI state switch delay due to cross slot scheduling	CATT

- R4-2000578	CR for DCI based BWP switch delay due to cross slot scheduling	CATT

- R4-2000642	RRM measurement relaxation for UE power saving	CMCC

- R4-2000963	Measurement relaxation for power saving	 LG Electronics Inc.

- R4-2000989	On RRM measurement relaxation for power saving	 OPPO

- R4-2001343	UE RRM Core requirements when applying UE power saving	 Nokia, Nokia Shanghai Bell

- R4-2001344	LS on introduction of carrier specific thresholds for UE Power Saving schemes	Nokia, Nokia Shanghai Bell

- R4-2001654	Discussion on measurement relaxation in power saving	Huawei, HiSilicon

- R4-2001754	Discussions on RRM impact of NR UE power saving	Ericsson

- R4-2001794	Discussion on RRM measurement relaxation in IDLE/INACTIVE mode	MediaTek inc.

- R4-2002137	RRM measurement relaxation for power saving	 Qualcomm