3GPP TSG RAN Meeting #87e				        		   RP-200316

Electronic Meeting, March 16-19, 2020

# Source:	CATT

# Title:	New WID on further enhancement for NR RRM requirement in Rel-17

# Document for:	Approval

Agenda Item:	15

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

Title: 	New WID on NR RRM requirement improvement

Acronym: NR_RRM_fenh

Unique identifier: 	TBD {A number to be provided by MCC at the plenary}

NOTE:	For new WIs/SIs leave the Unique identifier empty or you can make a proposal for an Acronym.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  |  |  |
| No | X |  | X | X | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, we define them as work tasks. If you are in doubt, please contact MCC.

2.2	Parent and child Work Items

| Parent and child Work Items |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one WID. Therefore the table above should just include the feature WI Unique ID and title and Nature of relationship is "parent WID".

2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

NOTE:	Also related or dependent WIs in other TSGs should be indicated.

3	Justification

NR introduced quite many features in one release and thus there are many RAN4 RRM requirements to be specified. In RAN4, there are still many remaining Rel-15 RRM issues. These remaining issues have been agreed to be solved in Rel-16, e.g. CSI-RS based L3 measurement requirement, the second pair of BLER for RLM, multiple SCells activation/deactivation, SRS carrier/port switching, CGI reading and RRM requirement related with FR2 inter-band CA if needed etc.

In Rel-15, SRS carrier and port switching and CGI reporting are quite important UE feature, the corresponding RRM requirement including core and performance requirements shall be defined. In Rel-15, RAN1 made the agreement to support two pairs of the RLM thresholds, the second pair of IS/OOS BLERs is used for VoIP service, it is important to define the requirements to support this functionality.  Since in Rel-15, UE is required to support up to 8 DL CCs, thus, it is quite straightforward to define multiple SCells activation/deactivation requirements.

CSI-RS was designed in RAN1 used for beam management and mobility management, and CSI-RS based mobility plays an important role in mobility and network deployment / optimization. CSI-RS based measurement is more refined and flexible than SSB based measurement. Moreover, SSB of intra-frequency cells are fully overlapped according to the configuration principle designed by RAN1/2, resulting in underestimated SS-SINR for the case of low interferer load, CSI-RS based RRM, can solve this underestimation issue.

This new WID is proposed to specify NR legacy RRM requirements described in objective in Rel-16.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of this work item is to specify the following NR RRM enhancement requirements in R17:

- RLM enhancement requirement

  - Specify the second IS/OOS BLER pair for VoNR service;

- RRM requirement for SRS antenna port switching for NR SA, NR-DC, EN-DC and NE-DC including

  - Interruption requirements of the following cases

    - NR SRS antenna port switching impacting LTE CC

    - NR SRS antenna port switching impacting NR CC

(3)    Other leftovers from Rel-16 are to be added later.

4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

- Specify the RRM test cases if necessary: test cases for RLM enhancement requirement, RLM, interruption and SRS antenna port switching requirement, etc.

4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

5	Expected Output and Time scale

| New specifications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR No. | Title | For info at TSG# | For approval at TSG# | Remarks |
| TR | 38.xxx |  |  |  |  |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.133 | NR; Requirements for support of radio resource management | TSG-RAN #xx | Core part |
| 38.331 | NR; Radio Resource Control (RRC) Protocol specification | TSG-RAN #xx | Core part |

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

6	Work item Rapporteur(s)

7	Work item leadership

RAN WG4

8	Aspects that involve other WGs

RAN WG2

NOTE:	For RAN WIDs: Section 8 applies only to WGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

9	Supporting Individual Members

Supporting IM name

CATT