3GPP TSG RAN Meeting #87e								RP-200324

Electronic Meeting, March 16-19, 2020

Agenda Item:	9.7

Source:	Apple Inc.

Title:	Views on two DRX groups in the same cell group

Document for:	Discussion/Decision

# Introduction

Current NR only supports single CDRX configuration for all the cells in a CG (cell group) which follows the LTE legacy. However, one of the major improvement from NR compared to LTE is the introduction of Radio Access designs to support mmWave, or FR2, deployment that allows cellular communication to have access to spectrum with much wider bandwidth at high frequency band. FR2 deployment has at least 2 major challenges, including

- The coverage limitation due to the large path loss and penetration loss from high frequency band

- Power consumption and thermal challenges especially for UE implementation

Therefore, to solve the coverage or reliability issue with FR2 communication, it is nature to use FR1 and FR2 CA in which PCell or PSCell resides in FR1 to ensure ubiquitous coverage and robust communication while SCells can be deployed on FR2 to provide data enhancement layers for large amount of data or high peak rate data communication.

With FR1 and FR2 CA deployment, configuration the same CDRX configuration for cells in FR1 and FR2 is not efficient enough to address the different usage and design challenges between FR1 and FR2, at least in the following areas

- FR2 has different power consumption and thermal problem compared to FR1

- FR2 cells are mainly to handle large amount of data or to achieve high date rate, it can be less "active" than the FR1 cells to maintain reasonable UE power consumption and thermal dissipation

As results, as part of TEI, RAN2 is looking at the solution to allow 2 DRX groups for a cell group mainly targeting to support FR1 and FR2 CA. The main use case of 2 DRX groups is for FR1 and FR2 CA in which one DRX group can be configured for FR1 and the other DRX group can be configured for FR2. Each DRX group can have different DRX related configurations such as short/long DRX cycle, drx-InactivityTimer and drx-onDurationTimer.

The progress of two DRX groups design in RAN2 is slow due to concerns and unwillingness of some companies to enable this feature. Consequently, RAN2 sent LS to both RAN1 and RAN4 inquiring the impact of this feature on RAN1 and RAN4 in R2-1916597, and the main contents are duplicated as below,

RAN2 discussed a power saving enhancement when both FR1 and FR2 cells are configured via Carrier Aggregation. With this enhancement the FR2 cells can be configured with a separate (and shorter) drx-InactivityTimer and drx-OnDurationTimer compared to the FR1 cells. The length of Long DRX cycle and Short DRX cycle, if configured, are common for both FR1 and FR2. This enhancement enables the FR2 cells to go to sleep more quickly and thereby reduce power consumption as depicted below: RAN2 reached the following agreements: Conditional on R1 acceptance: A separate drx-InactivityTimer and drx-onDurationTimer can be configured for the secondary DRX group. R2 understands that this has zero or almost zero impact in R1 and R4 The combination of cross-carrier scheduling and secondary DRX group is not supported FFS if timers for FR2 DRX configuration are shorter than timers for FR1 DRX configuration. The intention is to apply secondary DRX configuration to FR2 and existing DRX configuration to FR1 RAN2 has the intention to introduce this enhancement under TEI16 and would like to check that this has zero or very little impacts to RAN1 and RAN4. 2. Actions: To RAN1 ACTION: RAN2 respectfully asks RAN1 to confirm there is no RAN1 impact, or little impact that is acceptable. To RAN4 ACTION: RAN2 respectfully asks RAN4 to confirm there is no RAN4 impact or little impact that is acceptable.

Current status in RAN2 is that RAN2 work on the secondary DRX group is put on hold until RAN2 can receive positive LS reply from RAN1 and RAN4. However, in the latest RAN1 meeting RAN1#100e, a LS reply was delayed, at least until next meeting, since it is hard to make a judgement in RAN1 whether impact is “little”.

In this contribution, we share our view on this topic and propose for RAN2 to continue the work without hinging on the LS reply.

# Views on secondary DRX group

## RAN2 view

As we discussed above, we believe two DRX groups in a cell group is a very useful feature to solve the practical issues related to FR2 deployment. For FR2 deployment, we need to consider both the coverage and UE power consumption issues, which are typically two conflicting requirement. For FR2 deployment, the coverage can be ensured by FR1 and FR2 NR-CA. In FR1 and FR2 NR-CA, it is nature to configure different DRX pattern to adapt to the different use case for FR1 and FR2. It is reasonable to configure longer DRX cycle, longer drx-InactivityTimer and/or longer drx-onDurationTimer for cells in FR2 compared to cells in FR1, since FR2 is more targeted for data enhancement layer and faces more stringent power and thermal concern. UE should not be required to monitor DCI on FR2 as frequently as FR1.

Unfortunately, a LS reply confirming no impact or little impact that is acceptable from either RAN1 or RAN2 is hard to reach. Different companies always have different opinion and the question itself is also quite subjective to judge whether the impact is “little” or “acceptable”. If there are consensus to work on this feature in the first place, the LS, most likely, wouldn’t have been sent out. While consensus is clearly required for a LS reply, we are worried this can easily create deadlock and prevent secondary DRX group from being design in RAN2.

As results, we have following observation

Observation: RAN2 should try to minimize the risk of deadlock that may be created by replying on the LS reply from RAN1 and RAN4. The deadlock may cause the secondary DRX group work to be delayed unnecessarily.

## RAN1 view

During the RAN1 discussion in RAN1#100e, companies raised a few topics on which secondary DRX group design may have impact on RAN1. Below we share our view on those topics, which mainly focus on the interaction between WUS and two DRX groups

- Impact on WUS. Currently, PDCCH monitoring related specification in RAN1 is captured in 38.213. We believe some modification to 38.213 might be inevitable based on RAN2 secondary DRX group design, especially the interaction with WUS. However, the main design task for secondary DRX group with WUS is to specify the active time for each cell when one DRX cycle may correspond to multiple WUS occasions, for example, when we consider the typical use case that FR1 cells can be configured with short DRX cycle and FR2 cells can be configured with long DRX cycles. This can be design task for RAN2 and will be captured in RAN2 specification 38.321.

- The potential impact on CSI measurement and report. As long as active time can be clearly defined for each CC which is a design task for RAN2 and which is also captured in RAN2 specification 38.321, we do not see impact on the RAN1 specification since it is clear that UE does not need to measure CSI reference signal unless it is in active time. For CSI reporting, 38.214 is also clear whether UE needs to report CSI reporting based on RRC parameters. We quoted the related 38.214 as below

- The potential impact on SCell dormancy behavior. It was agreed that WUS can only be configured on SpCell, i.e. PCell on MCG or PSCell on SCG. We believe dormancy behavior handles the power saving in a larger time scale, but more efficient than SCell activation and deactivation. We don’t think dormancy operation has to rely on the condition that SCell follows the same CDRX configuration as PCell. Therefore, we don’t think there is impact of secondary DRX group on SCell dormancy

Based on the above discussion, we have the following observation

Observation: Impact of secondary DRX group on RAN1, if exists, is acceptable

## RAN4 view

In order to consider the dual DRX case, we can compare this case with dual DRX for DC. In the current RRM requirements, there are two main aspects which will probably be impacted by introducing second DRX group for FR2 in FR1+FR2 CA: cell identification/measurement and interruption requirement.

The cell identification/measurement includes all measurement related UE activities, e.g. cell identification on SCC, cell measurement on SCC, deactivated SCell measurement and so on. However, if DRX is used, all of those requirement are based on DRX cycle. Based on the RAN2 solution, only two configurations of second DRX can be different from FR1 CC DRX: drx-InactivityTimer and drx-OnDurationTimer, and therefore the second DRX would have the same starting point and cycle periodicity as first DRX on the time domain, which was illustrated in the figure of RAN2 LS. Since, in RAN2 design, first DRX on FR1 CC will always have same DRX cycle as second DRX on FR2 CC and all RRM requirements of measurement are based on DRX cycle, it will not have any impact on identification/measurement related requirement in DRX status.

Observation: Second DRX group on FR2 CC will not impact any identification/measurement related requirements in DRX status for RAN4, e.g., cell identification on SCC, cell measurement on SCC, deactivated SCell measurement and so on.

In RAN4, if dual DRX is configured in EN-DC, no interruption is allowed. Coming to FR1+FR2 CA case, we also think interruption shall not be allowed in the DRX on-duration time because that would remarkably impact the UE and system performance and that’s the motivation to avoid interruption for dual DRX case in EN-DC. To remove the requirement on interruption time, it is closely related to capability whether UE can support per-FR Measurement Gap (MG), which is RAN4 Rel-15 FG 3-1, i.e. independentGapConfig. For UE that supports per-FR MG, UE can toggle RF individually on FR1 CC and FR2 CC to save power, and meanwhile, we don’t expect interruption within any DRX on-duration.

Based on the above discussion, to avoid unnecessary delay and potential delay due to LS between RAN2 and RAN1/RAN4, we have the following observation

Observation: To serve power saving purpose, the secondary DRX group on FR2 CC for FR1 + FR2 CA is only supportable to the UE with per-FR MG capability, and no new interruption requirement will be introduced in RAN4.

Based on the discussion of our view from WG RAN1/2/4 on the secondary DRX group, we have the following proposal

Proposal 1: RAN2 should continue the design and specification of secondary DRX group per cell group,

targeting the use case of FR1 and FR2 NR-CA.

# Conclusion

As part of Rel-16 RAN2 TEI, companies start to discuss the introduction of the secondary DRX group in a cell group, especially for the purpose for FR1 and FR2 CA. The progress of the work in RAN2 is on hold while waiting for LS reply from RAN1 and RAN4. We think this feature is very useful to solve some practical deployment concern of FR2, allowing the device and NW to achieve good coverage and robustness by FR1 and FR2 CA, while still maintain good UE power consumption by allowing FR2 carrier to operate in a more power saving friendly DRX configuration, while FR1 carrier to operate with different DRX configuration more tailored to ensure low latency data scheduling. The LS between RAN2 and RAN1/RAN4 may cause deadlock and unnecessary delay of this feature. Therefore, we have the following proposal

Proposal 1: RAN2 should continue the design and specification of secondary DRX group per cell group, targeting the use case of FR1 and FR2 NR-CA.

# References

- R2-1916597, LS on secondary DRX group, Ericsson, RAN2 #108 meeting