3GPP TSG-RAN WG4 Meeting #94-e	R4-2001575

# Online, 24th February – 6th March, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0512 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on UE sidelink timing requirements for NR V2X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the agreements in R4-1909993, the UE sidelink transmit timing are introduced for NR V2X. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To define UE sidelink timing requirements in section 12.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE sidelink timing requirements for NR V2X will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

12.2	UE Transmit Timing

12.2.1	Introduction

This clause contains requirements of transmission timing for V2X sidelink communication when:

-	GNSS is used as the synchronization reference source;

-	NR Cell is used as the synchronization reference source;

-	E-UTRAN Cell is used as the synchronization reference source;

-	SyncRef UE is used as the synchronization reference source.

12.2.2	GNSS as synchronization reference source

The requirements in this subclause are applicable when the reference timing used by the UE for V2X sidelink communication is derived from GNSS.

The sidelink transmissions takes place  before the subframe starting boundary as defined in TS 38.331 [2], where  = 0 and=0.

The transmission timing error for sidelink transmissions shall be less than or equal to Te where the timing error limit value Te is defined in Table 12.2.2-1.

Table 12.2.2-1: Te Timing Error Limit

| Frequency Range of sidelink | Te_ |
| --- | --- |
| 1 | 12*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6]. |  |

12.2.3	NR Cell as synchronization reference source

The requirements in this subclause are applicable when the reference timing used for sidelink transmissions is a NR serving cell on a non-V2X sidelink carrier.

The sidelink transmissions takes place  before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell, where  = 0 and=0.

The transmission timing error for sidelink transmissions shall be less than or equal to Te where the timing error limit value Te is defined in Table 12.2.3-1.

Table 12.2.3-1: Te Timing Error Limit

| Frequency Range of sidelink | SCS of SSB signals ( kHz) | SCS of sidelink signals (kHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 14*64*Tc |
|  |  | 30 | 12*64*Tc |
|  |  | 60 | 12*64*Tc |
|  | 30 | 15 | 10*64*Tc |
|  |  | 30 | 12*64*Tc |
|  |  | 60 | 9*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6]. |  |  |  |

12.2.4	E-URTAN Cell as synchronization reference source

The requirements in this subclause are applicable when the reference timing used for sidelink transmissions is an E-UTRAN serving cell on a non-V2X sidelink carrier.

The sidelink transmissions takes place  before the reception of the first detected path (in time) of the corresponding E-UTRAN downlink frame from the reference cell, where  = 0 and=0.

The transmission timing error for sidelink transmissions shall be less than or equal to Te where the timing error limit value Te is defined in Table 12.2.4-1.

Table 12.2.4-1: Te Timing Error Limit

| Frequency Range of sidelink | E-UTRAN downlink bandwidth (MHz) | Te_ |
| --- | --- | --- |
| 1 | ≥3 | 14*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6]. |  |  |

12.2.5	SyncRef UE as synchronization reference source

The requirements in this subclause are applicable when the reference timing used for deriving sidelink transmission is from SyncRef UE transmitting sidelink synchronization signals.

The sidelink transmissions takes place  before the reception of the first detected path (in time) of the corresponding timing reference frame from the SyncRef UE, where  = 0 and=0.

The transmission timing error for sidelink transmissions shall be less than or equal to Te where the timing error limit value Te is defined in Table 12.2.5-1.

Table 12.2.5-1: Te Timing Error Limit

| Frequency Range of sidelink | SCS of sidelink signals (kHz) | Te |
| --- | --- | --- |
| 1 | 15 | 12*64*Tc |
|  | 30 | 8*64*Tc |
|  | 60 | 5*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6]. |  |  |

<End of Change 1>