3GPP TSG-RAN WG4 Meeting #94-e	R4-2001420

Electronic Meeting, 24 February – 6 March, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | 0065 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.817-02: Measurement uncertainty for FR2 OTA additional spurious emissions requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2020-01-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | World Radio Conference (WRC-19) has reached an agreement on limits of unwanted emission power in a specified bandwidth with the Earth Exploration Satellite Service (EESS) band. In this CR the measurement uncertainty analysis is captured into the TR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Measurement uncertainty for FR2 OTA additional spurious emissions requirements is captured into TR by re-using the uncertainty for OBUE and mandatory spurious emissions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Measurement uncertainty analysis for FR2 OTA additional spurious emissions requirements is not captured and therefore the background for the requirement is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of changes >

### 12.7.3	Additional Spurious Emissions requirements

#### 12.7.3.1	FR1

The measurement uncertainty for BS type 1-O out of band TRP requirements is based on the AAS BS MU analysis in TR 37.843 [9].

The additional spurious emission requirements including the co-existence with other BS in the same geographical area are based on existing co-existence with other 3GPP bands so the frequency ranges for the uncertainty assessment are different from the mandatory spurious emissions. In this case the uncertainty budgets for the in-band chambers are considered in the MU analysis. As the TRP level is at a low power level the measurement system dynamic range uncertainty is also considered as with the receiver spurious emissions.

Adding the MUperpoint and the SE we have:

,		f ≤ 3 GHz

,		3 GHz < f ≤ 4.2 GHz

,		4.2 GHz < f ≤ 6 GHz

By adding the MU per point and the SE values (see clause 12.10) together we have the following total MU:

MUtotal = 2.6 dB,		f ≤ 3 GHz

MUtotal = 2.6 dB,		3 GHz < f ≤ 4.2 GHz

MUtotal = 3.0 dB,		4.2 GHz < f ≤ 6 GHz

For additional requirements for co-existence between 3GPP bands the TT values are as follows:

MUtotal = 2.6 dB,		f ≤ 3 GHz

MUtotal = 2.6 dB,		3 GHz < f ≤ 4.2 GHz

MUtotal = 3.0 dB,		4.2 GHz < f ≤ 6 GHz

For PHS, and public safety additional requirements the TT=0dB.

The MUperpoint value in TR 37.843 [9] is:

MUperpoint = 2.48 dB,		f ≤3 GHz

MUperpoint = 2.52 dB,		3 GHz < f ≤ 4.2 GHz

MUperpoint = 2.93 dB,	,	4.2 GHz < f ≤ 6 GHz

By adding the MU per point and the SE values (see clause 12.10) together we have the following total MU:

MUtotal = 2.6 dB,		f ≤ 3 GHz

MUtotal = 2.6 dB,		3 GHz < f ≤ 4.2 GHz

MUtotal = 3.0 dB,		4.2 GHz < f ≤ 6 GHz

The test tolerance for co-existence between 3GPP systems is equal to the MU so the TT values are as follows:

TT = 2.6 dB,		f ≤ 3 GHz

TT = 2.6 dB,		3 GHz < f ≤ 4.2 GHz

TT = 3.0 dB,		4.2 GHz < f ≤ 6 GHz

The test tolerance for additional requirement which are based on regulation (for example co-existence with PHS systems) is zero.

#### 12.7.3.2	FR2

The analysis done for mandatory spurious emissions in section 12.7.1.2 applies also for additional spurious emissions.

## 12.8	Conformance testing for OTA co-location requirements

< End of changes >