3GPP TSG-RAN Meeting #87e	RP-200095

# Electronic Meeting, March 16-19, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.807 | CR | 0001 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the status of 66-71 GHz spectrum after WRC-19 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_beyond_52GHz |  |  |  |  |  | Date: |  |  | 2020-03-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Decisions from WRC-19 on 66-71 GHz are not captured. Those decisions were made before the completion of the study item. WRC-19 (28 October to 22 November 2019) identified 66-71 GHz for IMT via footnote 5.J113. Note 1 in Tables 4.2.1-1 and 4.2.1-2 is no longer relevant after the decisions at WRC-19. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of WRC-19 decisions on IMT identification of 66-71 GHz. Corrections of obsolete text refering to studies under AI 1.13 of WRC-19. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The technical report approved in Dec 2019 at the end of the study does not reflect the correct status of the 66-71 GHz spectrum at that time. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.1, 4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

====================<Unchanged text is omitted>========================================

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TD RP-181435: "New SID: Study on NR beyond 52.6 GHz".

[3]	3GPP TR 38.805: "Study on New Radio access technology; 60 GHz unlicensed spectrum".

[4]	3GPP TR 38.913: "Study on New Radio access technology; Next Generation Access Technologies".

[5]	WRC-15 Resolution 238: "Studies on frequency-related matters for International Mobile Telecommunications identification including possible additional allocations to the mobile services on a primary basis in portion(s) of the frequency range between 24.25 and 86 GHz for the future development of International Mobile Telecommunications for 2020 and beyond".

[6]	International Telecommunication Union, "CPM Report to WRC-19: Chapter 2, agenda item 1.13, Method C2, "	https://www.itu.int/md/R15-CPM19.02-R-0001/en

[7]	APG19-3/INP-35, Australia, "PRELIMINARY VIEWS ON WRC-19 AGENDA ITEMS 1.13, 1.16, 9.1 (ISSUES 9.1.1, 9.1.5 AND 9.1.8)", APG19-3, March, 2018, Australia

[8]	APG19-3/INP-87, China, "PRELIMINARY VIEWS ON WRC-19 AGENDA ITEMS 1.13, 1.16, 9.1 (ISSUES 9.1.1, 9.1.5, 9.1.8) ", APG19-3, March, 2018, Australia

[9]	Independent Communications Authority of South Africa. Standard procedures and guideline regarding the use of radio frequency spectrum in the E-band (71 – 76 GHz paired with 81 – 86 GHz). April 28, 2017.

[10]	Independent Communications Authority of South Africa. National Radio Frequency Plan 2018. May 25, 2018.

[11]	Independent Communications Authority of South Africa. Electronic Communications Act (781/2016): Amendment to the Radio Frequency Spectrum Regulations 2015. No. 404436, November 22, 2016.

[12]	Independent Communications Authority of South Africa. Radio Frequency Spectrum Regulations (279/2015): Amendment to the Radio Frequency Spectrum Regulations, 2015. November 22, 2016.

[13]	Canada Spectrum Management and Telecommunications. Canadian Table of Frequency Allocations 2018 Edition. April 2018.

[14]	Canada Spectrum Management and Telecommunications. Technical Requirements for Fixed Line-of-Sight Radio Systems Operating in the Bands 71-76 GHz and 81-86 GHz. May 2017.

[15]	MIIT, "Notice on wireless technical applications with very low transmission power (short distance) in 60GHz. 2006-082" (关于60GHz 频段微功率 (短距离) 无线电技术应用有关问题的通知 2006-082) http://www.miit.gov.cn/n1146295/n1146592/n1146754/n1235566/n1235603/n1235609/n1235611/c3176263/part/3176264.pdf

[16]	MIIT, "Technical requirement of wireless devices with very low transmission power (short distance 2005-423)." (微功率 (短距离) 无线电设备的技术要求 2005-043) http://www.miit.gov.cn/n1146295/n1146592/n1146754/n1235566/n1235603/n1235609/n1235611/c3176241/part/3176242.pdf

[17]	ARIB STD-B43 v2.1, "PORTABLE MILLIMETER-WAVE DIGITAL TRANSMISSION SYSTEM FOR TELEVISION PROGRAM CONTRIBUTION," Jan. 2018.

[18]	ARIB STD-T69 v4.0, "LOW POWER DATA COMMUNICATION SYSTEM/MILLIMETER-WAVE VIDEO TRANSMISSION EQUIPMENT," Sept. 2016.

[19]	ARIB STD-T117 v1.0, "LOW POWER DATA COMMUNICATION SYSTEM/60 GHz-BAND WIRELESS LAN FOR VERY HIGH THROUGHPUT DATA COMMUNICATIONS," Sept. 2016.

[20]	ARIB STD-T48 v2.2, "MILLIMETER-WAVE RADAR EQUIPMENT FOR SPECIFIED LOW POWER RADIO STATION," Dec. 2015.

[21]	ARIB STD-T111 v1.1, "79 GHz BAND HIGH-RESOLUTION RADAR," Mar. 2017.

[22]	Republic of Korea Communications Agency. Republic of Korea Frequency Allocations Status. May 2018.

[23]	Republic of Korea National Radio Research Agency. National Radio Research Agency Publication 2014-12. July 02, 2014.

[24]	Republic of Korea Ministry of Science and ICT. Ministry of Science and ICT Publication 2018-3. July 02, 2014.

[25]	Republic of Korea Ministry of Science and ICT. Ministry of Science and ICT Publication 2018-4. July 02, 2014.

[26]	NFAP-18	"National Frequency Allocation Plan – 2018", Government of India Ministry of Communications.

[27]	IMDA (Infocomm Media Development Authority), IMDA TS SRD, April 2018. https://www.imda.gov.sg/-/media/imda/files/regulation-licensing-and-consultations/ict-standards/telecommunication-standards/radio-comms/imdatssrd.pdf?la=en

[28]	ACMA (Australian Communications and Media Authority), "Radiocommunications (Low Interference Potential Devices) Class Licence 2015", 2018. https://www.legislation.gov.au/Details/F2018C00500

[29]	No. 2018-71 Ministry of Science and ICT notice

[30]	No. 2018-26 National Radio Research Agency notice

[31]	No. 2018-36 Ministry of Science and ICT notice

[32]	Ministry of Transportation and Communications, "TABLE OF RADIO FREQUENCY ALLOCATIONS OF THE REPUBLIC OF CHINA", Feb. 2017, http://www.motc.gov.tw/post/home.jsp?id=364&parentpath=0.

[33]	China, "People's republic of China Regulations on the Radio Frequency Allocation", The People's Posts and Telecommunications Press, 2010.

[34]	IMDA (Infocomm Media Development Authority), "Singapore Spectrum Allocation Chart", https://www.imda.gov.sg/-/media/imda/files/regulation-licensing-and-consultations/frameworks-and-policies/spectrum-management-and-coordination/spectrumchart.pdf?la=en

[35]	ACMA, "Australian Radiofrequency Spectrum Plan 2017-including general information", https://www.acma.gov.au/theacma/australian-radiofrequency-spectrum-plan-spectrum-planning-acma

[36]	The Wireless Planning & Coordination (WPC) Wing of the Ministry of Communication & Information Technology, "License Exemption for SRD Device GSR 1047(E) dated 18/10/ 2018"

[37]	International Telecommunication Union, "ITU-R, The Radio Regulations," Edition of 2016, https://www.itu.int/pub/R-REG-RR (Edition of 2016)

[38]	3GPP TR 22.826: "Study on communication services for critical medical applications"

[39]	3GPP TR 33.853: "Technical report on key issues and potential solutions for Integrity protection of the User Plane"

[40]	CEPT ECC, "ERC Recommendation 12-12: Radio frequency channel, arrangement for Fixed Service Systems operating in the band 55.78 to 57.0 GHz," January 2015.

[41]	CEPT ECC, "THE EUROPEAN TABLE OF FREQUENCY ALLOCATIONS AND APPLICATIONS IN THE FREQUENCY RANGE 8.3 kHz to 3000 GHz (ECA TABLE)", March 2019.

[42]	CEPT ECC, "ERC Recommendation 70-03: Relating to the use of Short Range Devices (SRD)," June 2019.

[43]	CEPT ECC, "Recommendation 02-05: Unwanted Emissions," March 2012.

[44]	CEPT ECC, "ERC Recommendation 74-01: Unwanted emissions in the spurious domain," May 2019.

[45]	Official Journal of the European Union, " COMMISSION IMPLEMENTING DECISION (EU) 2017/1483 of August 2017 amending Decision 2006/771/EC on harmonisation of the radio spectrum for use by short-range devices and repealing Decision 2006/804/EC," August 2017.

[46]	CEPT ECC, "ECC RECOMMENDATION (05)02: USE OF THE 64-66 GHz FREQUENCY BAND FOR FIXED SERVICE," February 2009.

[47]	CEPT ECC, "ECC Decision (09)01: Harmonised use of the 63.72-65.88 GHz frequency band for Intelligent Transport Systems (ITS)," March 2009.

[48]	CEPT ECC, "ECC RECOMMENDATION (05)07: RADIO FREQUENCY CHANNEL ARRANGEMENTS FOR FIXED SERVICE SYSTEMS OPERATING IN THE BANDS 71-76 GHz AND 81-86 GHz," May 2013.

[49]	CEPT ECC, "ECC Decision (04)03: The frequency band 77-81 GHz to be designated for the use of Automotive Short Range Radars," March 2015.

[50]	CEPT ECC, "EC Decision 2004/545/EC: Commission Decision of 8 July 2004 on the harmonisation of radio spectrum in the 79 GHz range for the use of automotive short-range radar equipment in the Community (notified under document number C(2004) 2591)(Text with EEA relevance)," August 2004.

[51]	CEPT ECC, "ECC Recommendation (18)02: Radio frequency channel/block arrangements for Fixed Service systems operating in the bands 92-94 GHz, 94.1-100 GHz, 102-109.5 GHz and 111.8-114.25 GHz," September 2018.

[52]	ETSI, " ETSI EN 302 217-2: Fixed Radio Systems; Characteristics and requirements for point-to-point equipment and antennas; Part 2: Digital systems operating in frequency bands from 1,3 GHz to 86 GHz; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU," June 2016.

[53]	CEPT ECC, "ECC RECOMMENDATION (05)07: RADIO FREQUENCY CHANNEL RRANGEMENTS FOR FIXED SERVICE SYSTEMS OPERATING IN THE BANDS 71-76 GHz AND 81-86 GHz," May 2013.

[54]	CEPT ECC, "ECC Report 80: ENHANCING HARMONISATION AND INTRODUCING FLEXIBILITY IN THE SPECTRUM REGULATORY FRAMEWORK," March 2006.

[55[	Provisional Final Acts WRC-19, https://www.itu.int/pub/R-ACT-WRC.13-2019/en.

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

B	transmission bandwidth

G	antenna gain

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BS	Base Station

CEPT	European Conference of Postal and Telecommunications Administrations

ECC	Electronic Communications Committee

ECO	European Communications Office

EESS	Earth Exploration Satellite Service

EIRP	Equivalent Isotropic Radiated Power

ERC	European Research Council

FDD	Frequency Duplex Division

FS	Fixed Service

FSS	Fixed Satellite Service

HDMI	High-Definition Multimedia Interface

IAB	Integrated Access Backhaul

ICASA	Independent Communication Authority of South Africa

IMT	International Mobile Telecommunications

ISM	Industrial, Scientific and Medical

ISS	International Space Station

ITU	International Telecommunication Union

LBT	Listen Before Talk

MCOT	Maximum Channel Occupancy Time

MSS	Mobile Satellite Service

NR	New Radio

OCB	Occupied Bandwidth

OOBE	Out-Of-Band Emission

PSD	Power Spectral Density

PTP	Point to point

RAS	Radio Astronomy Service

RLAN	Radio Local Area Network

RLS	Radio Location Service

RNS	Radio Navigation Satellite

RNSS	Radio Navigation Satellite Service

SI	Study Item

SID	Study Item Description

SRD	Short Range Device

SRS	Space Radiocomunication Stations

TDD	Time Duplex Division

UE	User Equipment

V2X	Vehicle to Everything

WAN	Wide Area Network

WAS	Wireless Access System

# 4	Operational Requirements

## 4.1	Overview of Global Spectrum Availability

In 2015, the international telecommunication union (ITU) proposed 11 millimetre-wave bands between 24 and 86 GHz to be studied towards WRC-19 for possible identification for IMT. See Table 4.1-1 with the bands studied under AI 1.13 of WRC-19 [5].

Table 4.1-1 Bands studied under AI 1.13 of WRC-19

| Candidate frequency bands which have allocations to the mobile service on a primary basis | Candidate frequency bands which required a co-primary mobile allocation |
| --- | --- |
| 24.25-27.5 GHz [Note1] | 31.8-33.4 GHz, |
| 37-40.5 GHz | 40.5-42.5 GHz |
| 42.5-43.5 GHz | 47-47.2 GHz |
| 45.5-47 GHz |  |
| 47.2-50.2 GHz |  |
| 50.4-52.6 GHz |  |
| 66-76 GHz |  |
| 81-86 GHz |  |
| NOTE 1: When conducting studies in the band 24.5-27.5 GHz, to take into account the need to ensure the protection of existing earth stations and the deployment of future receiving earth stations under the EESS (space-to-Earth) and SRS (space-to-Earth) allocation in the frequency band 25.5-27 GHz |  |

WRC-19 (28 October to 22 November 2019) identified 66-71 GHz for IMT via footnote 5.J113 [55]: In Regions 1 and 3, and Brazil, and Region 2 the frequency band 66-71 GHz is identified for use by administrations wishing to implement the terrestrial component of International Mobile Telecommunications (IMT). This identification does not preclude the use of this frequency band by any application of the services to which it is allocated and does not establish priority in the Radio Regulations. Resolution COM4/7 (WRC-19) applies.

Table 4.1-2 provides information on the allocation within the frequency range 52.6 GHz to 116 GHz in ITU Radio Regulation [37]. The column with comments contains (a subset of) information on protection requirements for incumbent services. For the full details please refer to the Radio Regulations

Table 4.1-2: Frequency bands in the range 52.6 to 116 GHz in radio regulation

| Frequency band (GHz) | Allocated to Mobile Service on a primary basis | Allocated to Fixed Service on a primary basis | Comments |
| --- | --- | --- | --- |
| 52.6-54.25 | No | No | EESS (passive) and SRS (passive), All emissions are prohibited in this band, footnote 5.340 |
| 54.25-55.78 | No | No | EESS (passive) and SRS (passive) |
| 55.78-59 | Yes | Yes | EESS (passive) and SRS (passive) This band available for high-density applications in the fixed service, footnote 5.547 |
| 59-59.3 | Yes | Yes | EESS (passive) and SRS (passive) Radiolocation |
| 59.3-64 | Yes | Yes | Radiolocation |
| 64-65 | Yes | Yes | This band available for high-density applications in the fixed service, footnote 5.547 |
| 65-66 | Yes | Yes | This band available for high-density applications in the fixed service, footnote 5.547 |
| 66-71 | Yes | No | Identified for IMT via footnote 5.J113 [55] |
| 71-76 | Yes | Yes | This band is mostly used by Fixed Services. |
| 76-81 | No | No | Radiolocation |
| 81-86 | Yes | Yes | This band is mostly used by Fixed Services. |
| 86-92 | No | No | EESS (passive) and SRS (passive), All emissions are prohibited in this band, footnote 5.340 |
| 92-94 | Yes | Yes | Radiolocation |
| 94-94.1 | No | No | Radiolocation |
| 94.1-95 | Yes | Yes | Radiolocation |
| 95-100 | Yes | Yes | Radiolocation |
| 100-102 | No | No | EESS (passive) and SRS (passive), All emissions are prohibited in this band, footnote 5.340 |
| 102-105 | Yes | Yes | N/A |
| 105-109.5 | Yes | Yes | SRS (passive) |
| 109.5-111.8 | No | No | EESS (passive) and SRS (passive), All emissions are prohibited in this band, footnote 5.340 |
| 111.8-114.25 | Yes | Yes | SRS (passive) |
| 114.25-116 | No | No | EESS (passive) and SRS (passive), All emissions are prohibited in this band, footnote 5.340 |

Within the range 52.6 to 116 GHz, the frequency bands 66-76 GHz (including 66-71 and 71-76 GHz) and 81-86 GHz were studied under WRC-19 Agenda Item 1.13 for potential IMT identification. Results of sharing and compatibility studies, potential technical and regulatory conditions are included in Draft CPM Report [6], and the final decisions were made in WRC-19 with respect to IMT identification or no IMT identification, along with the corresponding technical and regulatory conditions.

For 71-76 GHz, studies were carried out for the FS, RLS and FSS (space-to-Earth) indicating that sharing with FS and FSS is feasible. However, additional limits of the IMT BS and UE unwanted emissions is needed to protect RLS in the adjacent frequency band 76-81 GHz.

For 81-86 GHz, studies were carried out for the FS, FSS (Earth-to-space), RAS (in band and adjacent band), EESS (passive) and RLS. Studies were not needed for the SRS (passive), as this service is dealing with sensors around other planets and no interference issue is expected. Studies were also not carried out for the MSS. The results of those studies indicate that sharing with FS, FSS and RAS (in band and adjacent band) is feasible. Notice that additional limits of the IMT BS and UE unwanted emissions would be needed to ensure protection of EESS (passive) in the adjacent frequency band 76-81 GHz and RLS in the adjacent frequency band 86-82 GHz. Table 4.1-3 and Table 4.1-4 depicts the limits of the IMT BS and UE maximum unwanted emissions levels according to the individual compatibility studies.

Table 4.1-3: Limits of unwanted emission levels from IMT BS and UE operating on 71-76 and 81-86 GHz frequency bands into the 76-81 GHz frequency band according to WRC-19 compatibility studies

| Limits of unwanted emissions into 76-81 GHz from IMT stations | 76-77 GHz dB(W/200 MHz) | 77-81 GHz dB(W/200 MHz) |
| --- | --- | --- |
| BS | [TBD/−29.6/−31.5/] | [TBD/−33] |
| UE | [TBD/−20] | [TBD/−35] |

Table 4.1-4: Limits of unwanted emission levels from IMT BS and UE operating on 81-86 GHz into 86-92 GHz frequency band according to WRC-19 compatibility studies

| Limits of unwanted emissions into 86-92 GHz from IMT stations | dB(W/100 MHz) |
| --- | --- |
| BS | [TBD/-43.6/-49.9/-31.3/-20] |
| UE | [TBD/-43.5/-49.8/-31.2/-19.9] |

## 4.2	Country Specific Spectrum Availability and Spectrum Regulatory Requirements

### 4.2.1	Country Specific Licensing situation

Table 4.2-1 and 4.2-2 provide an overall summary of the current licensing situation for data communication (Mobile and Fixed, excluding radar based telemetry) between frequencies 52.6GHz and 100GHz for various countries under ITU region 1, 2, and 3. Symbol 'U', and 'L', represent unlicensed spectrum and licensed spectrum (including light licensing and other licensing forms), respectively. Notice that Table 4.2-1 and 4.2-2 does not include information about the service allocated to each frequency band. It neither includes information about the frequency bands studied in AI 1.13 WRC-19 which are listed in Table 4.1-1. Also, it is worthwhile noticing that type of licensing for each frequency band is a national decision.

Table 4.2.1-1: Current Licensing situation for various countries between frequency 52.6GHz and 71GHz

| Region | Country | Frequency (GHz) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 52.6-54.25 | 54.25-55.78 | 55.78-56.9 | 56.9-57 | 57-58.2 | 58.2-59 | 59-59.3 | 59.3-64 | 64-65 | 65-66 | 66-71 |
| ITU Region 1 | Europe/CEPT |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Israel |  |  |  |  |  |  |  |  |  |  |  |
|  | South Africa |  |  |  |  | U (Mobile) |  |  |  | U (Mobile) |  |  |
| ITU Region 2 | USA |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Canada |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Brazil |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Mexico |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
| ITU Region 3 | China |  |  |  |  |  |  | U (Mobile) |  |  |  |  |
|  | Japan |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Korea |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | India |  |  |  |  |  |  |  |  |  |  |  |
|  | Taiwan |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Singapore |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Australia |  |  |  |  | U (Mobile) |  |  |  |  |  |  |

Table 4.2.1-2: Current Licensing situation for various countries between frequency 71GHz and 100GHz

| Region | Country | Frequency (GHz) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 71-74 | 74-76 | 76-77 | 77-81 | 81-84 | 84-86 | 86-92 | 92-94 | 94-94.1 | 94.1-95 | 95-100 |
| ITU Region 1 | Europe/CEPT | L (Fixed) |  |  |  | L (Fixed) |  |  | L (Fixed) | L (Fixed) |  |  |
|  | Israel |  |  |  |  |  |  |  |  |  |  |  |
|  | South Africa | L (Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
| ITU Region 2 | USA | L (Fixed /Mobile) |  |  |  | L (Fixed /Mobile) |  |  | L (Fixed /Mobile) |  | L (Fixed /Mobile) |  |
|  | Canada | L (Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
|  | Brazil | L (Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
|  | Mexico | L (Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
| ITU Region 3 | China |  |  |  |  |  |  |  |  |  |  |  |
|  | Japan | L (Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
|  | Korea | L (Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
|  | India |  |  |  |  |  |  |  |  |  |  |  |
|  | Taiwan | L(Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
|  | Singapore | L (Fixed) |  |  |  | L (Fixed) |  |  |  |  |  |  |
|  | Australia |  |  |  |  |  |  |  |  |  |  |  |

====================<Unchanged text is omitted>========================================