3GPP TSG-RAN WG4 Meeting #96-e						 R4-2012264

Electronic Meeting, 17 August – 28 August, 2020

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0925 | rev | 2 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.133: PRS RSTD requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2020-8-6 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | NR DL RSTD measurement requirements needs to be defined in TS 38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change #1: NR DL RSTD measurement requirements are introduced in Section 9.9. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR DL RSTD measurement requirements are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Start of Change 1 >>

## 9.9	  NR measurements for positioning

### 9.9.2	RSTD measurements

### 9.9.2.1	Introduction

The requirements in clause 9.9.2 shall apply provided the UE has received NR-DL-TDOA-RequestLocationInformation message from LMF via LPP [34] requesting the UE to report DL RSTD measurements defined in TS 38.215 [4].

### 9.9.2.2	Requirements Applicability

The requirements in clause 9.9.2 apply, provided:

-	PRS-RSTD related side conditions given in clause 10.1.23 for FR1 and FR2 are fulfilled, for a corresponding Band.

### 9.9.2.3	Measurement Capability

UE PRS RSTD measurement capability is as reported by the UE according to TS 37.355[34].

### 9.9.2.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the periodic measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

The reported RSTD measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in sections 10.1.23.3 for RSTD.

### 9.9.2.5	Measurements Period Requirements

When physical layer receives last of NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Section 9.9.2.3) DL RSTD measurements, defined in TS 38.215 [4], during TRSTD,Total defined further in this section.

When measurement gaps and processing time T have overlap between different positioning frequency layers, TRSTD,Total is defined as:

TRSTD,Total=i=1LTRSTD,i+ L-1*maxTeffect,i

Where ,

i is the index of positioning frequency layer,

L is total number of positioning frequency layers, and Teffect,i is the periodicity of PRS-RSTD measurement in positioning frequency layer i as defined further in this section.

Editor’s note: FFS the RSTD measurement period when measurement gaps and processing time T do not have overlap between different positioning frequency layers.

TPRS-RSTD,i is the measurement period for PRS RSTD measurement in i positioning frequency layer as specified below

TPRS-RSTD,i=CSSFPRS,i*NRxBeam,i*NPRS,islotN'LPRS,iN*Nsample-1*Teffect,i+Tlast ,

where:

- NRxBeam,i is the UE Rx beam sweeping factor. In FR1, NRxBeam,i = 1; and in FR2  NRxBeam,i = [8].

- CSSFPRS,i is the carrier-specific scaling factor for the positioning frequency layer i as defined in clause 9.1.5.2 as CSSFwithin_gap,i.

- Nsample is the number of PRS RSTD samples and Nsample= [4].

- Tlast is the measurement duration for the last PRS RSTD sample, including the sampling time and processing time, Tlast = Ti + LPRS,i ,

- Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i

- Tavailable_PRS,i= LCMTPRS,i,MGRPi, the least common multiple between TPRS,i and MGRPi.

- LPRS,i is the time duration  as defined in clause 5.1.6.5 of TS 38.214 [26, 5.1.6.5].

- NPRS,islot is the maximum number of DL PRS resources in positioning frequency layer i configured in a slot.

- {N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth supported by UE as specified in clause 4.2.7.2 of TS 38.306 [14].

- N’ is UE capability for number of DL PRS resources that it can process in a slot as specified in clause 4.2.7.2 of TS 38.306 [14].

If positioning frequency layer i has more than one DL PRS resource set with different PRS periodicities, the maximum PRS periodicity among DL PRS resource sets is used to derive the measurement period of that positioning frequency layer.

If handover occurs while RSTD measurements are being performed then the UE shall complete the on-going positioning measurement session. The UE shall also meet the RSTD measurement and accuracy requirements. However, in this case the RSTD measurement period TRSTD,total.HO shall be as follows:

TRSTD, total,HO=TRSTD, Total+K*Teffect+THO

Where,

- K is the number of times handover occurs during TPRS-RSRP,total.HO;

- Teffect is the largest Teffect,i among all PRS layers;

- THO is the time during which the RSTD measurement may not be possible due to handover; it can be up to Tinterrupt as defined in clause 6.1.

## << End of Change 1 >>