3GPP TSG-RAN4 Meeting #96-e	R4-2012282

# Electronic Meeting, 17 – 28 August, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1083 | rev | 2 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for measurement requirements for PRS-RSRP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2020-07-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | PRS-RSRP measurement requirements are not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define PRS-RSRP measurement requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Core requirements for the WI objective are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 9.9.3	PRS-RSRP measurements

#### 9.9.3.1	Introduction

The requirements in clause 9.9.3.5 shall apply provided the UE has received a message from LMF via LPP [34] requesting the UE to report PRS-RSRP measurements defined in TS 38.215 [4].

#### 9.9.3.2	Requirements applicability

The requirements in clause 9.9.3 apply, provided:

-	PRS-RSRP related side conditions given in clause 10.1.24 are met for a corresponding Band.

#### 9.9.3.3	Measurement Capability

UE PRS-RSRP measurement capability is as reported by the UE according to TS 37.355.

#### 9.9.3.4	Measurement Reporting Requirements

This requirement assumes that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

The reported PRS-RSRP measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in sections 10.1.24.3.

Editor’s note: the measurement reporting requirements for aperiodic reports are FFS.

The PRS-RSRP measurement accuracy for all measured PRS resources shall be fulfilled according to the accuracy as specified in the clauses 10.1.24.

#### 9.9.3.5	Measurement Period Requirements

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Section 9.9.3.3) PRS-RSRP measurements, defined in TS 38.215 [4], from configured PRS resources for configured TRPs on configured PRS frequency layers, within TPRS-RSRP,total ms.

If measurement gaps and processing time T have overlap between different frequency positioning frequency layers,

TPRS-RSRP, total=i=1LTPRS-RSRP,i+L-1*maxTeffect,i

Editor’s note: FFS the PRS-RSRP measurement period when measurement gaps and processing time T do not have overlap between different positioning frequency layers.

where i is the index of PRS frequency layer,

TPRS-RSRP,i is the measurement period for PRS-RSRP measurements in frequency layer i as defined further in this section,

L is total number of positioning frequency layers, and Teffect,i is the periodicity of PRS-RSRP measurement in frequency layer i as defined further in this section.

TPRS-RSRP,i=CSSFi*NRxBeam,i*NPRS,islotN'LPRS,iN*Nsample-1*Teffect,i+Tlast

where

CSSFi is the carrier specific scaling factor for PRS-RSRP measurements specified in clause 9.1.5.2,

NRxBeam,i is the scaling factor for Rx beam sweeping, and NRxBeam,i=1 if PRS layer i is in FR1 and NRxBeam,i=8 if PRS layer i is in FR2,

LPRS,i is the time duration specified in clause 5.1.6.5 of TS 38.214 [26, section 5.1.6.5],

NPRS,islot is the maximum number of DL PRS resources of frequency layer i configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth supported by UE as specified in clause 4.2.7.2 of TS 38.306 [14],

N’ is UE capability for number of DL PRS resources that it can process in a slot as specified in clause 4.2.7.2 of TS 38.306 [14],

Nsample is the number of PRS-RSRP measurement samples and Nsample= [4],

Teffect,i is periodicity of PRS-RSRP measurement in frequency layer i:

Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i

where Tavailable_PRS,i= LCMTPRS,i,MGRPi, the least common multiple between TPRS,i and MGRPi

TPRS,i is the maximum PRS resource periodicity among all PRS resources in frequency layer i,

MGRPi is the measurement gap pattern periodicity in frequency layer i.

If handover occurs while PRS-RSRP measurements are being performed then the UE shall complete the ongoing positioning measurement session. The UE shall also meet the PRS-RSRP measurement and accuracy requirements. However in this case the PRS-RSRP measurement period TPRS-RSRP,total.HO shall be as follows:

TPRS-RSRP, total,HO=TPRS-RSRP, total+K*Teffect+THO   ms

where

K is the number of times handover occurs during TPRS-RSRP,total.HO;

Teffect is the largest Teffect,i among all PRS layers;

THO is the time during which the PRS-RSRP measurement may not be possible due to handover; it can be up to Tinterrupt as defined in clause 6.1.

<End of Change 1>