3GPP TSG-RAN4 WG4 Meeting # 96-e	R4-2011728

# Electronic meeting, August 17- 28, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0468 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for intra-band UL CA non-contiguous CA requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-Core |  |  |  |  |  | Date: |  |  | 2020-07-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For non-contiguous RB allocation of contiguous UL CA, note1 and note2 on outer1 and outer2 allocations MPR only take effect on Pi/2 BPSK and QPSK, the MPR for high order modulation has different limitation factor. For congtiguous RB allocation, case for RBs only allocate on one CC is not included. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Note1 in table 6.2A.2.4-2 is only valid for Pi/2 BPSK and QPSK with 2dB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For non-contiguous RB allocation of contiguous UL CA , MPR for high order modulation is not correct. Contiguous RB allocation definition is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR’s revision history: |  |
| --- | --- |

<Start of change>

### 6.2A.2	UE maximum output power reduction for CA

#### 6.2A.2.1	Void

#### 6.2A.2.2	Void

#### 6.2A.2.3	UE maximum output power reduction for Inter-band CA

For inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in clause 6.2.2 apply for each uplink component carrier.

#### 6.2A.2.4	UE maximum output power reduction for Intra-band contiguous CA

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2A.1.4-1 with contiguous RB allocation is specified in Table 6.2A.2.4-1 for UE power class 3 CA bandwidth classes B and C.

In case the modulation format is different on different component carriers then the MPR is determined by the rules applied to higher order of those modulations.

Table 6.2A.2.4-1: Contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | QPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | 16QAM | 1.5 | 3.5 | 2.5 | 7 |
|  | 64QAM | 3.0 | 4.0 | 5 | 7 |
|  | 256QAM | 5.5 | 6.0 | 7 | 7.5 |
| CP-OFDM | QPSK | 2.0 | 4.0 | 3.5 | 8 |
|  | 16QAM | 2.5 | 4.0 | 3.5 | 8 |
|  | 64QAM | 3.5 | 4.0 | 5 | 8 |
|  | 256QAM | 6.5 | 6.5 | 7 | 8 |

For CA bandwidth class B and bandwidth class C with contiguous RB allocation, the following parameters are defined to specify valid RB allocation ranges for Inner and Outer RB allocations:

Contiguous RB allocation is defined as RBStart1 + LCRB1 = NRB1, and RBStart2 = 0, where RBStart1, LCRB1, and NRB1 are for CC1, RBStart2, LCRB2, and NRB2 are for CC2, CC1 is the component carrier with lower frequency.

In contiguous CA, a contiguous allocation is an inner allocation if

RBStart,Low  ≤  RBStart_CA  ≤  RBStart,High, and NRB_alloc  ≤  ceil(NRB,agg /2),

where

RBStart,Low = max(1, floor(NRB_alloc /2)) , RBStart,High = NRB,agg – RBStart,Low – NRB,alloc,

with

NRB_alloc= (NRB1 - RBStart1)∙ 2^µ1 + (RBStart2 + LCRB2 ) ∙ 2^µ2, NRB,agg=NRB1∙2^µ1+ NRB2∙2^µ2.

If LCRB1 =0, RBStart_CA = NRB1∙2^µ1+ RBStart2∙2^µ2,

if LCRB1 > 0, RBStart_CA = RBStart1∙2^µ1.

A contiguous allocation that is not an Inner contiguous allocation is an Outer contiguous allocation

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2A.1.5-1 with non-contiguous RB allocation is specified in Table 6.2A.2.4-2 for UE power class 3 CA bandwidth classes B and C.

Table 6.2A.2.4-2: non-contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer11 | Outer22 | inner | Outer11 | Outer22 |
| DFT-s-OFDM | Pi/2 BPSK | 2 | 5.5 | 11.5 | 2.5 | 6 | 13 |
|  | QPSK | 2 | 5.5 |  | 2.5 | 6 |  |
|  | 16QAM | 2.5 | 5.5 |  | 3 | 6 |  |
|  | 64QAM | 4.5 | 6 |  | 5 | 6 |  |
|  | 256QAM | 6 | 6.5 |  | 6.5 | 6.5 |  |
| CP-OFDM | QPSK | 2.5 | 6.5 | 12 | 3.5 | 7 | 14 |
|  | 16QAM | 3 | 7 |  | 3.5 | 7 |  |
|  | 64QAM | 5 | 7 |  | 5 | 7 |  |
|  | 256QAM | 7.5 | 7.5 |  | 7.5 | 7.5 |  |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz |  |  |  |  |  |  |  |

For CA bandwidth classes B and C with non-contiguous RB allocation, the following parameters are defined to specify valid RB allocation ranges for Inner, Outer1 and Outer2 RB allocations:

Non-Contiguous RB allocation is defined as RBStart1 + LCRB1 < NRB1, or RBStart2 > 0 , where RBStart1, LCRB1, and NRB1 are for CC1, RBStart2, LCRB2, and NRB2 are for CC2, CC1 is the component carrier with lower frequency.

In contiguous CA, a non-contiguous RB allocation is a non-contiguous Inner RB allocation if the following conditions are met:

RBStart,Low  ≤  RBStart_CA  ≤  RBStart,High and NRB_alloc ≤  ceil((BWChannel_CA / 3 – BWgap ) / 0.18MHz),

where

NRB_alloc = (NRB1 - RBStart1)∙ 2^µ1 + (RBStart2 + LCRB2 ) ∙ 2^µ2, RBStart_CA = RBStart1∙2^1

RBStart,Low = max(1, floor(NRB_alloc + (BWgap – BWGB,low)/0.18MHz))

RBStart,High = floor((BWChannel_CA – 2 ∙ BWgap – BWGB,low)/0.18MHz – 2 ∙ NRB_alloc)

BWGB,low =Foffset,low – (NRB1∙12+1)∙SCS1/2

BWgap is the bandwidth of the gap between NRB1 and NRB2 possible allocations of CC1 and CC2 respectively.

In contiguous CA, a non-contiguous RB allocation is a non-contiguous outer 1 RB allocation if the following conditions are met:

RBStart,Low  ≤  RBStart_CA  ≤  RBStart,High and NRB_alloc ≤  ceil((3 BWChannel_CA / 5 – BWgap) / 0.18MHz)

where

RBStart,Low = max(1, 2 ∙ NRB_alloc – floor( (BWChannel_CA – 2 ∙ BWgap + BWGB,low)/0.18MHz)),

RBStart,High = floor((2 ∙ BWChannel_CA – 3 ∙ BWgap – BWGB,low) / 0.18MHz – 3 ∙ NRB_alloc)

NRB_alloc , RBStart_CA , BWgap and BWGB,low are as defined for the Inner region.

In contiguous CA, a non-contiguous allocation is an Outer 2 allocation if it is neither an non-contiguous Inner allocation nor an Outer 1 allocation.

<End of change>