3GPP TSG RAN Meeting #89e																		RP-201694

Electronic Meeting, September 14 - 18, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.331 | CR | 5962 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of REL-16 parameter value for Access Stratum release indicator |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 03/09/2020 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the RRC signalling, the "Access stratum release indicator" IE needs to be updated to allow UE to indicate to the Network that its Access Stratum is REL-16. The Access Stratum release indicator is defined in TS 25.306, subclause 4.10 as follows: “Access stratum release indicator: This is defined as the release of the UTRA layer 1, 2, and 3 specifications that is applicable for the UE e.g. R'99, Rel-4.” REL-16 includes a new feature ‘NR SRVCC to UTRAN’ (see TS 25.300) and functionality was specified to avoid impacts to 3G RRC. In case of continued access of UTRAN cell after SRVCC call ends, e.g. due to poor NR coverage, it may be useful to have a REL-16 release stratum indicator. Further this is considered as cleaner approach. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The "Access stratum release indicator" IE in 10.2.39, 10.3.3.42 and 10.3.3.42o is updated with REL-16 value. The corresponding update is made in the ASN.1 in 11.3. Impact Analysis Impacted functionality: UE Access Stratum Release Indicator. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not approved, the UE cannot indicate to the Network that its Access Stratum is REL-16. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.39, 10.3.3.42, 10.3.3.42o, 11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | 25.306 CR 0525 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This issue is inherited from RAN6 and was discussed on the RAN6 reflector. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modification

### 10.2.39	RRC CONNECTION REQUEST

RRC Connection Request is the first message transmitted by the UE when setting up an RRC Connection to the network.

RLC-SAP: TM

Logical channel: CCCH

Direction: UE  UTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| Radio Bearer IEs |  |  |  |  |  |
| Predefined configuration status information | MP |  | Boolean | TRUE indicates the UE has all pre- configurations stored with the same value tag as broadcast in the cell in which the RRC connection establishment is initiated | REL-5 |
| UE information elements |  |  |  |  |  |
| Initial UE identity | MP |  | Initial UE identity 10.3.3.15 |  |  |
| Establishment cause | MP |  | Establishment cause 10.3.3.11 |  |  |
| Protocol error indicator | MD |  | Protocol error indicator 10.3.3.27 | Default value is FALSE |  |
| >UE Specific Behaviour Information 1 idle | OP |  | UE Specific Behaviour Information 1 idle 10.3.3.51 | This IE shall not be included in this version of the protocol |  |
| Domain indicator | MP |  | CN domain identity 10.3.1.1 |  | REL-6 |
| CS Call type | CV-CS-Domain |  | Enumerated (speech, video, other) | One spare value is needed | REL-6 |
| UE capability indication | OP |  | Enumerated (HS-DSCH, HS-DSCH+E-DCH) | Absence of this IE implies that neither HS-DSCH nor E-DCH are supported by the UE | REL-6 |
| MBMS Selected Services | OP |  | MBMS Selected Services Short 10.3.9a.7d |  | REL-6 |
| UE Mobility State Indicator | MD |  | Enumerated (High-mobilityDetected) | Absence of this IE implies that, according to [4] either the High mobility state is not applicable or it has not been detected by the UE. | REL-7 |
| Support for F-DPCH | CV-SupportForHS |  | Enumerated (TRUE) | Presence of this information element indicates that F-DPCH is supported by the UE. Absence of this information element indicates that F-DPCH is not supported | REL-6 |
| Support for Enhanced F-DPCH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support enhanced F-DPCH | REL-7 |
| HS-PDSCH in CELL_FACH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-PDSCH reception in CELL_FACH state. Note 1 | REL-7 |
| MAC-ehs support | OP |  | Enumerated (TRUE) | The presence of this IE indicates that UE supports MAC-ehs, octet aligned transport block table, the use of special value of HE field to indicate end of an SDU for RLC AM and different HS-SCCHs in contiguous TTIs. The absence of this IE indicates that the UE does not support either MAC-ehs, octet aligned transport block table or the use of special value of HE field to indicate end of an SDU for RLC AM or different HS-SCCHs in contiguous TTIs. | REL-7 |
| DPCCH Discontinuous Transmission support | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support DPCCH Discontinuous Transmission | REL-7 |
| Support of common E-DCH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support E-DCH enhanced random access in CELL_FACH state and Idle mode. Note 1 | REL-8 |
| Multi cell support | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support dual cell operations on adjacent frequencies | REL-8 |
| Dual cell MIMO support | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support dual cell with MIMO operation on adjacent frequencies. | REL-9 |
| More than two cell or MIMO mode with four transmit antennas support | OP |  | Enumerated (higher rate, lower rate) | For FDD, the absence of this IE indicates that the UE does not support multi-cell operation on more than two cells. For 1.28 Mcps TDD, the absence of this IE indicates that the UE does not support MU-MIMO. If present, higher rate indicates uplink and downlink MU-MIMO, lower rate indicates uplink MU-MIMO | REL-10 |
|  |  |  |  | For FDD, the absence of this IE indicates that the UE does not support multi-cell operation on more than two cells or MIMO mode with four transmit antennas. For 1.28 Mcps TDD, the absence of this IE indicates that the UE does not support MU-MIMO. If present, higher rate indicates uplink and downlink MU-MIMO, lower rate indicates uplink MU-MIMO | REL-11 |
| Pre-redirection info | OP |  | Pre-redirection info 10.3.3.25a | The presence of this IE indicates the UE support of radio access technologies that the UE could be directed to, and that the UE was not redirected from EUTRA. | REL-8 |
| Support of MAC-i/is | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support MAC-i/is operation. | REL-8 |
| Support of SPS operation | OP |  | Enumerated (TRUE) | For 1.28 Mcps TDD only. The absence of this IE indicates that the UE does not support SPS operation. | REL-8 |
| Support for CS Voice over HSPA | OP |  | Enumerated (TRUE) | The IE indicates the UE’s support for CS Voice over HSPA, if set. | REL-8 |
| System Information Container Stored Indicator | OP |  | Enumerated (TRUE) | The presence of this IE indicates that the UE is accessing the cell using System Information stored in the variable SYSTEM_INFORMATION_CONTAINER | REL-9 |
| Support of the first Frequency Band | OP |  | Enumerated (TRUE) | The presence of this IE indicates the UE supports the 1st band broadcased in SIB5/5bis. For 1.28 Mcps TDD, the IE also indicates support band e if no band is broadcasted in SIB5/5bis. | REL-10 |
| Support of the second Frequency Band | OP |  | Enumerated (TRUE) | The presence of this IE indicates the UE supports the 2nd band broadcased in SIB5/5bis. For 1.28 Mcps TDD, the IE also indicates support band f if no band is broadcasted in SIB5/5bis. | REL-10 |
| CSFB Indication | OP |  | Enumerated (TRUE) | The presence of this IE indicates that the RRC Connection Request is due to CSFB call initiated in E-UTRA. |  |
| Measurement information elements |  |  |  |  |  |
| Measured results on RACH | OP |  | Measured results on RACH 10.3.7.45 |  |  |
| Access stratum release indicator | MP |  | Enumerated (REL-4, | Absence of the IE implies R99. The IE also indicates the release of the RRC transfer syntax supported by the UE. 3 spare values are needed. | REL-4 |
|  |  |  | REL-5, |  | REL-5 |
|  |  |  | REL-6, |  | REL-6 |
|  |  |  | REL-7, |  | REL-7 |
|  |  |  | REL-8, |  | REL-8 |
|  |  |  | REL-9, |  | REL-9 |
|  |  |  | REL-10, |  | REL-10 |
|  |  |  | REL-11, |  | REL-11 |
|  |  |  | REL-12, |  | REL-12 |
|  |  |  | REL-13, |  | REL-13 |
|  |  |  | REL-14, |  | REL-14 |
|  |  |  | REL-15, |  | REL-15 |
|  |  |  | REL-16) |  | REL-16 |

NOTE 1:	For 1.28 Mcps TDD, UE supporting HS-PDSCH in CELL_FACH always supports E-DCH enhanced random access in CELL_FACH state and Idle mode, and vice versa.

| Condition | Explanation |
| --- | --- |
| CS-Domain | This IE is mandatory present if the IE "Domain indicator" has the value "CS domain". Otherwise it is not needed. |
| SupportForHS | This IE is mandatory present if the UE supports HS-PDSCH, otherwise it is not needed. |

If the encoded message does not fill a transport block, the RRC layer shall insert padding according to subclause 12.1.

2nd Modification

#### 10.3.3.42	UE radio access capability

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Access stratum release indicator | MP |  | Enumerated (R99) | Indicates the release of the UE according to [35]. The IE also indicates the release of the RRC transfer syntax supported by the UE. |  |
|  | CV-not_rrc_connectionSetupComplete |  | Enumerated(REL-4, | 3 spare values are needed. | REL-4 |
|  |  |  | REL-5, |  | REL-5 |
|  |  |  | REL-6, |  | REL-6 |
|  |  |  | REL-7, |  | REL-7 |
|  |  |  | REL-8, |  | REL-8 |
|  |  |  | REL-9, |  | REL-9 |
|  |  |  | REL-10, |  | REL-10 |
|  |  |  | REL-11, |  | REL-11 |
|  |  |  | REL-12, |  | REL-12 |
|  |  |  | REL-13, |  | REL-13 |
|  |  |  | REL-14, |  | REL-14 |
|  |  |  | REL-15, |  | REL-15 |
|  |  |  | REL-16) |  | REL-16 |
| DL capability with simultaneous HS-DSCH configuration | CV-not_iRAT_HoInfo |  | Enumerated(32kbps, 64kbps, 128kbps, 384kbps) |  | REL-5 |
| PDCP capability | MP |  | PDCP capability 10.3.3.24 |  |  |
| RLC capability | MP |  | RLC capability 10.3.3.34 |  |  |
| Transport channel capability | MP |  | Transport channel capability 10.3.3.40 |  |  |
| RF capability FDD | OP |  | RF capability FDD 10.3.3.33 |  |  |
| RF capability TDD | OP |  | RF capability TDD 10.3.3.33b | One "TDD RF capability" entity shall be included for every Chip rate capability supported. |  |
|  |  | 1 to 2 |  | Note 1 | REL-4 |
| RF capability TDD 1.28 Mcps | CV-iRAT_HoInfo |  | RF capability TDD 1.28 Mcps 10.3.3.33c | Note 1 | REL-4 |
| Physical channel capability | MP |  | Physical channel capability 10.3.3.25 |  |  |
| UE multi-mode/multi-RAT capability | MP |  | UE multi-mode/multi-RAT capability 10.3.3.41 |  |  |
| Security capability | MP |  | Security capability 10.3.3.37 |  |  |
| UE positioning capability | MP |  | UE positioning capability 10.3.3.45 |  |  |
| Measurement capability | OP |  | Measurement capability 10.3.3.21 |  |  |
| Measurement capability TDD | OP |  | Measurement capability TDD 10.3.3.21b |  | REL-8 |
| Device type | MD |  | Enumerated (DoesNotBenefitFromBatteryConsumptionOptimisation) | Absence of this value means that the device does benefit from NW-based battery consumption optimisation. UE may set the value to DoesNotBenefitFromBatteryConsumptionOptimisation when it does not foresee to particularly benefit from NW-based The IE is not needed in the INTER RAT HANDOVER INFO message. | REL-6 |
| Support for System Information Block type 11bis | OP |  | Enumerated (TRUE) |  | REL-6 |
|  |  |  |  | The IE shall be present and set to TRUE | REL-7 |
| Support for F-DPCH | CV-SupportForHS |  | Enumerated (TRUE) | Presence of this information element indicates that F-DPCH is supported by the UE. Absence of this information element indicates that F-DPCH is not supported | REL-6 |
| MAC-ehs support | OP |  | Enumerated (TRUE) | The presence of this IE indicates that UE supports MAC-ehs, the use of special value of HE field to indicate end of an SDU for RLC AM, octet aligned transport block table and different HS-SCCHs in contiguous TTIs. The absence of this IE indicates that the UE does not support either MAC-ehs, octet aligned transport block table or the use of special value of HE field to indicate end of an SDU for RLC AM or different HS-SCCHs in contiguous TTIs. | REL-7 |
| UE specific capability Information LCR TDD | OP |  | Enumerated (NF, TriRxUniTx, TriRxTriTx, HexRxUniTx, HexRxTriTx, HexRxHexT, | For 1.28 Mcps TDD only | REL-7 |
|  |  |  | TwoRxUniTxDiscontiguous, TwoRxTwoTxDiscontiguous, TwoRxUniTxContiguous, TwoRxTwoTxContiguous) | TwoRxUniTxDiscontiguous and TwoRxTwoTxDiscontiguous mean that the UE is capable of supporting two non-adjacent carriers. TwoRxUniTxContiguous and TwoRxTwoTxContiguous mean that the UE is only capable of supporting two adjacent carriers. 6 spare values needed. Note 3. | REL-10 |
| Support for E-DPCCH Power Boosting | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the E-DPCCH Power Boosting is not supported. The presence of this IE indicates that the E-DPCCH Power Boosting is supported by the UE. | REL-7 |
| Support of common E-DCH | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support E-DCH enhanced random access in CELL_FACH state and Idle mode. | REL-8 |
| Support of MAC-i/is | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support MAC-i/is operation. | REL-8 |
| Support of Common E-RGCH based interference control | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support Common E-RGCH based interference control in CELL_FACH state. | REL-11 |
| Support of Fallback to R99 PRACH | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support Fallback to R99 PRACH in CELL_FACH state and IDLE mode. | REL-11 |
| Support of Concurrent deployment | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support Concurrent deployment of 2ms and 10ms TTI in a cell in CELL_FACH state and IDLE mode. | REL-11 |
| Support of TTI alignment and Per HARQ process | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support TTI alignment and Per HARQ process activation and de-activation in CELL_FACH state and IDLE mode. | REL-11 |
| Support of SPS operation | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | For 1.28 Mcps TDD only. The absence of this IE indicates that the UE does not support SPS operation. | REL-8 |
| Support of Control Channel DRX operation | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | For 1.28 Mcps TDD only. The absence of this IE indicates that the UE does not support Control Channel DRX operation | REL-8 |
| Support of CSG | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support access control based on CSG | REL-8 |
| Support for Two DRX schemes in URA_PCH and CELL_PCH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support Two DRX schemes in URA_PCH and CELL_PCH | REL-7 |
| Support for E-DPDCH power interpolation formula | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support E-DPDCH power interpolation formula when 16QAM is not configured | REL-7 |
| Support for absolute priority based cell re-selection in UTRAN | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support absolute priority based cell re-selection to UTRA inter-frequency | REL-8 |
| Support of MU-MIMO | OP |  | Enumerated (Uplink, Uplink and Downlink) | The absence of this IE indicates that the UE does not support MU-MIMO. This IE is used for 1.28 Mcps TDD only | REL-10 |
| Radio Access Capability Band Combination List | OP | 1 to 16 |  | The absence of this IE indicates that the UE does not support Dual Band Operation. | REL-9 |
| >Band Combination | MP |  | Integer(1..256) | The integer value n indicates that the nth DB-DC Configuration (A,B) in table 5.0AA in [21] is supported by the UE. | REL-9 |
| >Support for Dual Band Dual Cell E-DCH operation | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support Dual Band Dual Cell E-DCH operation on the corresponding band combination indicated by the IE "Band Combination". | REL-13 |
| >Supported Carrier Combination | OP |  |  | Absence of this IE means that the UE only supports the carrier combination (1,1) for this band combination (A,B). If carrier combination (X,Y) is supported, then carrier combination (M,N) is supported, where 1<=M<=X and 1<=N<=Y, so the corresponding value for that carrier combination (M,N) should be set to TRUE. Carrier combination (X,Y) for a band combination (A,B) refers to a block of X contiguous carriers in Band A and a block of Y contiguous carriers in Band B. | REL-10 |
| >>Carrier Combination (1,2) | MP |  | Boolean | The value TRUE means that carrier combination (1,2) is supported for this band combination (A,B). | REL-10 |
| >>Carrier Combination (2,1) | MP |  | Boolean | The value TRUE means that carrier combination (2,1) is supported for this band combination (A,B). | REL-10 |
| >>Carrier Combination (1,3) | MP |  | Boolean | The value TRUE means that carrier combination (1,3) is supported for this band combination (A,B). | REL-10 |
| >>Carrier Combination (3,1) | MP |  | Boolean | The value TRUE means that carrier combination (3,1) is supported for this band combination (A,B). | REL10 |
| >>Carrier Combination (2,2) | MP |  | Boolean | The value TRUE means that carrier combination (2,2) is supported for this band combination (A,B). | REL-10 |
| >>Carrier Combination (1,4) | MP |  | Boolean | The value TRUE means that carrier combination (1,4) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (4,1) | MP |  | Boolean | The value TRUE means that carrier combination (4,1) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (1,5) | MP |  | Boolean | The value TRUE means that carrier combination (1,5) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (5,1) | MP |  | Boolean | The value TRUE means that carrier combination (5,1) is supported for this band combination (A,B). | REL11 |
| >>Carrier Combination (1,6) | MP |  | Boolean | The value TRUE means that carrier combination (1,6) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (6,1) | MP |  | Boolean | The value TRUE means that carrier combination (6,1) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (1,7) | MP |  | Boolean | The value TRUE means that carrier combination (1,7) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (7,1) | MP |  | Boolean | The value TRUE means that carrier combination (7,1) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (2,3) | MP |  | Boolean | The value TRUE means that carrier combination (2,3) is supported for this band combination (A,B). | REL11 |
| >>Carrier Combination (3,2) | MP |  | Boolean | The value TRUE means that carrier combination (3,2) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (2,4) | MP |  | Boolean | The value TRUE means that carrier combination (2,4) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (4,2) | MP |  | Boolean | The value TRUE means that carrier combination (4,2) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (2,5) | MP |  | Boolean | The value TRUE means that carrier combination (2,5) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (5,2) | MP |  | Boolean | The value TRUE means that carrier combination (5,2) is supported for this band combination (A,B). | REL11 |
| >>Carrier Combination (2,6) | MP |  | Boolean | The value TRUE means that carrier combination (2,6) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (6,2) | MP |  | Boolean | The value TRUE means that carrier combination (6,2) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (3,3) | MP |  | Boolean | The value TRUE means that carrier combination (3,3) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (3,4) | MP |  | Boolean | The value TRUE means that carrier combination (3,4) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (4,3) | MP |  | Boolean | The value TRUE means that carrier combination (4,3) is supported for this band combination (A,B). | REL11 |
| >>Carrier Combination (4,4) | MP |  | Boolean | The value TRUE means that carrier combination (4,4) is supported for this band combination (A,B). | REL11 |
| >>Carrier Combination (3,5) | MP |  | Boolean | The value TRUE means that carrier combination (3,5) is supported for this band combination (A,B). | REL-11 |
| >>Carrier Combination (5,3) | MP |  | Boolean | The value TRUE means that carrier combination (5,3) is supported for this band combination (A,B). | REL-11 |
| >Supported Multiflow Combination | OP |  |  | Presence of this IE indicates that UE supports Multiflow for this band combination. Note 5. | REL-11 |
|  |  |  |  | Note 6 | REL-13 |
| >>Number of cells in band A | MP |  | Integer(1..6) | Total number of cells in band A that can be configured to a UE as a part of Multiflow configuration. In this version of the specification, the maximum value is 2. | REL-11 |
|  |  |  |  | In this version of the specification, the maximum value is 3. | REL-13 |
| >>Number of frequencies in band A | MP |  | Integer(1..3) | Total number of frequencies in band A, across which the Multiflow can be configured. In this version of the specification, the maximum value is 1. | REL-11 |
|  |  |  |  | In this version of the specification, the maximum value is 2. | REL-13 |
| >>Number of cells in band B | MP |  | Integer(1..6) | Total number of cells in band B that can be configured to a UE as a part of Multiflow configuration. In this version of the specification, the maximum value is 2. | REL-11 |
| >>Number of frequencies in band B | MP |  | Integer(1..3) | Total number of frequencies in band B, across which the Multiflow can be configured. In this version of the specification, the maximum value is 1. | REL-11 |
| Support of multi-cell configuration in inter-RAT handover | CV-iRAT_HoInfo |  | Enumerated (TRUE) |  | REL-11 |
| Support of TX Diversity on DL Control Channels by MIMO Capable UE when MIMO operation is active | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the MIMO capable UE does not support TX diversity on DL Control Channels (HS-SCCH, F-DPCH, E-AGCH, E-HICH, E-RGCH) when MIMO is active, P-CPICH is configured on antenna 1, and S-CPICH on antenna 2. | REL-7 |
|  |  |  |  | The absence of this IE indicates that the MIMO capable UE does not support TX diversity on DL Control Channels (HS-SCCH, F-DPCH, E-AGCH, E-HICH, E-RGCH, F-TPICH) when MIMO is active, P-CPICH is configured on antenna 1, and S-CPICH on antenna 2. | REL-11 |
| Support of enhanced TS0 | OP |  | Enumerated (TRUE) | For 1.28 Mcps TDD only. The absence of this IE indicates that the UE does not support the enhanced TS0 | REL-9 |
| Support for cell-specific Tx diversity configuration for dual-cell operation | CV- Dual_Cell_HSDPA_supported |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support cell-specific Tx diversity configuration when configured for dual-cell operation. | REL-8 |
| CSG proximity indication capability | CV-not_iRAT_HoInfo |  | CSG proximity indication capability 10.3.3.8a |  | REL-9 |
| Neighbour Cell SI acquisition capability | CV-not_iRAT_HoInfo |  | Neighbour Cell SI acquisition capability 10.3.3.21c |  | REL-9 |
| Extended measurements Support | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | TRUE means that the UE supports the extended measurement identity range 17 to 32. | REL-9 |
| Support for dual cell with MIMO operation in different bands | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support dual cell with MIMO operation in different bands | REL-10 |
| UE based network performance measurements parameters | CV-not_iRAT_HoInfo |  | UE based network performance measurements parameters 10.3.3.53 |  | REL-10 |
| Support of UTRAN ANR | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support UTRAN ANR. | REL-10 |
| IMS Voice capability | CV-not_iRAT_HoInfo |  | IMS Voice capability 10.3.3.14b |  | REL-9 |
| Multiflow capability | CV-not_iRAT_HoInfo |  | UE Multiflow capability 10.3.3.21ba |  | REL-11 |
| Support of MAC-ehs window size extension | OP |  | Enumerated (TRUE) | For 1.28 Mcps TDD only, the absence of this IE indicates that the UE does not support IE "MAC-ehs window size extension". Note 4. | REL-9 |
| Support of UM RLC re-establishment via reconfiguration | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support UM RLC re-establishment via reconfiguration. | REL-10 |
| Support of HS-DPCCH power offset extension | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support the HS-DPCCH power offset extension. | REL-11 |
| Support of STTD on DL Control Channels when Multiflow operation is active | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that a Multiflow capable UE does not support STTD on DL Control Channels (HS-SCCH, F-DPCH, E-AGCH, E-HICH, E-RGCH, F-TPICH) when Multiflow operation is active. | REL-11 |
| Support of F-TPICH feedback from the Multiflow assisting cell | CV-not_iRAT_HoInfo |  | Enumerated(TRUE) | The absence of this IE indicates that the UE does not support reception of the F-TPICH feedback from the Multiflow assisting cell. | REL-12 |
| Support of DSAC and PPAC update in CELL_DCH | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support DSAC and PPAC update in CELL_DCH. | REL-12 |
| Support of retrievable configurations | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support retrievable configurations. | REL-13 |
| Support of URA_PCH with seamless transition | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support URA_PCH with seamless transition. | REL-13 |
| Support of improved synchronized RRC procedures | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support improved synchronized RRC procedures. | REL-13 |
| Support of enhanced state transition | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support enhanced state transition. | REL-13 |
| Support of simultaneous setup and release of RABs and RBs | CV-not_iRAT_HoInfo |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support simultaneous setup and release of RABs and RBs. | REL-14 |
| Note 1: The second entity of the "RF capability TDD" is not needed in the INTER RAT HANDOVER INFO message: if both TDD 3.84/7.68 Mcps and TDD 1.28 Mcps are supported, the "RF capability TDD 1.28 Mcps" entity shall be used for TDD 1.28 Mcps; the "UE power class" in the "RF capability TDD" entity shall apply for both chip rates. |  |  |  |  |  |
| Note 2: For 1.28 Mcps TDD, UE supporting E-DCH enhanced random access in CELL_FACH state and Idle mode always supports HS-PDSCH reception in CELL_FACH, CELL_PCH and URA_PCH state and CELL_FACH HS-DSCH DRX operation. |  |  |  |  |  |
| Note 3: Two non-adjacent carriers can be in the same band or in two different bands. |  |  |  |  |  |
| Note 4: This IE should always be set to “TRUE” in this version of the specification. |  |  |  |  |  |
| Note 5: If the Multiflow scenario dual-band three cells on two frequencies is supported in band combination A&B (2 cells in band A and 1 cell in band B), then it is also supported in band combination B&A (2 cells in band B and 1 cell in band A). |  |  |  |  |  |
| Note 6: If the Multiflow scenario dual-band four cells on three frequencies is supported in band combination A&B (with 3 cells in band A and 1 cell in band B), then it is also supported in band combination B&A (with 3 cells in band B and 1 cell in band A), and in band combination A&B (with 2 cells in band A and 2 cells in band B), and in band combination B&A (with 2 cells in band B and 2 cells in band A). |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| not_rrc_connectionSetupComplete | The IE is not needed in the RRC CONNECTION SETUP COMPLETE message. Otherwise the IE is mandatory present. |
| not_iRAT_HoInfo | The IE is not needed in the INTER RAT HANDOVER INFO message. Otherwise, it is optional. |
| iRAT_HoInfo | The IE is optional in the INTER RAT HANDOVER INFO message. Otherwise, the IE is not needed. |
| Dual_Cell_HSDPA_supported | This IE is optionally present if Dual-Cell HSDPA is supported. Otherwise it is not needed. The IE is not needed in the INTER RAT HANDOVER INFO message. Otherwise, it is optional. |
| SupportForHS | This IE is mandatory present if the UE supports HS-PDSCH, otherwise it is not needed. The IE is not needed in the INTER RAT HANDOVER INFO message. |

#### 10.3.3.42o	UE radio access capability compressed

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Access stratum release indicator | MP |  | Enumerated (R99, REL-4, REL-5, | 3 spare values are needed. | REL-5 |
|  |  |  | REL-6, |  | REL-6 |
|  |  |  | REL-7, |  | REL-7 |
|  |  |  | REL-8, |  | REL-8 |
|  |  |  | REL-9, |  | REL-9 |
|  |  |  | REL-10, |  | REL-10 |
|  |  |  | REL-11, |  | REL-11 |
|  |  |  | REL-12, |  | REL-12 |
|  |  |  | REL-13, |  | REL-13 |
|  |  |  | REL-14, |  | REL-14 |
|  |  |  | REL-15, |  | REL-15 |
|  |  |  | REL-16) |  | REL-16 |
| Total AM RLC buffer size exceeds 10 kByte | MP |  | BOOLEAN |  | REL-5 |
| RF capability compressed | MP |  | RF capability compressed 10.3.3.32a |  | REL-5 |
| Support for System Information Block type 11bis | OP |  | Enumerated (TRUE) |  | REL-6 |
|  |  |  |  | The IE shall be present and set to TRUE | REL-7 |
| MAC-ehs support | OP |  | Enumerated (TRUE) | The presence of this IE indicates that UE supports MAC-ehs, the use of special value of HE field to indicate end of an SDU for RLC AM, octet aligned transport block table and different HS-SCCHs in contiguous TTIs. Absent if either MAC-ehs, octet aligned transport block table or the use of special value of HE field to indicate end of an SDU for RLC AM or different HS-SCCHs in contiguous TTIs are not supported | REL-7 |
| Security capability indication | OP |  | Enumerated (TRUE) | TRUE indicates UE security capabilities beyond R99 requirements, see 10.3.3.37 | REL-7 |
| GANSS support indication | OP |  | Enumerated (TRUE) | TRUE indicates that the UE supports GANSS | REL-7 |

3rd Modification

## 11.3	Information element definitions

InformationElements DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

hiPDSCHidentities,

hiPUSCHidentities,

hiRM,

maxAC,

maxAdditionalMeas,

maxAddPos,

<Partially omitted>

-- ***************************************************

--

--     USER EQUIPMENT INFORMATION ELEMENTS (10.3.3)

--

-- ***************************************************

AccessGroupIdentity ::=				INTEGER (0..15)

AccessStratumReleaseIndicator ::=	ENUMERATED {

rel-4, rel-5, rel-6, rel-7, rel-8,

rel-9, rel-10, rel-11, rel-12,

rel-13, rel-14, rel-15, rel-16, spare3,

spare2, spare1 }

-- TABULAR : for ActivationTime, value 'now' always appear as default, and is encoded

-- by absence of the field

ActivationTime ::=					INTEGER (0..255)

End of Modifications