3GPP TSG RAN meeting #89e										RP-201758

Electronic Meeting, September 14 - 18, 2020

## Status Report to TSG

Agenda item:			9.8.10

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_IAB_enh |  |  |  |  |
| Unique ID | 860050 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-201293 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 06/2021 | Performance part: 12/2021 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 5% | Performance Part: 0% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN-2 |
| --- | --- | --- |
| Rapporteur | Name | Georg Hampel |
|  | Company | Qualcomm Incorporated |
|  | Email | ghampel@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1 #102-e

Conclusion

At least the inter-carrier DC scenario can be considered in Rel-17. Further discussion in RAN3/RAN Plenary may be necessary for the intra-carrier DC scenario.

Agreement

Reuse by IAB-MT of existing Inter-frequency DC is considered as a starting point to support concurrent BH links to two parents.

- FFS: Reuse of multi-TRP transmission resource allocation features (if intra-freq DC scenario is supported for IAB)

- FFS: Additional specification effort to support IAB

Agreement

At least existing Rel-16 bands supporting IAB can be considered when evaluating the feasibility/impact of supporting different multiplexing cases.

Agreement

The Rel-16 semi-static and dynamic resource allocation mechanisms are the starting point for supporting Rel-17 multiplexing cases.

- FFS: Applicability for different IAB-DU resource types

- FFS: Cell-specific/semi-static signals and channels at the IAB-DU and/or IAB-MT

Agreement

- Based on the WID, the following multiplexing cases are in scope for potential support in Rel-17:

  - Multiplexing Case A: Simultaneous MT-Tx/DU-Tx

  - Multiplexing Case B: Simultaneous MT-Rx/DU-Rx

  - Multiplexing Case C: Simultaneous MT-Rx/DU-Tx

  - Multiplexing Case D: Simultaneous MT-Tx/DU-Rx

- Further study for for Case A and Case B at least the following scenarios:

  - Single or multi-panel IAB nodes operating in unpaired spectrum (FR1 and FR2 bands)

- Further study for Case C and Case D at least for the following scenarios:

  - Multi-panel IAB nodes operating in unpaired spectrum (FR1 and FR2 bands)

- FFS: Required level of specification impact to support the different cases. Any additional specification support in Rel-17 should be conditioned on feasibility from an interference and reliability perspective on a per-link and network basis

Agreement

- Case 7 timing is supported in Rel-17 for IAB-nodes operating in multiplexing scenario Case 2 (simultaneous MT-Rx/DU-Rx)

- Case 6 timing is supported in Rel-17 for IAB-nodes operating in multiplexing scenario Case 1 (simultaneous MT-Tx/DU-Tx)

  - RAN1 should strive to minimize specification impact due to this feature

- FFS: Whether Case 7 timing is supported in Rel-17 for IAB-nodes operating in multiplexing scenario Case 4 (simultaneous MT-Tx/DU-Rx)

#### 2.1.2	Remaining Open issues

10% of the items defined in the RAN1 WID objectives have been accomplished. The following items defined in the WID objectives require further work:

Duplexing enhancements:

- Specification of enhancements to the resource multiplexing between child and parent links of an IAB node.

- Specification of IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed, to support simultaneous operation (transmission and/or reception) by IAB-node’s child and parent links.

## 2.2	RAN2

#### 2.2.1	Agreements

None. Work has not started.

#### 2.2.2	Remaining Open issues

All objectives from WID on enhancements to IAB for NR.

## 2.3	RAN3

#### 2.3.1	Agreements

RAN3 #109-e

Agreements on topology adaptation enhancements:

- The following cases for inter-donor migration are studied:

a) IAB-MT is migrated between IAB-donors.

b) IAB-MT is simultaneously connected to two IAB-donors

c) IAB-DU is simultaneously connected to 2 donor-CUs (common understanding is that we won’t break F1 interface principles)

d) IAB-MT performs RLF recovery at new IAB-donor.

- The migration mechanism should allow to migrate to another donor all or some devices (the IAB nodes and/or UEs directly or indirectly served by the top-level IAB node).

- We assume that all parent-child relations are retained at the new donor

- UEs and IAB-MTs should not be forced into connection re-establishment in order to migrate to a new donor

- The following information should be made available to the new donor:

1. Contexts of all involved UEs,

2. Contexts of all involved MTs,

3. Contexts of all involved DUs,

4. Backhaul and topology-related information,

5. IP address information

- Current signaling is taken as baseline for inter-donor migration of UEs and IAB-MTs

- As baseline, IAB-MT migration should use a separate procedure w.r.t. the migration of the co-located IAB-DU, the served UEs and the served MTs

Agreements on reduction of service interruption:

- Topological redundancy should be considered as one mean among others for service interruption reduction.

- We shall consider how to reconfigure descendant nodes in order to reduce service interruption during migration

- Discuss mitigation of packet loss and reduction of unnecessary transmissions during IAB-node migration.

Agreements on topology redundancy:

- Consider Scenario 1 and 2 for CP/UP separation:

  - Scenario 1: F1-C via M-NG-RAN node (non-donor node) + F1-U via S-NG-RAN node (donor node)

  - Scenario 2: F1-U via M-NG-RAN node (donor node) + F1-C via S-NG-RAN node (non-donor node)

- Analyze Scenario 1 and Scenario 2 for inter-Donor Topology Redundancy, with the principle that an IAB-DU only have F1 interface with one Donor-CU:

  - Scenario 1: the IAB is multi-connected with 2 Donors.

  - Scenario 2: the IAB’s parent/ancestor node is multi-connected with 2 Donors.

- Routing Enhancement via descendant node can be discussed later or after RAN2 decision.

- Local re-routing scenario other than RLF can be discussed later or after RAN2 decision.

- Inter-Donor-DU re-routing can be discussed later or after RAN2 decision.

- Deprioritize Multi-Route Support with data split in IAB.

- Multi-MT Support is FFS in RAN3 pending RAN2

Agreements on transport enhancements:

- UP-based and CP-based approaches for DL congestion mitigation in IAB networks are complementary.

- In IAB DL end-to-end flow control, the access node sends feedback to the donor-CU-UP.

- Discuss the improvements to DDDS for IAB UP-based congestion mitigation (e.g. packet marking, highest PDCP SN received from parent node, receiving data rate, received data volume).

- The measures taken by the donor-CU-CP based on the CP-based approach are up to implementation.

- End-to-end UL flow control is deprioritized in Rel17.

Agreements on support for duplexing enhancements:

- IAB-specific CLI management should be considered in Rel-17. The specific scenarios should be first defined in RAN1 and potential enhancements should take the existing Rel-16 CLI measurements/signaling as the starting point.

- Wait for RAN1 on SDM/FDM support

#### 2.3.2	Remaining Open issues

5% of the items defined in the RAN3 WID objectives have been accomplished. All objectives from WID on enhancements to IAB for NR require further work.

## 2.4	RAN4

#### 2.4.1	Agreements

None. Work has not started.

#### 2.4.2	Remaining Open issues

All objectives from WID on enhancements to IAB for NR.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1 #102-e

| TDoc | Title | Source |
| --- | --- | --- |
| R1-2005260 | Resource multiplexing between backhaul and access for IAB duplexing enhancements | Huawei, HiSilicon |
| R1-2005261 | Enhancements for simultaneous operation of MT and DU | Huawei, HiSilicon |
| R1-2006382 | Discussions on IAB resource multiplexing enhancements | LG Electronics |
| R1-2006383 | Discussion on Other Enhancements for Simultaneous Operation | LG Electronics |
| R1-2006384 | Discussion on RACH procedure for IAB enhancement | LG Electronics |
| R1-2006403 | Considerations on enhancement of guard symbols for IAB | Huawei, HiSilicon |
| R1-2006581 | Power Control: It’s an Enhancement, and Here’s Our Views | Sharp |
| R1-2006903 | IAB enhancements to resource multiplexing | Ericsson |
| R1-2006904 | Other enhancements for simultaneous child and parent link operation in IAB | Ericsson |
| R1-2006905 | Enhancements to IAB for NR – overview of Rel-17 WI | Ericsson |
| R1-2007241 | Summary #1 of [102-e-NR-eIAB-01] | Moderator (AT&T) |
| R1-2007287 | Summary of [102-e-NR-eIAB-02] | Moderator (Qualcomm) |
| R1-2007310 | Summary#2 of [102-e-NR-eIAB-02] | Moderator (Qualcomm) |
| R1-2007322 | Summary #2 of [102-e-NR-eIAB-01] | Moderator (AT&T) |
| R1-2007352 | Summary#3 of [102-e-NR-eIAB-02] | Moderator (Qualcomm) |
| R1-2007388 | Session notes for 8.10 (Enhancements to Integrated Access and Backhaul) | Ad-hoc Chair (Samsung) |

RAN2 #111-e

| TDoc | Title | Source |
| --- | --- | --- |
| R2-2006624 | Consideration on Topology, Routing and Transport Enhancements | CATT |
| R2-2006625 | Consideration Issues on Inter-CU migration | CATT |
| R2-2006626 | Consideration on Enhancements to Reduce Service Interruption | CATT |
| R2-2006946 | Congestion handling and traffic splitting in IAB | Intel Corporation |
| R2-2006947 | Enhancements to establish efficient topologies | Intel Corporation |
| R2-2006948 | Backhaul failure recovery enhancments | Intel Corporation |
| R2-2006960 | Multi-hop scheduling and local routing enhancements for IAB | AT&T |
| R2-2006961 | Enhancements to support IAB topology adaptation | AT&T |
| R2-2006964 | Workplan for Rel-17 IAB | Qualcomm Incorporated (WI Rapporteur) |
| R2-2006965 | Simulations on fairness support in IAB topology | Qualcomm Incorporated |
| R2-2006966 | IAB flow and congestion control enhancements | Qualcomm Incorporated |
| R2-2006967 | Enhancements to BH RLF recovery for Rel-17 IAB | Qualcomm Incorporated |
| R2-2007019 | Topology optimization in IAB | NEC |
| R2-2007023 | Discussion on the Rel-17 scope of IAB enhancement | Fujitsu |
| R2-2007165 | Discussion on RLF handling enhancements | vivo |
| R2-2007166 | Discussion on congestion mitigation enhancements | vivo |
| R2-2007167 | Consideration of Inter-CU IAB Migration | vivo |
| R2-2007200 | Scoping out issues of topology-wide fairness, multi-hop latency and congestion mitigation | Samsung Electronics GmbH |
| R2-2007201 | On topology-wide fairness | Samsung Electronics GmbH |
| R2-2007295 | Consideration on routing enhancement | LG Electronics Inc. |
| R2-2007312 | Miscellaneous enhancements for IAB network | ZTE, Sanechips |
| R2-2007313 | Initial considerations on inter-donor migration | ZTE, Sanechips |
| R2-2007314 | Discussion on duplexing enhancement | ZTE, Sanechips |
| R2-2007487 | On the CP/UP separation | Nokia, Nokia Shanghai Bell |
| R2-2007488 | Inter-donor topology adaptation | Nokia, Nokia Shanghai Bell |
| R2-2007501 | Scope of topology adaptation issues for Rel-17 IAB | Samsung Electronics Romania |
| R2-2007658 | Other Enhancements to IAB for NR | Ericsson |
| R2-2007659 | User plane Latency in Multi-hop IAB Systems | Ericsson |
| R2-2007660 | Scenarios of topology adaptation for IAB network | Ericsson |
| R2-2007689 | Separation of CP/UP for improved CP robustness | AT&T |
| R2-2007773 | Initial consideration of topology adaptation enhancements for eIAB | Kyocera |
| R2-2007840 | Rel. 17 IAB enhancements for fairness, multi-hop latency reduction, and congestion mitigation | Futurewei Technologies |
| R2-2007863 | Consideration of inter-CU migration | Huawei, HiSilicon |
| R2-2007864 | Discussion on RLF handling issues | Huawei, HiSilicon |
| R2-2007865 | Further enhancements for R17 IAB | Huawei, HiSilicon |
| R2-2007984 | RAN2 impacts of Rel.17 IAB topology adaptation enhancements | Futurewei Technologies |
| R2-2008024 | Correction of text on measIdleCarrierListEUTRA and measIdleCarrierListNR | LG Electronics France |
| R2-2008025 | Enhancements of Topological Resilience | LG Electronics |
| R2-2008026 | BH RLF enhancements | LG Electronics |

RAN3 #109-e

| TDoc | Title | Source |
| --- | --- | --- |
| R3-204660 | Considerations on reducing signal overhead during Inter-donor IAB node migration | KDDI Corporation |
| R3-204698 | Considerations on differentiation inter-donor/intra-donor migration | KDDI Corporation |
| R3-204732 | Discussion on inter IAB donor-CU topology adaptation | CATT |
| R3-204733 | Discussion on group handover in inter IAB donor | CATT |
| R3-204734 | Discussion on reducing the service interruption for IAB | CATT |
| R3-204735 | Consideration on Multi-Hop Performance: QoS, Latency, Fairness | CATT |
| R3-204745 | Inter-CU topology adaptation procedure | Intel Corporation |
| R3-204746 | Multi-route support in IAB | Intel Corporation |
| R3-204747 | Enhancement to IAB DL Flow Control | Intel Corporation |
| R3-204794 | Work plan for Rel-17 IAB | Qualcomm Incorporated (WI Rapporteur) |
| R3-204795 | IAB inter-donor IAB-node migration | Qualcomm Incorporated |
| R3-204796 | Enhancements to intra-donor IAB-node-migration | Qualcomm Incorporated |
| R3-204797 | IAB-enhancements for topological redundancy | Qualcomm Incorporated |
| R3-204798 | IAB flow and congestion control enhancements | Qualcomm Incorporated |
| R3-204799 | Simulations on fairness support in IAB topology | Qualcomm Incorporated |
| R3-204800 | IAB support for RAN shared between PLMN and NPN | Qualcomm Incorporated |
| R3-204867 | Separation of CP/UP for improved CP robustness | AT&T |
| R3-204868 | Congestion Indication to CU-CP | AT&T |
| R3-204869 | Support for CLI management by IAB nodes | AT&T |
| R3-204870 | Support for IAB resource multiplexing enhancements | AT&T |
| R3-204917 | Inter-Donor CU topology adaptation | Lenovo,Motorola Mobility |
| R3-205063 | Consideration on IAB Node Migration for Mobile Scenarios | LG Electronics |
| R3-205064 | Consideration on topological redundancy | LG Electronics |
| R3-205162 | Discussion on inter-donor IAB-node migration procedure | ZTE, Sanechips |
| R3-205163 | Discussion on service interruption reduction in IAB node migration | ZTE, Sanechips |
| R3-205164 | Consideration on R17-IAB mobility enhancement | ZTE, Sanechips |
| R3-205167 | Considerations on CP/UP separation | ZTE, Sanechips |
| R3-205168 | Consideration on multi-hop latency for IAB network | ZTE, Sanechips |
| R3-205169 | Consideration on R17-IAB E2E flow control | ZTE, Sanechips |
| R3-205170 | Discussion on fairness in IAB scheduling | ZTE, Sanechips |
| R3-205171 | Discussion on IAB duplexing enhancements | ZTE, Sanechips |
| R3-205172 | Discussion on RAN-sharing in IAB network | ZTE, Sanechips |
| R3-205221 | Inter-donor Migration in IAB Networks – General Principles | Ericsson |
| R3-205222 | Inter-donor Migration Mechanism in IAB Networks | Ericsson |
| R3-205223 | Handling of In-flight Packets at Handover in IAB Networks | Ericsson |
| R3-205224 | Multi-connectivity for IAB Nodes | Ericsson |
| R3-205225 | Downlink End-to-End Flow Control in IAB Networks | Ericsson |
| R3-205226 | CLI Management Support for IAB | Ericsson |
| R3-205249 | Consideration on RLF Recovery | LG Electronics |
| R3-205251 | Issue on inter-donor IAB-node migration | LG Electronics |
| R3-205264 | Discussion on Inter-Donor IAB Node Migration | Nokia, Nokia Shanghai Bell |
| R3-205265 | discussion on Inter-CU topology redundancy | Nokia, Nokia Shanghai Bell |
| R3-205292 | Inter-CU handover procedure analysis | Huawei |
| R3-205293 | Inter-CU RLF recovery procedure | Huawei |
| R3-205294 | CHO and DAPS in R17 IAB | Huawei |
| R3-205295 | Discussion on IAB topological redundancy | Huawei |
| R3-205296 | Routing enhancement for IAB | Huawei |
| R3-205297 | Discussion on IAB E2E flow control | Huawei |
| R3-205352 | Considerations on IAB-DU configuration update during Inter-CU migration | ZTE, Sanechips |
| R3-205411 | Discussion on inter-donor IAB node migration | Samsung |
| R3-205412 | Discussion on Service Interruption Reduction in IAB | Samsung |
| R3-205413 | CR on service interruption reduction for intra-CU IAB migration | Samsung |
| R3-205414 | Discussion on topology redundancy in IAB | Samsung |
| R3-205415 | CR on CP-UP separation over Xn (Rel-17) | Samsung |
| R3-205416 | Discussion on Congestion Mitigation in IAB | Samsung |
| R3-205417 | Discussion on latency reduction in IAB | Samsung |
| R3-205418 | Discussion on PDCP duplication support in IAB | Samsung |
| R3-205425 | Inter-CU BH RLF recovery procedure for IAB node | Huawei |
| R3-205466 | CB: # 6_IAB_Inter-DonorMigration - Summary of email discussion | Ericsson - moderator |
| R3-205467 | CB: # 7_IAB_ServiceInterruptionReduction - Summary of email discussion | Qualcomm - moderator |
| R3-205468 | CB: # 8_IAB_TopologyRedundancy - Summary of email discussion | Nokia - moderator |
| R3-205469 | CB: # 9_IAB_CongestionMitigation - Summary of email discussion | Ericsson - moderator |
| R3-205470 | CB: # 10_IAB_MultiHopPerf - Summary of email discussion | CATT - moderator |
| R3-205471 | CB: # 11_IAB_CLImgmt - Summary of email discussion | AT&T - moderator |
| R3-205472 | CB: # 12_IAB_Others_RANsharing - Summary of email discussion | ZTE - moderator |
| R3-205488 | CB: # 13_IAB_MPXenh - Summary of email discussion | ZTE - moderator |

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template